3GPP TSG RAN WG1 #106bis-e			R1-2109499

# e-Meeting, October 11th – 19th, 2021

Agenda Item:		8.6.2

Source:		Samsung

Title:		RAN1 aspects for RAN2-led features for RedCap

Document for:	Discussion and decision

# Introduction

The following objectives were agreed in RAN#91-e [1].

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1] - The existing UE capability framework is used; changes to capability signalling are specified only if necessary. - Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] - Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

This contribution discusses remaining RAN1 aspects for RAN2-led features for RedCap related to the above objectives. RAN1#106-e agreements are listed in the Appendix.

# Definition of RedCap UE type

In RAN1#106-e it was agreed the following:

Agreement - A RedCap UE type from RAN1 point of view supports a maximum bandwidth of 20MHz for FR1 and 100MHz for FR2 - Further discuss whether to capture also one or more of the following capabilities to RedCap UE type description - Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers - Supports either FD-FDD or Type A HD-FDD operation for FR1 FDD bands - Supports either DL up to 64 QAM or up to 256 QAM for FR1 - Does not support CA/DC

We think that further discussion should happen in UE features AI.

# Early indication of RedCap UEs

In RAN1#106-e it was agreed the following:

Agreement: Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - FFS how to support enable/disable the early indication - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning - FFS: whether/how to address RA-RNTI overlapping issue - FFS the possibility of supporting Msg3 for the early indication Whether/how to support early indication of RedCap Ues in Msg3 in Rel-17 is up to RAN2 Conclusion - Whether there is RA-RNTI overlapping issue and how to address RA-RNTI overlapping issue in the early indication of RedCap UEs in Msg1 in Rel-17 is up to RAN2.

In RAN2#115-e it was agreed the following:

Agreements online: A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem)

Regarding the early identification in Msg3, since RAN2 has agreed a solution to support identification in Msg3, we think that no further discussion is needed in RAN1.

Regarding the early identification in Msg1, separate PRACH resources, PRACH preamble partitioning or separate initial UL BWPs can be used. In RAN1#106-e it was also agreed that separate PRACH resources or PRACH preamble partitioning can be used also within an UL BWP that is an initial UL BWP only used by RedCap UEs. One consideration on the use of separate PRACH preambles or PRACH resources associated to RedCap UEs is that with separate ROs (configured by a new PRACH configuration index) the regular RACH procedure is affected in the sense that a gNB needs to accommodate multiple SSB-RO mapping patterns, including the association of beams to the RO configuration. The use of separate PRACH preambles is preferable since it does not affect the Rel-15/16 SSB-RO association, however both separate PRACH preambles and separate ROs can be allowed and the gNB can configure one or the other, or a combination of PRACH preambles and ROs. This would give additional flexibility to the gNB scheduling and use different configurations depending on whether the initial UL BWP is shared or separate for RedCap UEs and non-RedCap UEs.

Proposal 1: Support at least separate sets of PRACH preambles for early identification of a RedCap UE in Msg1 for shared initial UL BWP and separate initial UL BWP (if supported).

In RAN1#105-e it was agreed the following:

Agreement: - Support 2-step RACH for RedCap UEs as an optional feature - FFS details of early indication in MsgA, e.g.: - Separation of 2-step RACH resources or MsgA preambles - Separation of initial UL BWP - Using a new indication in MsgA PUSCH part - Note: Discussion on 4-step RACH for early indication should be prioritised

When in a cell there are some RedCap UEs with 2-step RACH capability, separate sets of ROs and/or separate sets of PRACH preambles can be configured to be used with a 2-step RA procedure or a 4-step RA procedure. If for example separate sets of PRACH preambles are configured for early identification of RedCap UEs and non-RedCap UEs, the gNB can use separate set of ROs to be used by RedCap UEs that support/perform 2-step PRACH or by RedCap UEs that support/perform 4-step RACH. A UE that supports a 2-step RACH procedure and 4-step RACH procedure can choose whether to use 2-step RACH or 4-step RACH resources for the RA procedure. In case of separate UL BWPs, the access with 2-step RACH procedure can be possible in one or all UL BWPs.

Proposal 2: Support separate sets of PRACH preambles or separate sets of ROs for early identification of RedCap UEs supporting 2-step RACH procedure.

# Considerations on RAN2 LS R1-2108714 (R2-2109218)

From RAN2 LS Agreements online: - RAN2 Working Assumption: by default, all non-RedCap UE capabilities are applicable for RedCap UE, and therefore only for non-RedCap capabilities that are not applicable for RedCap UE, we clarify in the definitions for parameters in TS38.306, the value or feature is not applicable for RedCap UE

RAN1 sent an LS to RAN2 in R1-2108631 informing RAN2 about the RAN1 agreements in RAN1#106-e, which are consistent with the above RAN2 agreement - no further action is required.

From RAN2 LS Agreements: - The number of DRBs supported by RedCap UEs is less than legacy value (which is 16). There will be a single mandatory value (FFS if 4 or 8). FFS if it will be possible to have an optional capability - “RRC processing delay” is not relaxed for RedCap UE - PDCP/RLC AM 12 bits SN is mandatory for RedCap UE, and PDCP/RLC AM 18bits SN is optional supported by RedCap UE; FFS on how to capture this in specification - NE-DC, and (NG)EN-DC are not supported by RedCap UE; FFS on how to capture it in the specifications - DAPS and CAPC related capabilities are not applicable for RedCap UE; [8/20] FFS on CHO. FFS on how to capture this in the specification; Agreements via email - from offline 109: - Maximum 8 DRBs is mandatory supported by RedCap UEs. - From RAN2 perspective, inter RAT mobility related capabilities are applicable for RedCap UE; - From RAN2 perspective, measurement related capabilities are applicable for RedCap UE; - From RAN2 perspective, URLLC related capabilities are applicable for RedCap UE except those affected by CA/DC; - From RAN2 perspective, IAB related capabilities are not applicable for RedCap UE, i.e. the RedCap UE is not expected to act as IAB node; - Do not introduce capability signalling on the supported Rx number for RedCap UE since the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework;

In addition to the capabilities applicable to RedCap UEs mentioned above, RAN1 should discuss/confirm the applicability of Rel-17 features to RedCap UEs. Additional information on R17 features applicable to RedCap UEs may be added in a reply LS to RAN2.

Proposal 3: Further discuss applicability of Rel-17 features to RedCap UEs in RAN1, and include related information in a reply LS to RAN2.

# Conclusion

This contribution discusses RAN1 aspects of RAN2-led features for RedCap UEs. The observations made in this contribution are summarized as below:

Proposal 1: Support at least separate sets of PRACH preambles for early identification of a RedCap UE in Msg1 for shared initial UL BWP and separate initial UL BWP (if supported).

Proposal 2: Support separate sets of PRACH preambles or separate sets of ROs for early identification of RedCap UEs supporting 2-step RACH procedure.

Proposal 3: Further discuss applicability of Rel-17 features to RedCap UEs in RAN1, and include related information in a reply LS to RAN2.

# Reference

[1] RP-210918, “Revised WID on support of reduced capability NR devices”, RAN#91-e, March 2021.

[2] R1-2108714 (R2-2109218), “LS on capability related RAN2 agreements for RedCap”, RAN2#115-e, August 2021.

# Appendix

RAN1#106-e 8.6.2 Agreements

Agreement

Confirm the following working assumption with the modifications in red:

- For 4-step RACH, support the early indication of RedCap UEs at least in Msg1.

  - The early indication in Msg1 can be configured to be enabled/disabled via SIB

    - FFS how to support enable/disable the early indication

  - FFS details e.g.: From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported)

    - separate PRACH resource

    - PRACH preamble partitioning

    - FFS: whether/how to address RA-RNTI overlapping issue

  - FFS the possibility of supporting Msg3 for the early indication

Whether/how to support early indication of RedCap UEs in Msg3 in Rel-17 is up to RAN2.

Conclusion

- Whether there is RA-RNTI overlapping issue and how to address RA-RNTI overlapping issue in the early indication of RedCap UEs in Msg1 in Rel-17 is up to RAN2.

Agreement

- Send an LS to RAN2 informing RAN2-related agreements in AI8.6.2 in RAN1#106-e

  - FFS details

Conclusion

- There is no consensus in RAN1 on whether to have the access barring indication in DCI scheduling SIB1, and RAN1 can come back if triggered by RAN2.

Agreement

- For the RedCap UE capabilities, current definition of Rel-15/16 L1 UE capabilities mandatory without capability signalling in TR38.822 is reused by default, unless any update is agreed

  - Note: UE capabilities related to CA, DC and wider max UE bandwidth are not applicable to RedCap UEs

  - FFS: whether any L1 UE capabilities mandatory/optional with capability signalling are not applicable to RedCap UEs

Above agreement to be incorporated into agreed draft LS R1-2108615

Agreement

         A RedCap UE type from RAN1 point of view supports a maximum bandwidth of 20MHz for FR1 and 100MHz for FR2

         Further discuss whether to capture also one or more of the following reduced capabilities to RedCap UE type description

o    Supports either 1 or 2 Rx branches and corresponding maximum DL MIMO layers

o    Supports either FD-FDD or Type A HD-FDD operation for FR1 FDD bands

o    Supports either DL up to 64 QAM or up to 256 QAM for FR1

o    Does not support CA/DC

Above agreement to be incorporated into agreed draft LS R1-2108615

Final LS R1-2108631 is endorsed.