3GPP TSG RAN WG1 #106bis-e		   R1-2109638

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.14.1

Source: 	Intel Corporation

Title:	Performance evaluation results for XR and CG

Document for:       Discussion/Decision

- Introduction

In the previous RAN 1 meetings, substantial progress was achieved on traffic model and evaluation assumptions [1] [2] [3]. In this paper, we provide initial evaluation results for Uplink and Downlink capacity and coverage.

- Evaluation assumptions

In the following table, we summarize the assumptions used in the SLS evaluations.

| Parameter | Value |
| --- | --- |
| Layout | Dense Urban, 21cells with wraparound ISD: 200m |
| UE distribution | 80% indoor, 20% outdoor |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25m |
| UE height | hUT = 3(nfl – 1) + 1.5 Outdoor UEs: nfl = 1 Indoor UEs: nfl ~ uniform(1,N) with N ~ uniform(4,8) |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna parameters | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (2,1,2,4,8;4,8) |
| BS Tx Power | 44dBm for 20 MHz simulated |
| UE antenna pattern | Omni-directional, 0 dBi, |
| UE Antenna parameters | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,2;1,2), (dH, dV) = (0.5, 0.5)λ |
| Downtilt | 102 |
| TDD configuration | DDDSU |
| System BW | 20 MHz |
| Scheduler | MU-MIMO PF Scheduler |
| CSI | SRS based |
| Traffic | 30 Mbit/s video [6Mbits/s per 20 MHz simulated], 60 fps, [STD, Max, Min]: [10.5, 150, 50]% of mean packet size, truncated Gaussian |
| Jitter | STD = 2ms, Range [+/-4ms], truncated Gaussian |
| Packet Delay Budget (PDB) | 10ms (DL), 30ms, 10ms (UL) |

- Initial evaluation results

Initial evaluation results for Dense Urban scenario for Uplink and Downlink are shown in the following figures. For all figures in this section, each marker corresponds to the following number of UEs dropped per cell: [1, 5, 7, 10, 15] for Downlink and Uplink.

## 3.1 Capacity evaluation

## 3.1.1 Downlink

Evaluation results for CG, AR/VR single stream traffic are shown in this section. We consider the baseline evaluation assumptions agreed in [2], where the traffic model has 30Mbps data rate with 60 frames per second and the PDB requirement is 15ms for CG and 10ms for AR/VR.

Fraction of successful UEs

Figure 1: DU, 30 Mbps AR/VR, the capacity of a system with 10ms PDB is 7 UEs and 7.5 UEs with and without jitter, and that of a system with 15ms PDB is 7.4 UEs and 8.2 UEs with and without jitter. Capacity with PDB 15ms is slightly higher than PDB 10ms.

Observation 1: For DU Downlink Scenario VR/AR, 30Mbps, 10ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 1 System capacity of VR/AR (30Mbps) application in FR1 DL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 7 | 7 | 90% |  |
| 7.5 | 7 | 95.71% | Note 1 |
| Note 1. No jitter |  |  |  |

Observation 2: For DU Downlink Scenario CG, 30Mbps, 15ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 2 System capacity of CG (30Mbps) application in FR1 DL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 7.4 | 7 | 93.202% |  |
| 8.2 | 8 | 90.14% | Note 1 |
| Note 1. No jitter |  |  |  |

## 3.1.2 Uplink

Evaluation results for AR single stream are shown in this section for an aggregated traffic of scene/video/data/voice (Truncated Gaussian Model, 10Mbps, PDB = 30ms, 60 frames per second, no jitter).

Fraction of successful UEs

Figure 2: DU, 10Mbps AR/VR, the capacity of a system with 15ms PDB is lower than a system with 30ms PDB.

Observation 3: for DU Uplink Scenario scene/video/data/voice-stream, 10Mbps, 30ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 3 System capacity of scene/video/data/voice (10Mbps) application in FR1 UL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 14.7 | 14 | 91.33% |  |
| 14.47 | 14 | 90.79% | Note 1 |
| Note 1. No jitter |  |  |  |

Observation 4: In Dense Urban deployment scenario, XR capacity increases as the PDB increases for both DL and UL.

Observation 5: XR capacity decreases with jitter by 6.67 and 9.76 percent for DL VR/AR single stream and CG single stream respectively, and 1.56 percent for UL AR single stream.

## 3.2 Coverage evaluation

XR coverage is evaluated based on the optional methodology 2, where only one UE is randomly dropped in the entire network (or in all the cells) and associated with one of the 3 center cells (or gNBs). XR coverage is defined as 5%-tile point in the CDF curve of coupling gain for all the satisfied UEs. For downlink, a single stream model for VR/CG is used and for uplink pose/control stream model for AR is used.

| Figure 9: UMa, 30 Mbps VR for DL, and pose/control for UL. UL coverage is larger than DL. Coverage increases as the PDB increases. | Figure 10: DU, 30 Mbps VR for DL, and pose/control for UL. Coverage is limited by ISD. |
| --- | --- |

Observation 6: Based on the optional methodology 2, the coverage (the 5%-tile point in the CDF curve of coupling gain) is summarized in the table below.

Table 4 Coverage of CG and VR for FR1 Dense Urban

|  |  | DU | UMa |
| --- | --- | --- | --- |
| CG | DL | -135.5dB | -148.20dB |
|  | UL | -134.7dB | -140.05dB |
| VR | DL | -134.8dB | -144.65dB |
|  | UL | -134.6dB | -139.80dB |

Observation 7: For both DL and UL, XR coverage increases as the PDB is increased.

# 4. Conclusions

In this contribution, we provide an initial set of simulation results with the agreed traffic model and simulation assumptions for capacity KPI for the following cases:

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | AR (single stream) | 10 | 60 | 30 |

The observations are summarized below.

Observation 1: For DU Downlink Scenario VR/AR, 30Mbps, 10ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 1 System capacity of VR/AR (30Mbps) application in FR1 DL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 7 | 7 | 90% |  |
| 7.5 | 7 | 95.71% | Note 1 |
| Note 1. No jitter |  |  |  |

Observation 2: For DU Downlink Scenario CG, 30Mbps, 15ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 2 System capacity of CG (30Mbps) application in FR1 DL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 7.4 | 7 | 93.202% |  |
| 8.2 | 8 | 90.14% | Note 1 |
| Note 1. No jitter |  |  |  |

Observation 3: for DU Uplink Scenario scene/video/data/voice-stream, 10Mbps, 30ms PDB, 100MHz bandwidth, DDDSU TDD format, the capacity result is summarized as follows.

Table 3 System capacity of scene/video/data/voice (10Mbps) application in FR1 UL Dense Urban scenario

| MU-MIMO |  |  | Notes |
| --- | --- | --- | --- |
| Capacity | C1=floor(Capacity) | % of satisfied UEs when #UEs/cell =C1 |  |
| 14.7 | 14 | 91.33% |  |
| 14.47 | 14 | 90.79% | Note 1 |
| Note 1. No jitter |  |  |  |

Observation 4: In Dense Urban deployment scenario, XR capacity increases as the PDB increases for both DL and UL.

Observation 5: XR capacity decreases with jitter by 6.67 and 9.76 percent for DL VR/AR single stream and CG single stream respectively, and 1.56 percent for UL AR single stream.

Observation 6: Based on the optional methodology 2, the coverage (the 5%-tile point in the CDF curve of coupling gain) is summarized in the table below.

Table 4 Coverage of CG and VR for FR1 Dense Urban

|  |  | DU | UMa |
| --- | --- | --- | --- |
| CG | DL | -135.5dB | -148.20dB |
|  | UL | -134.7dB | -140.05dB |
| VR | DL | -134.8dB | -144.65dB |
|  | UL | -134.6dB | -139.80dB |

Observation 7: For both DL and UL, XR coverage increases as the PDB is increased.

# References

- Chairman’s Notes of RAN1 #104b-e

- Chairman’s Notes of RAN1 #105-e

- Chairman’s Notes of RAN1 #106-e