3GPP TSG RAN WG1 #106bis-e		R1- 2110258

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.12.3

Source:	ASUSTeK

Title:	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Document for:		Discussion and Decision

- Introduction

The support of NR Multicast and Broadcast Services (MBS) is introduced in Rel-17 as a work item [1] to achieve group-scheduling. The objectives of NR MBS are specified in [1] and some agreements on the basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs were reached at RAN1#102-e through RAN1#106-e meeting [2]~[7]. In this contribution, we discuss the remaining issues about supporting group-scheduling for RRC_IDLE/RRC_INACTIVE UEs for the MBS WI and some proposals are given.

- Discussion

2-1. Configurations for the CFR for broadcast reception

It was agreed in the last meeting [7] that at least one set of parameters configured for PDSCH for broadcast reception with GC-PDSCH and one set of parameters configured for PDCCH for broadcast reception with GC-PDCCH are included in the configuration of the CFR for broadcast reception for RRC_IDLE/INACTIVE UEs.

Agreement: From RAN1 perspective, the CFR for broadcast reception of RRC_IDLE/INACTIVE UEs, includes at least the following configurations: - One set of parameters configured for PDSCH for broadcast reception with GC-PDSCH - One set of parameters configured for PDCCH for broadcast reception with GC-PDCCH - FFS: whether some parameters configured for PDSCH/PDCCH are optional/needed for the supported cases of CFR. - FFS: If necessary, depending on the cases supported, starting PRB and the number of PRBs The reference for starting PRB is Point A. (Following the same approach to determine reference for starting PRB as that defined in AI8.12.1.)

In our opinion, the parameters should all be configured in a CFR-Config RRC IE, which is a per-CFR configuration. The starting PRB and the number of PRBs of the CFR should be configured in the CFR-Config. Also, the current SLIV indication mechanism can be reused to indicate the starting PRB and the number of PRBs of the CFR. If more than one broadcast service can be supported, a list of G-RNTI(s) that the UE will monitor on the CFR as well as the CFR-Config are signalled in SIB1 or the newly introduced MBS-specific SIBx. The scheduling throughput may not be the first priority of broadcast reception of RRC_IDLE/ INACTIVE UEs. Therefore, for example for pdsch-Config-Broadcast, only the basic parameters in the current PDSCH-Config are necessary for broadcast reception for RRC_IDLE/ INACTIVE UEs, e.g. pdsch-TimeDomainAllocationList, resourceAllocation, and rbg-Size, to simplify the implementation.

Proposal 1: The current SLIV indication mechanism can be reused to indicate the starting PRB and the number of PRBs of the CFR.

Proposal 2: Only the basic parameters in the current PDSCH-Config are necessary for broadcast reception for RRC_IDLE/ INACTIVE UEs, e.g. pdsch-TimeDomainAllocationList, resourceAllocation, and rbg-Size, to simplify the implementation.

2-1. MCCH change notification indication

The following agreement is reached at the RAN1#105-e meeting [6],

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start: - Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification; - Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification; Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2.

For MCCH change notification indication, two alternatives are given. Alt 1 uses a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification, which is not preferred in our opinion because it requires an RRC_IDLE/RRC_INACTIVE UE to constantly monitor an additional DCI with CRC scramble by an additional RNTI. In this case, power consumption is increased due to the extra monitoring of another RNTI. Instead, Alt 2 is preferred. Because the DCI size of GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH is aligned with DCI format 1_0 with CRC scrambled by C-RNTI in the CSS, some field(s), e.g. one or more HARQ feedback related field (if HARQ operation is not supported for broadcast), in DCI format 1_0 with CRC scrambled by C-RNTI can be transformed or re-interpreted as the MCCH change notification indication field.

Proposal 3: For MCCH change notification indication, only Alt 2 is supported.

- Conclusion

According to the discussion above, some observations and proposals are given.

Proposal 1: The current SLIV indication mechanism can be reused to indicate the starting PRB and the number of PRBs of the CFR.

Proposal 2: Only the basic parameters in the current PDSCH-Config are necessary for broadcast reception for RRC_IDLE/ INACTIVE UEs, e.g. pdsch-TimeDomainAllocationList, resourceAllocation, and rbg-Size, to simplify the implementation.

Proposal 3: For MCCH change notification indication, only Alt 2 is supported.

Reference

- RP-201038, ”WID revision: NR Multicast and Broadcast Services”, Huawei, HiSilicon.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #102-e, e-Meeting, August 17th - 28th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #103-e, e-Meeting, October 26th - November 13th, 2020.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104-e, e-Meeting, January 25th - February 5th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #104b-e, e-Meeting, April 12th – 20th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- Chairman Notes, 3GPP TSG RAN WG1 meeting #106-e, e-Meeting, August 16th – 27th, 2021.