3GPP TSG RAN WG1 #106bis-e                                                        R1-2110442

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.10.1

Source:	Moderator (AT&T)

Title:	Feature Lead Summary #1 of 8.10.1

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the discussion in RAN1#106-e for the following email discussion:

[106bis-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: October 14

- Final check point: October 19

# Simultaneous Operation of Access and Backhaul Links

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Company | Proposals |
| --- | --- |
| Huawei, HiSilicon R1-2108765 | Proposal 1: An IAB node shall report conditions for different multiplexing cases per {MT-CC, DU cell}-pair as follows: - Multiplexing case A (Simultaneous MT-Tx/DU-Tx): Case #6 timing, additional uplink PC parameters, guard band, and recommended Tx beam - Multiplexing case B (Simultaneous MT-Rx/DU-Rx): Additional downlink PC parameters, guard band, recommended Rx beam, and orthogonal DMRS ports - Multiplexing case C (Simultaneous MT-Rx/DU-Tx): Additional downlink PC parameters, guard band, recommended Rx beam, and orthogonal DMRS ports - Multiplexing case D (Simultaneous MT-Tx/DU-Rx): Additional uplink PC parameters, guard band, and recommended Tx beam Proposal 2: TCI state ID and SRS ID are adopted for the reporting of recommended DL beams and UL beams respectively based on MAC CE signaling. Proposal 3: To support the parent node indicating a set of restricted beams to IAB node, dedicated sounding reference signals (SRS) should be defined/configured. Proposal 4: To facilitate the co-existence of TDM and FDM slots and backward compatibility, a two-step H/S/NA configuration is supported in Rel-17 - Step 1: Configure time domain H/S/NA for each slot in one period by reusing Rel-16 mechanism - Step 2: Configure frequency domain H/S/NA using SLIV-based indication for a subset of the slots within one period, which overrides the time domain H/S/NA configuration in the slots Proposal 5: To support soft resource availability in the frequency domain, a single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell. Proposal 6: For intra-CC FDM, the guard band should be placed in MT resources. Proposal 7: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. Proposal 8: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes: - SS/PBCH block - CORESET for Type0-PDCCH CSS set - PRACH Proposal 9: Donor CU can provide two sets of resource configurations to each DU cell: a basic resource configuration for access UEs and one additional resource configuration for child IAB node MTs, and the DU cell should maintain two sets of DU resource configurations simultaneously. Proposal 10: Only Case #6 and Case#7 timing modes for IAB-MT should be taken into account for the enhancements of Desired/Provided Guard Symbols. |
| Nokia, Nokia Shanghai Bell R1-2108826 | Proposal 2.1: The minimum bandwidth resolution of an RB set is equal to the RBG size of the smallest BWP configured. Proposal 2.2: The number of RB sets that can be configured within a carrier is limited to M. - Appropriate values of M to be discussed Proposal 2.3: Rules specifying the interaction of TDM, FDM, SDM config, and multiplexing capability indication should be discussed after further details related to the operation of TDM, FDM, and SDM are finalized. Proposal 2.4: Support Alt. 1 by extending the number of availability combinations to support the number of RB sets configured within the available frequency resource (e.g. carrier, BWP). - FFS whether Rel-17 DCI format 2_5 shall be enhanced to support additional availability combinations Proposal 2.5: Indicate beam restrictions independently for nodes operating in case A and case B multiplexing modes. Proposal 2.6: Support explicit indication of DU SSB during multiplexing operation by a parent node towards a child IAB node. Proposal 2.7: Support indication of beam restriction via beam identifier on parent link (e.g. TCI state for DL, SRI for UL), such that the IAB DU determines corresponding beam restrictions via association. Proposal 2.8: Specify new MAC-CE for beam preference indication from an IAB node towards its parent DU. Proposal 2.9: Indication of preferred IAB MT beam (e.g., TCI state for DL, SRI for UL) is provided from an IAB node to a parent separately for case A and case B multiplexing mode. Proposal 2.10: Beam preference indication for DL beams can be indicated via TCI or SSB index. Beam preference for UL beams can be provided via SRI. Proposal 2.11: A beam indication should be assumed to be a request for a transmission either in UL or DL that will be fulfilled once a resource grant has been provided with the given beam configuration. Observation 2.1: Unless additional agreement is made a node operating in either case#6 or case#7 timing mode and changing link direction would necessitate a change in timing mode as well. Observation 2.2: For an IAB node switching from either Rel-17 enhanced multiplexing mode (e.g., case A or case B) requires only a single indication of required guard symbols when switching to Rel-16 TDM operation. Observation 2.3: When an IAB node switches between Rel-16 TDM multiplexing and Rel-17 case A multiplexing indication of desired guard symbols for changing operation within either the MT or DU is unnecessary. Observation 2.4: Required guard symbols for switching operation between Rel-17 case A multiplexing mode and Rel-16 TDM mode can be determined by the parent node via Rel-16 guard symbol indication and timing control for case #1 and case #6 timing mode. Proposal 2.12: Enhance MAC-CE for guard symbol indication to indicate, in symbols, the slot boundary offset of the UL Tx for the child node switching between case#7 and case#1, with details related to range of symbols discussed later. |
| vivo R1-2108995 | Proposal 1: IAB MT reports to parent node the supported timing mode, guard symbol, interference information, DL/UL power control information and desired DL/UL beam to assist parent node to indicate the applicability of a given multiplexing case. - The required guard symbol should be reported per timing mode. Proposal 2: Support both semi-static and dynamic adaptation of an IAB-node’s multiplexing operation on a given set of time/frequency resources. Proposal 3: Support to indicate the allowance of a given multiplexing case on a given set of time/frequency resources by parent node/CU. Proposal 4: Support IAB node to report the expected operation (e.g., simultaneous operation or TDM operation) on a given set of time/frequency resources to parent node/CU. Proposal 5: Rel-17 H/S/NA overides Rel-16 H/S, while Rel-16 NA should not be overridden. Proposal 6: For DCI format 2_5, Alt 1 should be supported. i.e., A single DCI format 2_5 can be received indicating availability for multiple RB sets. Proposal 7: The child node can dynamically report the recommended TCI(s)/SRI(s) to the parent node per multiplexing case. Proposal 8: MAC CE is used to report the recommended TCI(s)/SRI(s) to the parent node. Proposal 9: When child node reports recommended TCI(s) to parent node, the associated per beam interference is included in the beam reporting. - The associated interference includes MT-to-MT CLI in case of simultaneous to DU RX and MT TX at IAB node. - The associated interference includes self-interference from IAB DU to IAB MT in case of simultaneous to DU TX and MT RX. Proposal 10: Regarding applicability of the indicated restricted beams from parent node to child IAB-DU, the following should be supported. - The beam restriction is applied to the occasions of simultaneous DU TX and MT TX at child IAB node. - The beam restriction is not applied cell-specific signaling transmission, e.g., SSB and periodic CSI-RS. Proposal 11: Parent node indicates a set of restricted beams in the form of CRI of child DU. Proposal 12: Clarify the applicability of the beam restriction indications from different parent nodes in case of dual connectivity of the child IAB node. Proposal 13: New types of guard symbols should be defined if Case 6 timing mode is supported for the following switching cases - Case 1 DU TX to Case 6 MT TX. - Case 1 DU RX to Case 6 MT TX. - Case 6 MT TX to Case 1 DU TX. - Case 6 MT TX to Case 1 DU RX. Proposal 14: New types of guard symbols should be defined if Case 7 timing mode is supported - Case 1 MT RX to Case 7 DU RX. - Case 1 MT TX to Case 7 DU RX. - Case 7 DU RX to Case 1 MT RX. - Case 7 DU RX to Case 1 MT TX. Proposal 15: RAN1 further study how to derive the guard symbol for the switching between Case 6 and Case 7 timing mode. Proposal 16: For a given IAB node operating in Case 6 or Case 7 timing mode, RAN 1 further study the impact on guard symbol of the parent node for switching between Case 6 DU RX and MT TX/RX, and guard symbol of the child node for switching between Case 7 MT TX and DU RX/TX. Proposal 17: The indication of timing mode is associated with indication of multiplexing mode, i.e., Case 6 timing is associated to multiplexing case A and Case 7 timing is associated to multiplexing case B. Proposal 18: IAB-node is also indicated by parent node, when Case 7 timing is performed at the IAB-node, e.g., via indication of the associated multiplexing case. |
| Vivo R1-2108997 | Observation 1: When PRACH resources configured by both rach-ConfigCommonIAB and rach-ConfigCommon are available, Rel-16 child IAB-node will prioritize the PRACH resource configured by rach-ConfigCommonIAB for radio access purpose. Observation 2: Always using the PRACH resources configured by rach-ConfigCommonIAB once rach-ConfigCommonIAB is configured may incur large random access latency for an IAB-node. Proposal 1: Support a PRACH occasion selection mechanism, which gives the child IAB-node sufficient flexibility to use PRACH resources configured by both RACH-ConfigCommonIAB and rach-ConfigCommon. Proposal 2: Specify a mechanism for an IAB node to select between the PRACH resources configured by rach-ConfigCommon and that configured by rach-ConfigCommonIAB. |
| ZTE, Sanechips R1-2109261 | Observation 1: The benefit to specify the relationship between N and the RGB_MT is not clear, but arising a lot of following issues and normative works. Observation 2: In addition to a reference SCS, a reference carrier also needs to be defined for frequency domain H/S/NA configuration, e.g., at least the starting frequency location and bandwidth corresponding to the carrier including the RB sets need to be known first. Observation 3: Dynamic H/S/NA configuration selection between Rel-17 frequency domain H/S/NA configuration and Rel-16 time domain H/S/NA configuration may lead to inconsistent understanding of H/S/NA frequency on an IAB-node DU. Observation 4: Alt. 3 has less specification impact and is better compatible with Rel-16 availability indication of soft resources. Observation 5: It is sufficient to indicate the preferred beams for IAB node MT to its parent node. Observation 6: For a given time resource, actual multiplexing case adopted by an IAB-node can be determined according to timing mode indication, power control parameters provided by its parent node, beams used by IAB node DU and MT, resource type (i.e. DL, F, UL) of IAB node DU and MT etc. Proposal 1: N is a number of PRBs configured per DU cell by the CU regardless single or multiple BWPs are configured at the IAB MT. Proposal 2: For semi-static frequency domain H/S/NA configuration: • The reference SCS defined in Rel-17 is used to determine the frequency domain granularity, i.e., per RB set, of the frequency domain H/S/NA configuration. • The SCS defined in Rel-16 for the D/F/U configuration is reused to determine the time domain granularity, i.e., per D/U/F resource type within a slot, of the frequency domain H/S/NA configuration. Proposal 3: The carrier corresponding to the reference SCS in the carrier list of the IAB-node DU cell is used as the reference carrier to determine RB sets for frequency domain H/S/NA configuration. Proposal 4: A parent node should be made aware of the SCS-specific carriers and frequency domain H/S/NA configuration of its child node DU’s cell(s). Proposal 5: Rel-17 frequency domain H/S/NA configuration, if provided, always overrides Rel-16 time domain H/S/NA configuration. Proposal 6: Rel-16 DCI Format 2_5 is full reused to indicate soft resource availability for TDM and FDM slots (i.e. Alt. 3): • For TDM slot, availability indication is valid for all soft symbols in the slot • For FDM slot, availability indication is valid for all soft RB sets in the slot. Proposal 7: The IAB-node MT indicates recommended beams to its parent node by MAC CE signalling. Proposal 8: For the child node indication of recommended beams, IAB-MT DL beams are defined by TCI state ID, IAB-MT UL beams are defined by SRI. Proposal 9: Indication of IAB MT “not preferred” beams to its parent node is not supported. Proposal 10: Restricted beams of its child IAB-node DU indicated by parent node are defined by RS ID (e.g., SSB index). Proposal 11: For a given time resource, the applied timing mode can be explicitly indicated by the parent node, while the expected multiplexing case at the given time resource can be implicitly determined based on the timing mode indication. |
| ZTE, Sanechips R1-2109263 | Observation 1: If more than one SCS-specific carrier for an IAB-node DU cell is reported to CU, IAB-node DU should know which carrier (e.g. either which one of the SCS-specific carriers or the carrier defined/determined by CU) is used by CU to define the RB sets for H/S/NA resource configuration in frequency domain. Proposal 1: A reference carrier should be defined to determine the RB sets for frequency domain H/S/NA configuration, e.g., the carrier corresponding to the reference SCS can be used as the reference carrier. Proposal 2: Parent nodes should be made aware of the carrier frequency information of an IAB DU cell (i.e., its child node). • FFS: how to indicates of the carrier frequency information to the parent node |
| Samsung R1-2109510 | Proposal 1: FDM H/S/NA configuration includes time domain information. Proposal 2: Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource. Proposal 3: Guard band can be generated by each IAB node or parent IAB. Proposal 4: The following H/S/NA resource types are supported for FDM - H type for frequency resource: the IAB DU can assume it can use the frequency resource regardless of the MT’s configuration on the frequency resource - S type for frequency resource: the IAB DU can assume it can use the frequency resource without an impact on the MT’s configuration on the frequency resource - NA type for frequency resource: the IAB DU cannot assume it can use the frequency resource Proposal 5: For the beam restriction, beam information including TCI states ID, reference signal ID for CSI-RS or SSB is reported to parent IAB. Proposal 6: For the beam restriction, MAC-CE signaling from the parent DU includes which beam is restricted or not restricted in an order based on the reported TCI state ID. Proposal 7: For the beam recommendation, MAC-CE signaling from the child IAB is reported to parent IAB. Proposal 8: Both semi-static and dynamic adaptation for IAB node's multiplexing operations are supported. Proposal 9: Only one set of signaling for Desired/Provided Guard Symbols considering maximum values of all possible timing switching can be supported. |
| Intel Corporation R1-2109629 | Observation 1: For the switching of Case#1 Case#6 timing at an IAB-node, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU TX/MT TX is allowed in Case#6 timing: - IAB-DU TX in slot n to IAB-MT TX in slot (n+1) - IAB-MT TX in slot n to IAB-DU TX in slot (n+1) Observation 2: For the switching of Case#6 Case#1 timing at an IAB-node, new guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition. Observation 3: For the switching of Case#1 Case#7 timing, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU RX/MT RX is allowed in Case#7 timing: - IAB-DU RX in slot n to IAB-MT RX in slot (n+1) - IAB-MT RX in slot n to IAB-DU RX in slot (n+1) Observation 4: For the switching of Case#7 Case#1 timing at an IAB-node, guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition. Observation 5: For the switching of Case#6 Case#7 timing at an IAB-node, it is equivalent to the combination of Case#6 Case#1 switching and Case#1 Case#7 switching. New guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition. Observation 6: For the switching of Case#7 Case#6 timing at an IAB-node, it is equivalent to the combination of Case#7 Case#1 switching and Case#1 Case#6 switching. New guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition. Proposal 1: Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource. Proposal 2: For frequency-domain soft resource availability indication, support a single DCI format 2_5 to indicate availability for multiple RB sets which corresponding to the same time resources of the child IAB-DU cell. Proposal 3: Limit the maximum number of semi-static configured frequency-domain soft regions per resource type or per slot. Proposal 4: New guard symbols are needed for switching between Case#1/Case#6/Case#7 timing as in Table 1. Proposal 5: When simultaneous operation(s) are supported, the corresponding transition guard symbols defined in Rel-16 IAB are not needed as in Table 2. |
| NTT DOCOMO, INC. R1-2109697 | Proposal 1: For a given RB set at a symbol, if both Rel-17 frequency domain H/S/NA and Rel-16 time domain H/S/NA are provided, whether frequency domain or time domain H/S/NA is applied can be determined at least based on following rules. - Time domain H/S/NA is applied, and DU can Tx/Rx on all the RBs at the symbol, if the following condition is met. - If a given symbol is DU hard in time domain, and - MT does not Tx/Rx at the symbol, - Or the simultaneous MT/DU operation corresponding to the MT/DU DL/UL configuration at the symbol is not supported/indicated, e.g., the symbol is configured as MT DL and DU DL, but MT-Rx/DU-Tx is not supported. - Or if a given symbol is DU soft in time domain, and - MT does not Tx/Rx at the symbol, - Or Soft-IA is indicated for at least one RB set, and the simultaneous MT/DU operation corresponding to the MT/DU DL/UL configuration at the symbol is not supported/indicated, e.g., the symbol is configured as MT DL and DU DL, but MT-Rx/DU-Tx is not supported - Otherwise, frequency domain H/S/NA is applied, and DU can Tx/Rx on the RBs configured/indicated as hard/soft-IA at the symbol. Proposal 2: Support Alt.1 for soft resource availability in the frequency domain, i.e., - Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell. Proposal 3: AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set. Proposal 4: IAB-MT reports recommended beams per multiplexing case. Proposal 5: Legacy beam reporting framework can be reused for IAB-MT reporting of recommended beams. Proposal 6: A set of restricted DU beams are indicated per MT beam. Proposal 7: With parent node’s indication of restricted beam, whether a beam is used by DU is up to DU implementation. Proposal 8: Support simultaneous restricted DU beam indication across MT CCs. A list of MT CCs can be configured. If a set of restricted DU beams is indicated for one MT CC in the list, the indication applies to all the MT CCs in the list. Proposal 9: Parent node indicates whether Case#6 or Case#7 timing is performed via UL scheduling DCI. Proposal 10 : If case#6 timing mode is required for simultaneous MT-Tx/DU-Tx, simultaneous MT-Tx/DU-Tx can be performed only if case#6 UL Tx timing is indicated by parent node; otherwise, simultaneous Tx cannot be performed. Proposal 11: On a DU flexible symbol, parent node can configure/indicate IAB node MT Tx/Rx, and IAB node DU will Tx/Rx on the symbol based on MT transmission/reception direction and its multiplexing capability. Proposal 12: The indication of guard symbols can be extended as Table II to indicate guard symbols for transition between MT Tx with case#6/7 timing and DU Tx/Rx. Proposal 13: Whether different value range of the number of guard symbols is needed for the new timing modes needs to be studied. Proposal 14: Guard band can be handled by IAB node implementation. |
| Huawei, HiSilicon R1-2109755 | Proposal 1: Number of guard symbols for MT and DU flexible symbols = min(Number of guard symbols for MT and DU downlink switching, Number of guard symbols for MT and DU uplink switching) Proposal 2: Introduce negative value of Ng to support efficient resource utilization. |
| ETRI R1-2109816 | Observation 1: No need to support an explicit parent acknowledgement for multiplexing capability adaptation. Proposal 1: Introduce L2 (MAC CE) based report/indication on conditions/parameters to facilitate adaptation between multiplexing operation modes, where the report/indication includes the following contents, at least: - Supported multiplexing mode (FDM and/or SDM). - Required timing mode. - Required DL/UL power control support. - Information on the “non-preferred” beam. - Required number of guard symbols for switching of multiplexing mode. - Required guard band for FDM Proposal 2: Introduce higher-layer parameter(s) to configure the following information: - Periodicity or timing of report/indication on conditions/parameters to facilitate adaptation between multiplexing operation modes. - Recommendations on the conditions/parameters from parent node perspectives. Proposal 3: Support Alt. 1 (a single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell) for soft resource availability in the frequency domain. - FFS, whether to additionally support Alt. 2 or Alt. 3 on top of Alt. 1. - Clarify whether the terminology “all the soft resources” in Alt. 3 includes “all the soft resources configured by Rel-16 and Rel-17 signalings at a given time” or not. Proposal 4: Extend the mapping between values of resourceAvailability elements and types of soft symbols/resources for the time/frequency resources configured with both Rel-17 frequency domain H/S/NA configuration and the Rel-16 time domain H/S/NA configuration Proposal 5: Consider specification supports on handling of cell-specific or semi-static signals/channels when the signals/channels are associated with the non-preferred DU beams. Proposal 6: Consider specification supports to report non-preferred MT beams for simultaneous operations. Proposal 7: Preferred(recommended)/non-preferred(restricted) beams can be reported/indicated by one or more among the following options: - Option 1: TCI state ID - Option 2: Spatial relation information ID - Option 3: RS ID (including CSI-RS, SRS, SSB, etc.) Proposal 8: Support X>1, where X is the maximum numbers of the preferred(recommended)/non-preferred(restricted) beams for simultaneous operations. - Consider X=4, at least. Proposal 9: Support explicit indication/association of the simultaneous operation mode for the preferred(recommended)/non-preferred(restricted) beam report and/or indication. Proposal 10: RAN1 to strive concurrent support of “Rel-16 simultaneous TCI/spatial relation update” and “Rel-17 simultaneous DU/MT operations”. |
| CEWiT, IIT-M, IIT-B and Saankhya Labs R1-2109839 | Proposal 1: Mechanism to inform multiplexing capability of parent nodes to IAB node is supported Proposal 2: Signalling of multiplexing capability and supported modes from IAB node to CU and parent-DU is supported Proposal 3: Following alternatives are proposed for configuring active mode of operation at an IAB node - Alt1: Donor configuring the active mode of operation of IAB node - Alt2: Parent node configuring the active mode of operation of IAB node Proposal 4: Dynamic switching between multiplexing modes of IAB node is supported under the control of parent node Proposal 5: Mechanism to fall back to TDM mode from simultaneous mode at IAB node is supported Observation 1: Frequent switching between multiplexing modes leads to signaling storm Proposal 6: Define minimum duration for active mode of operation Observation 2: For an IAB node - The TA associated with UL-Tx at IAB-MT and guard required for switching to/from UL-Tx changes based on active mode of operation of IAB node and its parent node - The timing of UL-Rx at IAB-DU and guard required for switching to/from UL-Rx changes based on active mode of operation of IAB node and its child node Proposal 7: Parent node update TA and guard symbols required for switching to/from UL-Tx before mode switching at IAB node or its parent node Proposal 8: IAB node inserts additional guard symbols for switching to/from UL-Rx at IAB-DU before mode switching at IAB node or its child node Proposal 9: An IAB node apply either the time domain H/S/NA configuration or frequency domain H/S/NA configuration for a given resource based on the active mode of operation Proposal 10: The switching between time domain H/S/NA configuration and frequency domain H/S/NA configuration occur based on the granularity of active mode of operation Proposal 11: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell Proposal 12: In case of IAB-MT, the resource configuration given by tdd-UL-DL-ConfigurationDedicated-IAB-MT override the configuration of cell specific signal/channel |
| AT&T R1-2109920 | Proposal 1: To support dynamic adaptation of multiplexing capability for DU flexible resources configured as soft, a DCI Format 2_5 indication from the parent node can be used to jointly indicate a desired transmission direction (D or U) at the child along with an indication of whether the given soft time or frequency resource is indicated as available. Proposal 2: The Rel-17 frequency-domain H/S/NA configuration is only provided for RB sets in a given slot which have a different H/S/NA resource type from the corresponding Rel-16 H/S/NA configuration. Proposal 3: MAC-CE signaling from the parent node to the child node can indicate a set of restricted IAB-specific SSB beams at the IAB-DU of the child node. Proposal 4: The child node can dynamically report to the parent node both IAB-MT DL and IAB-MT UL beams based on SSB and CSI-RS using the existing beam-management framework (e.g. on the basis of different TCI states configured at the IAB-MT). Proposal 5: An IAB-node is explicitly indicated when Case 6 and Case 7 timing is performed at the IAB-node via MAC-CE signaling based on whether desired/provided guard symbols for Case 6 and/or Case 7 timing are provided. |
| Lenovo, Motorola Mobility R1-2109936 | Observation 1: Separate application of time- and frequency-domain H/S/NA may result in resources wasted when FDM cannot be realized at a moment. Observation 2: Separate application of time- and frequency-domain H/S/NA is a special case of joint application. Observation 3: The best-effort approach is insufficient for non-TDM operation in the presence of varying beam, power, interference, and timing alignment constraints. Observation 4: When non-TDM multiplexing between upstream and downstream links are intended, sending availability indication for IAB-DU resources solely based on IAB node’s non-TDM capability may result in shortage of resources for IAB-MT due to beamforming, power, interference, or timing alignment constraints at a moment. Proposal 1: Support joint application of time- and frequency-domain H/S/NA configurations. Proposal 2: Specify rules for determining H/S/NA resource types for time-frequency resources based on separate configurations in time and frequency domains. Proposal 3: Support conditional availability indication as a balance between existing best-effort non-TDM and TDM-only availability indication. Proposal 4: Extend DCI 2-5 for frequency-domain availability indication by allowing a unit of N PRBs in addition to the unit of 1 slot. Alt. 2 is preferable for the sake of flexibility. Proposal 5: No need to introduce scaling of N based on system BW or IAB-MT BWP. Proposal 6: Support per-panel granularity by allowing dynamic indication of restricting/using beams and their associated antenna panels for specific time-frequency resources. Proposal 7: Support early beam/panel indications for the upstream link in addition to downstream beam indications. Proposal 8: Support applicability of beam/panel indications to specific time-frequency resources. No need to specify indication of simultaneous operation mode in the standard. Proposal 9: Support signaling IAB node capabilities for timing alignment and multi-panel and baseband characteristics. Proposal 10: Support dynamic signalling for communicating operation constraints such as beam, power, interference, and timing alignment constraints along with information of time, frequency, and spatial resources. Proposal 11: Support indication of recommended beams to the parent node as part of dynamic signaling of “conditions/parameter” agreed in RAN#105-e. Proposal 12: Support MAC CE signaling for activation/deactivation of Case 6 timing. Proposal 13: Support MAC CE signaling for activation/deactivation of Case 7 timing. The activation MAC CE message can carry an additional TA value as offset to the legacy TA value. Proposal 14: Support RRC configuration for Case-6 and Case-7 timing alignment in addition to dynamic signaling. Proposal 15: Signalling for timing alignment indication is linked to multiplexing capability of the IAB node as well as whether the IAB node requires enhanced timing alignment. Proposal 16: Support a capability signalling, e.g., number of IFFT/FFT windows, to indicate whether the IAB node requires timing alignment between IAB-MT and IAB-DU operations. If negative, the IAB node can transmit unaligned OFDM symbols (Case A) and receive/process unaligned OFDM symbols (Case B), which simplifies signalling and configuration significantly. |
| Apple Inc. R1- 2110051 | Proposal 2: For simultaneous operation, either parent IAB-DU or IAB-DU, for example depending on data priority order of the link, can initiate and control the flow, and indicate the desired parameters to the other link. Proposal 3: For the case of parent IAB-DU indicating IAB node about the receiving beams/panels for Lp,DL and LA,UL / Lc,UL links, the indication can be dynamic and/or semi-static: - Dynamic e.g. through TCI index in DCI for scheduling PDSCH Lp,DL - Semi-static, e.g. through MAC-CE/RRC to IAB-MT or through F1 interface to IAB-DU Proposal 4: For the case of IAB indicating desired reception beams/panels on Lp,DL and LA,UL / Lc,UL links, the indication to parent IAB-DU can be dynamic and/or semi-static: - Dynamic e.g. through UCI transmission from IAB-MT to parent DU - Semi-static, e.g. through MAC-CE from IAB-MT or through F1 interface from IAB-DU Proposal 5: To achieve a simultaneous reception within an IAB node, i.e. DU-Rx & MT-Rx, a two-step indication is deployed - Step1: Parent IAB-DU indicates initial beam/panel, K0, SLIV, etc, and a grace time for IAB node in which IAB-MT can indicate its desired parameters like beam/panel, K0, SLIV, etc - Step2: IAB indicates its desired parameters (beam/panel, K0, SLIV, Tx power, etc), for example, on PUCCH indicated by DL DCI from parent IAB-DU Proposal 6: To utilize SDM resource multiplexing, support PDSCH allocation for IAB-MT with crossing the slot border. |
| LG Electronics R1-2110101 | - Simultaneous operation schemes between MT and DU Proposal 1: Discuss whether the minimum resource size N is configured independently for DL and UL. Proposal 2: The composition of the minimum resource for configuring the frequency domain granularity is based on the location of the CRB #0 of the corresponding MT. Proposal 3: A RB set for frequency domain H/S/NA configuration is composed by one or multiple consecutive RB groups where a RB group is composed by N PRBs. Proposal 4: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set at a symbol, an IAB node only applies one of them based on the applied multiplexing operation mode. Proposal 5: A single DCI format 2_5 can be received indicating availability for multiple RB sets which corresponds to the same time resources of the child IAB-DU cell. Proposal 6: Discuss whether the soft resource availability for time domain and frequency domain are indicated using different fields or the same field. Proposal 7: In case the frequency-domain H/S/NA is configured, this information can be used for deciding SDM operation of the IAB node. In addition, this information can be used for determining whether beam restriction for SDM operation is applied or not. Proposal 8: For IAB-nodes that require timing alignment for simultaneous operation, - If MT is configured Case#6 for MT UL Tx, MT and DU can perform simultaneous Tx/Tx when the MT has UL Tx direction and the DU has DL Tx direction within the time resource applying Case #6 timing. - If Case#7 is allowed for DU UL Rx, MT and DU can perform simultaneous Rx/Rx when the MT has DL Rx direction and the DU has UL Rx direction within the time resource applying Case #7 timing. - When the MT and DU of the IAB-node have a Tx/Rx resource direction combination that is not capable of simultaneous operation within the time resource applying Case #6 or Case #7 timing, the MT and DU can be operated in TDM-manner. Proposal 9: Discuss whether/how to handle overlapping of time resource for Case #6 timing and Case #7 timing. Proposal 10: For the case of an IAB-node that does not require any timing alignment for simultaneous operation, the IAB-node can perform simultaneously without restriction on time resource or within the explicitly allowed time resource if the combination of Tx/Rx direction between MT and DU is capable of. Proposal 11: It is supported to report the applicability of the given multiplexing operation mode to the parent node. Proposal 12: Guard band for FDM between MT and DU is determined by DU and allocated in DU frequency resource. Proposal 13: Support separation of CDM group between DU and MT for SDM operation of IAB-node. Proposal 14: The IAB-node and the parent node determine the number of guard symbols implicitly according to the timing case applied by the IAB-node in a specific time resource. Proposal 15: To indicate the number of guard symbols required for Case #6 and Case #7 timing, the switching scenarios only when the number of guard symbols is different compared to Case #1 timing case are considered. |
| Qualcomm Incorporated R1-2110206 | Proposal 2.1: Support Rel-17 frequency-domain H/S/NA configuration to provide time-varying frequency-domain H/S/NA pattern over a number of slots. Proposal 2.2: Support Rel-17 frequency-domain H/S/NA configuration by extending the gNB-DU cell resource configuration IE used by the Rel-16 time-domain configuration to the granularity of an RB set, where selected slots and RB sets can be optionally specified. Proposal 2.3: For the size of an RB set, support additional candidate values of {96,128,160} to the agreed list of {2,4,8,16,32,64} so that it can be sufficient to configure 2 or 3 RB sets for a max carrier bandwidth. Observation 2.1 For supporting soft resource availability in frequency-domain, - Alt1 is more flexible than Alt3 in that it can dynamically indicate availability of all or a subset of RB sets to the child node based on applicability of simultaneous operation mode and FDM requirement. - Alt1 has less over-the-air signaling overhead than Alt2. Proposal 2.4: Support Alt1 with a single DCI format 2_5 to indicate availability for multiple RB sets corresponding to the same time resource of the child IAB-DU cell. Proposal 2.5: Update resourceAvailability mapping table defined in TS38.213 so that the indication of availability can be applied over soft resources in frequency-domain for DL or UL or Flexible symbols. - E.g. replacing “soft DL/UL/Flexible symbols” with “soft resources in DL/UL/Flexible symbols” Observation 3.1: For a supported simultaneous operation mode at an IAB-node, the assistant information on whether FDM is required or not can be beneficial - for the donor-DU to determine more efficient H/S/NA resource pattern, - for the parent node to make more optimized decision on coordinating soft resources with its child node. Proposal 3.1: Support indication of whether FDM is required or not for a simultaneous operation mode to donor-CU and/or parent node. Proposal 3.2: Support dynamic indication by the IAB-node to its parent-node to indicate applicability of simultaneous operation modes. - FFS: indicated in the same signaling for required conditions. - FFS: support of dynamic overriding Rel-16 time-domain configuration by Rel-17 frequency-domain configuration based on indication of applicability of simultaneous operation modes. Observation 3.2: Rel-17 supports quite a few new types of switching across combinations of MT/DU timing cases, and MT/DU multiplexing modes. Each switch type may require a different number of guard symbols. However, supporting an indication of desired/provided guard symbols for all switch types will lead to a large signaling overhead. Observation 3.3: To support extension of Desired/Provided Guard Symbols to a new UL RX timing at a DU (based on Case 7) will further require providing the parent-node if/when the IAB-DU adopts Case 7 UL RX timing. This is not justified, due to its large signaling overhead, and questionable benefit. Proposal 3.3: MAC-CE signaling of Desired/Provided Guard Symbols is extended to support the following switch types: - MT TX based on Case 7 UL timing to/from DU RX, and MT TX based on Case 7 UL timing to/from DU TX - MT TX based on Case 6 UL timing to/from DU RX, and MT TX based on Case 6 UL timing to/from DU TX Proposal 3.4: An IAB-DU decides about its UL RX timing reference (e.g., whether to adopt Case 1, or Case 7) without any indication from its parent-node. Observation 3.4: An IAB-MT’s Case 6 UL TX timing is to facilitate enhanced duplexing at the IAB-node, while IAB-MT’s Case 7 UL TX timing is to facilitate enhanced duplexing at the parent-node. An IAB-node cannot map Case 7 UL TX timing (of IAB-MT) to any subset of its time resources, without explicit indication from the parent-node. It is desired to have a unified design for indication of different UL TX timing cases. Proposal 3.5: An IAB-MT is provided with Timing Case Indication that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot). Timing Case Indication is provided via MAC-CE. Proposal 3.6: Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG). Observation 3.5: An IAB-MT’s need for Case 6 UL TX timing may depend on: - MT CC, and/or (MT CC, DU cell) pair - IAB-node’s multiplexing mode: (MT TX, DU TX) and/or (MT TX, DU RX) - MT’s TX beam - Whether MT’s TX is FDMed with DU’s communications or overlaps in the frequency domain. Proposal 3.7: IAB-MT indicates, to its parent-node, whether it desires Case 6 UL TX timing, for a given MT CC, to facilitate simultaneous IAB-MT’s TX and IAB-DU’s TX and/or RX. - FFS: whether this indication can further be beam specific. - FFS: whether this indication can further depend on whether IAB-MT’s TX is FDMed with IAB-DU’s communications or not. Observation 4.1: Indication of “not-preferred” IAB-MT’s beam(s) (in addition or instead of recommended beam(s)) may provide more efficient and flexible assistance information. It can also effectively be used to indicate required fallback to TDMing, when an enhanced multiplexing mode of operation is not supported. Proposal 4.1: Further support indication of not-preferred IAB-MT’s beam(s) (for both TX and RX) to the parent-node. Observation 4.2: The parent-node should further be provided with information about the resources for which indicated recommended/not-preferred beam(s) are applicable. Proposal 4.2: The recommended and/or not-preferred IAB-MT’s beam(s), indicated by the IAB-MT to its parent-node, can be provided for: - A specific multiplexing mode – i.e., (MT RX, DU RX), (MT TX, DU TX), (MT RX, DU TX), (MT TX, DU RX) - A given (MT CC, DU cell) pair - The cases where concurrent MT’s and DU’s communications are FDMed or overlap in the frequency domain. - The indication is provided via MAC-CE. Proposal 4.3: The recommended and/or not-preferred IAB-MT’s beam(s) are indicated based on IAB-MT’s TCI state IDs (for DL RX beam(s)), and SRI IDs (for UL TX beam(s)). Proposal 4.4: The indication of the restricted IAB-DU’s beam(s), provided by the parent-node to the IAB-MT, is only applicable to the IAB-DU’s DL TX beams. Observation 4.1: The parent-node may know about the IAB-DU’s STC and CSI-RS configurations and use them a reference to indicate IAB-DU’s beam(s). Different IAB-DU’s STCs may be associated with different beamforming configurations. Proposal 4.5: The indication of the restricted IAB-DU’s beam(s), provided by the parent-node to the IAB-MT, is via referring to any of the following - IAB-DU’s CSI-RS resource indices - IAB-DU’s SSB and STC indices Proposal 4.6: The restricted IAB-DU’s beam(s), indicated by the parent-node to the IAB-MT, can be provided for: - A specific multiplexing mode – i.e., (MT TX, DU TX), (MT RX, DU TX) - A given MT’s TX/RX beam - A given (MT CC, DU cell) pair - The cases where concurrent MT’s and DU’s communications are FDMed or overlap in the frequency domain. Proposal 4.7: The indication of the restricted IAB-DU’s beam(s), provided by the parent-node, is considered as a recommendation, without any mandated behavior at the IAB-DU, at least for the IAB-DU’s HARD resources. - FFS: expected behaviour of the IAB-DU regarding the indication of restricted beams within its SOFT resources. |
| Ericsson R1-2110331 | Proposal 1 The frequency domain granularity, i.e., the N RBs in an RB set is related to MT’s BWP and number of PRBs in an RBG, NRB(RBG), as given in Table 1. Proposal 2 In case that an IAB-MT cell is configured with multiple BWPs, support per-BWP frequency domain H/S/NA configuration, including a specific N value. Proposal 3 In case that an IAB-DU cell is overlapping with multiple IAB-MT cells, the IAB-DU cell is configured with one N value which is derived based on the largest RBG size of all IAB-MT cells and is common to all the MT-BWPs. Proposal 4 If a resource is configured as Hard, or explicitly indicated as Soft-IA the IAB-DU can transmit, receive, or either transmit or receive according to its configuration only if it does not impact the IAB-MT’s actual ability to operate in any other resource, including the same RB-Set and other RB-Sets. Proposal 5 If a resource is configured as Soft, the IAB-DU can transmit, receive, or either transmit or receive only if it does not impact the IAB-MT’s actual ability to operate in any resource, including the same RB-Set and other RB-Sets. This applies to implicitly Indicated Available IAB-DU Soft resource. Proposal 6 For one IAB-DU cell, support operation in either TDM or FDM modes on a per-slot basis. Proposal 7 Support indicating both time-domain and frequency-domain H/S/NA in the same gNB-DU Cell Resource Configuration (9.3.1.107 in TS 38.473 [8]), where the frequency-domain H/S/NA is provided in terms of RB sets, or groups of RB sets. Proposal 8 To reduce signaling overhead, the IAB-DU may be provided with frequency domain H/S/NA configurations only to the RB sets whose configuration is different from the time domain H/S/NA. Proposal 9 For explicit availability indication of frequency-domain Soft resources over time, support to re-use the structure of the Rel-16 availabilityCombinations with enhancements. Proposal 10 For explicit availability indication of frequency-domain Soft resources, each availabilityCombination of availabilityCombinations can indicate availability for single or multiple RB sets. Proposal 11 Support multiple frequency domain DCI format 2_5 associated to the same time slot. Proposal 12 Rel-17 enhancement on DCI format 2_5 should at least consider extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources. FFS: Further enhancement to further increase the number of IAB-DU cells that can be indicated AI. Proposal 13 One resourceAvailability element of the availabilityCombinations table is reserved to signal to the IAB-DU that “No new Availability Indication is provided” in DCI format 2_5 for slots which receive multiple Availability Indicators. Proposal 14 Explicit availability indication overrides SDM beam restrictions in Soft resources. Proposal 15 RS ID (e.g., SSB index) is used by a parent node to indicate beams of an IAB-DU in the direction of which simultaneous operation is restricted. Proposal 16 RS ID is used by a child node to indicate a set of preferred beams to the parent node. Proposal 17 Interference measurement report, even if specified, is not included in the beam report. Proposal 18 Interference measurement discussions are limited to Agenda Item 8.10.2. Proposal 19 The IAB-node may report to the parent node a set of multiplexing conditions on the required timing mode, power control, beam indication.  For simultaneous MT-TX/DU-TX, the multiplexing conditions include whether the IAB-node requires: o Case-6 timing o Parent node indication of restricted IAB-DU beams o IAB-node indication to its parent node on its MT’s UL TX power control  For simultaneous MT RX/DU RX, the multiplexing conditions include whether the IAB-node: o Supports Case-7 timing o Requires IAB-node indication of recommended IAB-MT beams o Requires IAB-node indication to support parent node DL power control Proposal 20 Whether or not to switch to FDM/SDM should be based on an ACK/NACK response from the parent IAB-node. Proposal 21 Fallback to TDM does not need an ACK response from the parent IAB-node. Proposal 22 Desired/Provided Guard Symbols are signaled in multiple groups that covers all relevant switching combinations among Case #1, Case #6 and Case #7 timing alignments. |

ISSUE 2.1: FREQUENCY DOMAIN MULTIPLEXING

Proposal 2.1.1a: Rel-17 frequency domain H/S/NA configuration includes time domain information with the same granularity as the Rel-16 H/S/NA configuration.

Discussion: Views on proposal 2.1.1a?

| Company | Comments |
| --- | --- |

Proposal 2.1.2a: Different frequency domain H/S/NA resource types can be indicated individually for each RB set over a subset of slots where the frequency domain H/S/NA configuration is different from the time domain H/S/NA configuration.

- Additional signaling details (e.g. bitmap, slot pattern, etc.) can be left up to RAN3

Discussion: Views on proposal 2.1.2a?

| Company | Comments |
| --- | --- |

Proposal 2.1.3a: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, an IAB node applies the frequency domain H/S/NA only if simultaneous operation is currently applied for the indicated resources.

Discussion: Views on proposal 2.1.3a?

| Company | Comments |
| --- | --- |

Proposal 2.1.4a: Support additional candidate values of {96,128,160} for the size of an RB set, N.

Discussion: Views on proposal 2.1.4a?

| Company | Comments |
| --- | --- |

Proposal 2.1.5a: In case that an IAB-MT cell is configured with multiple BWPs, one of the following alternatives is supported:

- Per-BWP frequency domain H/S/NA configuration (including the value of N) is provided.

- A single value of N is common across all MT BWPs overlapping the IAB-DU cell

Discussion: Views on proposal 2.1.5a?

| Company | Comments |
| --- | --- |

Proposal 2.1.6a: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- FFS: Extension of AvailabiltyCombination to include multiple resourceAvailabilty per RB set

- FFS: Update resourceAvailability mapping table defined in TS38.213 so that the indication of availability can be applied over soft resources in frequency-domain for DL or UL or Flexible symbols.

- FFS: Extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources.

Discussion: Views on proposal 2.1.6a?

| Company | Comments |
| --- | --- |

ISSUE 2.2: SPATIAL DOMAIN MULTIPLEXING

Proposal 2.2.1a: RS ID is used for a parent node to indicate beams of an IAB-DU in the direction of which simultaneous operation is restricted

- At least SSB ID is supported

- FFS: Informing the parent node of RS configuration for CSI-RS or SRS at the child IAB-DU

Discussion: Views on proposal 2.2.1a?

| Company | Comments |
| --- | --- |

Proposal 2.2.2a: The restricted beam indication from the parent node to the child IAB-DU may be provided per:

- Multiplexing mode

- MT TX/RX beam

- {MT CC, DU cell} pair

Discussion: Views on proposal 2.2.2a?

| Company | Comments |
| --- | --- |

Proposal 2.2.3a: The recommended beam indication from the child IAB-MT to the parent node are provided via MAC-CE using the IAB-MT’s TCI state IDs (for DL RX beam(s)), and SRI IDs (for UL TX beam(s)).

Discussion: Views on proposal 2.2.3a?

| Company | Comments |
| --- | --- |

ISSUE 2.3: MULTIPLEXING OPERATION ADAPTATION

Proposal 2.3.1a: In Rel-17, the semi-static indication of the multiplexing capability of an IAB node for the case of no-TDM between IAB MT and IAB DU is additionally provided with respect to each transmission-direction combination (per MT CC/DU cell pair):

- Support for FDM (FFS: desired guard band)

- Support for SDM (i.e. parent beam restrictions and/or child beam recommendations)

- Supported timing modes

- Support for DL Tx power adjustment

Discussion: Views on proposal 2.3.1a?

| Company | Comments |
| --- | --- |

Proposal 2.3.2a: MAC-CE signaling is used for the request/report of the following conditions/parameters to facilitate dynamic adaptation between multiplexing operation modes:

From the child to the parent:

- Desired DL TX power adjustment

- Desired IAB-MT PSD range for UL TX power control

From the parent to the child:

- Applied DL TX power adjustment

- Slots where Case 6 timing is applied at the child

- Slots where Case 7 timing is applied at the parent

FFS: Mapping of conditions/parameters to a set of time slots and MT Tx/Rx beams

FFS: Indication of additional dynamic “support” of a feature (e.g. timing mode, power control, beam restrictions/recommendations for simultaneous operation cases) on top of higher-layer signaling

Discussion: Views on proposal 2.3.2a?

| Company | Comments |
| --- | --- |

Proposal 2.3.3a: An IAB node cannot operate under a given non-TDM multiplexing mode until all conditions and parameters required are explicitly acknowledged by the parent node.

Discussion: Views on proposal 2.3.3a?

| Company | Comments |
| --- | --- |

ISSUE 2.4: GUARD SYMBOL INDICATION ENHANCEMENTS

Proposal 2.4.1a: The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between Case #1, Case #6, and/or Case #7 timing (e.g. only indicated when the number of guard symbols for switching is different compared to the individual timing cases themselves).

Discussion: Views on proposal 2.4.1a?

| Company | Comments |
| --- | --- |

# Resource allocation for dual-connectivity scenarios (i.e. IAB-MT with concurrent BH links with two parent nodes)

From the eIAB WID:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx)

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

Summary of input contributions:

| Huawei, HiSilicon R1-2108765 | Proposal 11: To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 12: To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 13: To support the per-backhaul-link resource configuration, the donor CU can provide parent node-specific resource configurations to an IAB node and its parent nodes. |
| --- | --- |
| Nokia, Nokia Shanghai Bell R1-2108826 | Observation 3.1: RAN2 will need to determine the method of coordinating to avoid DCI format 2_0 conflicts between parent DUs with a common donor CU and common child IAB MT. Observation 3.2: RAN2 will need to determine the method signaling resource configurations between parent DUs of a DC IAB node for both inter- and intra-donor scenarios. Observation 3.3: Selecting Alt. 1 for either scenario for resolving DCI format 2_0 conflicts between parents of an inter-donor DC IAB should be expected to impact RAN2 and RAN3. Proposal 3.1: Select Alt.6 for inter-donor DC scenarios where a slot format conflict occurs, and one of the conflicted resources is semi-static flexible. Proposal 3.2: Select Alt.5 for inter-donor DC scenarios where a slot format conflict occurs, and both conflicted resources are semi-static flexible. Proposal 3.4: Support per-backhaul link per-DU resource configuration for DC IAB nodes. |
| vivo R1-2108995 | Observation 1: Current NR-DC requires two legs to use different carrier, which means NR-DC is not available when single carrier is deployed according the current spec, while intra-frequency DC can remove this barrier. Proposal 19: RAN1 supports both intra-carrier DC and inter-carrier DC. Proposal 20: To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 21: To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: - Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 22: RAN1 specifies IAB DU behavior when IAB DU receives conflicted indications for Rel-17 features from two parent nodes, e.g., restricted beam indication, multiplexing case indication, etc. Proposal 23: Support child node to report soft resource availability from one parent node to the other parent node in DC scenario. |
| ZTE, Sanechips | Proposal 12: To handle the potential indication conflict of symbols configured as semi-static flexible by one or both parent nodes in inter-donor DC scenarios： • The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 13: When IAB MT detects more than one DCI format 2_5, the IAB-MT expects that each of the more than one DCI formats 2_5 from the same CG indicates a same value for the availability combination of the soft resources in the slot. Proposal 14: For explicitly availability indication case, a per-cell IAB-DU soft resource is considered as available with respect to a IAB MT serving cell only if the IAB MT is explicitly indicated as IA by DCI 2_5 from the CG that the serving cell belongs to. |
| Samsung | Proposal 10: If a conflict occurs between different parents, the IAB MT is expected to perform as scheduled by MCG. Proposal 11: If a collect occurs between different parents and there exist the essential channel and signal by SCG only, the IAB MT can prioritize scheduling by SCG. |
| Intel | Proposal 6: For the semi-static DU resource configurations, additionally support per-backhaul link (e.g. per child IAB-MT link) configuration. Proposal 7: In inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers: - Support Alt. 6 (The IAB-MT is expected to operate according to the non-flexible configuration) to handle potential indication conflict of symbols configured as semi-static flexible by one parent node. - Support Alt. 5 (If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG) to handle potential indication conflict of symbols configured as semi-static flexible by both parents. |
| NTT DOCOMO | Proposal 15: To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios, support Alt.1, i.e., - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI 2_0 and dynamic scheduling grants from different parents. Proposal 16: To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios, support Alt.1, i.e., - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI 2_0 and dynamic scheduling grants from different parents. |
| ETRI | Proposal 11: Clarify that there is no impact on IAB-MT behavior due to conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT, when the IAB node supports SDM between DU and MT resources at least. Proposal 12: Support Alt. 1 for potential indication conflict handling. - Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. |
| CEWiT, IIT-M, IIT-B and Saankhya Labs R1-2109839 | Proposal 13: In case symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios, the IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents Proposal 14: In case symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios and if a conflict occurs, the IAB MT is expected to perform as scheduled by MCG |
| AT&T R1-2109920 | Proposal 6: Per-backhaul link NA resource configurations which can override the per-IAB-DU configuration are supported in Rel-17. Coordination signaling to exchange the per-link resource configurations between up to two parent IAB-nodes/donors should be additionally supported in Rel-17. |
| Lenovo, Motorola Mobility R1-2109936 | Proposal 17: Support signaling from an IAB node in the DC mode to its parent node for informing the parent node of the status of availability of soft resources. Further discuss specification of mechanisms to handle availability indication collisions by two parent nodes in the DC mode. Proposal 18: To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation. Proposal 19: To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios, support Alt. 1. The matter can be left to implementation. Proposal 20: Do not support per-backhaul-link resource configuration in Rel-17. Proposal 21: RAN1 to discuss resource conflicts in the DC mode due to conflicting Case-7 timing. |
| Apple | Proposal 1: For the case of inter-donor DC if IAB-MT incapable of simultaneous transmission and reception is indicated by conflicting allocations from multi-parents, IAB-MT is expected to operate as indicated by MCG. |
| LG Electronics R1-2110101 | - Dual-connectivity to support multiple parent DUs Proposal 16: In case of conflict of symbols configured as semi-static flexible by one parent node, but not by the other in inter-donor DC scenarios and if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. Proposal 17: In case of conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios and if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the IAB MT is expected to perform as scheduled by MCG. Proposal 18: Discuss how to handle the case where the MT cannot perform simultaneous Tx/Tx or simultaneous Rx/Rx for two serving cells due to timing misalignment in CA/DC environment. |
| Qualcomm Incorporated R1-2110206 | Proposal 5.1: Support Alt5/6 for TDD confliction resolution, which may have slightly advantage and flexibility over Alt1. Observation 5.1 Per-BH link resource config allows an IAB-node to override its Rel-16 DU-cell config for a specified child BH link without impacting to other child nodes, and it is more flexible than Rel-16 approach using per DU-cell config together with knowledge of child’s config. Proposal 5.2 Support per-BH-link resource configuration. - Defer the discussion on whether the configuration shall be provided to the dual-connected child node after agreement of TDD prioritization rules. |
| Ericsson R1-2110331 | Proposal 23 To handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB-MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the dual-connected IAB-MT is expected to operate according to the non-flexible configuration. Proposal 24 To handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB-MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers, the dual-connected IAB-MT is expected to perform as scheduled by MCG, if a conflict occurs. Proposal 25 In DC scenarios, support per-link-NA configuration in Rel-17 IAB. |

ISSUE 3.1: FLEXIBLE SYMBOL CONFLICTS

Proposal 3.1.1a:

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 6. The IAB-MT is expected to operate according to the non-flexible configuration.

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Discussion: Views on proposal 3.1.1a?

| Company | Comments |
| --- | --- |

ISSUE 3.2: MULTI-PARENT RESOURCE COORDINATION

Proposal 3.2.1a: In DC scenarios, support per-child MT link-NA resource configuration.

Discussion: Views on proposal 3.2.1a?

| Company | Comments |
| --- | --- |

# Summary

Proposed Agreement: Rel-17 frequency domain H/S/NA configuration includes time domain information with the same granularity as the Rel-16 H/S/NA configuration.

Proposal Agreement: Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 6. The IAB-MT is expected to operate according to the non-flexible configuration.

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Proposed Agreement: In DC scenarios, support per-child MT link-NA resource configuration.