3GPP TSG-RAN WG1 Meeting #106bis-e 	R1-21xxxxx

# e-Meeting, October 11th – 19th, 2021

Title:	[Draft] Reply LS on the physical layer aspects of small data transmission

Response to:

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	ZTE [RAN1]

To:	RAN2

Contact Person:

Name:	Ziyang Li

E-mail Address:	li.ziyang1@zte.com.cn

Attachments:	None

1. Overall Description:

RAN1 has further discussed the remaining physical layer issues for CG-SDT and RA-SDT, and the following agreements have been achieved.

Mapping ratio and association period - Mapping ratio of SSB to CG PUSCH is configured per CG configuration. - FFS whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations. - For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16} - FFS {1/8,1/4,1/2} Multi-layer PUSCH transmission - Multi-layer PUSCH transmission is not supported for CG-SDT. Repetitions - PUSCH repetition is not supported for CG-SDT Validation of PUSCH occasion - No need to define UL/DL pattern type of validation rule specific for paired spectrum at least for non-RedCap UEs. - FFS the case for RedCap UEs - A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion - FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion Default SSB set - When SSB set indication is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1. Search space and CORESET - RAN1 confirms the working assumption in RAN2 that UE-specific search space is configured for UEs performing CG-SDT. This does not exclude the configuration of CSS for UEs performing CG-SDT. - CORESET for UE performing RA-SDT should be a common CORESET. L1 feedback - RAN1 cannot reach consensus on reusing CG-DFI mechanism for CG-SDT for operation in licensed band. Pathloss determination - The pathloss for CG-SDT PUSCH power control can be determined by the measurement of selected SSB associated with the CG PUSCH. QCL assumption - For CG-SDT, the UE can assume the PDCCH carrying the DCI has the same DM-RS antenna port quasi co-location properties as for a SSB associated to the CG PUSCH transmission e.g. for detection of retransmission DCI in response to a CG PUSCH transmission.

Regarding the five questions from RAN2 in LS R1-2108715, the responses from RAN1 are provided as follows:

Q1: For both RA-SDT and CG-SDT, RAN2 assumes that common PUCCH resources (i.e. those that are shared with non-SDT UEs) can also be used for HARQ-ACK feedback for Msg4 /MsgB and subsequent SDT transmissions. Can RAN1 confirm this? A1: - RAN1 confirms that common PUCCH resources (i.e. those that are shared with non-SDT UEs) can also be used for HARQ-ACK feedback for Msg4 /MsgB and subsequent SDT transmissions. Q2: For RA-SDT and CG-SDT, for Msg4 /MsgB and subsequent SDT transmissions, does RAN1 think there is a need for any other PUCCH resources than the above and if needed, can RAN1 define these? A2: - RAN1 thinks there is no need for any other PUCCH resources than common PUCCH resources shared with non-SDT UEs. Q3: Is there any other L1 configuration needed for both RA-SDT and CG-SDT to support the subsequent data transmissions from RAN1 perspective? A3: - From RAN1’s perspective, there is no other L1 configuration for RA-SDT and CG-SDT to support subsequent data transmission. Q4: Do RAN1 have any concerns to support RA-SDT on the non-initial BWP? NOTE: It has already been agreed in RAN2 that CG-SDT resource can be configured on either initial BWP or separate SDT BWP, if confirmed by RAN1. A4: - It’s RAN1’s common understanding that RA-SDT resource cannot be configured on non-initial BWP. - RAN1 cannot reach a consensus on whether to confirm RAN2 agreement that CG-SDT resource can be configured on separate SDT BWP with the concern on necessity. Q5: Does RAN1 think that BFD/BFR procedure is required for SDT and if needed, can RAN1 define the necessary procedure to support this? A5: - It’s RAN1’s common understanding that BFD/BFR procedure is not required for SDT in Rel-17. - FFS: whether or not to support reporting the beam change to gNB.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

4. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107-e			11th Nov – 19th Nov 2021 	    		e-meeting