3GPP TSG RAN WG1 Meeting #106-bis-e															R1-2110404

e-Meeting, October 11 – 19, 2021

Source: 	Moderator (Intel Corporation)

Title:	Issue Summary for initial access aspects of NR extension up to 71 GHz

Agenda item:	8.2.1

Document for:	Discussion

# Introduction

In this contribution, we discuss aspects related to initial access for extending NR up to 71 GHz based on submitted contributions to RAN1 #106-bis-e. The main issues discussed in the following section for initial access are detailed design for synchronization signal block (SSB), CORESET#0, PRACH related issues, and discovery reference signal (DRS) related operations.

During the last RAN Plenary, the WID has been updated to reflect the approved numerologies for initial access. The following is copy of the WID objectives relevant for initial access.

- Physical layer aspects including [RAN1]: - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion. - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

# Summary of issues

## 2.1 SSB Aspects

### 2.1.1 DRS Related Aspects (and other MIB design other than CORESET#0/Type0-PDCCH)

- From [1] Huawei/HiSilicon:

  - For SSB with 120 kHz SCS, confirm the working assumption on 64 candidate SSBs within a half frame.

  - For SSB with 480 kHz and 960 kHz SCS, 64 candidate SSBs is sufficient for operation without shared spectrum while 128 candidate SSBs should be supported for operation with shared spectrum.

  - For operation with shared spectrum and for 480 kHz and 960 kHz SSBs, indicate the 7th bit of the candidate SSB index by borrowing the 4th LSB of SFN in the PBCH payload. Indicate the 4th LSB of SFN with spare bit in MIB payload.

  - Support discovery burst transmission window (DBTW) for all three numerologies in shared spectrum in 52.6GHz to 71GHz. At least  NSSBQCL=16,32,64 should be indicated in MIB for all three numerologies.

  - Configure DBTW length in SIB1 for operation with shared spectrum in 52.6GHz to 71GHz with the following values:

    - 480 kHz SCS: {72, 32, 24, 16, 8, 4} slots = {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms

    - 960 kHz SCS: {64, 32, 24, 16, 8, 4} slots = {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms

  - No indication for licensed or unlicensed operation is required in MIB.

  - Use of LBT should be indicated in SIB1 to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0. Common DCI size should be assumed for DCI format 1-0/0-0 in CSS no matter LBT is ON or OFF.

  - In operation with shared spectrum in 60 GHz, for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

    - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

    - if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

  - Regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense in ssb-PositionsInBurst configured in SIB1, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

  - the MIB content and PBCH payload in Table [1]-6 and Table [1]-7should be supported for 120 kHz, 480 kHz and 960 kHz SSB.

    - Table [1]-6 MIB and PBCH payload bit allocation for 120kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |
| --- | --- | --- | --- |
|  |  | 120kHz | 120kHz |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  | 5 |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |
|  | 8 |  |  |
|  | 9 |  |  |
|  | 10 |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (Sec 3.1 Table 1) |
|  | 13 |  |  |
|  | 14 |  |  |
|  | 15 |  |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero | searchSpaceZero (Sec 3.3 Table 4) |
|  | 17 |  |  |
|  | 18 |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |
|  | 20 | cellBarred | cellBarred |
|  | 21 | intraFreqReselection | intraFreqReselection |
|  | 22 | spare | Spare bit |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |
|  | 27 | half frame indication | half frame indication |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |

  - Table [1]-7 MIB and PBCH payload bit allocation 480kHz and 960kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |  |
| --- | --- | --- | --- | --- |
|  |  | 120kHz | 480kHz and 960kHz |  |
|  |  |  | DBTW OFF | DBTW ON |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |  |
|  | 1 |  |  |  |
|  | 2 |  |  |  |
|  | 3 |  |  |  |
|  | 4 |  |  |  |
|  | 5 |  |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |  |
|  | 8 |  |  |  |
|  | 9 |  |  |  |
|  | 10 |  |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |  |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (sec 3.2 Table 2) |  |
|  | 13 |  |  |  |
|  | 14 |  |  |  |
|  | 15 |  | searchSpaceZero (Sec 3.3 Table 5) |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero |  |  |
|  | 17 |  |  |  |
|  | 18 |  |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 20 | cellBarred | cellBarred |  |
|  | 21 | intraFreqReselection | intraFreqReselection |  |
|  | 22 | spare | Spare bit | 4th LSB of SFN |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN | 7th bit of candi. SSB index (sec 2.1) |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |  |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |  |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |  |
|  | 27 | half frame indication | half frame indication |  |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |  |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |  |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |  |

- From [2] Futurewei:

  - For FR2-2 120 kHz SCS support SS/PBCH DBTW.

  - For 480/960 kHz SS/PBCH DBTW should not be supported.

  - If DBTW is supported use i modNSSBQCL where i is the candidate SS/PBCH block index to establish a QCL relation between different SS/PBCH indexes.

  - For 120kHz SS/PBCH SCS indicate that DBTW is enabled in SIB1 and indicate LBT disabled either in MIB or in SIB1.

  - For 480/960 kHz SS/PBCH SCS use the field subCarrierSpacingCommon to indicate LBT disabled.

  - Consider whether the ssb-PositionsInBurst definition needs to be updated to support higher SCS SSB.

- From [3] Spreadtrum:

  - Confirm the working assumption that DBTW is supported at least for 120kHz SCS.

  - DBTW is supported for 480/960kHz SCS.

  - Confirm the working assumption that the number of candidate SSBs in a half frame is 64 for 120kHz SCS.

  - The number of candidate SSBs in a half frame is more than 64 and not great than 128 for 480/960kHz SCS.

  - The maximum DBTW length for 480/960kHz SCS can be 2ms and 1ms respectively.

  - The gap slots (slots without SSB) for 480/960kHz SCS can be different from that of 120kHz SCS.

- From [4] ZTE, Sanechips:

  - The following design of candidate SSBs with SCS 480/960 kHz in a half frame can be considered: First symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

    - If DBTW is not supported or DBTW is disabled

      - For 480kHz SCS, the 64 candidate SSBs are located in 32 slots, with 2  slots spacing between every 8 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

      - For 960kHz SCS, the 64 candidate SSBs are located in 32 slots, with 4  slots spacing between every 16 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35

    - If DBTW is supported and it is enabled

      - Additional 64 candidate SSB can be defined after the above original 64 candidate SSBs in the half frame

  - Discovery burst transmission window (DBTW) should be supported for all approved SSB SCS in FR2-2, including 120 kHz, 480 kHz and 960 kHz.

  - In order to reduce the impact of standardization caused by indicating candidate SSB indices, the maximum number of candidate SSBs defined in the half-frame can be kept unchanged (maintain 64) or limited to 128 for 480/960 kHz SSB SCS.

  - Four candidate values {8,16,32,64} for NSSBQCL are preferred from the perspective of reducing bit overhead.

  - For Rel-17 above 52.6GHz, it is recommended that the UE derives the QCL relation between candidate SSBs by the value of i modNSSBQCL , where i is the candidate SSB index.

  - Enable/disable of DBTW can be implicitly indicated by comparing the value of  NSSBQCL in MIB and DBTW length, and explicit signaling is not needed for this purpose.

- From [5] vivo:

  - Support DBTW in un-licensed band/LBT case from 52.6 GHz to 71 GHz for SSB with all supported SCSs.

  - Do not support explicit indication of DBTW on/off in MIB.

  - Support to use DBTW lengths {0.5, 1, 2, 3, 4, 5} msec for SCS 120 kHz, and FFS small values for SCS 480 kHz and 960 kHz.

  - Do not support LBT on/off indication in MIB.

  - The following fields could be considered to indicate the value of Q in PBCH:

    - subCarrierSpacingCommon

    - LSB of ssb-SubcarrierOffset

    - Coreset#0 and Type#0 PDCCH indication

  - When DBTW is enabled with indicated value of Q, how to interpret the meaning of ssbPositionsInBurst should be studied.

- From [8] NEC:

  - DBTW should be supported for SSB transmission with 120 kHz and 480/960 kHz SCS.

  - The long term sensing could be considered as an approach to enabling/disabling DBTW.

  - DBTW indication could be indicated per beam for SSB transmission.

  - Unlicensed/licensed operation indication should not be indicated in MIB.

  - LBT on/off indication should not be indicated in MIB.

  - The value of Q should be no lower than 16 at least.

  - The candidate SSB indication in NR-U should be reused with enhancement to indicate DBTW enabling/disabling and Q value jointly in MIB.

  - If DBTW is additionally supported for 480/960kHz SCS SSB transmission, 128 SSB candidates should be supported.

- From [9] CATT:

  - DBTW for 480/960 kHz SSB SCS can be supported with up to 128 candidate SSB index.

  - To indicate 7th bit of the candidate SSB index for 480/960 kHz SSB SCS, following schemes can be further considered and down-selected:

    - Borrowing the subCarrierSpacingCommon in MIB

    - Borrowing the 4th LSB of SFN, and  move  4th LSB of SFN  to  subCarrierSpacingCommon in MIB

    - Borrowing half frame bit  , with all candidate SSBs are assumed to be put in first half frame when DBTW is enabling.

  - For NR operation in 60 GHz unlicensed spectrum, the discovery burst transmission window (DBTW) shall be supported for 120 KHz SSB at least when gNB configures more than 56 SSB transmissions.

  - sub-set of SSBs can be transmitted as NO-LBT and the other sub-set SSBs are transmitted as DBTW if the exempt Short Control Signaling rules can be applied by local region rule.

  - For number of  NSSBQCL ，four states {16, 32, 64, reserved/DBTW disabled} is recommend.

  - On DBTW length for SCS 480/960 KHz (if supported), scale factor is applied comparing to value of SCS 120 KHz,

  - For 120 kHz SSB, signaling in MIB can indicate enable/disable of DBTW.

  - If LBT ON/OFF state is indicated in MIB/PBCH, joint coding can be used for indication of LBT ON/OFF, DBTW enabling/disabling and one bit information for candidate SSB index.

  - If LBT ON/OFF state is not indicated in MIB/PBCH, it can be indicated in DCI 1_0 scrambled by SI-RNTI.

- From [11] Ericsson:

  - If a DBTW is supported (not our preference), it should only be supported for 120 kHz SSB SCS and not for 480/960 kHz SSB SCS.

  - Confirm the working assumption that no additional (compared to the already supported 64) candidate SS/PBCH block positions are introduced.

  - Conclude that a DBTW is not supported.

  - If a DBTW is supported (not our preference) select one of the following options:

    - Option 1: Q and DBTW on/off indicated in MIB using the subCarrierSpacingCommon field

      - Q = [64, 32] where Q=64 indicates DBTW off

      - DCI 1_0 size is the same for LBT on/off (unlicensed/licensed)

    - Option 2: Q and DBTW on/off indicated in SIB1

      - The subCarrierSpacingCommon field is ignored

      - Default assumption for Q depends on the agreed value range of Q and can be left to UE implementation

      - Q = [48, 32, 16, 8]. Absence of the parameter in SIB1 indicates DBTW off.

      - DCI 1_0 size is the same for LBT on/off (unlicensed/licensed)

    - Option 3: Q indicated in SIB1 and DBTW on/off indicated in MIB using the subCarrierSpacingCommon field

      - Default assumption for Q (if DBTW on) depends on the agreed value range of Q and can be left to UE implementation

      - Q = [48, 32, 16, 8]. The parameter is only configured in SIB1 if DBTW is on

      - DCI 1_0 size is the same for LBT on/off (unlicensed/licensed)

- From [12] Nokia, NSB:

  - The design for DBTW, if supported, is common to different sub-carrier spacings.

  - Confirm the working assumption on number of SSB candidate locations in a half frame for 120kHz:

  - For 480kHz and 960kHz, the number of SSB candidate locations in a half frame is 64.

  - If DBTW is supported, NSSBQCL=32,64 is supported. FFS for need for other values.

  - Provide LBT on/off and DBTW indication in SIB1. (Note: licenced/unlicenced operation is assumed to be already part of SIB1 via frequency band information.)

  - Do not provide separate, additional indication for DBTW on/off in MIB. (Note it would be possible to provide the indication implicitly e.g. part of NSSBQCL.)

- From [13] Samsung:

  - Support discovery burst transmission window for all SCSs on the 60 GHz unlicensed spectrum.

    - The indication of Q can be in MIB for a best effort, and if not possible, in SIB1;

    - The indication of DBTW disabling can be joint coded with the indication of Q, if Q is indicated in MIB; and the indication can use 1 bit in MIB, if Q is not indicated in MIB;

    - For 480 kHz and 960 kHz SCS, support 128 candidate SS/PBCH block locations within a half frame, and use one PHY bit in PBCH payload to indicate the extra candidate SS/PBCH block index (e.g. 7th LSB);

    - For initial access, different synchronization raster entries are applied for licensed and unlicensed operations; for non-initial access, support an explicit indication of licensed or licensed operation when configuring a cell.

- From [15] Intel:

  - For DRS based on SSBs with SCS 120 kHz:

    - Lmax=64 and reuse Case D slot pattern for placement of SSB candidates

    - NSSBQCL is indicated in MIB

      - At least the subCarrierSpacingCommon bit from MIB is reinterpreted to indicate NSSBQCL∈16,64

      - Consider 1 bit from pdcch-ConfigSIB1 in MIB to indicate NSSBQCL from the extended set, e.g., NSSBQCL∈8,16,32,64

        - Alternatively, the spare bit from PBCH payload could be used to indicate NSSBQCL∈8,16,32,64 in addition to the subCarrierSpacingCommon bit from MIB

    - DBTW on/off is identified based on comparison of the DBTW length with the time duration occupied by transmission of NSSBQCL SSBs

    - DBTW length is signalled in SIB1

    - Licensed vs. unlicensed operation is not signalled in MIB

      - Align sizes of DCI 1_0 scrambled with SI-RNTI between licensed and unlicensed modes of operation

  - For DRS based on SSBs with SCS 480 kHz/960 kHz:

    - Lmax=128 and SSB candidate slots are arranged according to Proposal 2

    - One bit from MIB is used for indexing additional SSB candidates

      - The subCarrierSpacingCommon bit from MIB is reinterpreted for this purpose

    - NSSBQCL is indicated in MIB

      - One bit from pdcch-ConfigSIB1 in MIB is repurposed to indicate at least NSSBQCL∈16,64

    - The spare bit from PBCH payload could be used to indicate NSSBQCL∈8,16,32,64 in addition to 1 bit from pdcch-ConfigSIB1 in MIB

    - DBTW length is fixed and not signalled

    - DBTW on/off is explicitly signalled in SIB1

    - Licensed vs. unlicensed operation is not signalled in MIB

      - Align sizes of DCI 1_0 scrambled with SI-RNTI between licensed and unlicensed modes of operation

- From [16] NTT Docomo:

  - DBTW should be supported irrespective of SCS.

    - In a certain region, e.g., Japan, sensing needs to be performed for initiating any transmission by any device in 60 GHz.

  - Support to confirm the working assumption that the number of candidate SSBs with 120 kHz SCS in a half frame is 64

  - Support 64 candidate SSBs with both 480 and 960 kHz SCS

  - For DBTW to be supported in Rel-17 NR 52.6 – 71 GHz, similar to Rel-16 NR-U, support to indicate QCL parameter in MIB

    - Support to use subCarrierSpacingCommon for QCL parameter indication in MIB

  - For DBTW to be supported in Rel-17 NR 52.6 – 71 GHz, following information can be implicitly indicated via subCarrierSpacingCommon

    - Enabling/disabling of DBTW

    - Licensed/unlicensed band

    - LBT on/off

- From [17] Pansonic:

  - DBTW is supported regardless of SCS.

  - The number of candidate SSB positions is 64.

  - For NSSBQCL values, total of 4 states are supported (e.g., {8, 16, 32, 64}).

  - For the indication of Q, SIB1 is used except the signaling method to use MIB are clarified.

  - If Q is indicated in MIB, DBTW enabled/disabled is indicated in MIB (implicitly Q=64). If Q is indicated in SIB1, DBTW enabled/disabled is indicated in SIB1.

  - DBTW lengths for 480/960 kHz SCS are scaled from 120 kHz SCS.

- From [18] Sony:

  - Discovery Burst Transmission Window should be supported for all SCSs.

  - Enabling/disabling DBTW and NSSBQCL should be signalled in MIB

    - Indication of disabling DBTW should be jointly coded with NSSBQCL

      - Parameter to signal NSSBQCL indicates {8, 16, 32, or 64}

      - NSSBQCL = 64 implies disabling DBTW

  - For indication of QCL relation and disabling DBTW in MIB, subCarrierSpacingCommon and reserved state of pdcchConfig-SIB1 should be used.

- From [19] ETRI:

  - Propose to support DBTW for all SSB SCSs and the same DBTW lengths with Rel-16 NR-U.

- From [20] Lenovo, Motorola Mobility:

  - For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, potential enhancements related to periodic transmission of DRS such as SSB/PBCH/CORESET#0 are needed including:

    - performing directional LBT prior to the transmission of SSB according to the ssb-PositionsInBurst

    - directional LBT on multiple beams at the same time at the beginning of the DRS window

    - Cat 2 LBT (depending on the gap) before actual transmission

- From [21] Interdigital:

  - Support Discovery Burst (DB) and Discovery Burst Transmission Window (DBTW) in unlicensed spectrum operations that require LBT to enhance the initial access operation in beyond 52.6GHz spectrum.

  - Consider indicating enable/disable of DBTW in initial access operations based on a range of the sync raster offset.

  - Consider the enhancements to indicate the mode of operation regarding the enable/disable of the DBTW, on/off of the LBT, and the license regime based on the combination of Sync. raster offset and MIB indication.

  - Support enhancements on the reference tables in indication of Q parameter for up to 64 SSB beams in initial access operations for unlicensed spectrum in beyond 52.6GHz, e.g., subsamples of the Q parameter.

  - Support candidate SSB positions more than 64 for 120kHz SSB.

- From [22] LG Electronics:

  - For FR2-2, UE always assumes DBTW is enabled for 120 kHz SSB reception.

  - Total of 4 states (e.g., {8, 16, 32, 64}) of NSSBQCL values are supported by using 2 bits of the followings.

    - subCarrierSpacingCommon

    - LSB(s) of ssb-SubcarrierOffset

    - dmrs-TypeA-Position

  - No MIB indication to identify operation with or without shared spectrum channel access, but SIB indication or synchronization raster differentiation to identify operation with or without shared spectrum channel access.

  - Do not indicate LBT on/off in PBCH. DCI format 1_0 size should be aligned regardless of LBT on or off unless synchronization rasters are used to identify operation with or without shared spectrum channel access.

  - Discuss how to signal actually transmitted SSBs via ssb-PositionsInBurst when NSSBQCL can be indicated to be less than 64 in MIB.

- From [23] Sharp:

  - Adopt DBTW for SSB with 120 or 480 or 960 kHz SCS in FR2-2 operation.

  - One MIB payload bit is used for indication of candidate SSB index for Lmax>64. Another MIB payload bit indicates Q related information. DBTW enabled/disabled is not explicitly indicated via MIB. These two bits are repurposed from the bit for subCarrierSpacingCommon indication and the LSB for ssb-SubcarrierOffset indication.

- From [24] Apple:

  - If DBTW is introduced, for above 52.6GHz frequency band, consider the following:

    - Re-purposing the 1-bit 'subCarrierSpacingCommon'

    - If more than one bit is needed, re-purposing 1-bit MSB of controlResourceSetZero in MIB or providing one more bit information by selecting one sequence from two candidates to scramble CRC bits of PBCH payload.

  - LBT on/off can be implicitly indicated based on the indication of DBTW enable/disable.

  - Licensed/unlicensed band can be signaled in SIB1.

  - The same DCI size is used for DCI format 1_0 monitored in a common search space in both licensed and unlicensed band with existing padding operation.

- From [25] Convida Wireless:

  - If impact of LBT failure is not addressed, increasing the number of SSB candidate positions to above 64 to increase transmission opportunities to cope with LBT failure could be considered.

  - Increased number of candidate SSB positions for unlicensed/shared spectrum channel access with LBT could be considered for SCSs of 480KHz and 960KHz for 52.6 GHz-71 GHz.

- From [26] Qualcomm:

  - do not support discovery burst transmission window (DBTW) for SSB for SCS 480 and 960 kHz

  - for an unlicensed band that requires LBT, if DBTW for SSB is adopted for 120 kHz SSB:

    - MIB signaling to support indication of NSSBQCL for 120 kHz SSB

    - Minimize the number of bits needed to signal NSSBQCL (1 or 2 bits) and thus the values (2 or 4 values)

    - Enabling/disabling DBTW can be implicit in the NSSBQCL value

    - Consider getting the bits needed from one or more of the following: controlResourceSetZero, subCarrierSpacingCommon

    - Confirm the working assumption that the number of candidate positions when DBTW is enabled = 64 for 120 kHz SSB

    - Consider having a subset of the SSBs (< 64) transmitted under the short control signal assumption while another subset can be best effort or have multiple positions per beam (have a NSSBQCL within the subset)

  - consider increasing the size of the DCI 1_0 for NR licensed, by adding a field, to align with the size of the corresponding DCIs for the unlicensed operation.

- From [27] WILUS:

  - We propose to support discovery burst transmission window (DBTW) for at least 120kHz SCS which makes it possible to define candidate SSB positions within the DBTW. In addition to 120kHz SCS, DBTW should be applicable for 480/960 kHz SSB SCS on supporting NR above 52.6GHz.

  - Before confirming the working assumption that the number of candidates SSBs in a half frame is 64 for 120kHz SSB, it would be necessary to consider a method for compensating for the insufficient opportunity of the SSB transmission due to LBT failures in order to perform the operation in the unlicensed band of above 52.6GHz.

  - It should be further considered that the additional candidate SS/PBCH block locations within a DBTW can be set to the closest slot locations after LBT failure at candidate SS/PBCH blocks locations as defined in FR2.

#### Summary of Discussions

The following are previous agreements on DRS aspects.

RAN1 #105e Agreement: For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS - If DB supported - FFS: What signals/channels are included in DB other than SS/PBCH block - If DBTW is supported - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - The following points are additionally FFS: - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported Agreement: - For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0 - FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - FFS: details of how to inform UEs of the configuration of DBTW Agreement: FFS: Support DBTW at least for 120kHz - FFS whether DBTW will be applicable for 480/960 kHz SSB SCS - If DBTW is supported for 480/960kHz SSB: - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW, , and DBTW length) are supported by dedicated signaling. - For 120kHz SSB, support mechanism to distinguish at least the following scenarios: - Case 1) (Unlicensed with LBT off) + DBTW disabled - Case 2) (Unlicensed with LBT on) + DBTW enabled - Case 3) (Unlicensed with LBT on) + DBTW disabled - Case 4) (Licensed) + DBTW disabled - FFS: Whether/how LBT on/off is indicated in MIB - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling - FFS: whether all above cases need an explicit indication - FFS: Whether a single indication can be used for combination of more than one cases - For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods: - Option 1) signaling in MIB - Option 1-1) disabling DBTW is jointly coded with - Option 1-2) indicated by other bit fields in MIB - FFS: among options 1-1 and 1-2 - Option 2) distinct GSCN used by the SSB - Option 3) By comparing the value of in MIB and DBTW length after UE reads SIB1 or by comparing the value of in MIB and default DBTW length of 5 ms before UE reads SIB1. - FFS: whether to support option 1, 2, 3, or any combination of the options. - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods Agreement: If DBTW is supported, - Working assumption: MIB signaling to support - Alt A) indication of at least for 120kHz SSB - In this case, the total number of values of to not exceed 4 - Alt B) Explicit indication of SSB index and/or SSB candidate location - FFS on the details of signaling - FFS between Alt A, or B, or supporting both - Supported DBTW lengths - Alt 1) 0.5, 1, 2, 3, 4, 5 msec - Note: same as Rel-16 FR1 NR-U - Alt 2) maximum 5 msec - FFS other values - FFS between Alt 1 and 2 - Number of candidate positions when DBTW is enabled - For 120kHz SSB - FFS between 64 or 80 - If DBTW is additionally supported for 480/960kHz SSB - FFS between 64 or 128 RAN1 #106e Conclusion: RAN1 will continue discussions to develop solutions for supporting DBTW Agreement: For DBTW with 120kHz SCS (if supported), support DBTW lengths {0.5, 1, 2, 3, 4, 5} msec - Note: this should be the same as Rel-16 NR-U DBTW lengths. Working assumption: For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

The following is a summary of company position of various aspects of DRS.

- Supporting DBTW

  - Support: Huawei/HiSilicon, Futurwei (120kHz only), ZTE/Sanechips, vivo, NEC, Nokia/NSB, Intel, Docomo, Panasonic, Sony, ETRI, Interdigital, Sharp, WILUS

  - Do not support: Ericsson (if supported only for 120kHz only), Qualcomm (not support for 480/960kHz)

- Indication of DBTW (for initial access)

  - in MIB (either explicit or implicit with Q=64):

    - implicit: ZTE/Sanechip, NEC, Samsung (if Q is indicated in MIB), Docomo, Panasonic, Sony, Sharp, Apple, Qualcomm (for 120kHz)

    - no explicit signaling in MIB: vivo

    - explicit: CATT, Samsung (if Q is not indicated in MIB)

  - in SIB1:

    - Futurewei, Nokia/NSB

  - raster:

    - Interdigital, Samsung

- Supporting means of conveying candidate SSB location & SSB beams

  - Supported NSSBQCL values:

    - 120kHz {16,32,64} : Huawei/HiSilicon

    - 480/960kHz {16,32,64} : Huawei/HiSilicon

    - {8,16,32,64}: ZTE/Sanechips, Intel (if 2 bit for Q), Panasonic, Sony

    - Min 16: NEC

    - {16, 32, 64, reserved/DBTW disabled}: CATT

    - {32, 64} 64 serves DBTW disable: Ericsson (if DBTW supported, if Q indicated in MIB, as one option)

    - {8,16,32,48} : Ericsson (if DBTW supported, if Q indicated in SIB1, as one option)

    - {32, 64}: Nokia/NSB

    - {16,64}: Intel (if 1 bit for Q)

  - Potential bits for required signaling (e.g. additional SSB index, Q) for supporting DBTW in MIB

    - subCarrierSpacingCommon: Huawei/HiSilicon, vivo, Ericsson (if DBTW supported, as one option), Intel, Docomo, Sony, LGE, Apple, Qualcomm (for 120kHz)

    - controlResourceSetZero: vivo, Intel (for 480/960kHz), Sony, Apple, Qualcomm (for 120kHz)

    - searchSpaceZero: Huawei/HiSilicon, vivo

    - some bits of k_SSB: vivo, LGE

    - dmrs-typeA-position: LGE

    - spare bit (not the Msg Extension bit): Intel

  - Placement of candidate SSB index in PHY portion of PBCH (not in MIB) (requires moving 1 bit of SFN from PHY portion of PBCH to MIB.

    - Support: Huawei/HiSilicon, Samsung

    - FFS: CATT

  - NSSBQCL indication in SIB1

    - Ericsson (if DBTW supported, as one option)

- Supported DBTW lengths for 480/960 kHz (if supported)

  - For 480kHz:

    - {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms : Huawei/HiSilicon

    - Max 2 ms: Spreadtrum

    - Scaled version of 120kHz case: CATT, Panasonic

    - Fixed to single value: Intel

  - For 960kHz:

    - {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms : Huawei/HiSilicon

    - Max 1ms: Spreadtrum

    - Scaled version of 120kHz case: CATT, Panasonic

    - Fixed to single value: Intel

- Number of SSB candidates for DBTW

  - For 120kHz:

    - confirm WA

      - Huawei/HiSilicon, Spreadtrum, Ericsson, Nokia/NSB, Intel, Docomo, Qualcomm, ETRI

    - Wait for confirming WA after decision for 480/960kHz is made:

      - CATT

    - Support additional values of n

      - NEC

  - For 480/960kHz:

    - 64: Huawei/HiSilicon (licensed), ZTE (if DBTW not supported/disabled), Docomo, Panasonic

    - 64 < 128 ≤ 128: Spreadtrum

    - < 64: Interdigital, Convida

    - 128: Huawei/HiSilicon (unlicensed), ZTE (if DBTW supported/enabled), NEC, CATT, Samsung, Intel

- ssb-PositionsInBurst in SIB1

  - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted; if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

    - Huawei/HiSilicon

  - FFS: Futurewei, vivo, LGE

- Indication of licensed and unlicensed operation in MIB:

  - Support: [Docomo]

  - Not support: Huawei/HiSilicon, NEC, Intel, LGE, Apple

- Indication of LBT

  - MIB: Futurewei (480/960kHz), [Docomo], Apple (implicit with DBTW)

  - SIB1: Nokia/NSB, Intel, [LGE],

  - If indicated, joint encoding with DBTW enable/disable: CATT

  - If not indicated, provide indication in DCI 1_0 scrambled by SI-RNTI: CATT

- DCI sizes between licensed and unlicensed

  - Same size for CSS DCI 1_0/0_0: Huawei/HiSilicon, Ericsson, Intel, LGE, Apple, Qualcomm

#### <Moderator’s Suggestion for Discussions>

Discuss further on the following proposals and issues. The proposals listed are not unanimously supported by all companies. However, more numbers of companies seem to support the proposal. Therefore, moderator suggests using the proposal as basis for further discussions (even if they may not be agreeable).

Issue #1) Whether or not to support DBTW and number of SSB candidates

##### Proposal 1.1-1 – suggested for GTW discussion

- Support DBTW for 120kHz, 480kHz, and 960kHz cases

##### Proposal 1.1-2 – suggested for GTW discussion

- If DBTW is supported for 480 and 960 kHz, support 128 candidate SSB positions

Issue #2) Potential bits for required signaling for supporting DBTW in MIB

Discuss and identify which bits are available for required signaling for supporting DBTW in MIB

- subCarrierSpacingCommon

  - Seems to be unanimous support from all companies

- controlResourceSetZero

- searchSpaceZero

- some bits of k_SSB

- dmrs-typeA-position

- spare bit (not the Msg Extension bit)

Issue #3) Indication of DBTW & NSSBQCL

##### Proposal 1.1-3

- If DBTW is supported, support implicit indication DBTW may be disabled with NSSBQCL = 64 configuration.

##### Proposal 1.1-4

- If DBTW is supported, for NSSBQCL values:

  - If 2 bits are available in MIB for NSSBQCL, at least support {16, 32, 64}

  - If 1 bit is available in MIB for NSSBQCL, support {32, 64}

Issue #4) DCI size

##### Proposal 1.1-5

- Same DCI size for DCI 1_0 and 0_0 in CSS between licensed and unlicensed operation in 60 GHz.

  - Bit will be padded to the format with smaller DCI between licensed and unlicensed operation to match the DCI size between them.

  - Existing DCI size alignment in TS38.213 applies to DCI 1_0 and 0_0 in CSS.

Issue #5) DBTW lengths

##### Proposal 1.1-6

- If DBTW is supported, the following DBTW length are supported for 480 and 960 kHz:

  - {2.25, 1, 0.75, 0.5, 0.25, 0.125} msec for 480 kHz.

  - {1, 0.5, 0.375, 0.25, 0.125, 0.0625} msec for 960 kHz

Issue #6) Indication of licensed/unlicensed and LBT/no LBT in MIB

##### Proposal 1.1-7

- Indication of licensed and unlicensed operation is not explicitly indicated in MIB content payload.

- Indication of use of LBT or no-LBT is not explicitly indicated in MIB content payload.

  - If explicit indication of DBTW disabled is supported, use of no-LBT may be inferred from DBTW disabled indication.

Issue #7) ssb-PositionsInBurst in SIB1

##### Proposal 1.1-8

- For ssb-PositionsInBurst in SIB1,

  - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

  - if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

### 2.1.2 SSB Resource Pattern

- From [1] Huawei/HiSilicon:

  - Support following patterns for SSB with 480 kHz and 960 kHz SCS:

    - For operations without shared spectrum:

      - {2,9}+14n, (n=0,1,2,…,31) for both 480 kHz and 960 kHz SCS.

    - For operations with shared spectrum:

      - {2,9}+14n, (n=0,1,2,…,31,40,…,71) for 480 kHz SCS;

      - {2,9}+14n, (n=0,1,2,…,63) for 960 kHz SCS.

- From [2] Futurewei:

  - For SS/PBCH transmission at 480kHz and respectively 960kHz use “n” values that correspond to SS/PBCH transmission gaps of 8 slots and respectively 16 slots to allow low latency traffic transmissions.

- From [4] ZTE, Sanechips:

  - The following design of candidate SSBs with SCS 480/960 kHz in a half frame can be considered: First symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

    - If DBTW is not supported or DBTW is disabled

      - For 480kHz SCS, the 64 candidate SSBs are located in 32 slots, with 2 slots spacing between every 8 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

      - For 960kHz SCS, the 64 candidate SSBs are located in 32 slots, with 4 slots spacing between every 16 consecutive slots to avoid prolonged occupation, i.e. n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35

    - If DBTW is supported and it is enabled

      - Additional 64 candidate SSB can be defined after the above original 64 candidate SSBs in the half frame

- From [5] vivo:

  - Non-LBT scenario: the value of ‘n’ for SCS 480 kHz and 960 kHz can be set as:

    - n=0,1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,40,41,44,45,48,49,52,53,56,57,60,61,64,65,68,69.

  - LBT scenario: the value of ‘n’ for SCS 480 kHz and 960 kHz can be set as:

    - n=0,1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,40,41,44,45,48,49,52,53,56,57,60,61,64,65,68,69, 80,81,84,85,88,89,92,93,96,97,100,101, 104,105, 108, 109,120,121,124, 125, 128, 129,132,133,136,137,140,141,144,145,148,149

- From [8] NEC:

  - Additional n values of 4, 9, 14 and 19 should be supported to indicate 80 candidate SSBs in DBTW at least for 120 kHz SCS SSB pattern.

  - The indication of additional candidate SSBs based on additional values should be investigated.

- From [9] CATT:

  - Confirming the working assumption of candidate SSB index number of 120 kHz SCS can be postponed to after the decision on maximum number of can SSB index supported for 480/960 kHz.

- From [11] Ericsson:

  - For SS/PBCH block with 120 kHz SCS, no new values of n are supported. Hence the Case D pattern from Rel-15 is supported.

  - For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame, and n = 0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37.

- From [12] Nokia, NSB:

  - Support in for 480kHz and 960kHz SSB pattern design with slots without SSB candidate locations at every 0.25ms.

  - Define SSB slot pattern for 480kHz and 960kHz sub-carrier spacing so that 8 consecutive slots are contain SSB candidate locations, followed by 4 slots are left unoccupied (by SSBs), until all SSBs locations are accounted. Determine the slot indexes n for candidate locations as follows:

    - The slot indexes n={0,1,2,3,4,5,6,7,

    - 12,13,14,15,16,17,18,19,

    - 24,25,26,27,28,29,30,31,

    - 36,37,38,39,40,41,42,43}

- From [13] Samsung:

  - For 480 kHz and 960 kHz, the first symbols of candidate SS/PBCH block have indexes 2, 9+14⋅n, wherein:

    - n=0,1,…, 31 for 480 kHz SCS and operation without shared spectrum channel access;

    - n=0, 1, …, 31, 40, 41, …, 71 for 480 kHz SCS and operation with shared spectrum channel access;

    - n=0,1,…, 31 for 960 kHz SCS and operation without shared spectrum channel access;

    - n=0, 1, …, 63 for 960 kHz SCS and operation with shared spectrum channel access.

- From [15] Intel:

  - For SSB SCS 120 kHz, reuse Case D pattern for SSB candidate slot positions within a half-frame.

  - Consider SSB pattern in a slot with 3 SSB containing slots, each slot with 2 SSB position, followed by 1 non-SSB carrying slot for 480 kHz and 6 SSB carrying slots followed by 2 non-SSB carrying slots for 960kHz.

    - For 480kHz and 960kHz SCS based SSB, first symbols of the candidate SSB have indexes {2,9} + 14×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

    - For 480kHz, n = {0,1,2, 4,5,6, 8,9,10, 12,13,14, 16,17,18, 20,21,22, 24,25,26, 28,29,30, 32,33,34,  36,37,38, 40,41}, {42, 44,45,46, 48,49,50, 52,53,54, 56,57,58, 60,61,62, 64,65,66, 68,69,70, 72,73,74, 76,77,78, 80, 81, 82, 84}.

      - The second set of n values could be used to enable larger number of candidate SSBs, i.e., L max=128

    - For 960kHz, n = {0,1,2,3,4,5, 8,9,10,11,12,13, 16,17,18,19,20,21, 24,25,26,27,28,29, 32,33,34,35,36,37, 40,41}, {42,43,44,45, 48,49,50,51,52,53, 56,57,58,59,60,61, 64,65,66,67,68,69, 72,73,74,75,76,77, 80,81,82,83}.

      - The second set of n values could be used to enable larger number of candidate SSBs, i.e., L max=128

- From [16] NTT Docomo:

  - For SSB slots “n” with 480/960 kHz SCS, the following alternatives can be considered where we prefer Alt 3 the best::

    - Alt 1: Reuse “n” values defined for Case D in Rel-15/16

    - Alt 2: Define “n” values as a set of consecutive slots

    - Alt 3: Define “n” values with more number of non-SSB slots between two set of consecutive SSB slots within a SSB burst

- From [17] Panasonic:

  - For SSB slot position, Case D SSB patten is reused (i.e., n = 0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37).

- From [19] ETRI:

  - Confirm the working assumption on the number of candidates SSBs for 120kHz as an agreement.

- From [20] Lenovo, Motorola Mobility:

  - For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, for higher subcarrier spacings (numerologies) such as 960kHz for SSB, to allow the beam switching between contiguous SSBs and between SSB and CORESET, a gap (for example a symbol gap or post-fix) should be supported for beam switching at least for 960kHz

- From [21] Interdigital:

  - Support using gap slots in Case D SSB pattern in SCS 120kHz for the candidate SSB positions, wherein multiple subsets of candidate SSB indexes per gap slot are considered.

- From [22] LG Electronics:

  - For 480/960 kHz SSB, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame (as per agreement made in RAN1#106-e), and values of ‘n’ are consecutive integers (i.e., n = 0, 1, 2, …, 31).

- From [23] Sharp:

  - Non-contiguous values of n with 3 or 4 gap slots between SSB slots should be considered.

- From [26] Qualcomm:

  - for 480 kHz/960 kHz SSB pattern, consider the following options:

    - For 480 kHz SCS, select one of:

      - SSB slots (n) = {1, 2, 3, 4} + 6*m, where m = 0, 1, …, 7, or

      - SSB slots (n) = {n1, n2}

        - {n1} = {1, 2, 3, 4} + 6*m, where m = 0, 1, 2, 3

        - {n2} = {33, 34, 35, 36} + 6*m, where m = 0, 1, 2, 3

    - For 960 kHz SCS:

      - SSB slots (n) = {2, 3, 4, 5, 6, 7, 8, 9} + 12*m, where m = 0, 1, …, 7

    - Keep the 20 ms initial access SSB pattern period

#### Summary of Discussions

In RAN1 #105e the following agreement was made.

Agreement: - For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, X} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame. - Alt 1: X = 8 - Alt 2: X = 9 Agreement: For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- SSB pattern for 120kHz

  - Use SSB case D from Rel-15 NR

    - Ericsson, Interdigital

- SSB slot pattern for 480 kHz:

  - Continuous slots, {2,9}+14n, (n=0,1,2,…,31)

    - Huawei/HiSilicon (for licensed), Samsung (for licensed), LGE

  - 8 slot gap every 32 slots, (n=0,1,2,…,31,40,…,71)

    - Huawei/HiSilicon (for unlicensed), Samsung (for unlicensed)

  - Gap every 8 slots

    - Futurewei

  - 2 slots gap every 8 slots, n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

    - ZTE, Sanechips, Ericsson, Panasonic

  - n=0,1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,40,41,44,45,48,49,52,53,56,57,60,61,64,65,68,69

    - vivo

  - 4 slot gap every 8 slots, n={0,1,2,3,4,5,6,7,12,13,14,15,16,17,18,19,24,25,26,27,28,29,30,31, 36,37,38,39,40,41,42,43}

    - Nokia/NSB

  - 1 slot gap every 3 slots, n = {0,1,2, 4,5,6, 8,9,10, 12,13,14, 16,17,18, 20,21,22, 24,25,26, 28,29,30, 32,33,34,  36,37,38, 40,41}, {42, 44,45,46, 48,49,50, 52,53,54, 56,57,58, 60,61,62, 64,65,66, 68,69,70, 72,73,74, 76,77,78, 80, 81, 82, 84}

    - Intel

  - N slot gap every M slots

    - Docomo

  - 3 or 4 slot gap every M slots

    - Sharp

  - 4 slot gap every 8 slots, n={2, 3, 4, 5, 6, 7, 8, 9} + 12*m

    - Qualcomm

- SSB slot pattern for 960 kHz:

  - Continuous slots, {2,9}+14n, (n=0,1,2,…,31)

    - Huawei/HiSilicon (for licensed), Samsung (for licensed), LGE

  - Continuous slots, (n=0,1,2,…,63)

    - Huawei/HiSilicon (for unlicensed), Samsung (for unlicensed)

  - Gap every 16 slots

    - Futurewei

  - 4 slot gap every 8 slots, n={0,1,2,3,4,5,6,7,12,13,14,15,16,17,18,19,24,25,26,27,28,29,30,31, 36,37,38,39,40,41,42,43}

    - Nokia/NSB

  - 8 slot gap every 16, n=0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35

    - ZTE, Sanechips

  - 2 slot gap every 6 slots, {0,1,2,3,4,5, 8,9,10,11,12,13, 16,17,18,19,20,21, 24,25,26,27,28,29, 32,33,34,35,36,37, 40,41}, {42,43,44,45, 48,49,50,51,52,53, 56,57,58,59,60,61, 64,65,66,67,68,69, 72,73,74,75,76,77, 80,81,82,83}

    - Intel

  - N slot gap every M slots

    - Docomo

  - 2 slots gap every 8 slots, n=0, 1, 2, 3, 4, 5, 6, 7, 10, 11, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37

    - Panasonic, Ericsson

  - 4 slot gap every 8 slots, n={2, 3, 4, 5, 6, 7, 8, 9} + 12*m

    - Qualcomm

#### <Moderator’s Suggestion for Discussions>

Discuss further on the following proposals, including further aspects that should be discussed together with Proposal 1.2-1 and 1.2-2.

##### Proposal 1.2-1

- Use SSB pattern case D from Rel-15 NR for 120 kHz SSB pattern

##### Proposal 1.2-2 – suggested for GTW discussion (with possible down-selection in GTW)

- Supported value of n for 480/960kHz SSB slot pattern:

  - ALT 1) contiguous, n = 0, 1, …, Lmax

  - ALT 2) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB

    - FFS: whether same pattern will apply to 480kHz and 960kHz (i.e same N and M for 480 and 960 kHz), or scaled version pattern will apply between 480 and 960 kHz (i.e. N and M for 480kHz, 2N and 2M for 960 kHz)

    - FFS: whether n will start from 0 or N

  - Moderator’s note: If Alt 2 is selected, RAN1 should work further during RAN1 #106bis-e to settle the final slot pattern (i.e. determine values of N and M and FFS aspects)

### 2.1.3 CORESET#0 Configuration

- From [1] Huawei/HiSilicon:

  - For CORESET for Type0-PDCCH in 52.6GHz to 71GHz spectrum, support the following:

    - For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, support multiplexing pattern 1 and multiplexing pattern 3 as per Agreement in RAN1 104-e.

    - For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz, support multiplexing pattern 1 only.

    - For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {960, 960} kHz, support multiplexing pattern 1 only.

  - For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, in addition to the supported values of (NRBCORESET, NsymbCORESET ) from Rel-15, support NRBCORESET= 96 with NsymbCORESET={1,2} for multiplexing pattern 1.

  - Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={120, 120} kHz:

    - For CORESET#0 with 24 RBs and 48 RBs: the same as supported values in Table 13-8 of 38.213.

    - For CORESET#0 with 48 RBs: additional RB offsets values of 0 and 28 RBs can be considered for multiplexing pattern 1.

    - For CORESET#0 with 96 RBs: RB offsets values of 0 and 76 RBs can be considered for multiplexing pattern 1.

  - Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

    - For CORESET#0 with 24 RBs: the same as supported values in Table 13-8 of 38.213.

    - For CORESET#0 with 48 RBs: In addition to the offset of 14 RBs already supported in Rel-16, additional values of 0 and28 RBs can be considered for multiplexing pattern 1.

  - The parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 listed in Table [1]-4 and Table [1]-5 should be supported. For 480kHz and 960 kHz SCS, the scaling factor X in Table 5 is 12μ-3 when DBTW is OFF and 12μ-4 when DBTW is ON.

    - Note: DBTW OFF is indicated in MIB using a value of NSSBQCL

  - To find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum, RAN1 should uniquely determine the hypothetical on-synch raster SSB that serves as the reference for the offset to the off-synch raster SSB in case more than one synch rasters are included in a channel bandwidth.

- From [2] Futurewei:

  - Rel 17 FR2-2 the SS/PBCH and CORESET#0 for Type0-PDCCH should have only the same SCS.

  - Use O from the set {0, 5, 2.5, 5+2.5} for 120 kHz, {0, 5, 2.5/X, 5+2.5/X} for 480 kHz, and {0, 5, 2.5/(2*X), 5 + 2.5/(2*X)} for 960 kHz, with X values TBD.

- From [4] ZTE, Sanechips:

  - In addition to multiplexing pattern 1, multiplexing pattern 3 for three approved SCS combinations of SSB and Type0-PDCCH can also be considered in FR2-2.

    - (SSB, Type0-PDCCH): SCS (120 kHz, 120 kHz)

    - (SSB, Type0-PDCCH): SCS (480 kHz, 480 kHz)

    - (SSB, Type0-PDCCH): SCS (960 kHz, 960 kHz)

  - For {SSB, CORESET# for Type0-PDCCH} SCS = {120, 120} kHz, even though RAN4 has agreed the minimum CBW is increased to 100 MHz, at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 should still be supported.

- From [5] vivo:

  - Support Multiplexing pattern 1 and 3 for SCS 120 kHz, and support Multiplexing pattern 3 for SCS 480 kHz and 960 kHz when operation in FR2-2.

  - Support 96 RB for SCS 120kHz and 480 kHz. Do not support 96 RB for SCS 960kHz.

  - If the sync raster/ channel raster is designed  with FR 2-1, the existing RB offset design can be reused for SCS 480 kHz and 960 kHz. Otherwise, the RB offset should be re-designed.

  - For {SSB, PDCCH} SCS {120, 120} kHz, {480, 480} kHz and {960, 960} kHz, the tables for PDCCH monitoring occasions for type0-PDCCH CSS set configuration defined for FR2-1 in Rel-15 can be reused with little adjustment.

- From [9] CATT:

  - The subCarrierSpacingCommon field in MIB can be saved and repurposed.

  - Multiplexing pattern 2 or 3 can be used for further multiplexing SSB/CORSET#0 with periodic CSI-RS/paging PDCCH&PDSCH in frequency.

  - For SSB and CORESET#0/Type0-PDCCH with 120 KHz SCS, support the following combinations of SSB/CORESET multiplexing pattern, number of RB and symbols for CORESET.

    - {mux pattern 1, 48 PRB CORESET, 1 symbol CORESET}

    - {mux pattern 1, 48 PRB CORESET, 2 symbol CORESET}

    - {mux pattern 3, 48 PRB CORESET, 2 symbol CORESET}

  - The configuration of {0, if i is even}, {NsymbCORESET, if i is odd} can be supported, considering for SCS=120 KHz use case, the gNB could use implementation to avoid beam switching gap issue if it choose to.

  - The default TDRA table for pattern 1 in TS 38.214 can be enhanced, e,g  at least {S=6 ,L=7}, {S=2，L=11}  is supported.

- From [10] Xiaomi:

  - Detail parameters modification for controlResourceSetZero configuration should be based on channel and sync raster design in RAN4.

- From [11] Ericsson:

  - RAN1 should strive to design a common CORESET0 configuration table for use for all 3 supported SCS combinations (120,120), (480,480), and (960, 960).

  - If RAN4 defines a floating channelization with a sync raster granularity in line with the design, add offset values 2 and 26 for the option of 48 RB CORESET0 and make Table 13-8 in 38.213 applicable also for operation with 480 and 960 kHz SCS.

  - Reuse existing Table 13-12 in 38.213 for operation with 480 and 960 kHz SCS. For subcarrier spacings 480 and 960 kHz, select Alternative 1 to define offset values.

- From [12] Nokia, NSB:

  - For CORESET#0 with 120kHz sub-carrier spacing, consider supporting also NRBCORESET={96} for multiplexing pattern 1.

  - For SSB and CORESET#0 with 480kHz sub-carrier spacing with SSB and CORESET#0 multiplexing pattern 3, following configuration options could be considered:

    - NsymbCORESET={2}

    - NRBCORESET={24, 48}.

  - Support the following ’O’ values for both 480 and 960 kHz sub-carrier options: {0, 1.5, 5, 6.5} ms.

  - Support values {1,2} for the number of search space sets per slot, and values {1, 1/2} for the shift M. Additionally, given room in table also M={2} could be supported.

  - Support first symbol index configuration options 0 and {0, if  is even}, {7, if  is odd}

  - Support Type0-PDCCH CSS configuration presented in Table [12]-7 for multiplexing pattern 1.

| O | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 0 |
| 0 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 1.5 | 1 | 1 | 0 |
| 1.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 5 | 1 | 1 | 0 |
| 5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 6.5 | 1 | 1 | 0 |
| 6.5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 0 | 1 | 2 | 0 |
| 5 | 1 | 2 | 0 |

  - Consider also SSB and CORESET#0 multiplexing pattern 3 for 120kHz SSB.

  - Pending on the UE minimum BW capability, consider also SSB and CORESET#0 multiplexing pattern 3 for 480kHz SSB.

- From [13] Samsung:

  - For CORESET#0 configuration with 120 kHz SCS,

    - additional CORESET#0 RB offsets are needed;

    - support 96 RB as the number of RBs for CORESET#0.

  - For CORESET#0 configuration with 480 kHz and 960 kHz SCS,

    - support multiplexing pattern 3;

    - support 96 RB as the number of RBs for CORESET#0;

    - further study the RB offset based on RAN4 design of channel and synchronization rasters.

  - Type0-PDCCH configuration, support all configurations from Rel-15 table except for the changes to O values:

    - For 120 kHz, O∈0, 5, 2.5, 5+2.5;

    - For 480 kHz, O∈0, 5, 2.5/2, 5+2.5/2;

    - For 960 kHz, O∈0, 5, 2.5/4, 5+2.5/4.

  - For 480 kHz and 960 kHz SCS, a UE only monitors one slot for Type0-PDCCH:

    - Alt 1: the one slot is slot n0 for all cases;

    - Alt 2: the one slot is slot n0 for M=1 and M=1/2, and configured from slot n0 and n0+1 for M=2.

- From [14] Mediatek:

  - Support adjusting the time-domain offset between SSB and CORESET #0 for 480/960 kHZ SCS.

- From [15] Intel:

  - Support 96 PRB CORESET for {SS/PBCH, PDCCH} equal to {120,120}, {480,480} and {960,960} kHz with NsymbCORESET= {1, 2}.

  - Support the following CORESET#0 RB offset values for {120, 120} kHz, {480, 480}, {960, 960} kHz for multiplexing patterns 1 and 3:

    - For CORESET#0 with 24 RBs: [0] for multiplexing pattern 1 and –20 if kssb =0 (-21 if kssb > 0) for multiplexing pattern 3.

    - For CORESET#0 with 48 RBs: [0], for multiplexing pattern 1 and –20 if kssb =0 (-21 if kssb > 0) for multiplexing pattern 3.

      - FFS: inclusion of RB offset of [1]

    - For CORESET#0 with 96 RBs: [0] for multiplexing pattern 1 and –20 if kssb =0 (-21 if kssb > 0) for multiplexing pattern 3.

    - Modify Table 13.8 in TS 38.213 to support the proposed RB offset when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

    - Support addition of a new table in 38.213 for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz or {960, 960} kHz.

  - For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz,

    - Support O = {0, 2.75, 5, 7.75} for 480kHz (in case Lmax = 128)

    - Support O = {0, 1.5, 5, 6.5} for 960kHz {in case Lmax = 128)

  - The equation for determining the slot number for PDCCH monitoring is modified to account for the non-contiguous numbering of the SSB slot pattern for {SSB, Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz.

    - Support the following modified equation:

    - For 480 kHz:  n0=O∙2μ+i∙M mod Nslotframe, μ where i=i+2∙i/6 and  0 ≤i ≤Lmax-1

    - For 960 kHz:  n0=O∙2μ+i∙M mod Nslotframe, μ where i=i+4∙i/12 and  0 ≤i ≤Lmax-1

- From [16] NTT Docomo:

  - If time allows, support the following for 480/960 kHz SCS, considering the support of two sets of SSB-CORESET#0 multiplexing within a slot:

    - More number of RBs for CORESET#0 PDCCH

    - Enhance Default PDSCH TDRA Table A

  - If time allows, support smaller ‘O’ value especially for 960 kHz SCS

- From [17] Pansonic:

  - 96 PRB CORESET for 120 kHz SCS is supported.

- From [21] Interdigital:

  - Introduce the enhancements on SS/PBCH block transmission patterns to deliberately include the CORESET#0 and SIB1 in fixed time locations along with the corresponding SS/PBCH block to ensure the channel occupancy as much as possible, in the initial access operations with 120kHz SCS for unlicensed spectrum in beyond 52.6GHz.

- From [22] LG Electronics:

  - Reuse Table 13-12 in TS 38.213 specification for type0-PDCCH CSS set configuration with 120/480/960 kHz, except for O values for 480/960 kHz.

- From [23] Sharp:

  - For Type0-PDCCH CSS set configuration rows where the first symbol index is given by {0, if i is even}, {NsymbCORESET, if i is odd}, the configuration rows should be modified such that gap symbols between different beams can be supported.

  - ‘O’ is from the set {0, 5, X5, 5+ X5} for 480 kHz, and {0, 5, X6, 5 + X6} for 960 kHz, where X5 and X6 stand for durations that count for consecutive transmission of SSB burst of 480kHz SCS and 960kHz SCS, respectively.

- From [24] Apple:

  - In addition to 24 and 48 PRBs, 96 PRBs can be considered for CORESET#0 BW with 120kHz SCS.

  - For Type0-PDCCH Mos determination, the offset values are defined as O=0,2.5Xu,5,5+2.5Xu for 480kHz and 960kHz SCS respectively, where Xu=2u-4.

- From [26] Qualcomm:

  - for FR2-2, CORESET0 SCS = SSB SCS for all SCSs

  - consider minimizing the overhead of beam switching gaps by supporting multiplexing pattern 3

  - for ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, the following set of parameters are supported for SS/PBCH block and CORESET multiplexing pattern 1:

| Number of search space sets per slot | M | First symbol index |
| --- | --- | --- |
| 1 | 1 | 0 |
| 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 2 | 1/2 | {0, if is even}, {+ 1, if is odd} |
| 1 | 2 | 0 |

    - Note: the number of entries corresponding the same {number of SS per slot, M, first symbol index} tuple (listed above) will depend on supported ‘O’ for each tuple

    - For 960 kHz, re-interpret the offsets of O = {0, 2.5, 5, 7.5} from Table 13-12 as O = {0, 1.25, 5, 6.25}

#### Summary of Discussions

In RAN1 #104e and #105e the following agreement was made.

Agreement: For CORESET#0 and Type0-PDCCH search space configured in MIB: - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz. - FFS: Supporting additional values - FFS: Supported values for SSB to CORESET#0 offset RBs - If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported, - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz - If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported, - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz - If 240 kHz SSB SCS is agreed to be supported, - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz - FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960) - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz) Agreement: For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - Support the following set of parameters. SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols. 1; 24; 2. 1; 48; 1. 1; 48; 2. - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design. - FFS: addition other set of parameters

The following are a summary of company views on CORESET#0 configuration aspects.

- For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz

  - controlResourceSetZero

    - Addition of 96 PRB CORESET#0 with {1,2} symbols

      - Support: Huawei/HiSilicon, vivo, Nokia/NSB, Samsung, Intel, Panasonic, Apple

      - Do not support:

    - Addition of mux pattern 3

      - Support: Huawei/HiSilicon (according to RAN1#104-e agreement), ZTE/Sanechips, vivo, [CATT], Nokia/NSB, Intel

      - Do not support:

    - RB offset values for Mux 1

      - 24 RB

        - 0, 4 : Huawei/HiSilicon

        - 0: Intel

      - 48 RB

        - 0, 28: Huawei/HiSilicon

        - 2, 14, 26: Ericsson

        - 0, 1: Intel

      - 96 RB

        - 0, 76: Huawei/HiSilicon

        - 0: Intel

  - searchSpaceZero

    - Use Table 13-12 (originally intended for {120,120} kHz)

      - Samsung, Intel, LGE

    - Use Table 13-12 (originally intended for {120,120} kHz) except O values (2.5 and 7.5)

      - Huawei/HiSilicon

- For {SSB, CORESET#0/Type0-PDCCH} = {480, 480} and {960, 960} kHz

  - controlResourceSetZero

    - Addition of 96 PRB CORESET#0

      - Support: vivo (for 480kHz), Intel

      - Do not support: vivo (for 960kHz)

    - Addition of mux pattern 3

      - Support: ZTE/Sanechips, [CATT], Nokia/NSB (for 480kHz), Samsung, Intel, Qualcomm

      - Do not support: Huawei/HiSilicon

    - RB offset values for Mux 1

      - 24 RB

        - 0, 4 : Huawei/HiSilicon, Ericsson (for 960kHz)

        - 0: Intel

      - 48 RB

        - 0, 14, 28: Huawei/HiSilicon

        - 2, 14, 26: Ericsson

        - 0: Intel

      - 96 RB

        - 0: Intel

    - RB offset values for Mux 3

      - -20/-21 depend on k_ssb

      - N, where N is number of RBs for CORESET

  - searchSpaceZero

    - Use Table 13-12 (originally intended for {120,120} kHz) except O values

      - Huawei/HiSilicon, Nokia/NSB, Intel, LGE

    - O values

      - {0, 5/X, 5, 5 + 5/X} with X = 2^(µ-3) for DBTW OFF, X = 2^(µ-4) for DBTW ON

        - Huawei/HiSilicon

      - {0, 5, 2.5/X, 5+2.5/X} for 480 kHz and {0, 5, 2.5/(2X), 5+2.5/(2X)} for 960 kHz

        - Futurewei

      - {0, 1.5, 5, 6.5}

        - Nokia/NSB

      - {0, 1.25, 5, 6.25} for 480 kHz and {0, 0.625, 5, 5.625} for 960 kHz

        - Samsung, Apple

      - {0, 2.75, 5, 7.75} for 480 kHz and {0, 1.5, 6, 6.5 } for 960 kHz

        - Intel

      - {0, 5, X, 5 +X} for 480kHz and {0, 5, Y, 5+Y} for 960kHz, X and Y are slot duration number that correspond to SSB burst

        - Sharp

      - {0, 2.5, 5, 7.5} for 480 kHz and {0, 1.25, 5, 6.25} for 960 kHz

        - Qualcomm

- Other proposals

  - Common CORESET and SS table for all SCS

    - Ericsson

  - For 480 kHz and 960 kHz, whether to monitor Type0-PDCCH in n0 only or in {n0, n0+1}

    - Samsung

  - Update PDCCH monitoring equation to account to non-contiguous numbering of SSB slots pattern for 480/960kHz

    - Intel

  - Enhancement of default PDSCH TDRA Table A

    - NTT Docomo, CATT

#### <Moderator’s Suggestion for Discussions>

Discussion and decisions are needed for the following issues:

- For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz

  - Whether or not to include 96 PRB CORESET

    - 9 company support, no objections so far

  - Whether or not to support mux pattern 3 – RAN1 seemed to have agreed to this in RAN1 #104-e

  - searchspaceZero - Use Table 13-12 as is or with modifications (e.g. O values, removal of entries, etc)

  - RB offset values for 24, 48, [96] PRB CORESET: FFS

- For {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and = {960, 960} kHz

  - Whether or not to include 96 PRB CORESET

  - Whether or not to support mux pattern 3

    - 7 companies support, 2 companies do not support

  - searchspaceZero - Use Table 13-12 with modification of O values

    - whether or not to use different O value depending on whether DBTW is ON/OFF

    - {0, 5, X, 5 +X} for 480kHz and {0, 5, Y, 5+Y} for 960kHz, values of X and Y FFS

  - RB offset values for 24, 48, [96] PRB CORESET: FFS

- Other proposals that require discussions

  - Common CORESET and SS table for all SCS

  - For 480 kHz and 960 kHz, whether to monitor Type0-PDCCH in n0 only or in {n0, n0+1}

  - Update PDCCH monitoring equation to account to non-contiguous numbering of SSB slots pattern for 480/960kHz

  - Enhancement of default PDSCH TDRA Table A

    - Discuss in Section 2.1.5

Moderator would like to encourage companies to initiate some discussion on RB offset values for CORESET. RAN1 has 1 more meeting left (in November) before completion of release 17 for RAN1. RAN4 does not have a meeting until November, and if RAN4 does not complete the raster design by November, then RAN1 may not be able complete the specification. Therefore, moderator suggests companies to investigate into RB offset values needed based on currently available raster proposals in RAN4. RAN1 can potentially make tentative proposals for few potential raster scenarios (being considered in RAN4). This way RAN1 at least has some idea on how many entries will be used for CORESET#0 and is able to pick out a final set as soon as RAN4 concludes on the raster design.

Issue #1)

##### Proposal 1.3-1

- For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, support multiplexing pattern 1 with 96 PRB CORESET#0, and {1, 2} symbol durations

Issue #2)

##### Proposal 1.3-2

- For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz,

  - use Table 13-8 in TS38.213 for multiplexing pattern 1,

  - (if supported) use Table 13-15 in TS38.213 for multiplexing pattern 3.

Issue #3)

##### Proposal 1.3-3

- For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, use the following table for multiplexing pattern 1:

  - FFS: whether or not to use different X value depending on whether DBTW is ON/OFF

  - FFS: whether or not to use same or different X value for 480 and 960 kHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 2 | 2.5 X | 1 | 1 | 0 |
| 3 | 2.5 X | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 6 | 0 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 7 | 2.5 X | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 8 | 5 | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 9 | 7.5 5 + X | 1 | 1 | 0 |
| 10 | 7.5 5 + X | 2 | 1/2 | {0, if is even}, {7, if is odd} |
| 11 | 7.5 5 + X | 2 | 1/2 | {0, if is even}, {, if is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

##### Proposal 1.3-4

- If supported, for ‘searchSpaceZero’ configuration for {480, 480} kHz and {960, 960} kHz, use the following table for multiplexing pattern 3:

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 31) |
| --- | --- | --- |
| 0 |  | 2, 9 in i=2k, i=2k+1 |
| 1 ~ 15 | Reserved |  |

Issue #4)

Discuss further on the following issue:

- For 480 kHz and 960 kHz, whether to monitor Type0-PDCCH in n0 only or in {n0, n0+1}

Issue #5)

Discuss further on the following issue:

- Update PDCCH monitoring equation to account to non-contiguous numbering of SSB slots pattern for 480/960kHz

### 2.14 ANR/CGI Reporting Aspects

- From [3] Spreadtrum:

  - The mechanism of two offsets in MIB defined for NR-U, i.e. Alt 2), can be reused for UE to determine CORESET#0/Type0-PDCCH.

- From [7] OPPO:

  - For ANR design, RAN1 considers one of the two options

    - Option 1: RAN1 holds ANR discussion until RAN4 concludes the channelization, LBT bandwidth and sync raster relationship.

    - Option 2: RAN1 does not follow R16 baseline solution and redesign ANR.

- From [9] CATT:

  - There is no need to study additional method(s) to enable support to obtain neighbor cell SIB1 contents related to CGI reporting in Rel-17.

- From [13] Samsung:

  - No need to support extra method for providing the CORESET#0/Type0-PDCCH configuration for ANR purpose.

- From [26] Qualcomm:

  - for ANR, do not consider additional methods (compared to current NR) to signal the NCGI

#### Summary of Discussions

3 Companies expressed that there is no need to consider additional methods to support neighbor cell SIB1 reading. 2 Companies mentioned the possibility of supporting offset based neighbor cell CORESET#0 determination, similar to what was defined for Rel-16 NR-U.

#### <Moderator’s Suggestion for Discussions>

From moderator’s understanding whether offset based neighbor cell determination is possible may depend on raster design in RAN4. Therefore, based on limited discussion from companies, and potential dependency on RAN4 decision, moderator suggest de-prioritize discussion in this meeting.

### 2.1.5 Various other aspects on SSB Design

- From [3] Spreadtrum:

  - SSB with 240kHz SCS can be down-prioritized.

  - Supporting initial cell selection with 480kHz SSB should be an optional UE capability separately from supporting other processing with 480/960kHz SCS.

  - The SSB-based TRS/CSI-RS validation can be supported.

- From [7] OPPO:

  - The raster step size for 120kHz and 480kHz are 3*17.28MHz and 15*17.28MHz, respectively, leading to a total number of raster entries 428.

- From [9] CATT:

  - Symbol #6 and symbol #13 can be reserved for beam switching. Neither PDCCH nor PDSCH can be transmitted on the reserved symbols.

  - The default TDRA table for pattern 1 in TS 38.214 can be enhanced, e,g  at least {S=6 ,L=7}, {S=2，L=11}  is supported.

- From [12] Nokia, NSB:

  - It is possible to apply SCSe to one part of actually transmitted SSBs and LBT procedure for other/rest of the SSBs.

  - Consider semi-static or predetermined mechanism to determine which SSBs are under SCSe and which under LBT in certain time windows.

- From [13] Samsung:

  - RAN1 clarifies that the configurable SCS for initial BWP configured by SIB1 can be 120 kHz, 480 kHz, and 960 kHz.

  - For 480 and 960 kHz, support the following 4 configurations for NR carrier RSSI measurement:

    - Configuration #0: {0, 1};

    - Configuration #1: {0, 1, …, 5};

    - Configuration #2: {0, 1, …, 8};

    - Configuration #3: {0, 1, …, 12}.

- From [20] Lenovo, Motorola Mobility:

  - For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, with higher subcarrier spacings (numerologies), coverage enhancement of channels and signals used for initial access should be considered for NR beyond 52.6 GHz

  - For initial access in NR unlicensed bands between 52.6 GHz and 71 GHz, with directional LBT based channel access mechanism, indication of sensing beams can be considered during the initial access

- From [25] Convida Wireless:

  - SSB coverage enhancement should be studied for higher SCS.

#### Summary of Discussions

- Companies have provided the following issues

  - Raster design

  - Capability aspect for initial access

  - Support of SSB based TRS/CSI-RS validiation

  - TDRA table update

  - Short control signal exemption applicability to signals

  - RSSI symbol update due to new SSB design

  - Coverage enhancement

  - Sensing beam indication

#### <Moderator’s Suggestion for Discussions>

For the following issues, moderator has provided comments on whether to further discuss during this meeting.

- Raster design

  - Should be discussed in RAN4

- Capability aspect for initial access

  - Should be discussed in 8.17.2

- Support of SSB based TRS/CSI-RS validiation

  - Moderator asks the proponent company to provide further information on what needs to be considered and specified in RAN1.

- TDRA table update

  - Moderator suggest to discuss further this meeting

- Short control signal exemption applicability to signals

  - Should be discussed in 8.2.6 channel access agenda

- RSSI symbol update due to new SSB design

  - Moderator suggest to discuss further this meeting

- Coverage enhancement

  - Moderator suggest to de-prioritize this discussion as coverage enhancement was explicitly de-scoped from the WID

- Sensing beam indication

  - Moderator thinks 8.2.6 channel access agenda might be a better suited agenda for discussion

Further discuss on the following proposals.

Issue #1) TDRA table update

Currently, Type0-PDCCH uses default TDRA A and C for CORESET multiplexing pattern 1 and 3, respectively. Please provide further comments on whether TDRA table should be updated and if so how it should be updated.

Issue #2) RSSI symbol update due to new SSB design for 480 and 960 kHz

##### Proposal 1.5-1

- For 480 and 960 kHz, support the following 4 configurations for NR carrier RSSI measurement:

  - Configuration #0: {0, 1};

  - Configuration #1: {0, 1, …, 5};

  - Configuration #2: {0, 1, …, 8};

  - Configuration #3: {0, 1, …, 12}.

## 2.2 PRACH Aspects

### 2.2.1 PRACH Sequence and Format

- From [1] Huawei/HiSilicon:

  - Additionally support L=571 for 480 kHz PRACH.

- From [2] Futurewei:

  - Do not support PRACH length L=571 for 480kHz PRACH.

- From [4] ZTE, Sanechips:

  - Support sequence length 571 for 480KHz PRACH SCS for 52.6 to 71 GHz.

- From [5] vivo:

  - Support 120KHz and 480KHz as candidate SCS of initial UL BWP.

  - Support 480KHz and 960KHz SCS in addition to 120KHz SCS for PRACH.

- From [11] Ericsson:

  - We are open to further discuss whether or not L = 571 is supported for 480 kHz.

- From [12] Nokia, NSB:

  - Support L=571 for PRACH with 480kHz.

- From [13] Mediatek:

  - Support only sequence length L=139 when PRACH SCS=480 kHz.

- From [15] Intel:

  - Support PRACH formats A1~A3, B1~B4, C0, C2 for LRA= 571 with SCS 480 kHz, i.e., μ=5.

- From [22] LG Electronics:

  - The 120 kHz PRACH SCS with sequence lengths L=571 and L=1151 are not required for the licensed spectrum where the regulatory requirements are not defined on PSD limit.

  - PRACH with sequence length L=571 can be supported for the 480 kHz SCS in addition to L=139 for initial/non-initial access and 960 kHz SCS PRACH with L=139 is only supported for non-initial access.

- From [23] Sharp:

  - Only support L = 139 for PRACH with 480kHz and 960 kHz SSB SCS.

- From [24] Apple:

  - Support PRACH length L=571 for 480 kHz PRACH.

#### Summary of Discussions

The following are previous agreements on PRACH sequence and formats.

Agreement: - For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2. - For non-initial access use cases, - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively. - FFS: support of sequence length L = 571, 1151 - FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access Agreement: Do not support PRACH length L=571, 1151 for 960kHz PRACH and at least L =1151 for 480kHz PRACH.

- Supported sequence lengths

  - PRACH length L=571 for 480kHz

    - Support: Huawei/HiSilicon, ZTE/Sanechips, Nokia/NSB, Intel, LGE, Apple,

    - Do not support: Futurewei, Sharp

- Supported subcarrier spacing for initial UL BWP

  - 120 kHz, 480 kHz: vivo

#### <Moderator’s Suggestion for Discussions>

Further discussion on following proposals.

##### Proposal 2.1-1

- Additionally support PRACH length L=571 for 480kHz

##### Proposal 2.1-2

- Support 120 kHz and 480 kHz subcarrier spacing for initial UL BWP

### 2.2.2 RACH Occasion Resources

- From [1] Huawei/HiSilicon:

  - For 480 kHz and 960 kHz PRACH, support one gap symbol between consecutive ROs in time domain at least for Formats A1, B1, and A1/B1.

  - For PRACH format A1, B1 and A1/B1, the first symbols for each RO in a reference slot can be derived using equation (2) if a gap symbol between consecutive ROs is introduced. The same triples of l0,NtRA,slot,NdurRA   in Table 6.3.3.2-4 of 38.211 are reused.

    - Note: Equation (2) guarantees that no RO straddles between slots nslotRA-1 and nslotRA.

- From [2] Futurewei:

  - If when the LBT is required prior to RACH transmissions there is no necessary to add extra gaps between successive RO in the same PRACH slot.

  - For 480kHz and 960 kHz SCS reuse Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum, where the slot index is scaled up by 4 and respectively by 8 as per prior agreement. For 120 kHz SCS use the Table 6.3.3.2-4 as is.

  - Update the table 8.1-2 to indicate the necessary Ngap for higher SCS.

- From [4] ZTE, Sanechips:

  - For 480kHz and 960kHz, it is unnecessary to introduce gap between ROs for LBT and/or beam switching.

  - For 480kHz and 960kHz, it is unnecessary to allow for additional nslotRA values if the maximum that can be configured for the number of FD ROs is less than 8.

- From [5] vivo:

  - The gaps between the consecutive ROs should be supported for LBT and/or beam switching.

  - The ROs for a given PRACH configuration spanned more than one PRACH slot should not be supported.

- From [6] Fujitsu:

  - Gaps between time-domain ROs in a slot are needed not only for LBT/beam switching, but also to avoid strong inter-RO interference due to power ramping up and rolling down.

  - For FR2-2, support gaps between time-domain ROs in a slot. The gaps can be enabled and configured by RRC signaling.

- From [9] CATT:

  - The reference slot duration corresponds to 60 kHz SCS. PRACH slot index nslotRA  corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

  - For 480/960 kHz PRACH slots configuration, higher PRACH slot density or higher RO density in time domain can be supported to compensate the impact from MSGS –FDM decreasing and LBT/beam switching GAP.

    - For 480KHz SCS,  PRACH slot density can be 2 or 4 times comparing to than 120KHz SCS

    - For 960KHz SCS,  PRACH slot density can be 4 times comparing to 120KHz SCS

  - If gap for LBT or beam switching is needed before UE transmit a msg-1, one RO can be disabled by RRC in a 60 KHz  reference  slot, and UE can perform  LBT or  beam switching on the disable RO.

- From [10] Xiaomi:

  - Inconsecutive RO time domain configuration should be supported at least for 480 and 960 kHz case.

- From [11] Ericsson:

  - Do not support additional nslotRA values if the maximum number of configured FD ROs is less than 8.

  - Do not specify gaps between consecutive PRACH occasions. If needed, gaps to account for gNB receive beam switching time can be created purely by gNB implementation based on the gNB's own knowledge of the switching time.

  - There is no need to further consider additional nslotRA values for the case when a PRACH slot cannot contain all time domain PRACH occasions

  - Support the following values of nslotRA

    - When number of PRACH slots in a reference slot is 1,

      - nslotRA=7 for 480kHz and nslotRA=15 for 960kHz PRACH

    - When the number of PRACH slots in a reference slot is 2,

      - nslotRA=3,7 for 480kHz and nslotRA=7,15 for 960kHz PRACH

- From [12] Nokia, NSB:

  - Do not introduce LBT gap between consecutive ROs.

  - Re-use the FR2-1 PRACH configuration for 120kHz sub-carrier spacing.

- From [13] Samsung:

  - Support non-consecutive RO configuration to alleviate the RACH LBT failure.

  - Postpone the decision of additional nslotRA values until the gap generation method has been determined.

- From [14] Mediatek:

  - RAN 1 to discuss the value of Ngap for NR operation to 52.6-71 GHz.

- From [15] Intel:

  - If gaps between consecutive ROs are necessary, gNB is able to configure PRACH with a large number of repetitions where some extra repetitions may be skipped and, thus, serve as gaps between ROs.

- From [16] NTT Docomo:

  - For RO configuration for PRACH with 480/960 kHz SCS, no need to consider either LBT or beam switching gap for RO design in 52.6 – 71 GHz

- From [21] Interdigital:

  - Do not support gap insertion between consecutive ROs in time domain as it causes inefficiency and application ambiguity.

  - Consider the enhancements to RO configuration without inserting gaps in between consecutive ROs.

  - For 52.6 – 71 GHz with 120kHz, 480kHz, and 960kHz PRACH, inserting gaps to achieve non-consecutive RACH occasions is not supported.

  - For 52.6 – 71 GHz, support sharing and extending the COT for LBT-free PRACH transmission in the consecutive ROs.

- From [22] LG Electronics:

  - When LBT is used to transmit the PRACH preamble, consider to insert CCA gap between adjacent RACH occasions in time domain (e.g. X usec or Y symbol) to avoid inter-UE LBT blocking due to the propagation delay of PRACH transmitted in an earlier RO.

  - The starting PRACH slot index for 480/960 kHz is given by:

    - when the number of PRACH slots in a reference slot is 1,

      - nslotRA=7-X for 480 kHz and nslotRA=15-X for 960 kHz PRACH

    - when the number of PRACH slots in a reference slot is 2,

      - nslotRA=3-X, 7-X for 480 kHz and nslotRA=7-X,15-X for 960 kHz PRACH

    - where X is the number of additional slots to provide a gap between all of consecutive RACH occasions corresponding to a PRACH configuration index in Table 6.3.3.2-4 of TS 38.211, based on the configured number of symbols for the gap required for LBT and/or beam switching.

      - Note: If a PRACH slot cannot contain all time domain PRACH occasions corresponding to a PRACH configuration index in Table 6.3.3.2-4 of TS 38.211 including gap(s) between consecutive PRACH occasions to account for LBT and/or beam switching, then X=0.

- From [23] Sharp:

  - Gaps between consecutive ROs should be supported at least for LBT purposes.

  - A starting symbol index of a PRACH occasion is given by l=l0+ntRANdurRA+NgapRA+14nslotRA. If non-zero duration gaps are configured between consecutive ROs and the ROs would span multiple PRACH slots, nslotRA=6 and 14 for 480 and 960 kHz SCS, respectively. Otherwise, nslotRA=7 and 15 for 480 and 960 kHz SCS, respectively.

- From [24] Apple:

  - Maximum 4 PRACH ROs can be configured for 120kHz SCS with L=571.

  - Maximum 2 PRACH ROs can be configured for 120kHz SCS with L=1151.

  - If a gap between consecutive PRACH occasions is not configured or not supported,

    - When number of PRACH slots in a reference slot is 1, nslotRA=7 for 480kHz and nslotRA=15 nslotRA=15 for 960kHz PRACH.

    - When number of PRACH slots in a reference slot is 2,nslotRA=3,7 for 480kHz and nslotRA=7,15 nslotRA=15 for 960kHz PRACH.

  - Pending confirmation from RAN4 on 59ns beam switching time, a SIB1-configurable gap between time-domain ROs cand be considered.

  - Keep the same nslotRAnslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8

- From [26] Qualcomm:

  - a maximum of 4 and 2 FD multiplexed ROs for SCS = 120 kHz and sequence length = 571 and 1151, respectively

  - for SCS = 120 kHz, if the maximum number of FD ROs are reduced, consider ways to increase the TD ROs (to maintain the same capacity) with minimal specification impact

  - for higher RACH SCS (480 and 960 kHz), consider including a gap between ROs which can be symbol-level (for gNB beam switching delay) or RO-level (for LBT)

  - for 480 kHz and 960 kHz PRACH:

    - ROs for a given PRACH configuration may need extra PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

      - Option A: TDM "RO + gap" until all required number of ROs are satisfied (even if they extend to an extra slot)

      - Option B: split the number of ROs as evenly as possible among multiple slots such that the pattern is the same for all slots (distribute the "RO + gap" among slots)

    - For the extra slots (if needed) consider the following 2 alternatives:

      - Alt1: the extra slots are added such that the distribution of the slots is even within the RACH reference slot

      - Alt2: the extra slots are added next to the original slots

#### Summary of Discussions

The following are previous agreements on PRACH sequence and formats.

Agreement: For 480 and 960kHz PRACH: - At least the same RO density in time domain (i.e. number of specified RO per reference slot according the PRACH configuration index) as for 120kHz PRACH in FR2 is supported - FFS: Support gap between consecutive ROs in time domain and the details to derive the gap Agreement: For 480 and 960kHz PRACH, - When a PRACH slot can contain all time domain PRACH occasions corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching, - and when number of PRACH slots in a reference slot is 1, - nslotRA=7 for 480kHz and nslotRA=15 for 960kHz PRACH - and when the number of PRACH slots in a reference slot is 2, - nslotRA=3,7 for 480kHz and nslotRA=7,15 for 960kHz PRACH - FFS: nslotRA values, when a PRACH slot cannot contain all time domain PRACH occasions, corresponding to a PRACH Config. Index in Table 6.3.3.2-4 of 38.211 including gap(s) between consecutive PRACH occasions (if supported) to account for LBT and/or beam switching. - FFS: whether to allow for additional nslotRA values if the maximum that can be configured for the number of FD RO’s is less than 8 (due to BW limitation)

The following is a summary of company views.

- Gap between consecutive ROs

  - Support: Huawei/HiSilicon, vivo, Fujitsu, [CATT], [Xiaomi], Samsung, LGE, Sharp, Qualcomm

  - Do not support: ZTE/Sanechips, [Futurewei], Ericsson, Intel, Nokia/NSB, NTT Docomo, Interdigital

- Do not support ROs that span more than one PRACH slot

  - vivo

- RO disabling RRC configuration to provide gap for LBT or beam switching

  - CATT

- Maximum 4 PRACH ROs can be configured for 120kHz SCS with L=571. Maximum 2 PRACH ROs can be configured for 120kHz SCS with L=1151.

  - Apple, Qualcomm

- Do not support additional nslotRA values if the maximum number of configured FD ROs is less than 8.

  - Ericsson, ZTE/Sanechips, Apple

- nslotRA values

  - If number of PRACH slots per reference slot is 1,

    - Confirm WA, nslotRA=7 for 480kHz and nslotRA=15 for 960kHz PRACH

      - Ericsson, Apple

    - nslotRA=7-X for 480 kHz and nslotRA=15-X for 960 kHz PRACH

      - LGE

    - If gap between RO is configured, nslotRA=6 for 480kHz and nslotRA=14 for 960kHz PRACH

      - Sharp

  - If number of PRACH slots per reference slot is 2,

    - Confirm WA, nslotRA=3,7 for 480kHz and nslotRA=7,15 for 960kHz PRACH

      - Ericsson, Apple

    - nslotRA=3-X, 7-X for 480 kHz and nslotRA=7-X,15-X for 960 kHz PRACH

      - LGE

#### <Moderator’s Suggestion for Discussions>

Whether or not gap is supported between consecutive RO is the most controversial and critical issue that seems to impact other discussion for RO design. Suggest discussing and conclude on this aspect first. Please further discuss on the following proposal.

##### Proposal 2.1-1 – suggested for GTW discussion

- Support gap between consecutive ROs for 480kHz and 960kHz

  - FFS: whether supporting gaps is fixed in specification or RRC configured by gNB

##### Proposal 2.1-2 – suggested for GTW discussion

- Do not support gap between consecutive ROs for 480kHz and 960kHz

### 2.2.3 RAR Window & RA Preamble ID

- From [1] Huawei/HiSilicon:

  - For the same RO density per reference slot as 120 KHz PRACH, the RA-RNTI corresponding to 480 kHz and 960 kHz ROs can be generated according to equation (5) by compressing the t_id to tid2μ-3.

    - When some ROs are backward shifted to the immediately preceding slot of the specified slot nslotRA due to the use of a gap symbol between consecutive ROs, support a 1 bit indication field in the DCI scheduling RAR/MsgB to resolve the PRACH slot ambiguity.

  - Support maximum of 40 ms for ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum. Support indicating two LSBs of SFN at which gNB has received msg1 (MsgA) in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI).

- From [2] Futurewei:

  - For 480kHz and 960kHz use the following formula for RA-RNTI

    - RA-RNTI = 1 + s_id + 14 × t_id + 14 × 160 × f_Id + 14 × 160 × 8 × ul_carrier_Id

    - and divide the RAR window in N segments where each segment is 160 slots, and signal the segment index in the DCI that schedules the MSG2/B.

- From [4] ZTE, Sanechips:

  - For higher PRACH SCS (480 and/or 960 kHz), consider the following options for further down-selection of RA-RNTI enhancements: option 2, 3, or 7

- From [5] vivo:

  - For larger PRACH SCS (480KHz/960KHz), the following options can be considered for RA-RNTI calculation:

    - Alt.1: Modify the RA-RNTI formula as following and introduce some contention resolution mechanism to resolve the conflict.

    - RA-RNTI = (1+s_id+14×t_id+14×X×f_id +14×X×8×ul_carrier_id) mod A

    - Alt.2: Reuse the current RA-RNTI formula while introducing additional indicator field to indicate the time-frequency resource together with RA-RNTI.

    - Alt.3: Depending on the RO configuration pattern, reuse/modify the RA-RNTI formula and express the slot indexes t_id based on a new specific subcarrier spacing.

- From [6] Fujitsu:

  - When calculating RA-RNTI for 480kHz and 960kHz PRACH, the following should be considered to uniquely identify a RO:

    - t_id is determined in a way that more than one slot can have the same t_id; and

    - DCI scheduling RAR indicates the local index among the slots having the same t_id.

- From [9] CATT:

  - For supporting Msg1 transmission with 480 KHz/960 KHz SCS, RA-RNTI is divided into two parts. One part of RA-RNTI is carried by DCI, and the remaining 16-bit of RA-RNTI could be used to scramble CRC of the DCI1. Two possible options are:

    - Option A:

      - RA-RNTI = (1 + s_id + 14 × t_id + 14 ×80*2u-3× f_id + 14 × 80*2u-3 × 8 × ul_carrier_id) mod 215

      - inDCI_bit = floor ((1 + s_id + 14 × t_id + 14 ×80*2u-3× f_id + 14 × 80*2u-3 × 8 × ul_carrier_id) /215)

      - s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

      - t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 640)

    - Option B:

      - RA-RNTI = 1 + s_id + 14 ×(t_id mod 80) + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

      - inDCI_bit = tid/80

      - s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

      - t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 640)

- From [10] Xiaomi:

  - Confirm the working assumption that for 120 kHz SSB, the number of candidates SSBs in a half frame is 64.

  - For 480/960 kHz SSB, candidates SSB index can be up to 128.

  - Whether DBTW or Q is needed can be decoded together with Q value.

- From [11] Ericsson:

  - For 480/960 kHz PRACH, reuse the RA-RNTI expressions from Rel-15/16, with the additional statement that for 480/960 kHz PRACH, t_id should be determined based on a subcarrier spacing of 120 kHz.

  - Postpone further discussions of RA-RNTI design until the PRACH configuration design is completed.

- From [12] Nokia, NSB:

  - Reuse RA-RNTI formula defined for 120 kHz SCS also for the cases PRACH is configured with 480 or 960 kHz SCS where

    - sid assumes 480/960 kHz SCS

    - tid assumes 120 kHz SCS

- From [15] Intel:

  - RA-RNTI computation equation should be adjusted to avoid overflow in case of PRACH SCS 480 kHz and 960 kHz;

    - Support the following modified equation for RA-RNTI computation:

      - RA-RNTI=1+s_id+14×t_id / max1, 2μ-3  +14×80×f_id+14×80×8×ul_carrier_id,

      - where t_id is based on the value of μ specified in clause 5.3.2 of TS 38.211.

- From [19] ETRI:

  - Propose to reuse the current equation with minor modifications for RA preamble ID calculation.

    - RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

      - t_id is the index of 120kHz slot that contains RO in a system frame

      - s_id is the index of the first OFDM symbol of RO based on the value of μ specified in clause 5.3.2 of TS 38.211

      - If additional PRACH slots are configured, the index(s) of the first OFDM symbol of ROs may be configure not to overlap each other between two PRACH slots within a 120kHz slot.

- From [22] LG Electronics:

  - Since the same RO density in time domain as for 120 kHz PRACH in FR2 is maintained regardless of whether there is a gap between ROs, RA-RNTI/MSGB-RNTI associated with the PRACH occasion for 480 and 960 kHz SCS using the existing RA-RNTI equation, the following options can be considered:

    - In the case of mapping RA-RNTI to hypothetical 480/960 kHz PRACH slot assuming that the gap between RACH occasions is zero,

      - Option 1: Reuse the existing RA-RNTI/MSGB-RNTI equation by reinterpreting the slot indexes t_id based on a new specific subcarrier spacing as the slot indexes of 120 kHz SCS (e.g., floor(t_id/n) where n=4 for 480 kHz SCS and n=8 for 960 kHz).

    - In the case of mapping RA-RNTI to actual 480/960 kHz PRACH slot,

      - Option 2: Divide the RAR window for RA-RNTI (or msg2 window for MSGB-RNTI) into N sub-periods (where each sub-period is 80 slots using the used SCS) + signal the sub-period index using the DCI that schedules the MSG2/MSGB.

- From [23] Sharp:

  - Assuming RO density per reference slot is unchanged, without modifying the formula and definition of s_id. Modify the definition of t_id as the slot index referring to 120kHz SCS.

- From [24] Apple:

  - modifying the existing calculation equation or redefine t_id based on 120kHz SCS to solve the RA-RNTI overflowing problem:

    - RA-RNTI=1+sid+14*tid+14*max80,Nslotframe,u*fid+14*max80,Nslotframe,u*8*ulcarrieridmod216

- From [26] Qualcomm:

  - for higher RACH SCS (480 and 960 kHz), consider the following options for the RA-RNTI:

    - Case 1: no extra RACH slots needed/configured

      - Same RA-RNTI equation as Rel-15/16

      - t_id is the index of the first slot (based on 120 kHz numerology) of the PRACH occasion in a system frame (0 ≤ t_id < 80)

    - Case 2: extra RACH slots needed/configured (but with the same number of ROs per reference slot)

      - Same RA-RNTI equation as Rel-15/16

      - s_id is the index of the first OFDM symbol of the PRACH occasion within the one or more slots spanned by the ROs excluding any gaps (0 ≤ s_id < 14)

      - t_id is the index of the first slot (based on 120 kHz numerology) of the PRACH occasion in a system frame (0 ≤ t_id < 80)

    - Case 3: extra RACH slots needed/configured (with more number of ROs per reference slot)

      - Option A: Extend s_id to more than 14:

        - RA-RNTI = (1 + s_id + S × t_id + S × 80 × f_id + S × 80 × 8 × ul_carrier_id) mod 216

        - s_id is the index of the first OFDM symbol of the PRACH occasion within the one or more slots spanned by the ROs excluding any gaps (0 ≤ s_id < S), S can take value > 14

        - t_id is the index of the first slot (based on 120 kHz numerology) of the PRACH occasion in a system frame (0 ≤ t_id < 80)

      - Option B:

        - Same RA-RNTI equation as Rel-15/16

        - t_id is the index of the first slot (based on 120 kHz numerology) of the PRACH occasion in a system frame (0 ≤ t_id < 80)

        - And signaling in the DL DCI that schedules the MSG2/MSGB the 480/960 kHz slot index within the 120 kHz slot

#### Summary of Discussions

The following list of options are from last meetings discussion.

- Plain Modulus Category - Option 1) - RA-RNTI=1+sid+14∙tid+14∙max80,Nslotframe,u∙fid+14∙max80,Nslotframe,u∙8∙ulcarrier-idmod216 - PRACH Sub-segmentation Method Category - Option 2) - RA-RNTI=1+sid+14∙tidmod80+14∙80∙fid+14∙80∙8∙ulcarrier-id - The same PRACH slot location in each 120kHz slot duration - Option 3) - Segment the PRACH into N segments - RA-RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier-id - tid is the index of the PRACH slot that contains the PRACH occasion in a segment. - In DCI: RA-indication = Segment index - Option 4) - Segment the PRACH into N segments - RA-RNTI=1+sid+14×tid+14×80*2u-3×fid+14×80*2u-3×8×ulcarrieridmod215 - In DCI: RA-indication=1+sid+14×tid+14×80*2u-3×fid+14×80*2u-3×8×ulcarrierid/215 - Option 5) - Segment the PRACH into N segments - RA-RNTI=1+sid+14∙tidmodM+14∙80∙fid+14∙M∙8∙ulcarrier-id - In DCI: RA-indication=tid/M - Option 6) - RA-RNTI=1+sid+14∙tidmod160+14∙160∙fid+14∙160∙8∙ulcarrier-id - In DCI: RA-indication=tid/160 - Compressing some indices Category (may require a matching RO configuration to work properly) - Option 7) - RA-RNTI=1+sid+14∙tid+14∙80∙fid+14∙80∙8∙ulcarrier-id - tid is the index of the first 120kHz slot that contains the PRACH occasion in a system frame. - sid is the index of the first OFDM symbol of the PRACH occasion based on the value of μ specified in clause 5.3.2 of TS 38.211. - Option 8) - RA-RNTI = 1 + s_id + 14 × floor(t_id / max1, 2μ-3) + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id, - t_id is based on the value of μ specified in clause 5.3.2 of TS 38.211.

The following is summary of company views.

- Alt 1) Plain Modulus Category, some example in option 1

  - vivo

- Alt 2) PRACH Sub-segmentation Method Category, some examples in option 2 ~ 6

  - Huawei/HiSilicon, Futurewei, ZTE/Sanechips, vivo, Fujitsu, CATT, LGE, Qualcomm

- Alt 3) Compressing some indices Category (may require a matching RO configuration to work properly), some examples in option 7 ~ 8

  - ZTE/Sanechips, Ericsson, Intel, vivo, Fujitsu, Nokia/NSB, ETRI, LGE, Sharp, Apple, Qualcomm

#### <Moderator’s Suggestion for Discussions>

RO design needs to be further progressed in order to assess which scheme is most suitable for fixing the RA-RNTI overflow issues. Suggest discussing this further once RO gap issue has been resolved and nslotRA are determined.

### 2.2.4 Other aspects on PRACH

- From [2] Futurewei:

  - Support short control signaling LBT exception for RACH transmissions.

#### Summary of Discussions

One company provided inputs on applicability of short control signal exemption for PRACH transmission.

#### <Moderator’s Suggestion for Discussions>

Moderator suggest discussing short control signal exemption aspects under 8.2.6 channel access agenda.

## 2.3 Others Aspects

- From [1] ZTE, Sanechips:

  - The existing parameter subCarrierSpacingCommon in MIB should be captured into Rel-17 RRC parameter table, as it will no longer be used to indicate the SCS of CORESET#0 in FR2-2.

#### Summary of Discussions

One company provided inputs on RRC parameters needed for initial access.

#### <Moderator’s Suggestion for Discussions>

Moderator suggest discussing the RRC parameters related issues under 8.2 RRC parameter discussion thread, “[106bis-e-R17-RRC-60GHz] Email discussion on Rel-17 RRC parameters for supporting NR from 52.6 GHz to 71 GHz – Jing (Qualcomm).”

# Reference

- R1-2108767, “Initial access signals and channels for 52-71GHz spectrum,” Huawei, HiSilicon

- R1-2108782, “Initial access for Beyond 52.6GHz,” FUTUREWEI

- R1-2108902, “Discussion on initial access aspects for NR for 60GHz,” Spreadtrum Communications

- R1-2108934, “Discussion on the initial access aspects for 52.6 to 71GHz,” ZTE, Sanechips

- R1-2108959, “Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz,” vivo

- R1-2109032, “Considerations on initial access for NR from 52.6GHz to 71 GHz,” Fujitsu

- R1-2109070, “Discusson on initial access aspects,” OPPO

- R1-2109120, “Discussion on initial access aspects supporting NR from 52.6 to 71 GHz,” NEC

- R1-2109208, “Initial access aspects for up to 71GHz operation,” CATT

- R1-2109401, “On initial access aspects for NR from 52.6-71 GHz,” Xiaomi

- R1-2109433, “Initial Access Aspects,” Ericsson

- R1-2109442, “Initial access aspects,” Nokia, Nokia Shanghai Bell

- R1-2109476, “Initial access aspects for NR from 52.6 GHz to 71 GHz,” Samsung

- R1-2109557, “Remaining issues on initial access of 52.6-71 GHz NR operation,” MediaTek Inc.

- R1-2109598, “Discussion on initial access aspects for extending NR up to 71 GHz,” Intel Corporation

- R1-2109665, “Initial access aspects for NR from 52.6 to 71 GHz,” NTT DOCOMO, INC.

- R1-2109741, “Initial access aspects for NR from 52.6 GHz to 71 GHz,” Panasonic Corporation

- R1-2109777, “Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz,” Sony

- R1-2109808, “Discussion on initial access aspects for NR from 52.6 to 71GHz,” ETRI

- R1-2109897, “Initial access aspects for NR from 52.6 GHz to 71GHz,” Lenovo, Motorola Mobility

- R1-2109903, “Discussion on initial access channels and signals for operation in 52.6-71GHz,” InterDigital, Inc.

- R1-2109961, “Initial access aspects to support NR above 52.6 GHz,” LG Electronics

- R1-2109992, “Initial access aspects,” Sharp

- R1-2110021, “Initial access signals and channels,” Apple

- R1-2110109, “NR SSB design consideration for 52.6 GHz to 71 GHz,” Convida Wireless

- R1-2110172, “Initial access aspects for NR in 52.6 to 71GHz band,” Qualcomm Incorporated

- R1-2110320, “Discussion on initial access aspects for NR beyond 52.6GHz,” WILUS Inc.