3GPP TSG RAN WG1#106bis-e	R1-211xxxx

e-Meeting, October 11th – 19th, 2021

Title:	[DRAFT] Reply LS on L2 buffer size reduction

Reply to:

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	Intel [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Debdeep Chatterjee

#### Email Address:	debdeep (dot) chatterjee (at) intel (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on L2 buffer size reduction for Rel-17 RedCap in R1-2108713 (R2-2109198). RAN1 has discussed the issue during RAN1 #106-e and RAN1 #106bis-e meetings, and the status of the RAN1 discussions is summarized below.

- RAN1 discussed various options for use of peak rate scaling factor as potential means of L2 buffer size reduction for Rel-17 RedCap but has not arrived at a consensus on whether and how to pursue L2 buffer size reduction as a cost/complexity reduction feature.

  - RAN1 does not intend to continue discussions on the issue unless further indication is received from RAN2.

- In addition to the options of maintaining Rel-15 specifications or defining that peak rate scaling factors as not applicable for Rel-17 RedCap UEs, RAN1 also discussed the following options towards optimizing peak rate scaling factor for RedCap for L2 buffer size reduction:

  - Relaxing the product of max number of layers, max modulation order, and scaling factor < 4, and/or

  - Reducing the scaling factor < 0.4

- While it was observed that Rel-15 specifications with the same scaling factors and constraints may still be available for RedCap UEs (in case of no spec changes), RAN1 could not converge on whether the cost/complexity benefits are sufficient to justify the above options for optimization of peak rate scaling factor for RedCap  changes in Rel-17.

- It was noted the proponent companies for optimizing peak rate scaling factor for RedCap towards L2 buffer size reduction could converge to a single proposal, i.e. relaxing the product to be smaller value (4->[1.5]) while keeping the existing scaling factor unchanged for Rel-17 RedCap.

- It was also noted by multiple companies in RAN1 that more effective UE cost/complexity reduction features with the same performance impact, but high cost/complexity reduction potential, were discussed and not pursued by RAN1 during the SI phase. Thus, such companies consider L2 buffer size reduction via peak rate scaling factor optimization as out-of-scope for the current WI.

2. Actions:

To RAN2 group

ACTION: RAN1 respectfully asks RAN2 to kindly take the above into account in future RAN2 discussions.

3. Date of Next TSG-RAN1 Meetings:

RAN1#107-e	November 11th – November 19th, 2021	E-meeting