3GPP TSG RAN WG1 Meeting #106bis-e	R1-21xxxxx

e-Meeting, October 11th – October 19th, 2021

Agenda Item:	8.9

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on 106bis-e-R17-RRC-NB-IoT-eMTC

Document for:	Discussion and Decision

# Introduction

In email discussion post RAN1#106-e, there were initial draft of RRC parameters for the WI [1].

This documents provides the proposals and summary of discussions of the corresponding email discussion on RRC parameters.

[106bis-e-R17-RRC-NB-IoT-eMTC] Email discussion on Rel-17 RRC parameters for Rel-17 NB-IoT and eMTC – Yubo (Huawei)

The RRC parameter list is located at:

# Discussion

## Support of 16-QAM for unicast in UL and DL for NB-IoT

### Issue 1-1: Configuration of PUR

For the configuration of NPUSCH for PUR, there are two entries in brackets as following in the RRC parameter list.

| [NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.211, 36.213 |  |  |  | enable16QAM-ul in PUR-config-NB | New | enable 16QAM for NPUSCH in PUR procedure | Enable/disable the use of 16QAM for NPUSCH in PUR procedure | Enable | disable | Per UE | UE specific | 36.331] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [NB_IOTenh4_LTE_eMTC6 | 16QAM for NB-IoT | 36.211, 36.213 |  |  |  | npusch-MCS | FFS New or existing | the TBS index for NPUSCH for PUR | the TBS index for NPUSCH for PUR when 16QAM is configured | 14,15,…,21 |  | Per UE | UE specific | 36.331] |  |

The following are proposed regarding this issue:

| Sourcing | proposals |
| --- | --- |
| [4] | Proposal 9: For supporting 16-QAM in PUR procedure, - One IE is introduced in pur-PhysicalConfig to enable the use of 16-QAM in NPUSCH - The field multiTone in npusch-MCS in PUR NPUSCH configuration is modified to include MCS 0-21. - One IE is introduced in pur-PhysicalConfig to enable the use of 16-QAM in NPDSCH - Power ratios of NRS and NPDSCH are given in pur-PhysicalConfig |
| [8] | - Proposal 7 To support 16-QAM for NPDSCH and NPUSCH in PUR procedure, - 16-QAM can be enabled/disabled by UE specific RRC signaling in PUR-Config-NB for NPDSCH and NPUSCH separately. - When 16-QAM is enabled for NPUSCH, the MCS indices, RU indices and UL power control parameter are indicated in PUR-Config-NB. - Note1: It’s up to RAN2 whether a new parameter or the legacy parameter is used to indicate the RU indices. - Note 2: There may be additional parameters if agreed. - When 16-QAM is enabled for NPDSCH, the DL power allocation is indicated in PUR-Config-NB. |

For the enabler of 16-QAM for NPUSCH in PUR procedure, based on the inputs, the following is proposed:

Proposal 1: 16-QAM is enabled/disabled for NPUSCH in PUR procedure by a UE specific RRC signaling

- Option 1: in PUR-config-NB

- Option 2: in pur-PhysicalConfig

- Option 3: up to RAN2

For the indication of MCS indices, the following is proposed:

Proposal 2: If 16-QAM is enabled, the MCS indices of PUR NPUSCH is signaled by:

- Option 1: modification of field multitone to include MCS 0~21

- Option 2: a new field to signal the MCS 14~21

- Option 3: up to RAN2

The details of RRC parameters for NPDSCH in PUR procedure can be discussed if it’s agreed.

Please input your comments for the above proposal:

| Companies | Comments |
| --- | --- |
| Ericsson | For both Proposal 1 and Proposal 2, we think that Option-3 (i.e., “up to RAN2”) should be selected, especially because in our undestanding there is already a running CRs on TS 36.331 touching upon the PUR configuration |

### Issue 1-2: Configuration for downlink power allocation

This will be discussed once it’s agreed.

### Issue 1-3: Configuration for uplink power control

This will be discussed once it’s agreed.

### Issue 1-4: Others

Please input your comments on issues other than the above ones:

| Companies | Comments |
| --- | --- |

## Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

Please input your comments in below table:

| Companies | Comments |
| --- | --- |

## Support a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

The following are proposed:

| Sourcing | proposals |
| --- | --- |
| [15] | Proposal 1: Parameters on configuration of the maximum DL TBS for multicast and PUR in eMTC are not needed. |

Please input your comments in below table:

| Companies | Comments |
| --- | --- |
| Ericsson | If multicast were supported, there will be an impact in the configuration of SC-MTCH provided via SC-MCCH. Thus, supporting multicast won’t be transparent, and as we expressed in the previous e-meeting, multicast used along with the new DL TBS of 1736 bits does not seem to be a relevant scenario that can provide significant gains as to justify the specification impacts (e.g., there might be very few UEs that implement multicast + larger TBS). Thus, we are only OK with supporting the larger TBS for connected mode features + PUR. |

# Summary

# References

- R1-2108684, [Post-106-e-Rel17-RRC-09] Summary of email discussion on RRC parameters for Enhancements for NB-IoT and LTE-MTC, Moderator (Huawei), RAN1#106-e, Aug., 16th-27th, 2021.

- R1-2108777	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

- R1-2109174	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

- R1-2109314	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-2109320	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

- R1-2109337	Discussion on UL and DL 16QAM for NB-IoT	ZTE, Sanechips

- R1-2109559	Remaining Issues on supporting 16QAM in NB-IOT R17	MediaTek Inc.

- R1-2110316	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

- R1-2108778	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

- R1-2109175	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

- R1-2109315	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

- R1-2109338	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE, Sanechips

- R1-2110317	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

- R1-2110318	On the support of 16-QAM for unicast in UL and DL for TDD NB-IoT	Ericsson

- R1-2110372	Discussion on RRC parameters for max DL TBS of 1736 bits	Huawei, HiSilicon