3GPP TSG RAN WG1 #107-e                                                                                                               R1- 2111203

Meeting, 11th Nov – 19th Nov., 2021

Source:               TCL Communication

Title:                    Discussion on PUSCH repetition type A enhancement

Agenda item:      8.8.1.1

Document for:    Discussion and Decision

- Introduction

In RAN#106bis-e, enhancements on PUSCH repetition type A for coverage enhancement were discussed and the following agreements were made [1].

Agreement Following working assumption is confirmed The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32. Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots. - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s) Agreement For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots. Conclusion: For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots. Observation - Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions. Agreement - For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2. - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2. - Note: The available slot determination is to be specified. - For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig. - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig. - Note: The available slot determination is to be specified. Agreement - Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots. - Any other RRC configuration is not considered for the determination of available slots. Agreement - The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots. The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.

In this contribution, we further analyze the potential enhancements and provide our views on type A PUSCH repetition enhancement.

2. Discussion

2.1 The use case of maximum number of repetitions

It’s was agreed that Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists for DCI foramt 0-1 and DCI foramt 0-2, while whether the increase of maximum number of repetitions also applies for the repetition factor configured in PUSCH-config and/or ConfiguredGrantConfig has not been determined.

2.1.1 Maximum number of repetition for repK/ pusch-AggregationFactor

In Rel-16, the maximum number of repetition was extended from 8 to 16, but the RRC parameter pusch-AggregationFactor configures the number of repetitions for PUSCH, where the candidate value set of pusch-AggregationFactor = {2, 4, 8}. In other words, the maximum repetition factor configured in PUSCH-Config was kept to be 8 as in Rel-15. Similar way should be reused in Rel-17. The maximum number of pusch-AggregationFactor should not be increased. In addition, the considerations should also be apply to repK.

Proposal 1: The maximum number of repetition factors of pusch-AggregationFactor in PUSCH-config should not be increased.

Proposal 2: The maximum number of repetition factor of repK in ConfiguredGrantConfig should not be increased.

2.1.2 Maximum number of repetition for DCI format 0_0

In Rel-15/16, the TDRA table used for DCI format 0_0 is shown in table 1, the Number of repetitions configured in TDRA table was not supported for DCI format 0_0. In addition, the objective of Rel-17 WI for PUSCH repetition Type A enhancements is to simply increase the maximum number of repetitions without more spec impact. Thus, similar mechanism in Rel-16 should be reused.

Table 1.  Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallbackRAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SP-CSI-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Proposal 3: For Rel-17 PUSCH type A enhancement, does not support the increased maximum number of repetitions in TDRA list for DCI format 0_0.

2.1.3 Maximum number of repetition for type 1 CG

In Rel-16, maximum number of repetition 16 is supported, as highlighted yellow below. A straightforward way is reused the same mechanism for Rel-17 CG type 1, the TDRA list for DCI format 0_1 and TDRA list for DCI format  0_2 can be reused.

When PUSCH resource allocation is semi-statically configured by higher layer parameter configuredGrantConfig in BWP-UplinkDedicated information element, and the PUSCH transmission corresponding to a configured grant, the following higher layer parameters are applied in the transmission: - For Type 1 PUSCH transmissions with a configured grant, the following parameters are given in configuredGrantConfig unless mentioned otherwise: - For the determination of the PUSCH repetition type, if the higher layer parameter pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured and set to 'pusch-RepTypeB', PUSCH repetition type B is applied; otherwise, PUSCH repetition type A is applied; - For PUSCH repetition type A, the selection of the time domain resource allocation table follows the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1. - For PUSCH repetition type B, the selection of the time domain resource allocation table is as follows: - If pusch-RepTypeIndicatorDCI-0-1 in pusch-Config is configured and set to 'pusch-RepTypeB', pusch-TimeDomainResourceAllocationListDCI-0-1 in pusch-Config is used; - Otherwise, pusch-TimeDomainResourceAllocationListDCI-0-2 in pusch-Config is used. - It is not expected that pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured with 'pusch-RepTypeB' when none of pusch-RepTypeIndicatorDCI-0-1 and pusch-RepTypeIndicatorDCI-0-2 in pusch-Config is set to 'pusch-RepTypeB'. - The higher layer parameter timeDomainAllocation value m provides a row index m+1 pointing to the determined time domain resource allocation table, where the start symbol and length are determined following the procedure defined in Clause 6.1.2.1; …… - For Type 2 PUSCH transmissions with a configured grant: the resource allocation follows the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI. - The PUSCH repetition type and the time domain resource allocation table are determined by the PUSCH repetition type and the time domain resource allocation table associated with the UL grant received on the DCI, respectively, as defined in Clause 6.1.2.1. For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK.

Proposal 4: Type 1 CG-PUSCH supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions and the TDRA list for DCI format 0_1 or 0_2 can be reused.

2.2 RRC configuration

In NR Rel-16, the number of repetitions is configured in the PUSCH time domain allocation list.

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

k2                                      INTEGER(0..32)                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PUSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16

PUSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k2-r16                                     INTEGER(0..32)          OPTIONAL,   -- Need S

puschAllocationList-r16                    SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16,

...

}

PUSCH-Allocation-r16 ::=  SEQUENCE {

mappingType-r16                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r16                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r16                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r16                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r16                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL,   -- Cond Format01-02

...

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

In Rel-17, In addition to {1, 2, 3, 4, 7, 8, 12, 16}, the additional value set {20, 24, 28, 32} for repetition factor is also supported. To minimize the specification impact, similar way in Rel-16 can be reused. For example, additional maximum repetitions can be added into the Rel-16 time domain allocation list. As highlighted yellow.

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)

omit

pusch-TimeDomainAllocationListDCI-0-2-r17               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r17 }

OPTIONAL,   -- Need M

pusch-TimeDomainAllocationListDCI-0-1-r17               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r17 }

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

-- ASN1START

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PUSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation

PUSCH-TimeDomainResourceAllocation ::=  SEQUENCE {

...

}

Omit

PUSCH-Allocation-r17 ::=  SEQUENCE {

mappingType-r17                           ENUMERATED {typeA, typeB}                     OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbolAndLength-r17                  INTEGER (0..127)                              OPTIONAL,   -- Cond NotFormat01-02-Or-TypeA

startSymbol-r17                           INTEGER (0..13)                               OPTIONAL,   -- Cond RepTypeB

length-r17                                INTEGER (1..14)                               OPTIONAL,   -- Cond RepTypeB

numberOfRepetitions-r17                   ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16,n20,n24,n28,n32} OPTIONAL,   -- Cond Format01-02

...

}

-- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

- Conclusion

In this contribution, the following proposals have been made:

Proposal 1: The maximum number of repetition factors of pusch-AggregationFactor in PUSCH-config should not be increased.

Proposal 2: The maximum number of repetition factor of repK in ConfiguredGrantConfig should not be increased.

Proposal 3: For Rel-17 PUSCH type A enhancement, does not support the increased maximum number of repetitions in TDRA list for DCI format 0_0.

Proposal 4: Type 1 CG-PUSCH supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions and the TDRA list for DCI format 0_1 or 0_2 can be reused.

References

[1] 3GPP RAN1, Chair’s Notes RAN1#106bis-e

[2] R1-2110596, “FL Summary #4 on Enhancements on PUSCH repetition type A”