3GPP TSG RAN WG1 Meeting #107bis-e                                 R1-2200013

e-Meeting, January 17th – 25th, 2022

Source:	New H3C

Title:	Discussion on propagation delay compensation enhancements

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

In the RAN1 #107-e meeting, enhancements for time synchronization in Rel.17 IIOT/URLLC were discussed. Some agreements were made as below [1].

Agreement

For Rel-17

- Support RTT-based PDC method

- Support PDC method based on legacy TA-based mechanism

  - No RAN1/RAN4 specification impact expected

Agreement

For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC.

Agreement

Send an LS to RAN2 and RAN4 with the content including:

- The agreements made in RAN1#107-e for propagation delay compensation.

- Ask RAN4 to define the following for RTT-based propagation delay compensation:

  - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking

  - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate)

  - gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate)

- Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1.

Conclusion

For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP.

Note: No RAN1 specification impact is expected for this conclusion

Agreement

For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signalling.

Conclusion

The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4.

In this contribution, we further provide our considerations on the evaluation on overall error of the time synchronization and the propagation delay compensation enhancements.

- Discussion

It was agreed in RAN1#107-e meeting that for RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC. In our view, if TRS and PRS are configured for PDC measurement simultaneously, SRS-Resource-ID should be included in the Rx-Tx measurement report provided from the gNB to the UE in order to pair the SRS and TRS/PRS for a gNB Rx-Tx time difference. It is also beneficial for mitigating the Rx/Tx timing errors.

Proposal 1: The Rx-Tx measurement report provided from the gNB includes SRS-Resource-ID.

In addition, if TRS and PRS are configured for PDC measurement simultaneously, dl-PRS-ID/csi-RS (The ID of a CSI-RS resource) should be included in the Rx-Tx measurement report provided from the UE to the gNB in order to pair the SRS and TRS/PRS for a gNB Rx-Tx time difference.

Proposal 2: The Rx-Tx measurement report provided from the UE includes dl-PRS-ID/csi-RS (The ID of a CSI-RS resource).

Based on the agreement in RAN1#107-e meeting, compensation, the Rx-Tx time difference is reported via RRC signalling which is defined in TS 38.331. So it is better to leave detailed signalling design to RAN2.

Proposal 3: The detailed RRC signalling design for Rx-Tx time difference measurement reporting for RTT-based PDC is up to RAN2.

- Conclusion

In this contribution, we discuss about the TSN supported by 5GS with the following observation and proposal.

Proposal 1: The Rx-Tx measurement report provided from the gNB includes SRS-Resource-ID.

Proposal 2: The Rx-Tx measurement report provided from the UE includes dl-PRS-ID/csi-RS (The ID of a CSI-RS resource).

Proposal 3: The detailed RRC signalling design for Rx-Tx time difference measurement reporting for RTT-based PDC is up to RAN2.

- Reference

- Chairman’s Notes, RAN1#107-e, final, November 11th – 19th, 2021