3GPP TSG RAN WG1 Meeting #107bis-e	    R1-2200035

e-Meeting, January 17-25, 2021

Agenda Item:	8.15.7

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for UE power saving enhancements

Document for:	Discussion and Decision

- Introduction

This contribution includes our views on UE features for Rel-17 UE power saving enhancement based on RAN1 agreements/conclusions to address FFS/controversial issues from email discussion for UE power saving features since RAN1#107-e meeting.

- Discussion

## UE feature for paging enhancement (UE feature 29-1)

In RAN1#107, the following UE feature 29-1 was agreed to be introduced. Regarding the details highlighted by yellow, it is still FFS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |

Regarding the type of the UE feature, we think it should be “per band” type, considering it could reduce the IODT work to deploy the feature. Even if the UE type is defined as “per UE”, it should be at least “per UE” with FR1/FR2 differentiation.

Proposal 1: Make the following update for UE feature 29-1:

- Update the UE feature 29-1 as ‘per band’ or “per UE” with FR1/FR2 differentiation.

## TRS resources for idle/inactive UEs (UE feature 29-2)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |

It was agreed in RAN1#107-e that the UE feature 29-2 is introduced with some FFS details of highlight, as shown above. We have the following proposals for the FFS part of the UE feature:

- The UE feature 29-2 should be ‘per band’, considering it could accelerate the deployment of the feature. Even if the UE feature is defined as “per UE” type, it should be at least with FR1/FR2 differentiation.

- Similarly as paging enhancement, we suggest to use “optional” in the table and leave RAN2 to decide ‘optionaa with capatility signalling’ or ‘optional without capability signalling’. The column of “Need for the gNB to know if the feature is supported” should be also updated accordingly.

- ‘Consequence if the feature is not supported by the UE’ can be updated to “UE does not support TRS occasions for idle/inactive UEs”.

Proposal 2: Make the following update on the FFS part of UE feature 29-2:

- Use “optional” in the table and leave RAN2 to decide “optional with capatility signalling” or “optional without capability signalling”.

- Update the feature type as “per band” or “per UE” with FR1/FR2 differentiation.

- Upate the content of column “Consequence if the feature is not supported by the UE” as “UE does not support TRS occasions for idle/inactive UEs”.

A suggested update is provided as an example.

## PDCCH monitoring adaptation within an active BWP (29-3)

The current UE features for PDCCH monitoring adaptation are shown below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

The following updates are suggested:

- There are proposals to pause the timer of SSSG(s) during an indicated skipping durations for case 4. This implies the UE behaviours of 29-3d in specification may be different from the simply combination of 29-3a and 29-3b. Therefore, a separate feature 19-3d should be supported.

- During the discussion, companies have commented that the state transitions among three SSSGs would be more complicated than the state transitions between two SSSGs. Concisdering additional complexity would be introduced on UE implementation, we propose to remove the square brackets of 29-3c and endorse the feature 29-3c.

- The corresponding columns of “Consequence if the feature is not supported by the UE” can be updated to the opposite of the name of the feature group, i.e. the feature is not supported.

- The UE features should be all “per band” features, considering it could accelaerate deployment of the feature on some bands. Even if the UE feature is defined as “per UE” type, it should be at least with FR1/FR2 differentiation.

Proposal 3: Make the following update on the FFS part of UE feature 29-3:

- Remove the square brackets of 29-3c and endorse the corresponding UE feature.

- Remove the square brackets of 29-3d and endorse the corresponding UE feature.

- The UE features of 29-3a, 29-3b, 29-3c and 29-3d are ‘per band’ UE feature or “per UE” with FR1/FR2 differentiation.

- Conclusion

In this contribution, we have following proposals on UE features for Rel-17 UE power saving enhancement:

Proposal 1: Make the following update for UE feature 29-1:

- Update the UE feature 29-1 as ‘per band’ or “per UE” with FR1/FR2 differentiationy

Proposal 2: Make the following update on the FFS part of UE feature 29-2:

- Use “optional” in the table and leave RAN2 to decide “optional with capatility signalling” or “optional without capability signalling”.

- Update the feature type as “per band” or “per UE” with FR1/FR2 differentiation.

- Upate the content of column “Consequence if the feature is not supported by the UE” as “UE does not support TRS occasions for idle/inactive UEs”.

Proposal 3: Make the following update on the FFS part of UE feature 29-3:

- Remove the square brackets of 29-3c and endorse the corresponding UE feature.

- Remove the square brackets of 29-3d and endorse the corresponding UE feature.

- The UE features of 29-3a, 29-3b, 29-3c and 29-3d are ‘per band’ UE feature or “per UE” with FR1/FR2 differentiation.

- References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO), RAN1#106-bis

- R1-2112139, Summary on UE features for UE power saving enhancements, Moderator (NTT DOCOMO, INC.)

- Chair’s note of RAN1#106b-e, 11-19, Oct. 2021.