3GPP TSG RAN WG1 Meeting #107bis-e	                                       R1-2200040

E-meeting, January 17th – 25th, 2022

Agenda Item:	8.15.3

Source:	Huawei, HiSilicon

Title:		     Rel-17 UE features for enhanced IIoT/URLLC

Document for:	Discussion and Decision

Introduction

After the RAN1#106-e meeting, the preliminary RAN1 UE features list for Rel-17 [1] was provided, and further updated in RAN1#107-e [2]. In this paper, our views for Rel-17 UE features for enhanced IIoT/URLLC are given.

Discussions on Rel-17 UE features for enhanced IIoT/URLLC

UE feedback enhancements for HARQ-ACK

- FG 25-1

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- There is a typo “Idenfify” in the column of components.

- For the type, we are fine with per UE. Looking at the SPS HARQ-ACK deferral mechanism itself, the motivation to support finer granularity e.g. per band or FSPC, is not clear to us.

- FG 25-2/25-3/25-3a

| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Comments:

- FG 25-2: Fine with the current version, including the cells in yellow.

- FG 25-3b: Add one of {25-3, 25-3a} as the prerequisite group.

- FG 25-4/25-5/25-6/25-7

| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Comments:

- FG 25-6:

  - Change “Per UE” to “Per band” to align with the granularity of the prerequisite.

  - Add a component for the maximum number of configured CCs for CB generation and a component for the maximum number of configured HARQ processes for CB generation. According to the agreement below, smaller CB size is achieved by either limited to a subset of configured CC or a subset of configured HARQ processes, UE capability should be set on the maximum number of configured CC or the maximum number of configured HARQ processes.

Agreement One enhanced Type 3 HARQ-ACK codebook is RRC configured either as: - a subset of CC, i.e., all HARQ processes of the subset of CCs are part of the codebook, OR pdsch-HARQ-ACK-enhType3perCC; Configure the one enhanced Type 3 HARQ-ACK codebook using per CC configuration; (1..maxNrofServingCells) of Integer (0,1). - a subset of configured HARQ processes per CC, i.e., different subsets of HARQ processes can be configured for each CC. pdsch-HARQ-ACK-enhType3perHARQ; Configure the one enhanced Type 3 HARQ-ACK codebook using a per HARQ process and CC configuration; (1..maxNrofServingCells) of Bit String (Size (16)).

  - Delete 10-16 from the prerequisite feature group for FG 25-6. Enhanced type 3 HARQ-ACK codebook feedback is to achieve smaller size compared to Rel-16 type 3 HARQ-ACK codebook, thus the support of enhanced type 3 doesn't necessarily mean the support of Rel-16 type 3 HARQ-ACK codebook.

  - For enhanced type 3 HARQ-ACK codebook, different UE capabilities should be set for combination of enhanced type 3 HARQ-ACK codebook with slot-based PUCCH and sub-slot based PUCCH, similar as FG 25-8 for type 1 HARQ-ACK codebook. Therefore, FG 25-6 should be limited to slot-based PUCCH while add a new FG 25-6a for sub-slot PUCCH.

| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback for slot PUCCH | 1. Support feedback of enhanced type 3 HARQ-ACK codebook for slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of configured CCs Y for an enhanced type 3 codebook for slot PUCCH. 6. Supported maximum number of configured HARQ processes Z for an enhanced type 3 codebook for slot PUCCH. | 10-16 | Yes | N/A |  | Per band | N/A | N/A | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured For component 5, the UE indicates its capability in the number of configured CCs: {1,...,Y} For component 6, the UE indicates its capability in the number of configured HARQ processes: {1,...,Z} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25. NR_IIOT_URLLC_enh | 25-6a | Enhanced type 3 HARQ-ACK codebook feedback for sub-slot PUCCH | 1. Support feedback of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for sub-slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of configured CCs Y for an enhanced type 3 codebook for sub-slot PUCCH. 6. Supported maximum number of configured HARQ processes Z for an enhanced type 3 codebook for sub-slot PUCCH. | 25-6 11-3 | Yes | N/A |  | Per band | N/A | N/A | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,2,4,8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured For component 5, the UE indicates its capability in the number of CCs: {1,...,Y} For component 6, the UE indicates its capability in the number of configured HARQ processes: {1,...,Z} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 25-7: Fine with the current FG 25-7, including the cells in yellow.

- FG 25-8

| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-8: Change “Per UE” to “Per FS” to align with the prerequisite.

- FG 25-9/25-10

| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |

Comments:

- FG 25-9/25-10: For FFS whether to separate the capability for different numerologies, we think it should be independent UE capability for same numerology and different numerologies to leave more flexibility at the UE side.

CSI feedback enhancements

- FG 25-11

| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-11: Fine with the current FG 25-11, including cells in yellow.

Enhancements for unlicensed band URLLC/IIoT

- FG 25-12

| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-12: Add basic feature components agreed for a UE that can operate as an initiating device in the semi-static channel access mode as follows. It can be decided whether the condition on the gap duration in component 2 needs to be captured or not, i.e., “a gap greater than 16us”.

| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. Sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Intra-UE Multiplexing/Prioritization

- FG 25-14/25-15

| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Comments:

- FG 25-14: Fine with the current FG 25-14, including cells in yellow.

- FG 25-15:

  - The square bracket for FG 25-15 can be removed based on the agreement in RAN1#107-e.

  - Add a component for the maximum value of additional time d3 for LP CG-PUSCH and HP DG-PUSCH collision resolution. The detail for the candidate value for d3 still need to wait for the progress from intra-UE multiplexing.

| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | FFS: Candidate value set for component 2: d3 = {0, 1,…,2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 25-16

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-16:

  - For FFS whether to separate capability for different UCI type, we are open to discuss. However we may not need to split in such a detailed way for every different UCI type.

  - Change 11-3 to one of {11-4, 11-4a}, as 11-4/11-4a is the UE capability of supporting two HARQ-ACK codebook with different priorities.

  - Delete 12-1 from the prerequisite feature group for FG 25-16. 12-1 is to define prioritization of overlapping channel/signals with two priority levels in physical layer, while 25-17 here is to define multiplexing of overlapping channel/signals with two priority levels in physical layer. There is no need to couple these two capabilities. It also aligns with #capability 1 UE as agreed #107-e meeting. In addition, if 12-1 is removed, then we can add one component “Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format” to FG 25-16.

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 12. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 23. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 34. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 45. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 56. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 67. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 78. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3 one of {11-4, 11-4a}, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 25-18

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

- FG 25-18:

  - Remove [at least] as it is already concluded not to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17 during last meeting.

  - The terminology of “PUCCH/PUSCH” covers three cases: PUCCH + PUCCH, PUSCH + PUSCH, and PUSCH + PUSCH. And FG 25-18 only refers to the case of “PUCCH + PUSCH”. We propose to change the component as “Support simultaneous PUCCH and PUSCH transmissions on different cells for inter-band CA.” to eliminate confusion.

  - Change “Per UE” to “Per BC”, since the capability for UE would dependent on the CA band combination.

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH PUCCH and PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UEBC | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Propagation delay compensation enhancements

- FG 25-19/25-19a/25-20

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS FFS whether to merge FG 25-19 and FG 25-19a | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS FFS whether to merge FG 25-19 and FG 25-19a | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure |  | Yes | N/A |  | Per UE | no | no | N/A |  | Optional with capability signaling |

Comments:

- FG 25-19/25-19a:

  - For FFS whether to merge FG 25-19 and FG 15-19a, we think it should be kept as independent UE capabilities to leave more flexibility at the UE side.

  - Delete 25-19 from the prerequisite feature group for FG 25-19a. RTT-based Propagation delay compensation based on CSI-RS for tracking and DL PRS should be independent UE capabilities to leave more flexibility at the UE side.

- FG 25-10: Fine with the current including cells in yellow.

- References

R1-2108697, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1 #106-e

R1-2112902	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.), RAN1 #107-e