3GPP TSG-RAN WG1 Meeting #107bis-e		R1-2200073

E-Meeting, January 17th – 25th, 2022

Title:	[Draft] Reply LS on MBS broadcast reception on SCell and non-serving cell

Response to:	R2-2111625

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Na Li

E-mail Address:	lina5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for sending their agreements and questions in LS R2-2111625 on MBS broadcast reception on SCell and non-serving cell.

Please find RAN1 answers on the questions below.

From RAN2 point of view, the UE may receive MBS broadcast service from SCell in intra-PLMN case and if supported this may be a separate UE capability. Send an LS to RAN1 to ask to check the feasibility of MBS broadcast reception on SCell.

- Answer1: For broadcast reception, RAN1 has agreed that the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs and the SCS and CP are the same.

- To support MBS broadcast reception on SCell, UE needs to get broadcast related configuration about CFR, PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH/MTCH by SIBx. One potential way is to allow UE to monitor MIB, SIB 1, SIBx and MCCH on SCell, and another potential way is to get the configuration of SIBx and MCCH via dedicated RRC signaling. Considering the change notification of SIB on PCell is based on paging, it is not expected to support paging on SCell from RAN1 perspective.

From RAN2 point of view, the connected UE may if supported receive MBS broadcast service from non-serving cell in intra-PLMN case, under the condition this does not have any impact to operation on serving cell(s). This may be a separate UE capability. Send an LS to RAN1 to ask to check the feasibility.

Answer2: From RAN1 perspective, if it is a separate UE capability to support MBS broadcast service from non-serving cell, it can be up to UE implementation and has no specification impact.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #108-e	21 February – 03 March 2022	E-Meeting