3GPP TSG RAN WG1 #107bis-e                                                        R1-2200100

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Discussion on UE features for IIoT and URLLC

Agenda Item:	8.15.3

Document for:	Discussion and Decision

Introductions

In RAN1#107 e-meeting, UE features for IIoT and URLLC are discussed and some agreements are achieved[1]. In this contribution, we share our view on remaining issues of UE features for IIoT and URLLC.

URLLC/IIOT UE features

FG 25-1

- Prerequisite FG

In Rel-15, downlinkSPS is configured per UE corresponding to FG 5-18 since only one SPS configuration is supported per cell group. In Rel-16, sps-r16 is defined per band i.e., FG12-2 indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. The complexity for FG 25-1 applied for single SPS configuration, may not be the same for multiple SPS configurations, so we think prerequisite feature groups should include both FG 5-18 or FG12-2, but not be “5-18 and 12-2”.

- Type

The prerequisite feature groups of FG 25-1 are FG 5-18 or FG 12-2. Considering type of FG 12-2 is per Band, the type of FG 25-1 should be at least per band.

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2. Type of FG 25-1 should be at least per band.

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenftify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-185-18 or 12-2 | Yes | N/A |  | Per UEBand | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-3 & 25-3a

For FG 25-3a, the pre-requisite 30-5, i.e., dynamic slot-based repetition, corresponds coverage enhancement feature, while FG 25-3a targets meeting URLLC requirements. There is no strong correlation between these two features. A URLLC UE may only implement sub-slot based repetitions without supporting slot-based repetitions. Therefore, the pre-requisite 30-5 is not needed and should be removed from FG 25-3a.

Since prerequisite feature groups of FG 25-3 include both FG 11-3 and FG 4-23 and the type of FG 11-3 is Per FeatureSetUplink, to align with the granularity of the prerequisite, we think the type of FG 25-3 should be per FS.

Similarly, since prerequisite feature groups of FG 25-3a include FG 25-3 and the type of FG 25-3 should be Per FS as discussed above, the type of FG 25-3a should also be per FS.

Proposal 2: For FG 25-3 and 25-3a, the type should be Per FS. The pre-requisite 30-5 should be removed from FG 25-3a.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |

FG 25-4 & FG 25-5 & FG25-6

FG 10-16 is the prerequisite feature groups of FG 25-4/5/6, which type is Per band. To align with the granularity of the prerequisite, the type of FGs 25-4 to 25-6 should be Per band.

Proposal 3: The type of FGs 25-4 to 25-6 should be Per band.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1.Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2.Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UEBand | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

FG 25-7

For first bullet in components column of FG 25-7, we think ‘sub-slot’ should be added.

FG 25-7 is related to FG 11-1 and 11-4, FGs 11-1 and 11-4 can be added as prerequisite feature groups.

The type of prerequisite group FG 11-4 is Per FeatureSetUplink, to align with the granularity of the prerequisite, we think the type of FG 25-7 should be per FS.

Proposal 4: For FG 25-7 ‘sub-slot’ should be added in components column. FGs 11-1 and 11-4 can be added as prerequisite feature groups. The type of FG 25-7 is per FS.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 11-1,11-4 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-9 & FG 25-10

- Whether to separate the capability for different numerologies

PUCCH cell switching with same or different numerologies may require different UE capabilities. PUCCH cell switching with same numerology should be the baseline and PUCCH cell switching with different numerologies should be deemed as a more advanced UE feature. Supporting separate UE capability provide flexible UE implementation. We suggest split FG 25-9 and FG 5-10 into separate UE capabilities, i.e., the one for the capability for same numerology and the other for different numerologies between switchable carriers.

- Applicable cases

For FG 25-9, semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI. This should be captured in the corresponding components column.

For FG 25-10, according to the agreement, in addition to HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

- HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH

- triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI

For FG 25-10, these applicable cases should be explicitly described in the components.

- Type

PUCCH cell switching is only used for TDD scenario, the type should be per Feature Set per Band combination.

Proposal 5: For FG 25-9 and 25-10, support separate UE capability for different numerologies. The applicable cases should be explicitly described in the components. The type should be Per FSBC.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching with the same numerology | 1.Semi-static PUCCH cell switching with the same numerology using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-9a | Semi-static PUCCH cell switching with the different numerologies | 1.Semi-static PUCCH cell switching with the same or different numerologies using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication with the same numerology | PUCCH cell switching with the same numerology based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10a | PUCCH cell switching based on dynamic indication with the different numerologies | PUCCH cell switching with the same or different numerologies based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |

FG 25-16

For FG 25-16, the prerequisite feature groups are FG 11-3 which is per FeatureSetUplink. Therefore, the type of FG 25-16 should be Per FS.

Proposal 6: The type of FG 25-16 should be Per FS.

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-12

For FG 25-12, the following descriptions should also be captured in components column to ensure that a UE can operate as an initiating device in the semi-static channel access mode.

- 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO.

- Determination of COT initiator assumption based on rules for configured UL

- Validating COT initiator assumption indicated in UL scheduling DCI

Proposal 7: For FG 25-12, capture the following descriptions in components column.

| 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL. 4. Validating COT initiator assumption indicated in UL scheduling DCI. | 10-1a |
| --- | --- | --- | --- |

FG 25-18

In latest RAN1 meeting, no consensus is achieved to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17, [at least] can be deleted.

Proposal 8: For FG 25-18, [at least] can be deleted.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-19 & FG 25-19a

Positioning capability is an optional feature. URLLC UE supporting propagation delay compensation capability needn’t requires support of positioning capability. It is reasonable that UE without positioning capability supports PDC. In this case, FG 25-19 can be only supported by UE. It is unnecessary to merge FG 25-19 and FG 25-19a.

Proposal 9: It is unnecessary to merge FG 25-19 and FG 25-19a.

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |

Conclusion

In the contribution, we have some investigations on UE feature for URLLC/IIOT, and propose that,

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2. Type of FG 25-1 should be at least per band.

| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenftify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-185-18 or 12-2 | Yes | N/A |  | Per UEBand | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: For FG 25-3 and 25-3a, the type should be Per FS. The pre-requisite 30-5 should be removed from FG 25-3a.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |

Proposal 3: The type of FGs 25-4 to 25-6 should be Per band.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1.Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2.Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UEBand | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

Proposal 4: For FG 25-7 ‘sub-slot’ should be added in components column. FGs 11-1 and 11-4 can be added as prerequisite feature groups. The type of FG 25-7 is per FS.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 11-1,11-4 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: For FG 25-9 and 25-10, support separate UE capability for different numerologies. The applicable cases should be explicitly described in the components. The type should be Per FSBC.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching with the same numerology | 1.Semi-static PUCCH cell switching with the same numerology using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-9a | Semi-static PUCCH cell switching with the different numerologies | 1.Semi-static PUCCH cell switching with the same or different numerologies using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication with the same numerology | PUCCH cell switching with the same numerology based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10a | PUCCH cell switching based on dynamic indication with the different numerologies | PUCCH cell switching with the same or different numerologies based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UEFSBC | No | No | N/A |  | Optional with capability signaling |

Proposal 6: The type of FG 25-16 should be Per FS.

| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: For FG 25-12, capture the following descriptions in components column.

| 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL. 4. Validating COT initiator assumption indicated in UL scheduling DCI. | 10-1a |
| --- | --- | --- | --- |

Proposal 8: For FG 25-18, [at least] can be deleted.

| 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 9: It is unnecessary to merge FG 25-19 and FG 25-19a.

| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |

References

R1-2112902 Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.) RAN WG1 #107