3GPP TSG RAN WG1 Meeting #107bis-e					R1-2200102

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Discussion on UE features for UE power saving enhancements

Agenda Item:	8.15.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our proposals for Rel-17 power saving UE features. The discussion is based on the latest version of UE features summary in R1-2112900. [1].

Following agreements were made in this RAN1-107 meeting.

Agreement

- FG 29-1 is kept as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Agreement

- Component 2 in FG 29-2 for support receiving L1 indication for TRS availability is kept

Agreement

- FG 29-3 is split into multiple FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: 2 search space sets group switching

  - [FG 29-3c]: 3 search space sets group switching

  - [FG 29-3d]: 2 search space sets group switching with PDCCH skipping

  - FFS: A UE reporting 29-3a and 29-3b does not implicitly mean the UE supports FG 29-3d

  - FFS: FG for 3 search space sets group switching with PDCCH skipping

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of PDCCH skipping Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 2 search space sets grouping switching Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 3 search space sets grouping switching Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2 search space sets grouping switching with PDCCH skipping Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

- Discussion on Rel-17 UE Power saving features

29-1 Paging enhancement

Agreement

- FG 29-1 is kept as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

- Subgroup indication

Subgrouping indication is up to RAN2.

- Descriptions of the components

According to the RAN plenary decision [2], PDCCH based PEI is agreed. New DCI format and only Behv-A supported is agreed. Thus it should be captured in the component descriptions.

Proposal 1:

- Support UE subgroup indication is either separated from 29-1 or as a RAN2 feature to be discussed in RAN2.

- Update the descriptions of 29-1 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1-1 | Paging enhancement | 1. Support paging early indication 1. Support of configured window for detection of DCI format XXX with CRC scrambled with YYY for paging early indication 2. Support of Behv-A if UE does not detect PEI for all monitored PEI occasion(s) for the PO 2. Support UE subgroup indication |  |
| 29. NR_UE_pow_sav_enh | 29-1-2 | Paging enhancement | Support UE subgroup indication (it is up to RAN2 whether/how to separate the capability for UE subgroup indication) | 29-1-1 |

29-3 PDCCH monitoring adaptation within an active BWP

It is agreed in RAN1#106-bis that,

Agreement The bit mapping of DCI indication PDCCH monitoring adaptation is as follows, - For Case 1 (i.e., PDCCH skipping), the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=1 - ‘0’ is Beh 1 and ‘1’ is Beh 1A - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=2 or 3 - ‘00’ is Beh 1 - ‘01’ is Beh 1A with skipping duration 1 - ‘10’ is Beh 1A with skipping duration 2 - ‘11’ is Beh 1A with skipping duration 3 if M=3, reserved if M=2 - For Case 2 (i.e., 2 SSSG switching) , the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘0’ is Beh 2 and ‘1’ is Beh 2A - For Case 3 (i.e., 3 SSSG switching) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - [‘11’ is reserved] - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors, - FFS details bit mapping - FFS: For Case 5 (i.e., 3 SSSG switching and skipping) - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - ‘11’ is Beh 1A - FFS Timer behavior when Beh 1A is indicated - Note: The UE can be configured to be indicated by DCI a value of X (i.e., skipping duration) among M RRC configured values by scheduling DCIs indicating PDCCH schedules data - FFS whether to restrict Skipping duration to be shorter than SSSG initial timer value - FFS whether the configuration is same or different for DCI format x_1 and DCI format x_2

5 cases are described and 4 of them are agreed to be supported with different UE behaviours

- The motivation to support more than 3 SSSG has been discussed. Considering specification effort for supporting more than 2 SSSGs and different understanding of urgency, it would be natural to have a sperate capability indication for UE supporting more than 2 SSSGs. Hence support of case 3 should be separated.

- Considering the different mechanisms/usage to support PDCCH monitoring adaptation, it is natural to sperate 1/1A and 2/2A/2B. Hence support of case 1 and 2 should be separated.

- A combination of PDCCH skipping and SSSG switching would requires additional specification support and hence case 4 can be separated

Proposal 2:

- Update the descriptions of 29-3 as follows,

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 SSSG switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 SSSG switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 SSSG switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

- Conclusions

This contribution addresses our proposals for Rel-17 power saving UE features.,

Proposal 1:

- Support UE subgroup indication is either separated from 29-1 or as a RAN2 feature to be discussed in RAN2.

- Update the descriptions of 29-1 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1-1 | Paging enhancement | 1. Support paging early indication 1. Support of configured window for detection of DCI format XXX with CRC scrambled with YYY for paging early indication 2. Support of Behv-A if UE does not detect PEI for all monitored PEI occasion(s) for the PO 2. Support UE subgroup indication |  |
| 29. NR_UE_pow_sav_enh | 29-1-2 | Paging enhancement | Support UE subgroup indication (it is up to RAN2 whether/how to separate the capability for UE subgroup indication) | 29-1-1 |

Proposal 2:

- Update the descriptions of 29-3 as follows,

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 SSSG switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 SSSG switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 SSSG switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

# Reference

- R1-2112900	Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.)

- RP-212611,		Moderator’s summary for discussion [93e-18-PowSav-WI]: Final Round, 	Moderator (CMCC)