3GPP TSG RAN WG1 #107bis-e		R1-2200105

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Discussion on UE features for MBS

Agenda Item:	8.15.12

Document for:	Discussion and Decision

- Introduction

In last meeting, Rel-17 NR UE features list has been discussed and updated in [1]. In this contribution, our views on MBS part are further provided based on [1].

- Discussion

For broadcast, multiple G-RNTIs facilitate different services, and thus, support of  G-RNTIs should be included in broadcast feature 33-1.

Proposal 1 Include ‘support of  G-RNTIs’ in broadcast feature 33-1.

For multicast, the capability for group-common PDSCH reception doesn’t have to be coupled with HARQ-ACK feedback. In general, there exist three reactions when UE receives group-common PDSCH: no HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; and sends ACK/NACK based HARQ-ACK feedback. Thus, neither ACK/NACK nor NACK-only based HARQ -ACK feedback should be included in 33-2.

Proposal 2 Neither ACK/NACK nor NACK-only based HARQ -ACK feedback should be included in 33-2.

Furthermore, when UE doesn’t support sending HARQ-ACK feedback, there will be no PTP retransmission or PTM retransmission and thus, support of PTP retransmission and support of PTM retransmission for multicast should be separated from feature 33-2.

Proposal 3 Separate ‘support of PTP retransmission for multicast’ from 33-2.

Proposal 4 Separate ‘support of PTM retransmission for multicast’ from 33-2.

Also, the HARQ-ACK codebook should be separated from multiplexing of unicast PDSCH and group-common PDSCH, and similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities.

Proposal 5 Separate ‘support of FDM-ed HARQ-ACK codebook’ from 33-3-2.

Proposal 6 Separate ‘support of TDM-ed HARQ-ACK codebook’ from 33-3-3.

Furthermore, as there is no difference between FDMed and TDMed type-2 codebok, it is not necessary to differentiate them, and thus, option 1 in [1] should be supported.

Proposal 7 Regarding HARQ-ACK codebook

- Add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for Type-2 HARQ-ACK codebook for multicast

According the achieved agreements in appendix, capability for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast should be added and it could be combined with the capability for TDM-ed Type-1 HARQ-ACK codebook for multicast.

Proposal 8 Add capability for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast in the capability for TDM-ed Type-1 HARQ-ACK codebook for multicast

From the achieved agreements, support of intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and support of FDM between unicast PDSCH and group-common PDSCH should also apply to broadcast. Thus, 33-3-2 and 33-3-3 should take 33-1 or 33-2 as prerequisite feature groups.

Proposal 9 Take 33-1 or 33-2 as prerequisite feature groups for 33-3-2 and 33-3-3.

Similar as dynamic scheduled group-common PDSCH reception, capability of ACK/NACK and NACK-only based HARQ -ACK feedback should be separated from data reception in 33-5-1.

Proposal 10 Separate ACK/NACK and NACK-only based HARQ -ACK feedback from 33-5-1.

The corresponding track changes with respect to R1- 2112902 is shown below:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support slot-level repetition for MTCH - Support of G-RNTIs for broadcast. FFS details of N |  | Yes | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast |  | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | - Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling - Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast - FFS details. | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1, 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1 or 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | ACK/NACK based HARQ-ACK feedback for multicast | - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling. | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | TDM-ed Type-1 HARQ-ACK codebook for multicast | - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast | 33-2, 33-3-3, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | FDM-ed Type-1 HARQ-ACK codebook for multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2, 33-3-2, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-4 | Type 2 HARQ-ACK codebook for multicast | - Support Type-2 HARQ-ACK codebook for multicast. | 33-2, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-5 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling HARQ-ACK feedback | 33-2 | Yes | Optional with capability signalling |

- Conclusion

In this contribution, we provide our further considerations on MBS part in the following proposals:

Proposal 1 Include ‘support of  G-RNTIs’ in broadcast feature 33-1.

Proposal 2 Neither ACK/NACK nor NACK-only based HARQ -ACK feedback should be included in 33-2.

Proposal 3 Separate ‘support of PTP retransmission for multicast’ from 33-2.

Proposal 4 Separate ‘support of PTM retransmission for multicast’ from 33-2.

Proposal 5 Separate ‘support of FDM-ed HARQ-ACK codebook’ from 33-3-2.

Proposal 6 Separate ‘support of TDM-ed HARQ-ACK codebook’ from 33-3-3.

Proposal 7 Regarding HARQ-ACK codebook

- Add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for Type-2 HARQ-ACK codebook for multicast

Proposal 8 Add capability for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast in the capability for TDM-ed Type-1 HARQ-ACK codebook for multicast

Proposal 9 Take 33-1 or 33-2 as prerequisite feature groups for 33-3-2 and 33-3-3.

Proposal 10 Separate ACK/NACK and NACK-only based HARQ -ACK feedback from 33-5-1

- Appendix

Agreements: (RAN1#102) For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot. Agreements: (RAN1#103) Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. Agreements: (RAN1#103) Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. - Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot - Case 2: support TDM among multiple group-common PDSCHs in a slot - Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 5: support FDM among multiple group-common PDSCHs in a slot - FFS: maximum number of PDSCHs in a slot simultaneous received per UE Agreement: (RAN1#104bis) At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L Agreement: (RAN1#106bis) For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Agreement: (RAN1#107) Support enabling/disabling HARQ-ACK for NACK-only based feedback. - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI.

- References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.)