3GPP TSG RAN WG1 #107bis-e                                              R1-2200115
e-Meeting, January 17th – 25th, 2022

Source: 	ZTE Corporation

Title: 	Discussion on remaining issues for Type A PUSCH repetitions for Msg3

Agenda item:	8.8.3

Document for:	Discussion/Decision

Introduction

In RAN1#107-e, the critical issues for Msg3 repetition have been resolved according to the discussion summarized in [1]. In this contribution, we provide our views on the left-over issues for Msg3 PUSCH repetition, and some TPs are also provided for the editor CRs [2][3].

Indication of MCS index for Msg3 re-transmission with repetitions

- In RAN1#107-e, it was agreed to use MCS information filed for repetition indication of Msg3 repetition. For Msg3 initial transmission, the 2 MSB and 2 LSB bits of the MCS information field in RAR UL grant are used for indicating the repetition factor and the MCS index respectively, and both the candidate values for repetition factor and MCS index can be configured by SIB1. For Msg3 re-transmission, the repetition factor is indicated the same way as initial transmission, i.e., using 2 MSB bits of the MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI for selecting one value from the four candidate values configured by SIB1. However, how to indicate the MCS index for re-transmission is not concluded yet.

Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreement - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission

- For Msg3 re-transmission, the legacy MCS information field has a total of 5 bits, and the 3 LSB bits can be used for indicating MCS index in case the 2 MSB bits are used for repetition indication. To avoid any RRC impact, one straightforward way is to use the 3 LSB bits to select one MCS index from a fixed MCS set with index from 0~7. This would not impact the flexibility of MCS indication as the MCS index is typically not larger than 7 even considering relatively large Msg3 payload as discussed in [1].

Based on above, we have the following proposal. Correspondingly, the text proposal is also provided.

- Proposal 1: In case of repetition of Msg3 re-transmission, use the 3 LSB bits of the MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI for indicating one MCS index from a fixed MCS set with index from 0~7.

- Adopt Text Proposal #1 for TS38.214 h00.

Text Proposal #1 for TS38.214 h00

#### 6.1.4.1 Modulation order and target code rate determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a fallbackRAR UL grant or for a MsgA PUSCH transmission, or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI, and if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3 - if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel; - elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 5.1.3.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. else - if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 6.1.4.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. end <Unchanged parts are omitted>

How to interpret the information filed for indication of Msg3/CFRA repetition

Regarding how a UE is managed to know whether to use legacy interpretation or new interpretation on the bit field indicating the number of repetitions, the following two options were agreed in RAN1#106-e.

Agreement Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. - Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. - Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

In RAN1#107-e, it was agreed that if the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}, which means Option 1 is supported. Similarly, Option 1 can also be applied in case the four candidate repetition factors are configured to achieve a unified solution. This can also avoid any additional specification work for Option 2.

In addition, the following working assumption was reached for repetition of CFRA PUSCH. Similar approach of Option 1 can be applied for CFRA PUSCH. The only difference is a UE requests Msg3 repetition by separate PRACH preamble while requests repetition of CFRA PUSCH based on UE capability reporting (i.e., FG 30-6) based on the working assumption below.

Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

Based on above, we have the following proposal and corresponding TP.

Proposal 2: Regarding how a UE should interpret the selected information field for indication of the number of repetitions, no additional signaling is needed.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

- Note that, a UE is considered to request repetition of CFRA PUSCH if the UE reports UE capability FG 30-6.

- Adopt Text Proposal #2 for TS38.213 h00.

Text Proposal #2 for TS38.213 h00

- 8.3 PUSCH scheduled by RAR UL grant - <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321] for contention based random access procedure or if the UE reports capability of CFRA PUSCH repetition for contention-free random access procedure, The UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - <Unchanged parts are omitted>

Conclusion

According to the analysis given above, we have the following proposals and corresponding TPs:

- Proposal 1: In case of repetition of Msg3 re-transmission, use the 3 LSB bits of the MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI for indicating one MCS index from a fixed MCS set with index from 0~7.

- Adopt Text Proposal #1 for TS38.214 h00.

Text Proposal #1 for TS38.214 h00

#### 6.1.4.1 Modulation order and target code rate determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a fallbackRAR UL grant or for a MsgA PUSCH transmission, or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI, and if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3 - if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel; - elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 5.1.3.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. else - if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI have an in-order one-to-one mapping with MCS index 0~7 in Table 6.1.4.1-1 to determine the MCS index IMCS. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. end <Unchanged parts are omitted>

Proposal 2: Regarding how a UE should interpret the selected information field for indication of the number of repetitions, no additional signaling is needed.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

- Note that, a UE is considered to request repetition of CFRA PUSCH if the UE reports UE capability FG 30-6.

- Adopt Text Proposal #2 for TS38.213 h00.

Text Proposal #2 for TS38.213 h00

- 8.3 PUSCH scheduled by RAR UL grant - <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE transmits a PRACH using a resource associated with a corresponding Msg3 PUSCH transmission with repetitions [11, TS 38.321] for contention based random access procedure or if the UE reports capability of CFRA PUSCH repetition for contention-free random access procedure, Tthe UE repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - <Unchanged parts are omitted>

- Reference

3GPP RAN1#107-e, R1-2112836 Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3, Moderator (ZTE Corporation).

3GPP TS38.213 h00.

3GPP TS38.214 h00.