3GPP TSG RAN WG1 #107bis-e	R1-2200123

e-Meeting, January 17th – 25th, 2022

Title: 	Discussion on Rel-17 UE features for NR MBS

Source: 	ZTE

Agenda item:	8.15.12

Document for:	Discussion/Decision

Introduction

After RAN1#107-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for MBS.

Discussion

Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed.

For multicast reception, to avoid UE fragmentation and facilitate commercial deployment, it is fundamental to group some necessary UE features into basic feature group FG33-2. Otherwise, it may jeopardize the commercial deployment. From our perspective, at least the following can be grouped together into FG33-2.

1.	Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI.

2.	Support of CFR configuration for multicast.

3.	Support of CORESET and common search space configuration for multicast.

4.	Support of DCI format 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast.

5.	Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots.

6.	Support ACK/NACK based HARQ-ACK feedback

7.	Support PTM retransmission for multicast.

8.	Support PTP retransmission for multicast.

Proposal 1:

Define FG33-1 as basic feature group for broadcast

Define FG33-2 as basic feature group for multicast with at least the following components

1.	Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI.

2.	Support of CFR configuration for multicast.

3.	Support of CORESET and common search space configuration for multicast.

4.	Support of DCI format 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast.

5.	Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots.

6.	Support ACK/NACK based HARQ-ACK feedback

7.	Support PTM retransmission for multicast.

8.	Support PTP retransmission for multicast.

Another issue is how to capture agreements for FG33-3-3. From our perspective, UE can share the same maximum PDSCH receptions in a slot per CC between unicast and multicast. Thus, the FG33-3-3 can be updated as following.

Proposal 2: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. -  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. |
| --- | --- | --- |

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for MBS with the following proposal.

Proposal 1:

Define FG33-1 as basic feature group for broadcast

Define FG33-2 as basic feature group for multicast with at least the following components

1.	Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI.

2.	Support of CFR configuration for multicast.

3.	Support of CORESET and common search space configuration for multicast.

4.	Support of DCI format 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast.

5.	Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots.

6.	Support ACK/NACK based HARQ-ACK feedback

7.	Support PTM retransmission for multicast.

8.	Support PTP retransmission for multicast.

Proposal 2: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. |
| --- | --- | --- |

Reference

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, RAN1#107-e.