3GPP TSG RAN WG1 #107bis-e		R1-2200154

e-Meeting, January 17th – 25th, 2022

Source:	CATT

Title:	Discussion on Type A PUSCH repetitions for Msg3

Agenda Item:	8.8.3

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, type A PUSCH repetitions for Msg3 was extensively discussed. A set of agreements were achieved for Msg3 PUSCH enhancement [1].

Agreement - Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition. - Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed. - Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon. Conclusion - There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition. Agreement - RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots. Agreement For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules. - The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused. - The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused. - The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused. Agreement - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Agreement - Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. - Note: there is no specification impact. Working assumption - support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

In this contribution, we further discuss the remaining issue on the MCS indication for Msg3 PUSCH re-transmission.

Discussion

After a long discussion across four meetings, it is finally determined to use the MCS information field to indicate the repetition number for both initial transmission and re-transmission of Msg3 PUSCH when repetition is requested. Specifically, for initial Msg3 PUSCH scheduled by RAR grant, 2 MSBs of the MCS information field are used to indicate the repetition number of Msg3 PUSCH transmission from four candidate repetition factors. The MCS index is indicated through the remaining 2 bits of the MCS information field. Then, as to the Msg3 PUSCH re-transmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, 2 MSBs of the MCS information field are used to indicate the repetition number of Msg3 PUSCH transmission from four candidate repetition factors which are the same as initial transmission. However, how to indicate the MCS index of re-transmission is still inconclusive for the case when repetition is requested.

Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission

In current NR, for TBS and MCS indication of Msg3 re-transmission when repetition is not requested, the following two ways are supported:

- Explicit indication

For explicit indication, both the modulation order Qm and coding rate R are indicated, and the TBS is calculated through explicit MCS indication (and time-frequency resource). In the case of transform precoding is disabled, 0≤IMCS≤28 in Table 5.1.3.1-1 of TS 38.214 is used to indicate Qm and R [2]. When the transform precoding is enabled, 0≤IMCS≤27 in Table 6.1.4.1-1 of TS 38.214 is used to indicate Qm and R. In this case, the candidate MCS indices for re-transmission are the same with initial transmission. Note that, it is important for gNB to guarantee that the TBS of initial transmission and that of re-transmission are aligned.

- Implicit indication

For implicit indication, the TBS of the Msg3 re-transmission is assumed to be as the TBS of the initial transmission. In the case of transform precoding is disabled, 29≤IMCS≤31 in Table 5.1.3.1-1 of TS 38.214 is used to indicate the modulation order Qm [2]. Meanwhile, the coding rate can be calculated according to the Qm, TBS and the time-frequency resource. When the transform precoding is enabled, the only difference is that the Qm should be indicated by 28≤IMCS≤31 in Table 6.1.4.1-1 of TS 38.214, while other procedures are the same.

The two brief MCS index tables for Msg3 transmission when repetition is not requested are shown below. The lines in yellow are specially used for explicit indication. The lines in cyan are used for implicit indication.

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| … | ... | ... | … |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/ q | 0.2344 |
| 1 | q | 314/ q | 0.3066 |
| … | ... | ... | … |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Therefore, for TBS and MCS indication of Msg3 re-transmission when repetition is requested, the following two principles shall be followed:

- When explicit indication is applied, the 4 MCS indices for initial transmission shall be included in the MCS table for re-transmission, indicating the modulation order Qm and coding rate R.

- When implicit indication is applied, the TBS of the Msg3 re-transmission should be fixed to the TBS of the initial Msg3 transmission. The MCS field only indicates the modulation order Qm for Msg3 re-transmission. The coding rate R is automatically calculated based on the (same) TBS, Qm and the allocated time-frequency resource.

Concretely, since the 3 LSBs of MCS field are left in DCI format 0_0 with CRC scrambled by TC-RNTI, there are totally 8 states can be used to indicate the candidate MCS indices for Msg3 re-transmission. A most straightforward way is to use 4 of the states (e.g. 0~3) to indicate the 4 MCS indices for Msg3 initial transmission when repetition is requested, for supporting explicit indication.

Proposal 1: For MCS indication of Msg3 re-transmission when repetition is requested, 4 states of the 3 LSB bits of MCS field (e.g. 0~3) in DCI format 0_0 with CRC scrambled by TC-RNTI are used to indicate 4 same MCS indices as initial transmission.

- 4 states are corresponding to MCS 0~3 when mcs-Msg3Repetition is not configured.

- 4 states are corresponding to the 1st ~ 4th value of mcs-Msg3Repetition, when configured.

Meanwhile, the remaining 4 states (e.g. 4~7) can be used to indicate the last few lines in the existing MCS index table with reserved coding rate to indicate the MCS of Msg3 re-transmission when repetition is requested, for supporting implicit indication. Specifically, last 3 lines in Table 5.1.3.1-1 are used to indicate the MCS index for Msg3 re-transmission if transform precoding is disabled. And the last 4 lines in Table 6.1.4.1-1 are used to indicate the MCS index for Msg3 re-transmission if transform precoding is enabled [2]. The mapping relationship between codepoint of 3 LBSs of MCS field and MCS index is shown below.

Table 1: Proposed MCS table for Msg3 re-transmission when repetition is requested.

| Codepoint | IMCS |  |  |  |
| --- | --- | --- | --- | --- |
|  | Transform precoding is disabled |  | Transform precoding is enabled |  |
|  | mcs-Msg3Repetition is configured | mcs-Msg3Repetitions is not configured | mcs-Msg3Repetition is configured | mcs-Msg3Repetitions is not configured |
| 000 | First value of mcs-Msg3Repetition | 0 | First value of mcs-Msg3Repetition | 0 |
| 001 | Second value of mcs-Msg3Repetition | 1 | Second value of mcs-Msg3Repetition | 1 |
| 010 | Third value of mcs-Msg3Repetition | 2 | Third value of mcs-Msg3Repetition | 2 |
| 011 | Fourth value of mcs-Msg3Repetition | 3 | Fourth value of mcs-Msg3Repetition | 3 |
| 100 | 29 |  | 28 |  |
| 101 | 30 |  | 29 |  |
| 110 | 31 |  | 30 |  |
| 111 | reserved |  | 31 |  |

Hence, we have the following proposals.

Proposal 2: For MCS indication of Msg3 re-transmission when repetition is requested, X states of the 3 LSB bits of MCS field (e.g. 4~7 or 4~8) in DCI format 0_0 with CRC scrambled by TC-RNTI are used to indicate the last X lines in the existing MCS index table, in which the modulation order is indicated and the code rate is reserved.

- X=3 states are corresponding to the last 3 lines in Table 5.1.3.1-1 when transform precoding is disabled.

- X=4 states are corresponding to the last 4 lines in Table 6.1.4.1-1 when transform precoding is enabled.

Proposal 3: Adopt the above Table 1 as the MCS table for Msg3 re-transmission when repetition is requested.

Conclusion

This contribution discussed the mechanisms of enhancements for Msg3 PUSCH. We have the following proposals:

Proposal 1: For MCS indication of Msg3 re-transmission when repetition is requested, 4 states of the 3 LSB bits of MCS field (e.g. 0~3) in DCI format 0_0 with CRC scrambled by TC-RNTI are used to indicate 4 same MCS indices as initial transmission.

- 4 states are corresponding to MCS 0~3 when mcs-Msg3Repetition is not configured.

- 4 states are corresponding to the 1st ~ 4th value of mcs-Msg3Repetition, when configured.

Proposal 2: For MCS indication of Msg3 re-transmission when repetition is requested, X states of the 3 LSB bits of MCS field (e.g. 4~7 or 4~8) in DCI format 0_0 with CRC scrambled by TC-RNTI are used to indicate the last X lines in the existing MCS index table, in which the modulation order is indicated and the code rate is reserved.

- X=3 states are corresponding to the last 3 lines in Table 5.1.3.1-1 when transform precoding is disabled.

- X=4 states are corresponding to the last 4 lines in Table 6.1.4.1-1 when transform precoding is enabled.

Proposal 3: Adopt the above Table 1 as the MCS table for Msg3 re-transmission when repetition is requested.

References

- Chairman's Notes RAN1#107-e, 3GPP TSG RAN WG1 Meeting #107-e, e-Meeting, November 11th – 19th, 2021.

- TS 38.214, Physical layer procedures for data, v16.5.0.