3GPP TSG RAN WG1#107bis-e	        	R1-2200175

E-meeting, January 17th – 25h, 2022

Agenda Item:					    8.2.3

Source:	Sony

Title:	Remaining issues on enhancements for PUCCH formats 0/1/4

Document for:	Discussion and decision

# List of agreements

During RAN1#107-e meeting, the following agreements on enhancements for PUCCH formats 0/1/4 were made [1]- [2]:

Agreement

For DMRS of enhanced PF1 support a single sequence of length equal to the total number of mapped REs of the PUCCH resource is used. The spreading factor for DMRS of enhanced PF1 is defined in the same way as Rel-16.

- Note: this clarifies Alt-1 from the RAN1#104 agreement

Agreement

In the RAN1#106bis-e agreements on construction of PUCCH resource sets prior to dedicated PUCCH configuration, the following is supported for PUCCH resource set index 15 in Table 9.2.1-1:

- X=NRB

Agreement

- For DMRS of enhanced (multi-RB) PF4, Type-2 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is supported if dmrs-UplinkTransformPrecodingPUCCH is configured and pi2BPSK is configured for the PUCCH resource.

- FFS: For DMRS of enhanced (multi-RB) PF4, whether or not Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is supported if pi2BPSK is configured for the PUCCH resource.

  - If Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is not supported for DMRS of enhanced (multi-RB) PF4 in FR2-2 if pi2BPSK is configured for the PUCCH resource, it will be separately discussed whether to support FG 16-6b in FR2-2 in UE feature discussion.

- Update the prior agreement from RAN1#104bis-e as follows:

Agreement (RAN1#104bis-e):

  - For DMRS of enhanced PF4, if pi2BPSK is not configured for the PUCCH resource, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

In view of the above, only some FFS points remain to be discussed during RAN1#107bis-e, on which we provide our views below.

# Discussion

During RAN1#107-e discussions [2], companies could not agree whether Type-1 low PAPR sequences of length equal to the total number of mapped REs of the PUCCH resource is supported when pi2BPSK is configured for the PUCCH resource, for DMRS of enhanced (multi-RB) PF4.

Based on earlier agreements in RAN1#104bis-e and RAN1#107-e, as well as on the specification text in [3], Sec. 6.4.1.3.3.1, the allowed low PAPR sequences for DMRS of enhanced (multi-RB) PF4 depending on the dmrs-UplinkTransformPrecodingPUCCH, pi2BPSK flags may be summarized as follows:

Table 1. Allowed low PAPR sequences for DMRS of enhanced (multi-RB) PF4.

| dmrs-UplinkTransformPrecodingPUCCH | pi2BPSK of PUCCH resource | Low PAPR sequence |
| --- | --- | --- |
| False | False | Type-1 (as per RAN1#104bis-e agreement and [3]). |
| False | True | Type-1 or N/A? |
| True | False | Type-1 (as per RAN1#104bis-e agreement and [3]). |
| True | True | Type-2 (as per RAN1#107-e agreement). |

There is some uncertainty about what the correct behavior is for the case when dmrs-UplinkTransform PrecodingPUCCH = false and pi2BPSK = true. Our reading of [3], Sec. 6.4.1.3.3.1 is that whenever either of dmrs-UplinkTransformPrecodingPUCCH or pi2BPSK is not set, then DMRS of PUCCH shall be generated according to [3], Sec. 6.3.2.2.

In turn, [3], Sec. 6.3.2.2 mandates low PAPR Type-1 sequences as per clause 5.2.2.

No further constraints regulating the usage of π/2-BPSK modulation for DMRS of PUCCH (e.g., a clause forbidding the combination dmrs-UplinkTransformPrecodingPUCCH = false and pi2BPSK = true) were found in other 3GPP specifications, such as [4]. This being the state of things, a close interpretation of the current 3GPP specification requires that Type-1 low PAPR sequences are used whenever dmrs-UplinkTransform PrecodingPUCCH is set to false and pi2BPSK to true.

Observation 1: If dmrs-UplinkTransformPrecodingPUCCH = false and pi2BPSK = true, then the current 3GPP specification requires that Type-1 low PAPR sequences are used for DMRS of regular (single-RB) PF4.

To minimize the amount of standardization effort, and considering that the regular R17 time frame has already ended, we therefore make the following proposal:

Proposal 1: To minimize standardization effort, for DMRS of enhanced (multi-RB) PF4, use Type-1 low PAPR sequences of length equal to the total number of mapped REs of the PUCCH resource whenever dmrs-UplinkTransformPrecodingPUCCH is set to false and pi2BPSK is set to true.

Note that even though pi2BPSK is set to true for the PUCCH resource, π/2-BPSK modulation is actually not applied. In effect, this case is equivalent to the case when dmrs-UplinkTransformPrecodingPUCCH and pi2BPSK are both set to false. Therefore, no additional UE implementation complexity is incurred.

Observation 2: Since for DMRS of enhanced (multi-RB) PF4, the case dmrs-UplinkTransform PrecodingPUCCH = false, pi2BPSK = true is handled identically to the case dmrs-UplinkTransform PrecodingPUCCH = false, pi2BPSK = false, no additional UE implementation complexity is incurred by Proposal 1.

Additionally, we note that transform precoding is always used for UCI transmissions of PF4. Whether π/2-BPSK or QPSK modulation is used depends on the pi2BPSK flag. This is summarized in Table 2.

Table 2. Allowed modulations for UCI transmissions of regular (single-RB) and enhanced (multi-RB) PF4. Transform precoding is always used.

| dmrs-UplinkTransformPrecodingPUCCH | pi2BPSK of PUCCH resource | Low PAPR sequence |
| --- | --- | --- |
| N/A | False | QPSK |
| N/A | True | π/2 – BPSK |

According to previous R17 agreements, we have that nOfPRBs-r17 ∈ {1,2,3,4,5,6,8,9,10,12,15,16} for the PF4 PUCCH resource. For these numbers of PRBs, the corresponding values of the cubic metric (CM) [5], a proxy for the PAPR [6], are shown in Figure 1, for DMRS of  PF4, and Figure 2, for UCI of PF4, for the sequences proposed in Table 1 and Table 2, respectively.

Figure 1. Comparison of cubic metric (CM) of low PAPR sequences for DMRS of enhanced (multi-RB) PF4. Transform precoding is applied to Type-2 sequences, but not to Type-1 sequences.

Figure 2. Comparison of the cubic metric (CM) of UCI of enhanced (multi-RB) PF4. Transform precoding is applied to both QPSK and π/2-BPSK.

To obtain low PAPR transmissions, it is clear that PUCCH symbols carrying DMRS and UCI should both have similar values of PAPR. One way to achieve this is to make sure that within one slot, Type-1 DMRS symbols go hand in hand with QPSK UCI symbols. Likewise, Type-2 DMRS and π/2-BPSK symbols should be configured in the same slot. From  Table 1 and Table 2, it can be seen that the flag combination dmrs-UplinkTransformPrecodingPUCCH = false, pi2BPSK = true produces Type-1 DMRS and π/2-BPSK UCI waveforms, which have disparate CMs, and should therefore be avoided in practice. To minimize standardization impact, this can be left up to the network.

Observation 3: The flag combination dmrs-UplinkTransformPrecodingPUCCH = false, pi2BPSK = true produces DMRS and UCI waveforms with disparate values of the cubic metric (CM), and should therefore be avoided in practice.

Proposal 2: Avoiding the flag combination dmrs-UplinkTransformPrecodingPUCCH = false, pi2BPSK = true is up to the network.

# Conclusions

This document considered enhancements to Rel-17 for PUCCH formats 0/1/4. The following observations and proposals have been made:

Observation 1: If dmrs-UplinkTransformPrecodingPUCCH = false and pi2BPSK = true, then the current 3GPP specification requires that Type-1 low PAPR sequences are used for DMRS of regular (single-RB) PF4.

Observation 2: Since for DMRS of enhanced (multi-RB) PF4, the case dmrs-UplinkTransform PrecodingPUCCH = false, pi2BPSK = true is handled identically to the case dmrs-UplinkTransform PrecodingPUCCH = false, pi2BPSK = false, no additional UE implementation complexity is incurred by Proposal 1.

Observation 3: The flag combination dmrs-UplinkTransformPrecodingPUCCH = false, pi2BPSK = true produces DMRS and UCI waveforms with disparate values of the cubic metric (CM), and should therefore be avoided in practice.

Proposal 1: To minimize standardization effort, for DMRS of enhanced (multi-RB) PF4, use Type-1 low PAPR sequences of length equal to the total number of mapped REs of the PUCCH resource whenever dmrs-UplinkTransformPrecodingPUCCH is set to false and pi2BPSK is set to true.

Proposal 2: Avoiding the flag combination dmrs-UplinkTransformPrecodingPUCCH = false, pi2BPSK = true is up to the network.

# References

- “Draft minutes report RAN1#107-e_v010,” RAN1 chairman report.

- 3GPP R1-2111466, “FL summary for Enhancements for PUCCH formats 0/1/4.” Ericsson.

- 3GPP TS 38.211 v16.7.0, “NR; Physical channels and modulation.”

- 3GPP TS 38.331 v16.7.0 “NR; Radio Resource Control (RRC); Protocol specification.”

- K. Kim, J. No and D. Shin, "On the Properties of Cubic Metric for OFDM Signals," in IEEE Signal Processing Letters, vol. 23, no. 1, pp. 80-83, Jan. 2016.

- 3GPP R1-060023, “Cubic metric in 3GPP-LTE,” Jan. 2006.