3GPP TSG RAN WG1 #107bis-e	R1-2200217

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.15.2

Source:	Samsung

Title:	UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

This contribution includes comments for Rel-17 UE features for supporting NR from 52.6 GHz to 71 GHz, as drafted in R1-2112902 [1].

# Comments to FG 24-1 and 24-1a

RAN and RAN1 have agreed to support 120 kHz SCS as the default numerology for supporting NR from 52.6 GHz to 71 GHz, and support 480 kHz and 960 kHz as optional numerologies subject to UE’s capability. Based on this principle, basic support for NR from 52.6 GHz to 71 GHz with 120 kHz SCS should be a mandatory UE feature, with the type of the UE feature as per band.

Meanwhile, only essential operation with 120 kHz SCS should be considered as component(s) of the basic support, wherein essential operation can be understood as the minimum set of features supporting development scenarios in FR2-2. Considering there are agreed development scenarios with DL only or UL only on FR2-2, it’s beneficial to distinguish DL and UL as two separate basic FR2-2 feature groups, without using each other as a prerequisite feature group.

Proposal 1: For FG 24-1 and 24-1a, keep current formulation to separate DL and UL operations as two basic FGs.

# Comments to FG 24-1b and 24-1c

Wideband PRACH is motivated by operation with shared spectrum channel access, but in general applicable to both operations with and without shared spectrum channel access. It may not be needed as basic FG, since PRACH with sequence length 139 was supported as basic FG for all the SCSs.

Multi-RB PUCCH format is an enhancement considered for operation with shared spectrum channel access only, as identified in RP-213637, so it should not be included as the basic FG.

Proposal 2: FG 24-1b and 24-1c should not be included as basic UL FGs, and FG 24-1c should be for operation with shared spectrum channel access only.

# Comments to FG 24-8 ad 24-9

It was agreed to support 32 DL and UL HARQ processes, using same solution as in NTN, but for UE features, the FGs of supporting 32 DL and UL HARQ processes should be separate from the corresponding FGs for NTN, since the type of UE to support those FGs can be different. Also, when the agreement was made, the motivation is clearly for 480 and 960 kHz SCSs only, to address the enlarged number of transmissions due to shorter slot duration, so the discussion should not be repeated for 120 kHz in the UE feature discussion. Lastly, the supporting of FG 24-8 and 24-9 should be per FSPC.

Proposal 3: For FG 24-8 and FG 24-9:

- Keep the FGs separately from supporting 32 HARQ processes in NTN;

- Remove “FFS: 120 kHz” in the two FGs;

- “Type” of the FGs are per FSPC.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: For FG 24-1 and 24-1a, keep current formulation to separate DL and UL operations as two basic FGs.

Proposal 2: FG 24-1b and 24-1c should not be included as basic UL FGs, and FG 24-1c should be for operation with shared spectrum channel access only.

Proposal 3: For FG 24-8 and FG 24-9:

- Keep the FGs separately from supporting 32 HARQ processes in NTN;

- Remove “FFS: 120 kHz” in the two FGs;

- “Type” of the FGs are per FSPC.

# Reference

[1] R1-2112902, Summary of UE features for supporting NR from 52.6 GHz to 71 GHz, Moderators (AT&T), Nov, 2021.