3GPP TSG RAN WG1 #107bis-e	R1-2200218

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.3

Source: 	Samsung

Title: 				     UE features for enhanced IIoT and URLLC

Document for:	Discussion and Decision

Introduction

This contribution discussed UE features for Rel-17 IIoT/URLLC.

Discussion

In [1], following UE features are endorsed as the following table. In this contribution, it will discuss yellow highlight parts.

| Features | Index | Feature group | Components |  | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional | Samsung’s views |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK |  | 5-18 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 |  | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE |  | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK |  | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations |  | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - For prerequisite feature group, 4-1, 4-3, 4-4 and 4-5 should be considered, and those entries have FR1/2 differentiation. So, “Need of FR1/FR2 differentiation” needs to be changed as “Yes”. - Confirm all other yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value |  | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI |  | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling | - For prerequisite feature group, 25-4 and 25-5 are enough to support 25-6. - Confirm all other yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration |  | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling | - “Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier” - Separating feature for the case of different numerologies. - Adding “support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier” |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling | - Separating feature for the case of different numerologies. - Adding “support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier” |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Confirm all yellow highlight parts |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. |  | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling | - Confirm the yellow highlight part |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. |  | 10-1a, 25-12 | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH |  | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling | - Prefer to merge 25-14 and 25-15. |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell |  | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling | - Prefer to merge 25-14 and 25-15. |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type |  | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Remove FFS for different UCI type. |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling | - Based on related conclusions in RAN1#107-e, it should be changed as “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.” |
| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS FFS whether to merge FG 25-19 and FG 25-19a |  | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling | For PRS based propagation delay compensation, it requires PRS estimation, which is different from CSI-RS based estimation. Therefore, we prefer to keep them separated. Moreover, regarding on the Note for FG 25-19a, we don’t see the need. Even for PRS based propagation delay compensation, there is no need to refer to positioning capability, because for propagation delay compensation, it only requires DL timing estimation for PRS but no need to support the whole procedure and signaling for positioning. Moreover, it is not clear on whether SRS for positioning or SRS for propagation delay compensation (if the configuration of SRS for propagation delay compensation is agreed to be introduced). Therefore, we prefer to further discuss SRS related issues after we have more agreement or conclusion in 8.3.4. |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS FFS whether to merge FG 25-19 and FG 25-19a |  | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |  |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure |  |  | Yes | N/A |  | Per UE | no | no | N/A |  | Optional with capability signaling |  |

- Necessity of considering different TA(Timing Advance) impact for PUCCH cell switching

For PUCCH cell switching, it is likely that some UEs are configured to transmit two PUCCHs simultaneously if two cells are included in different TAG (Timing Advance Group). For example, as shown in Figure 1, there are two cells for supporting PUCCH cell switching. If a UE is scheduled to transmit first PUCCH at slot 0 in CC1 and then transmit second PUCCH at slot 1 in CC0, the UE needs to transmit both PUCCH simultaneously in the overlapped time when those cells are configured to different TAG having TA values. So, this issue would increase UE implementation’s complexity since it requires simultaneous PUCCH transmissions in a PUCCH group although this is not intended operation of PUCCH cell switching.

Figure 1. PUCCH cell switching in case of different TA

In order to reduce UE implementation’s complexity, we would like to propose the following new UE feature. With that, a UE can do overlapped PUCCH transmission as shown in Figure 1 if the UE reports the following UE feature. Otherwise, it should be avoided by gNB scheduling.

| 25. NR_IIOT_URLLC_enh | 25-x | PUCCH cell switching with different TAGs | - Support PUCCH cell switching between two cells in different TAG | 6-11 25-9 and/or 25-10 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion

This contribution considered the UE features list for Rel-17 URLLC/IIoT. Proposals are included in above table. Also, new UE feature is suggested for handling the issue of different TAG for PUCCH cell switching.

Reference

[1] R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e