3GPP TSG RAN WG1 #107bis-e	R1-2200221

# e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.8

Source:	Samsung

Title:	UE features for NR Coverage Enhancement

Document for:	Discussion and decision

- Introduction

This contribution discusses the remaining issues on UE features for Rel-17 Coverage Enhancement WI based on the latest RAN1 UE features list [1].

- RAN2 guidelines for UE capability definitions

In RAN1#107-e, the major discussion point of FG for coverage enhancement was whether to split/merge the corresponding FGs. During the discussion, it was pointed out that RAN2 guidelines [2] had provided to avoid defining  functionality that has no RRC configuration but is dependent on capability bits (copied below for the reference). Unless otherwise justified, it is still valid for Rel-17 UE capability discussion:

RAN2 guidelines for UE capability definitions [2] … 5 Avoid defining functionality that has no RRC configuration but is dependent on capability bits. The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting the reported capabilities. A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

- Feature Group for Enhancements on PUSCH Repetition Type A

In case of PUSCH repetition Type A enhancements, it was argued whether to split/merge FG 30-1/30-1a and FG 30-2/30-2a, respectively. Previously, we preferred to merge in light of simplifying the UE capability signaling and no meaningful UE implementation differences. On the other hand, RAN1 has ended up with the introduction of repK-r17 applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH while numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition Type A. This seems to resolve the raised concern during RAN1#107-e with respect to above RAN2 guidelines.

In this regard, we are fine to keep current FG structure for FG 30-1/1a/2/2a for the sake of progress. Further, we suggest to remove FFFs in FG 30-1a/2a.

Proposal 1: Keep current FG structure for FG 30-1/1a/2/2a and remove FFSs.

| 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. |
| --- | --- | --- |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 |
| 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. |
| 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 |

- Feature Group for TBoMS

FG 30-3 has the debating point, i.e., whether to split the FG according to DG and CG. Again, we still do not see the need to split current FG 30-3. It was argued that FG associated with DG and CG were split in Rel-15 (e.g., FG 5-16/5-17). On the contrary, in Rel-16, a single capability (FG 11-5) consists of DG and CG. Moreover, NO SPLIT would be compliant with above RAN2 guidelines [2].

Proposal 2: Update FG 30-3 by removing FFS.

| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG |
| --- | --- | --- |

- Feature Group for DM-RS bundling

FG 30-4a/30-4b/30-4c define specific DM-RS bundling capability combined with PUSCH Type A repetition, PUSCH Type B repetition, and PUSCH TBoMS, respectively with the FFS of possible merging. Similar with above discussions, a single capability for DM-RS bundling would be enough. For example, if a UE supports DM-RS bundling and PUSCH TBoMS respectively, it means the UE also supports DM-RS bundling for TBoMS. In case of FG 30-4d, having separate RRC signalling of enabling/disabling DM-RS bundling for PUCCH, we are fine to keep it as a separate FG.

Another FFS point is whether to split to back-to-back transmission and non-back-to-back transmission. In RAN1#107-e, it was commented that there was an agreement of non-back-to-back transmission as a UE capability. However, it does not necessarily mean splitting back-to-back and non-back-to-back transmission. Moreover, if we introduce FGs for non-back-to-back transmission, what is the relation with FG 30-4g (Restart DM-RS bundling after the events that violate power consistency and phase continuity)? We would like to get a clarification on this before we introduce FGs dedicated for non-back-to-back transmission.

Proposal 3: Merge FGs 30-4b, and 30-4c into FG 30-4a and remove the corresponding FFS.

Proposal 4: RAN1 to clarify splitting FG for non-back-to-back transmission in relation with FG 30-4g (Restart DM-RS bundling after the events that violate power consistency and phase continuity).

| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A and PUCCH FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |
| --- | --- | --- |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |

- Mandatory/optional

All Rel-17 FGs should be “Optional with capability signaling”. We may need to decide which FG is the basic FG for Rel-17 Coverage Enhancement.

Proposal 5: Set “Optional with capability signaling” for all feature groups under Rel-17 Coverage Enhancement.

- Conclusions

We propose the following based on above discussion. Basically, the underlying assumption is to avoid introducing too specific/fragmented UE capability to support Rel-17 Coverage Enhancement:

Proposal 1: Keep current FG structure for FG 30-1/1a/2/2a and remove FFSs.

| 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. |
| --- | --- | --- |
| 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 |
| 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. |
| 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 |

Proposal 2: Update FG 30-3 by removing FFS.

| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG |
| --- | --- | --- |

Proposal 3: Merge FGs 30-4b, and 30-4c into FG 30-4a and remove the corresponding FFS.

Proposal 4: RAN1 to clarify splitting FG for non-back-to-back transmission in relation with FG 30-4g (Restart DM-RS bundling after the events that violate power consistency and phase continuity).

| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A and PUCCH FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |
| --- | --- | --- |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |

Proposal 5: Set “Optional with capability signaling” for all feature groups under Rel-17 Coverage Enhancement.

References:

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2001513, Guidelines for UE capability definitions, RAN2

- R1-2112141, Summary on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.)

# Annex: Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e [2]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions FFS whether to merge with FGs 30-4a/4b/4c FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | [Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH FFS whether to merge with FG 30-4f | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling FFS whether to merge with FG 30-4e | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | [4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | MRepetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |