3GPP TSG RAN WG1 #107bis-e	        		             R1-2200223

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.12

Source: 	Samsung

Title: 	UE features for NR MBS

Document for:	Discussion and Decision

Introduction

This contribution considers UE features list for Rel-17 MBS. In RAN1#107-e, the followings were agreed.

Agreement

- FG 33-6-2 is kept as “Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE” as follows

- FG 33-6-3 is kept as “More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot” as follows

| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 |

Agreement

- FG 33-2 is separated as following FGs

  - FG 33-2: current FG 33-2

    - FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback as a component of FG 33-2

  - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

  - FFS whether to separate the capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast

  - FFS whether to separate the capability for support of PTP retransmission for multicast

  - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast |  |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a |

Agreement

- Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3

  - Option 1:

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for Type-2 HARQ-ACK codebook for multicast

  - Option 2:

    - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast

  - Option 3:

    - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2

    - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

Discussions

In [1], the preliminary RAN1 UE features list for Rel-17 was discussed.

Mandatory/optional and DCI format

For FG 33-1, this feature should be optional. Instead, it is proposed to note that this is a basic FG for MBS.

| Features | Index | Feature group | Components | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. | Optional without capability signalling This is a basic FG for MBS. |

Pre-requisite

For 33-3-2/3, pre-requisite can be either 33-1 or 33-2, but should not be both.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1 or 33-2 |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. - FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1 or 33-2 |

Adding G-CS-RANTI for FG 33-6-1

For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast. |

Separating the capability for support of PTP retransmission for multicast for FG 33-2

As discussed in our contribution [3], it is beneficial for a UE to indicate the support of combining a PTM initial transmission and a PTP retransmission in case of different circular buffer. A new capability should be defined to indicate whether the UE can or cannot combine. Therefore, we propose to separate component 8 of FG 33-2 and use it as a pre-requisite of the combining capability.

Conclusions

This contribution considered the UE features list for Rel-17 MBS. The above changes are proposed.

- References

[1] RAN1 Chair’s Notes for RAN1#107-e, Nov, 2021.

[2] R1-2112145, “Summary on UE features for NR MBS,” Moderator (NTT DOCOMO, INC.), Nov. 2021.

[3] R1-2200213, “Remaining issues on group scheduling for RRC_CONNECTED UEs”, Jan. 2022