3GPP TSG RAN WG1 #107bis-e			R1-2200238

e-Meeting, January 17th – 25th, 2022

Source:	NTT DOCOMO, INC.

Title:	Joint channel estimation for PUSCH

Agenda Item:	8.8.1.3

Document for: 	Discussion

- Introduction

In this contribution, we discuss remaining issues on joint channel estimation for PUSCH in coverage enhancements.

- Discussion on joint channel estimation for PUSCH

Configured (nominal) time domain window for CG-PUSCH

- Reason for change

At RAN1#106-e meeting, the configured time domain window (TDW) design was discussed, and the following agreement was made [1].

Agreement: - For PUSCH repetition type A counting based on physical slots - The start of the first configured TDW is the first physical slot for the first PUSCH transmission. - The end of the last configured TDW is the last physical slot for the last PUSCH transmission. - For PUSCH repetition type A counting based on available slots - The start of the first configured TDW is the first available slot for the first PUSCH transmission. - The end of the last configured TDW is the last available slot for the last PUSCH transmission. - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

The above agreement ensures that the first configured TDW is the first physical slot or available slot for the first PUSCH transmission. However, TDW determination for CG-PUSCH has not been discussed sufficiently at RAN1 meetings. The mechanism based on the above agreements increases gNB burden to capture the correct configured TDW positions for CG-PUSCH with RV sequences {0,0,0,0}, {0,3,0,3}, which achieves low latency communications. Since the initial transmission for CG-PUSCH could be any transmission occasions associated with RV=0 if startingFromRV0 is set to ‘on’ and RV sequence is {0,0,0,0} or {0,3,0,3}, the start of the first configured TDW also could be any transmission occasions associated with RV=0. As UE can choose which transmission occasion associated with RV=0 is the initial transmission for CG-PUSCH, gNB might end up false alarm or miss detection of the actual initial transmission as shown in Fig.1. In that case, the misalignment of configured TDW positions between UE and gNB might be caused not only for the first configured TDW but also the other subsequent TDWs, because the start of TDWs could be determined by the end of previous TDWs. To avoid this error propagation issue, we propose the configured TDW starts at the first transmission occasion at least for CG-PUSCH. When this proposed determination is applied, the same configured TDW positions can be determined regardless of miss detection or false alarm, as shown in Fig.1.

Figure 1. Configured TDW position for CG-PUSCH.

- Summary of change

The start of the first configured TDW should be the first physical/available slot for the first PUSCH transmission occasion for CG PUSCH.

- Consequences if not approved

The false alarm or miss detection of the first PUSCH transmission results in the misalignment of configured TDWs.

Proposal 1: Adapt the following text proposal.

Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.214--------------------------- <Unchanged parts omitted> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUCCH transmissions of a PUCCH repetition: - The start of the first nominal TDW is the first slot determined for the first PUCCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUCCH transmission. - The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------

- Group common TPC commands

At RAN1#106bis-e meeting, TPC command indicating the action during time domain window was discussed and the following agreement was made [2]

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

The number of TPC commands UE can receive within a configured TDW when UE is not configured to accumulate TPC commands is under discussion. According to the agreement, when UE receives more than one TPC commands indicating the action within a configured TDW, UE just applies the last received TPC command after the configured TDW. As this UE behavior is clear and simple, there is no need to put the constraint on the number of expected TPC commands indicating actions during a configured TDW.

Proposal 2: No need to put the constraint on the number of expected TPC commands indicating actions during a configured TDW.

- Conclusion

In this contribution, we discussed joint channel estimation for PUSCH in coverage enhancements. Based on the discussion we made following text proposal and view.

Proposal 1: Adapt the following text proposal.

Text proposal 1:

-------------------------- Start of Text Proposal for TS 38.214--------------------------- <Unchanged parts omitted> 6.1.7 UE procedure for determining time domain windows for bundling DM-RS For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, and for PUCCH transmissions of PUCCH repetition, when PUCCH-DMRS-Bundling is enabled, the UE determines one or multiple nominal TDWs, as follows: - For PUSCH transmissions of repetition Type A, PUSCH repetition Type B and TB processing over multiple slots, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUSCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUSCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots of N∙K PUSCH transmissions, and where: - For PUSCH transmissions of PUSCH repetition Type A, N=1 and K is the number of repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of PUSCH repetition Type B, N=1 and K is the number of nominal repetitions, as defined in Clause 6.1.2.1. - For PUSCH transmissions of TB processing over multiple slots, N is the number of slots used for TBS determination and K is the number of repetitions of the number of slots N used for TBS determination, as defined in Clause 6.1.2.1. - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213]. - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot for the first PUSCH transmission occasion. - The end of the last nominal TDW is the last slot for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUCCH transmissions of a PUCCH repetition: - The start of the first nominal TDW is the first slot determined for the first PUCCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUCCH transmission. - The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 --------------------------

Proposal 2: No need to put the constraint on the number of expected TPC commands indicating actions during a configured TDW.

References

- 3GPP, RAN1#106e, RAN1 Chairman’s Notes, Sep. 2021.

- 3GPP, RAN1#107-e, RAN1 Chairman’s Notes, Nov. 2021.