3GPP TSG RAN WG1 #107bis-e   	           R1-2200246

e-Meeting, January 17th – 25th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel-17 FeMIMO UE features

Agenda Item:	8.15.1

Document for: 	Discussion and Decision

Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #107e is captured in R1-2112902. In this contribution, we provide our views on Rel.17 FeMIMO UE features.

Discussions

  - Multi-beam Operation

We believe FG23-1-1 and FG23-1-2 should be reported per band.

For FG23-1-1, we have the following suggestion:

- Component 2: “[, SS-RSRP]” is not necessary in addition to “SSBRI-RSRP”, and hence we can delete it.

- Component 3: MAC CE-based TCI state indication should be basic feature of FG23-1-1. Hence, the component values should be {“MAC CE”, “MAC CE+DCI”}.

- Component 4: The number of TCI state pools across all BWPs/CCs should not be limited by UE capability. In Rel.15, TCI state pool (list) is configured per BWP in a CC, and we should not restrict gNB to configure one TCI state pool per BWP in a CC by UE capability.

- Component 5/6: In Rel.15, the number of configured TCI state for PDSCH per CC is reported, however, the number of configured TCI states across all BWPs/CCs is not reported. Hence, only component 6 should be reported.

  - For the candidate values of component 6, in Rel.15, UE is mandated to report the number of RRC configured TCI states as “64” in FR2 and “the maximum number of allowed SSBs” in FR1. Since Rel.17 unified TCI framework replaces all of Rel.15/16 beam indication mechanism, the mandatory value of Rel.15/16 TCI state should be supported in Rel.17 TCI state. That is, UE is mandated to report the number of RRC configured TCI states per BWP in a CC as “64” in FR2 and “the maximum number of allowed SSBs” in FR1.

- Component 7/8: In Rel.15, the number of activated TCI state for PDSCH per BWP in a CC is reported, however, the number of activated TCI states across all BWPs/CCs is not reported. Hence, component 7/8 should be reported per BWP in a CC.

- Component 10: Component 10 should be a basic feature of FG23-1-1. In RAN1#107e, we made the following WA, which means once Rel.17 TCI state is configured, gNB cannot configure Rel.15/16 TCI state/QCL in the same band. Hence, component 10 cannot be an optional feature.

Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI

- Component 12: In RAN1#106e meeting, the number of RRC configured PCIs per CC for M-TRP inter cell was agreed as X<=7. We believe UE would report the same/similar value for L1/L2 inter cell mobility and M-TRP inter cell. Hence, we suggest to add candidate value of component 12 as {1, …, 7} per CC.

For FG23-1-2, we have the following suggestion:

- Component 3-4: We believe the number of PCIs for beam measurement should be common for both FR1 and FR2.

- Component 5-7: We don’t think we should introduce new UE capability to report the number of SSBs for inter-cell beam reporting.

For FG23-1-3, we have the following suggestion

- Component 1: Regarding the text in bracket, we think it should be kept. From our perspective, it is better to clarity that the report is included in PHR report.

- Component 2: Considering multiple entry PHR, PHRs and PMPRs of multiple CCs are included in one MAC CE, it is better to clarify the maximum number of reported P-MPR and SSBRI/CRI pairs is per CC or across all CCs. In our view, the maximum number of reported P-MPR and SSBRI/CRI pairs should be per CC.

- Component 3: Similar as component 2, it is better to clarify the maximum number of candidate RS(s) is per CC or across all CCs. In our view, the maximum number of candidate RS(s) is related to the number of RS(s) that UE can measure. Thus, the maximum number of candidate RS(s) can be across all CCs. Regarding candidate value, similar as DL beam measurement, {4,8,16,32,64} can be considered.

Proposal 1: Adopt the following for Rel-17 Multi-beam Operation.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | For both intra- and inter-cell beam management: 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS (PLRS and other, including handling of beam alignment/misalignment for PLRS) 3. Supported mode of [MAC-CE/MAC-CE+DCI-based TCI state indication (including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment) 6. The maximum number of configured TCI states per CC in a band 7. Support number of MAC-CE activated joint TCI states across per BWP in a CC 8. The maximum number of MAC-CE activated separate DL and UL TCI states across per BWP in a CC 9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE 11. The minimum beam application time in Y symbols 12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states per CC |  |  |  |  | per band |  |  |  | 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values {“MAC CE”, “MAC CE+DCI”} 12. candidate values {1, … , 7} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] For component 6, UE supports FG23-1-1 shall report at least “64” in FR2 and “the maximum number of allowed SSBs” in FR1. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of L1-RSRP measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of L1-RSRP reporting of Kmax SSBRI-RSRP beams, with at least one beam associated with a PCI different from serving cell PCI 3. The number of RRC-configured PCI(s) different from serving cell PCI for beam measurement | [2-24, 2-29] |  |  |  | per band |  |  |  | {Support, Not support} [2. Candidate value of {1,2,3,4}] 3. candidate values {1, …, 7} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs per CC 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation across all CCs |  |  |  |  |  |  |  |  | 2. Candidate value of {1,2,3, 4} 3. Candidate value {4,8,16,32,64} | Optional with capability signalling |

  - Multi-TRP for PDCCH

For FG23-2-1, we have following suggestion.

- Component 1: regarding the text in bracket, we think it should be kept. In our view, if a UE supports PDCCH repetition, it supports both TDM and FDM repetition and also supports repetition for Type3 CSS. There is no need to have separate capability for FDM or TDM or Type3 CSS, and it is better to have the text in bracket to make it clear.

- Component 2: since it was concluded that there is no consensus to support RRC configuration of number of BDs, to align the understand of number of BDs between gNB and UE, only one number of BDs should be reported by UE. Thus, the text in bracket, i.e., [or more], should be removed.

- Component 3: similar as component 2, only one number of BDs should be reported by UE, the text “or {2,3}” should be removed.

- Component 4: since it was agreed that UE capability for such overlap is introduced, we support to have component 4. Meanwhile, we think a note should be added to clarify how the number of overlaps is counted when N PDCCH candidates are overlapped, i.e., counted as one overlap or counted as N overlaps. In our understanding, if N PDCCH candidates are overlapped, the number is counted as one.

For FG23-2-2, we think 23-6-1 and 23-6-2 should be removed from the prerequisite FG, because in our view, this FG is only related to non-SFN PDCCH repetition and is not related to SFN PDCCH. Meanwhile, we support to keep the text in bracket.

For FG23-2-3, we think this FG should be removed. As analyzed above, there is no need to have separate capability for Type3 CSS.

Proposal 2: Adopt the following for Rel-17 M-TRP PDCCH

| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS 2. Support of reporting one number(s) as required number of BDs for the two PDCCH candidates 3. If 3 is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs If 2 is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs. 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component2: {2,3} Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] Note: for component4, if N PDCCH candidates are overlapped, the number of overlaps is counted as one. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | 23-2-1 |  |  |  |  |  |  | FR2 only |  |

  - Multi-TRP for PUSCH, PUCCH

For M-TRP PUSCH repetition, in our view, M-TRP PUSCH repetition type A and M-TRP PUSCH repetition type B can be merged into one FG, because in our understanding, other components in this FG are common for repetition type A and repetition type B, e.g., if UE supports cyclic mapping, it supports for both repetition type A and repetition type B, thus, there is no need to have separate FGs for repetition type A and type B.

Furthermore, CB based and NCB based operation can be split into two components since UE may have different capability of whether to support CB or NCB PUSCH operation. And regarding number of SRS resources, it can be clarified that UE supports two CB or NCB SRS resource sets.

Proposal 3: Adopt the following for Rel-17 M-TRP PUSCH

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) 2. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) 3. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A 4. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 5. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 6. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) 7. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A 8. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) 9. Support of A-CSI report on two PUSCH repetitions 10. Support of SP-CSI report on two PUSCH repetitions 11: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 12. Support multi-TRP PUSCH repetition with CB based PUSCH operations and corresponding parameters including two CB SRS resource sets 13. Support multi-TRP PUSCH repetition with NCB based PUSCH operations and corresponding parameters including two NCB SRS resource sets |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For M-TRP PUCCH, for FG23-3-2, we have following suggestion.

- Component3: the text in bracket should be removed, because it is duplicate with the text at the end of the sentence.

- Component4: the text in bracket should be removed. It was agreed that for DCI format 1_0, single TPC field is applied.

Proposal 4: Adopt the following for Rel-17 M-TRP PDCCH

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats1_1 / 1_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Inter-cell multi-TRP

For component 2, 3, and 4 which are related to the number of RRC-configured PCIs different from serving cell, component 2 can be deleted and component 3 and 4 can be kept considering the duplication. In addition, based on agreements, component 3 is for case1 and component 4 is for case2, and we think both FR1 and FR2 can be considered for both cases. For component 5, since we have agreed case1 and case2 cannot be enabled simultaneously, it is important to define a default case for inter-cell MTRP operation, otherwise, it is not friendly to deploy this feature in NW. For this default case, we think new component is not needed. Instead, a note is sufficient.

Proposal 5: Adopt the following for Rel-17 inter-cell M-TRP.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. Supported max number of RRC-configured PCIs different from serving cell PCI for case1 (X1) SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI 3. Supported max number of RRC-configured PCIs different from serving cell PCI for case2 (X2) SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI Note: default case to be supported, case2 with X2=1 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Beam management for multi-TRP

For 23-5-1, for component 1, to align the wording, it is suggested to keep ‘group’ and delete the ‘[pair/][in one reporting instance]’. For other components, since component 2 and 5 are naturally supported for group-based L1-RSRP beam reporting, separate components are not needed and can be removed, while component 3, 4, and 6 can be kept.

For23-5-2. Component 2 is also naturally supported and separate component is not needed. To make the description more complete, ‘two BFD-RS sets’ can be added into component 1. In component 8, it is important to keep both single DCI and multi-DCI as both cases are supported for MTRP BFR. In addition, based on latest agreement, a new component should be added for support of an association configuration between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for multi-DCI based MTRP BFR.

Proposal 6: Adopt the following for Rel-17 beam management for M-TRP.

| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per group 2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs 3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs 4. Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set, for the two BFD-RS sets 2. Maximum number of CSI-RS resources of both BFR-RS sets across all CCs 3. Maximal number of different SSBs of both BFD-RS sets across all CCs 4. Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs 5. Support up to two PUCCH-SR resources for MTRP BFRQ 6. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP 7. Support of single DCI/multi-DCI based BFR enhancement 8. support of an association configuration between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for multi-DCI based MTRP BFR |  |  |  |  |  |  |  |  | Candidate values: {1, 2,…} | Optional with capability signalling |

  - HST-SFN deployment

We have the following comments:

- For FG23-6-1 and FG23-6-2,

  - During the RAN1 discussion for Rel.17 HST, the assumption of the basic operation is a combination of “1) SFN-PDCCH + SFN-PDSCH”, and some UE vendors claim difficulty of supporting other combinations. Hence, the basic capability of FG23-6-1 and FG23-6-2 should be the combination of “1) SFN-PDCCH + SFN-PDSCH”, and other combinations should be reported in separate components. The other combinations are “2) S-TRP PDCCH + SFN-PDSCH” and “3) SFN-PDCCH + S-TRP PDSCH”. Note that 3) is only supported in SFN scheme A based on the previous agreements.

  - The most of function between FR23-6-1 and FG23-6-2 are common, and hence we agree FG23-6-1 is pre-requisite feature of FG23-6-2.

  - We believe FG23-6-1 and FG23-6-2 should be reported per band.

  - For FG23-6-2, SFN scheme B is supported in both FR1 and FR2 by RAN1#107e agreement. Hence, “[FR1 only]” should be deleted.

- For FG23-6-1a and FG23-6-2a,

  - FG23-6-1a and FG23-6-2a can be reported per UE, as an optional capability.

- For FG 23-6-4,

  - Default beam is useful for both HST and URLLC, and we should add URLLC in addition to HST for the name of FG23-6-4.

  - We agree all of components in FG23-6-4 are needed. Alternatively, we can create more general component(s) which combines the multiple components.

Proposal: Adopt the following for HST-SFN deployment

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for PDSCH scheduled by scheme A PDCCH 2. Support of scheme A for PDSCH scheduled by single TRP PDCCH] and default QCL assumption with two TCI states for PDSCH 3. Support of single TRP PDSCH scheduled by scheme A PDCCH and default QCL assumption with one TCI states for PDSCH |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for PDSCH scheduled by scheme B PDCCH 2. Support of scheme B for PDSCH scheduled by single TRPPDCCH and default QCL assumption with two TCI states for PDSCH | 23-6-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 |  |  |  | Per UE |  | Yes |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup for HST/URLLC | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - SRS

For FG23-8-1, we have the following two comments:

- RAN1 has agreed to support cross carrier triggering of Rel-17 enhanced A-SRS, by reusing the same manner as Rel-16 A-SRS for determining reference slot. As no dedicated capability is defined for cross carrier triggering of A-SRS even in Rel-15 and Rel-16, we believe FG23-8-1 should explicitly cover the support of cross carrier triggering of Rel-17 enhanced A-SRS as well.

- There is a part with bracket on the newly supported DCI field. We believe it should be captured as it is since FG23-8-1 should cover both the functionality to indicate the next available slot and the one to use the new DCI field for indicating t value. We do not see the need to define them as separate capabilities.

For FG23-8-3, there is an FFS on Component 1 candidate values. The (explicitly) agreed transceiver architectures are 1T6R, 1T8R, 2T6R, 2T8R and 4T8R, which should be captured. A remaining issue would be whether to capture downgrading configurations for each transceiver architecture. For the existing architectures (i.e. with Rx less than 4), some configurations to use less Rx ports than the implemented ones are supported in Rel-16, which achieves more SRS configuration flexibility and smaller SRS overhead for DL CSI acquisition. Since the same benefit is expected even with larger number of Rx ports, we believe it would be good to support such downgrading configurations together.

For FG23-8-7, the part with bracket does not capture the latest status correctly since RAN1 has agreed to support start RB location hopping even for A-SRS. Therefore, an update to capture it is needed.

Proposal: Adopt the following for Rel-17 SRS.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement for both self/cross carrier triggering | Support of determining aperiodic SRS location based on available slot and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: - 't1r6' for 1T6R - ‘t2r6' for 2T6R - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R - ‘t1r6-t2r6' for 1T6R/2T6R - ‘t1r8' for 1T8R - ‘t2r8' for 2T8R - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R - ‘t1r8-t2r8' for 1T8R/2T8R - ‘t4r8' for 4T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R1T8R/2T8R/4T8R | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  |  |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS | 23-8-6 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - M-TRP CSI

We think 23-7-3 could be merged into 23-7-1 with introduction of a default value as basic feature of MTRP CSI enhancement. We prefer the default value of Ks,max =4 but we can also accept Ks,max=2. For 23-7-2, we do not think it is needed but we can live with it. Current candidate values for 23-7-2 is not aligned with the combinations.

Proposal: Adopt the following for Rel-17 M-TRP CSI.

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  | [Per band / per BC] |  |  |  | [Component 2 candidate value set: {4, 5, 6, 7, 8} Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | 23-7-1 |  |  |  | Per band |  | FR2 only |  | Note: ‘NCJT’ and ‘single-TRP’ are not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |

  - FDD CSI

We’re generally fine with current version. For component 23-9-3, we think it is fine to support separate capabilities for M=1 and M=2. For component 1 in 23-9-2 and component 2 in 23-9-4, similar as MTRP CSI, we do not think it is needed but we can live with it.

Proposal: Adopt the following for Rel-17 port selection CSI.

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 - Support rank 1,2 - Support parameter combinations with M=1 | 2-35 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | [23-9-1] |  |  |  | FFS |  |  |  | [Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations [Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1] 2. Support parameter combinations with M=2 3. Support of DFT FD bases of size N | 23-9-1 |  |  |  | [per band and per BC] |  |  |  | [Component 1 candidate values - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] [Component 4 candidate values: {N=2 only, N=2 and N=4}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II separate capabilities for M = 1 and M = 2 | 23-9-1[, 23-9-2] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II [2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2] | 23-9-2 |  |  |  | [per band and per BC] |  |  |  |  | Optional with capability signalling |

Summay

Based on the discussions on Rel.17 FeMIMO UE features in Session 2, we propose to adopt the updated UE feature tables for each UE feature.