3GPP TSG RAN WG1 #107bis-e			R1-2200249

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.3

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	[draft] Summary on UE features for enhanced IIoT and URLLC

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.15.3 regarding UE features for enhanced IIoT and URLLC and captures the following email discussion.

[107bis-e-R17-UE-features-eIIoT-URLLC-01] Email discussion UE features for enhanced IIoT and and URLLC – Shinya (DOCOMO) - 1st check point: January 20 - Final check point: January 25

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e [1], there are following feature groups for enhanced IIoT and URLLC.

- 25-1	SPS HARQ-ACK deferral in case of TDD collision

- 25-2	Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

- 25-3	Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8

- 25-3a	Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

- 25-3b	Inter-subslot frequency hopping for PUCCH repetitions

- 25-4	One-shot HARQ ACK feedback triggered by DCI format 1_2

- 25-5	PHY priority handling for one-shot HARQ ACK feedback

- 25-6	Enhanced type 3 HARQ-ACK codebook feedback

- 25-7	Triggered HARQ-ACK codebook re-transmission

- 25-8	Semi-static HARQ-ACK codebook for sub-slot PUCCH

- 25-9	Semi-static PUCCH cell switching

- 25-10	PUCCH cell switching based on dynamic indication

- 25-11	4-bits subband CQI

- 25-12	UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations

- 25-13	UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations

- 25-14	PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH

- [25-15]	PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH

- 25-16	HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

- 25-18	Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

- 25-19	RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS

- 25-19a	RTT-based Propagation delay compensation based on DL PRS and SRS

- 25-20	Propagation delay compensation based on legacy TA procedure

The issues to be discussed are tagged and colour coded with High priority, Medium priority, or Low priority, considering RAN2 impact especially for capability signaling design.

- 25-1: SPS HARQ-ACK deferral in case of TDD collision

In [1], FG 25-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-18 | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - There is a typo “Idenfify” in the column of components. - For the type, we are fine with per UE. Looking at the SPS HARQ-ACK deferral mechanism itself, the motivation to support finer granularity e.g. per band or FSPC, is not clear to us. |
| --- | --- | --- |
| [4] | vivo | - Prerequisite FG In Rel-15, downlinkSPS is configured per UE corresponding to FG 5-18 since only one SPS configuration is supported per cell group. In Rel-16, sps-r16 is defined per band i.e., FG12-2 indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. The complexity for FG 25-1 applied for single SPS configuration, may not be the same for multiple SPS configurations, so we think prerequisite feature groups should include both FG 5-18 or FG12-2, but not be “5-18 and 12-2”. - Type The prerequisite feature groups of FG 25-1 are FG 5-18 or FG 12-2. Considering type of FG 12-2 is per Band, the type of FG 25-1 should be at least per band. Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2. Type of FG 25-1 should be at least per band. 25. NR_IIOT_URLLC_enh; 25-1; SPS HARQ-ACK deferral in case of TDD collision; 1. Idenftify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK; 5-185-18 or 12-2; Yes; N/A; Per UEBand; No (TDD only); No; N/A; Optional with capability signaling. |
| [7] | Samsung | - Confirm all yellow highlight parts |
| [8] | NTT DOCOMO, INC. | - FG 25-1: SPS HARQ-ACK deferral in case of TDD collision - Type should be per UE - The prerequisite feature groups are FG 5-18 or FG 12-2 to cover multiple SPS configurations case |
| [9] | Spreadtrum Communications | - Prerequisite FG - Prerequisite feature groups should include FG 5-18 or FG12-2. - Type - Per band is preferred. The reason is FG 12-2 is per band. So FG 25-1 should at least per band. |
| [10] | Qualcomm Incorporated | First, it is observed that the type of many features is “per UE” in R1-2108679. We think this is very problematic. Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. Figure 1: Example process to set the support of mandatory features with capability indication Due to the above concern, we make the following proposal. Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. In the following, we analyze each UE feature and make some proposals to capture the key changes on UE features for Rel-17 IIOT/URLLC. With regards to the Feature 25-1 (SPS HARQ Deferral in TDD), the feature is not necessary at FDD. Prerequisite for this feature is SPS support - feature 5-18 and therefore the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 2: Feature 25-1 (SPS HARQ Deferral in TDD collision) is for TDD only. Furthermore, it should a per Feature Set Per Component Carrier (FSPC) feature rather than a per UE feature. 25. NR_IIOT_URLLC_enh; 25-1; SPS HARQ-ACK deferral in case of TDD collision; 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot 4. Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK; 5-18; Yes; N/A; Per UE Per FSPC; No (TDD only); No; N/A; Optional with capability signaling. |
| [11] | OPPO | Prerequisite feature groups: Regarding the prerequisite feature groups for FG 25-1, from our point of view, FG 5-18 (downlinkSPS) and FG 12-6 (SPS periodicity shorter than 10 ms) should be included, otherwise, there would be no need for SPS HARQ-ACK defer. However, adding FG 12-2 (Multiple SPS configurations) is unnecessary since UE may choose to implement SPS HARQ-ACK defer feature without support of multiple SPS configurations. Type: To align with the prerequisite feature groups, FG 25-1 should be per UE. Proposal 1: The type of FG 25-1 is per UE. Proposal 2: The prerequisite feature groups for FG 25-1 include FG 5-18 (downlinkSPS) and FG 12-6 (SPS periodicity shorter than 10 ms). |
| [12] | Intel Corporation | From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-1 SPS HARQ-ACK deferral in case of TDD collision Regarding the issue with testing mentioned by some companies last meeting, we think these need to be discussed not per each UE feature but in general for all work items and agenda items. If accepted, then some ‘per UE’ features can be promoted to per FSPC/BC/FS etc. |
| [13] | Ericsson | From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378) Adopt “Per UE” type for FG 25-1 to FG 25-10. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-1: - Per UE |

Discussion

Medium priority question 2-1:

- Companies are encouraged to provide views on whether the type of FG 25-1 should be per UE or per band or per FSPC

  - Per UE: Huawei, HiSilicon, Samsung, DOCOMO, OPPO, Intel, Nokia, NSB

    - Motivation to support finer granularity is not clear

    - As FG 5-18 and FG 12-6 (if it is a prerequisite FG)

    - Complexity does not scale with bands

    - Not clear that band differentiation would impact the feature and the corresponding testing

    - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

  - Per band: vivo, Spreadtrum

    - As FG 12-2 (if it is a prerequisite FG)

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| vivo | We would be fine if the majority support the type of FG25-1 is per UE. |
| Apple | As in our contribution (RAN WG1 #106bis-e, R1-2110067), due to prerequisite’s type is per band, we prefer to have it as “per band” |
| New H3C | We support the type of FG25-1 is per UE |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| ZTE | Per UE |
| Nokia, NSB | Per UE. Regarding Qualcomm’s observations on 3GPP process for IODT confirmation we need to clarify that there is no such process, not even for Rel-15 features. It is very confusing that this is brought up here as if it was a fact, as the figure itself is from RP-190273 (RAN#83, Nokia) which was not endorsed by RAN Plenary, in part due to opposition by Qualcomm itself. In any case happy to see Qualcomm is now more receptive to the process proposed by Nokia for Rel-15 features onward, but until it is endorsed by RAN we have to assume no such process exists, unfortunately. Regarding the concerns on NTN, this is non-issue, which can be dealt with in the same manner as we dealt with licensed/unlicensed issue in Rel-16. |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Huawei, HiSilicon | Fine with “per UE”. |
| QC2 | The Nokia contribution RP-190273 proposed a specific procedure of how to convert a pre-existing IODT bit, i.e., how to make a 'mandatory with capability' feature to 'mandatory without capability' feature during a later release. This was not agreed. But here we were just referring to the understanding of how sufficient IODT opportunity is determined. It is a common view coming from 3G days where IODT test need separate commercial infra coming from two different vendors. Nokia contribution was not endorsed is not due to this common view not acceptable. It is not endorsed due to other specific issues related the conversion. Nothing is confusing here, unless Nokia think IODT test does not need separate commercial infra coming from two different vendors. |
| Ericsson | Per UE The functionalities under FG 25-1 are not affected when testing for different bands. The issue raised by QC is a generic issue on licensed/unlicensed/NTN. It’s nothing specific to FG25-1. |
| Nokia, NSB | Regarding QC’s comment above, we would like to clarify that RP-190273 did not propose a procedure to convert a pre-existing IODT bit into mandatory without capability. Anyway this is not relevant for the discussion of Rel-17 URLLC features, which should be the focus of this document. |
| QC2 | The Nokia contribution RP-190273 proposed a specific procedure of how to convert a pre-existing IODT bit, i.e., how to make a 'mandatory with capability' feature to 'mandatory without capability' feature during a later release. This was not agreed. But here we were just referring to the understanding of how sufficient IODT opportunity is determined. It is a common view coming from 3G days where IODT test need separate commercial infra coming from two different vendors. Nokia contribution was not endorsed is not due to this common view not acceptable. It is not endorsed due to other specific issues related the conversion. Nothing is confusing here, unless Nokia thinks IODT test does not need separate commercial infra coming from two different vendors. |

Low priority question 2-2:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 25-1

  - FG 5-18: Samsung

  - FG 5-18 or 12-2: vivo, DOCOMO, Spreadtrum

    - To cover multiple SPS configurations case

  - FG 5-18 and 12-6: OPPO

    - Otherwise, there would be no need for SPS HARQ-ACK defer

    - FG 12-2 is unnecessary since UE may choose to implement SPS HARQ-ACK defer feature without support of multiple SPS configurations

| Company | Comment |
| --- | --- |
| Nokia/NSB | We agree with OPPO, that multiple SPS configurations with still 10ms periodicity do not really require SPS HARQ-ACK deferral, but the issue is mainly arising from shorter SPS periodicities (i.e. 12-6). So in this respect 12-6 alone should be sufficient (as 5-18 is a pre-requisite for 12-6 already) |
| Huawei, HiSilicon | We are fine with only listing FG 5-18 as the prerequisite. In our understanding, even only 5-18 is listed here, it doesn’t mean that SPS HARQ-ACK deferral cannot work in case of multiple SPS configurations given by FG 12-2. As long as both 25-1 and 12-2 are reported, then they can work together. However, if people really want to reflect 12-2 in the prerequisite, then we are fine with FG 5-18 or 12-2 as well. As to FG 12-6, it is not necessary in our understanding, we don’t need to restrict the potential application scenario here. |
| Ericsson | Prefer keep prerequisite as 5-18, i.e., no change The functions under FG 25-1 do not require 12-2 or 12-6 to be implemented. |
| QC 2 | FG 5-18 and FG 12-6 should be the prerequisites for FG 25-1. For the UE to support SPS HARQ Deferral (FG 25-1), the UE has to support FG 5-18 (SPS) and FG 12-6 (SPS with periodicity less than 10 ms). This is the case in which the SPS HARQ deferral is meaningful. |

Low priority question 2-3:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-1 which do not have capability signaling impacts, e.g.,

  - Revise component 1 as “Idenftify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot”

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Support |
| QC | Support |

- 25-2 to 25-3b: PUCCH Repetition enhancements

In [1], FGs 25-2 to 25-3b are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK | 25-3 30-5 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3b | Inter-subslot frequency hopping for PUCCH repetitions | 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations | TBD | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-2: Fine with the current version, including the cells in yellow. - FG 25-3b: Add one of {25-3, 25-3a} as the prerequisite group. |
| --- | --- | --- |
| [4] | vivo | For FG 25-3a, the pre-requisite 30-5, i.e., dynamic slot-based repetition, corresponds coverage enhancement feature, while FG 25-3a targets meeting URLLC requirements. There is no strong correlation between these two features. A URLLC UE may only implement sub-slot based repetitions without supporting slot-based repetitions. Therefore, the pre-requisite 30-5 is not needed and should be removed from FG 25-3a. Since prerequisite feature groups of FG 25-3 include both FG 11-3 and FG 4-23 and the type of FG 11-3 is Per FeatureSetUplink, to align with the granularity of the prerequisite, we think the type of FG 25-3 should be per FS. Similarly, since prerequisite feature groups of FG 25-3a include FG 25-3 and the type of FG 25-3 should be Per FS as discussed above, the type of FG 25-3a should also be per FS. Proposal 2: For FG 25-3 and 25-3a, the type should be Per FS. The pre-requisite 30-5 should be removed from FG 25-3a. 25.NR_IIOT_URLLC_enh; 25-3; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE; 4-23 11-3; Yes; N/A; Per UEFS; No; No; N/A; Optional with capability signaling. 25.NR_IIOT_URLLC_enh; 25-3a; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK; 25-3 30-5; Yes; N/A; Per UEFS; No; No; N/A; Optional with capability signaling. |
| [5] | ZTE | Index 25-2 to 25-3a: A remaining issue of last meeting is whether the type of FGs 25-2 to 25-3a should be per UE or per FS or per FSPC. From our perspective, Per UE is preferred. Proposal 1: The type of FGs 25-2 to 25-3a should be kept as per UE. |
| [6] | CATT | - FG 25-3b The prerequisite feature group for FG25-3b is TBD, which should be support of sub-slot based PUCCH repetition in our view. Accordingly, it is proposed that the prerequisite feature group for FG25-3b is 25-3. Proposal 1: The prerequisite feature group for FG25-3b is 25-3. |
| [7] | Samsung | - 25-2: Confirm all yellow highlight parts - 25-3: Confirm all yellow highlight parts - 25-3a: Confirm all yellow highlight parts - 25-3b: - For prerequisite feature group, 4-1, 4-3, 4-4 and 4-5 should be considered, and those entries have FR1/2 differentiation. So, “Need of FR1/FR2 differentiation” needs to be changed as “Yes”. - Confirm all other yellow highlight parts |
| [8] | NTT DOCOMO, INC. | - FG 25-2: Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 - Type should be per UE - FG 4-23 can be removed from the prerequisite feature groups. UE could report FG 25-2 without dependency with FG 4-23. - FG 25-3: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 - Type should be per UE - FGs 4-23 and 11-3 can be kept as prerequisite feature groups - FG 25-3a: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication - Type should be per UE - FGs 25-3 and 30-5 can be kept as prerequisite feature groups - FG 25-3b: Inter-subslot frequency hopping for PUCCH repetitions - Type should be per UE - FG 25-3 can be added as prerequisite feature group |
| [9] | Spreadtrum Communications | - Prerequisite FG - For FG 25-2, prerequisite feature group is 4-23 - For FG 25-3, prerequisite feature group are 4-23 and 11-3 - For FG 25-3a, prerequisite feature group are 25-3 and 30-5 - For FG 25-3b, prerequisite feature group are 25-3 - Type - Per FS for 25-2/3/3a/3b, to align with the granularity of the prerequisites, because FG 11-3 is per FS. |
| [10] | Qualcomm Incorporated | With regards to 25-2 - Repetitions for PUCCH Format 0 and 2 over multiple slots-it would be useful to clarify that the HARQ Codebook is slot-based. The prerequisite for 25-2 is 4-23 – support for PUCCH repetitions- therefore the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 3: For feature 25-2 (Repetitions for PUCCH Format 0 and 2 over multiple slots) a clarification needs to be made that the feature is for slot-based codebook. Furthermore, the feature should be supported per Feature Set Per Component Carrier (FSPC) rather than per UE. With regards to 25-3 - Repetitions for PUCCH Format 0, 1, 2, 3, 4 over multiple sub-slots-the prerequisites for 25-2 are: - 4-23 – support for PUCCH repetitions - and - feature 11-3 - support for more than 1 PUCCH HARQ within a slot. Therefore, the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 4: For feature 25-3 (Repetitions for PUCCH Format 0, 1, 2, 3, 4 over multiple sub-slots) the feature should be supported per Feature Set Per Component Carrier (FSPC). 25. NR_IIOT_URLLC_enh; 25-2; Repetitions for PUCCH format 0, and 2 over multiple slots (for slot based codebook) with K = 2, 4, 8; Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8; 4-23; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-3; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that supported by the UE; 4-23 11-3; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-3a; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication; Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. Note: Dynamic PUCCH repetition factor indication is only supported for HARQ-ACK; 25-3 30-5; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-3b; Inter-subslot frequency hopping for PUCCH repetitions; 1. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Formats 0, 1, 2, 3 and 4 for 7OS slot-based PUCCH configurations. 2. Support inter-subslot frequency hopping for PUCCH repetition operation of PUCCH Format 0 and Format 2 for 2OS slot-based PUCCH configurations; TBD; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. |
| [11] | OPPO | Prerequisite feature groups: For FG 25-2, FG 4-23 should be deleted since the two feature groups are not relevant. For FG 25-3a, it is preferred to remove FG 30-5 from the prerequisite feature groups since there is no dependency for these two features although the mechanism of dynamic indication of repetition number is the same. Type: To align with their prerequisite feature groups, we support per UE for FG 25-2, per FS for FG 25-3 and FG 25-3a. Proposal 3: Remove FG 4-23 from the prerequisite feature groups for FG 25-2. Proposal 4: Remove FG 30-5 from the prerequisite feature groups for FG 25-3a. Proposal 5: The type of FG 25-2 is per UE. The type of FG 25-3 and 25-3a is per FS. |
| [12] | Intel Corporation | - 25-3a Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication - Regarding the pre-requisite of 30-5 (dynamic slot-based repetition), we still think it should be removed. In our view, there is no direct relation with CovEnh feature. Slot-based repetitions target coverage improvement, while sub-slot based repetitions target better latency-reliability tradeoff. In that sense, a URLLC UE may choose to implement sub-slot based repetitions without support slot-based repetitions. - 25-3b Inter-subslot frequency hopping for PUCCH repetitions - Sub-slot PUCCH repetition operation and frequency hopping should be pre-requisites From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-2 Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 - 25-3 Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 - 25-3a Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication - 25-3b Inter-subslot frequency hopping for PUCCH repetitions |
| [13] | Ericsson | From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378) - Adopt “Per UE” type for FG 25-1 to FG 25-10. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-2: - Per UE - 25-3: - Per UE - 25-3a: - The pre-requisite feature group from Cov. Enh. WI of the dynamic PUCCH repetition indication should be 30-5. - Per UE - 25-3b: - Add 25-3 as pre-requisite. - Per UE |

Discussion

Medium priority question 3-1:

- Companies are encouraged to provide views on whether the type of FGs 25-2 to 25-3b should be per UE or per FS or per FSPC

  - FG 25-2

    - Per UE: Huawei, HiSilicon, ZTE, Samsung, DOCOMO, OPPO, Intel, Ericsson, Nokia, NSB

      - As FG 4-23

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per FS: Spreadtrum

    - Per FSPC: Qualcomm

  - FG 25-3

    - Per UE: ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per FS: vivo, Spreadtrum, OPPO

      - As FG 11-3

    - Per FSPC: Qualcomm

  - FG 25-3a

    - Per UE: ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per FS: vivo, Spreadtrum, OPPO

      - As FG 11-3

    - Per FSPC: Qualcomm

  - FG 25-3b

    - Per UE: Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

      - FR1/FR2 differentiation: Samsung

        - As FGs 4-1, 4-3, 4-4 and 4-5 (if they are the prerequisite FGs)

    - Per FS: Spreadtrum

      - As FG 11-3

    - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| vivo | For FG 25-2, we are fine with per UE to align with the pre-requisite FG4-23. Similar reason, FG 25-3, FG 25-3a and FG 25-3b, the type should be per FS. |
| New H3C | We support the type of FG 25-2. Fg25-3, 25-3a/3b are per UE. |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| Nokia/NSB | Per UE. On Qualcomm arguments, please see previous section. |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Huawei, HiSilicon | Per UE for 25-2 while per FS for 25-3/3a, to align with the granularity of the prerequisites. For FG 25-3b, since FG 25-3 or FG 25-3a would be the pre-requisite, then it can be reported as per FS also similar as FG 25-3/3a. |
| Ericsson | Per UE. The functionalities under these feature groups are not affected when testing for different bands. The issue raised by QC is a generic issue on licensed/unlicensed/NTN. It’s nothing specific to these feature groups. |

Low priority question 3-2:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 25-2 to 25-3b

  - FG 25-2

    - FG 4-23 is kept: Huawei, HiSilicon, Samsung, Spreadtrum,

    - FG 4-23 can be removed: DOCOMO, OPPO

      - UE could report FG 25-2 without dependency with FG 4-23

  - FG 25-3

    - FG 4-23 and 11-3: vivo, Samsung, DOCOMO, Spreadtrum

  - FG 25-3a

    - FG 30-5 should be removed (i.e. only FG 25-3): vivo, OPPO, Intel

      - No strong correlation between FGs 25-3a and 30-5

    - FG 30-5 should be kept (i.e. FG25-3 and 30-5): Samsung, DOCOMO, Spreadtrum, Nokia, NSB

  - FG 25-3b

    - Add one of {25-3, 25-3a}: Huawei, HiSilicon

    - FG 25-3: CATT, DOCOMO, Spreadtrum, Nokia, NSB

    - FG 4-1, 4-3, 4-4, and 4-5: Samsung

    - FG 25-3, 25-3a, 4-1, 4-3, 4-4, and 4-5: [Intel]

| Company | Comment |
| --- | --- |
| Nokia/NSB | We are fine with the current pre-requisite FG for 25.2, 25.3, 25.3a For 25-3b, 25-3 should be added as pre-requisite FG |
| Intel | Still prefer removing 30-5 from 25-3a |
| Huawei, HiSilicon | - For FG 25-2, it looks ok to us to keep 4-23, since it should be straightforward that UEs supporting 25-2 for short format slot based repetition should be able to support slot based repetition for other formats. However, would be fine to remove it also, since there is no strong dependency it seems. - For FG 25-3, both FG 4-23 and 11-3 should be kept. In our understanding, sub-slot PUCCH repetitions would result in slot PUCCH repetitions in some cases also, thus 4-23 should be there. - For FG 25-3a, we agree that no need to add 30-5 from coverage as the prerequisite. Sub-slot based repetition can apply same mechanism that designed for slot based repetition in terms of dynamic number indication, but as to the UE capability no strong dependency For FG 25-3b, the prerequisite can be “FG 25-3 or FG 25-3a”. If only FG 25-3 is listed there, the question is whether hopping can be done in case of dynamic indication of the repetitions, in our understanding the hopping can be applicable either. |
| Ericsson | - For 25-2: Delete 4-23 as prerequisite. FG 4-23 is for PF1/3/4, not relevant for PF0/2 - For 25-3: Delete 4-23 as prerequisite. FG 4-23 is for slot based repetition, and not necessarily required for sub-slot based repetition. 11-3 is sufficient for 25-3. - For 25-3a: Delete 30-5 as prerequisite. The reason 25-3a is introduced is that FG for URLLC and FG for coverage enhancement are separate. Thus 25-3a is independent of 30-5. FG 25-3 is sufficient as prerequisite for FG 25-3a. - For 25-3b: FG 25-3 as prerequisite. |
| QC | - For 25-2: Ok to keep 4-23 as prerequisite (no strong opinion). - For 25-3: both FG 4-23 and 11-3 should be kept. With regards to FG 4-23, indeed, it is the feature of slot-based repetition. The UE can theoretically support subslot based repetition (FG 25-3) without supporting slot-based repetition. This is however absurd. If the UE needs to apply repetition, the implementation should be in place independently of the slot configuration. - For 25-3: both 25-3 and 30-5 should be kept. - For 25-3b: 25-3 as a prerequisite |

Low priority question 3-3:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 25-2 to 25-3b which do not have capability signaling impacts, e.g.,

  - Revise the name of FG 25-2 as “Repetitions for PUCCH format 0, and 2 over multiple slots (for slot based codebook) with K = 2, 4, 8”

| Company | Comment |
| --- | --- |
| Nokia/NSB | We do not see a need for the addition in red proposed above. |
| Huawei, HiSilicon | Fine with the change. |
| Ericsson | Support the change above. Also, a minor editorial change for the note in Component column of 25-3: “Note: The support of FG 25-3 doesn’t imply an increase of the maximum number of PUCCHs per slot that are supported by the UE” |
| QC | Ok with the addition/clarification. |

- 25-4 to 25-7: Retransmission of cancelled HARQ-ACK

In [1], FGs 25-4 to 25-7 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-6: - Change “Per UE” to “Per band” to align with the granularity of the prerequisite. - Add a component for the maximum number of configured CCs for CB generation and a component for the maximum number of configured HARQ processes for CB generation. According to the agreement below, smaller CB size is achieved by either limited to a subset of configured CC or a subset of configured HARQ processes, UE capability should be set on the maximum number of configured CC or the maximum number of configured HARQ processes. Agreement One enhanced Type 3 HARQ-ACK codebook is RRC configured either as: - a subset of CC, i.e., all HARQ processes of the subset of CCs are part of the codebook, OR pdsch-HARQ-ACK-enhType3perCC; Configure the one enhanced Type 3 HARQ-ACK codebook using per CC configuration; (1..maxNrofServingCells) of Integer (0,1). - a subset of configured HARQ processes per CC, i.e., different subsets of HARQ processes can be configured for each CC. pdsch-HARQ-ACK-enhType3perHARQ; Configure the one enhanced Type 3 HARQ-ACK codebook using a per HARQ process and CC configuration; (1..maxNrofServingCells) of Bit String (Size (16)).. - Delete 10-16 from the prerequisite feature group for FG 25-6. Enhanced type 3 HARQ-ACK codebook feedback is to achieve smaller size compared to Rel-16 type 3 HARQ-ACK codebook, thus the support of enhanced type 3 doesn't necessarily mean the support of Rel-16 type 3 HARQ-ACK codebook. - For enhanced type 3 HARQ-ACK codebook, different UE capabilities should be set for combination of enhanced type 3 HARQ-ACK codebook with slot-based PUCCH and sub-slot based PUCCH, similar as FG 25-8 for type 1 HARQ-ACK codebook. Therefore, FG 25-6 should be limited to slot-based PUCCH while add a new FG 25-6a for sub-slot PUCCH. 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback for slot PUCCH; 1. Support feedback of enhanced type 3 HARQ-ACK codebook for slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of configured CCs Y for an enhanced type 3 codebook for slot PUCCH. 6. Supported maximum number of configured HARQ processes Z for an enhanced type 3 codebook for slot PUCCH.; 10-16; Yes; N/A; Per band; N/A; N/A; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured For component 5, the UE indicates its capability in the number of configured CCs: {1,...,Y} For component 6, the UE indicates its capability in the number of configured HARQ processes: {1,...,Z}; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-6a; Enhanced type 3 HARQ-ACK codebook feedback for sub-slot PUCCH; 1. Support feedback of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for sub-slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of configured CCs Y for an enhanced type 3 codebook for sub-slot PUCCH. 6. Supported maximum number of configured HARQ processes Z for an enhanced type 3 codebook for sub-slot PUCCH.; 25-6 11-3; Yes; N/A; Per band; N/A; N/A; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,2,4,8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured For component 5, the UE indicates its capability in the number of CCs: {1,...,Y} For component 6, the UE indicates its capability in the number of configured HARQ processes: {1,...,Z}; Optional with capability signaling. - FG 25-7: Fine with the current FG 25-7, including the cells in yellow. |
| --- | --- | --- |
| [4] | vivo | FG 10-16 is the prerequisite feature groups of FG 25-4/5/6, which type is Per band. To align with the granularity of the prerequisite, the type of FGs 25-4 to 25-6 should be Per band. Proposal 3: The type of FGs 25-4 to 25-6 should be Per band. 25.NR_IIOT_URLLC_enh; 25-4; One-shot HARQ ACK feedback triggered by DCI format 1_2; 1.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2.Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value; 10-16 11-1; Yes; N/A; Per UEBand; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-5; PHY priority handling for one-shot HARQ ACK feedback; Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI; 10-16 11-4; Yes; N/A; Per UEBand; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback; 1.Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2.Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 10-16; Yes; N/A; Per UEBand; No; No; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured; Optional with capability signaling. For first bullet in components column of FG 25-7, we think ‘sub-slot’ should be added. FG 25-7 is related to FG 11-1 and 11-4, FGs 11-1 and 11-4 can be added as prerequisite feature groups. The type of prerequisite group FG 11-4 is Per FeatureSetUplink, to align with the granularity of the prerequisite, we think the type of FG 25-7 should be per FS. Proposal 4: For FG 25-7 ‘sub-slot’ should be added in components column. FGs 11-1 and 11-4 can be added as prerequisite feature groups. The type of FG 25-7 is per FS. 25. NR_IIOT_URLLC_enh; 25-7; Triggered HARQ-ACK codebook re-transmission; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 11-1,11-4; Yes; N/A; Per UEFS; No; No; N/A; Optional with capability signaling. |
| [5] | ZTE | Index 25-4 to 25-7: A remaining issue of last meeting is whether the type of FGs 25-4 to 25-7 should be per UE or per FS or per FSPC. From our perspective, Per UE is preferred. Proposal 2: The type of FGs 25-4 to 25-7 should be kept as per UE. Index 25-6 and 25-7: The two indices are separately for the Enhanced type 3 HARQ-ACK codebook feedback and Triggered HARQ-ACK codebook re-transmission. The common things are both supporting DCI format 1_2 and two HARQ-ACK codebooks / PUCCH config. The feature of supporting DCI format 1_2 is 11-1 and the feature of supporting two HARQ-ACK codebooks / PUCCH config is 11-4. So we propose to include 11-1 and 11-4 both in the prerequisite feature column of index 25-6 and 25-7. Proposal 3: Include 11-1 and 11-4 both in the prerequisite feature group column of index 25-6 and 25-7. Index 25-7: Also in 25-7, the component of description for Triggered HARQ-ACK codebook re-transmission is not clear. The “earlier PUCCH slot” is not clear to aim the cancelled HARQ-ACK codebook, below adjustment can clarity the retransmission is for the cancelled HARQ-ACK codebook, the cancellation of the HARQ-ACK codebook is due to various reasons, such as conflicts with the HP channel or Dl symbols, or cancellation based on CI. So we propose: Proposal 4: The following adjustment is proposed for component of 25-7. - 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot of the cancelled HARQ-ACK based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) - 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4). |
| [7] | Samsung | - 25-4: Confirm all yellow highlight parts - 25-5: Confirm all yellow highlight parts - 25-6: - For prerequisite feature group, 25-4 and 25-5 are enough to support 25-6. - Confirm all other yellow highlight parts - 25-7: Confirm all yellow highlight parts |
| [8] | NTT DOCOMO, INC. | - FG 25-4: One-shot HARQ ACK feedback triggered by DCI format 1_2 - Component 1 can be revised as: Support feedback of type 3 HARQ-ACK codebook with full size or reduced size with either one of low or high priority, triggered by a DCI 1_2 scheduling a PDSCH - Component 2 can be revised as: Support feedback of type 3 HARQ-ACK codebook with full size or reduced size with either one of low or high priority, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value - Type should be per UE - FGs 10-16 and 11-1 can be kept as prerequisite feature groups - FG 25-5: PHY priority handling for one-shot HARQ ACK feedback - Component can be revised as: Support transmission of type 3 HARQ-ACK codebook with full size or reduced size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI - Type should be per UE - FGs 10-16 and 11-4 can be kept as prerequisite feature groups - FG 25-6: Enhanced type 3 HARQ-ACK codebook feedback - Component 1 can be revised as: Support feedback of enhanced type 3 HARQ-ACK codebook with or without priority indicator in the triggering DCI, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) - Type should be per UE - FGs 10-16 can be kept as prerequisite feature group. Also, FGs 11-1 and 11-4 can be added as prerequisite feature groups. - FG 25-7: Triggered HARQ-ACK codebook re-transmission - Component 1 can be revised as: Support HARQ-ACK re-transmission from an earlier PUCCH slot of the cancelled HARQ-ACK based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) - Type should be per UE - FGs 11-1 and 11-4 can be added as prerequisite feature groups. |
| [9] | Spreadtrum Communications | FG 25-4: - Prerequisite FGs are 10-16 and 11-1 - Type: Per band, to keep consistence with FG 10-16. FG 25-5: - Prerequisite FGs are 10-16 and 11-4 - Type: Per FS, to keep consistence with FG 11-4. FG 25-6: - Components: - Split a separate FG (FG 25-6a) for component 4. Because enhanced Type 3 HARQ-ACK codebook feedback can be used without two priorities. Component 4 is not an essential combined part for enhanced Type 3 CB. - Two separate FGs for DCI format 1_1 and DCI format 1_2 (FG 25-6b). Because DCI 1_1 for one-shot HARQ-ACK CB defined in 10-16, however, DCI 1_2 defined in 25-4. It is nature to give two FGs for Enhanced type 3 HARQ-ACK codebook feedback, which also keep consistency with Rel-16 and Rel-17 UE feature. - Add new sub-components for components 3, i.e. with PDSCH scheduling, without PDSCH scheduling and Enhanced Type 3 codebook indicator field in DCI according to the following agreements. Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the FDRA field is valid, then a PDSCH is scheduled - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE selects the 1st indexed e-Type 3 HARQ-ACK CB in the M configured e-Type 3 HARQ-ACK CBs. - Prerequisite FGs: - FG 25-6: 10-16; - FG 25-6a: 25-5 - FG 25-6b: 25-4 - Type: - FG 25-6: Per band, to keep consistence with FG 10-16. - FG 25-6a: Per FS, to keep consistence with FG 25-5 - FG 25-6b: Per band, to keep consistence with FG 25-4 FG 25-7: - Components: - Split a separate FG (FG 25-7a) for component 2. Because HARQ-ACK re-transmission can be used without two priorities. Component 2 is not an essential combined part for FG 25-7. - Two separate FGs for DCI format 1_1 and DCI format 1_2 (FG 25-7b). - Prerequisite FGs: - FG 25-7a: 11-4 - FG 25-7b: 11-1 - Type: - FG 25-7: Per band. Similar as enhanced Type 3 HARQ-ACK CB, HARQ-ACK re-transmission also need to be per band. - FG 25-7a: Per FS, to keep consistence with FG 11-4 - FG 25-7b: Per band, to keep consistence with FG 25-7 |
| [10] | Qualcomm Incorporated | With regards to 25-4 -One-shot HARQ feedback triggered by DCI 1_2-the prerequisites are: - 10-16 (support for 1-shot HARQ-ACK feedback) and - 11-1 (monitoring DCI 1_2) . Therefore the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 5: Feature 25-4 (One-shot HARQ feedback triggered by DCI 1_2) should be supported per Feature Set Per Component Carrier (FSPC). With regards to 25-5 - PHY layer priority for One-shot HARQ feedback - the prerequisites are: - 10-16 (support for 1-shot HARQ-ACK feedback) and - 11- 4 (2 HARQ-ACK codebooks with up to 1 sub-slot based HARQ-ACK codebook). Therefore the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 6: Feature 25- 5 (PHY layer priority for One-shot HARQ feedback) should be supported per Feature Set Per Component Carrier (FSPC). With regards to 25-6 - Enhanced Type 3 HARQ-ACK feedback - the prerequisites are: - 10-16 (support for 1-shot HARQ-ACK feedback). Therefore, the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 7: Feature 25- 6 (Enhanced Type 3 HARQ-ACK feedback) should be supported per Feature Set Per Component Carrier (FSPC). With regards to 25-7 - Triggered HARQ-ACK codebook retransmission - the feature should be supported per band. Proposal 8: Feature 25-7 (Triggered HARQ-ACK codebook retransmission) should be supported per Feature Set Per band. 25. NR_IIOT_URLLC_enh; 25-4; One-shot HARQ ACK feedback triggered by DCI format 1_2; 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value; 10-16 11-1; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-5; PHY priority handling for one-shot HARQ ACK feedback; Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI; 10-16 11-4; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback; 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 10-16; Yes; N/A; Per UE Per FSPC; No; No; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-7; Triggered HARQ-ACK codebook re-transmission; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); Yes; N/A; Per UE Per Band; No; No; N/A; Optional with capability signaling. |
| [11] | OPPO | FG 25-4~25-6: Type 3 and e-Type 3 HARQ-ACK CB Separate capability for different indication method for selection of one e-Type 3 CB In last RAN1 meeting, the following agreement was achieved to indicate one e-Type 3 HARQ-ACK CB from more than one (M>1) configured e-Type 3 HARQ-ACK CBs. Agreement If more than one (M>1) enhanced Type 3 HARQ-ACK codebook is configured and the triggering DCI with the ‘one-shot HARQ-ACK request’ set to ‘1’, - If the FDRA field is not valid, i.e. all “1s” or all “0s” as per Rel-16, then PDSCH is not scheduled: - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE uses the MCS field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the FDRA field is valid, then a PDSCH is scheduled - If a new field with N=ceiling(log2 (M)) bits is configured in the triggering DCI, the UE uses this new field to indicate one of M configured e-Type 3 HARQ-ACK CBs - If the new field is not configured in the triggering DCI, the UE selects the 1st indexed e-Type 3 HARQ-ACK CB in the M configured e-Type 3 HARQ-ACK CBs According to the above agreement, UE may use different method to select of one e-Type 3 HARQ-ACK CB depending on whether the triggering DCI schedules PDSCH when the new field is not configured. From perspective of UE implementation, UE does not need to implement two different behaviors for the same functionality since this would surely increase UE complexity. So, we prefer to separate capability for different indication method to select one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured. Prerequisite feature groups: For FG 25-6, it uses FG 10-16 (Rel-16 One-shot HARQ ACK feedback) as prerequisite feature group. This seems not reasonable to us since UE may choose to implement the Rel-17 enhanced Type 3 HARQ-ACK codebook feedback for partial HARQ-ACK retransmission to increase the system spectrum efficiency, but choose not to support the FG of Rel-16 One-shot HARQ ACK feedback since the feedback is so redundant. The logic is, a Rel-17 feature may be implemented by UE supporting only Rel-15 functionalities, instead of Rel-16. In such a case, it is preferred not to use FG 10-16 as the prerequisite feature group for FG 25-6. Type: To align with their prerequisite feature groups, we support per band for FG 25-4~25-6. Components: The components of FG 25-6 include support of enhanced Type 3 CB triggered by DCI format 1_1 AND DCI format 1_2. However, similar logic as the above, UE may only choose to implement the triggering mechanism in DCI format 1_1 OR DCI format 1_2, so it is preferred to change the wording “DCI format 1_1 and DCI format 1_2” in the components of FG 25-6 to “DCI format 1_1 and/or DCI format 1_2”. Proposal 6: The type of FG 25-4~25-6 is per band. Proposal 7: For FG 25-6: - Separate capability for different indication method for selection of one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured. - Remove FG 10-16 from the prerequisite feature groups for FG 25-6. - For the components part: change the wording “DCI format 1_1 and DCI format 1_2” to “DCI format 1_1 and/or DCI format 1_2”. FG 25-7: Triggered HARQ-ACK codebook re-transmission Components: It was agreed in RAN1 #10bis-e that for one-shot triggering of HARQ-ACK re-transmission on PUCCH, the HARQ-ACK codebook per PHY priority per HARQ-ACK CB type is constructed by appending the retransmission HARQ-ACK CB to the HARQ-ACK CB indicated to be transmitted. To our understanding, this agreement brings additional complexity for UE implementation since UE needs to check if there are indicated HARQ-ACK CB transmission as well as triggered retransmission HARQ-ACK CB, if it is, UE needs to perform HARQ-ACK CB construction as agreed. So we suggest to add description of the HARQ-ACK CB construction in components of FG 25-7. Agreement For one-shot triggering of HARQ-ACK re-transmission on PUCCH, - in case the dynamic Type 2 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 2 HARQ-ACK codebook to be re-transmitted to the Type 2 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority. - in case the semi-static Type 1 HARQ-ACK codebook is configured, the HARQ-ACK codebook per PHY priority on the indicated PUCCH is constructed by appending the Type 1 HARQ-ACK codebook to be re-transmitted to the Type 1 HARQ-ACK codebook of the indicated PUCCH (carrying new, initial HARQ-ACK information) per PHY priority. In addition, similar with FG 25-6, the wording “DCI format 1_1 and DCI format 1_2” in the components of FG 25-7 is changed to “DCI format 1_1 and/or DCI format 1_2”. Moreover, the wording of “PUCCH slot” should be changed to “PUCCH slot/sub-slot” to align with the following RAN1 agreement: Agreement For one-shot HARQ re-transmission on PUCCH, the triggering DCI dynamically indicates a ‘HARQ re-tx offset’ which is used to define the offset in number of PUCCH slots/sub-slots between the triggering DCI and the PUCCH slot/sub-slot of the HARQ-ACK codebook to be re-transmitted. For the triggering DCI received in slot/sub-slot m, indicating the HARQ-ACK re-tx in slot/sub-slot m+k and indicating HARQ_retx_offset, the PUCCH slot/sub-slot n of the HARQ-ACK codebook to be re-transmitted is determined as either: - Alt. 1: n = m - HARQ_retx_offset - Alt. 2: n = m + k - HARQ_retx_offset - FFS: value range of the HARQ-retx_offset Proposal 8: Modify FG 25-7 as: - Add description of HARQ-ACK CB construction in Components of FG 25-7 - Change the wording “DCI format 1_1 and DCI format 1_2” in the components to “DCI format 1_1 and/or DCI format 1_2” - Change the wording “PUCCH slot” in the components to “PUCCH slot/sub-slot” 25. NR_IIOT_URLLC_enh; 25-7; Triggered HARQ-ACK codebook re-transmission; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot/sub-slot based on the triggering information in DCI format 1_1 and/or DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 3. Support HARQ-ACK codebook construction by appending the HARQ-ACK codebook to be re-transmitted to the HARQ-ACK codebook of the indicated PUCCH per PHY priority and per HARQ-ACK CB Type.; Yes; N/A; Per UE; No; No; N/A; Optional with capability signaling. |
| [12] | Intel Corporation | From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-4 One-shot HARQ ACK feedback triggered by DCI format 1_2 - 25-5 PHY priority handling for one-shot HARQ ACK feedback - 25-6 Enhanced type 3 HARQ-ACK codebook feedback - 25-7 Triggered HARQ-ACK codebook re-transmission |
| [13] | Ericsson | From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378) - Adopt “Per UE” type for FG 25-1 to FG 25-10. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-4/5/6/7: - Confirm the FG components (i.e. remove yellow highlight) - Per UE |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether to add following components into FGs 25-6

  - Component for the maximum number of configured CCs for CB generation

    - Support: Huawei, HiSilicon

      - Smaller CB size is achieved by limited to a subset of configured CC

  - Component for the maximum number of configured HARQ processes for CB generation

    - Support: Huawei, HiSilicon

      - Smaller CB size is achieved by limited to a subset of configured HARQ processes

| Company | Comment |
| --- | --- |
| DOCOMO | There is no need to add the components into FG 25-6. There is no such agreement in 8.3.1 agenda to define a maximum number of configured CCs or a maximum number of configured HARQ processes. |
| vivo | No, we support the FG10-16 should be the Prerequisite FG of the FG 25-6, hence we do not see the necessity of the additional components. |
| Apple | Support |
| OPPO | No need to add the FG as pointed by DOCOMO that there is no agreement. |
| New H3C | We have the similar view with DOCOMO. |
| QC | No need to add any limitation on the maximum number of CCs, or no the maximum number of HARQ Processes to be included in a Rel. 17 Type 3 HARQ. There is not such an agreement and from implementation point of view, it is not justified either. Rel. 16 Type 3 HARQ CB, which is a sub-case of Rel. 17 Type 3 HARQ CB, does not have such a limitation either. |
| Samsung | It may be dependent on whether or not to confirm prerequisite feature group of 10-16. For example, if 10-16 is confirmed as prerequisite feature group, the proposal is not necessary since the UE already supports full size of type 3 codebook. |
| LG | It is not necessary to add the components for the maximum number. Moreover, prerequisite FG 10-6 already supports entire CCs and HARQ processes for CB. |
| ZTE | No need to add the two components in question 4-1. |
| Nokia, NSB | Not needed |
| Intel | Not needed, constructing a subset of full type 3 CB does not introduce complexity. Type 3 CB does not have this limitation, thus eType 3 does not have to have it either. |
| Huawei, HiSilicon | We think it is better to add the two components into FG 25-6, though we can accept without it. In our understanding, the main complexity difference for type 3 or enhanced type 3 is the potential overall payload size of the codebook, you can consider type 3 as the most challenging one while enhanced type 3 is kind of relaxation of type 3. Then it would be good to set different capability on different level of payload size for enhanced type 3 codebook also. According to the agreement below, smaller CB size is achieved by either limited to a subset of configured CC or a subset of configured HARQ processes, thus if we want to set different UE capability for different level of payload size, it is straightforward to let UE report the maximum number of configured CC or the maximum number of configured HARQ processes to achieve it. ========== Agreement The following enhanced Type 3 CB types of smaller size are supported, the CB to contain either: - the HARQ processes of a subset of configured CCs, or - a subset of configured HARQ processes (specific to CCs) FFS: additional enh. Type 3 CB types ========== |
| Moderator | Summary of companies view - Component for the maximum number of configured CCs for CB generation - Support: Huawei, HiSilicon, Apple - Smaller CB size is achieved by limited to a subset of configured CC - Not support: DOCOMO, vivo, OPPO, New H3C, QC, Samsung (if FG 10-16 is prerequisite), LG, ZTE, Nokia, NSB, Intel, Ericsson - No agreement in AI 8.3.1 - FG 10-16 (One-shot HARQ ACK feedback) should be prerequisite, which supports entire CCs and HARQ processes for CB - Not justified from implementation point of view - Component for the maximum number of configured HARQ processes for CB generation - Support: Huawei, HiSilicon, Apple - Smaller CB size is achieved by limited to a subset of configured HARQ processes - Not support: DOCOMO, vivo, OPPO, New H3C, QC, Samsung (if FG 10-16 is prerequisite), LG, ZTE, Nokia, NSB, Intel, Ericsson - No agreement in AI 8.3.1 - FG 10-16 should be prerequisite, which supports entire CCs and HARQ processes for CB - Not justified from implementation point of view Given that majority companies don’t think those components are necessary for FG 25-6 assuming FG 10-16 should be prerequisite, following proposal is made [GTW1] High priority proposal 4-1: - FG 10-16 is confirmed as a prerequisite FG of FG 25-6 |
| FL2 | Based on the discussion in the GTW session on Jan 18, all companies seem fine with the proposal in principle but there was a question whether FG 10-16 is applicable to licensed band or not. As kindly shared by Cassio/Nokia over the RAN1 reflector, copied as below, it is captured in TS38.306. --- We had a discussion today on whether FG 10-16 is applicable for licensed spectrum or not. Please note this is already captured in 38.306: oneShotHARQ-feedback-r16 Indicates whether the UE supports one shot HARQ ACK feedback comprised of the following functional components: - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH; - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value. This capability is also applicable to a frequency band that does not require shared spectrum access.; Band; No; N/A; N/A. Hence it should be fine to confirm the agreement. --- Therefore, the same proposal is set to be agreed quickly in the next GTW session. No further comments are necessary if you are fine with the proposal [GTW2] High priority proposal 4-1: - FG 10-16 is confirmed as a prerequisite FG of FG 25-6 |
| Ericsson | Support High priority proposal 4-1 |
| Samsung | Fine with FL’s proposal. |
| Nokia, NSB | Support |
| LG | Support the proposal. |
| ZTE | Support |
| vivo | Support |
| FL3 | Following agreement was made in the GTW session on Jan 20. Agreement - FG 10-16 is confirmed as a prerequisite FG of FG 25-6 |

[FL1] High priority question 4-2:

- Companies are encouraged to provide views on whether/how to separate FG 25-6

  - One for slot based PUCCH and the other for sub-slot based PUCCH

    - Support: Huawei, HiSilicon

      - Similar as FG 25-8 for type 1 HARQ-ACK codebook

  - One for the components 1-3 and the other for the component 4

    - Support: Spreadtrum

      - Enhanced Type 3 HARQ-ACK codebook feedback can be used without two priorities

  - One for DCI format 1_1 and the other for DCI format 1_2

    - Support: Spreadtrum

      - Similar as FGs 10-16 and 25-4 for type 3 HARQ-ACK codebook

  - Separate FG for different indication method for selection of one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured

    - Support: OPPO

      - UE may use different method to select of one e-Type 3 HARQ-ACK CB depending on whether the triggering DCI schedules PDSCH when the new field is not configured

| Company | Comment |
| --- | --- |
| DOCOMO | We don’t think separate capabilities are needed since there is no additional UE complexity for any of above aspects. |
| vivo | We do not see the necessity for separate FG for following: - Separate FG for different indication method for selection of one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured. For others, we are open for discuss. |
| OPPO | Support separate capability for different indication method for selection of one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured. From perspective of UE implementation, UE does not need to implement two different behaviors for the same functionality since this would surely increase UE complexity. We are open to the other capability separation. |
| New H3C | We support two proposals from Spreadtrum. |
| QC | There is no justification on separating components 1-4 (DCI 1_1 or DCI 1_2, or the support of up to 8 Type 3 HARQ CB sizes and the dynamic indication of one of those 8 via DCI) |
| Samsung | For the first proposal, we would like to understand the motivation clearly because the reason seems not same as that of introducing FG 25-8. For the second proposal, it is our understanding that current component 4 is only valid for a UE supporting two HARQ-ACK codebooks. So, there is no need to do separation. For the third proposal, we would like to understand the motivation clearly because the reason seems not same as that of having FGs 10-16 and 25-4. For the fourth proposal, why UE complexity is increased on the case where the proponent pointed out? All related information is included in the same DCI. So, there is no additional impact for a UE to implement both method. |
| LG | For 25-6, UE behaviors are basically same regardless of DCI format and indicated priorities. We don’t think the separation is not necessary. |
| ZTE | No need to separate capabilities. |
| Nokia, NSB | No need to separate FG 25-6. |
| Intel | Seems no separation is necessary. If a UE supports eType3 for DCI 1_1, then for DCI 1_2 there is no additional implementation complexity. |
| Huawei, HiSilicon | We support separate UE capabilities for slot-based PUCCH and sub-slot based PUCCH, i.e. one for slot based PUCCH and the other for sub-slot based PUCCH - As shown in FG 25-8 for type 1 HARQ-ACK codebook, different UE capabilities are set for combination of type 1 HARQ-ACK codebook with slot-based PUCCH and sub-slot based PUCCH, then why not for enhanced type 3 HARQ-ACK codebook? - Combination of enhanced type 3 with slot based PUCCH and sub-slot PUCCH obviously will have different requirements on the UE complexity, e.g. the overall payload bits to be supported per slot are different. |
| Spreadtrum | For proposal 2, enhanced Type 3 HARQ-ACK codebook feedback can be used without two priorities. Component 4 is not an essential combined part for enhanced Type 3 CB. For proposal 3, even if UE support FG 11-4, it may not support FG 11-4b (DL priority indication in DCI with mixed DCI formats). Such as only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP. Thus, for the current component 3, it implies that only mixed DCI formats to schedule enhanced type 3 HARQ-ACK codebook with mixed priority. It requires more complexity for UE implementation. So we propose to separate DCI formats for the single DCI format or mixed DCI formats. |
| Moderator | Summary of companies view - One for slot based PUCCH and the other for sub-slot based PUCCH - Support: Huawei, HiSilicon - Similar as FG 25-8 for type 1 HARQ-ACK codebook - Not support: DOCOMO, LG, ZTE, Nokia, NSB, Intel - no additional UE complexity - Open to discuss: vivo, OPPO - Need clarification: Samsung - reason seems not same as that of introducing FG 25-8 - One for the components 1-3 and the other for the component 4 - Support: Spreadtrum, New H3C - Enhanced Type 3 HARQ-ACK codebook feedback can be used without two priorities - Not support: DOCOMO, QC, Samsung, LG, ZTE, Nokia, NSB, Intel - no additional UE complexity - component 4 is only valid for a UE supporting two HARQ-ACK codebooks - Open to discuss: vivo, OPPO - One for DCI format 1_1 and the other for DCI format 1_2 - Support: Spreadtrum, New H3C - Similar as FGs 10-16 and 25-4 for type 3 HARQ-ACK codebook - Not support: DOCOMO, LG, ZTE, Nokia, NSB, Intel - no additional UE complexity - Open to discuss: vivo, OPPO - Need clarification: Samsung - reason seems not same as that of having FGs 10-16 and 25-4 - Separate FG for different indication method for selection of one e-Type 3 CB from the configured more than one e-Type 3 CBs when the new field is not configured - Support: OPPO - UE may use different method to select of one e-Type 3 HARQ-ACK CB depending on whether the triggering DCI schedules PDSCH when the new field is not configured - Not support: DOCOMO, vivo, Samsung, LG, ZTE, Nokia, NSB, Intel - no additional UE complexity Given that majority companies don’t think FG 25-6 needs to be separated, following proposal is made [GTW1] High priority proposal 4-1: - Components 1 to 4 in FG 25-6 are confirmed |
| FL2 | This proposal could not be discussed sufficiently in the GTW session on Jan 18. Companies are invited to provide view whether proposal 4-1 is acceptable or not. Please also try to address the concern from other sides. [FL2] High priority proposal 4-1: - Components 1 to 4 in FG 25-6 are confirmed |
| Huawei, HiSilicon (updated) | We don’t support proposal 4-1, because we still think separate UE capabilities are needed for slot-based PUCCH and sub-slot based PUCCH, i.e. FG 25-6 for slot based PUCCH and FG 25-6a for sub-slot based PUCCH. An example is given as below: 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback for slot PUCCH; 1. Support feedback of enhanced type 3 HARQ-ACK codebook for slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 10-16; Yes; N/A; Per band; N/A; N/A; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-6a; Enhanced type 3 HARQ-ACK codebook feedback for sub-slot PUCCH; 1. Support feedback of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks for sub-slot PUCCH. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook for sub-slot PUCCH using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 25-6 11-3; Yes; N/A; Per band; N/A; N/A; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,2,4,8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured; Optional with capability signaling. Reasons are as below: - As shown in FG 25-8 for type 1 HARQ-ACK codebook, different UE capabilities are set for combination of type 1 HARQ-ACK codebook with slot-based PUCCH and sub-slot based PUCCH, then similar structure should be applied to enhanced type 3 HARQ-ACK codebook also. - Note that in Rel-16 only type 2 HARQ-ACK codebook is supported for sub-slot PUCCH, which means that if UE reports the support of sub-slot PUCCH, then it means it supports sub-slot PUCCH with type 2 HARQ-ACK codebook, otherwise only slot-based PUCCH with type 2 HARQ-ACK codebook is supported, which implicitly means that there is separate UE capabilities for the support of type 2 HARQ-ACK codebook with slot based PUCCH and sub-slot based PUCCH also. - Combination of enhanced type 3 with slot based PUCCH and sub-slot PUCCH obviously will have different requirements on the UE complexity, at least from the overall payload bits to be processed per slot perspective. For slot-based PUCCH, only one PUCCH with one enhanced type 3 HARQ-ACK codebook needs to be processed in each slot, if we don't consider the case of two simultaneous HARQ-ACK codebook introduced in Rel-16. While for sub-slot based PUCCH, especially for 2-symbol sub-slot configuration, then UE needs to process for 7 PUCCHs in a slot, the overall payload bits to be treated is 7 times of that for slot-based PUCCH. So it should be clear that different UE capabilities are required for slot-based PUCCH case and sub-slot based PUCCH case. |
| Intel | We are fine with current FL2 proposal. |
| Ericsson | Support High priority proposal 4-1. We still don’t think it’s necessary to split 25-6. The UE behavior is similar for slot-based and subslot-based PUCCH. - FG 25-8 does not justify splitting 25-6. Type-1 HARQ-ACK codebook has slot-based based vs subslot-based is due to the artifact that Rel-16 ran out of time and didn’t converge on sub-slot based Type-1 CB. - If the UE implementation is concerned with more payload bits in a slot, then the UE can indicate ‘no’ to 25-6 when indicating support of the subslot based HARQ-ACK (11-3). |
| Spreadtrum | One for the components 1-3 and the other for the component 4: In component 4, it requires to mix HP and LP HARQ-ACK to be mixed in one codebook, which is quite different from the HARQ-ACK codebook generation in Rel-15/16, because one codebook can only can same priority HARQ-ACK bits, never mixed them together, especially according to HARQ process. In Rel-17, when support intra-UE multiplexing, HP HARQ-ACK codebook and LP codebook bits never concatenate together into one CB, we only support separate coding for them. Before coding, the original bits always separate. So for enhanced Type 3 HARQ-ACK CB, it require additional design for CB generation, it certainly has additional requirements for UE behaviours. One for DCI format 1_1 and the other for DCI format 1_2. This proposal is also considering the mixed operation of HP and LP indicated by both DCI format 1_1 and 1_2 (11-4b), or only support one DCI format 1_1 or 1_2 (11-4). Clearly, FG 11-4b is more complex, when there are two DCI formats have priority indicator fields. There are mixed HP and LP from different DCI formats. We also fine to add this as a case for the new FG of different priority of e-type 3 HARQ-ACK CB. Such as : 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback with single priority; 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 10-16; Yes; N/A; Per UE band; No; No; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured; Optional with capability signaling. 25-6a; Enhanced type 3 HARQ-ACK codebook feedback with two priorities; Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in one triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4), or two triggering DCI formats ((for a UE supporting 11-4b)); 10-16; Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. For easy reference, here is 11-4b and 11-4: 11-4b; DL priority indication in DCI with mixed DCI formats; - Support of priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP; 11-1, 11-4. 11-4 Component 4: Supports a DCI format (from the formats /1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP |
| DOCOMO | Support the FL2 proposal. Regarding the separated capabilities for slot based and sub-slot based PUCCH, we still don’t think there is much additional UE complexity but we could be fine to introduce separate capabilities if majority companies think it is problematic for implementation. Regarding the two motivations raised by Huawei, we think separate capability for sub-slot based PUCCH for Type 1 HARQ-ACK CB as FG25-8 is motivated by different HARQ-ACK CB construction rule for sub-slot based and slot based type 1 HARQ-ACK CB. However, the (enhanced) type 3 HARQ-ACK CB construction rule is common for (enhanced) type 3 HARQ-ACK CB transmitted on sub-slot or slot based PUCCH resource. On the other hand, regarding payload size issue, it would be true that sub-slot based PUCCH leads to some additional UE complexity compared to slot-based PUCCH. |
| Samsung | We are fine with FL’s proposal. Having said that, we share the concern by Huawei regarding sub-slot/slot because sub-slot configuration would require more PUCCH transmission in a slot. So, we are open for this aspect. |
| Nokia, NSB | We support FL2 proposal. |
| LG | We support the proposal . |
| ZTE | Support FL2 proposal |
| vivo | Support FL2 proposal |
| Moderator | Summary of companies view - Support: Intel, E///, DCM, Samsungm Nokia/NSB, LG, ZTE, vivo - Not support - Separate slot-based PUCCH and sub-slot based PUCCH: HW/HiSi, [Samsung], Apple - Separate component 4: SPRD - Separate DCI 1_1 and DCI 1_2: Given majority companies support proposal 4-1, the same proposal is set for GTW session [GTW2] High priority proposal 4-2: - Components 1 to 4 in FG 25-6 are confirmed |
| FL3 | This proposal was discussed in the GTW session on Jan 20 but no consensus was achieved. One compromised proposal was made from HW/HiSi that the capabilities for slot-based and sub-slot based capabilities are not separated from FG 25-6 but adding FGs for the number of PUCCH in a sub-slot for eType 3 HARQ-ACK CB, similar to Rel-16. Companies are invited to further provides view on whether either of original or compromised proposal is acceptable or not. Please also try to address the concern from other side [FL3] High priority proposal 4-2: - Components 1 to 4 in FG 25-6 are confirmed [FL3] High priority compromised proposal 4-2’: - Components 1 to 4 in FG 25-6 are confirmed - Support FGs for the number of PUCCH in a sub-slot for eType 3 HARQ-ACK CB, FFS details |
| vivo | Our first preference is High priority proposal 4-2. But we are open for High priority compromised proposal 4-2’. One clarification for proposal 4-2’ is to further discuss to add one FG for the number of PUCCH in a sub-slot for eType 3 HARQ-ACK CB, right? [FL] Thanks! Will be fixed |
| DOCOMO | Either original proposal 4-2 or compromised proposal 4-2’ is acceptable to us. |
| OPPO | We can accept both proposal 4-2 and 4-2’. |
| Huawei, HiSilicon | Thank you very much for the great effort from Moderator. Our suggested compromised solution is to add a component (e.g. component 5) as below, if we don’t introduce separate capabilities for slot-based PUCCH and sub-slot based PUCCH for enhanced type 3 HARQ-ACK codebook: - Supported maximum number of actual PUCCH transmissions for enhanced type 3 HARQ-ACK codebook feedback within a slot Candidate values for the component is: {1, 2, 3, 4, 5, 6, 7} As we explained during the GTW: 1. The UE cannot just reply on saying NO to FG 11-3 (i.e. sub-slot PUCCH) to separate the capabilities for sub-slot PUCCH with enhanced type 3 HARQ-ACK codebook, because UE may still want to support sub-slot based PUCCH with type 2 HARQ-ACK codebook. Our thinking is that the report support of FG 11-3 would mean that UE support the combination of sub-slot PUCCH with type 2 HARQ-ACK codebook, then if UE further support sub-slot PUCCH with enhanced type 3 CB need some separate capability reporting, or as the compromised solution to rely on the newly added component to control the potential number of PUCCHs to be transmitted for type 3 in a slot. 2. FG 11-3 itself, we don’t have the component to report the supported maximum number of actual PUCCH transmission in a slot, therefore we need to add the component to 25-6 here. Based on the above analysis, we would suggest to modify proposal 4-2’ as below: [FL3] High priority compromised proposal 4-2’: - Components 1 to 4 in FG 25-6 are confirmed - Add a component (e.g. component 5) to FG 25-6 as below: - Supported maximum number of actual PUCCH transmissions for enhanced type 3 HARQ-ACK codebook feedback within a slot Candidate values for the component is: {1, 2, 3, 4, 5, 6, 7} |
| Nokia, NSB | We support 4-2. |
| LG | We prefer original proposal but are open to discuss both. We would like to clarify some points. Firstly, ‘what the maximum number of PUCCH transmission for enhanced type-3 means’ should be clarified. We have some of interpretation on the wording: If UE report the maximum number as X, - gNB should schedule PUCCHs less than X in every slots. - gNB should schedule PUCCHs less than X in a slot where enhance type-3 indicated to be transmitted. - gNB should schedule less than two of enh.type-3 codebook in every slots. On the other hand, though we don’t have the restriction on FG 11-3 itself, but we have similar restriction on FG 11-4, at least not to increase the PUCCH transmission when UE is configured with different priorities. How they can work jointly and does it really additionally necessary even if enhance type-3 codebook wouldn’t increase the number of PUCCH transmission in a slot? In addition, we think FG 10-16 requires more capable UE, than 25-6 in some aspect. And FG 10-16 currently works with FG 11-3 without such restriction. For our understanding, FG 25-6 would means simplified type-3 HARQ-ACK procedure, rather than requiring additional complexity. We are wondering which parts of FG 25-6 requires the restriction comparing to FG 10-16. |
| Spreadtrum | We are fine to support FL’s proposal, and also fine with HW’s update. Thank you for your great effort. In addition, minor change as FL proposed for FG25-7 can be adopted for FG25-6 too. The component 1 is changed as: 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and/or DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) A clarify in the Note can be: “For component 4, PHY priority indication in two triggering DCI formats is only supported if UE reports 11-4b”, to make it more clear. |
| QC 2 | For finer UE type separation, it is reasonable to support separation of the feature 25-6 per PUCCH slot or sub-slot configuration. The effort to find a compromise is appreciated, but the compromised proposal does not target the source differentiating PUCCH slot to sub-slot configuration. Good to clarify in the compromised proposal that the higher than 1 numbers of maximum Rel. 17 Type 3 HARQ CBs, e.g. 2-7, applies for sub-slot based configuration. |
| Ericsson | Support proposal 4-2 |
| Moderator | Summary of companies view - proposal 4-2: vivo, DCM, Nokia/NSB, LG, SPRD, E/// - compromised proposal 4-2: vivo, DCM, [LG], SPRD - compromised proposal 4-2’: HW/HiSi, [LG], SPRD Given a number show flexibility to accept compromised proposal 4-2/4-2’, 4-2’ proposed by HW/HiSi is set with the following update. Also, clarification is necessary for the comment from LG. - Components 1/3 are updated based on the comment from SPRD, while it would need further discussion similar to proposal 4-4b - A note is added based on the comment from SPRD - Clarification for candidate values for component 5 is added based on the comment from QC [GTW3] High priority compromised proposal 4-2’: - FG 25-6 is updated as follows 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback; 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot; 10-16; Yes; N/A; Per UE; No; No; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. The values higher than 1 can be applied to sub-slot based configuration only.; Optional with capability signaling. |
| Spreadtrum | We support the compromised proposal 4-2’. |
| Moderator | Following agreement was made in the GTW session on Jan 24 Agreement - FG 25-6 is updated as follows 25. NR_IIOT_URLLC_enh; 25-6; Enhanced type 3 HARQ-ACK codebook feedback; 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot; 10-16; Yes; N/A; Per UE; No; No; N/A; For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. The values higher than 1 can be applied to sub-slot based configuration only.; Optional with capability signaling. |

[FL1] High priority question 4-4:

- Companies are encouraged to provide views on whether/how to separate FG 25-7

  - Separate FG per DCI format

    - Support: Spreadtrum

  - Separate component 2 as another FG

    - Support: Spreadtrum

      - HARQ-ACK re-transmission can be used without two priorities

| Company | Comment |
| --- | --- |
| DOCOMO | We don’t think separate capabilities are needed. It seems DCI format and priority handling does not change UE complexity a lot for FG 25-17. |
| vivo | Similar handling as FG25-6 can be used for FG25-7 on whether to split the FG for different priorities and different DCI formats. We are open for discussion. |
| OPPO | We are open to the above capability separation. |
| New H3C | We support this proposal. |
| QC | No need for separate capabilities. |
| Samsung | For the first proposal, we would like to understand motivation first. For the second proposal, it is our understanding that current component 2 is only valid for a UE supporting two HARQ-ACK codebooks.. |
| LG | For 25-6, UE behaviors are basically same regardless of indicated priorities. We don’t think the separation is not necessary at least for DCI format. We are fine with the separation based on the component 2. |
| ZTE | Open to separate component 2 as another FG. |
| Nokia, NSB | No need to separate FG 25-7. |
| Intel | Seems no separation is necessary. If a UE supports Triggered HARQ-ACK codebook for DCI 1_1, then for DCI 1_2 there is no additional implementation complexity. |
| Spreadtrum | For proposal 1, HARQ-ACK re-transmission can be used without two priorities. Component 2 is not an essential combined part. For proposal 2, even if UE support FG 11-4, it may not support FG 11-4b (DL priority indication in DCI with mixed DCI formats). Such as only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP. Thus, for the current component 1, it implies that only mixed DCI formats to schedule HARQ-ACK re-transmission with mixed priority. It requires more complexity for UE implementation. So we propose to separate DCI formats for the single DCI format or mixed DCI formats. |
| Moderator | Summary of companies view - Separate FG per DCI format - Support: Spreadtrum, New H3C - Not support: DOCOMO, QC, LG, Nokia, NSB, Intel - DCI format handling does not change UE complexity a lot - Open to discuss: vivo, OPPO - Similar handling as FG 25-6 - Need clarification of the motivation: Samsung - Separate component 2 as another FG - Support: Spreadtrum, New H3C, LG - HARQ-ACK re-transmission can be used without two priorities - Not support: DOCOMO, QC, Samsung, Nokia, NSB, Intel - priority handling does not change UE complexity a lot - current component 2 is only valid for a UE supporting two HARQ-ACK codebooks. - Open to discuss: vivo, OPPO, ZTE - Similar handling as FG 25-6 Given that there is no majority view, further discussion in GTW session would be necessary |
| FL2 | This proposal could not be discussed in the GTW session on Jan 18. Companies are invited to try to address the concern from other sides. |
| Ericsson | Do not support the proposals to split FG 25-6. For this FG, UE implementation complexity is not affected by DCI format or PHY priority. |
| Spreadtrum | - Separate component 2 as another FG, and Separate FG per DCI format For FG 27-7 HARQ-ACK re-tx, in case of HARQ process collision, there two Alts for certain HARQ process IDs of the requested HARQ CB to be retransmitted is replaced by new HARQ bits, new content or original bits. Clearly, if the priorities of the certain HARQ process changed in this period, UE have to operate accordingly. In addition, as same as FG 27-6, Considering the mixed operation of HP and LP indicated by both DCI format 1_1 and 1_2 (11-4b), or only support one DCI format 1_1 or 1_2 (11-4). We also fine to add this as a case for the new FG of different priority of HARQ-ACK re-tx. 25. NR_IIOT_URLLC_enh; 25-7; Triggered HARQ-ACK codebook re-transmission with single priority; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. 25-7a; Triggered HARQ-ACK codebook re-transmission with two priority; Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration based on PHY priority indication in one triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4), or two triggering DCI formats ((for a UE supporting 11-4b)); Yes; N/A; Per band; No; No; N/A; Optional with capability signaling. In this meeting, early triggering is confirmed, and a new FG was agreed, so the following FG can be discussed too. 25. NR_IIOT_URLLC_enh; 25-7b; Early triggering of HARQ-ACK codebook re-transmission; 1. the triggering is supported before the initial PUCCH transmission slot; Yes; N/A; Optional with capability signaling. Agreement RAN1 confirms the following RAN1#107-e working assumption: Working Assumption For one-shot triggering of HARQ re-transmission, in addition to one-shot triggering of HARQ re-transmission after the initial PUCCH transmission slot, the triggering is supported before the initial PUCCH transmission slot - Re-transmission triggering does not change processing for the initial PUCCH transmission (i.e., HARQ multiplexing / dropping / transmission) - The UE expects the PUCCH carrying the HARQ-ACK re-transmission to be scheduled in a slot/sub-slot after the initial PUCCH transmission slot/sub-slot. - The support for the triggering before the initial PUCCH transmission slot is subject to separate UE capability indication. |
| vivo | Similar as FG25-6, so, no need of separation. |
| Moderator | Summary of companies view - Separate FG per DCI format - Support: Spreadtrum, New H3C - Not support: DOCOMO, QC, LG, Nokia, NSB, Intel, E///, vivo - DCI format handling does not change UE complexity a lot - Open to discuss: vivo, OPPO - Similar handling as FG 25-6 - Need clarification of the motivation: Samsung - Separate component 2 as another FG - Support: Spreadtrum, New H3C, LG - HARQ-ACK re-transmission can be used without two priorities - Not support: DOCOMO, QC, Samsung, Nokia, NSB, Intel, E///, vivo - priority handling does not change UE complexity a lot - current component 2 is only valid for a UE supporting two HARQ-ACK codebooks. - Open to discuss: vivo, OPPO, ZTE - Similar handling as FG 25-6 - Separate FG for the support for the triggering before the initial PUCCH transmission slot - Support: SPRD - Based on the agreement Given majority companies prefer not to separate FG per DCI format or separate component 2, following proposals are made [GTW2] High priority proposal 4-4a: - FG 25-7 is not separated per DCI format [GTW2] High priority proposal 4-4b: - Component 2 in FG 25-7 is not separated as another FG In addition, Spreadtrum proposed to separate an FG for the support for the triggering before the initial PUCCH transmission slot based on the agreement in this RAN1 meeting. Therefore, following proposal is made [GTW2] High priority proposal 4-4c: - Add an FG for the support for the triggering before the initial PUCCH transmission slot as follows 25. NR_IIOT_URLLC_enh; 25-7a; HARQ-ACK codebook re-transmission triggered before the initial PUCCH transmission slot; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) before the initial PUCCH transmission slot 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 25-7; Yes; N/A; Per UE; No; No; N/A; Optional with capability signaling. |
| FL3 | These proposals could not be discussed in the GTW session on Jan 20. Companies are invited to provide view whether these proposals are acceptable not. Please also try to address the concern from other side. [FL3] High priority proposal 4-4a: - FG 25-7 is not separated per DCI format [FL3] High priority proposal 4-4b: - Component 2 in FG 25-7 is not separated as another FG [FL3] High priority proposal 4-4c: - Add an FG for the support for the triggering before the initial PUCCH transmission slot as follows 25. NR_IIOT_URLLC_enh; 25-7a; HARQ-ACK codebook re-transmission triggered before the initial PUCCH transmission slot; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) before the initial PUCCH transmission slot 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4); 25-7; Yes; N/A; Per UE; No; No; N/A; Optional with capability signaling. |
| vivo | We support above High priority proposal 4-4a and High priority proposal 4-4b. For High priority proposal 4-4c, based on AI 8.3.1 HARQ in GTW session, it related to the introducing the UE capability reporting on the UE supported value of the minimum value and maximum value for HARQ_retx_offset. We are open for it. |
| DOCOMO | We support proposal 4.4-a and proposal 4.4-b but we would suggest the following modification to solve the companies’ concern. For component 1, regarding Spreadtrum’s concern on mixed priority support, can we make small updates (“/or” is added) to address Spreadtrum’s concern? Component 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and/or DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) It is still not very clear to us what the potential UE complexity is due to component 2. It would be appreciated if proponents could elaborate more. We think FG separation is not needed for component 2. We are also fine to add a note that “component 2 is only valid for a UE supporting two HARQ-ACK codebooks”, as commented by Samsung. We are fine with Proposal 4.4-c. |
| ZTE | Support proposal 4-4a and 4-4b, the revision from DOCOMO is also fine. For proposal 4-4c, as the value range of offset are under discussed and not determined, it is better to suspend the proposal now. |
| Huawei, HiSilicon | 1. We are fine with proposal 4-4a. As to the modification from DOCOMO on component 1 to address Spreadtrum concern, it may not work because it is not clear whether the UE support DCI format 1_1, DCI format 1_2 or DCI format 1_1 plus DCI format 1_2. Or you mean it will allow UE to report which DCI format (s) to support? 2. We are fine with proposal 4-4b also. But also open to hear more motivations from Spreadtrum. 3. For proposal 4-4c, as commented by vivo, we may need to wait for more progress in agenda 8.3.1 first on the supported value of the minimum value and maximum value for HARQ_retx_offset. |
| Nokia, NSB | We support 4-4a and 4-4b. We do not support with the revision from DOCOMO above, as gNB would not know which DCI formats can be used for the UE in practice. Do not support current 4-4c. We would like to second the comment from vivo above. There seems to be the intention by some companies to make the HARQ-ACK offset range the UE supports a UE capability. Meaning, this would include the early triggering as a UE supporting earlier triggering would only indicate a positive range. So it would therefore (for companies suggesting such a range to be a UE capability), be better to add another (third) component into 25-7 and then have the min & max value of the value range based on UE capability signaling. |
| LG | We are fine with proposal 4-4a/b also with DOCOMO’s modification. For proposal 4-4c, we share similar view to vivo. It would be safer to wait discussion in AI 8.3.1 |
| Spreadtrum | We are fine with proposal 4-4a/b also support DOCOMO’s modification. Thank you for your great effort. Same as FG 25-6, would you clarify in the Note that: “For component 2, PHY priority indication in two triggering DCI formats is only supported if UE reports 11-4b For proposal 4-4c, we prefer to keep it, and keep the component opens, waiting more progress in AI 8.3.1. |
| QC | Support proposals 4-4a and 4-4b and 4-4c. |
| Ericsson | Support Proposal 4-4a, 4-4b. Do not support Proposal 4-4c. |
| Moderator | Summary of companies view - 4-4a - Support: vivo, DCM, ZTE, HW/HiSi, Nokia/NSB, LG, E///, QC - “DCI format 1_1 and DCI format 1_2” is revised to “DCI format 1_1 and/or DCI format 1_2” - it is not clear whether the UE support DCI format 1_1, DCI format 1_2 or DCI format 1_1 plus DCI format 1_2 - 4-4b - Support: vivo, DCM, ZTE, HW/HiSi, Nokia/NSB, LG, E///, QC - Add a note: “For component 2, PHY priority indication in two triggering DCI formats is only supported if UE reports 11-4b”: SPRD - 4-4c - Open: vivo - Support: DCM, QC - Not support: E/// - Add a component in FG 25-7 for supported value of the minimum value and maximum value for HARQ_retx_offset: Nokia/NSB - Wait for the progress in AI 8.3.1: ZTE, HW/HiSi, LG Given above, proposal 4-4a and 4-4b are updated as follows. Proposal 4-4c will be further discussed after some progress is made in AI 8.3.1 [GTW3] High priority proposal 4-4a: - FG 25-7 is not separated per DCI format - Component 1 is revised as “Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and/or DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1)” [GTW3] High priority proposal 4-4b: - Component 2 in FG 25-7 is not separated as another FG - Add a note: “For component 2, PHY priority indication in two triggering DCI formats is only supported if UE reports 11-4b” |
| Spreadtrum | We support the proposal 4-4a and 4-4b. |
| Moderator | Following agreements were made in the GTW session on Jan 24 Agreement - FG 25-7 is not separated per DCI format Agreement - Component 2 in FG 25-7 is not separated as another FG |

Medium priority question 4-5:

- Companies are encouraged to provide views on whether the type of FGs 25-4 to 25-7 should be per UE or per band or per FSPC

  - FG 25-4

    - Per UE: ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per band: vivo, Spreadtrum, OPPO

      - As prerequisite FGs

    - Per FSPC: Qualcomm

  - FG 25-5

    - Per UE: ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per band: vivo, OPPO

      - As prerequisite FG (10-16)

    - Per FS: Spreadtrum

      - As prerequisite FG (11-4)

    - Per FSPC: Qualcomm

  - FG 25-6

    - Per UE: ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per band: Huawei, HiSilicon, vivo, Spreadtrum, OPPO

      - As prerequisite FGs

    - Per FSPC: Qualcomm

  - FG 25-7

    - Per UE: Huawei, HiSilicon, ZTE, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

      - Complexity does not scale with bands

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per band: Spreadtrum, Qualcomm

      - Similar as enhanced Type 3 HARQ-ACK CB, HARQ-ACK re-transmission also need to be per band

    - Per FS: vivo

      - As prerequisite FGs

| Company | Comment |
| --- | --- |
| vivo | The type of FGs 25-4 to 25-7 should be per band. |
| New H3C | The type of FGs 25-4 to 25-7 should be per UE. |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| Nokia, NSB | Per UE Response to Qualcomm’s comments on section 2. |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Huawei, HiSilicon | Per band at least for 25-4/5/6 to align with the prerequisite. For 25-7, ok to do it per UE. |
| Ericsson | Per UE |

Low priority question 4-6:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 25-4 to 25-7

  - FG 25-4

    - FG 10-16 and 11-1: Samsung, DOCOMO, Spreadtrum

  - FG 25-5

    - FG 10-16 and 11-4: Samsung, DOCOMO, Spreadtrum

  - FG 25-6

    - FG 10-16 should be removed: Huawei, HiSilicon, OPPO

      - Enhanced type 3 HARQ-ACK codebook feedback is to achieve smaller size compared to Rel-16 type 3 HARQ-ACK codebook

    - FG 10-16 should be kept: DOCOMO, Spreadtrum

    - FG 11-1 and 11-4 should be added: ZTE, DOCOMO

    - FG 25-4 and 25-5: Samsung

  - FG 25-7

    - FG 11-1 and 11-4 should be added: vivo, ZTE, DOCOMO

| Company | Comment |
| --- | --- |
| Nokia, NSB | We are fine with the latest list of prerequisite FGs (no need for changes). Few comments still here on proposal by other companies: - 25-6: - neither 11-1, 11-4, 25-4 nor 25-5 can be added, as the enhanced Type 3 CB can be operated also without the support for DCI format 1_2 or PHY priority operation. Therefore, there is no need to have these a FGs - 11-16 should be kept, as the R16 type codebook can be also configured as one enhanced Type 3 CB. Therefore, R16 Type 3 CB needs to be supported as well. 25-7: 11-1 and 11-4 should not be added, as HARQ-ACK re-transmission can also be supported without PHY priority operation and the need to support DCI format 1_2. |
| Ericsson | Support the existing prerequisites for 25-4 to 25-7. No need to change |

Low priority question 4-7:

- Companies are encouraged to provide views on whether/how to revise components in FGs 25-4 to 25-7

  - FG 25-4

    - Component 1 can be revised as “Support feedback of type 3 HARQ-ACK codebook with full size or reduced size with either one of low or high priority, triggered by a DCI 1_2 scheduling a PDSCH”: DOCOMO

    - Component 2 can be revised as “Support feedback of type 3 HARQ-ACK codebook with full size or reduced size with either one of low or high priority, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value”: DOCOMO

  - FG 25-5

    - Component can be revised as “Support transmission of type 3 HARQ-ACK codebook with full size or reduced size using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI”: DOCOMO

  - FG 25-6

    - Component 1 can be revised as “Support feedback of enhanced type 3 HARQ-ACK codebook with or without priority indicator in the triggering DCI, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1)”: DOCOMO

    - Add new sub-components for components 3, i.e. with PDSCH scheduling, without PDSCH scheduling and Enhanced Type 3 codebook indicator field in DCI: Spreadtrum

    - Change the wording “DCI format 1_1 and DCI format 1_2” to “DCI format 1_1 and/or DCI format 1_2”: OPPO

  - FG 25-7

    - ‘sub-slot’ should be added to the first component: vivo

    - Component 1 can be revised as “Support HARQ-ACK re-transmission from an earlier PUCCH slot of the cancelled HARQ-ACK based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1)”: ZTE, DOCOMO

    - Change the wording “DCI format 1_1 and DCI format 1_2” to “DCI format 1_1 and/or DCI format 1_2”.: OPPO

    - Change the wording “PUCCH slot” in the components to “PUCCH slot/sub-slot”: OPPO

    - Add a component as “Support HARQ-ACK codebook construction by appending the HARQ-ACK codebook to be re-transmitted to the HARQ-ACK codebook of the indicated PUCCH per PHY priority and per HARQ-ACK CB Type”

| Company | Comment |
| --- | --- |
| Nokia, NSB | Do not agree with the proposed changes to 25-4 & 25-5: There could be also UEs not supporting reduced size (i.e. only R16 Type 3 CB operation) and support the related PHY priority handling & DCI format 1_2. 25-6: We do not see a needed change here, as - We do not agree with the suggested addition to component 1. The PHY priority handling is a different issue and not really integral part here. - We do not see a need for additional sub-components proposed for component 3 - We do not agree with the change to ‘DCI format 1 and/or DCI format 2’, as DCI format 1 should always be supported. Only DCI format 1 is conditional 25-7: - ‘sub-slot’ could be added as proposed by vivo & OPPO, but we don’t see this as essential - We do not agree with the proposed changes by ZTE & DOCOMO, as the re-transmission can also be triggered in case the HARQ-ACK codebook had not been correctly received. We do not have any such restriction agreed. - We do not agree with the proposed change by OPPO to ‘DCI format 1 and/or DCI format 2’, as DCI format 1 should always be supported. Only DCI format 1 is conditional - We could be fine with the additional component on the CB appending, but don’t see this as essential as this is integral part of the operation. |
| Ericsson | We are fine with not changing the component description |

Low priority question 4-8:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 25-4 to 25-7 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

In [1], FG 25-8 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-8: Change “Per UE” to “Per FS” to align with the prerequisite. |
| --- | --- | --- |
| [5] | ZTE | Index 25-8: The type definition from moderator on index 25-8 is “Per UE”. But we think “Per FS” is better for the feature group as the prerequisites of “Per sub-slot URLLC feature in Rel-16” is per FS. Proposal 5: The type of FGs 25-8 should be kept as per FS. |
| [7] | Samsung | Confirm all yellow highlight parts |
| [8] | NTT DOCOMO, INC. | - FG 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH - Type should be per UE - FGs 4-11 and 11-3 can be kept as prerequisite feature groups. |
| [9] | Spreadtrum Communications | - Prerequisite FGs: 4-11 and 11-3 - Type: per FS, to keep consistence with FG 11-3 |
| [10] | Qualcomm Incorporated | With regards to 25-8 - Semi-static HARQ-ACK codebook for sub-slot PUCCH - the prerequisites are: - 4-11 (support for semi-static HARQ-ACK feedback) and - 11-3 (support for more than 1 PUCCH HARQ within a slot). Therefore, the feature should be supported per Feature Set per Component Carrier (FSPC). Proposal 9: Feature 25-8 (Semi-static HARQ-ACK codebook for sub-slot PUCCH) should be supported per Feature Set Per Component Carrier (FSPC). 25. NR_IIOT_URLLC_enh; 25-8; Semi-static HARQ-ACK codebook for sub-slot PUCCH; Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration; 4-11 11-3; Yes; N/A; Per UE Per FSPC; No; No; N/A; Optional with capability signaling. |
| [11] | OPPO | Type: To align with their prerequisite feature groups, we support per FS for FG 25-8. Proposal 9: The type of FG 25-8 is per FS. |
| [12] | Intel Corporation | From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-8 Semi-static HARQ-ACK codebook for sub-slot PUCCH |
| [13] | Ericsson | From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378) - Adopt “Per UE” type for FG 25-1 to FG 25-10. |

Discussion

Medium priority question 5-1:

- Companies are encouraged to provide views on whether the type of FG 25-8 should be per UE or per FS or per FSPC

  - Per UE:Samsung, DOCOMO, Intel, Ericsson

    - Complexity does not scale with bands

    - Not clear that band differentiation would impact the feature and the corresponding testing

    - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

  - Per FS: Huawei, HiSilicon, ZTE, Spreadtrum, OPPO

    - to align with the prerequisite FGs

  - Per FSPC: Qualcomm

| Company | Comment |
| --- | --- |
| vivo | FG 25-8 should be per FS to align with the prerequisite FG |
| Apple | Per FS is preferred |
| New H3C | We support the type of FG 25-8 should be per UE |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| Nokia, NSB | Per UE |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Ericsson | Per UE. The functionalities for the FG are not affected when testing for different bands. The IODT issue is a generic issue on licensed/unlicensed/NTN. It’s nothing specific to the FG. |

Low priority proposal 5-2:

- Prerequisite feature groups for FG 25-8 are confirmed as FGs 4-11 and 11-3

  - Support: Samsung, DOCOMO, Spreadtrum

| Company | Comment |
| --- | --- |
| Nokia, NSB | Support the current prerequisite FGs |
| Huawei, HiSilicon | Fine with it. |
| Ericsson | Support |

Low priority question 5-3:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-8 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-9 to 25-10: PUCCH cell switching

In [1], FGs 25-9 to 25-10 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell switching | Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell switching based on dynamic indication | PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No (TDD only) | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-9/25-10: For FFS whether to separate the capability for different numerologies, we think it should be independent UE capability for same numerology and different numerologies to leave more flexibility at the UE side. |
| --- | --- | --- |
| [4] | vivo | - Whether to separate the capability for different numerologies PUCCH cell switching with same or different numerologies may require different UE capabilities. PUCCH cell switching with same numerology should be the baseline and PUCCH cell switching with different numerologies should be deemed as a more advanced UE feature. Supporting separate UE capability provide flexible UE implementation. We suggest split FG 25-9 and FG 5-10 into separate UE capabilities, i.e., the one for the capability for same numerology and the other for different numerologies between switchable carriers. - Applicable cases For FG 25-9, semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI. This should be captured in the corresponding components column. For FG 25-10, according to the agreement, in addition to HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to: - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI For FG 25-10, these applicable cases should be explicitly described in the components. - Type PUCCH cell switching is only used for TDD scenario, the type should be per Feature Set per Band combination. Proposal 5: For FG 25-9 and 25-10, support separate UE capability for different numerologies. The applicable cases should be explicitly described in the components. The type should be Per FSBC. 25.NR_IIOT_URLLC_enh; 25-9; Semi-static PUCCH cell switching with the same numerology; 1.Semi-static PUCCH cell switching with the same numerology using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI; Yes; N/A; Per UEFSBC; No; No; N/A; Optional with capability signaling. 25.NR_IIOT_URLLC_enh; 25-9a; Semi-static PUCCH cell switching with the different numerologies; 1.Semi-static PUCCH cell switching with the same or different numerologies using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI; Yes; N/A; Per UEFSBC; No; No; N/A; Optional with capability signaling. 25.NR_IIOT_URLLC_enh; 25-10; PUCCH cell switching based on dynamic indication with the same numerology; PUCCH cell switching with the same numerology based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies; Yes; N/A; Per UEFSBC; No; No; N/A; Optional with capability signaling. 25.NR_IIOT_URLLC_enh; 25-10a; PUCCH cell switching based on dynamic indication with the different numerologies; PUCCH cell switching with the same or different numerologies based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies; Yes; N/A; Per UEFSBC; No; No; N/A; Optional with capability signaling. |
| [5] | ZTE | Index 25-9 and 25-10: The type definition from moderator on index 25-9 and 25-10 is “Per UE”. But we think “Per BC” is better for the two feature group. For a certain UE, not all the band combination is suitable for PUCCH switching, for example, one UE could switch between carrier 1 and carrier 2, but carrier 3 is not allowed for switching. So the feature of supporting Semi-static PUCCH carrier switching or dynamic PUCCH carrier switching seems more suitable as Per BC definition. Proposal 6: The type of the feature group 25-9 and 25-10 is proposed to change to Per BC. |
| [7] | Samsung | - 25-9: - “Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier” - Separating feature for the case of different numerologies. - Adding “support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier” - 25-10: - Separating feature for the case of different numerologies. - Adding “support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier” - Necessity of considering different TA(Timing Advance) impact for PUCCH cell switching For PUCCH cell switching, it is likely that some UEs are configured to transmit two PUCCHs simultaneously if two cells are included in different TAG (Timing Advance Group). For example, as shown in Figure 1, there are two cells for supporting PUCCH cell switching. If a UE is scheduled to transmit first PUCCH at slot 0 in CC1 and then transmit second PUCCH at slot 1 in CC0, the UE needs to transmit both PUCCH simultaneously in the overlapped time when those cells are configured to different TAG having TA values. So, this issue would increase UE implementation’s complexity since it requires simultaneous PUCCH transmissions in a PUCCH group although this is not intended operation of PUCCH cell switching. Figure 1. PUCCH cell switching in case of different TA In order to reduce UE implementation’s complexity, we would like to propose the following new UE feature. With that, a UE can do overlapped PUCCH transmission as shown in Figure 1 if the UE reports the following UE feature. Otherwise, it should be avoided by gNB scheduling. |
| [8] | NTT DOCOMO, INC. | - FG 25-9: Semi-static PUCCH cell switching - Regarding the FFS whether to separate the capability for different numerologies, we prefer not to separate the capability for different numerologies. There is no additional UE complexity for different numerology case and same numerology case. - Type should be per UE - FG 25-10: PUCCH cell switching based on dynamic indication - Regarding the FFS whether to separate the capability for different numerologies, we prefer not to separate the capability for different numerologies. There is no additional UE complexity for different numerology case and same numerology case. - Type should be per UE |
| [9] | Spreadtrum Communications | - Components: - FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers - Type: FG 25-9 to 25-10: per BC |
| [10] | Qualcomm Incorporated | With regards to the Feature 25-9, the feature is not necessary at FDD. The feature should be supported, for obvious reasons, per Feature Set per Band combination. Proposal 10: Feature 25-9 (semi-static PUCCH cell switch) and 25-10 (dynamic PUCCH cell switch) are for TDD only. Furthermore, they should be per band combination rather than per UE. 25. NR_IIOT_URLLC_enh; 25-9; Semi-static PUCCH cell switching; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE Per BC; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10; PUCCH cell switching based on dynamic indication; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE Per BC; No (TDD only); No; N/A; Optional with capability signaling. |
| [11] | OPPO | The high priority discussion point is whether to separate the capability for different numerologies. To our understanding, if PUCCH cell switching for same numerology and different numerologies are both supported, it should be based on separate UE capability similar with cross-carrier scheduling with same or different numerologies. This is because for PUCCH cell switching with different numerologies, additional UE implementation is needed to handle the timing conversion for different SCS. In addition, it was agreed in last RAN1 meeting that Rel-17 supports PUCCH cell switching between 2 cells, we suppose this should be added to the components of FG 25-9 and FG 25-10. Agreement PUCCH cell switching between 2 cells is supported in Rel-17. Proposal 10: For FG 25-9 and FG 25-10: - Separate the UE capability for same numerology and different numerology; - Add the description of supported cells for PUCCH cell switching to the components of FG 25-9 and 25-10. |
| [12] | Intel Corporation | - 25-9 Semi-static PUCCH cell switching - 25-10 PUCCH cell switching based on dynamic indication - Regarding the FFS whether to separate the capability for different numerologies, we are supportive. As it can be seen in current discussions, the different numerologies require additional handling mechanisms for the cases of SCS reference larger or smaller than SCS of switched cell. |
| [13] | Ericsson | From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378) - Adopt “Per UE” type for FG 25-1 to FG 25-10. It has been discussed last meeting whether each of these features should be split into two capabilities, one for the same numerology between two cells and the other for different numerologies between two cells. From our point of view, such split is not needed. During the discussion of PUCCH cell switching, the possibility of having different numerologies has been a central assumption in design. Hence, the corresponding agreements were inclusive with respect to numerologies being the same or different, and there is no agreement to differentiate the behaviour for the same or different numerologies. Do not split FG 25-9 and FG 25-10 each into two capabilities, one for the same numerology and the other for different numerologies between two cells for cell switching. |
| [14] | MediaTek Inc. | PUCCH cell switching (FG25-9 & FG25-10): PUCCH cell switching feature should have UL CA as Prerequisite FG. Also, the type for FG25-9 & FG25-10 should be per band combination. Given the different required UE implementation complexities, the PUCCH cell switching should be reported separately for the case where the carriers have different numerologies. Index; Feature group; Components; Prerequisite FG; Type; Note; Mandatory/Optional. 25-9; Semi-static PUCCH cell switching; FG6-6; Per BC; Optional. 25-10; PUCCH cell switching based on dynamic indication; FG6-6; Per BC; Optional. - Change the Type of FG25-9 & FG25-10 from “Per UE” to “Per BC”, and add FG6-6 as “Prerequisite FG” for these two FGs. - FG25-9 & FG25-10 are reported separately for the case where the carriers have different numerologies. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-9: - No need for separate capabilities for different numerologies - Per UE - 25-10: - No need for separate capabilities for different numerologies - Per UE |

Discussion

[FL1] High priority proposal 6-1:

- FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers

  - Support: Huawei, HiSilicon, vivo, Samsung, Spreadtrum, OPPO, Intel, MediaTek

    - to leave more flexibility at the UE side

    - PUCCH cell switching with same numerology should be the baseline and PUCCH cell switching with different numerologies should be deemed as a more advanced UE feature

    - for PUCCH cell switching with different numerologies, additional UE implementation is needed to handle the timing conversion for different SCS

  - Not support: DOCOMO, Ericsson, Nokia, NSB

    - no additional UE complexity for different numerology case and same numerology case

    - no agreement to differentiate the behaviour for the same or different numerologies

| Company | Comment |
| --- | --- |
| DOCOMO | As commented in our contribution, we prefer not to separate the capability for different numerologies. There is no additional UE complexity for different numerology case and same numerology case. |
| vivo | As reasons provided above, we support split FGs 25-9 and 25-10 into ones for the capability for same numerology and the others for different numerologies between switchable carriers. |
| OPPO | Support to split the capability for same and different numerology since additional UE behavior is needed for PUCCH cell switching between cells with different numerologies |
| New H3C | We support FL proposal on • FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers |
| QC | Support. UE implementation for PUCCH cell switch with different numerology is definitely more complicated then with same numerology. Flexibility should be allowed at UE to implement PUCCH cell switch with same numerology as baseline firstly and add supporting for different numerologies later. |
| LG | We support the proposal. Though we had removed complex situation from discussion in AI 8.3.1, there are still some UE behavior is necessary to handle different numerologies. |
| Nokia, NSB | No need to separate the capabilities. |
| Intel | Support |
| Huawei, HiSilicon | We think it should be independent UE capability for same numerology and different numerologies, as the reasons summarized in the proposal. |
| Moderator | Summary of companies view - Support: Huawei, HiSilicon, vivo, Samsung, Spreadtrum, OPPO, Intel, MediaTek, New H3C, QC, LG - to leave more flexibility at the UE side - PUCCH cell switching with same numerology should be the baseline and PUCCH cell switching with different numerologies should be deemed as a more advanced UE feature - for PUCCH cell switching with different numerologies, additional UE implementation is needed to handle the timing conversion for different SCS - Not support: DOCOMO, Ericsson, Nokia, NSB - no additional UE complexity for different numerology case and same numerology case - no agreement to differentiate the behaviour for the same or different numerologies Given that majority companies support to split FGs 25-9 and 25-10 into ones for the capability for same numerology and the others for different numerologies between switchable carriers, following proposal is made [GTW1] High priority proposal 6-1: - FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers as follows 25. NR_IIOT_URLLC_enh; 25-9; Semi-static PUCCH cell switching for same numerology between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-9a; Semi-static PUCCH cell switching for different numerologies between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for different numerologies between switchable carriers; 25-9; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10; PUCCH cell switching based on dynamic indication for same numerology between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10a; PUCCH cell switching based on dynamic indication for different numerologies between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for different numerologies between switchable carriers; 25-10; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. |
| FL2 | This proposal could not be discussed in the GTW session on Jan 18. Companies are invited to provide view whether proposal 6-1 is acceptable or not. Please also try to address the concern from other sides. [FL2] High priority proposal 6-1: - FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers as follows 25. NR_IIOT_URLLC_enh; 25-9; Semi-static PUCCH cell switching for same numerology between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-9a; Semi-static PUCCH cell switching for different numerologies between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for different numerologies between switchable carriers; 25-9; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10; PUCCH cell switching based on dynamic indication for same numerology between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10a; PUCCH cell switching based on dynamic indication for different numerologies between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for different numerologies between switchable carriers; 25-10; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. |
| Huawei, HiSilicon (updated) | We support proposal 6-1. |
| Intel | Support proposal 6-1 |
| Ericsson | Do not support splitting 25-9 and 25-10. During the discussion of PUCCH cell switching, the possibility of having different numerologies has been a central assumption in design. Hence, the corresponding agreements were inclusive with respect to numerologies being the same or different, and there is no agreement to differentiate the behaviour for the same or different numerologies. |
| DOCOMO | We can accept the proposal for compromise if majority companies think such splitting is necessary. |
| OPPO | Support proposal 6-1. |
| Nokia, NSB | If the FGs separated for same and different numerologies, is it correct assumption that any combination of different numerologies are then supported by the corresponding FGs? Moreover, it seems that it is not the numerology that defines additional different handling but it is more then PUCCH slot length here. Having two CCs with the same numerology but different slot/sub-slot length (based on the proposed 25-9a/b / 25-10a/b) is having more handling needed whereas e.g. having a 15 kHz CC with 7-OS subslot and 30kHz CC with 14-OS/slot-based PUCCH slots is simpler to operate. So having numerology as a distinguishing point from UE complexity is not really applicable. |
| LG | Support the proposal. |
| vivo | Support proposal 6-1 |
| Moderator | Summary of companies view - Support: Huawei, HiSilicon, vivo, Samsung, Spreadtrum, OPPO, Intel, MediaTek, New H3C, QC, LG, DCM, LG, vivo - to leave more flexibility at the UE side - PUCCH cell switching with same numerology should be the baseline and PUCCH cell switching with different numerologies should be deemed as a more advanced UE feature - for PUCCH cell switching with different numerologies, additional UE implementation is needed to handle the timing conversion for different SCS - Not support: Ericsson, Nokia, NSB - no additional UE complexity for different numerology case and same numerology case - no agreement to differentiate the behaviour for the same or different numerologies Given majority companies support proposal 6-1, the same proposal is set for GTW session [GTW2] High priority proposal 6-1: - FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers as follows 25. NR_IIOT_URLLC_enh; 25-9; Semi-static PUCCH cell switching for same numerology between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-9a; Semi-static PUCCH cell switching for different numerologies between switchable carriers; Semi-static PUCCH cell switching using configured time-domain domain pattern of applicable PUCCH cell / carrier for different numerologies between switchable carriers; 25-9; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10; PUCCH cell switching based on dynamic indication for same numerology between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for same numerology between switchable carriers FFS whether to separate the capability for different numerologies; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-10a; PUCCH cell switching based on dynamic indication for different numerologies between switchable carriers; PUCCH cell switching based on dynamic indication in the DCI scheduling the PUCCH for different numerologies between switchable carriers; 25-10; Yes; N/A; Per UE; No (TDD only); No; N/A; Optional with capability signaling. |
| FL3 | This proposal was discussed in the GTW session on Jan 20 but no consensus was achieved. One compromised proposal was made from HW/HiSi that the capabilities for same numerology and for different numerologies are not separated from FG 25-9/10 but adding a note that if UE supporting FG 25-9/10 also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carriers. Another compromise was made from Nokia that FG 25-9/10 are not separated by same/different numerology but by number of overlapping PUCCH slot between switchable carriers. Companies are invited to further provides view on whether either of original or compromised proposal is acceptable or not. Please also try to address the concern from other side [FL3] High priority proposal 6-1: - FGs 25-9 and 25-10 are split into ones for the capability for same numerology and the others for different numerologies between switchable carriers [FL3] High priority compromised proposal 6-1a: - FGs 25-9 and 25-10 are not split into multiple FGs - Add a note in FG 25-9/10 that if UE supporting FG 25-9/10 also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carriers [FL3] High priority proposal 6-1b: - FGs 25-9 and 25-10 are split into ones for the capability for same number of overlapping PUCCH slots and the others for different number of overlapping PUCCH slots between switchable carriers |
| QC | We don’t support Proposal 6-1a. 6-9/6-9a are Rel-15 UE features for CA and EN-DC with different numerologies. We don’t want to link this Rel-15 UE feature with a Rel-17 UE feature. With this proposal, we don’t know how to indicate a UE support FGs 6-9 and 6-9a, but only support PUCCH cell switch with same numerology? We support the direction suggested by proposal 6-1b. We are open to discuss how to reformulate the proposal using terminology of PUCCH slot/sub-slot. |
| vivo | We support FL High priority proposal 6-1 and would be fine with proposal 6-1a if the complexity for PUCCH carrier switching between different SCSs is similar as cross-CC scheduling with different SCSs. We are not sure whether High priority proposal 6-1b is needed or not, the complexity may be similar for the overlapped PUCCH resources with different length in the same PUCCH carrier or different PUCCH carrier. |
| DOCOMO | Our first preference is FL3 proposal 6-1a. We can also accept Proposal 6-1b for progress. |
| OPPO | Based on the discussion on the GTW, we have a clarification question for proposal 6-1: if proposal 6-1 is adopted, for FG of same numerology, does the capability needs to be further split between same and different PUCCH slot/sub-slot length? |
| ZTE | Agree the intention to split. As the same question with OPPO, maybe 6-1b is more accurate. |
| Huawei, HiSilicon | 1. We are fine with proposal 6-1. 2. We are fine with proposal 6-1a also. In our understanding, if UE already support 6-9 and 6-9a, then it should be able to support PUCCH switching between cells with different numerology. However, without this note, it would mean that if a UE report the supporting of FG 25-9/10, even the UE doesn’t support 6-9 and 6-9a, the UE has to support PUCCH switching between cells with different numerology, which to us is a big step. 3. We are fine with the direction of proposal 6-1b also, and open to discuss the details of these FGs. |
| Nokia, NSB | We have a preference for 6-1a, but if needed we can further consider 6-1b for sake of progress. |
| LG | WE support proposal 6-1 or 6-1a. It seems clear. |
| Ericsson | Support 6-1a |
| Moderator | Summary of companies view - 6-1: vivo, HW/HiSi, LG - 6-1a: vivo, DCM, HW/HiSi, Nokia/NSB, LG, E///, OPPO - 6-1b: QC, DCM, ZTE, HW/HiSi, Nokia/NSB, OPPO Given majority companies are generally fine with 6-1a and/or 6-1b, these proposals are set for GTW session for further discussion [GTW3] High priority compromised proposal 6-1a: - FGs 25-9 and 25-10 are not split into multiple FGs - Add a note in FG 25-9/10 that if UE supporting FG 25-9/10 also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carriers [GTW3] High priority proposal 6-1b: - FGs 25-9 and 25-10 are split into ones for the capability for same number of overlapping PUCCH slots and the others for different number of overlapping PUCCH slots between switchable carriers |
| Moderator | Following compromised proposals were discussed in the GTW session on Jan 24 but no consensus was achieved. Let’s further discuss directly over the RAN1 reflector whether we can converge on either one. High priority compromised proposal 6-1a: - FGs 25-9 and 25-10 are not split into multiple FGs - Add a note in FG 25-9/10 that if UE supporting FG 25-9/10 also supports both FGs 6-9 and 6-9a, the UE support the case of different numerologies between switchable carriers Support: vivo, DCM, HW/HiSi, Nokia/NSB, LG, E///, OPPO High priority alternative proposal 6-1a’: - FGs 25-9 and 25-10 are split into ones for the capability for same length of overlapping PUCCH slots and the others for different length of overlapping PUCCH slots between switchable carriers Support: QC, DCM, ZTE, HW/HiSi, Nokia/NSB, OPPO |
| Moderator | These proposals were discussed over RAN1 reflector but no consensus was achieved. Let’s further discuss in the next RAN1 meeting. |

[FL1] High priority question 6-2:

- Companies are encouraged to provide views on whether to add an FG for PUCCH cell switching with different TAGs

  - Support: Samsung (see above feedback in [7] for detail)

| Company | Comment |
| --- | --- |
| DOCOMO | Not support to add an FG for PUCCH cell switching with different TAGS. There is no agreement that simultaneous PUCCH transmission on different CCs with TAGs is permitted. On the contrary, we have an agreement that overlapping of UCI on different CCs is not expected. Conclusion (@RAN1#107-e) For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell. The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell. |
| vivo | It is preferred to first discuss in RAN1 whether to support the case of PUCCH cell switching (in semi-static and/or dynamic manner) with different TAGs. If such case is supported, we prefer to set separate FGs. |
| OPPO | We do not see the necessity to add the FG due to the conclusion cited by DOCOMO for dynamic switching and the following agreement for semi-static switching: Agreement For PUCCH cell switching based on semi-static operation, adopt Alt. 4, i.e., the UE does not expect a semi-static PUCCH cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot. |
| New H3C | The motivation of this proposal isn’t clear to us and the proponent need clarify ir. |
| Samsung | Motivation is clear. In Rel-15/16, a UE only transmits actual PUCCH at a time in a PUCCH group. We think that this principle should hold even if PUCCH cell switching is considered. However, without any restriction about whether cells for PUCCH cell switching will be same or different TAG group, it is likely that two cells may have different TA value. It means that a UE may transmit two PUCCHs simultaneously in a PUCCH group due to different TA value, as shown in the figure in our contribution. So, the motivation of having the capability is to give some flexibility of UE implementation. Regarding conclusion/agreement cited by DOCOMO/OPPO, our understanding is that it is about logical slot timing, not actual slot timing. However, we are open to limit the case as in same TAG. So, if majority of companies think that this case is only for same TAG, we would like to suggest to add note “Note: this features applies to cells in the same TAG” in 25-9/10 for the clarification without having additional separate UE capability. |
| LG | We think the case where PUCCH transmission are overlapped in different cells is not supported. If it is allowed by current specification, it would be preferred to discuss in RAN1 whether to support or prohibit such case. |
| Nokia, NSB | Not clear if needed |
| Intel | Open to consider, potentially after discussion in AI 8.3.1 |
| Huawei, HiSilicon | This may need some discussion first before making decision on what to do in UE feature here, it is not clear yet whether any additional |
| Moderator | Summary of companies view - Support: Samsung (see above feedback in [7] for detail) - Compromised proposal: to add note “Note: this feature applies to cells in the same TAG” in 25-9/10 for the clarification without having additional separate UE capability - Not support: DOCOMO, OPPO, Nokia, NSB - no agreement that simultaneous PUCCH transmission on different CCs with TAGs is permitted - an agreement that overlapping of UCI on different CCs is not expected - Need further discussion whether to support such a case: vivo, LG, Intel - Need clarification: New H3C Given that proponent suggest another proposal for clarification, following proposal is made [GTW1] High priority proposal 6-2: add note “Note: this feature applies to cells in the same TAG” in 25-9/10 |
| FL2 | This proposal could not be discussed in the GTW session on Jan 18. Companies are invited to provide view whether proposal 6-2 is acceptable or not. Please also try to address the concern from other sides. [FL2] High priority proposal 6-2: - Add note “Note: this feature applies to cells in the same TAG” in FG 25-9/10 |
| Intel | Proposal 6-2 is acceptable |
| Ericsson | We are fine with High priority proposal 6-2 |
| DOCOMO | Not support the FL2 proposal. With the FL2 proposal, it seems that PUCCH cell switching is only applied to cells in the same TAG, and not allowed for cells in different TAGs. Based on current RAN1 agreements, we don’t think such limitation is common understanding by all companies. We share similar view as vivo that RAN1 discussion is needed first. |
| Samsung | Accept the proposal. If multiple TAG might be considered in the feature, we strongly suggest to add what we originally suggest in the first round since it definitely affects UE implementation. |
| OPPO | Proposal 6-2 is acceptable |
| Nokia, NSB | It is not clear if there is a need for such note. It does not say clearly if it applies to cells of different TAG, and it is not good practice to have ambiguous text in UE capabilities. |
| LG | We can accept the proposal. |
| ZTE | We can accept the proposal as the open issues are not very clear after introducing the different TAGs. |
| vivo | We are fine with this proposal |
| Moderator | Summary of companies view - Support: Samsung, Intel, OPPO, LG, ZTE, vivo, QC, HW/HiSi, E///, SPRD, New H3C - Not support: DOCOMO, Nokia/NSB - no agreement that simultaneous PUCCH transmission on different CCs with TAGs is permitted - an agreement that overlapping of UCI on different CCs is not expected - Need further discussion in AI 8.3.1: vivo, DCM Given more companies support the proposal, the same proposal is set for GTW session [GTW2] High priority proposal 6-2: - Add note “Note: this feature applies to cells in the same TAG” in FG 25-9/10 |
| FL3 | Following proposal was made in the GTW session on Jan 20. Agreement - Add note “Note: this feature applies to cells in the same TAG only” in FGs 25-9/10 |

Medium priority question 6-3:

- Companies are encouraged to provide views on whether the type of FGs 25-9 and 25-10 should be per UE or per BC or per FSPC

  - FG 25-9

    - Per UE: DOCOMO, Ericsson, Nokia, NSB

      - Not clear that band differentiation would impact the feature and the corresponding testing

      - In line with RAN2 recommendation that FSPC indication should be avoided as much as possible (R2-2002378)

    - Per BC: ZTE, Spreadtrum, Qualcomm, MediaTek

      - not all the band combination is suitable for PUCCH switching

    - Per FSPC: vivo(?)

      - only used for TDD scenario

  - FG 25-10

    - Per UE: DOCOMO, Ericsson, Nokia, NSB

    - Per BC: ZTE, Spreadtrum, Qualcomm, MediaTek

      - not all the band combination is suitable for PUCCH switching

    - Per FSPC: vivo(?)

      - only used for TDD scenario

| Company | Comment |
| --- | --- |
| vivo | We support the type of FGs 25-9 and 25-10 is per BC |
| New H3C | The type of FG 25-9and FG 25-10 should be Per BC. |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| Nokia, NSB | Per UE |
| Intel | Per BC, since essentially the feature is about operation/switching between cells/carriers, where some combinations may not be easily supported by a UE. |
| Ericsson | Per UE |

Low priority question 6-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 25-9 to 25-10

  - FG 6-6: MTK

| Company | Comment |
| --- | --- |

Low priority question 6-5:

- Companies are encouraged to provide views on whether to add applicable cases in component descriptions of FGs 25-9 and 25-10, e.g.,

  - FG 25-9

    - semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI

    - support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier

  - FG 25-10

    - PUCCH carrier switching based on dynamic indication is applicable to

      - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier

      - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

      - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

      - HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH

      - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size for HARQ-ACK retransmission based on the indication in the triggering DCI

    - support PUCCH carrier switching only among two different TDD cells with PUCCH configured on the NUL carrier

| Company | Comment |
| --- | --- |
| Nokia, NSB | We don’t see the need for the suggested additions above. Moreover, we are e.g. currently still discussing if some changes to the dynamic PUCCH cell switching applicability may be done. And therefore, better to be not too specific here. |
| Ericsson | Wait for further development in AI 8.3.1 to add details |

Low priority question 6-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 25-9 and 25-10 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-11: 4-bits subband CQI

In [1], FG 25-11 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-11: Fine with the current FG 25-11, including cells in yellow. |
| --- | --- | --- |
| [7] | Samsung | Confirm all yellow highlight parts |
| [8] | NTT DOCOMO, INC. | - FG 25-11: 4-bits subband CQI - Type should be per UE - FG 2-32 can be added as prerequisite feature group. |
| [10] | Qualcomm Incorporated | 25. NR_IIOT_URLLC_enh; 25-11; 4-bits subband CQI; Subband CQI reporting with 4 bits per subband; Yes; N/A; Per UE Per Band; No; No; N/A; Optional with capability signaling. |
| [12] | Intel Corporation | From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-11 4-bits subband CQI |
| [13] | Ericsson | Add FG 2-32 “Basic CSI feedback” as a prerequisite for FG 25-11 “4-bits subband CQI”. Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-11: - Per UE |

Discussion

Medium priority question 7-1:

- Type of FG 25-11 is per UE or per Band

  - Per UE: Huawei, HiSilicon, Samsung, DOCOMO, Intel, Ericsson, Nokia, NSB

  - Per Band: Qualcomm

| Company | Comment |
| --- | --- |
| Apple | Per band is okay |
| New H3C | We support type of FG 25-11 should be per UE |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| ZTE | Per UE is fine. |
| Nokia, NSB | Per UE |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Ericsson | Per UE |

Low priority proposal 7-2:

- FG 2-32 is added as a prerequisite feature group for FG 25-11

  - Support: DOCOMO, Ericsson

| Company | Comment |
| --- | --- |
| QC | 2-32 is a mandatory Rel-15 UE feature without capability signaling. Whether add or not add it as prerequisite, UE has to support it anway. |
| Huawei, HiSilicon | Fine though we think it is not necessary since 2-32 is anyway mandatory feature. |
| Ericsson | Prefer to add. This is not a critical issue and we are fine if majority does not see the need. |

Low priority question 7-3:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-11 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-12 to 25-13: UE initiating a semi-static channel occupancy

In [1], FGs 25-12 to 25-13 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a, 25-12 | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-12: Add basic feature components agreed for a UE that can operate as an initiating device in the semi-static channel access mode as follows. It can be decided whether the condition on the gap duration in component 2 needs to be captured or not, i.e., “a gap greater than 16us”. 25. NR_IIOT_URLLC_enh; 25-12; UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations; 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. Sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI; 10-1a; Yes; N/A; Per band; N/A; N/A; N/A; The signaling is per band but is only expected for a band where shared spectrum channel access must be used; Optional with capability signaling. |
| --- | --- | --- |
| [4] | vivo | For FG 25-12, the following descriptions should also be captured in components column to ensure that a UE can operate as an initiating device in the semi-static channel access mode. - 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. - Determination of COT initiator assumption based on rules for configured UL - Validating COT initiator assumption indicated in UL scheduling DCI Proposal 7: For FG 25-12, capture the following descriptions in components column. 25-12; UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations; 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL. 4. Validating COT initiator assumption indicated in UL scheduling DCI.; 10-1a. |
| [7] | Samsung | Confirm the yellow highlight part |
| [8] | NTT DOCOMO, INC. | - FG 25-12: UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations - Regarding additional components, we are fine to add following ones to ensure which features are included in FG 25-12 - Component 2: 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. - Component 3: Determination of COT initiator assumption based on rules for configured UL - Component 4: Validating COT initiator assumption indicated in UL scheduling DCI |
| [13] | Ericsson | The remain issue with respect to UE features for operation on shared spectrum was whether to update the component description of FG 25-12 as the following: Table 2: FG 25-12 with proposed updates for component description 25-12; UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations; 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI. From our perspective, the proposed additions are important to be included and tested to ensure proper intra-operability. Adopt the proposed changes in red in Table 2 for FG 25-12. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-12: - Confirm the FG components (i.e. remove yellow highlight) |

Discussion

Low priority proposal 8-1:

- Following components are added in FG 25-12

  - Component 2: 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO.

  - Component 3: Determination of COT initiator assumption based on rules for configured UL

  - Component 4: Validating COT initiator assumption indicated in UL scheduling DCI

    - Support: Huawei, HiSilicon, vivo, DOCOMO, Ericsson

| Company | Comment |
| --- | --- |
| Qualcomm | Support |
| Huawei, HiSilicon | We support Proposal 8-1 with the following update to Component 2: - Component 2: 9us sSensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. The reason for deleting “ 9us” is to align with the Channel Access Type indicated in DCI by Row index 3 of Table 7.3.1.1.1-4A in TS 38.212 v17.0.0 when the UE is configured with ue-SemiStaticChannelAccessConfig. This is also aligned with introducing 16us sensing slot beside the default 9us sensing slot to the channel access procedures and related dynamic signaling for gNB-only initiated semi-static channel occupancy by endorsing CR0021 in R1-2112751 for TS37.213 and CR0077 in R1-2112750 for TS38.212, respectively. Also, from companies’s views, it seems that the “[ ]” can be removed and the condition for performing sensing within UE initiated COT “a gap greater than 16us from” can be kept for more accuracy. |
| LG | Support |
| Ericsson | Support. We are also fine with Huawei update to component 2. |

- 25-14 to [25-15]: PHY prioritization of overlapping DG-PUSCH and CG-PUSCH with different priorities

In [1], FGs 25-14 to 25-15 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | New H3C Technologies Co., Ltd. | Based on the following agreements in RAN1#107-e, the square brackets of FG 25-15 can be removed. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated.. Furthermore, FG 25-14 and FG 25-15 should be merged into one feature group, FG 25-14 can be modified as follows: PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH with different PHY priorities; Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH Support PHY prioritization of overlapping high-priority G-PUSCH and low-priority CG-PUSCH. |
| --- | --- | --- |
| [3] | Huawei, HiSilicon | - FG 25-14: Fine with the current FG 25-14, including cells in yellow. - FG 25-15: - The square bracket for FG 25-15 can be removed based on the agreement in RAN1#107-e. - Add a component for the maximum value of additional time d3 for LP CG-PUSCH and HP DG-PUSCH collision resolution. The detail for the candidate value for d3 still need to wait for the progress from intra-UE multiplexing. 25. NR_IIOT_URLLC_enh; [25-15]; PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH; 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time).; 12-1; Yes; N/A; Per band; N/A; N/A; N/A; FFS: Candidate value set for component 2: d3 = {0, 1,…,2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively.; Optional with capability signaling. |
| [5] | ZTE | Index 25-15: As we have the agreement on the handling of the collision between LP CG PUSCH and HP DG PUSCH in RAN1#107-e meeting. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively.. For FG 25-15, the square brackets as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” should be removed. No need to merge FG 25-14 and FG 25-15 in order to keep the flexibility of UE features. Proposal 7: The following modifications are proposed for feature group 25-15: - Remove the [] of index 25-15. |
| [7] | Samsung | Prefer to merge 25-14 and 25-15. |
| [8] | NTT DOCOMO, INC. | - FG 25-14: PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH - Type can be per band - FG 12-1 can be prerequisite feature group - FG 25-15: PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH - Square brackets should be removed based on the agreement in RAN1#107-e - Type can be per band - FG 12-1 can be prerequisite feature group |
| [9] | Spreadtrum Communications | FG [25-15]: confirm the FG 25-15, square brackets can be removed |
| [10] | Qualcomm Incorporated | With regards to feature 25-14, 25-15, 25-16, they all have prerequisite 12-1 which is per FS. Therefore, they should be defined as per FS as well. Proposal 11: Features 25-14, 25-15, and 25-16 are defined as per FS rather than per UE. 25. NR_IIOT_URLLC_enh; 25-14; PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH; Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH; 12-1; Yes; N/A; Per band Per FS; N/A; N/A; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; [25-15]; PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH; Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell; 12-1; Yes; N/A; Per band Per FS; N/A; N/A; N/A; Optional with capability signaling. |
| [11] | OPPO | In RAN1 #106bis-e meeting, FG 25-14 and 25-15 are kept with square brackets for further discussion since companies think it can wait until more progress in AI 8.3.3 is made. In last RAN1 meeting, the square brackets of FG 25-14 are removed since we achieved agreement on the handling of collision between HP CG PUSCH and LP DG PUSCH in RAN1 #106bis-e meeting. In RAN1 #107e meeting, the following agreement was reached to solve overlapping between LP CG and HP DG, so we think the square brackets of FG 25-15 should also be moved. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Proposal 11: Remove the [] for FG 25-15. |
| [13] | Ericsson | In our view a single UE feature should be defined for overlapping DG-PUSCH vs CG-PUSCH of different priorities. It does not make sense that a UE would support one of 25-14/[15], but not the other. Thus we propose to expand 25-14 to include the components of [25-15]. Merge [25-15] into 25-14. The description is revised to “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different priorities”. As shown in the agreements of AI 8.3, the UE is expected to apply cancellation per actual repetition, if (LP/HP) DG-PUSCH and/or (HP/LP) CG-PUSCH is repeated. Thus it’s necessary to add the support of PUSCH with repetition in the merged 25-14 and [25-15]. Add support of PUSCH with repetition in the components for merged 25-14 and [25-15]. Furthermore, the additional time d3 in the agreement of AI 8.3 should be added as a component to [25-15] (to be merged with 25-14. Add a component for the additional cancellation time d3, together with its candidate value set: “Additional number of symbols (d3) of the preparation time needed for the high priority DG-PUSCH transmission that cancels a low priority CG-PUSCH transmission. Candidate values for d3 are: {0, 1,…,2μ+1}” In summary, we suggest to update 25-14 to the following: 25. NR_IIOT_URLLC_enh; 25-14; PHY prioritization of overlapping PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different priorities; - Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH - Cancellation is applied per actual repetition, if low-priority DG-PUSCH and/or high-priority CG-PUSCH is repeated. - Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell - Cancellation is applied per actual repetition, if high-priority DG-PUSCH and/or low-priority CG-PUSCH is repeated. - Additional number of symbols (d3) of the preparation time needed for the high-priority DG-PUSCH transmission that cancels a low priority CG-PUSCH transmission. Candidate values for d3 are: {0, 1,…,2μ+1}; 12-1. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-14: - Note that FG 25-14 has been confirmed last meeting already though the whole row is marked in yellow highlight in [2]. (See agreements listed under FG 25-15 below.) - Per UE - 25-15: - Per UE - Square brackets and yellow highlights can be removed, i.e. UE feature is needed, as RAN1 finalized the details of this feature during RAN1#107-e (UE capability reporting on d3 would need to be added to the ’Note’ column): |

Discussion

[FL1] High priority proposal 9-1:

- FG 25-15 is kept as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” (i.e. delete square brackets in FG 25-15) as follows

| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | FFS: Candidate value set for component 2: d3 = {0, 1,…,2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Support: H3C, Huawei, HiSilicon, ZTE, DOCOMO, Spreadtrum, OPPO, Nokia, NSB

| Company | Comment |
| --- | --- |
| DOCOMO | Support the proposal and also fine to merge FG 25-15 with FG 25-14. |
| vivo | Support to keep FG 25-15. |
| OPPO | Support the proposal. |
| QC | Support the proposal. But we are not supportive with merging FG 25-15 and 25-14, because the difficulty level at UE to implement these two features are different. |
| Samsung | Okay with the proposal, but it should be discussed together with question 9-2. |
| LG | Support the proposal. |
| Nokia, NSB | Support |
| Intel | Support |
| Huawei, HiSilicon | Support. For the detail of d3 signaling, we can wait for the progress in AI 8.3.3. |
| Moderator | While all companies are fine to confirm FG 25-15, this can be come back when question 9-2 is resolved |
| FL2 | An agreement was made in the GTW session on Jan 18 as captured in question 9-2. |

[FL1] High priority question 9-2:

- Companies are encouraged to provide views on whether to merge FG 25-14 and 25-15

  - Support: H3C, Samsung, Ericsson

    - It does not make sense that a UE would support one of 25-14/[15], but not the other

  - Not support: ZTE

    - in order to keep the flexibility of UE features

| Company | Comment |
| --- | --- |
| DOCOMO | Fine to merge FG 25-14 and FG 25-15. |
| vivo | We prefer NOT to merge the FG 25-14 and 25-15. |
| OPPO | Fine to merge FG 25-14 and FG 25-15. |
| QC | Don’t support this proposal, because the difficulty level at UE to implement these two features are different. UE should be allowed to implement one feature, while not the other one. |
| LG | We prefer to leave FGs separated (Not to merge). We already have separated discussion for those FGs in RAN1 due to their differences. |
| ZTE | New timeline requirement for FG 25-15 is the working assumption, better to be individual FG. |
| Nokia, NSB | Do not support (as we support Proposal 9.1 above) |
| Intel | Prefer not to merge. |
| Huawei, HiSilicon | Prefer not to merge FG 25-14 and FG 25-15. The mechanisms for the two FGs are different, and the impact on UE is different. |
| Moderator | Summary of companies view - Support: H3C, Samsung, Ericsson, DOCOMO, OPPO - It does not make sense that a UE would support one of 25-14/[15], but not the other - Not support: ZTE, vivo, QC, LG, ZTE, Nokia, NSB, Intel, MTK, HW/HiSi, Apple - in order to keep the flexibility of UE features - difficulty level at UE to implement these two features are different Given that there is no majority view, further discussion in GTW session is necessary |
| FL2 | Following agreement was made in the GTW session on Jan 18. Agreement - FG 25-15 is kept as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” (i.e. delete square brackets in FG 25-15) as follows 25. NR_IIOT_URLLC_enh; [25-15]; PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH; 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). FFS whether to merge with FG 25-14; 12-1; Yes; N/A; Per band; N/A; N/A; N/A; FFS: Candidate value set for component 2: d3 = {0, 1,…,2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively.; Optional with capability signaling. Regarding the FFS whether to merge with FG 25-14, this was discussed in the GTW session on Jan 18 but no consensus was achieved. Companies are encouraged to further provide view on the FFS. Please also try to address the concern from other sides. |
| Ericsson | For the sake of progress, we are OK with not merging FG 25-14 and 25-15. Note that we also suggest to add components/clarifications to the components of 25-14 and 25-15. - Edits for FG 25-14: - Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH on a BWP of a serving cell - Cancellation is applied per actual repetition, if low-priority DG-PUSCH and/or high-priority CG-PUSCH is repeated. - Add a component to FG 25-15: - Cancellation is applied per actual repetition, if high-priority DG-PUSCH and/or low-priority CG-PUSCH is repeated. |
| DOCOMO | We are fine to keep them separated based on the GTW discussion. UE would need additional UE complexity for the cancelation with the new timeline. |
| LG | We still prefer separated FG. If we consider UE with skipUplinkTxDynamic=”true”, PHY prioritization between HP DG and LP CG is totally different from between HP CG and LP DG. We think there could be beneficial point. |
| ZTE | Better to separate the FGs. |
| Moderator | Summary of companies view - Not merge: ZTE, vivo, QC, LG, ZTE, Nokia, NSB, Intel, MTK, HW/HiSi, Apple, E///, DCM - in order to keep the flexibility of UE features - difficulty level at UE to implement these two features are different - Merge: H3C, Samsung, OPPO - It does not make sense that a UE would support one of 25-14/15, but not the other Given majority companies prefer not to merge FG 25-14 and 25-15, following proposal is made [GTW2] High priority proposal 9-2: - FGs 25-14 and 25-15 are not merged |
| FL3 | This proposal could not be discussed in the GTW session on Jan 20. Companies are invited to provide view whether this proposal is acceptable or not. Please also try to address the concern from other side [FL3] High priority proposal 9-2: - FGs 25-14 and 25-15 are not merged |
| vivo | Support FL High priority proposal 9-2. |
| DOCOMO | We are fine with the proposal. |
| OPPO | Fine with the proposal. |
| ZTE | Support 9-2 |
| Huawei, HiSilicon | Support proposal 9-2. |
| Nokia, NSB | Support |
| LG | Support the proposal 9-2 |
| Ericsson | We can accept proposal 9-2 |
| Moderator | Given all companies are fine with the proposal, the same proposal is set for GTW session [GTW3] High priority proposal 9-2: - FGs 25-14 and 25-15 are not merged |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given no concern has not been received for more than 24 hours. This proposal is set for email approval after final check. |
| Moderator | Following agreement was made over the RAN1 reflector. Agreement - FGs 25-14 and 25-15 are not merged |

Medium priority question 9-3:

- Companies are encouraged to provide views on whether the type of FG 25-14 and 25-15 should be per UE or per band or per FS

  - Per UE: Nokia, NSB

  - Per band: Huawei, HiSilicon, DOCOMO

  - Per FS: Qualcomm

    - As prerequisite FG

| Company | Comment |
| --- | --- |
| vivo | Per FS to align with prerequisite FG. |
| New H3C | We support the type of FG 25-14 and 25-15 should be per band |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| Nokia, NSB | Per UE, no need to follow the granularity of pre-requisites, similarly as in Rel-16. |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Huawei, HiSilicon | Fine to go to Per FS as well considering the prerequisite. |
| Ericsson | Per UE |

Low priority proposal 9-4:

- Prerequisite feature groups for FG 25-14 and 25-15 are confirmed as FG 12-1

  - Support: Huawei, HiSilicon, DOCOMO, Spreadtrum, [Ericsson], Nokia, NSB

| Company | Comment |
| --- | --- |
| Nokia, NSB | Support |
| Ericsson | Support |

- 25-16: Intra-UE multiplexing with different priorities

In [1], FGs 25-16 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type | 11-3, 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | New H3C Technologies Co., Ltd. | Based on the following guideline for Rel-17 intra-UE multiplexing framework in RAN#94-e, Capabilility#1 only will be handled in Q1 2022 and there is no more time to address other capability (capability#3). In this case, one FG is enough for HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH we suggest removing FFS sentence in FG 25-16. "RAN to guide RAN1 to focus on the discussions on Capabilility#1 only in Q1 2022 for Rel-17 intra-UE multiplexing framework". |
| --- | --- | --- |
| [3] | Huawei, HiSilicon | - FG 25-16: - For FFS whether to separate capability for different UCI type, we are open to discuss. However we may not need to split in such a detailed way for every different UCI type. - Change 11-3 to one of {11-4, 11-4a}, as 11-4/11-4a is the UE capability of supporting two HARQ-ACK codebook with different priorities. - Delete 12-1 from the prerequisite feature group for FG 25-16. 12-1 is to define prioritization of overlapping channel/signals with two priority levels in physical layer, while 25-17 here is to define multiplexing of overlapping channel/signals with two priority levels in physical layer. There is no need to couple these two capabilities. It also aligns with #capability 1 UE as agreed #107-e meeting. In addition, if 12-1 is removed, then we can add one component “Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format” to FG 25-16. 25. NR_IIOT_URLLC_enh; 25-16; HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH; 1. Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 12. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 23. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 34. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 45. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 56. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 67. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 78. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type; 11-3 one of {11-4, 11-4a}, 12-1; Yes; N/A; Per UE; No; No; N/A; Optional with capability signaling. |
| [4] | vivo | For FG 25-16, the prerequisite feature groups are FG 11-3 which is per FeatureSetUplink. Therefore, the type of FG 25-16 should be Per FS. Proposal 6: The type of FG 25-16 should be Per FS. 25. NR_IIOT_URLLC_enh; 25-16; HARQ-ACK with different priorities multiplexing using on a PUCCH/PUSCH; 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type; 11-3, 12-1; Yes; N/A; Per UEFS; No; No; N/A; Optional with capability signaling. |
| [5] | ZTE | Index 25-16 For the usage of parentheses in component of 25-16, we should keep the commonality on each part of the components. So we propose to remove the parentheses for item 4 and 5. Proposal 8: The following adjustment is proposed for component of 25-16. 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination.. |
| [6] | CATT | - FG25-16 There is an FFS for FG25-16 on whether to separate capability for different UCI type. We do not think separate capability for different UCI type is needed. Therefore, it is proposed to delete the FFS. Proposal 2: Separate capability for different UCI type for FG25-16 is not supported and the corresponding FFS is removed. |
| [7] | Samsung | Remove FFS for different UCI type. |
| [8] | NTT DOCOMO, INC. | - FG 25-16: HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH - Regarding the FFS whether to separate capability for different UCI type, we don’t support further separating capability for different UCI types. We think separate capabilities for different UCI types will increase scheduling complexity and thus it would be hard to implement/operate the PUCCH/PUSCH multiplexing of different priorities at gNB side. - Type should be per UE - FGs 11-3 and 12-1 can be kept as prerequisite feature groups for FG 25-16. |
| [9] | Spreadtrum Communications | - Prerequisite FGs: 11-3. We do not think 12-1 is a prerequisite. At least for capability 1, a UE does not need to partial cancel LP PUCCH/PUSCH. - Type: per FS, to keep consistence with FG 11-3 |
| [10] | Qualcomm Incorporated | With regards to feature 25-14, 25-15, 25-16, they all have prerequisite 12-1 which is per FS. Therefore, they should be defined as per FS as well. Proposal 11: Features 25-14, 25-15, and 25-16 are defined as per FS rather than per UE. 25. NR_IIOT_URLLC_enh; 25-16; HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH; 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to separate capability for different UCI type; 11-3, 12-1; Yes; N/A; Per band Per FS; No; No; N/A; Optional with capability signaling. |
| [11] | OPPO | For FG 25-16 and 25-17, there is an FFS about whether to separate capability for different UCI type. From our perspective, we do not see the need to split the FGs since no additional UE implementation complexity is observed for different UCI type. In addition, if separate capability is involved, more standardization work may be needed for UE supporting multiplexing for one UCI type while not for other UCI type. Regarding to the prerequisite feature groups for FGs 25-16 and 25-17, we prefer to remove FG 12-1 since it is unnecessary to couple Rel-16 intra-UE prioritization capability with Rel-17 intra-UE multiplexing capability. UE may choose to implement FG 25-16 and 25-17, while not to implement 12-1. This is because for a UE capable of intra-UE multiplexing almost no longer needs to perform cancelation between UL channels with different priorities. Proposal 12: It is preferred not to separate capability for different UCI types. Proposal 13: Remove 12-1 from the prerequisite feature groups of FG 25-17. |
| [12] | Intel Corporation | - 25-16 HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH - Regarding the FFS whether to separate capability for different UCI type, we have a slight preference of not doing that to allow a greater network scheduling flexibility to support intra-UE multiplexing use cases. From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-16 HARQ-ACK with different priorities multiplexing using on a PUCCH/PUSCH |
| [15] | Nokia, Nokia Shanghai Bell | - 25-16: - No need for separate capabilities per UCI type - Per UE |

Discussion

[FL] High priority proposal 10-1:

- “FFS whether to separate capability for different UCI type” is removed from FG 25-16

  - Support: H3C, CATT, Samsung, DOCOMO, OPPO, Intel, Nokia, NSB

    - separate capabilities for different UCI types will increase scheduling complexity and thus it would be hard to implement/operate the PUCCH/PUSCH multiplexing of different priorities at gNB side

    - no additional UE implementation complexity is observed for different UCI type

    - Less standardization work

  - Open: Huawei, HiSilicon

| Company | Comment |
| --- | --- |
| vivo | We are fine to remove the “FFS […]” bullet. |
| QC | Although we prefer separate capabilities to allow more UE implementation flexibility, but we can compromise to remove this FFS to simplify the implementation/operation at gNB side. |
| ZTE | Support the proposal 10-1 |
| Nokia, NSB | Support |
| Intel | Support removing the FFS |
| Huawei, HiSilicon | We are fine to remove the FFS bullet also. |
| LG | We are also fine to remove the FFS bullet. |
| Ericsson | Support |

Medium priority question 10-2:

- Companies are encouraged to provide views on whether the type of FG 25-16 should be per UE or per FS

  - Per UE: DOCOMO, Intel, Nokia, NSB

    - complexity does not scale with bands

  - Per FS: vivo, Spreadtrum, Qualcomm

    - As prerequisite FG

| Company | Comment |
| --- | --- |
| MediaTek | Per FS |
| vivo | Per FS. |
| New H3C | Per FS |
| QC | Per UE capability signaling does not work. Below is the 3GPP process of IODT confirmation. In theory, this applies to any feature. One can ignore the 6 months part for new (e.g. Rel-17) features. The key part is that test opportunity has to exist with two separate commercial infra coming from two different vendors. If we make a feature per-UE, then we make it a necessity that the UE implements and tests the feature in unlicensed (unconditionally) and in NTN (conditioned on how NTN features will be handled in general). As long as unlicensed/NTN base stations don't get this feature implemented, testing is not possible, and the feature is going to get effectively disabled across the board. We can discuss whether per FSPC, per FS, per BC, or per band is needed. But per UE does not work. |
| ZTE | Per UE |
| Nokia, NSB | Per UE |
| Intel | Per UE from technical perspective. The potential testing issue should be kept aside for now, and if confirmed, may be brought to change the signaling. |
| Huawei, HiSilicon | Per FS to align with the prerequisites. |
| Ericsson | Per UE |

Low priority question 10-3:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 25-16

  - One of {11-4, 11-4a}: Huawei, HiSilicon

    - Delete 12-1: no need to couple these two capabilities

  - 11-3 and 12-1: DOCOMO

  - Only 11-3: Spreadtrum, OPPO

    - Delete 12-1: At least for capability 1, a UE does not need to partial cancel LP PUCCH/PUSCH. no need to couple these two capabilities

| Company | Comment |
| --- | --- |
| MediaTek | - As there is no cancelation behavior defined for FG25-18, FG12-1 can’t be prerequisite. Thus, it should be FG12-1 removed from the prerequisite feature groups for FG 25-16. - FG11-3 is related to “More than one PUCCH for HARQ-ACK transmission within a slot” for the same priority. Thus, it is not clear to us why this FG should be prerequisite for multiplexing between different priorities. - FG11-4 “Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE.” chould be a prerequisite FG. |
| New H3C | Support Only 11-3 |
| QC | 12-1 should be removed. 12-1 is Rel-16 dropping/prioritization between different priorities. Rel-17 multiplexing between different priorities does not depends on Rel-16 dropping/prioritization. |
| ZTE | At least support 11-3 |
| Nokia, NSB | Should be 11-4 instead of 11-3 (two HARQ-ACK codebooks need to be constructed to enable the multiplexing in the first place). We suggest to keep 12-1, as this also contains the PHY priority configuration (which is needed for the multiplexing operation in the first place). |
| Huawei, HiSilicon | - Change 11-3 to one of {11-4, 11-4a}, as 11-4/11-4a is the UE capability of supporting two HARQ-ACK codebook with different priorities. - Delete 12-1 from the prerequisite feature group for FG 25-16. 12-1 is to define prioritization of overlapping channel/signals with two priority levels in physical layer, while 25-17 here is to define multiplexing of overlapping channel/signals with two priority levels in physical layer. There is no need to couple these two capabilities. It also aligns with #capability 1 UE as agreed #107-e meeting. - Since 12-1 should be removed, we can add one component “Support configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format” to FG 25-16” to reflect different priorities for CG-PUSCH, DG-PUSCH and SR. |
| Ericsson | Only 11-4 (“Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slotbased + slot-based, or slot-based + subslot based) simultaneously constructed for supporting HARQACK codebooks with different priorities at a UE.”). Regarding 12-1: we agree that 12-1 is for prioritization (i.e., cancellation/dropping), and should not be coupled here. In AI 8.3.3 discussion, currently ‘Capability #1’ is under discussion, and one reason was the concern on the Rel-16 cancellation timeline. Thus ‘Capability #1’ discussion assumes that 12-1 is not necessarily implemented. |

Low priority question 10-4:

- Companies are encouraged to provide views on whether to add components for support of PHY priority to FG25-16, e.g.,

  - Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format

| Company | Comment |
| --- | --- |
| Nokia, NSB | This very much depends on the prerequisite FGs (if 12-1 is kept, then no need to add that component). |
| Huawei, HiSilicon | Yes if we delete 12-1 from the prerequisite as described above. In addition, if 11-4 or 11-4a is added as the prerequisite, then we can only add the following component, since different priorities of HARQ-ACK is already reflected by 11-4 or 11-4a. - Support configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format |
| Ericsson | Support removing 12-1 as prerequisite and add more component details accordingly. Details can be discussed after the prerequisites are agreed. |

Low priority question 10-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-16 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-18: Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

In [1], FG 25-18 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [3] | Huawei, HiSilicon | - FG 25-18: - Remove [at least] as it is already concluded not to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17 during last meeting. - The terminology of “PUCCH/PUSCH” covers three cases: PUCCH + PUCCH, PUCCH + PUSCH, and PUSCH + PUSCH. And FG 25-18 only refers to the case of “PUCCH + PUSCH”. We propose to change the component as “Support simultaneous PUCCH and PUSCH transmissions on different cells for inter-band CA.” to eliminate confusion. - Change “Per UE” to “Per BC”, since the capability for UE would dependent on the CA band combination. 25. NR_IIOT_URLLC_enh; 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH PUCCH and PUSCH transmissions on different cells [at least] for inter-band CA.; Yes; N/A; Per UEBC; No; No; N/A; Optional with capability signaling. |
| --- | --- | --- |
| [4] | vivo | In latest RAN1 meeting, no consensus is achieved to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17, [at least] can be deleted. Proposal 8: For FG 25-18, [at least] can be deleted. 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA.; Yes; N/A; Per UE; No; No; N/A. |
| [5] | ZTE | Index 25-18: This feature group is supporting parallel PUCCH and PUSCH transmission across CCs in inter-band CA. The type of this feature group is proposed to “Per BC”. The reason is similar with previous proposal as not all band combination is supporting parallel PUCCH and PUSCH transmission. Proposal 9: The type of the feature group 25-18 is proposed to change to Per BC. |
| [6] | CATT | - FG25-18 The following conclusions were made in RAN1#107-e. So the FG25-18 is proposed to be updated according to the conclusions. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.. Proposal 3: Adopt the following update to FG25-18. 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA.; Yes; N/A; Per UE; No; No; N/A; Optional with capability signaling. |
| [7] | Samsung | Based on related conclusions in RAN1#107-e, it should be changed as “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.” |
| [8] | NTT DOCOMO, INC. | - FG 25-18: Parallel PUCCH and PUSCH transmission across CCs in inter-band CA - Type should be per UE - ”[at least]” can be removed based on the conclusion in RAN1#107-e |
| [9] | Spreadtrum Communications | - Prerequisite FGs: 25-16. Because only different priority of PUCCH and PUSCH can parallel transmission in Rel-17, which means it only exits based on FG 25-16. Conclusion RAN1 107e There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17.. - Type: per FS, to keep consistence with FG 25-16 |
| [10] | Qualcomm Incorporated | With regards to feature 25-18. It is a feature for particular band combinations. UE may be able to support this feature for some band combinations but not other band combinations. It is natural to define this feature per band combination. Proposal 12: Feature 25-18 is defined as per BC rather than per UE. 25. NR_IIOT_URLLC_enh; 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA.; Yes; N/A; Per UE Per BC; No; No; N/A; Optional with capability signaling. |
| [12] | Intel Corporation | From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-18 Parallel PUCCH and PUSCH transmission across CCs in inter-band CA |
| [13] | Ericsson | Thus, Rel-17 only supports simultaneous PUCCH/PUSCH transmission of different priorities on different cells for inter-band CA. Accordingly FG 25-18 can be finalized as below: 25. NR_IIOT_URLLC_enh; 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA.. FG 25-18a component description is revised to: “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.” |
| [14] | MediaTek Inc. | Regarding FG25-18, the support of parallel PUCCH and PUSCH transmission across CCs in inter-band CA should be reported per band combination. Index; Feature group; Components; Prerequisite FG; Type; Note; Mandatory/Optional. 25-18; Parallel PUCCH and PUSCH transmission across CCs in inter-band CA; Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA.; Per BC; Optional. - Change the Type of FG25-18 from “Per UE” to “Per BC”. |

Discussion

Medium priority question 11-1:

- Companies are encouraged to provide views on whether the type of FG 25-18 should be per UE or per BC or per FS

  - Per UE: DOCOMO, Intel

    - complexity does not scale with bands

  - Per BC: Huawei, HiSilicon, ZTE, Qualcomm, MediaTek

    - the capability for UE would dependent on the CA band combination

  - Per FS: Spreadtrum

    - As FG 25-16

| Company | Comment |
| --- | --- |
| MediaTek | The UE capability of supporting simultaneous PUCCH and PUSCH transmissions is dependent on the CA band combination. |
| vivo | Per BC. |
| Apple | Per FS, this is about Putting PUCCH on a particular band |
| New H3C | Per BC |
| QC | Per UE does not work, based on the same reason explained for other UE features. |
| Nokia, NSB | Per UE |
| Intel | Per BC (per UE seems an omission in our tdoc) |
| LG | Per BC |
| Ericsson | Per BC |

Low priority question 11-2:

- Companies are encouraged to provide views on how to update the component description for FG 25-18, e.g,

  - Support simultaneous PUCCH/ and PUSCH transmissions of different priorities on different cells [at least] for inter-band CA.

| Company | Comment |
| --- | --- |
| QC | The above suggested wording update from FL looks good to us. |
| ZTE | Support the update |
| Nokia, NSB | Support in principle, but maybe we should be more specific here saying the PUSCH and PUCCH need to be in different bands (as there could be inter-band CA, but only transmissions in one band at a time and then this then the simultaneous PUSCH and PUCCH transmission would not be applicable. |
| Huawei, HiSilicon | Support with reasons given below: - Remove [at least] as it is already concluded not to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17 during last meeting. - The terminology of “PUCCH/PUSCH” covers three cases: PUCCH + PUCCH, PUSCH + PUSCH, and PUSCH + PUSCH. And FG 25-18 only refers to the case of “PUCCH + PUSCH”. We propose to change the component as “Support simultaneous PUCCH and PUSCH transmissions on different cells for inter-band CA.” to eliminate confusion. |
| Ericsson | Support the revision |

Low priority question 11-3:

- Companies are encouraged to provide views on whether to add FG 25-16 as a prerequisite feature group for FG 25-18

  - Support: Spreadtrum

    - Because only different priority of PUCCH and PUSCH can parallel transmission in Rel-17

| Company | Comment |
| --- | --- |
| QC | FG 25-16 and FG 25-18 are two parallel features. We don’t see why one should be prerequisite of the other. |
| Nokia, NSB | Do not agree, as we did not agree yet to support the joint operation of simultaneous PUSCH and PUCCH and Rel-17 Intra-UE mux. But some type of PHY priority prerequisite FG should be added here (e.g. 11-3 and 12-1) |
| Huawei, HiSilicon | No need to add 26-16 in our understanding. If the point of is to emphasize different priorities, then the following two components from Rel-16 can be added: - Support 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - Support configuration of PHY priority level for CG PUSCH, and dynamic indication of priority level for dynamic PUSCH with a single DCI format |
| Ericsson | Do not add 25-16 as prerequisite. Although the two feature can work together (details pending further discussion in AI 8.3.3), they can work independent of each other as well. |

Low priority question 11-4:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-18 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 25-19 to 25-20: Propagation delay compensation

In [1], FGs 25-19 to 25-20 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS FFS whether to merge FG 25-19 and FG 25-19a | 2-51, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS FFS whether to merge FG 25-19 and FG 25-19a | 25-19, 13-1, 2-53 | Yes | N/A |  | FS | N/A | N/A | N/A | Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also. | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-20 | Propagation delay compensation based on legacy TA procedure | Support propagation delay compensation based on legacy TA procedure |  | Yes | N/A |  | Per UE | no | no | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | New H3C Technologies Co., Ltd. | Because CSI-RS and PRS signal can be independently configured and correspond to different UE capability, it is better to keep separated FG 25-19 and FG 25-19a. So we suggest removing FFS sentence in FG 25-19 and FG 25-19a as follows: FFS whether to merge FG 25-19 and FG 25-19a |
| --- | --- | --- |
| [3] | Huawei, HiSilicon | - FG 25-19/25-19a: - For FFS whether to merge FG 25-19 and FG 15-19a, we think it should be kept as independent UE capabilities to leave more flexibility at the UE side. - Delete 25-19 from the prerequisite feature group for FG 25-19a. RTT-based Propagation delay compensation based on CSI-RS for tracking and DL PRS should be independent UE capabilities to leave more flexibility at the UE side. - FG 25-10: Fine with the current including cells in yellow. |
| [4] | vivo | Positioning capability is an optional feature. URLLC UE supporting propagation delay compensation capability needn’t requires support of positioning capability. It is reasonable that UE without positioning capability supports PDC. In this case, FG 25-19 can be only supported by UE. It is unnecessary to merge FG 25-19 and FG 25-19a. Proposal 9: It is unnecessary to merge FG 25-19 and FG 25-19a. 25. NR_IIOT_URLLC_enh; 25-19; RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS; Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS; 2-51, 2-53; Yes; N/A; FS; N/A; N/A; N/A; Optional with capability signaling. 25. NR_IIOT_URLLC_enh; 25-19a; RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS; Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS; 25-19, 13-1, 2-53; Yes; N/A; FS; N/A; N/A; N/A; Note: FG 13-1 is now only reported to LMF. If UE reports the support of this FG, it needs to report FG 13-1 to gNB also.; Optional with capability signaling. |
| [5] | ZTE | Index 25-19 and 25-19a: Both FG 25-19 and 25-19a are used to report the capability that whether or not a UE supports Rel-17 propagation delay compensation. The only difference between them is the reference signal for measurement. This leads to the prerequisite feature groups are also different since they reflects the capability of the reference signals for measurement. Therefore, they can be merged to form two prerequisite feature groups sets, where one set corresponds to 25-19 and the other set corresponds to 25-19a. The network can determine the UE complete capability of Rel-17 propagation delay compensation based on the merged 25-19 and corresponding prerequisite feature groups. Proposal 10: Merge 25-19a into 25-19 and the corresponding prerequisite feature groups is {2-51, 2-53} or {25-19, 13-1, 2-53}. Proposal 11: The component of 25-19 is revised to: Support Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS or DL PRS and SRS. |
| [6] | CATT | - FG25-19/25-19a For FG25-19 and 25-19a, there is an FFS on whether to merge the two FGs. The difference is that RTT-based PDC is based on CSI-RS for FG25-19 and based on PRS for FG25-19a. We do not have a strong view but think it would be preferable if the two FGs can be merged. Proposal 4: It is preferable that FG25-19 and FG25-19 are merged. |
| [7] | Samsung | For PRS based propagation delay compensation, it requires PRS estimation, which is different from CSI-RS based estimation. Therefore, we prefer to keep them separated. Moreover, regarding on the Note for FG 25-19a, we don’t see the need. Even for PRS based propagation delay compensation, there is no need to refer to positioning capability, because for propagation delay compensation, it only requires DL timing estimation for PRS but no need to support the whole procedure and signaling for positioning. Moreover, it is not clear on whether SRS for positioning or SRS for propagation delay compensation (if the configuration of SRS for propagation delay compensation is agreed to be introduced). Therefore, we prefer to further discuss SRS related issues after we have more agreement or conclusion in 8.3.4. |
| [8] | NTT DOCOMO, INC. | - FG 25-19: RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS - Regarding the FFS whether to merge FG 25-19 and FG 25-19a, we think it can be merged - Type should be per UE - FG 2-51 and FG 2-53 can be prerequisite feature groups - FG 25-19a: RTT-based Propagation delay compensation based on DL PRS and SRS - Regarding the FFS whether to merge FG 25-19 and FG 25-19a, we think it can be merged - Type should be per UE - FG 13-1 and FG 2-53 can be prerequisite feature groups - FG 25-20: Propagation delay compensation based on legacy TA procedure - Type should be per UE |
| [12] | Intel Corporation | - 25-19 RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS - 25-19a RTT-based Propagation delay compensation based on DL PRS and SRS - Regarding the FFS whether to merge FG 25-19 and FG 25-19a, we clearly don’t see the need since DL PRS support may require additional implementation efforts comparing to CSI-RS based - Depending on discussion on “SRS for positioning” support, additional row e.g., “RTT-based Propagation delay compensation based on DL PRS and SRS for positioning” may be introduced From technical point of view, complexity of the following capabilities does not scale with bands, and therefore ‘Per UE’ signalling type is appropriate: - 25-20 Propagation delay compensation based on legacy TA procedure |
| [13] | Ericsson | Thus two FG need to be introduced for propagation delay compensation, one for legacy TA-based method, the other for RTT-based method. A UE may support the legacy TA-based FG for poor synchronization accuracy on the Uu interface, and/or support the RTT-based FG for tighter synchronization accuracy. Two feature groups are introduced for supporting propagation delay compensation in Rel-17, one for legacy TA-based method, the other for RTT-based method. Specifically, we suggest the FGs and components below. Further details for the components (e.g., gNB-based or UE-based compensation) can be discussed when the relevant decisions are made in RAN2. Table 3: Exemplary FG for propagation delay compensation. Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Mandatory/Optional. Legacy TA-based propagation delay compensation; Support legacy TA based propagation delay compensation for time synchronization of the Uu interface; N/A; Yes; Per UE; No; No; N/A; Optional with capability signalling. RTT-based propagation delay compensation; Support RTT-based propagation delay compensation for time synchronization of the Uu interface; N/A; Yes; Per UE; No; No; N/A; Optional with capability signalling. |
| [15] | Nokia, Nokia Shanghai Bell | - 25-19: - Per UE, FR1-only, with TDD/FDD differentiation. There is no need for per FS signaling due to dependency on 2-53. As already the case for Rel-16 MIMO, the UE will only support the feature for the bands and band combinations where the pre-requisites are fulfilled. Hence, this one can be reported with coarser granularity than its pre-requisites. - 25-19a: - Per UE, FR1-only, with TDD/FDD differentiation. There is no need for per FS signaling due to dependency on 2-53. As already the case for Rel-16 MIMO, the UE will only support the feature for the bands and band combinations where the pre-requisites are fulfilled. Hence, this one can be reported with coarser granularity than its pre-requisites. - 25-20: - Per UE |

Discussion

[FL1] High priority question 12-1:

- Companies are encouraged to provide views on whether to merge FG 25-19 and 25-19a

  - Merged: ZTE, CATT, DOCOMO, [Ericsson]

    - A UE may support the legacy TA-based FG for poor synchronization accuracy on the Uu interface, and/or support the RTT-based FG for tighter synchronization accuracy

  - Separate: H3C, Huawei, HiSilicon, vivo, Samsung, Intel

    - CSI-RS and PRS signal can be independently configured and correspond to different UE capability

    - to leave more flexibility at the UE side

| Company | Comment |
| --- | --- |
| DOCOMO | The difference between FG 25-19 and 25-19a is type of RS and thus UE complexity should be not much different. |
| vivo | Separate. Positioning capability is an optional feature. URLLC UE supporting PDC needn’t require support of positioning capability. PDC based on DL PRS may require additional implementation efforts compared to CSI-based PDC. |
| OPPO | Prefer separate FG 25-19 and 25-19a due to the reasons summarized above. |
| New H3C | Support separating FG 25-19 and 25-19a |
| LG | We support to merge those FGs. There seems no big difference in terms of UE complexity. |
| ZTE | We prefer to merge the two FGs. No flexibility lose if we can provide two sets of prerequisites for the FG 25-19 after merging, where UE can support FG 25-19 as long as it support one of the two sets. |
| Nokia, NSB | It is ok to have separate FGs, as the baseline synchronization schemes are not necessarily supported by the UE. |
| Intel | Prefer separate. A UE may not implement DL PRS without positioning support, and thus use only TRS. |
| Huawei, HiSilicon | Separate capabilities should be applied for different combinations of DL RS and UL RS, e.g. RTT-based PDC with TRS and SRS for MIMO, and RTT-based PDC with PRS and SRS. It is very clear that PRS is an optional capability for UE, it doesn’t make sense to request a UE to support PRS in order to support RTT-based PDC. By the way, depending on the outcome of PDC discussion in agenda 8.3.4, i.e. whether to support SRS for positioning, we may need some refinement and/or new FG here. |
| Moderator | Summary of companies view - Merge: ZTE, CATT, DOCOMO, [Ericsson], LG, ZTE - A UE may support the legacy TA-based FG for poor synchronization accuracy on the Uu interface, and/or support the RTT-based FG for tighter synchronization accuracy - Separate: H3C, Huawei, HiSilicon, vivo, Samsung, Intel, OPPO, New H3C, Nokia, NSB, Intel - CSI-RS and PRS signal can be independently configured and correspond to different UE capability - to leave more flexibility at the UE side Given that there is no majority view, further discussion in GTW session is necessary |
| FL2 | This proposal could not be discussed in the GTW session on Jan 18. Companies are invited to try to address the concern from other sides. |
| Huawei, HiSilicon (updated) | We still prefer separate UE capabilities. Besides the reasons we provided before, we think the reason given by the proponents for merging doesn’t justify the merge, because even with the current structure, UE still can report different capabilities for scenarios with relaxing requirements on synchronization accuracy (e.g. report FG 25-20) and scenarios with tighter synchronization accuracy (e.g. FG 25-19 and/or 25-19a). The support of scenarios with tighter synchronization accuracy doesn’t mean UE has to support both combination of TRS with SRS and combination of PRS with SRS. |
| Intel | It seems more companies are in favor of separate features. It is a safer option to go with at this point. |
| Ericsson | We are fine with not merging 25-19 and 25-19a. |
| DOCOMO | We are fine to have separate FGs based on the reasons provided by the proponents. |
| vivo | We support separate FGs. |
| Moderator | Summary of companies view - Merge: LG - A UE may support the legacy TA-based FG for poor synchronization accuracy on the Uu interface, and/or support the RTT-based FG for tighter synchronization accuracy - Separate: H3C, Huawei, HiSilicon, vivo, Samsung, Intel, OPPO, New H3C, Nokia, NSB, Intel, E///, DCM, ZTE - CSI-RS and PRS signal can be independently configured and correspond to different UE capability - to leave more flexibility at the UE side Given majority companies prefer separate FGs, following proposal is made [GTW2] High priority proposal 12-1: - FG 25-19 and 25-19a are not merged |
| FL3 | Following agreement was made in the GTW session on Jan 20 Agreement - FG 25-19 and 25-19a are not merged |

Medium priority question 12-2:

- Companies are encouraged to provide views on whether the type of FGs 25-19 to 25-20 should be per UE or per FS

  - FG25-19

    - Per UE: DOCOMO, [Ericsson], Nokia, NSB

      - no need for per FS signaling due to dependency on 2-53

      - FR1 only: Nokia, NSB

      - TDD/FDD differentiation is needed: Nokia, NSB

    - Per FS:

  - FG25-19a

    - Per UE: DOCOMO, Nokia, NSB

      - no need for per FS signaling due to dependency on 2-53

      - FR1 only: Nokia, NSB

      - TDD/FDD differentiation is needed: Nokia, NSB

    - Per FS:

  - FG25-20

    - Per UE: [Huawei], [HiSilicon], DOCOMO, Intel, [Ericsson] , Nokia, NSB

      - complexity does not scale with bands

| Company | Comment |
| --- | --- |
| vivo | We would be fine if the majority support the type of FG25-19, FG25-19a and FG 25-20 are per UE. |
| New H3C | We support the type of FG25-19, FG25-19a and FG 25-20 are per UE |
| ZTE | We think the type of FG 25-19 to 25-20 should be per UE. |
| Nokia, NSB | Per UE, no need to follow granularity of pre-requisite (see Rel-16). |
| Intel | Per UE |
| Huawei, HiSilicon | 1. Per FS for FG 25-19 and FG 25-19a, to align with the granularity of the prerequisite. 2. Support per UE for FG 25-20. |
| Ericsson | Per UE |

Low priority question 12-3:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 25-19 and 25-19a

  - FG 25-19

    - FG 2-51 and 2-53: DOCOMO

  - FG 25-19a

    - Delete FG 25-19 (i.e. FG 13-1 and 2-53): Huawei, HiSilicon, DOCOMO

      - to leave more flexibility at the UE side

    - FG 25-19, 13-1, and 2-53: ZTE

| Company | Comment |
| --- | --- |
| ZTE | It should be discussed after the question 12-1 is resolved. |
| Nokia, NSB | We do not see a need to change the current status of the prerequisite FGs (i.e. we think 25-19 should still be a prerequisite for 25-19a). |
| Huawei, HiSilicon | 1. Fine with FG 2-51 and FG 2-53 as the prerequisite for FG 25-19. 2. Slightly prefer to delete FG 25-19 from the prerequisite for FG 25-19a, because for UEs that will support PRS there is no need to request it to support RTT-based PDC with TRS. Of course, since TRS is a mandatory feature, it would be reasonable to assume PDC with TRS and SRS should be supported also for a UE supporting PDC with PRS and SRS, therefore we are fine not removing FG 25-19 also. |
| Ericsson | - Prerequisite for FG 25-19: 2-51, 2-53 (i.e., no change) - Prerequisite for FG 25-19a: 13-1, 2-53 (i.e., delete 25-19). There is no need to couple 25-19 and 25-19a. They can work independent of each other. |

Low priority question 12-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 25-19 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | For FG 25-19a, the following additional component should be added: - Max number of DL PRS Resources in the DL PRS Resource Set for PDC. Candidate Values = {1, 2, 4, 8, 16, 32, 64}. Note: 16, 32, 64 are only applicable to FR2 bands Similar as that for positioning method, the maximum number of PRS resources in the set should be UE capability also. |
| Ericsson | We don’t think components like “Max number of DL PRS Resources …” should be added. It’s adequate to have such capabilities reported under FG 13-1. |

- Conclusions

Following agreements were made in this RAN1 meeting.

Agreement

- FG 25-15 is kept as “PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH” (i.e. delete square brackets in FG 25-15) as follows

| 25. NR_IIOT_URLLC_enh | [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | 1. Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell 2. Additional number of symbols (d3) needed on top of Rel-16 cancellation time (which results N2+d1+d3 in total cancellation time). FFS whether to merge with FG 25-14 | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A | FFS: Candidate value set for component 2: d3 = {0, 1,…,2μ+1} symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 10-16 is confirmed as a prerequisite FG of FG 25-6

Agreement

- Add note “Note: this feature applies to cells in the same TAG only” in FGs 25-9/10

Agreement

- FG 25-19 and 25-19a are not merged

Agreement

- FG 25-6 is updated as follows

| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. The values higher than 1 can be applied to sub-slot based configuration only. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 25-7 is not separated per DCI format

Agreement

- Component 2 in FG 25-7 is not separated as another FG

Agreement

- FGs 25-14 and 25-15 are not merged

References

[1]	R1-2112902	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2200014	UE features for IIoT and URLLC	New H3C Technologies Co., Ltd.

[3]	R1-2200040	Rel-17 UE features for enhanced IIoT/URLLC	Huawei, HiSilicon

[4]	R1-2200100	Discussion on UE features for IIoT and URLLC	vivo

[5]	R1-2200121	Discussion on UE features for enhanced IIoT and URLLC	ZTE

[6]	R1-2200138	UE features for Rel-17 IIoT/URLLC	CATT

[7]	R1-2200218	UE features for enhanced IIoT and URLLC	Samsung

[8]	R1-2200248	Discussion on Rel.17 UE features for enhanced IIoT/URLLC	NTT DOCOMO, INC.

[9]	R1-2200286	UE features for enhanced IIoT and and URLLC	Spreadtrum Communications

[10]	R1-2200313	UE features for enhanced IIOT and URLLC	Qualcomm Incorporated

[11]	R1-2200346	Discussion on UE features for IIoT and URLLC	OPPO

[12]	R1-2200391	On UE features for enhanced IIoT and URLLC	Intel Corporation

[13]	R1-2200444	Rel-17 UE Features for IIoT/URLLC	Ericsson

[14]	R1-2200557	Views on UE features for enhanced IIoT and URLLC	MediaTek Inc.

[15]	R1-2200624	On UE features for enhanced IIoT and and URLLC	Nokia, Nokia Shanghai Bell