3GPP TSG RAN WG1 #107bis-e	                	                                 R1-2200252

e-Meeting, January 17th – 25th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for UE power saving enhancements

Agenda Item:	8.15.7

Document for: 	Discussion and Decision

Introduction

Updated Rel.17 UE feature list after RAN1#107-e including UE power saving enhancements has been agreed [1]. In this contribution, we present our views and updates regarding the UE features for UE power saving enhancements based on the list.

Rel-17 UE features for UE power saving

We itemized our suggested corrections to the existing UE features as follows.

- FG 29-1:

  - Type should be per UE

- FG 29-2:

  - For the Components: Regarding whether to separate the capability for receiving PEI based indication for TRS availability as another FG, we think need to separate the capability since we already agreed supporting PEI and Paging DCI based indication for TRS availability.

  - Regarding whether to support FG 29-2 as optional with capability signaling or optional without capability signaling, we think it can be left to RAN2 discussion.

  - Type should be per UE

  - The sentence in “Consequence if the feature is not supported by the UE” should be revised to “UE does not support TRS occasions for idle/inactive UEs”.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2-a | TRS resources for idle/inactive UEs indicating availability via Paging PDCCH | TRS receiving for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving Paging PDCCH indication for TRS availability |
|  | 29-2-b | TRS resources for idle/inactive UEs indicating availability via PEI | TRS receiving for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving PEI indication for TRS availability |

- FG 29-3a to 29-3d:

  - Type should be per UE

  - Regarding whether/how to separate the capabilities for PDCCH monitoring adaptation we think  they can be defined by four (or five) FGs as follows based on the progress in AI 8.7.2:

    - FG 29-3a: Support of PDCCH skipping

    - FG 29-3b: Support of  2 SSSG switching

    - FG 29-3c: Support of  3 SSSG switching

    - FG 29-3d: Support of  2 SSSG switching with PDCCH skipping

    - [FG 29-3e: Support of  3 SSSG switching with PDCCH skipping]

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |
| 29. NR_UE_pow_sav_enh | [29-3e] | 3 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 3 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |

According to the above comments, the following revision is proposed for UE power saving.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-2:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2-a | TRS resources for idle/inactive Ues indicating availability via Paging PDCCH | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving Paging PDCCH indication for TRS availability |
|  | 29-2-b | TRS resources for idle/inactive Ues indicating availability via PEI | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving PEI indication for TRS availability |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |
| 29. NR_UE_pow_sav_enh | [29-3e] | 3 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 3 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-2:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2-a | TRS resources for idle/inactive Ues indicating availability via Paging PDCCH | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving Paging PDCCH indication for TRS availability |
|  | 29-2-b | TRS resources for idle/inactive Ues indicating availability via PEI | TRS receiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving PEI indication for TRS availability |

- FG 29-3:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | FG 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |
| 29. NR_UE_pow_sav_enh | [29-3e] | 3 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 3 SSSGs with PDCCH skipping by scheduling DCI and timer based switching |

Reference

[1]	Moderators (AT&T, NTT DOCOMO, INC.), R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Nov. 2021.