3GPP TSG RAN WG1 #107bis-e			R1-2200258

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.12

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR MBS

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.12 regarding UE features for NR MBS and captures the following email discussion.

[107bis-e-R17-UE-features-MBS-01] Email discussion UE features for NR MBS – Shinya (DOCOMO) - 1st check point: January 20 - Final check point: January 25

In the Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e [1], there are following feature groups for NR MBS.

- 33-1	Broadcast

- 33-2	Dynamic scheduling for multicast

- 33-2b	DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

- 33-2-x	Multiple G-RNTIs for group-common PDSCHs

- 33-3-1	Slot-level repetition for group-common PDSCH

- 33-3-2	FDM-ed unicast PDSCH and group-common PDSCH

- 33-3-3	Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

- 33-4	NACK-only based HARQ-ACK feedback for multicast

- 33-5-1	SPS group-common PDSCH for multicast

- 33-5-2	Multiple SPS group-common PDSCH configuration

- 33-6-1	DL priority indication for multicast in DCI

- 33-6-2	Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE

- 33-6-3	More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

- 33-7	Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback

The issues to be discussed are tagged and colour coded with High priority, Medium priority, or Low priority, considering RAN2 impact especially for capability signaling design.

- 33-1: Broadcast

In [1], FG 33-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | Since DCI format 4_0 is defined in TS 38.212 for scheduling broadcast, DCI format 1_0 in the 3rd component needs to be updated to DCI format 4_0. MBS Broadcast includes MCCH and MTCH, both of which will be scheduled by G-RNTI. The 6th component needs to clarify that group-common PDSCH including MCCH and MTCH will be TDM-ed or either one will be TDM-ed with unicast in different slots. As discussed in [5], as motivated for unicast, MBS broadcast scheduling also needs to consider RB level and RE level rate matching. As mandatory for legacy UE supporting unicast, supporting semi-static rate-matching resource set configuration and supporting rate-matching around LTE CRS should be the components of FG33-1. Broadcast reception targets UEs for all RRC states. As commented by some companies, FG33-1 can be reported so that network can at least manipulate the transmissions of MBS broadcast and unicast properly for UE in RRC_CONNECTED state. From this sense, we can accept this capability reported to network. In addition, slot-level repetition configured by RRC should be one component of FG33-1. Otherwise, repetition will be probably not used from network scheduling since broadcast will be received by UEs in all RRC states. In addition, with an additional dedicated HARQ process reserved by the existing UE implementation should not be the basic assumption to support Rel-17 MBS broadcast reception as discussed in [5]. Whether UE supports an additional dedicated HARQ process for receiving MBS broadcast that is out of the maximum number of 16 HARQ processes per cell should be subject to a UE capability. If UE does not report such capability, UE is not expected to have additional dedicated HARQ process for MBS broadcast reception on top of the maximum number supported currently. Proposal 2: Updating FG33-1 and adding FG33-1-1 as follows in red: 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support of semi-static rate-matching resource set configuration. - Support of rate-matching around LTE CRS. - Support of semi-static configured slot-level repetition for group-common PDSCH; No Yes; Per UE; No; No; Optional without with capability signalling. 33-1-1; Broadcast; Support an additional dedicated HARQ process for receiving MBS broadcast.; 33-1; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [3] | vivo | For broadcast, multiple G-RNTIs facilitate different services, and thus, support of G-RNTIs should be included in broadcast feature 33-1. Proposal 1 Include ‘support of G-RNTIs’ in broadcast feature 33-1. |
| [4] | ZTE | Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed. |
| [5] | Samsung | For FG 33-1, this feature should be optional. Instead, it is proposed to note that this is a basic FG for MBS. Index; Feature group; Components; Mandatory/Optional. 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format [1_0] with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI.; Optional without capability signalling This is a basic FG for MBS.. |
| [6] | NTT DOCOMO | FG 33-1 is basic operations for broadcast reception. So we support to make FG33-1 as a basic FG. We don’t really see the need for capability signalling. Because it is one thing to say that a UE has the capability to receive broadcast and another to say that the UE actually receives broadcast. We doubt that capability signalling is useful for PDSCH scheduling. DCI format for broadcast is now named DCI format 4_0, so component 5 needs to be updated. Proposal 1: FG 33-1 is supported as a basic FG for MBS. Proposal 2: The capability signalling is not necessary for FG 33-1. Proposal 3: Update the components description of FG 33-1 as following. 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 14_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI.. |
| [7] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 5 of FG 33-1, DCI format 1_0 is adjusted as DCI format 4_0; |
| [8] | Qualcomm | 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. - Support of CFR configuration for broadcast. - Support of CORESET and common search space in a CFR for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between other PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - Support of slot-level repetition for group-common PDSCH for MTCH - FFS: Support of TRS for GC-PDCCH/PDSCH configured in a CFR for broadcast; Yes; Per Band; N/A; N/A; Optional without capability signalling. |
| [9] | OPPO | Based on the discussion for broadcast, the components list considers all of the basic functions according to the agreements in NR MBS. For other functions, e.g. FDM-ed unicast PDSCH and GC-PDSCH for broadcast, intra-slot FDM-ed unicast PDSCH and GC-PDSCH for broadcast, multiple G-RNTIs for broadcast and slot-level repetition for broadcast GC-PDSCH, should not be captured in this FG. - FG 33-1 is not adding more components other than the existing listed items as FFS. According to the discussion in last meeting, one question is that whether 33-1 FG Broadcast is a basic FG for NR MBS. According to previous UE feature discussion, basic FG means broadcast itself is supported as an optional function group, UE has to indicate that this FG is supported when the UE supports NR MBS. From our understanding, UEs are required to receive system information in a cell through broadcast, but receiving NR MBS services through broadcast is not mandatory. On the other hand, some UE may not be able to connect to cellular network, and receiving NR MBS services through broadcast is the only method if it is interested for MBS reception. Furthermore, MBS services through broadcast transmission can also be received by RRC_CONNETED UEs, so the design and component should consider all the UEs in different RRC states. Compared with other FGs in NR MBS, broadcast reception for UEs is more general and basic. Therefore, FG 33-1 is supported as a basic FG for Rel-17 NR MBS. - FG 33-1 is supported as a basic FG for Rel-17 NR MBS. For capability signaling, it is not necessary for a UE to send capability signaling to inform network. First, the use case and motivation for reporting this capability by UE is not clear. Furthermore, for the transmission type of broadcast, there is no target UEs, in other word, any UE can be the target. There is no need for network to know which UE has this kind of capability, which seems useless for network. - The capability signaling is not necessary for UEs supporting MBS broadcast in FG 33-1. |
| [11] | Apple | Regarding the component 6 of FG33-1, it’s related to simultaneous reception of multicast PDSCH and the PDSCH carrying SIB/paging/RA response. It can be categorized as three cases, i.e., simultaneous receptions of MTCH with MCCH, simultaneous receptions of multiple MTCHs with different G-RNTIs and simultaneous receptions of MTCH with SIB/paging/RA response. The inter-slot TDM reception can be applied to three cases. Proposal 1: Updating the component 6 of FG33-1 to support of inter-slot TDM reception, - between group common PDSCHs associated with different G-RNTIs, or - between group-common PDSCHs associated with G-RNTI and MCCH-RTI, or - between group-common PDSCH associated with MCCH-RNTI/G-RNTI and PDSCH associated with SIB/paging/RA response. Regarding the repetition of PDSCH carrying MTCH, the following agreements were reached in last RAN1 meeting. Thus, corresonping FG should be introduced to support MTCH repetition. In addition, similar as multicast PDSCH repletion, two different RRC parameters, i.e., pdsch-AggregationFactor and repetitionNumber, can be used to indicate the MTCH repetition. Therefore two separated FGs can be introduced. Agreement: [2] For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support: - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI. - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.. Proposal 2: Introducing two separated FGs to support MTCH repetition via configuration of pdsch-AggregationFactor or repetitionNumber respectively. |
| [12] | Xiaomi | For FG 33-1, it is extensively discussed whether multiple CFR can be configured for RRC IDLE/INACTIVE UEs. In RAN1#106 e-meeting, the following agreement was achieved: Agreement: Only one CFR can be configured for group-common PDCCH/PDSCH carrying MCCH for broadcast reception with UEs in RRC_IDLE/INACTIVE state.. Furthermore, the following agreement on the CFR configuration for broadcast reception with RRC_IDLE/RRC_INACTIVE UEs was achieved in RAN1#107 e-meeting. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx; - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx - PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.. Based on the above two agreements, it can be observed single CFR configuration should be captured in component 3) of FG33-1 in order to better reflect the RAN1 agreement. For the DCI format used to schedule broadcast, DCI format 4_0 is captured in TS38.212, hence component 5) should be modified accordingly. Furthermore, component 6) describe the TDM transmission between unicast PDSCH and group-common PDSCH in different slots. However, unicast PDSCH is only available after UE enters RRC_CONNECTED state. Proposal 1: For UE feature group 33-1, the following modification on component 3), 5) and 6) should be adopted: - 3) Support of single CFR configuration for broadcast. - 5) Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - 6) Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots in RRC_CONNECTED state. |
| [13] | MediaTek | In the latest approved TS 38.212 spec, the three DCI formats were defined for MBS broadcast and multicast that DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. In order to align the MBS DCI format naming, we suggest the following proposal: Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. Considering the concept of group-common PDCCH/PDSCH is used for broadcast and multicast, it may cause some reader confused. Therefore, we want to clarify the concept more clearly with some constrains, e.g., group-common PDCCH/PDSCH for multicast or broadcast, respectively. Proposal 2: Clarify the concept of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast. Regarding the discussion of which granularity type definition should be defined, from our view, the different carriers within the same band will have different characteristics or usage purpose, especially for considering the n28 is a shared band by CBN and CMCC. What’s more, considering the IODT testing cost, we prefer the type definition from UE features should be based on the granularity per FSPC. Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC. Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception since we also have agreed that the CFR for MCCH and MTCH can be configured by SIBx. Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;. Proposal 4: For FG 33-1, adding a note that “For component 3, only one CFR is supported for broadcast and the CFR is configured by SIBx”. For the slot-level repetition for MTCH, the following agreements were achieved in last meeting: Agreement For RRC_IDLE/INACTIVE UEs, for slot-level repetition for MTCH, support: - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format 1_0 with the G-RNTI. - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Broadcast - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.. Thus, we suggest to add new separate FGs to capture the repetition capability. Considering the two repetition schemes are scheduled by different mechanism, e.g., Config A is for semi-static and Config B is for dynamic indication, we prefer to split them as to two separate FGs, e.g., the first FG is for semi-static scheduling and the second FG is for dynamic scheduling. Proposal 5: Adding a new FG 33-1-1 to capture the semi-static slot-level repetition for broadcast MTCH. Proposal 6: Adding a new FG 33-1-2 to capture the dynamic slot-level repetition for broadcast MTCH. 33-1; Broadcast; - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI.; Yes; Per UE Per FSPC; No; No; For component 3, only one CFR is supported for broadcast and the CFR is configured by SIBx; Optional without capability signalling. 33-1-1; Semi-static slot-level repetition for broadcast MTCH; - Support of semi-static slot-level repetition for broadcast MTCH.; 33-1; Per FSPC; For component 1 the repetition number can be n2, n4, n8 slots if pdsch-AggregationFactor configured; Optional with capability signalling. 33-1-2; Dynamic slot-level repetition for broadcast MTCH; - Support of dynamic slot-level repetition for broadcast MTCH.; 33-1; Per FSPC; For component 1 the repetition number can be n2, n3, n4, n5, n6, n7, n8, n16 slots if repetitionNumber configured; Optional with capability signalling. |
| [14] | CMCC | The most fundamental question is whether the FG 33-1 is a basic FG for MBS. Considering to proceed the commercial deployment of MR MBS and reduce the market fragmentation, especially considering there are much different capabilities in Rel-17 NR MBS, e.g., DCI based HARQ-ACK enable/disable, insertion type-1 codebook for multicast, we think it is necessary to define the basic FG both for broadcast and multicast. Proposal 1. FG 33-1 is supported as a basic FG for MBS. Another remaining issue is capability signaling of FG 33-1. Considering this feature is applied for three RRC states and RRC_IDLE/IANCTIVE UEs can not report UE capability, we think the FG 33-1 should be optional without capability signaling. Proposal 2. The signaling of FG 33-1 should be optional without capability signaling. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-1: - Confirm it is per UE. Further discussion needed on whether there is a need for FR1/FR2 differentiation - Optional with capability signalling. The network needs to know if any UE supports the feature. Alternatively, it can be left for RAN2 to decide on this aspect, as currently done for some UE power saving features (e.g. FG 29-1). - Per UE |
| [16] | Ericsson | In the feature description, the DCI format numbers should be updated to reflect the newly introduced broadcast DCI format number. In addition, the feature should also list support for slot-level repetition of the MTCH PDSCH. Agreement: For broadcast reception with UEs in RRC_IDLE/INACTIVE states, support slot-level repetition for MTCH.. The changes in the feature are reflected changed marks below: 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support of slot-level repetition for MTCH.. |

Discussion

[FL1] High priority question 2-1:

- Companies are encouraged to provide views on whether to add a new component into FG 33-1 or separate FGs for support of slot-level repetition for MTCH

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Yes. We support slot-level repetition as component of FG33-1, otherwise, NW may not use it at all because some UE may not support it if it is separate FG. We also have proposed to add one more FG 33-1-1 Support an additional dedicated HARQ process for receiving MBS broadcast. It was signaling impact and should be discussed with high priority. |
| MediaTek | Considering the FG16-2b-5 as defined in Rel-16 and supporting of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 is optional, We think it is best to align with the legacy UE behavior and support slot-level repetition for MTCH can be as a separate FGs. |
| ZTE | We support to add a new component into FG33-1 for slot-level aggregation. Broadcast PDSCH is common to all UEs. If some UEs support while some other UEs don’t support slot-level aggregation, then the gNB has to take the worst case into account. |
| Qualcomm | We support adding the slot-level repetition for MTCH as component of FG33-1 |
| Xiaomi | We prefer to keep support of slot-level repetition for MTCH as a component of FG33-1. |
| CMCC | Support, otherwise NW cannot know which UE supporting PDSCH repetition and will not scheduling PDSCH with repetition. |
| CATT | We support adding a new component of FG33-1 for slot-level aggregation. |
| Nokia, NSB | Adding a component is enough. |
| vivo | We support to add slot-level repetition for MTCH as a new component into FG33-1 by considering broadcast recipient is not known to network. |
| NTT DOCOMO | We support to add a new component in FG 33-1 for slot-level repetition for MTCH. |
| OPPO | Slot-level repetition for MTCH can be a separate FG from FG 33-1. Besides, the components captured in current FG33-1 are aligned with the agreements as well as to fulfill the basic function requirements for broadcast reception, therefore, we suggest not adding new component into FG 33-1. |
| Lenovo, Motorola Mobility | Support. |
| Apple | Slot-level repetition for MTCH can be a separate FG from FG 33-1, as the repetition is not mandate feature to support broadcast service. |
| Moderator | Summary of companies view - Add a new component: Huawei, HiSilicon, ZTE, QC, Xiaomi, CMCC(?), CATT, Nokia, NSB, vivo, DOCOMO, Ericsson - Separate FG: MTK, OPPO, [Apple] Given that majority companies support to add a new component, following proposal is made [GTW1] High priority proposal 2-1: - A new component for support of slot-level repetition for MTCH is added into FG 33-1 Also, Huawei, HiSilicon proposed to add one more FG 33-1-1 Support an additional dedicated HARQ process for receiving MBS broadcast, to be discussed further in the next round. |
| FL2 | Following agreement was made in the GTW session on Jan 18. Agreement - A new component for support of higher layer configured slot-level repetition up to 8 for MTCH is added into FG 33-1 - FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG Regarding the FFS, we have not discussed much so far. Companies are encouraged to provide view on the FFS. Please also try to address the concern from other side. |
| ZTE | Regarding the “DCI indicated slot-level repetition up to [8 or 16] for MTCH”, we are ok to make it as a separate UE capability. It would be better if we can make it as a separate UE capability applicable to both broadcast and multicast. Limiting it to 8 is acceptable for us. |
| vivo | When considering DCI indicated slot-level repetition as a separate UE capability, we are not quite sure how to apply this feature for broadcast, under assumption that UEs in RRC idle/inactive can’t report such capability. More clarifications on this is appreciated. |
| Spreadtrum | Fine to split DCI indication as one separated FG. Regarding vivo’s question, our understanding is that gNB could apply DCI indicated repetition based on some prior information, e.g, and the capability information from connected state, MBS interest indication. If UE not support the capability, UE could choose not to receive broadcast information. |
| MediaTek | We support the DCI indicated repetition as a separate FG. Besides, regarding the repetition number, we think up to 8 repetition is enough since broadcast is a best effort reception. Thus, we suggest adding the following FG: 33-1-1; Dynamic slot-level repetition for broadcast MTCH; - Support of dynamic slot-level repetition for broadcast MTCH.; 33-1; For component 1 the repetition number can be n2, n3, n4, n5, n6, n7, n8, slots if repetitionNumber configured; Optional with capability signalling. |
| Qualcomm | Whether the DCI indicated slot-level repetition is a separate UE capability, we can follow the majority view. But it is unnecessary to limit to 8. It’s complicated that there are different restriction on the repetition number configuration in TDRA table, dependent on DCI format. |
| NTT DOCOMO | We share similar view with vivo. Since gNB cannot know the capabilities of IDLE UEs, gNB may not be able to use DCI indicated repetition unless it is included in the basic FG. |
| Xiaomi | We not sure the motivation of splitting semi-static slot-level repetition and dynamic slot-level repetition into two separate FGs. Considering it is agreed that semi-static slot-level repetition is included in FG 33-1, can proponents clarify what is the fundamental difference with respect to UE behaviour between semi-static and dynamic repetition? |
| Huawei, HiSilicon | DCI indicated slot-level repetition can be a separate FG. Whether restricting the number to 8, QC has a point that it might be more complicated if the number is different for different DCI format even though it could be a separate UE capability from that for unicast. For broadcast, all FGs as components for broadcast might be too challenging to UE implementation. For some separate UE FG for broadcast, if the capability can be reported to NW, NW may be able to use this information for better scheduling but it is up to NW how to use it. The motivation for DCI indication as separate FG was because semi-static is R15 feature and DCI indicating repetition is R16 FG and we suppose R15 FG might be more extensively used than the R16 FG. |
| Nokia, NSB | Limiting the FFS to 8 is acceptable to us. If it is straightforward for UEs to support this DCI controlled capability, then we would prefer to have it as part of FG 33-1. Having a mixed population of UEs will mean defaulting to the slower/less dynamic non-DCI method for ALL UEs sharing the broadcast service. |
| CATT | We share same concern with vivo. If the DCI indicated slot-level repetition is defined as a separate UE capability, it is difficult to ensure that all UEs that want to receive broadcast have this capability. Thus, it will result in inability to use slot-level repetition in most cases. |
| Ericsson | OK to have the DCI indicated slot level repetition in a separate capability. We agree with Qualcomm that the limitation to 8 is not necessary. |
| Moderator | Summary of companies view - Support another FG for DCI indicated slot-level repetition: ZTE, SPRD, MTK, HW/HiSi, Nokia/NSB, E/// - gNB could apply DCI indicated repetition based on some prior information - Up to 8: ZTE, MTK, Nokia/NSB - broadcast is a best effort reception - Up to 16: QC, HW/HiSi, E/// - applicable to both broadcast and multicast: ZTE - More clarification is necessary: vivo, Xiaomi, DCM, CATT Moderator understanding: If option 1 (separate FG from 33-1), for idle/inactive modes, NW does not know whether a UE supports this FG or not. If not supported and DCI indicates repetition, UE is not expected to receive broadcast information. For connected mode, gNB can configure a group of UEs not supporting this FG without DCI indication for repetition, and another group of UEs supporting this FG with DCI indication. If option 2 (Component in 33-1), UE supporting FG 33-1 always supports reception of DCI indicating repetition and to receive broadcast information in idle/inactive/connected modes This issue can be further discussed after the GTW session |
| FL3 | Companies are invited to further provide view whether to support the capability for DCI indicated slot-level repetition as a separate FG from 33-1 (Option 1) or as a component in FG 33-1 (Option 2) considering the moderator understanding as mentioned above. |
| Apple | As we commented online, the technical details of RRC indication and DCI indication of repetition number are not fully discussed under AI 8.12, so the difference is not clear. To be safe, we tend to prefer option1. As for the maximum repetition, up to 16 should be fine. |
| CMCC | Support option 2. If DCI based PDSCH repetition is a separate FG, gNB will never use it since doesn’t know the UE capability. |
| vivo | Support option 1. Even for unicast, DCI indicated slot-level repetition is a separate capability, we don’t see the motivation to include it in basic feature for multicast. |
| OPPO | As we agreed on GTW that higher layer configured slot-level repetition is added into FG 33-1, correspondingly the DCI indicated repetition should also be captured together within FG 33-1. We share the similar view with CMCC that a separate FG (Option 1) may lead to useless of this FG. |
| Huawei, HiSilicon | Slot-level repetition was optional in R15 and R16. Supporting both as components are too demanding for UE implementation. Considering it is broadcast which is supposed to be as robust as possible, semi-static repetition is sufficient. Dynamic repetition was introduced in R16 m-TRP, which may not be implemented by all UEs. Considering both sides from NW and UE, we support option1, i.e., only semi-static repetition is the component. |
| CATT | We are fine with option 1. |
| Spreadtrum | Prefer option 1. |
| Nokia, NSB | As mentioned by CMCC, the only safe alternative is option 2, as this is a broadcast feature. |
| Moderator | Summary of companies view - option 1 (separate FG from 33-1): Apple, vivo, HW/HiSi, CATT, SPRD, - option 2 (Component in 33-1): CMCC, OPPO, Nokia/NSB [GTW3] Still companies view is split. Further discussion is necessary in the GTW session |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given companies still prefer different options, no further discussion is planed in this meeting. Let’s further discuss in the next meeting. |

[FL3] High priority question 2-1a:

- Companies are encouraged to provide views on whether to add an FG for support of an additional dedicated HARQ process for receiving MBS broadcast

| Company | Comment |
| --- | --- |
| FL3 | This issue was raised by HW/HiSi. The motivation is copied as below In addition, with an additional dedicated HARQ process reserved by the existing UE implementation should not be the basic assumption to support Rel-17 MBS broadcast reception as discussed in [5]. Whether UE supports an additional dedicated HARQ process for receiving MBS broadcast that is out of the maximum number of 16 HARQ processes per cell should be subject to a UE capability. If UE does not report such capability, UE is not expected to have additional dedicated HARQ process for MBS broadcast reception on top of the maximum number supported currently. |
| Apple | Don’t need to discuss. As we had related agreement already on Jan.20 GTW. Conclusion Additional HARQ process(es) is(are) not introduced for Rel-17 MBS broadcast reception on serving cell. Note: The UE is not expected to support hardware for more HARQ processes for receiving broadcast in Rel-17 in addition to the maximum number of HARQ processes supported for receiving unicast in Rel-16, i.e. the HARQ process resources are shared between broadcast, unicast and multicast |
| OPPO | Same view with Apple that no necessary to discuss about it since the conclusion has been made on this issue. |
| Huawei, HiSilicon | Yes, agree. Given the new progress just achieved, now this proposal is not needed. |
| Moderator | More input is necessary |

Medium priority proposal 2-2:

- FG 33-1 is supported as a basic FG for MBS

  - Support: ZTE, Samsung, NTT DOCOMO, OPPO, CMCC

| Company | Comment |
| --- | --- |
| Samsung | Support |
| OPPO | As discussed and clarified in the past meetings on the “basic FG”, FG 33-1 can be supported as a basic FG for MBS as the broadcast transmission and reception by considering that it contains the basic functionalities of broadcast. |
| CATT | Support. |

Medium priority question 2-3:

- Companies are encouraged to provide views on whether capability ignalling is necessary for FG 33-1, i.e., whether to support as optional with capability ignalling or optional without capability ignalling

  - Optional with capability ignalling: Huawei, HiSilicon, Nokia, NSB

  - Optional without capability ignalling: NTT DOCOMO, Qualcomm, OPPO, CMCC, MediaTek

  - Optional: Samsung

| Company | Comment |
| --- | --- |
| Samsung | We would like to hear proponent arguments for each direction. This depends on whether “the gNB needs to know” is yes or no. |
| OPPO | Optional without capability signaling. First, the motivation and benefit to report capability to network. Second, broadcast has no target reception UEs, or all in-coverage UEs are the target UEs. There is no necessary to report this capability to network. In another words, UEs should support such kind of capability as the basic capability for supporting MBS reception. |
| LG Electronics | Optional without capability signaling gNB cannot know broadcast reception of UE in RRC_IDLE/INACTIVE because gNB does not know which cell an idle UE is located in. We do not understand how an idle UE can report capability without entering RRC_CONNECTED. |

Medium priority question 2-4:

- Companies are encouraged to provide views on whether the type of FG 33-1 should be per UE, per band or per FSPC

  - Per UE: Huawei, HiSilicon, Nokia, NSB

  - Per band: Qualcomm

  - Per FSPC: MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Medium priority question 2-5:

- Companies are encouraged to provide views on whether FR1/FR2 differentiation is necessary for FG 33-1

| Company | Comment |
| --- | --- |
| Qualcomm | Yes. But if the type of FG33-1 is per band, it implicitly saying FR1/FR2 differentiation for FG33-1. |
| LG Electronics | FR1/FR2 differentiation is necessary for FG 33-1 |

Low priority proposal 2-6:

- Components of FG 33-1 is revised as

  - Component 1: Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI.

  - Component 2: Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH.

  - Component 3: Support of single CFR configuration for broadcast.

  - Component 4: Support of CORESET and common search space in a CFR for broadcast.

  - Component 5: Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast.

  - Component 6: Support of inter-slot TDM between other PDSCH and group-common PDSCH for broadcast in different slots in RRC_CONNECTED state.

  - Add a component for support of semi-static rate-matching resource set configuration.

  - Add a component for support of rate-matching around LTE CRS.

  - Add a component for support of  G-RNTIs.

| Company | Comment |
| --- | --- |

Low priority question 2-7:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-1 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-2 to 33-2-x: Dynamic scheduling for multicast

In [1], FGs 33-2 to 33-2-x are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | To align with TS 38.212 CR, the DCI format 1_0 in the 4th component of FG33-2 should be updated to 4_1 (based on DCI format 1_0). There was discussion in RAN1#107-e meeting that whether DCI format 4_2 (based on DCI format 1_1) should be the component of FG33-2. Some UE vendor views that it is preferred to have a common feature set for broadcast and multicast as much as possible at the early stage of implementation of Rel-17 NR MBS, though DCI format 1_1 is mandatory for legacy UE receiving unicast. For early implementation and deployment, it could be compromised to both network and UE to define a separate UE capability whether support DCI format 4_2 for scheduling multicast. The UE capability for multicast and broadcast is preferred to be separate although the basic feature sets for them are strived to be common. When considering UE capability for supporting both multicast and broadcast, it would be preferred to describe it in FG for multicast instead of defining additional FG for supporting both. For example, for the 5th component of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots where the group-common PDSCH is intended for multicast, it could be updated to consider broadcast group-common PDSCH as well if UE supports FG33-1. As for FG33-1 broadcast, support of semi-static configured slot-level repetition of group-common PDSCH can be one of components of FG33-2 for multicast. Regarding whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback, whether to separate the capability for support of PTP retransmission for multicast, and whether to separate the capability for support of PTM retransmission for multicast, from network perspective, they are the components to be included in FG33-2 preferably. As discussed in RAN1#107-e meeting, from UE vendor perspective, these could be separate UE capabilities. As compromised between network and UE, support of NACK-only feedback and PTM retransmission can be the components and support of ACK/NACK based feedback and PTP retransmission can be separate UE capabilities. Proposal 3: Updating FG33-2, adding FG33-2-1, FF33-2-2, FG33-2-3, deleting FG33-2b, deleting FG33-3-1, deleting FG33-4, and deleting FG33-7 as follows in red: 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support of semi-static configured slot-level repetition for group-common PDSCH - Support of NACK-only based feedback for dynamic multicast scheduling - Support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling. - Support of PTM retransmission for dynamic multicast scheduling; Yes; Per UE; No; No; Optional with capability signalling. 33-2-1; Dynamic scheduling for multicast by DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-2-2; ACK/NACK-based feedback for dynamic scheduling for multicast; - Support of ACK/NACK based feedback for multicast dynamic scheduling. - Support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling for dynamic scheduling of multicast.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-2-3; PTP transmission for PTM HARQ retransmission; Support of PTP transmission for PTM HARQ retransmission for dynamic scheduling; 33-2-2; Yes; Per UE; No; No; Optional with capability signalling. 33-2-4; Group-common DCI indicating the enabling/disabling HARQ-ACK feedback for dynamic scheduling; Support of DCI-based enabling/disabling HARQ-ACK feedback per configuration of RRC ignalling for each G-RNTI.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. FG33-2-x is currently worded for the number of supported G-RNTIs for multicast. In light of avoiding device hardware impact to facilitate implementation and deployment of the feature, receiving multicast and broadcast may take the resources built for unicast from UE implementation perspective without changing the existing hardware, e.g., the maximum number of RNTIs UE supports that was dimensioned for unicast will be shared with multicast as well when to support MBS multicast. Therefore, in addition to the value of the maximum number of G-RNTIs supported for multicast in FG33-2-x, UE needs to report whether such values are the numbers that are to be reduced or are to be increased for FG33-2-x, so that network can make full use of UE capability for better supporting services for unicast, multicast, and broadcast. In addition, UE can report MII for the interested broadcast reception, reporting the number of G-RNTIs supported for broadcast may not be needed. FG33-2-x can be for multicast only. Proposal 4: Updating FG33-2-x as follows in red: 33-2-x; Multiple G-RNTIs for group-common PDSCHs; - The maximum number of G-RNTIs supported for multicast. FFS detailed values. - Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [3] | vivo | For multicast, the capability for group-common PDSCH reception doesn’t have to be coupled with HARQ-ACK feedback. In general, there exist three reactions when UE receives group-common PDSCH: no HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; and sends ACK/NACK based HARQ-ACK feedback. Thus, neither ACK/NACK nor NACK-only based HARQ –ACK feedback should be included in 33-2. Proposal 2 Neither ACK/NACK nor NACK-only based HARQ –ACK feedback should be included in 33-2. Furthermore, when UE doesn’t support sending HARQ-ACK feedback, there will be no PTP retransmission or PTM retransmission and thus, support of PTP retransmission and support of PTM retransmission for multicast should be separated from feature 33-2. Proposal 3 Separate ‘support of PTP retransmission for multicast’ from 33-2. Proposal 4 Separate ‘support of PTM retransmission for multicast’ from 33-2. |
| [4] | ZTE | For multicast reception, to avoid UE fragmentation and facilitate commercial deployment, it is fundamental to group some necessary UE features into basic feature group FG33-2. Otherwise, it may jeopardize the commercial deployment. From our perspective, at least the following can be grouped together into FG33-2. 1. Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. 6. Support ACK/NACK based HARQ-ACK feedback 7. Support PTM retransmission for multicast. 8. Support PTP retransmission for multicast. |
| [5] | Samsung | As discussed in our contribution [3], it is beneficial for a UE to indicate the support of combining a PTM initial transmission and a PTP retransmission in case of different circular buffer. A new capability should be defined to indicate whether the UE can or cannot combine. Therefore, we propose to separate component 8 of FG 33-2 and use it as a pre-requisite of the combining capability. |
| [6] | NTT DOCOMO | DCI formats for multicast are now named DCI format 4_1/4_2, so component 4 needs to be updated. Regarding support of DCI format 4_2, we don’t see the need to separate the capability. If DCI format 4_2 can’t be used unless all Ues in the group support DCI format 4_2, then the scheduling constraints would be too large. HARQ-ACK feedback and HARQ retransmissions would be basic operations to ensure the reliability of multicast services. We would like to keep component 6, 7 and 8 in FG33-2. Type should be per UE. Proposal 4: Update the components description of FG 33-2 as following. 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_11_0 / 4_21_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast.. Proposal 5: Type of FG 33-2 is per UE. Type of FG 33-2b should be the same as the prerequisite FG, FG 33-2 (i.e., per UE). Proposal 6: Type of FG 33-2b is per UE. Type of FG 33-2-x should be the same as the prerequisite FG, FG 33-2 (i.e., per UE). Proposal 7: Type of FG 33-2-x is per UE. |
| [7] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 4 of FG 33-2, DCI format 1_0/1_1 is adjusted as DCI format 4_1/4_2; In Rel-17 MBS, HARQ-ACK feedback is introduced for multicast, and ACK/NACK based HARQ feedback and NACK-only are included. For ACK/NACK based feedback, the PUCCH resource for each UE in the group is orthogonal by gNB’s implementation, to improve the reliability. But in our understanding, HARQ-ACK feedback is not the basic operation of multicast, as we have done in Rel-16 sidelink PC5, and also like FG33-4. In general, multicast transmission is more reliability than unicast. In addition, slot-level repetition is also can be utilized for the reliability. From the perspective of UE, power cost can be saved if it doesn’t support to feedback. Thus, we think whether to support HARQ-ACK feedback is up to UE capability. Proposal 2: Component 6 in FG33-2 can be split as one separated FG, and as one optional UE capability. Proposal 3: Component 7 in FG33-2 can be split as one separated FG, and as one optional UE capability. Proposal 4: Component 8 in FG33-2 can be split as one separated FG, and as one optional UE capability. |
| [8] | Qualcomm | 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 4_1with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - [Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 4_1 with CRC scrambled with G-RNTI] - ignalling; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-2a; Dynamic multicast with DCI format 4_2; - Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-2b; ACK/NACK-based feedback for Dynamic scheduling for multicast and PTM retransmission for Dynamic scheduling for multicast; - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC ignalling - Support of PTM retransmission for dynamically scheduled multicast based on G-RNTI; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-2c; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; - Support of DCI-based enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-RNTI for multicast by RRC ignalling by using DCI format 4_2; 33-2a, 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-2d; PTP retransmission for Dynamic scheduling for multicast; - Support of PTP retransmission for dynamically scheduled multicast based on C-RNTI; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-2e; TCI-state configuration for multicast group-common PDSCH; - Support of M’ TCI-state configurations within the PDSCH-Config-Multicast; 33-2; Yes; Per FSPC; N/A; N/A; FFS: values of M’; Optional with capability signalling. 33-2-x; Multiple G-RNTIs for group-common PDSCHs; Capability on max number of G-RNTIs for multicast FFS details.; 33-2; Yes; Per FSPC; N/A; N/A; FFS: Values = {2, …}; Optional with capability signalling. 33-2-y; Multiple G-CS-RNTIs for SPS group-common PDSCHs; Capability on max number of G-CS-RNTIs for multicast FFS details.; 33-5-1; Yes; Per FSPC; N/A; N/A; FFS: Values = {2, …}; Optional with capability signalling. |
| [9] | OPPO | For FG of dynamic scheduling for multicast, at least component 1~5 in the table above are considered as basic functions for a UE. For HARQ-ACK feedback functionality, including ACK/NACK based scheme, the default status/value is “disabled”. “support ACK/NACK based HARQ-ACK feedback” can be separated from FG 33-2. Correspondingly, “enabling/disabling ACK/NACK by RRC signaling” can also be separated from FG 33-2 since using RRC signaling to enable/disable ACK/NACK feedback is general. The current component 6 in square bracket can be separated to 33-2a. Considering about retransmission scheme of PTP and PTM, PTP retransmission for multicast for ACK/NACK based feedback is needed because the ACK/NACK feedbacks can be differentiated per UE through UL resources. PTM retransmission scheme for ACK/NACK based feedback can be then separated as another FG. The proposals for FG 33-2 with related table is provided below. - “Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling” can be separated from FG 33-2 as FG 33-2a. - “Support PTP retransmission for multicast” can be merged into FG 33-2a together with ACK/NACK based feedback for dynamic scheduling for multicast. - “Support PTM retransmission for multicast” can be separated from FG 33-2a as in 33-2c. 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. 33-2a; ACK/NACK-based feedback for dynamic scheduling for multicast; - Support ACK/NACK based HARQ-ACK feedback. - Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling. - Support PTP retransmission for multicast; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33-2c; Retransmission scheme for dynamic scheduling for multicast; Support PTM retransmission for multicast; 33-2, 33-2a; Yes; Per UE; No; No; Optional with capability signalling. |
| [10] | Intel | - FG 33-2 - Item 6: The support of ACK/NACK based HARQ feedback should be separated into a separate dependent feature group since the default mode is HARQ disabled and RRC is used to enable it. UEs may not support HARQ feedback. - Separate support of DCI 1_1 and rename 1_0/1_1 to 4_1/4_2 respectively - The PTP and PTM retransmission for MBS should be separated from 33-2 and added to dependent FG on ACK/NACK based HARQ feedback - FG 33-2b: - Add NACK-only based feedback option based on latest agreements. 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast. 33-2b; DCI-based enabling/disabling ACK/NACK-HARQ/ACK based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK and NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a. |
| [11] | Apple | For component 6 Support ACK/NACK based HARQ-ACK feedback, it would be preferred this component is treated as an independent FG, the fundamental function for multicast service is receiving the multicast data without HARQ feedback, any feedback mode is the enhanced feature which is subjected to UE capability. In additional, supporting enable/disabling HARQ-ACK feedback is another UE feature. For component 8 Support PTP re-transmission for multicast, this component can be removed from FG32-2 as independent FG. PTM is considered the basic function for multicast. Additional capability would be required for UE to support PTP retransmission. Proposal 3: Supporting the following components as independent FG from FG33-2, - Support of PTP retransmission for multicast - Support of ACK/NACK based HARQ-ACK feedback |
| [13] | MediaTek | Regarding the FG 33-2, it has the following remaining issues needed to be further discussed: - FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast - FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback - FFS whether to separate the capability for support of PTP retransmission for multicast - FFS whether to separate the capability for support of PTM retransmission for multicast. Regarding the CFR number for multicast reception, the following agreement was achieved in previous RAN1 meeting: Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.. Thus, we prefer to update the 2nd component based on the latest agreement. Proposal 7: For FG 33-2, adding a note that “for component 2, only one CFR is supported for multicast reception”. In previous RAN1 meeting, the HARQ-ACK enabling/disabling was agreed for multicast reception, and the corresponding agreement is copied as following. Agreement: Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback. Agreement: Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - ……………………………….. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. - ……………………….. Agreement: The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling. Agreement: If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.. According to the agreements, we can see that the HARQ-ACK can be disabled/enabled and the UE capability for enabling/disabling is introduced. In other words, the UE supporting multicast does not meant it is definitely to support HARQ-ACK. Thus, we suggest the HARQ-ACK feedback can be as a separate FG. Proposal 8: For FG 33-2, the HARQ-ACK feedback mechanism can be as a separate FG. Accordingly, the retransmission schemes (e.g., PTM scheme or PTP scheme) are not always needed since HARQ ACK can be disabled, and the two schemes also can be as a separate FGs. Proposal 9: For FG 33-2, the PTM retransmission for multicast can be as a separate FG. Proposal 10: For FG 33-2, the PTP retransmission for multicast can be as a separate FG. Regarding whether to separate the capability for support of DCI format 4_2 for multicast, we think the DCI format 4_1 for multicast is sufficient and the DCI format 4_2 can be defined as the similar with DCI format 1_2 that as a separate UE capability. Proposal 11: For FG 33-2, monitoring DCI format 4_2 can be as a separate FG. 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1 scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - [Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast FFS whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE Per FSPC; No; No; For component 2, only one CFR is supported for multicast reception; Optional with capability signalling. 33-2-1; Monitoring DCI format 4_2; Support monitoring DCI format 4_2 for DL scheduling; 33-2; Per UE Per FSPC; Optional with capability signalling. 33-2-2; ACK/NACK based HARQ-ACK feedback; Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.; 33-2; Per UE Per FSPC; Optional with capability signalling. 33-2-3; PTM retransmission for multicast; Support PTM retransmission for multicast; 33-2, 33-2-2; Per UE Per FSPC; Optional with capability signalling. 33-2-3; PTP etransmission for multicast; Support PTP retransmission for multicast; 33-2,33-2-2; Per UE Per FSPC; Optional with capability signalling. In the ongoing RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following. one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.. Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear. In addition, supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. We think one G-NRTI is sufficient for Rel-17 UE supporting multicast services. Proposal 12: Only one G-RNTI is sufficient for Rel-17 UE supporting multicast services. Proposal 13: For FG 33-2-X, RAN1 needs to re-consider the motivation for UE supporting multiple G-RNTIs. Proposal 14: For FG 33-2-X, adding a note that “For component 1, the candidate values of m is：{0,1}”. 33-2-x; Multiple G-RNTIs for group-common PDSCHs; - Capability The maximum on number (e.g., m) of G-RNTI for multicast groupcast FFS details.; 33-2; Yes; Per UE Per FSPC; No; No; For component 1, the candidate values of m is：{0,1}; Optional with capability signalling. |
| [14] | CMCC | As the discussion above, it is necessary to define basic FG bother for broadcast and multicast. Proposal 3. FG 33-2 is supported as a basic FG for MBS. Regarding the component of HARQ-ACK feedback, we think both NACK-only and ACK/NACK should be included, since if UE has supported ACK/NACK based HARQ feedback, it will introduce additional UE complexity to support NACK-only based HARQ feedback and selection of ACK/NACK based and NACK-only based HARQ feedback is usually according to the network PUCCH resource overhead not UE capability. If NACK-only is excluded from FG 33-2, it will cause gNB more complex to allocate and schedule PUCCH resources. Regarding the PTP retransmission for multicast, we think it should not be split, since the most reliability improvement gain for multicast service is provided by the combination of ACK/NACK based HARQ feedback and PTP retransmission. If PTP retransmission is excluded from FG 33-2, gNB can only use PTM retransmission which causes ACK/NACK based HARQ feedback has no technique advantage than NACK-only based HARQ feedback, and is also not the intention to introduce ACK/NACK based HARQ feedback for NR multicast. Regarding the DCI format 1_1, we think it should not be split as well, the most important consideration is the scheduling restriction of only using DCI format 1_0, e.g., the frequency scheduling graduality, priority indication and so on. To maintain the DCI format 1_1 as the basic feature for multicast is better for MBS commercial deployment. Regarding the HARQ-ACK feedback mode selection, since the selection signalling is based on UE-specific RRC signalling, it can be as a separate capability, but we also fine not to split it. Proposal 4. For FG 33-2: - Support both NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-2; - Don’t support to separate the capability for support of PTP retransmission for multicast; - Don’t support to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast; - The capability for HARQ-ACK feedback mode selection for dynamically scheduled group-common PDSCH for multicast can be separated. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-2: - Add 33-1 as pre-requisite - Merge 33-4 into this one - Per UE - 33-2-x: - Confirm FG and provide a proper FG numbering - Per UE |
| [16] | Ericsson | In the feature description, the DCI formats should be corrected to DCI 4_1 and 4_2 to reflect the changes in 38.212. Moreover, subfeature 6 is still in bracket, but reflects past agreements (comfirming the working assumption) on the HARQ functionality for MBS, which can be enabled/disabled via RRC. Therefore the brackets can be removed. Regarding the FFS to separate the capability for support of PTP retransmission for multicast, we do not see the need for a separate capability. The LBRM discussion concluded with the following agreement: Agreement For the LBRM/TBS determination for PTP retransmission of multicast, Option 2 is supported. - Option 2: based on the LBRM/TBS determination of the legacy unicast PDSCH transmission - Note: The UE is not required to soft combine the PTM initial transmission and the PTP retransmission in case of different circular buffer. This means that there is no additional complexity required from the UE to support PTP retransmission for multicast, and therefore no need for a separate capability. Similarly, PTM retransmission is not different from PTM transmission from the UE complexity perspective. Regarding the FFS on NACK only / and-or ACK NACK based HARQ feedback, it was agreed not to support configuration of both NACK only and ACK NACK feedback for the same G-RNTI. There is also already a separate capability for NACK-only feedback. Therefore, the “basic” feature for dynamic scheduling should only include ACK/NACK based feedback. The FFS can be removed and the brackets on item 6 in the feature can be removed. Regarding the support of DCI 4_2 (referred as multicast 1_1 in the UE feature document), we believe that UE should support both non-fallback and fallback DCI for multicast as part of the core functionality. Fragmenting the feature further will result in making the non-fallback DCI usable in practice. The changes in the feature are reflected changed marks below: 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast. |

Discussion

[FL1] High priority question 3-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-2.

  - Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast.

    - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson

    - Separate FGs: Huawei, HiSilicon, Qualcomm, MediaTek

  - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling.

    - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson

    - Separate FGs: Huawei, HiSilicon, vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, Apple, MediaTek

  - Support of PTM retransmission for multicast.

    - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast.

    - Keep in FG 33-2: Huawei, HiSilicon, ZTE, NTT DOCOMO, Ericsson

    - Separate FGs: vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, MediaTek

  - Support of PTP retransmission for multicast.

    - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast.

    - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson

    - Separate FGs: Huawei, HiSilicon, vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, Apple, MediaTek

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | From NW point of view, NW has more room for flexible scheduling if UE can support all these FGs/components. However, it may not be piratical for UEs to implement all of these. For compromise between NW and UE, support of NACK-only and PTM retransmission may be acceptable to both sides. |
| MediaTek | We support the all the above capabilities as a separate FG from FG 33-2 as we stated in our contribution. |
| ZTE | From our perspective, one compromise is to support type-1 ACK/NACK codebook (or type-2 ACK/NACK codebook) as a component of FG 33-2. From our perspective, ACK/NACK based feedback is more aligned with the legacy implementation, which seems to be a better choice to be included in FG 33-2. |
| Qualcomm | 1) DCI format 4_1 can be included in the basic format for multicast. Support DCI format 4_1 with configurable fields to be a separate FG. 2) need to separate each FG for support of ACK/NACK based HARQ-ACK feedback for multicast, RRC-configured enabling/disabling, GC-DCI-based enabling/disabling (on condition of DCI format 4_2), and RRC-configured feedback mode selection. 3) it can be merged with the support of ACK/NACK based HARQ-ACK feedback for multicast 4) it should be separate FG on condition of FG for ACK/NACK feedback for multicast. |
| Xiaomi | Support of PTM retransmission and support of PTP retransmission should be bundled with ACK/NACK based HARQ-ACK feedback. If support of ACK/NACK based HARQ-ACK feedback is a separate UE capability, support of PTM retransmission and support of PTP retransmission should also be split from FG 33-2. It doesn’t make sense UE support retransmission while it doesn’t support HARQ-ACK feedback. |
| CMCC | The main difference between NR MBS and LTE MBMS/SC-PTM is the introduction of HARQ-ACK feedback to improve the reliability, if such HARQ related features are not merged in the basic multicast feature, the technical advantage beyond LTE will be reduced. |
| CATT | We support keep all these capabilities in FG 33-2. For supporting ACK/NACK based HARQ-ACK feedback, we believe multicast has Qos requirements and it is necessary to support the ACK/NACK based HARQ-ACK feedback. |
| Nokia, NSB | We tend to agree with Huawei’s views here. NACK-only (i.e. merge 33-4 here) and PTM retransmission as components to 33-2 sound like a reasonable compromise. |
| LG Electronics | We prefer separate FGs for the followings: - Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast. - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. Support of PTM/PTP retransmission for multicast can be separate UE capabilities and it does not need to be merged with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast. |
| Vivo | We support separating the capabilities above from FG 33-2. In general, there can be three options when UE receives group-common PDSCH: no HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; and sends ACK/NACK based HARQ-ACK feedback. Thus, neither ACK/NACK nor NACK-only based HARQ –ACK feedback should be included in 33-2. |
| NTT DOCOMO | We would like to keep all these capabilities in FG 33-2. We think HARQ-ACK feedback and HARQ retransmissions are essential features to ensure the reliability of multicast services. |
| OPPO | Support the above 4 proposals to separate them from FG 33-2. |
| Moderator | Summary of companies view - Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast. - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson - Separate FGs: Huawei, HiSilicon, Qualcomm, MediaTek, LGE, OPPO - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson - Separate FGs: Huawei, HiSilicon, vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, Apple, MediaTek, LGE - Support of PTM retransmission for multicast. - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast. - Keep in FG 33-2: Huawei, HiSilicon, ZTE, NTT DOCOMO, Ericsson, Xiaomi - Separate FGs: vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, MediaTek - Whether to merge with the FG for support of NACK-only based HARQ-ACK feedback for multicast. - Keep in FG 33-2 for compromise: Huawei, Nokia, NSB - Support of PTP retransmission for multicast. - Whether to merge with the FG for support of ACK/NACK based HARQ-ACK feedback for multicast. - Keep in FG 33-2: ZTE, NTT DOCOMO, CMCC, Ericsson, Xiaomi - Separate FGs: Huawei, HiSilicon, vivo, Spreadtrum Communications, Qualcomm, OPPO, Intel, Apple, MediaTek Given there is no majority view, further discussion in GTW session is necessary |
| FL2 | This issue could not be discussed in the GTW session on Jan 18. As mentioned above (highlighted in yellow), a compromised proposal was suggested from Huawei and Nokia to merge FG 33-4 (NACK-only based HARQ-ACK feedback for multicast) into FG 33-2 and to keep component 7 (Support PTM retransmission for multicast) in FG 33-2. Therefore, following proposal is made with some revision for clarification. [FL2] High priority proposal 3-1: - FG 33-2 is updated as follows 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of a single CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 1_0 / 1_1 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - [Support of NACK-only ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling NACK-only ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 4_2 with CRC scrambled with G-RNTI for multicast FFS whether how to separate the capability for support of NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether how to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE; No; No; Optional with capability signalling. Companies are encouraged to provide view on whether proposal 3-1 is acceptable or not. Please also try to address the concern from other side |
| ZTE | From our perspective, at least one of the following combinations can be included in FG33-2 as a component. PTM retransmission; PTP retransmission; PTM & PTP retransmission. Type-1 HARQ-ACK; Combination#1; Combination#2; Combination#3. Type-2 HARQ-ACK; Combination#4; Combination#5; Combination#6. NACK-only; Combination#7; N/A; N/A. It would be good to hear companies’ preference on the above combinations. From our perspective, Combination#1 – Combinaiton#6 are preferred than Combination#7 since Type-1 HARQ-ACK and Type-2 HARQ-ACK codebook are there from Rel-15, which may have less implementation impact. In addition, it can give network information on which particular UE is sending ACK/NACK. |
| Vivo | We are not ok with proposal 3-1. Considering 33-2 as a basic feature, neither ACK/NACK nor NACK-only based HARQ –ACK feedback should be included in 33-2. Reliability can be achieved by slot-level repetition. |
| Spreadtrum | Not support proposal 3-1. For multicast, feedback should not be the basic component, like we have done in R16 sidelink. |
| MediaTek | Not support. Share the similar view with vivo/Spreadtrum that HARQ-ACK feedback should be split as a separate FG. Accordingly, the PTP/PTM retransmission also should be as a separated FG. |
| Qualcomm | Not support proposal 3-1. Similar view as vivo, Spreadtrum and MTK. |
| NTT DOCOMO | We support proposal 3-1. |
| Xiaomi | Just clarify our position a bit: we are supportive to separate feedback features from FG 33-1. Which we are not OK is to further split feedback and re-transmission into separate FGs. The point is that if a UE support feedback it automatically supports PDSCH re-transmission, in one way or another. |
| LG Electronics | Not support proposal 3-1. Similar view as vivo, Spreadtrum, MTK, and Qualcomm. |
| Huawei, HiSilicon | If most of UE vendors has concern to support NACK-only and PTM as components, we can also be fine with separate FGs for compromise. |
| Nokia, NSB | Support FL proposal |
| Moderator | Summary of companies view - Support: DCM, Nokia/NSB - Not support: vivo, SPRD, MTK, QC, Xiaomi. LGE, HW/HiSi, - feedback should not be the basic component - Reliability can be achieved by slot-level repetition Assuming reliability is the main concern, there would be two alternatives to address the concern - Alt 1: At least one of HARQ-ACK feedback is included in FG 33-2 - Alt 2: Slot level repetition (FG 33-3-1) is merged with FG 33-2 - Note: As discussed in question 4-2, semi-static repetition can be merged with FG 33-2 This issue will be discussed in the GTW session |
| FL3 | Following agreement was made in the GTW session on Jan 20. Let’s further discuss FFS parts. Agreement - Slot level repetition (FG 33-3-1) is merged with FG 33-2 - FFS whether to include semi-static repetition only or both semi-static and dynamic repetitions - FFS max value of dynamic repetition - FFS max value of semi-static repetition - Components 6/7/8 are separated from FG 33-2, FFS how to separate Regarding repetition, companies showed different preference in the GTW session. Let’s check where we stand now. [FL3] High priority question 3-1a: - Companies are encouraged to provide views on the following - Q1: Whether to include the capability for semi-static repetition only or both capabilities for semi-static and dynamic repetitions in FG 33-2. - Q2: Max value of dynamic repetition if included in FG 33-2 - Q3: Max value of semi-static repetition Regarding component 6/7/8, following proposal is made as a starting point for further discussion. [FL3] High priority proposal 3-1b: - Add following FGs - FG 33-2a: RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2c: PTM retransmission for multicast - FG 33-2d: PTP retransmission for multicast 33. NR_MBS; 33-2a; RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; PTM retransmission for multicast; Support of PTM retransmission for multicast; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. |
| MediaTek | priority question 3-1a: Regarding the concern of reliability of multicast reception, we have agreed that semi-static repetition was introduced in FG33-2. Besides, considering the dynamic indication was introduced in Rel-16 and as an optional feature, we think semi-static repletion is sufficient in FG 33-2 and dynamic repetition should be as a separate FG for enhancement. Regarding the maximum repetition number, we can compromise to accept 8 for semi-static repetition. Regarding the maximum repetition number of 16 for dynamic repetition, we think there is no necessary for MBS feature, especially for considering the new incoming UE to receive multicast. In addition, 16 repetition number will have larger UE impact than that of 8 repetition, the reason is that UE will accumulate more soft bits and the bit lengths will become larger. Thus, we suggest the FG can be modified as following: 33. NR_MBS; 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of a single CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 1_0 / 1_1 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support of semi-static slot-level repetition for group-common PDSCH for multicast. - [Support of NACK-only ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling NACK-only ACK/NACK based HARQ-ACK feedback configured by RRC ignalling.] - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. FFS whether to separate the capability for support of DCI format 1_1 4_2 with CRC scrambled with G-RNTI for multicast FFS whether how to separate the capability for support of NACK-only and/or ACK/NACK based HARQ-ACK feedback FFS whether how to separate the capability for support of PTP retransmission for multicast FFS whether to separate the capability for support of PTM retransmission for multicast; Yes; Per UE Per FSPC; No; No; For component 6, the candidate repetition number can be n2, n4, n8 slots if pdsch-AggregationFactor configured and UE can report one value from the candidate values; Optional with capability signalling. 33. NR_MBS; 33-2e; Dynamic slot-level repetition for group-common PDSCH for multicast; - Support of dynamic slot-level repetition for group-common PDSCH for multicast.; 33-2; Per UE Per FSPC; For component 1 the repetition number can be n2, n3, n4, n5, n6, n7, n8slots if repetitionNumber configured and UE can report one value from the candidate values; Optional with capability signalling. proposal 3-1b: support the proposal |
| ZTE | Regarding question 3-1a: Our understanding is that both semi-static repetition number configuration and dynamic repetition number indication should be included in FG33-2. In Rel-15, based on the following, UE is mandatory to support the semi-static configured repetition number up to 8. FG description: FG 5-17a PDSCH repetitions over multiple slots 1) K = 2, 4, 8 times repetitions Yes Type 4 No need Optional with capability signaling Signaling description from 38.331: pdsch-RepetitionMultiSlots ENUMERATED {supported} OPTIONAL, In Rel-16, based on the following, for dynamic repetition number indication, UE can indicate its maximum repetition number to one of {n2, n3, n4, n5, n6, n7, n8, n16}. supportInter-slotTDM-r16 SEQUENCE { supportRepNumPDSCH-TDRA-r16 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}, maxTBS-Size-r16 ENUMERATED {kb3, kb5, kb10, kb20, noRestriction}, maxNumberTCI-states-r16 INTEGER (1..2) } To avoid the concern from UE vendors, maybe we could try the following middle ground. 1) PDSCH repetition with semi-static configured repetition number is included in FG33-2. UE is mandatory to support {2, 4, 8 times repetitions } if UE indicates support of FG33-2. 2) PDSCH repetition with dynamically indicated repetition number is included in FG33-2. UE is allowed to report one of the maximum value of repetition number from {n2, n3, n4, n5, n6, n7, n8, n16}. Note that, the above two bullets are following the same handling from Rel-15/Rel-16. In addition, if companies have concern on the 16 repetitions for dynamic indication, they are always allowed to indicate the maximum value of repetition number that is less than 16. Regarding proposal 3-1b: For FG 33-2c and FG 33-2d, we are not sure whether they should be separate FGs or not for now. For example, they can be merged with the support of ACK/NACK based or NACK-only based feedback. Otherwise, it may end up with that UE supports UL feedback but doesn’t support retransmission. This is weird. |
| Apple | Question 3-1a: Q1: no strong preference now, the technical difference between semi-static and dynamic repetition is not clear. Need more progress in AI8.12. Q2/Q3: the maximum value 16 should be fine. We are OK with Proposal 3-1b. |
| LG Electronics | Question 3-1a: We slightly prefer to both capabilities for semi-static and dynamic repetitions in FG 33-2. Proposal 3-1b: We have sympathy with ZTE’s comment on this proposal. |
| CMCC | question 3-1a: Q1: support both semi-static and dynamic, otherwise network may use double resources to transmit the same multicast service. proposal 3-1b: We also think the PTM and PTP retransmission should be merged to HARQ-ACK feedback. |
| vivo | Question 3-1a: Q1: Support semi-static only. Even for unicast, DCI indicated slot-level repetition is a separate capability, we don’t see the motivation to include it in basic feature for multicast. Proposal 3-1b: We are fine to merge PTM retransmission to HARQ-ACK feedback, but leave PTP retransmission as a separate UE capability because it may involve ambiguity between PTP retransmission for multicast and unicast initial transmission when the same HPID is used. |
| OPPO | Question 3-1a: Q1: We prefer to include both semi-static and dynamic repetition in FG 33-2, while we did not quite clear about the fundamental difference between them. Q2/3: For Max value of repetition, we do not have strong view and 16 is acceptable for us. Proposal 3-1b: We support to separate the three FGs from FG 33-2, but we think at least one of 33-2c/d should be merged into 33-2a. It is because HARQ feedback-based retransmission has to be supported anyway, and at least one of the reTx schemes should be clearly supported to do the reTx, either one is OK. We prefer PTP (33-2d) can be merged into 33-2a. |
| Huawei, HiSilicon | 3-1a: Q1: semi-static repetition only is the component, to be keep minimum difference between multicast and broadcast. Q2: max value for dynamic can be 16 but can also accept 8. Q3: max value for semi-static is 8 to be the same as unicast. 3-1b: ok |
| CATT | Question 3-1a: Q1:Considering that dynamic repetition is an optional UE capability for unicast in Rel-16, we prefer to only include semi-static repetition in FG 33-2 to align with unicast design. Q3: Similar to unicast in Rel-16, the max value of dynamic semi-static repetition is 8. Question 3-1a: We share same view as OPPO. At least one of the FG 33-2c/d should be merged into 33-2a, and FG 33-2d is our preferred. If a UE support ACK-NACK-based feedback, the UE should also support the capability of ReTx for multicast. |
| Spreadtrum | Question 3-1a: Q1: prefer semi-static only. Q2: 8 Q3: 8 Proposal 3-1b: Ok |
| Nokia, NSB | Q1: Fine to have both Q2/Q3: for multicast there is no use of indicating a maximum number of repetitions per UE, as the network cannot use more than the minimum reported by any UE that is part of the multicast service. As pointed out by ZTE, since Rel-15 UEs are already mandated to support all values {2, 4, 8} if they support semi-static repetition. For dynamic repetition there is a different type of indication introduced in Rel-16, but again that is for unicast. For multicast we need a clear set of values that can be supported by *any* UE, as fragmentation creates more limitations in this type of service. Perhaps a compromise is to support indication of values {8, 16} for dynamic. In this way it is guaranteed that UEs support at least as much as for semi-static, and in certain scenarios it is possible to go beyond, i.e. 16 repetitions. For question 3-1b NACK-only feedback seems to be missing, though it is covered by component 6. This needs to be revised. |
| Moderator | Summary of companies view question 3-1a - Q1: Whether to include the capability for semi-static repetition only or both capabilities for semi-static and dynamic repetitions in FG 33-2. - semi-static repetition only: MTK, vivo, HW/HiSi, CATT, SPRD - both semi-static and dynamic repetitions: ZTE, LGE, CMCC, OPPO, Nokia/NSB - Need progress in AI8.12: Apple - Q2: Max value of dynamic repetition if included in FG 33-2 - 8: MTK, HW/HiSi, CATT, SPRD - 16, ZTE, Apple, OPPO, Nokia/NSB - Q3: Max value of semi-static repetition - 8: MTK, ZTE (mandatory), HW/HiSi, CATT, SPRD, Nokia/NSB (mandatory) - 16: Apple, OPPO, HW/HiSi proposal 3-1b Support: MTK, Apple, HW/HiSi, SPRD Merge 33-2c and 33-2d with 33-2a: ZTE, LGE, CMCC Merge 33-2c with 33-2a: vivo Merge 33-2d with 33-2a: OPPO, CATT For 3-1a, as suggested by ZTE and Nokia/NSB, one possible compromised proposal is to mandate UE supporting FG 33-2 to support 2/4/8 times semi-static repetitions while allow UE to report one of the maximum values for dynamic repetitions from {2, 3, 4, 5, 6, 7, 8, 16} [GTW3] High priority proposal 3-1a: - Both the capabilities for semi-static and dynamic repetitions are included in FG 33-2 - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions - For dynamic repetition, UE reports one of the maximum values from {2, 3, 4, 5, 6, 7, 8, 16} For 3-1b, a number of companies propose to merge at least one of 33-2c/2d to 33-2a while companies view is split. Therefore, the proposal is updated as follows [GTW3] High priority proposal 3-1b: - Add following FGs - FG 33-2a: RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - FG 33-2c: PTM retransmission for multicast - FG 33-2d: PTP retransmission for multicast 33. NR_MBS; 33-2a; RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling. FFS whether to merge with 33-2c/33-2d; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; PTM retransmission for multicast; Support of PTM retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. |
| Moderator | Following agreements were made in the GTW session on Jan 24. Agreement - Only the capability for semi-static repetition is included in FG 33-2 - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions - For dynamic repetition, a separate FG is introduced; UE reports one of the maximum values from {8, 16} Agreement - Add following FGs - FG 33-2a: Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - [FG 33-2c: PTM retransmission for multicast] - [FG 33-2d: PTP retransmission for multicast] 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling. At least 33-2c or 33-2d is merged, FFS which one or both; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; PTM retransmission for multicast; Support of PTM retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. |

[FL1] High priority question 3-2:

Companies are encouraged to provide views on whether to add following FGs in addition to FG 33-2.

- FG for support of M’ TCI-state configurations within the PDSCH-Config-Multicast.

- FG to indicate whether the UE can or cannot combine a PTM initial transmission and a PTP retransmission in case of different circular buffer size.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | FL proposal is a bit confusing. Is it proposing new FGs or additional components to FG33-2? In addition, these are open issues need to be solved in main agenda and then come back to the UE feature discussion. |
| MediaTek | From our understanding, the issue is still be discussed in AI8,12,1. We suggest deferring the discussion until we achieve some agreements/consensus. |
| ZTE | We propose to wait for the outcome from AI 8.12.1(or 2) first and then come back to this issue. |
| Xiaomi | We share the same views that we need to defer the discussion until we have any conclusion/agreement in main agenda. |
| CMCC | Wait the process in AI 8.12.1 |
| CATT | The issue is still discussed in AI 8.12.1. Thus, we propose to postpone this issue until some consensus were achieved in AI 8.12.1. |
| Nokia, NSB | The proposal is not totally clear to us. |
| Vivo | We agree with companies to revisit this issue after further progress achieved in MBS agenda. |
| OPPO | We share the similar view with HW/HiSi and MTK that both items can be discussed firstly in 8.12.1, and then they can be decided to be captured/discussed here. |
| Moderator | Given majority companies prefer to wait for the progress in AI 8.12.1, this issue can be come back when further progress is made in the AI |
| FL2 | No additional input is necessary for now |

[FL1] High priority question 3-3:

- Companies are encouraged to provide views on whether to add an FG for multiple G-CS-RNTIs for SPS group-common PDSCHs in addition to FG 33-2-x.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Is it proposing a new FG or an additional component to FG33-2-x? G-CS-RNTIs is for SPS, in general it can be kept as separate FG from that for dynamic scheduling. |
| MediaTek | We suggest focusing on the multiple G-RNTIs issue firstly. |
| ZTE | We are not sure why the number of G-CS-RNTIs have impact on RAN1. Actually, RAN1 cares more about the number of PDSCHs instead of the number of G-RNTIs. It seems better to leave this FG to RAN2 from our perspective. |
| Qualcomm | FG for multiple G-CS-RNTIs should be separate from that of G-RNTIs, which it is on condition of SPS multicast in FG 33-5-1. But we are fine to leave it to RAN2 decision. |
| CMCC | It seems like a RAN2-led UE feature. |
| CATT | We share same view with ZTE. |
| Nokia, NSB | Not clear if needed and if in RAN1 scope. |
| Spreadtrum | Share the same view with ZTE. |
| Vivo | We are ok to add an FG for multiple G-CS-RNTIs for SPS group-common PDSCHs |
| OPPO | Not support. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Summary of companies view - Support: HW/HiSi(?), vivo - Not support: OPPO - Leave to RAN2: ZTE, QC, CMCC, CATT, CATT, Nokia/NSB(?), SPRD, - Focus on multiple G-RNTIs at first: MTK Given that majority companies (including proponent) are OK to leave to RAN2, no further input is necessary. |

Medium priority question 3-4:

- Companies are encouraged to provide views on whether FG 33-2 is supported as a basic FG for MBS

  - Support: CMCC

| Company | Comment |
| --- | --- |

Medium priority question 3-5:

- Companies are encouraged to provide views on whether the type of FGs 33-2/33-2-x should be per UE or per FSPC.

  - FG 33-2

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, OPPO, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

  - FG 33-2-x

    - Per UE: Huawei, HiSilicon, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |
| Nokia, NSB | Per UE, differentiation cases brought up by Qualcomm can be easily resolved without such a costly solution as defining the ignalling per band. |

Low priority proposal 3-6:

- Components of FG 33-2 are revised as

  - Component 1: Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast.

  - Component 2: Support of a single CFR configuration for multicast.

  - Component 3: Support of CORESET and common search space configuration in a CFR for multicast.

  - Component 4: Support of DCI format 4_1 / 4_2 with CRC scrambled with G-RNTI for multicast.

  - Component 5: Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots.

  - Component 6: Support of NACK-only based HARQ-ACK feedback for dynamic multicast scheduling, and support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling.

- Component of FG 33-2-x is revised as “Capability on max number of G-RNTIs for multicast”.

- Name of FG 33-2b is revised as “DCI-based enabling/disabling HARQ-ACK feedback for dynamic scheduling for multicast”.

- Components of FG 33-2b is revised as “Support of DCI-based enabling/disabling ACK/NACK based and NACK-only HARQ-ACK feedback per configuration of RRC signaling for each G-RNTI”

- Components of FG 33-2-x is revised as

  - Component 1: The maximum number of G-RNTIs supported for multicast. FFS detailed values.

  - Add component 2: Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.

| Company | Comment |
| --- | --- |

Low priority question 3-7:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-2

| Company | Comment |
| --- | --- |

Low priority question 3-8:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-2 and 33-2-x which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-3-1: Slot-level repetition for group-common PDSCH

In [1], FG 33-3-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the Gnb to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | As for FG33-1 broadcast, support of semi-static configured slot-level repetition of group-common PDSCH can be one of components of FG33-2 for multicast. Proposal 3: Updating FG33-2, adding FG33-2-1, FF33-2-2, FG33-2-3, deleting FG33-2b, deleting FG33-3-1, deleting FG33-4, and deleting FG33-7 as follows in red: 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support of semi-static configured slot-level repetition for group-common PDSCH - Support of NACK-only based feedback for dynamic multicast scheduling - Support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling. - Support of PTM retransmission for dynamic multicast scheduling; Yes; Per UE; No; No; Optional with capability signalling. 33-3-1; Slot-level repetition for group-common PDSCH; - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [6] | NTT DOCOMO | There’s no big difference between semi-static slot-level repetition and dynamic slot-level repetition, except for the RRC parameter to indicate the number of repetitions. We don’t see the need to split semi-static slot-level repetition and dynamic slot-level repetition into separate FGs. Since support of unicast PDSCH repetition is per UE, support of multicast PDSCH repetition should also be per UE. Proposal 8: Support of semi-static slot-level repetition and dynamic slot-level repetition are kept in one FG. Proposal 9: Type of FG 33-3-1 is per UE. |
| [8] | Qualcomm | 33-3-1; Slot-level repetition for group-common PDSCH; - Support slot-level repetition for group-common PDSCH for multicast by using DCI format 4_2 with CRC scrambled with G-RNTI.; 33-2a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [9] | OPPO | For FG 33-3-1 slot-level repetition for group-common PDSCH, there is no necessary to split it into two FGs. Both semi-static and dynamic slot-level repetition schemes are equivalent mechanism but in different repetition types. The two repetition schemes can be indicated separately as two components in one FG rather than separating into two FGs. - For FG 33-3-1, it is not needed to split it into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. 33-3-1; Slot-level repetition for group-common PDSCH; - Support semi-static slot-level repetition for group-common PDSCH for multicast. - Support dynamic slot-level repetition for group-common PDSCH for multicast.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| [11] | Apple | Regarding the multicast PDSCH repetition, two different parameters, i.e., i.e., pdsch-AggregationFactor and repK, could configure the repetition. We update our views on this FG and prefer to split FG33-3-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition, which provide more flexibilities to UE implementation. Proposal 4: Spliting FG 33-3-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. |
| [12] | Xiaomi | For FG 33-3-1, it is still open on whether to split FG 33-3-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. From our perspective, slot-level PDSCH repetition impacts UE behavior on buffering and combining. Whether the slot-level repetition is configured by RRC signaling or the slot-level repetition is indicated via DCI does not make any difference on the UE behavior for PDSCH buffering and combining. Hence we don’t think it is necessary to further split FG 33-3-1. Proposal 3: There is no need to further split FG 33-3-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. |
| [13] | MediaTek | Regarding the slot-level repetition for group common PDSCH for multicast, we need to further discuss whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. From our view, considering the n16 repetition is optional for legacy UE, which means that even if the UE support the repetition, it does not mean the UE can support 16 PDSCH repetition. So, we suggest to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition Proposal 15: For FG 33-2-1, split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition. 33-3-1; Semi-static slot-level repetition for group-common PDSCH for multicast; - Support of semi-static slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition; 33-2; Yes; Per UE Per FSPC; No; No; For component 1 the repetition number can be n2, n4, n8 slots if pdsch-AggregationFactor configured; Optional with capability signalling. 33-3-1a; Dynamic slot-level repetition for group-common PDSCH for multicast; - Support of dynamic slot-level repetition for group-common PDSCH for multicast.; 33-2; Per UE Per FSPC; For component 1 the repetition number can be n2, n3, n4, n5, n6, n7, n8, n16 slots if repetitionNumber configured; Optional with capability signalling. |
| [14] | CMCC | One remaining issue is whether to separate semi-static and dynamic PDSCH repetition, and during last RAN1 meeting, some companies also proposed to wait for the PDSCH repetition discussion for broadcast. As the agreement in last RAN1 meeting, both semi-static (config A) and dynamic (config B) are supported for broadcast which is the same as multicast. Considering if different UE supports different configuration, it will make Gnb configure two MBS groups for one service which causes the reduction of spectrum efficiency, therefore, the split of FG 33-3-1 is not preferred. In addition, since the PDSCH repetition schemes are the same for both broadcast and multicast, and the split of semi-static and dynamic is also not preferred, we think FG 33-3-1 is both applied for broadcast and multicast. Proposal 5. For FG 33-3-1: - Don’t support to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition; - FG 33-3-1 is both applied for broadcast and multicast. |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether to merge FG 33-3-1 with FG 33-2.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Support of dynamically indicated repetition can be a separate FG and the semi-static configured repetition can be a component of FG 33-2. |
| MediaTek | Not support to merge the FG 33-3-1 with FG 33-2. As we stated in High priority question 2-1, supporting slot-level repletion is a separate FG is legacy behavior, we should keep the similar mechanism with that. Hence, we support slot level repetition can be as a separate FG. |
| ZTE | The difference between the dynamic indication and semi-static configuration repetition is the indication method. From our perspective, it is ok to combine them into one FG. |
| Qualcomm | If the slot-level repetition is included in FG 33-1 for broadcast, FG 33-3-1 can be merged into FG 33-2 for multicast. |
| CMCC | Support to merge. |
| CATT | Support to merge FG 33-3-1 with FG 33-2. |
| Vivo | Ok for merge |
| NTT DOCOMO | We support to merge. |
| OPPO | Not support to merge FG 33-3-1 with FG 33-2. Both dynamic and semi-static slot-level repetition can be merged into a separate FG. |
| Lenovo, Motorola Mobility | Support to merge due to same functionality |
| Apple | Not support to merge, repetition is not mandate feature for multicast service. |
| Moderator | This issue can be discussed further after some progress is made in question 4-2 |
| FL3 | This question can be further discussed together with question 3-1a |

[FL1] High priority question 4-2:

- Companies are encouraged to provide views on whether to split FG 33-3-1 into 2 separate FGs, i.e., 1st one for semi-static slot-level repetition, 2nd one for dynamic slot-level repetition.

  - Keep as a single FG: NTT DOCOMO, Qualcomm, OPPO, Xiaomi, CMCC

  - Split into 2 separate FGs: Apple, MediaTek

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Support of dynamically indicated repetition can be a separate FG and the semi-static configured repetition can be a component of FG 33-2. |
| MediaTek | Considering the different UE capability is needed for semi-static and dynamic repetition, we support that FG 33-3-1 cab be separated as 2 FGs, i.e., 1st one for semi-static slot-level repetition, 2nd one for dynamic slot-level repetition. |
| ZTE | If the semi-static configured repetition is moved to a component of FG 33-2, then it is ok to list dynamically indicated repetition as a separate FG. Otherwise, we would prefer to combine them into one FG. |
| Xiaomi | We don’t see the necessity to define two separate FGs on slot-level repetition. |
| CMCC | Don’t suppot to split into two FGs, otherwise gNB may use redundant PDCCH resources for scheduling the same MSB service, which one is semi-static indication and another is dynamic indication. |
| CATT | We support keep FG 33-3-1 in a single FG. |
| Spreadtrum | We are fine to split it into two FGs, and we are also fine with two components in one FG with the candidate value {supported, not supported} for each component. |
| OPPO | We think semi-static and dynamic slot-level repetition can be merged into one FG by considering about that there is not quite technical gap to separate them. |
| Lenovo, Motorola Mobility | No need to separate the two repetitions. |
| Apple | Support to split into two features to leave more implementation choices, the complexity for dynamic and semi-static repetition is different. |
| Moderator | Summary of companies view - Keep as a single FG: NTT DOCOMO, Qualcomm, OPPO, Xiaomi, CMCC, CATT, Lenovo, Motorola Mobility - Split into 2 separate FGs: Apple, MediaTek, Huawei, HiSilicon, ZTE (if semi-static one is combined with FG 32-2), Spreadtrum Given there is no majority view, further discussion in GTW session is necessary. |
| FL2 | This issue could not be discussed in the GTW session on Jan 18. Companies are encouraged to further provide view on this aspect especially trying to address the concern from other side |
| ZTE | Maybe the following alternatives could be a potential compromise. Alternative: 1. Semi-static slot-level repetition for group-common PDSCH for multicast for up to 8 repetition is included in FG33-2 as a component; 2. Separate UE capability for dynamic slot-level repetition for group-common PDSCH for multicast. |
| Vivo | We are ok with the compromised proposal from ZTE. |
| Spreadtrum | Prefer to split into two FGs. The reason is as companies say that they denotes different UE complexity. To solve CMCC’s concern, we are fine to put semi-static indication as the prerequisite FG of dynamic indication. |
| Xiaomi | We are unclear on the comments that dynamic repetition and semi-static repetition denotes different UE complexity. More clarification is needed. |
| Ericsson | We would prefer to merge at least semi-static indication of repetition into 33-2 (see previous question). |
| Moderator | Summary of companies view - Keep as a single FG: NTT DOCOMO, Qualcomm, OPPO, Xiaomi, CMCC, CATT, Lenovo, Motorola Mobility - gNB may use redundant PDCCH resources for scheduling the same MSB service - Split into 2 separate FGs: Apple, MediaTek, Spreadtrum - different UE complexity - Semi-static repetition is included in FG 33-2 and dynamic repetition is separate FG: ZTE, vivo, HW/HiSi, E/// As proposed by ZTE and also supported by some companies, a compromise is to include Semi-static repetition in FG 32-2 and to revise FG 33-3-1 as dynamic ignalling. This issue can be discussed together with proposal 3-1 |
| FL3 | This question can be further discussed together with question 3-1a |

Medium priority question 4-3:

- Companies are encouraged to provide views on whether the type of FG 33-3-1 should be per UE or per FSPC.

  - Per UE: NTT DOCOMO, OPPO

  - Per FSPC: Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority proposal 4-4:

- Components of FG 33-3-1 are revised as “Support slot-level repetition for group-common PDSCH for multicast by using DCI format 4_2 with CRC scrambled with G-RNTI.”

| Company | Comment |
| --- | --- |
| Qualcomm | If RAN1 agrees to support slot-level repetition by using DCI format 4_1 (to be discussed in 8.12.1), the revision is not needed. |

Low priority question 4-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-3-1

| Company | Comment |
| --- | --- |

Low priority question 4-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-3-1 which do not have capability ignalling impacts.

| Company | Comment |
| --- | --- |

- 33-3-2 to 33-3-3: Multiplexing of unicast PDSCH and group-common PDSCH

In [1], FGs 33-3-2 to 33-3-3 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination including supportedBandwidthDL, maxNumberMIMO-LayersPDSCH and supportedModulationOrderDL that can determine an maximum TB size in a slot. For UE supporting FDM-ed unicast PDSCH and one group-common PDSCH in a slot, the maximum TBS size should be reported and is supposed to be the total maximum TB size for both unicast and group-common PDSCH. If FG33-2 has no HARQ-ACK feedback or NACK-only as the component, it makes sense to separate the support of FDM-ed Type1/Type2 HARQ-ACK codebook from FG33-3-2 and define additional UE capability as is. Proposal 5: Updating FG33-3-2 and adding FG33-3-2a as follows in red: 33-3-2; FDM-ed unicast PDSCH and one group-common PDSCH in a slot; - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. - The maximum TB size supported in a slot; 33-1, 33-2; Yes; Per FSPC; NA; NA; Optional with capability signalling. 33-3-2a; ACK/NACK feedback multiplexing for FDM-ed unicast PDSCH and one group-common PDSCH; Support of FDM-ed ACK/NACK-based HARQ-ACK codebook for unicast and group-common PDSCH in a slot.; 33-3-2; Yes; Per FSPC; NA; NA; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. Similar rationale for the update for FG33-3-2, the maximum TBS size supported in a slot should be reported and reasonable to add another feature group for the ACK/NACK based feedback multiplexing for the TDMed scheduling. In addition, one agreement regarding Type-1 HARQ-ACK codebook generation for TDM-ed unicast and multicast based on the intersection of k1 values of unicast and multicast as UE capability was not reflected. Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability. Proposal 6: Updating FG33-3-3 and adding FG33-3-3a/3b as follows in red: 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - The maximum TB size supported in a slot; 33-1, 33-2; Yes; Per FSPC; NA; NA; Candidate values: For component 2: values of M = {2, 3…,7}; For component 3: values of N= {2, 3…,7}; For component 4: values of (K, L)= {(2,3),(3,2),(2,4), (4,2)…}; Optional with capability signalling. 33-3-3a; ACK/NACK feedback multiplexing for intra-slot TDM-ed unicast PDSCH and one group-common PDSCH based on the union of K1 sets; Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for M>= 1 unicast and N>=1 multicast in a slot. Note: For Type-1 HARQ-ACK codebook, it is based on the union of K1 sets for unicast and multicast.; 33-3-3; Per FSPC; N/A; N/A; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. 33-3-3b; ACK/NACK feedback multiplexing for intra-slot TDM-ed unicast PDSCH and one group-common PDSCH based on the intersection of K1 sets; Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for M>= 1 unicast and N>=1 multicast in a slot. Note: For Type-1 HARQ-ACK codebook, it is based on the intersection of K1 sets for unicast and multicast.; 33-3-3; Per FSPC; N/A; N/A; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. |
| --- | --- | --- |
| [3] | vivo | Also, the HARQ-ACK codebook should be separated from multiplexing of unicast PDSCH and group-common PDSCH, and similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities. Proposal 5 Separate ‘support of FDM-ed HARQ-ACK codebook’ from 33-3-2. Proposal 6 Separate ‘support of TDM-ed HARQ-ACK codebook’ from 33-3-3. Furthermore, as there is no difference between FDMed and TDMed type-2 codebok, it is not necessary to differentiate them, and thus, option 1 in [1] should be supported. Proposal 7 Regarding HARQ-ACK codebook - Add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - Add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - Add an FG for Type-2 HARQ-ACK codebook for multicast According the achieved agreements in appendix, capability for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast should be added and it could be combined with the capability for TDM-ed Type-1 HARQ-ACK codebook for multicast. Proposal 8 Add capability for type-1 codebook generation based on k1 values in the intersection of K_1 set for unicast and K_1 set for multicast in the capability for TDM-ed Type-1 HARQ-ACK codebook for multicast From the achieved agreements, support of intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and support of FDM between unicast PDSCH and group-common PDSCH should also apply to broadcast. Thus, 33-3-2 and 33-3-3 should take 33-1 or 33-2 as prerequisite feature groups. Proposal 9 Take 33-1 or 33-2 as prerequisite feature groups for 33-3-2 and 33-3-3. |
| [4] | ZTE | Another issue is how to capture agreements for FG33-3-3. From our perspective, UE can share the same maximum PDSCH receptions in a slot per CC between unicast and multicast. Thus, the FG33-3-3 can be updated as following. Proposal 2: Update the components description as following. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast.. |
| [5] | Samsung | For 33-3-2/3, pre-requisite can be either 33-1 or 33-2, but should not be both. Index; Feature group; Components; Prerequisite feature groups. 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast.; 33-1 or 33-2. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. - FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1 or 33-2. |
| [6] | NTT DOCOMO | We don’t see the need to separate the capability for HARQ-ACK codebook for FDM. If a UE supports FDM of unicast PDSCH and group-common PDSCH, then the UE should also support the corresponding HARQ-ACK codebook. Proposal 10: Support of HARQ-ACK codebooks for FDM-ed unicast PDSCH and group-common PDSCH are kept in FG 33-3-2. |
| [7] | Spreadtrum Communications | Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2 and 3 in FG33-3-2 as separated FG. Proposal 5: Component 2 and 3 in FG33-3-2 can be split as separated FG, and as optional UE capability. Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 6 and 7 in FG33-3-3 as separated FG. Proposal 6: Component 6 and 7 in FG33-3-3 can be split as separated FG, and as optional UE capability. In RAN11#106e meeting, we have the following agreement: Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Furthermore, in current 38.213 spec [3], Alt 1 has been captured as type1-Codebook-Generation-Mode = ‘mode1’ Thus, the following FG is preferred to be added to reflect the agreement and align with TS38.213 spec. Proposal 10: Suggest to add one new FG to reflect the following. - Supports type1-Codebook-Generation-Mode configured as mode 1 |
| [8] | Qualcomm | Among the options, we prefer Option 2 and propose the following changes. 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-1, 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-3-2a; FDM-ed pattern of feedback codebook for unicast PDSCH and group-common PDSCH; 1. Support of FDM-ed ACK/NACK-based Type-1 HARQ-ACK codebook for one unicast and one multicast in a slot. 2. Up to X (X>1) G-RNTI(s) configured to UE for the FDMed unicast and multicast Type-1 HARQ-ACK codebook; 33-2b, 33-3-2; Yes; Per FSPC; N/A; N/A; FFS: value of X; Optional with capability signalling. 33-3-2b; Maximum data rate of FDMed unicast PDSCH and group-common PDSCH for multicast; - 1. Up to X data rate of FDMed unicast PDSCH and group-common PDSCH(s) for multicast in a slot per CC..; 33-3-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; 33-1, 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-3-3a; Feedback multiplexing for intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast; - Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for M (M>= 1) unicast and N (N>=1) multicast in a slot.; 33-2b, 33-3-3; Yes; Per FSPC; N/A; N/A; Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both}; Optional with capability signalling. 33-3-3b; Maximum data rate of TDMed unicast PDSCH and group-common PDSCH for multicast; - Up to X data rate of TDMed unicast PDSCH(s) and group-common PDSCH(s) for multicast in a slot per CC.; 33-3-3; Yes; Per FSPC; N/A; N/A; FFS: value of X; Optional with capability signalling. 33-3-4; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; - Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [11] | Apple | For FG33-3-2, it’s so restrictive to limit only one unicast PDSCH and only one GC-PDSCH FDMed in a slot. Similar as FG3-3-3, it can support K TDMed unicast PDSCH in a slot and L TDMed GC-PDSCH in a slot, but unicast PDSCH and GC-PDSCH are FDMed. With this update, there is no codebooks construction issue, anyway the codebook for unicast PDSCH and for GC-PDSCH are generated separately and concatenated together. The total number of received unicast PDSCHs and GC-PDSCHs in a slot are subject to TDMed PDSCH reception capability defined in Rel-15/Rel-16, i.e.,{2/4/7}. Proposal 5: Supporting FDMed reception between K TDMed unicast PDSCHs and L TDMed multicast PDSCHs in a slot. For the separation of capability for HARQ-ACK codebook in FG33-3-2 and Fg33-3-3, as we comment earlier supporting HARQ-ACK feedback is a UE capability, then the supported HARQ-ACK codedbook types should be independent UE capability with the condition that HARQ-ACK feedback is supported. Proposal 6: Supporting HARQ-ACK codebook types as independent capability. It was agreed that RRC_CONNECTED UE can still receive the broadcast service. This is highly related to UE capability whether UE can receive the broadcast PDSCH with unicast/multicast PDSCH in FDM manner. The FG33-3-2 can be clarified as FDMed unicast PDSCH and multicast PDSCH. On top of FG33-3-2, a new FG can be introduced to support FDMed reception of broadcast PDSCH and unicast PDSCH/multicast PDSCH for RRC_CONNECTED UE. Agreement: [3] For broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE Ues and RRC_CONNECTED Ues when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE Ues and the SCS and CP are the same.. Proposal 7: introducing new FG to support FDMed reception of broadcast PDSCH and unicast PDSCH/multicast PDSCH for RRC_CONNECTED UE. |
| [12] | Xiaomi | In RAN1#107 e-meeting, it is extensively discussed how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3. Three options were achieved which are listed as below: - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast First of all, HARQ-ACK feedback for multicast is an optional capability. Hence it is not reasonable to capture it in FG33-2. Regarding to option2, the construction of type-2 HARQ-ACK codebook depends on the received DCI. To be specific, it is constructed based on the T-DAI and C-DAI carried by the received DCI. It is constructed in a dynamic manner and it doesn’t matter whether PDSCHs are TDMed or FDMed. Proposal 2: Separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 with option 1. |
| [13] | MediaTek | Regarding the FG 33-3-2, we need to further discuss whether/how to separate the capability for HARQ-ACK codebook. From our understanding, supporting FDM between unicast and GC-PDSCH does not mean FDM-ed HARQ-ACK codebook. Thus, we suggest to separate the FDM-ed HARQ-ACK from the FG 33-3-2. Proposal 16: For FG 33-3-2, the FDM-ed HARQ-ACK codebook capability should be as a separate FG. Regarding the prerequisite feature groups 33-1, we think the broadcast reception is “best-effort” reception and the UE in RRC IDLE/INACTIVE cannot report capability. So, we think FDMed reception for MBS is supported for multicast and not broadcast. Proposal 17: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted. 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1, 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. 33-3-2-1; HARQ-ACK codebook for FDMed unicast and multicast; - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast; 33-3-2; Per UE Per FSPC; Optional with capability signalling. Regarding the FG 33-3-3 we need to further discuss whether/how to separate the capability for HARQ-ACK codebook. We share the similar view as that of FG 33-3-2 and have the following proposals: Proposal 18: For FG 33-3-3, the TDM-ed HARQ-ACK codebook capability should be as a separate FG. Proposal 19: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook; 33-1, 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. 33-3-2-1; HARQ-ACK codebook for FDMed unicast and multicast; - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast; Per UE Per FSPC; Optional with capability signalling. |
| [14] | CMCC | Three options are listed in the following agreement to be further discussed how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3. Agreement - Down select one of the followings on how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3 - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Option 3: - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2 - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast As the comments from companies, it is not necessary to differentiate TDM/FDM-ed Type-2 HARQ-ACK codebook. Besides, if UE supports FDM-ed unicast and multicast multiplexing, HARQ-ACK codebook should also be included in this FG, thus we support Option 3 to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3. Proposal 6. Support Option 3 to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-3-2: - If CONNECTED_MODE Ues can receive broadcast without supporting multicast, component 1 may need to be separated from 2/3. More clarification needed here. - Per UE - 33-3-3: - Same comment as for 33-3-2. - Per UE |
| [16] | Ericsson | For both of these features, HARQ codebook support also should include unicast and multicast, as it is possible to create a common codebook. The following change is proposed: 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.. FFS whether/how to separate the capability for HARQ-ACK codebook. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.. FFS whether/how to separate the capability for HARQ-ACK codebook. |

Discussion

[FL1] High priority question 5-1:

- Companies are encouraged to provide views on whether/how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3, e.g.,

  - Option 1:

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

    - add an FG for Type-2 HARQ-ACK codebook for multicast

  - Option 2:

    - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast

  - Option 3:

    - Add the capability for Type-2 HARQ-ACK codebook for multicast as a component in FG 33-2

    - add an FG for FDM-ed Type-1 HARQ-ACK codebooks for multicast

    - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

Option 1: vivo, Xiaomi

Option 2: Huawei, HiSilicon, Qualcomm, MediaTek

Option 3: CMCC

| Company | Comment |
| --- | --- |
| MediaTek | Support option 2. Option1 is also ok for us. |
| ZTE | From our perspective, this issue is depending on the outcome of FG 33-2. If some kinds of codebook are included as a component for FG33-2, then the remaining ones can have separate FG(s). |
| CATT | Support option2. |
| Spreadtrum | Option 2 |
| vivo | We support option 1, as there is no difference between constructing TDM-ed and FDM-ed Type-2 HARQ-ACK codebook. |
| NTT DOCOMO | We support Option 3. |
| Lenovo, Motorola Mobility | Either Option 1 or 2 is fine with us. |
| Apple | Option 1, same view as vivo. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Summary of companies view Option 1: vivo, Xiaomi, MTK, Lenovo, Motorola Mobility, Apple Option 2: Huawei, HiSilicon, Qualcomm, MediaTek, CATT, SPRD, Lenovo, Motorola Mobility Option 3: CMCC, DCM, Can be discussed when FG33-2 is stable: ZTE As suggested by ZTE, this issue can be further discussed when FG33-2 is stable. No further comments are necessary for now |
| FL3 | As we made some progress on FG 33-2, let’s further discuss this issue. Option 3 can be preclude based on the discussion in the GTW session on Jan 20. [FL3] High priority question 5-1a: - Companies are encouraged to provide views on whether/how to separate the capability for HARQ-ACK codebook from FGs 33-3-2 and 33-3-3, e.g., - Option 1: - add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast - add an FG for Type-2 HARQ-ACK codebook for multicast - Support: vivo, Xiaomi, MTK, Lenovo, Motorola Mobility, Apple - Option 2: - add an FG for FDM-ed Type-1/2 HARQ-ACK codebooks for multicast - add an FG for TDM-ed Type-1/2 HARQ-ACK codebook for multicast - Support: Huawei, HiSilicon, Qualcomm, MediaTek, CATT, SPRD, Lenovo, Motorola Mobility |
| ZTE | We can accept with Option 1 above with the condition that at least PTM retransmission is merged with them. Otherwise, it may end up with that UE only supports UL feedback but doesn’t support retransmission. Also, we would propose to make one of them as the basic feedback mode for multicast. Otherwise, if different UEs supporting different feedback mode, it is impossible for network to group these UEs into one group for MBS. The whole picture could be the following assuming that TDM-ed Type-1 HARQ-ACK codebook is the basic feedback mode. FG 33-x: 1) TDM-ed Type-1 HARQ-ACK codebook for multicast; 2) PTM retransmission for multicast; FG 33-y: Prerequisite: FG 33-x - FDM-ed Type-1 HARQ-ACK codebook for multicast FG 33-z: Prerequisite: FG 33-x 1) Type-2 HARQ-ACK codebook for multicast |
| CMCC | Similar view as ZTE, the codebook should be merged with HARQ-ACK feedback. |
| Vivo | Support option 1 as there is no difference between constructing TDM-ed and FDM-ed Type-2 HARQ-ACK codebook. We are fine to merge PTM retransmission to HARQ-ACK feedback, but leave PTP retransmission as a separate UE capability. |
| OPPO | We support option 1. |
| Moderator | Summary of companies view - Option 1: - Support: vivo, Xiaomi, MTK, Lenovo, Motorola Mobility, Apple, ZTE, CMCC, OPPO - Option 2: - Support: Huawei, HiSilicon, Qualcomm, MediaTek, CATT, SPRD, Lenovo, Motorola Mobility [GTW3] Whether to include PTM retransmission or not may depend on the discussion for proposal 3-1b. Further discussion is necessary in the GTW session. |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given companies still prefer different options, no further discussion is planed in this meeting. Let’s further discuss in the next meeting. |

[FL1] High priority question 5-2:

- Companies are encouraged to provide views on whether to add following FGs in addition to FG 33-3-2.

  - FG for support of FDMed reception between K TDMed unicast PDSCHs and L TDMed multicast PDSCHs in a slot.

  - FG for support of FDMed reception of broadcast PDSCH and unicast PDSCH/multicast PDSCH for RRC_CONNECTED UE.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | FG 33-3-2 is for FDM-ed one unicast PDSCH and one group-common PDSCH which can be broadcast or multicast, which seems sufficient for this release. Otherwise, additionally supporting such FGs will cause additional specification efforts to specify UE behaviors. |
| MediaTek | Not support. In the previous agreement, we only agreed that supporting only one unicast PDSCH and one multicast PDSCH for FDMed case. |
| CMCC | Not support, we only agreed one unicast PDSCH and one multicast PDSCH to be FDMed in one slot. |
| CATT | One clarification for supporting only one unicast PDSCH and one multicast PDSCH for FDMed case are needed. As the figure shown, the multicast-1 is TDMed with unicast-1 and is FDMed with unicast-2. For this case, does it comply with supporting FDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability? |
| Nokia, NSB | Do not support |
| Spreadtrum | Not support |
| NTT DOCOMO | We don’t think those FGs are necessary. |
| Lenovo, Motorola Mobility | Not support. |
| Apple | In the codebook design, we already support the FDM +TDM reception. The below figure was used in last RAN1 discussion. I’m a little confused if one unicast PDSCH and one multicast PDSCH to be FDMed in one slot, what is the benefit to support FDM, why we discuss the so complicated HARQ feedback design. The second bullet can be revisited after the progress on AI8.8.1.3. Agreement: For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the sub-codebooks for each G-RNTI by appending one to another in ascending order of G-RNTI value. - The sub-codebook for each G-RNTI is generated per the k1 and TDRA configurations for the same G-RNTI as the legacy procedure. - FFS: whether/how to reduce the Type-1 codebook size when multiple G-RNTIs are configured. - Note: The maximum number of G-RNTI(s) configured to UE for the FDMed unicast and multicast Type-1 codebook is up to UE capability which will be discussed in UE features. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Summary of companies view - Support: - Not support: HW/HiSi, MTK, CMCC, Nokia/NSB, SPRD, DCM, Lenovo, Motorola Mobility - Need clarification: CATT, Apple Given that majority companies don’t support those FGs, no further input is necessary, but companies are invited to check the comments from CATT/Apple and to clarify the understanding. |
| ZTE | From our perspective, if a UE supports FDMed unicast and multicast PDSCH and can support more than 2 PDSCHs per slot, then the case mentioned in Apple and CATT’s figure can be supported. Otherwise, if a UE indicates support of FDMed unicast and multicast PDSCH, it always limits the number of PDSCHs per slot to 2. Is this the intention from other companies? |
| Xiaomi | The issue raised by CATT is under discussion in AI8.12.1. It is better to come back after we have a clear agreement/conclusion in main session. |
| Ericsson | We may need a new FG if the FDM discussion on “case 6” in the group scheduling agenda concludes that 1 multicast FDM with multiple TDM unicast in a slot is supported. |
| Moderator | As suggested by Xiaomi, this issue can be discussed after some progress is made in AI 8.12.1 |

[FL1] High priority question 5-3:

- Companies are encouraged to provide views on whether to add following FGs in addition to FG 33-3-3

  - FG for support of type1-Codebook-Generation-Mode configured as mode 1.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | My reading of FL proposal is that it is a separate UE FG with FG 33-3-3 as prerequisite, then it is fine. |
| MediaTek | We are ok to have a separate FG |
| Qualcomm | Ok to have a separate FG |
| CATT | We are ok to have a separate FG. |
| Spreadtrum | Ok to have a separated FG |
| Apple | Not clear the proposal, more details could be helpful. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Given majority companies support to have a separate FG, following proposal is made [FL2] High priority proposal 5-3: - Add an FG for support of mode 1 for type1 codebook generation as follows 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; TBD; Yes; Per UE; No; No; Optional with capability signalling. Companies are encouraged to provide view on whether proposal 5-3 is acceptable or not. Please also try to address the concern from other side |
| Spreadtrum | Support the proposal. The FG is used to reflect one previous agreement where for TDMed type-1 codebook, Alt 1 is one optional UE capability. In addition, Alt 1 has been captured into spec by naming ‘mode1’ for type1-Codebook-Generation-Mode. The following information is for your reference. In RAN11#106e meeting, we have the following agreement: Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Furthermore, in current 38.213 spec [3], Alt 1 has been captured as type1-Codebook-Generation-Mode = ‘mode1’, and copied below for reference. 38.213 h00 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ----------------------------------unrelated text omitted---------------------------------------- e) If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c - if the UE is not provided type1-Codebook-Generation-Mode = ‘mode1’, K1 is additionally provided by the union of dl-DataToUL-ACK-ForDCI Format4_1 - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - if the UE is provided type1-Codebook-Generation-Mode = ‘mode1’, the UE - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM ----------------------------------------unrelated text omitted-------------------------------------- If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = ‘mode1’, the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.. |
| Ericsson | Support. |
| Moderator | Given majority companies still support to have a separate FG, the same proposal is set for GTW session [GTW2] High priority proposal 5-3: - Add an FG for support of mode 1 for type1 codebook generation as follows 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; TBD; Yes; Per UE; No; No; Optional with capability signalling. |
| FL3 | No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 5-3: - Add an FG for support of mode 1 for type1 codebook generation as follows 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; TBD; Yes; Per UE; No; No; Optional with capability signalling. |
| Moderator | [GTW3] The same proposal is set for GTW session |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given no concern has not been received for more than 24 hours. This proposal is set for email approval after final check. |
| Moderator | Following agreement was made over the RAN1 reflector Agreement - Add an FG for support of mode 1 for type1 codebook generation as follows 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; TBD; Yes; Per UE; No; No; Optional with capability signalling. |

[FL1] High priority question 5-4:

- Companies are encouraged to provide views on whether to add a FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | It can be a separate UE FG. |
| ZTE | It seems the difference between multiplexing HARQ-ACK for unicast and multicast for the same or different HARQ-ACK codebook types is minor. We would prefer to have one FG for both multiplexing between the same codebook types and different codebook types. |
| Qualcomm | Support |
| CMCC | Support |
| CATT | OK |
| Spreadtrum | Support |
| Apple | OK |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Given majority companies support to have a separate FG, following proposal is made [FL2] High priority proposal 5-4: - Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. Companies are encouraged to provide view on whether proposal 5-3 is acceptable or not. Please also try to address the concern from other side |
| Ericsson | OK |
| Moderator | Given majority companies still support to have a separate FG, the same proposal is set for GTW session [GTW2] High priority proposal 5-4: - Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| FL3 | No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 5-4: - Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| ZTE | We won’t object this proposal. But we would like to seek some clarification from the UE proponents of this proposal, compared with the same HARQ-ACK codebook type, what would be the additional complexity for multiplexing different HARQ-ACK codebook types in the same PUCCH?, |
| Moderator | [GTW3] The same proposal is set for GTW session Following agreement was achieved in the GTW session on Jan 24 Agreement - Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

[FL1] High priority question 5-5:

- Companies are encouraged to provide views on whether to add a component into FG 33-3-2 and FG 33-3-3 or separate FGs for supported maximum TB size in a slot.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We’d like to hear other UE vendor’s view, when the supported maximum TB size in a slot is not reported as separate UE FG, how to perceive the max TB size in the slot UE can support, the same as UE supports for unicast?? |
| ZTE | For unicast, UE is capable of receiving multiple unicast TBs in one slot, the UE does not need to report the maximum TB size. Could the proponents of this proposal clarify the motivation to report the maximum TB size here? |
| CATT | The motivation to add the capability for reporting the supported maximum TB size in a slot is not clear. |
| Nokia, NSB | Not clear if needed |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | More input is necessary. If not yet provided, companies are encouraged to provide views on this issue |
| Qualcomm | Support The reason is that if FDM of unicast and multicast is supported, the number of PDSCHs in a slot may be larger than legacy case and the as well as the max TB size for UE processing could be different. |
| Huawei, HiSilicon | We support QC’s explanation of the motivation. We proposed to add components which seems clearer. |
| Ericsson | We Do not think it is needed. Regarding the number of PDSCH in a slot, the number should not be increased from what we already have in rel16, even for the case of FDM of unicast and multicast (we could have an agreement if needed in AI 8.12.1). |
| Moderator | Summary of companies view - Support: QC, HW/HiSi - if FDM of unicast and multicast is supported, the number of PDSCHs in a slot may be larger than legacy case - Clarification is needed: ZTE, CATT, Nokia/NSB - Not support: E/// This issue can be discussed after some progress is made in AI 8.12.1 |

Medium priority question 5-6:

- Companies are encouraged to provide views on whether the type of FGs 33-3-2 and 33-3-3 should be per UE or per FSPC.

  - Per UE: Nokia, NSB

  - Per FSPC: Huawei, HiSilicon, Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority proposal 5-7:

- The name of FG 33-3-3 is revised as “Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for multicast”

- Components of FG 33-3-2 are revised as

  - Component 2: Support FDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.

  - Component 3: Support FDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.

- Components of FG 33-3-3 are revised as

  - Components 1 to 5: Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.

    - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).

  - Component 6: Support TDM-ed Type-1 HARQ-ACK codebook for multicast and unicast.

  - Component 7: Support TDM-ed Type-2 HARQ-ACK codebook for multicast and unicast.

| Company | Comment |
| --- | --- |

Low priority question 5-8:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 33-3-2 and 33-3-3

| Company | Comment |
| --- | --- |
| Samsung | For 33-3-2/3, pre-requisite can be either 33-1 or 33-2, but should not be both. |

Low priority question 5-9:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-3-2 to 33-3-3 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-4: NACK-only based HARQ-ACK feedback for multicast

In [1], FG 33-4 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | Regarding whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback, whether to separate the capability for support of PTP retransmission for multicast, and whether to separate the capability for support of PTM retransmission for multicast, from network perspective, they are the components to be included in FG33-2 preferably. As discussed in RAN1#107-e meeting, from UE vendor perspective, these could be separate UE capabilities. As compromised between network and UE, support of NACK-only feedback and PTM retransmission can be the components and support of ACK/NACK based feedback and PTP retransmission can be separate UE capabilities. Proposal 3: Updating FG33-2, adding FG33-2-1, FF33-2-2, FG33-2-3, deleting FG33-2b, deleting FG33-3-1, deleting FG33-4, and deleting FG33-7 as follows in red: 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support of semi-static configured slot-level repetition for group-common PDSCH - Support of NACK-only based feedback for dynamic multicast scheduling - Support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling. - Support of PTM retransmission for dynamic multicast scheduling; Yes; Per UE; No; No; Optional with capability signalling. 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [6] | NTT DOCOMO | Type of FG 33-4 should be the same as the prerequisite FG, FG 33-2 (i.e., per UE). Proposal 11: Type of FG 33-4 is per UE. |
| [8] | Qualcomm | 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support of NACK-only based HARQ-ACK feedback. - Support PTM retransmission for dynamically scheduled multicast based on G-RNTI; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-4a; More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource; 1. Support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [9] | OPPO | For NACK-only based feedback scheme, it can be separated from ACK/NACK based feedback scheme. Both feedback schemes are considering in different use cases and scenarios. For example, when the number of UEs are large, NACK-only based feedback scheme can be applied with shared UL resources. ACK/NACK based feedback scheme is more general by reusing the similar design in unicast, which makes it applicable and easily deployed. “NACK-only based feedback” can be an independent feature group in 33-4 as discussed in the previous meetings. Similar with ACK/NACK based feedback, enabling/disabling of NACK-only based feedback by RRC signaling can be within the same FG, and DCI-based enabling/disabling scheme can be separated. Furthermore, the retransmission scheme for NACK-only based feedback can only consider PTM retransmission, which can be added as another component in FG 33-4. - “Support NACK-only based HARQ-ACK feedback, and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC ignalling” can be separated from FG 33-2 as FG 33-4. - “Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling” can be separated as in FG 33-4a. 33-4; NACK-only based feedback for dynamic scheduling for multicast; - Support NACK-only based HARQ-ACK feedback. - Support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC ignalling. - Support PTM retransmission for multicast.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-4a; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; No; No; Optional with capability signalling. |
| [10] | Intel | - FG 33-4: - Add component for support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling 33-4-1; ACK-NACK based HARQ-ACK feedback for multicast; - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast.; 33-2. 33-4-2; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC ignalling.; 33-2. |
| [13] | MediaTek | Regarding the NACK-only based HARQ-ACK feedback for multicast, the following agreements were achieved in last meeting: Agreement Support enabling/disabling HARQ-ACK for NACK-only based feedback. - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI.. It means that the NACK-only based HARQ-ACK feedback can be disabled/enabled by group-common DCI mechanism as that of ACK/NACK based HARQ-ACK feedback mechanism. Proposal 20: For FG 33-4, adding a separate FG 33-4-1 that “DCI-based enabling/disabling NACK only-based feedback for dynamic scheduling for multicast”. 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback.; 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. 33-4-1; DCI-based enabling/disabling NACK only-based feedback for dynamic scheduling for multicast; - Support of DCI-based enabling/disabling NACK only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE Per FSPC; Optional with capability signalling. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-4: - To be merged in 33-2, or at least to be mandatorily indicated for Ues supporting 33-2, as it is an essencial functionality for groupcast. - Per UE |

Discussion

[FL1] High priority question 6-1:

- Companies are encouraged to provide views on whether to merge FG 33-4 with FG 33-2.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | It can be the main difference from SC-PTM for multicast and UE can implement it as well. |
| MediaTek | Not support. The HARQ-ACK is an optional feature for multicast, and it doesn’t need to capture it with FG 33-2 |
| ZTE | If ACK/NACK based feedback is included in FG33-2, then it is ok to have a separate FG for NACK-only based feedback. |
| Qualcomm | Not support |
| CATT | We prefer to have a separate FG for NACK-only based HARQ-ACK feedback for multicast. |
| Nokia, NSB | Support |
| LG Electronics | We prefer a separate FG for NACK-only based feedback |
| Spreadtrum | Not support |
| vivo | Not support. |
| OPPO | Not support. ACK/NACK and NACK-only based feedback scheme should be in separated FGs as they are both optional features. |
| Lenovo, Motorola Mobility | Separate FG is preferred. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | This issue can be discussed together with proposal 3-1 |
| FL3 | Based on the discussion in the GTW session on Jan 20, FG 33-4 is not merged with FG 33-2 |

[FL1] High priority question 6-2:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-4.

  - Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling.

  - Support of more than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | The first can be a separate FG with 33-4 as prerequisite. The second one is still an open issue which can be firstly discussed in AI 8.12.2, if it is agreed, then yes, it can be a separate FG with FG 33-4 as prerequisite. |
| MediaTek | Share the similar view with HW. |
| ZTE | The second point is still pending the discussion on AI 8.12.2. |
| Qualcomm | Support to be separate capabilities |
| CATT | For the first bullet, we support having a separate FG for DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC ignalling. For the second bullet, the discussion can be postponed until some agreements/consensus are achieved in AL 8.12.2 |
| Nokia, NSB | Do not support, it depends on high-priority question 6-1 and 33-2. |
| LG Electronics | We prefer separate capabilities. |
| Spreadtrum | Share the same view with Huawei |
| vivo | We are ok to separate the 1st bullet from 33-4, and leave the 2nd bullet until there is an agreement. |
| OPPO | We share the similar view with HW/HiSi that DCI-based enabling/disabling feature can be a separated FG from 33-4, and the second proposal can be determined firstly in 8.12.2. |
| Lenovo, Motorola Mobility | Agree with Huawei. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Given majority companies are fine to separate the capability for the 1st bullet, following proposal is made [FL2] High priority proposal 6-2: - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling. Companies are encouraged to provide view on whether proposal 6-2 is acceptable or not. Please also try to address the concern from other side Regarding 2nd bullet, as suggest by many companies, this can be discussed after some progress is made in AI 8.12.2 |
| Spreadtrum | Support the proposal |
| Ericsson | OK |
| Moderator | Given majority companies support the proposal, the same proposal is set for GTW session [GTW2] High priority proposal 6-2: - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling. |
| FL3 | No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 6-2: - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling. |
| MediaTek | Support the proposal |
| Moderator | [GTW3] The same proposal is set for GTW session |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given no concern has not been received for more than 24 hours. This proposal is set for email approval after final check. |
| Moderator | Following agreement was made over the RAN1 reflector Agreement - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling. |

Medium priority question 6-3:

- Companies are encouraged to provide views on whether the type of FG 33-4 should be per UE or per FSPC

  - Per UE: NTT DOCOMO, Nokia, NSB

  - Per FSPC: Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority proposal 6-4:

- Components of FG 33-4 are revised as

  - Add a component for support of PTM retransmission for dynamically scheduled multicast based on G-RNTI

| Company | Comment |
| --- | --- |

Low priority question 6-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-4.

| Company | Comment |
| --- | --- |

Low priority question 6-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-4 which do not have capability ignalling impacts

| Company | Comment |
| --- | --- |

- 33-5-1 to 33-5-2: SPS group-common PDSCH for multicast

In [1], FGs 33-5-1 to 33-5-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | The following agreements achieved in RAN1#106bis-e and RAN1#107-e are not (completely) reflected in the UE feature groups for multicast SPS. Agreement For multicast in RRC_CONNECTED state, - Only SPS-Config-Multicast(s) configured in CFR for multicast can be activated/deactivated by GC-PDCCH with G-CS-RNTI. - SPS-Config-Multicast(s) configured in CFR for multicast cannot be activated by unicast PDCCH with CS-RNTI, but can be deactivated by unicast PDCCH with CS-RNTI. Agreement: For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED Ues. - Config A or Config B can be configured to UE: - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast. - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. - If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH. - For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is - Alt1: equal to 1. Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. Agreement For multicast SPS PDSCH without PDCCH scheduling, support the following: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - The configuration is per G-CS-RNTI. - Separate UE capability is needed from that for dynamic scheduling for multicast. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled. - The configuration is per G-CS-RNTI.. Similar to MBS broadcast FG33-1 and multicast FG33-2, support of slot-level repetition for SPS group-common PDSCH can be the component of FG33-5-1. Support of NACK-only based HARQ-ACK feedback for multicast SPS group-common PDSCH without PDCCH scheduling, support of PTM retransmission for multicast SPS, and support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC ignalling can be the components of FG33-5-1. Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling can be a separate UE capability. Support of ACK/NACK based HARQ-ACK feedback for multicast SPS group-common PDSCH without PDCCH scheduling, support of enabling/disabling ACK/NACK based HARQ-ACK feedback for multicast SPS scheduling per the configuration of RRC ignalling, and support of PTP retransmission for multicast SPS can be defined as an additional UE feature group for multicast SPS. Support of enabling/disabling HARQ-ACK feedback in the group-common DCI for multicast SPS activation per the configuration of RRC ignalling for each G-CS-RNTI can be a separate UE capability with either FG33-5-1 or FG33-5-1b as prerequisite UE feature. Support of SPS group-common PDSCH deactivated by unicast PDCCH with CS-RNTI can be a separate UE capability. Support of one or more multicast SPS activated with N multicast SPS configurations can be in different UE feature groups. Proposal 7: Updating FG33-5-1/2, adding FG33-5-1a/1b/1c/1d/2b as follows in red: 33-5-1; SPS group-common PDSCH for multicast; - Support of one SPS group-common PDSCH configuration for multicast. - Support of one SPS group-common PDSCH activated/deactivated by group-common PDCCH scrambled with G-CS-RNTI. - Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. - Support of ACK/NACK based feedback for multicast SPS activation/deactivation. - Support of NACK-only based HARQ-ACK feedback for multicast SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for multicast SPS. - Support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling. - Support of slot-level repetition for SPS group-common PDSCH.; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-5-1a; Multicast SPS scheduled by DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-1b; ACK/NACK based HARQ-ACK feedback for multicast SPS; - Support of ACK/NACK based HARQ-ACK feedback for multicast SPS group-common PDSCH without PDCCH scheduling. - Support of enabling/disabling ACK/NACK based HARQ-ACK feedback for multicast SPS scheduling per the configuration of RRC signaling - Support of PTP retransmission for multicast SPS.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-1c; Group-common DCI indicating the enabling/disabling HARQ-ACK feedback for multicast SPS; Support of enabling/disabling HARQ-ACK feedback in the group-common DCI for multicast SPS activation per the configuration of RRC ignalling for each G-CS-RNTI.; 33-5-1, 33-5-1b; Yes; Per UE; No; No; Optional with capability signalling. 33-5-1d; Multicast SPS deactivated by unicast PDCCH; Support of SPS group-common PDSCH deactivated by unicast PDCCH with CS-RNTI.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-2; Multiple SPS group-common PDSCH configuration with one activated; - Support [N>1] SPS group-common PDSCH configuration for multicast - Support of one SPS group-common PDSCH activated/deactivated by GC-PDCCH with G-CS-RNTI; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-2b; Multiple SPS group-common PDSCH configuration with multiple activated; - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [N>=M>1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [3] | vivo | Similar as dynamic scheduled group-common PDSCH reception, capability of ACK/NACK and NACK-only based HARQ –ACK feedback should be separated from data reception in 33-5-1. Proposal 10 Separate ACK/NACK and NACK-only based HARQ –ACK feedback from 33-5-1. |
| [6] | NTT DOCOMO | Component 5 (i.e., Support slot-level repetition for SPS group-common PDSCH) can be separated from FG 33-5-1 and merged with FG 33-3-1 (i.e., Slot-level repetition for group-common PDSCH). Since support of SPS unicast PDSCH is per UE, support of SPS GC-PDSCH should also be per UE. Proposal 12: Support of slot-level repetition for SPS group-common PDSCH is separated from FG 33-5-1 and merged with FG 33-3-1. Proposal 13: Type of FG 33-5-1 is per UE. Type of FG 33-5-2 should be the same as the prerequisite FG, FG 33-2 (i.e., per UE). Proposal 14: Type of FG 33-5-2 is per UE. |
| [7] | Spreadtrum Communications | Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2, 3 in FG33-5-1 as separated FG. Proposal 7: Component 2, 3 in FG33-5-1 can be split as separated FG, and as optional UE capability. Dynamic repetition number indication and semi-static repetition number indication for SPS group-common PDSCH are considered. It is similar to repetition indication for dynamic group-common PDSCH. In our mind, there is no necessary to have additional FG for SPS. We are fine that FG 33-3-1 is also applied for SPS group-common PDSCH. For the component 1 of FG33-5-2, if taking FG12-2 as reference, it shall be per CFR configuration, and further constraint on per cell or cell group can also be considered. Proposal 8: Component 1 in FG33-5-2 can be revised as ‘Support up to [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group’. Last meeting, we have discussed about the deactivation mechanism of the SPS group-common PDSCH. Although the benefit is not very clear to us, but finally for the majority, it is compromised to support the the following agreement. Agreement For multicast in RRC_CONNECTED state, - Only SPS-Config-Multicast(s) configured in CFR for multicast can be activated/deactivated by GC-PDCCH with G-CS-RNTI. - SPS-Config-Multicast(s) configured in CFR for multicast cannot be activated by unicast PDCCH with CS-RNTI, but can be deactivated by unicast PDCCH with CS-RNTI. In our understanding, there would be two typical types of Ues supporting MBS: one is MBS only Ues, and the other is UE supporting both unicast service and MBS service. At least for MBS only Ues, when not supporting unicast SPS PDSCH, there is no reason to mandatory Ues to support unicast CS-RNTI and related PDCCH interpretation, just only in order to release SPS group-common PDSCH. Thus, we have the following proposal. Proposal 11: Suggest to add one new FG to reflect the following. - The function of SPS group-common PDSCH deactivated by unicast PDCCH with CS-RNTI is optional UE capability |
| [8] | Qualcomm | 33-5-1; SPS group-common PDSCH for multicast; - Support of one SPS group-common PDSCH configuration for multicast - Support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast. - [Support of slot-level repetition for group-common PDSCH scheduled by using DCI format 4_1 with CRC scrambled with G-CS-RNTI] - FFS: HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release PDCCH.; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1a; SPS multicast using DCI format 4_2; - Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS transmission.; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1b; ACK/NACK-based HARQ ACK feedback for SPS multicast; - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1c; DCI-based enabling/disabling ACK/NACK-based feedback for SPS multicast; - Support of DCI-based enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-CS-RNTI for multicast by RRC ignalling by using DCI format 4_2; 33-5-1a, 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1d; PTP retransmission for SPS multicast; - Support PTP retransmission for SPS group-common PDSCH based on CS-RNTI; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1e; NACK-only-based HARQ ACK feedback for SPS multicast; - Support of NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-1f; Slot-level repetition of SPS group-common PDSCH for multicast by using DCI format 4_2; - Support of slot-level repetition for group-common PDSCH by using DCI format 4_2 with CRC scrambled with G-CS-RNTI; 33-5-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-5-2; Multiple SPS group-common PDSCH configuration; - Support [N>1] SPS group-common PDSCH configurations per CFR for multicast - Support [N>=M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per FSPC; N/A; N/A; FFS: value of N, M; Optional with capability signalling. |
| [9] | OPPO | For FG 33-5-1, at least supporting one SPS GC-PDSCH configuration for multicast is the basic function. The rest components 2~5 can be separated from FG 33-5-1. HARQ-ACK feedback is can be enabled/disabled with different feedback schemes. Furthermore, slot-level repetition can also be separated to an independent FG. - For FG 33-5-1, component 2/3/4 can be separated from this FG as in 33-5-4, 33-5-5 and 33-5-6. - For FG 33-5-1, component 5 can be separated from this FG into an independent FG 33-y. For FG 33-5-2, the two components should be considered separately. Supporting configuration of multiple SPS GC-PDSCH and activate 1 SPS at one time for multicast is considered basic and general, but supporting to activate more than 1 SPS at one time may have different impact on UE capability as well as unicast SPS. - For FG 33-5-2, support multiple SPS configurations and activated 1 SPS for multicast can be kept as one FG in 33-5-2. - For FG 33-5-2, support activated multiple SPS for multicast can be separated as in 33-5-3. 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-5-2; Multiple SPS group-common PDSCH configuration; - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33-5-3; Multiple SPS group-common PDSCH configuration; Support [M>1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2, 33-5-2; Yes; Per UE; No; No; Optional with capability signalling. 33-5-4; SPS group-common PDSCH for multicast; Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-5; SPS group-common PDSCH for multicast; Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-5-6; SPS group-common PDSCH for multicast; HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH.; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33-y; SPS group-common PDSCH for multicast; Support slot-level repetition for SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. |
| [13] | MediaTek | Regarding the SPS group-common PDSCH for multicast, we need to further discuss whether/how to separate the capabilities from FG 33-5-1. Actually, we think the capability for SPS should be align with the dynamic scheduling and have the following proposal: Proposal 21: For FG 33-5-1, the similar separate capability for dynamic scheduling can be reused for SPS group-common PDSCH for multicast. 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Only support ACK/NACK based HARQ feedback for SPS activation/deactivation - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1; 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. 33-5-1a; SPS group-common PDSCH for multicast; - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Per UE Per FSPC; Optional with capability signalling. 33-5-1b; SPS group-common PDSCH for multicast; - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.; 33-5-1; Per UE Per FSPC; Optional with capability signalling. 33-5-1c; PTM retransmission for SPS multicast; - Support PTM retransmission of SPS group common PDSCH for multicast; 33-5-1; Per UE Per FSPC; Optional with capability signalling. 33-5-1d; PTP retransmission for SPS multicast; - Support PTP retransmission of SPS group common PDSCH for multicast; 33-5-1; Per UE Per FSPC; Optional with capability signalling. 33-5-1e; Semi-static slot-level repetition for SPS group-common PDSCH; - Support of semi-static slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Per UE Per FSPC; For component 1 the repetition number can be n2, n4, n8 slots if pdsch-AggregationFactor configured; Optional with capability signalling. 33-5-1f; Dynamic slot-level repetition for SPS group-common PDSCH; - Support of dynamic slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Per UE Per FSPC; For component 1 the repetition number can be n2, n3, n4, n5, n6, n7, n8, n16 slots if repetitionNumber configured; Optional with capability signalling. |
| [14] | CMCC | There were also some discussions about whether to separate the capabilities in FG 33-5-1 similar the discussion in FG 33-2. In general, we think the same principle of multicast dynamic scheduling and multicast SPS should be applied, thus we suggest the following proposal. Proposal 7. For FG 33-5-1: - Support both NACK-only and ACK/NACK based HARQ-ACK feedback as a component of FG 33-5-1; - Don’t support to separate the capability for support of PTP retransmission for multicast; - Don’t support to separate the capability for support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission; - The capability for HARQ-ACK feedback mode selection for for SPS group-common PDSCH without PDCCH scheduling can be separated. Similar as the discussion in FG 33-3-1, we think the capability of PDSCH repetition for multicast SPS PDSCH without scheduling can be separated from FG 33-5-1, but the semi-static and dynamic can not be separated. Proposal 8. Support of repetition for SPS group-common PDSCH for multicast can be separated from FG 33-5-1 - Don’t support to separate semi-static slot-level repetition and dynamic slot-level repetition. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-5-1: - Per UE - 33-5-2: - Per UE |
| [16] | Ericsson | For SPS, retransmission can be done via PTM or PTP. Therefore how retransmission is supported for SPS should also be a separate feature component. Finally we propose that feature 4 is clarified to be relevant for SPS activation and release. The following change is proposed: 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common activation with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH - Support of PTM retransmission for SPS multicast - Support of PTP retransmission for SPS multicast. |

Discussion

[FL1] High priority question 7-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-5-1

  - Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

    - Keep in FG 33-5-1: CMCC

    - Separate FGs: Huawei, HiSilicon, Qualcomm

  - Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.

    - Keep in FG 33-5-1: CMCC, Ericsson

    - Separate FGs: Huawei, HiSilicon, vivo, Spreadtrum Communications, Qualcomm, OPPO, MediaTek

  - Support of NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling.

    - Keep in FG 33-5-1: Huawei, HiSilicon, CMCC, Ericsson

    - Separate FGs: vivo, Spreadtrum Communications, Qualcomm, OPPO, MediaTek

  - Support of slot-level repetition for SPS group-common PDSCH for multicast.

    - Whether to separate semi-static slot-level repetition and dynamic slot-level repetition.

    - Keep in FG 33-5-1: Huawei, HiSilicon, Ericsson

    - Separate FGs: NTT DOCOMO, Qualcomm, OPPO, MediaTek, CMCC

  - Support of DCI-based enabling/disabling ACK/NACK-based HARQ-ACK feedback per G-CS-RNTI for multicast by RRC signaling by using DCI format 4_2.

    - Separate FGs: Huawei, HiSilicon

  - Support of PTM retransmission for SPS group-common PDSCH based on CS-RNTI.

    - Keep in FG 33-5-1: Huawei, HiSilicon, Ericsson

    - Separate FGs: Qualcomm, OPPO, MediaTek

  - Support of PTP retransmission for SPS group-common PDSCH based on CS-RNTI.

    - Keep in FG 33-5-1: CMCC, Ericsson

    - Separate FGs: Huawei, HiSilicon, Qualcomm, OPPO, MediaTek

  - Support of SPS group-common PDSCH deactivation by unicast PDCCH with CS-RNTI.

    - Separate FGs: Spreadtrum Communications

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | In addition to our views expressed in FL’s summary, support of SPS group-common PDSCH deactivation by unicast PDCCH with CS-RNTI can be separate |
| MediaTek | Support to separate above capabilities from FG 33-5-1 |
| ZTE | Most of the issues are similar as that for dynamic-scheduling multicast. We propose to address the issue for dynamic-scheduling multicast first, and then we could have the same design for SPS here. |
| Xiaomi | We don’t see the necessity to split dynamic repetition and semi-static repetition. We are fine for the others. |
| CMCC | Reuse the same principle as dynamic scheduling. |
| CATT | We prefer to keep these capabilities in FG 33-5-1. |
| Nokia, NSB | This might lead to excessive fragmentation of capabilities. Agree with other companies that it is better to resolve similar issues first and apply same principles here for consistency. |
| Spreadtrum | Support all. We also have the same view with Huawei that support of SPS group-common PDSCH deactivation by unicast PDCCH with CS-RNTI can be separated UE capability. |
| NTT DOCOMO | We would like to prioritize the discussion for dynamic scheduling. FG design for SPS should follow the design for dynamic scheduling. |
| OPPO | OK to separate the components from FG 33-5-1 as mentioned above. |
| Apple | Support to separate above capabilities from FG 33-5-1 |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | This issue can be discussed when FG 33-2 is stable |
| FL3 | A proposal for updating FG 33-5-1 is made based on the agreement for FG 33-2 [FL3] High priority proposal 7-1: - Component 5 is kept in FG 33-5-1 - FFS whether to include semi-static repetition only or both semi-static and dynamic repetitions - FFS max value of dynamic repetition - FFS max value of semi-static repetition - Components 2/3/4 are separated from FG 33-5-1, FFS how to separate |
| MediaTek | The similar mechanism achieved in FG33-2 can be reused for SPS scheduling. |
| ZTE | We also think we can first address the issue for DG-PDSCH and then reuse the similar mechanism for SPS. |
| Apple | Agree with MediaTek. |
| LG Electronics | Agree with MediaTek. |
| Huawei, HiSilicon | Semi-static repetition can be the component with dynamic one as separate FG. |
| Nokia, NSB | Agree with MediaTek here. |
| Moderator | As suggested by many companies, this proposal can be further discussed when FG 33-2 is stable |

[FL1] High priority question 7-2:

- Companies are encouraged to provide views on whether to separate FG33-5-2 into a FG for support of one activated SPS group-common PDSCH configuration and a FG for support of [N>=M>1] activated SPS group-common PDSCH configurations.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | It can be separate to facilitate UE implementation so that it can be commercialized with high possibility. |
| MediaTek | Basically ok. |
| ZTE | Ok to have separate UE capability. They can also be included in the same UE capability but with a value to report the number of activated SPS. |
| Qualcomm | Support |
| Xiaomi | We are fine with the proposal. |
| Nokia, NSB | Not clear if needed, as commented by ZTE it can be done within same FG as well. |
| Spreadtrum | Fine with separated capability, and also fine to keep it in the FG while candidate value can include {1,2, …, M} |
| NTT DOCOMO | We share similar view with ZTE. |
| OPPO | Support to separated FG 33-5-2 into FGs including one activated SPS and more than one activated SPS. |
| Apple | Support the proposal |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | As commented by some companies, they can be included in the same FG but with a value to report the number of activated SPS. Therefore, following proposal is made [FL2] High priority proposal 7-2: - FG33-5-2 is not separated into multiple FGs - Supported number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values Companies are encouraged to provide view on whether proposal 7-2 is acceptable or not. Please also try to address the concern from other side |
| ZTE | Ok with the FL proposal 7-2. |
| vivo | Ok |
| Qualcomm | The number M of activated SPS group-common PDSCH should be max number M? |
| Xiaomi | OK with the proposal. |
| Ericsson | Support the proposal, and the update from Qualcomm. |
| Moderator | Given most companies are fine with the proposal, the same proposal is set for GTW session with a clarification [GTW2] High priority proposal 7-2: - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values |
| FL3 | No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 7-2: - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values |
| MediaTek | Sorry for late response the issue. From my observation, majority views are ok with the original proposal. Why not keep the original proposal? If majority views are ok with the current proposal, we want to make it clear from UE’s perspective and modify it as following: - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values - UE can report one value from the candidate values. |
| ZTE | Ok with the proposal. MediaTek’s version is also ok. |
| OPPO | OK with the updated version by MTK. |
| Spreadtrum | Either is fine to us. |
| Moderator | The proposal is update based on the comment from MTK [GTW3] High priority proposal 7-2: - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values - UE can report one value from the candidate values. |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given no concern has not been received for more than 24 hours. This proposal is set for email approval after final check. |
| Moderator | Following agreement was made over the RAN1 reflector Agreement - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values - UE can report one value from the candidate values. |

Medium priority question 7-3:

- Companies are encouraged to provide views on whether the type of FGs 33-5-1 and 33-5-2 should be per UE or per FSPC

  - FG 33-5-1

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, OPPO, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

  - FG 33-5-2

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, OPPO

    - Per FSPC: Qualcomm, MediaTek, Nokia, NSB

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority proposal 7-4:

- Components of FG 33-5-1 are revised as

  - Component 4: FFS: HARQ-ACK feedback for SPS group-common with PDCCH activation scheduling and SPS release PDCCH.

  - Add a component for support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast.

  - Add a component for support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast.

- Components of FG 33-5-2 are revised as

  - Component 1: Support [N>1] SPS group-common PDSCH configuration per CFR for multicast.

  - Component 2: Support [N>=M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast.

| Company | Comment |
| --- | --- |

Low priority question 7-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-5-1 and 33-5-2.

| Company | Comment |
| --- | --- |

Low priority question 7-6:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-5-1 and 33-5-2 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 33-6-1 to 33-6-3: HARQ-ACK with different priorities for multicast

In [1], FGs 33-6-1 to 33-6-3 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | For dynamic scheduling, the priority indication can be included in DCI format 4_2 for multicast. The priority index cannot be directly configured by RRC signalling, although it means low priority when the priority indication is not included in the DCI format. Therefore, separating UE capabilities of supporting priority configuration from supporting priority indication included in DCI format does not seem necessary. For multicast SPS scheduling, the priority index is configured together with the configuration for SPS. There could be an UE capability of support of priority configuration for multicast SPS but it is not needed to have a capability defined for support of priority indicator field in the DCI format just for multicast SPS scheduling. Proposal 8: Updating FG33-6-1/2/3, adding FG33-6-1a as follows in red: 33-6-1; DL priority indication in DCI for multicast dynamic scheduling; Support of priority indicator field configured in DCI format 4_2 with CRC scrambled with G-RNTI for multicast dynamic scheduling.; 33-2-1, 33-2-2,; Yes; Per UE; No; No; Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.; Optional with capability signalling. 33-6-1a; DL priority configuration for multicast SPS scheduling; Support of priority index configuration for multicast SPS scheduling.; 33-5-1; Yes; Per UE; No; No; Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.; Optional with capability signalling. 33-6-2; Two ACK/NACK based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE.; 1. Support of two ACK/NACK based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.; 33-2-2, 33-6-1; Yes; Per UE; No; No; Optional with capability signalling. 33-6-3; More than one PUCCH for ACK/NACK based HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-2; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [5] | Samsung | For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well. Index; Feature group; Components. 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI and G-CS-RNTI for multicast.. |
| [6] | NTT DOCOMO | Components description needs to be updated based on the agreed DCI format. Since support of priority indication for unicast is per UE, support of priority indication for multicast should also be per UE. Proposal 15: Update the components description of FG 33-6-1 as following. 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI format 4_21_1 with CRC scrambled with G-RNTI for multicast.. Proposal 16: Type of FG 33-6-1 is per UE. |
| [7] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 1 of FG 33-6-1, DCI format 1_1 is adjusted as DCI format 4_2; |
| [8] | Qualcomm | 33-6-1a; DL priority of multicast HARQ-ACK feedback for dynamically scheuled multicast; 1. Support of priority configured for multicast HARQ-ACK feedback of dynamically scheuled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.; 33-2b or 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-6-1b; DL priority indication for dynamically scheduled multicast in DCI; 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast.; 33-2a, 33-6-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-6-1c; DL priority of multicast HARQ-ACK feedback SPS multicast; 1. Support of priority configured for multicast HARQ-ACK feedback of SPS multicast; 33-5-1b or 33-5-1d; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-6-1d; DL priority indication for SPS multicast with PDCCH scheduling; 1. Support of priority indicator field configured in DCI format 4_2 with CRC scrambled with G-CS-RNTI for SPS multicast.; 33-5-1a, 33-6-1c; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-6-2; Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE.; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.; 33-6-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-6-3; More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [9] | OPPO | For FG 33-6-1, support of priority indicator field configured in DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - For FG 33-6-1, there is no need to separate the capability. 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI formats 4_1 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-6-1: - Per UE - 33-6-2: - Per UE - 33-6-3: - Per UE |

Discussion

[FL1] High priority question 8-1:

- Companies are encouraged to provide views on whether to separate following capabilities from FG 33-6-1.

  - Support of DL priority configuration (not indication) for multicast.

    - Whether to separate ACK/NACK-based HARQ-ACK feedback and NACK-only-based HARQ-ACK feedback.

    - Whether to separate dynamically scheduled multicast and SPS multicast.

  - Support of DL priority indication for multicast.

    - Whether to separate dynamically scheduled multicast and SPS multicast PDCCH activation.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | It would be better to clarify “DL priority configuration” is for SPS and DL priority indication is for DL scheduling if it is the intention from moderator because otherwise it might be confusing whether it is new functionality to be supported. If our understanding is correct, the bullet “Whether to separate dynamically scheduled multicast and SPS multicast under support of DL priority configuration (not indication) for multicast is unclear. ” |
| CMCC | Don’t see the motivation to differentiate ACK/NACK and NACK-only based HARQ feedback for DL priority configuration. |
| CATT | We prefer to keep them into FG33-6-1. |
| Nokia, NSB | Not clear if needed |
| NTT DOCOMO | We don’t support to separate FGs. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | More input is necessary. If not yet provided, companies are encouraged to provide views on this issue |
| Huawei, HiSilicon | I guess the motivation to separate them was because ACK/NACK and NACK-only will be probably separate UE FG. |

Medium priority question 8-2:

- Companies are encouraged to provide views on whether the type of FGs 33-6-1 to 33-6-3 should be per UE or per FSPC

  - FGs 33-6-1

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, OPPO, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

  - FGs 33-6-2

    - Per UE: Huawei, HiSilicon, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

  - FGs 33-6-3

    - Per UE: Huawei, HiSilicon, Nokia, NSB

    - Per FSPC: Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority proposal 8-3:

- Name of FG 33-6-1 is revised as “DL priority indication in DCI for multicast dynamic scheduling”.

- Components of FG 33-6-1 are revised as

  - Component 1: Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast.

- Name of FG 33-6-2 is revised as “Two ACK/NACK-based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with: different priorities for multicast or for unicast and multicast at a UE”.

- Components of FG 33-6-2 are revised as

  - Component 1: Supports of two ACK/NACK-based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.

- Name of FG 33-6-3 is revised as “More than one PUCCH for ACK/NACK-based HARQ-ACK transmission for multicast or for unicast and multicast within a slot”.

- Components of FG 33-6-3 are revised as

  - Component 1: Supports of two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.

| Company | Comment |
| --- | --- |

Low priority question 8-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-6-1 to 33-6-3.

| Company | Comment |
| --- | --- |

Low priority question 8-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-6-1 to 33-6-3 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| Samsung | For 33-6-1, we assume that activation DCI with G-CS-RNTI would also indicate priority. Then, G-CS-RNTI needs to be added as well. |

- 33-7: Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback

In [1], FG 33-7 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | Regarding whether to support NACK-only and/or ACK/NACK based HARQ-ACK feedback, whether to separate the capability for support of PTP retransmission for multicast, and whether to separate the capability for support of PTM retransmission for multicast, from network perspective, they are the components to be included in FG33-2 preferably. As discussed in RAN1#107-e meeting, from UE vendor perspective, these could be separate UE capabilities. As compromised between network and UE, support of NACK-only feedback and PTM retransmission can be the components and support of ACK/NACK based feedback and PTP retransmission can be separate UE capabilities. Proposal 3: Updating FG33-2, adding FG33-2-1, FF33-2-2, FG33-2-3, deleting FG33-2b, deleting FG33-3-1, deleting FG33-4, and deleting FG33-7 as follows in red: 33-2; Dynamic scheduling for multicast; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support of semi-static configured slot-level repetition for group-common PDSCH - Support of NACK-only based feedback for dynamic multicast scheduling - Support of enabling/disabling NACK-only based HARQ-ACK feedback per configuration of RRC signaling. - Support of PTM retransmission for dynamic multicast scheduling; Yes; Per UE; No; No; Optional with capability signalling. 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 33-2; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [6] | NTT DOCOMO | In RAN1#107-e, the following agreement was made [2]. Agreement Support enabling/disabling HARQ-ACK for NACK-only based feedback. - The relevant agreements made for ACK/NACK based feedback can be extended for the support of NACK-only, including: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI and the configuration is per G-RNTI. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled and the configuration is per G-RNTI.. NACK-only based feedback can be enabled/disabled as well as ACK/NACK based feedback. So there is no need to limit the components description to ACK/NACK based. Type of FG 33-7 should be the same as the prerequisite FG, FG 33-7 (i.e., per UE). Proposal 17: Update the components description of FG 33-7 as following. 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback. Proposal 18: Type of FG 33-7 is per UE. |
| [7] | Spreadtrum Communications | Since 33-2b has captured the function of supporting DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling, and we have already agreed to support DCI-based enabling/disabling for NACK-only based HARQ-ACK feedback, we suggest to revise FG33-7. Proposal 9: For FG33-7, suggest to revise [ACK/NACK based] as NACK-only. |
| [8] | Qualcomm | 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [9] | OPPO | After RAN1#107-e meeting, FG 33-7 is kept in the updated UE features list for Rel-17 NR document. However, this feature is captured as 33-2b. 33-7 can be removed from the feature list. - FG 33-7 can be removed from the feature list, because it has been captured into FG 33-2b. |
| [10] | Intel | - FG 33-7: - Add NACK-only feedback also to this FG 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK and NACK-only based] HARQ-ACK feedback; 33-2. |
| [12] | Xiaomi | In the updated RAN1 UE features list, FG 33-7 is captured for supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback.[1] However, FG 33-2b is already specified to support DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast, which fully covers the current FG 33-7. Proposal 4: FG 33-2b is sufficient and remove FG 33-7 in the MBS UE feature list. |
| [14] | CMCC | As the discussion in FG 33-2, we think both ACK/NACK based HARQ-ACK feedback and NACK-only based NARQ-ACK feedback should be as the component of FG 33-2 and GC-PDCCH based HARQ-ACK enabling/disabling is supported for these two HARQ-ACK feedback options, thus, we think NACK-only based feedback should also be included in FG 33-7. Proposal 9. NACK-only based feedback should also be included in FG 33-7. |
| [15] | Nokia, Nokia Shanghai Bell | - 33-7: - Per UE |

Discussion

[FL1] High priority question 9-1:

- Companies are encouraged to provide views on whether to remove FG 33-7.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | In general, the capability in FG33-7 is needed. The question is where it is grouped/indexed in the UE feature list. For example, whether it is group together with ACK/NACK based or NACK-only based or whether it is grouped together with HARQ-ACK for SPS. This proposal can wait a while after other related issues progress further. |
| ZTE | We share similar view with Huawei above. |
| Qualcomm | FG 33-7 is needed. |
| Xiaomi | As we analysed in our contribution, FG33-7 is redundant. |
| CATT | We prefer to remove FG 33-7. Since it has been captured in FG 33-2b. |
| NTT DOCOMO | FG 33-7 seems to be redundant. |
| OPPO | OK to remove FG 33-7 since it has been merged into FG 33-2b. |
| Apple | Share the views with Huawei, the proposal can wait for now. |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | As suggested by some companies, this issue can be discussed when related FGs are stable |
| FL3 | It seems FG 33-7 is not necessary since FG 33-2b covers ACK/NACK based case and proposal 6-2 covers NACK-only based case |
| Moderator | Sorry for late response. We think the FG 33-7 is needed. we share our understanding as following: FG 33-2b means that the RRC can configure the ACK/NACK enabling/disabling function, which is also aligned with the proposal 6-2 for NACK only case as copied following ref part. FG 33-7 means that the UE can support the DCI enable/disable the ACK/NACK or NACK only if RRC configure the function, that’s the reason why we want to deleted the wording of “ACK/NACK based”in FG33-7. The Prerequisite feature groups can be marked as yellow since it has not stable. Thus, we suggest keep the FG33-7 and delete “ACK/NACK based” wording and detailed component description can be further discussed if it is still not clear. 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; [33-2]; Yes; Per UE; No; No; Optional with capability signalling. Ref: 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33-7; Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback; 33-2; Yes; Per UE; No; No; Optional with capability signalling. No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 6-2: - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling.. |
| ZTE | Ok. Our understanding is that this FG applies to both type-1 and type2 codebook for multicast. |
| Moderator | This issue can be further discussed when related FGs are stable |

Medium priority question 9-2:

- Companies are encouraged to provide views on whether NACK-only based feedback should also be included in FG 33-7.

| Company | Comment |
| --- | --- |
| Qualcomm | Better to be a separate FG on condition of FG 33-4 |

Medium priority question 9-3:

- Companies are encouraged to provide views on whether the type of FG 33-7 should be per UE or per FSPC

  - Per UE: NTT DOCOMO, Nokia, NSB

  - Per FSPC: Qualcomm, MediaTek

| Company | Comment |
| --- | --- |
| Qualcomm | We object to make it as ‘per UE’ given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Low priority question 9-4:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-7

| Company | Comment |
| --- | --- |

Low priority question 9-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-7 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- Other FGs

This section discusses other FGs which are not included in [1].

Following feedbacks are provided in contributions for the RAN1#107bis-e meeting.

| [2] | Huawei, HiSilicon | As discussed in [3], introducing UE capability of 32 DL HARQ process can also help alleviate difficulty of network coordinating HARQ process allocation for unicast, multicast and broadcast. This can be an independent Rel-17 UE capability without any Rel-17 feature as prerequisite and can be reported per band as proposed in [4], so that this UE capability can be applicable to NR NTN, NR FR2-2 feature and NR MBS. Proposal 1: Define an independent Rel-17 UE capability of supporting 32 HARQ process numbers. |
| --- | --- | --- |
| [8] | Qualcomm | 33-8-1; PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-2; Up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-ConfigurationList for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b, 33-6-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-3; PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-4; Up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a seperate PUCCH-ConfigurationList for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4, 33-6-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-5; PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH; 1. Support of a SPS-PUCCH-AN-List for multicast ACK/NACK-based HARQ-ACK feedback of all SPS configuration(s), separate from that of SPS unicast configurations; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[FL1] High priority question 10-1:

- Companies are encouraged to provide views on whether to introduce new FGs for PUCCH resource configuration as follows:

  - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast

  - FG for up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast

  - FG for PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast

  - FG for up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast

  - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH

| Company | Comment |
| --- | --- |
| Spreadtrum | Since other FGs which are not included in [1] are listed here for further discussion, we notice that some proposals in our paper are not listed, so we list here for your consideration and further discussion. 33. NR_MBS; 33-8; Supporting mode 1 for type-1codebook construction; - Supports type1-Codebook-Generation-Mode configured as mode 1; 33-3-3-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; - Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. In addition, we also copy and paste some information from our paper to show the reason why new FGs are needed below for your reference. New FGs: In RAN11#106e meeting, we have the following agreement: Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement: - Alt 1: - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets. - Support of Alt. 1 is a UE capability Furthermore, in current 38.213 spec [3], Alt 1 has been captured as type1-Codebook-Generation-Mode = ‘mode1’, and copied below for reference. 38.213 h00 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ----------------------------------unrelated text omitted---------------------------------------- e) If the UE is configured to monitor PDCCH for multicast DCI formats for serving cell c - if the UE is not provided type1-Codebook-Generation-Mode = ‘mode1’, K1 is additionally provided by the union of dl-DataToUL-ACK-ForDCI Format4_1 - if the UE is not provided dl-DataToUL-ACK-ForDCI Format4_1, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} - if the UE is provided type1-Codebook-Generation-Mode = ‘mode1’, the UE - determines a first K1,UM set as K1∩K1,M, where K1,M is a set of slot timing values for the multicast DCI formats, a second K1,U\M set as K1\K1,UM, and a third K1,M\U set as K1,M\K1,UM ----------------------------------------unrelated text omitted-------------------------------------- If the UE is configured to monitor PDCCH for DCI formats with CRC scrambled by G-RNTI or G-CS-RNTI and is provided type1-Codebook-Generation-Mode = ‘mode1’, the UE separately applies the following pseudo-code for each of the first K1,UM set, the second K1,U\M set, and third K1,M\U set as the set of slot timing values K1, and for the corresponding sets of row indexes as R to obtain first, second, and third Type-1 HARQ-ACK sub-codebooks, and concatenates the first, second, and third, Type-1 HARQ-ACK sub-codebooks to obtain the Type-1 HARQ-ACK codebook.. Thus, the following FG is preferred to be added to reflect the agreement and align with TS38.213 spec. Proposal 10: Suggest to add one new FG to reflect the following. - Supports type1-Codebook-Generation-Mode configured as mode 1 Last meeting, we have discussed about the deactivation mechanism of the SPS group-common PDSCH. Although the benefit is not very clear to us, but finally for the majority, it is compromised to support the the following agreement. Agreement For multicast in RRC_CONNECTED state, - Only SPS-Config-Multicast(s) configured in CFR for multicast can be activated/deactivated by GC-PDCCH with G-CS-RNTI. - SPS-Config-Multicast(s) configured in CFR for multicast cannot be activated by unicast PDCCH with CS-RNTI, but can be deactivated by unicast PDCCH with CS-RNTI. In our understanding, there would be two typical types of UEs supporting MBS: one is MBS only UEs, and the other is UE supporting both unicast service and MBS service. At least for MBS only UEs, when not supporting unicast SPS PDSCH, there is no reason to mandatory UEs to support unicast CS-RNTI and related PDCCH interpretation, just only in order to release SPS group-common PDSCH. Thus, we have the following proposal. Proposal 11: Suggest to add one new FG to reflect the following. The function of SPS group-common PDSCH deactivated by unicast PDCCH with CS-RNTI is optional UE capability |
| Huawei, HiSilicon | We can be ok with these additional FGs. |
| Qualcomm | Support |
| CMCC | Support |
| Moderator | This issue can be discussed after the GTW session |
| FL2 | Based on the input so far, following proposals are made [FL2] High priority proposal 10-1a: - Add FGs for PUCCH resource configuration as follows: - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast - FG for up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast - FG for PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast - FG for up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast - FG for PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH 33-8-1; PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-2; Up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-ConfigurationList for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations; 33-2b, 33-6-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-3; PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a PUCCH-Config for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-4; Up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast; 1. Support of a seperate PUCCH-ConfigurationList for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured; 33-4, 33-6-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33-8-5; PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH; 1. Support of a SPS-PUCCH-AN-List for multicast ACK/NACK-based HARQ-ACK feedback of all SPS configuration(s), separate from that of SPS unicast configurations; 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. [FL2] High priority proposal 10-1b: - Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. Companies are encouraged to provide view on whether proposals 10-1a/b are acceptable or not. Please also try to address the concern from other side |
| ZTE | Regarding FL proposal 10-1a, we think it is better to wait for the outcome of FGs for ACK/NACK and NACK-only. Otherwise, it may end up with the situation that UE supports ACK/NACK but doesn’t support configuring PUCCH resource for it. Thus, at least for the case of ‘one PUCCH-Config’, this can be merged with the support of ACK/NACK and NACK-only, instead of having just one UE capability for the PUCCH resource configuration. |
| Spreadtrum | Fine with both proposals. |
| Ericsson | Support. |
| Moderator | As suggested by ZTE, proposal 10-1a can be further discussed after some progress is made for the capabilities of HARQ-ACK feedback Given companies are fine with proposal 10-1b, the same proposal is set for GTW session [GTW2] High priority proposal 10-1b: - Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. |
| FL3 | No objection has been received for more than 24 hours. If you have strong concern, please indicate it in this round discussion [FL3] High priority proposal 10-1b: - Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. |
| Moderator | [GTW3] The same proposal is set for GTW session |
| Moderator | This proposal could not be discussed in the GTW session on Jan 24. Given no concern has not been received for more than 24 hours. This proposal is set for email approval after final check. |
| Moderator | Following agreement was made over the RAN1 reflector Agreement - Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. |

- Conclusions

Following agreements were made in this RAN1 meeting

Agreement

- A new component for support of higher layer configured slot-level repetition up to 8 for MTCH is added into FG 33-1

  - FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG

Agreement

- Slot level repetition (FG 33-3-1) is merged with FG 33-2

  - FFS whether to include semi-static repetition only or both semi-static and dynamic repetitions

    - FFS max value of dynamic repetition

    - FFS max value of semi-static repetition

- Components 6/7/8 are separated from FG 33-2, FFS how to separate

Agreement

- Only the capability for semi-static repetition is included in FG 33-2

  - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions

  - For dynamic repetition, a separate FG is introduced; UE reports one of the maximum values from {8, 16}

Agreement

- Add following FGs

  - FG 33-2a: Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

  - [FG 33-2c: PTM retransmission for multicast]

  - [FG 33-2d: PTP retransmission for multicast]

| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling. At least 33-2c or 33-2d is merged, FFS which one or both | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Agreement

- Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows

| 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Add an FG for support of mode 1 for type1 codebook generation as follows

| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | TBD | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows

| 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-4 | Yes |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG33-5-2 is not separated into multiple FGs

  - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values

  - UE can report one value from the candidate values.

Agreement

- Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows:

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

[1]	R1-2112902	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2200030	Rel-17 UE features for NR MBS	Huawei, HiSilicon

[3]	R1-2200105	Discussion on UE features for MBS	vivo

[4]	R1-2200123	Discussion on Rel-17 UE features for NR MBS	ZTE

[5]	R1-2200223	UE features for NR MBS	Samsung

[6]	R1-2200257	Discussion on Rel.17 UE features for NR MBS	NTT DOCOMO, INC.

[7]	R1-2200288	UE features for MBS	Spreadtrum Communications

[8]	R1-2200318	UE features for MBS	Qualcomm Incorporated

[9]	R1-2200354	Discussion on UE feature for NR MBS	OPPO

[10]	R1-2200394	UE Features for NR MBS	Intel Corporation

[11]	R1-2200435	Views on Rel-17 MBS UE features	Apple

[12]	R1-2200456	Discussion on UE features for NR MBS	Xiaomi

[13]	R1-2200552	Views on UE features for NR MBS	MediaTek Inc.

[14]	R1-2200603	Discussion on UE features for NR MBS	CMCC

[15]	R1-2200628	On UE features for NR MBS	Nokia, Nokia Shanghai Bell

[16]	R1-2200664	views on NR MBS UE features	Ericsson