3GPP TSG RAN WG1 #107bis-e	R1-2200260

e-Meeting, January 17th - 25th, 2022

Title:	Remaining issues on the PDCCH monitoring enhancements for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.2

Document for:	Discussion and Decision

Introduction

In the RAN1 #107 e-meeting in November 2021, the following agreements related to PDCCH monitoring [1] were reached.

Agreement

- The following limits apply for multi-slot monitoring within the group of X slots

  - The maximum number of monitored PDCCH candidates per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 20.

  - The maximum number of monitored PDCCH candidates per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 20.

  - The maximum number of non-overlapped CCEs per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 32.

  - The maximum number of non-overlapped CCEs per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 32.

- Note: Search spaces monitoring within the group of X slots will be additionally defined

Agreement

- For Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS

  - A SS is monitored within Y consecutive slots within a slot group of X slots

  - The Y consecutive slots can be located anywhere within the slot group of X slots

    - Note: There is no requirement to align the Y consecutive slots across UEs or with slot n0

  - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups

  - BD attempts for all Group (1) SSs are restricted to fall within the same Y consecutive slots

- For Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

  - SS monitoring locations can be anywhere within a slot group of X slots, with the following exception

    - BD attempts for Type0-CSS for SSB/CORESET 0 multiplexing pattern 1, and additionally for Type0A/2-CSS if searchSpaceId = 0, occur in slots with index n0 and n0+X0, where n0 is as in Rel-15, X0=4 for 480 kHz SCS and X0=8 for 960 kHz SCS.

- Supported combinations of (X,Y)

  - A UE capable of multi-slot monitoring mandatorily supports

    - For SCS 480 kHz: (X,Y) = (4,1)

    - For SCS 960 kHz: (X,Y) = (8,1)

  - A UE capable of multi-slot monitoring optionally supports

    - For SCS 480 kHz: (X,Y) = (4,2)

    - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1)

      - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

- A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots

  - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots)

  - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3)

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot

    - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)]

  - The following supersedes FG3-5b and FG3-1 definition:

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD

  - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD

Working assumption

The following values are adopted as minimum value of  for 120/480/960 kHz

- Support only search space set group switching processing capability 1 with the following values

| μ | Minimum value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

Agreement

- SS set overbooking can be allowed with multi-slot PDCCH monitoring capability same as the current specification but applied per slot group, i.e., SS set overbooking is allowed for USS in PCell and PSCell, and UE expects no overbooking for CSS in PCell and PSCell and no overbooking in SCell.

- The dropping rule for multi-slot PDCCH monitoring capability is the same as the current specification but evaluated per slot group, i.e., a UE drops UE specific search space set(s) in a slot group with higher index when SS sets are overbooked.

- Additional dropping rules are not precluded

In this contribution, we give our views on the remaining issues of PDCCH monitoring enhancements for NR between 52.6 GHz ~ 71 GHz.

Discussion on PDCCH monitoring enhancements

  - 2.1 	Default PDCCH monitoring capability

The specific definition of multi-slot PDCCH monitoring capability for 480/960 kHz is shown in the following table according to the TS 38.213 draft CR [3]:

If a UE is provided monitoringCapabilityConfig for a serving cell, the UE obtains an indication to monitor PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs - per slot, as in Tables 10.1-2 and 10.1-3, if monitoringCapabilityConfig = r15monitoringcapability, or - per span, as in Tables 10.1-2A and 10.1-3A, if monitoringCapabilityConfig = r16monitoringcapability - per group of Xs slots according to combination Xs,Ys, as in Tables 10.1-2B and 10.1-3B, if monitoringCapabilityConfig = r17monitoringcapability If the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. ...omitted... If a UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell results to a separation of every two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for one or more of the multiple combinations X,Y, the UE monitors PDCCH on the cell according to the combination X,Y, from the one or more combinations X,Y, that is associated with the largest maximum number of MPDCCHmax,X,Y,μ and CPDCCHmax,X,Y,μ defined in Table 10.1-2A and Table 10.1-3A. The UE expects to monitor PDCCH according to the same combination X,Y in every slot on the active DL BWP of a cell. For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

  - According to the previous agreements/conclusions, a UE supporting 480/960 kHz SCS supports multi-slot PDCCH monitoring for 480/960 kHz SCS and for 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. In other words, it will not cause any ambiguity if per-slot monitoring is not supported for SCS 480/960 kHz and there is no need to introduce monitoringCapabilityConfig = r17monitoringcapability.

  - Proposal 1: There is no need to introduce monitoringCapabilityConfig = r17monitoringcapability if per-slot monitoring is not supported for SCS 480/960 kHz.

  - Moreover, according to TS 38.213, if the UE is not provided monitoringCapabilityConfig, the UE monitors PDCCH on the serving cell for a maximum number of PDCCH candidates and non-overlapping CCEs per slot. However, for SCS configuration μ=5 or μ=6 in Rel-17, only multi-slot PDCCH monitoring capability is supported and per-slot monitoring is still under discussion. There is a high probability that per-slot monitoring will not be supported in FR2-2 based on previous discussions, we prefer to clarify this in the specification that if UE does not provide any monitoring capability combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960 kHz. Moreover, for SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to more combinations Xs,Ys, different Xs,Ys may correspond to different BD/CCE budge, we suggest indicating the mandatory Xs,Ys combination.

  - Proposal 2: Further clarification in TS 38.213: if a UE does not provide any monitoring capability combinations Xs,Ys or if a UE indicates a capability to monitor PDCCH according to more combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960kHz.

  - 2.2 	BD/CCE budget

In the RAN1 #107 e-meeting, it was agreed that a UE capable of multi-slot monitoring optionally supports Xs,Ys = (4, 2) for SCS 480 kHz and Xs,Ys = (8, 4), (4, 2), (4, 1) for SCS 960 kHz. The BD/CCE budget of SCS 960kHz for  (4,2), (4,1) can be scaled based on the absolute time of a slot group, and the value of  is large enough to enable the highest CCE aggregation level. The following working assumption related to BD/CCE budge was made during the meeting:

Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

According to the working assumption, Table 1 and Table 2 provide the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ and  non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 1: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 20 | 20 | - | - |
| 6 | 10 | 10 | 20 | 20 |

Table 2: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 32 | 32 | - | - |
| 6 | 16 | 16 | 32 | 32 |

  - Proposal 3: Confirm the working assumption: BD/CCE budget of 960 kHz for (4,2), (4,1) is half that of X=8.

Table 1: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 20 | 20 | - | - |
| 6 | 10 | 10 | 20 | 20 |

Table 2: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 32 | 32 | - | - |
| 6 | 16 | 16 | 32 | 32 |

  - 2.3 	Search space set configuration

- In RAN1#104bis e-meeting, the following conclusion/agreement were made. For 120kHz SCS in FR2-2, only per-slot PDCCH monitoring capability is supported. Therefore, the search space configuration including monitoringSlotPeriodicityAndOffset , duration and monitoringSymbolsWithinSlot as shown in the following table can be the same as that for 120kHz in FR2-1.

SearchSpace ::= SEQUENCE { searchSpaceId SearchSpaceId, controlResourceSetId ControlResourceSetId OPTIONAL, -- Cond SetupOnly monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639), sl1280 INTEGER (0..1279), sl2560 INTEGER (0..2559) } OPTIONAL, -- Cond Setup duration INTEGER (2..2559) OPTIONAL, -- Need R - monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, -- Cond Setup - RAN1 #104bis-e - Conclusion: - For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. - Agreement: - For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

- Proposal 4: The search space configuration for 120kHz SCS in FR2-2 including monitoringSlotPeriodicityAndOffset , duration and monitoringSymbolsWithinSlot can be the same as that for 120kHz in FR2-1.

- Multi-slot PDCCH monitoring is based on slots within a slot group, when configuring the search space set by higher layer parameter monitoringSlotPeriodicityAndOffset and duration, the gNB needs to ensure that PDCCH monitoring periodicity ks, the duration TS is an integral multiple of X slots (X slots consists a slot group). For example, if a slot group includes four slots (X=4), the duration TS can be configured as 4, 8, 12, 16, ... of slots. We can introduce new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and add other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for multi-slot PDCCH monitoring in FR2-2 are an integral multiple of slot groups.

- Proposal 5: For multi-slot PDCCH monitoring in FR2-2, we suggest introducing new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and adding other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for 480/960 kHz are an integral multiple of slot groups.

Besides, for Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS, SS monitoring locations can be anywhere within a slot group of X slots. It is hard to indicate monitoring occasions by simply using the parameter monitoringSymbolsWithinSlot due to the lack of “monitoring slots location within the slot group” information. A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz.

Proposal 6: A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz.

  - 2.4  Search space set group switching

Search Space Set Group (SSSG) Switching was introduced in Rel-16 NR-U. In order to reduce access delay and save power, the SSSG monitoring periodicity/granularity is increased or decreased according to the channel access procedure. UE has to support frequent PDCCH monitoring outside gNB-COT and infrequent PDCCH monitoring inside COT. The existing SSSG switching is applied for SCS 15/30/60 kHz, for SSSG switching on large SCS values like 120, 480 and 960 kHz, UE needs to dynamically change between two PDCCH monitoring periodicity/granularity.  Besides, the switching time Pswitcℎ with all SCSs supported in above 52.6 GHz needs to be defined if SSSG switching is agreed to be supported. We agree with the following working assumption made in RAN1#107 e-meeting indicating values of Pswitcℎ for 120/480/960 kHz.

- Support only search space set group switching processing capability 1 with the following values

| μ | Minimum Pswitcℎ value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

Proposal 7: Confirm the working assumption: Support only search space set group switching processing capability 1 with the following values

| μ | Minimum Pswitcℎ value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

  - 2.5 	Cross-carrier scheduling

- As agreed in the RAN1 #107 e-meeting, the Npdsch value of 120 kHz SCS is scaled by 4 and 8 for 480 kHz and 960 kHz SCS respectively to maintain the same absolute time duration as that of 120 kHz SCS in FR2-1. Npdsch in 38.214 Section 5.5 is the UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH.

TS 38.214 Table 5.5-1: Npdsch as a function of the subcarrier spacing of the scheduling PDCCH

| µPDCCH | Npdsch [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | 14 |
| 5 | 56 |
| 6 | 112 |

- The cross-carrier scheduling here mainly refers to cross-carrier scheduling of a cell within 52.6-71GHz band from/to a cell within FR1/FR2-1. Although a cell within 52.6-71GHz band cross-carrier schedules other cells within FR1/FR2-1 is less likely, it should not be ruled out unless we find enough proof. In our understanding, a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling.

Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz band from/to a cell within FR1/FR2-1 is supported and a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling.

Conclusion

In this contribution, we discuss PDCCH monitoring enhancements for NR above 52.6 GHz band operation and have the following proposals.

  - Proposal 1: There is no need to introduce monitoringCapabilityConfig = r17monitoringcapability if per-slot monitoring is not supported for SCS 480/960 kHz.

  - Proposal 2: Further clarification in TS 38.213: if a UE does not provide any monitoring capability combinations Xs,Ys or if a UE indicates a capability to monitor PDCCH according to more combinations Xs,Ys, Xs,Ys = (4, 1) or (8, 1) is mandatorily supported for SCS configuration μ=5 or μ=6 respectively if per-slot monitoring is not supported for SCS 480/960kHz.

  - Proposal 3: Confirm the working assumption: BD/CCE budget of 960 kHz for (4,2), (4,1) is half that of X=8.

Table 1: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 20 | 20 | - | - |
| 6 | 10 | 10 | 20 | 20 |

Table 2: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell

|  | Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ |  |  |  |
| --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 4) |
| 5 | 32 | 32 | - | - |
| 6 | 16 | 16 | 32 | 32 |

- Proposal 4: The search space configuration for 120kHz SCS in FR2-2 including monitoringSlotPeriodicityAndOffset , duration and monitoringSymbolsWithinSlot can be the same as that for 120kHz in FR2-1.

Proposal 5: For multi-slot PDCCH monitoring in FR2-2, we suggest introducing new parameters (e.g. monitoringSlotPeriodicityAndOffset-r17 and duration-r17 ) and adding other appropriate periodicity/duration values in addition to the existing configurable values. Moreover, additional restrictions are needed to make sure that the values of periodicity and duration for 480/960 kHz are an integral multiple of slot groups.

Proposal 6: A new slot level bitmap (e.g. monitoringSlotsWithinSlotGroup) needs to be introduced to indicate which slots have monitoring occasions winthin a slot group for SCS 480/960 kHz.

Proposal 7: Confirm the working assumption: Support only search space set group switching processing capability 1 with the following values

| μ | Minimum Pswitcℎ value for UE processing capability 1 [symbols] |
| --- | --- |
| 3 | 40 |
| 5 | 160 |
| 6 | 320 |

Proposal 8: Cross-carrier scheduling of a cell within 52.6-71 GHz band from/to a cell within FR1/FR2-1 is supported and a UE supporting 480 kHz and 960 kHz SCS supports cross-carrier scheduling.

Reference

- RAN1 #107-e chairman’s notes

- R1-2112755, Feature lead summary #4 for B52.6 GHz PDCCH Monitoring Enhancements, Moderator (Lenovo)

- R1-2112931, Introduction for extending NR operation to 71 GHz, 38.213 CR