3GPP TSG RAN WG1 #107bis-e						                R1-2200314

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.5

Source: 	Qualcomm Incorporated

Title: 		        UE features for NR positioning enhancements

Document for:		Discussion and Decision

# Introduction

This contribution includes our comments and suggestions on the Positioning Enhancements features appeared in R1-2112902.

# Comments

For a subset of the Feature groups found in the R1-2112902, we provide a first collection of comments and suggested changes as shown below:

## FG 27-1-1

This feature needs to be split into 2, one for DL-TDOA and one for M-RTT due to the following reasons:

- Already the M-RTT UE feature groups are separated from the  DL-TDOA

- A UE may support RxTEG reporting in DL-TDOA, but for M-RTT it may support RxTxTEG reporting and not RxTEG reporting.

- The reporting IE for TDOA are different than the reporting IE for M-RTT.

Proposal 1: Support two separate rows for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” (27-1-1b)

- Per band reporting

Proposal 2: If a UE supports the RxTEG capability with a value=1, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.

## FG 27-1-4

We are supportive of separating a Feature Group for the maximum number of Rx TEGs measuring the same DL PRS resources simultaneously. At least in FR2, supporting measuring the same DL PRS resource with multiple Rx TEGs may require having multiple panels receiving simultaneously which is a feature that it will be useful to be clearly declared to the LMF. If a UE supports such a feature, the reporting delay may be an order of magnitude less compared to the case that the UE is measuring the same PRS resources with different Rx TEGs in a TDMed fashion.

Proposal 3: Support a separate UE capability (27-1-4a) for “The maximum number of  UE Rx TEGs for measuring the same DL PRS resource simultaneously”.

## FG 27-3-2

With regards to the PRS measurement outside MG, we make the following observations:

- A UE should be able to declare in each band, whether Type 1A, 1B, 2 or multiple of such options are supported. The 3 options correspond to different delay / overhead tradeoffs.

  - In Type 1A, a UE is allowed to dedicate all memory/processing across both NR & LTE for the purpose of achieving the lowest latency possible. However, this comes with the cost that other conflicting DL channels in either LTE or NR may be dropped.

  - In Type 1B, only the channels of the same band as the DL PRS are affected, however this means that the UE will have to reserve memory/processing resources to do simultaneous processing of the DL PRS in this band with any other communication channels in the remaining bands. So, the UE would declare larger latency as a UE capability. Similarly for Type 2.

Proposal 4: With regards to the PRS measurement outside MG, a UE should be able to declare support of one or more of the Type-1A, Type-1B, Type-2” capabilities.

## FG 27-3-2

With regards to the PRS measurement outside MG, it hasn’t yet been agreed how the UE will reporting the PRS processing timeline and the required time after the last PRS symbol before the UE is capable of reporting the measurements.

In short, in a similar way as it has been done in previous low-latency scenarios in NR specification (e.g. front-loaded DMRS designs, low-latency CSIRS, ACK/NAK PDSCH’s timeline designs), we propose the UE to report the following:

- Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window, for a given DL PRS bandwidth in MHz (B), such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol, where

- N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms

- T: {N+4, N+5, N+6, N+8} ms

In other words, the UE expects the configuration of a PRS processing window to be such that the gap between the last measured PRS symbol and the end of the PRS processing window is at least as large as T-N ms that the UE reported as capability.

Proposal 5: Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window, for a given DL PRS bandwidth in MHz (B), such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol, where

- N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms

- T: {N+4, N+5, N+6, N+8} ms

## FG 27-6, 27-6a, 27-6b, 27-6c (PRS Processing in RRC Inactive)

With regards to the RRC Inactive PRS processing, we make the following observations:

- Separate set of PRS processing capabilities are needed because, a UE, in RRC inactive, may have more memory/processing resources to dedicate to PRS processing compared to the RRC connected state.

- In RRC inactive state, we think it is more appropriate the UE to report the PRS processing capabilities assuming a single contiguous time window wherein all the “N” PRS symbols

- A UE in RRC Inactive, may be able to perform simultaneous processing between TDOA & AoD or RTT & AoD, whereas it may not be able to do so in RRC connected state. Therefore, we propose a UE to be able to report separate capabilities for those cases.

Proposal 5: With regards to PRS processing capabilities in RRC inactive, we propose the UE to be able to report

- Dedicated (N,T) PRS processing capabilities for RRC inactive PRS processing

- Simultaneous processing of DL-AoD with RTT in RRC Inactive

- Simultaneous processing of DL-AoD with DL-TDOA in RRC Inactive

## FG 27-15, 27-15b (SRS Transmission in RRC Inactive)

With regards to the RRC Inactive SRS capabilities, we make the following observations:

- A different capability is needed for a UE supporting SRS within initial BWP and outside initial BWP.

- For the capability of SRS outside initial BWP, a UE should be able to report whether:

  - Different numerology between the SRS and the initial UL BWP is supported.

  - SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB

Such capabilities are already reported for the BWP capabilities (6-1, 6-4) and will be needed for this new type of SRS which is not constrained within a legacy BWP.

- Add a Note: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.

- The SRS capabilities for RRC connected state are reported in a per band per band combination fashion. However, in RRC Inactive, there is no usage of such reporting since the UE does not have Carrier Aggregation configuration. Therefore, the capabilities in RRC inactive need to be reported in a “per-band” fashion.

Proposal 6: With regards to SRS Transmission capabilities in RRC Inactive configured outside initial UL BWP, we propose the following:

- Include a component for declaring that Different numerology between the SRS and the initial UL BWP is supported.

- Include a component for declaring that SRS operation without restriction on the BW is supported: BW of the SRS may not include BW of the CORESET#0 and SSB

- Include the following notes for clarification of the UE behavior:

- Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. The SRS-only BWP inherits the legacy BWP restrictions.

- Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.

# 5 Conclusions

We make the following proposals:

Proposal 1: Support two separate rows for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” (27-1-1b)

- Per band reporting

Proposal 2: If a UE supports the RxTEG capability with a value=1, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.

Proposal 3: Support a separate UE capability (27-1-4a) for “The maximum number of  UE Rx TEGs for measuring the same DL PRS resource simultaneously”.

Proposal 4: With regards to the PRS measurement outside MG, a UE should be able to declare support of one or more of the Type-1A, Type-1B, Type-2” capabilities.

Proposal 5: Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window, for a given DL PRS bandwidth in MHz (B), such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol, where

- N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms

- T: {N+4, N+5, N+6, N+8} ms

Proposal 6: With regards to SRS Transmission capabilities in RRC Inactive configured outside initial UL BWP, we propose the following:

- Include a component for declaring that Different numerology between the SRS and the initial UL BWP is supported.

- Include a component for declaring that SRS operation without restriction on the BW is supported: BW of the SRS may not include BW of the CORESET#0 and SSB

- Include the following notes for clarification of the UE behavior:

- Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. The SRS-only BWP inherits the legacy BWP restrictions.

- Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.

Proposal 7: Support the detailed changes shown in Appendix Section 4.

# 4 NR_pos_enh (Qualcomm)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs for UE-assisted DL TDOA positioning | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA | 13-1, 13-3 | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | per band | n/a | n/a | n/a | The candidate values are {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements. Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-1b | Support of UE-RxTEGs for Multi-RTT positioning | The maximum number of UE-RxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-1, 13-4 | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | Per band | n/a | n/a | n/a | The candidate values are {1, 2, 3, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements. Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4,13-8 | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32,64, 128, 256} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  |  | Per band | n/a | n/a | n/a | The candidate values are {1,2, 4, 8} | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | DL PRS RSRP of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | 13-5 | No |  |  | per band | n/a | n/a | n/a | Component 2 candidate values: {2,4,8,16,24} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | Yes | n/a | The candidate values are {12,16, 24, 32, 64} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set | 13-1 | No |  |  | per band | n/a | n/a | n/a | The candidate values are {1} If the UE does not provide the capability, the UE is assumed to support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing priority window - processing types | Supported PRS processing type(s) subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing priority window Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (i.e. both across NR & LTE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from a certain band are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. The DL signals/channels from a certain band are affected. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | 13-1 | No |  |  | per band | n/a | n/a | n/a | Candidate values: One or more of the {Type 1A, Type 1B, Type 2} Need for location server to know if the feature is supported Note: A UE may declare support of one or more of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - within a PRS processing priority window | - Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window, for a given DL PRS bandwidth in MHz (B), such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol, where - N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms - T: {N+4, N+5, N+6, N+8} ms Note: The UE expects the configuration of a PRS processing window to be such that the gap between the last measured PRS symbol and the end of the PRS processing window is at least as large as T-N ms. 4. Max number of DL PRS resources that UE can process in a slot in a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted DL-TDOA positioning | Support reporting LoS/NLoS indicator to LMF for RSTD measurements to LMF - Support Soft Value reporting - Support Hard value reporting | 13-3 | No |  |  | per band | n/a | n/a | n/a | The candidate value are {0, 0.1, …, 0.9, 1} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-2 | LOS/NLOS Indicator for M-RTT positioning | Support reporting LoS/NLoS indicator to LMF for UE Rx-Tx time difference measurements to LMF - LOS/NLOS indicator type: {softValue, hardValue} | 13-4 | No |  |  | per band | n/a | n/a | n/a | The candidate value are {0, 0.1, …, 0.9, 1} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | Support of UE-initiated on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | 13-1 | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process in a single contiguous time window, every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: Same values as the periodicities available for Long DRX cycle ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-17a | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-6a | Max number of positioning frequency layers UE supports across all positioning methods across all bands in RRC inactive state | 1. Max number of positioning frequency layers UE supports across all positioning methods across all bands in RRC Inactive state Values: {1, 2, 3, 4} | 27-6 | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-6b | Simultaneous DL-AoD and DL-TDoA processing in RRC inactive state | - Support of simultaneous processing for DL AoD and DL TDoA measurements in RRC Inactive state If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and DL TDoA measurements | 27-6 | No | N/A |  | Per band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-6c | Simultaneous DL-AoD and RTT processing in RRC inactive state | - Support of simultaneous processing for DL AoD and RTT measurements in RRC Inactive state If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and RTT measurements | 27-6 | No | N/A |  | Per band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-7 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report | 13-1 | No |  |  | Per UE | n/a | n/a | n/a | Values: {2,4,5,8,10,16,20,32} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning | 13-1 | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} | 13-1 | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for Positioning measurements | 1. Support of using UL MAC CE to request measurement gap for Positioning measurements | 13-1 | Yes |  | Using UL MAC CE to indicate measurement gap for Positioning measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation for Positioning | 1. Support of preconfiguration of MGs in RRC for Positioning measurements 2. Support of using DL MAC CE to activate the MG for Positioning measurements 3. Maximum number of preconfigured MGs for Positioning measurements | 13-1 | Yes |  | Using DL MAC CE to activate the preconfigured MG for Positioning measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} | 13-1 | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | 1. Support of TOA reporting for more than 2 additional paths for UE-assisted DL-TDOA. 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. | 13-1 | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths for Multi-RTT 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. | 13-1 | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state configured associated within initial BWP | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP Values = {1, 2, 4, 8, 12, 16} 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} | 13-8 | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
|  | 27-15b | Support of positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | 1. Support of SRS for Positioning configured outside initial UL BWP - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC. - Max number of SRS Resource Sets for positioning supported by UE. Values = {1, 2, 4, 8, 12, 16} - Max number of periodic SRS Resources for positioning. Values = {1,2,4,8,16,32,64} - Max number of periodic SRS Resources for positioning per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} - Different numerology between the SRS and the initial UL BWP is supported. - SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB | 27-15 | Yes |  |  | Per band |  |  |  | Need for location server to know if the feature is supported. - Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. The SRS-only BWP inherits the legacy BWP restrictions. - Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported. - Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP - Note 4: If component 7 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB. |  |
| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same Elements as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-17a | Support of PRS in RRC_INACTIVE | - Support of PRS measurement in RRC_INACTIVE - Support of reporting location information in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement Note: A UE may support component 1 without supporting component 2, but not the reverse | 13-1 | Yes |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18a | Support of PRS in RRC_INACTIVE state for DL-TDOA | - Support of PRS measurement in RRC_INACTIVE state for DL-TDOA - Support of reporting location information for DL-TDOA in RRC_INACTIVE state. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. | 27-17a | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18b | Support of PRS in RRC_INACTIVE state for DL-AoD | - Support of PRS measurement in RRC_INACTIVE state for DL-AoD - Support of reporting location information for DL-TDOA in RRC_INACTIVE state. Applicable for both UE-assisted and UE-based DL-AoD Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. | 27-17a | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of reporting location information for Multi-RTT in RRC_INACTIVE state. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive. | 27-17a | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same Elements as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |

# Appendix: NR_pos_enh (R1-2110587)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | [per band or FS] | n/a | n/a | n/a | The candidate values are {[1,] 2,[ 3,] 4, 6, 8[, 12, 16, 24, 32]} Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | [13-4, ]13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: per band or per FS | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8} [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first path for DL-AoD | 1.) Support of [measuring and reporting the] PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | [13-2 or 13-3, 13-4, 13-5, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | Yes | n/a | The candidate values are {[12, ]16, 24[, 32, 64]} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set ] | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-7 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | [1. Support of TOA reporting for more than 2 additional paths.] 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} 4. FFS: Applicability for initial BWP OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-17 | Support of [PRS measurement in RRC_INACTIVE] | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |