3GPP TSG RAN WG1 #107bis-e			R1-2200317

January 17th – 25th, 2022

Agenda item:	8.15.11

Source: 	Qualcomm

Title: 	UE Features for Sidelink Enhancements

Document for:	Discussion/Decision

- Introduction

In this contribution, we propose a set of UE features for NR Sidelink Enhancements based on the preliminary feature list provided in [1] and agreements in subsequent meetings. A proposed feature list based on the proposal in this contribution is provided in the next section.

In general, many of the new features are impacted by the currently supported featureset, e.g. the ability to process the additional information required for inter-UE coordination could be impacted by support of operations in other bands that would increase the baseband processing effort. Similarly transmitting or receiving such information would be impacted by other reception and transmission. Another example is sensing and partial sensing, which both require baseband processing taking away processing resources from other operations. Therefore, we propose that the FGs be defined per featureset.

Proposal 1: UE features for sidelink enhancements are defined per featureset.

The Rel-16 UE feature list for sidelink noted that Uu-related functionality, including configuration, is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1. The same should also apply to the Rel-17 sidelink features.

Proposal 2: Configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

RAN1 agreed to have an FG 32-2 that combines support for receiving PSFCH and S-SSB with an FFS on whether to split into two FGs. Reception of PSFCH and reception of S-SSB serve different purposes and have different implementation requirements. Therefore, there is no need to group them. We propose to separate them into two FGs. While the gNB would benefit from knowing whether a UE can receive PSFCH or not, the same does not hold for S-SSB reception.

Proposal 3: Split FG 32-2 into two FGs: one for reception of PSFCH and the other for the reception of S-SSB.

As part of the abovementioned agreement, it was noted that RAN1 would need to decide on how to report the total number of supported PSFCHs transmissions and receptions and whether that value should combine PSFCH for the purpose of feedback and inter-UE coordination. The limitation on the maximum number of simulataneous PSFCH transmissions is an RF limitation, unrelated to why any PFSCH is transmitted. Similarly, the UE’s ability to support a maximum number of simultaneous PSFCH receptions is unrelated to the purpose of a PSFCH. Hence, the number of simultaneous PSFCH receptions and/or transmissions should be reported jointly for inter-UE coordination and HARQ-ACK reporting.

Proposal 4: The number of simultaneous PSFCH receptions and/or transmissions is reported jointly for inter-UE coordination and HARQ-ACK reporting.

It was agreed to use separate inter-UE coordination UE capabilities into at least two FGs: one for Scheme 1 and one for Scheme 2. Both new FG have FFSs about further splitting. The ability to utilize preferred resource information is unrelated to the ability to utilize non-preferred resources and target different scenario that need not be supported by the same device. Therefore, the FGs for Scheme 1 with preferred resource indication, for Scheme 1 with non-preferred resource indication should be separated. Moreover, a UE that utilizes coordination information does not necessarily need to be able to generate coordination information, for example, a low power device can utilize the preferred resource set to select resources but might not be performing sensing and would not be able to generate such a set. Hence, those two aspects are listed in separate FGs for each variant.

Proposal 5: Scheme 1 with preferred resource is a separate FG from Scheme 1 with non-preferred resources.

Proposal 6: The capability to receive inter-UE coordination information is separate from the capability to generate and transmit inter-UE coordination information, at least for Scheme 1 with preferred resources.

RAN1 agreed to support reception of Scheme 1 inter-UE coordination using SCI-2 as an optional feature. We introduce a new FG, FG 32-6, to incorporate this agreement into the UE feature list.

Agreement

For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

- Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

  - For the indication of resource set, the following is supported:

    - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

      - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

      - FFS: UE capability details

      - 2nd SCI is UE RX optional

- Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

  - For the indication of resource set, the following is supported:

    - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

      - First resource location of each TRIV is separately indicated by the inter-UE coordination information

- FFS: Whether/How to use resource reservation information as coordination information

Lastly, RAN1 agreed to support the following condition for determining a conflict in Scheme 2 inter-UE coordination information as an optional feature. A new FG 32-7 is added to the table in the following section capturing this decision:

Working Assumption

A resource pool level (pre-)configuration can enable one of the following options:

- Option 1:

  - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - For the case when UE-A is a destination UE of a TB transmitted by UE-B

      - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

        - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

    - For the case when UE-A is a destination UE of a TB transmitted by another UE

      - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

        - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

- Option 4:

  - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - For the case when UE-A is a destination UE of a TB transmitted by UE-B

      - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

    - For the case when UE-A is a destination UE of a TB transmitted by another UE

      - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

  - Support of Option 4 is subject to UE capability

- FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- NR_SL_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | None | [Yes] | [Yes] |  | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | [No] | [No] |  | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. FFS whether any other components should be added | None | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. supported. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. FFS whether any other components should be added | None | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per FS] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5b | Inter-UE coordination preferred resource reception | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5c | Inter-UE coordination non-preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of non-preferred resource set in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5d | Inter-UE coordination preferred nonresource reception | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5e | Inter-UE coordination expected conflict indication transmission | 1) UE can detect the presence of expected/potential resource conflict and transmit an indication for inter-UE coordination in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-5f | Inter-UE coordination expected conflict indication reception | 1) UE can receive inter-UE coordination information indicating the presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-6 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receiver Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. | None | No | No |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-7 | Determination of expected conflcit in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | None | No | No |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

# Conclusion

In addition to the changes in Section 23, we proposed the following:

Proposal 1: UE features for sidelink enhancements are defined per featureset.

Proposal 2: Configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1.

Proposal 3: Split FG 32-2 into two FGs: one for reception of PSFCH and the other for the reception of S-SSB.

Proposal 4: The number of simultaneous PSFCH receptions and/or transmissions is reported jointly for inter-UE coordination and HARQ-ACK reporting.

Proposal 5: Scheme 1 with preferred resource is a separate FG from Scheme 1 with non-preferred resources.

Proposal 6: The capability to receive inter-UE coordination information is separate from the capability to generate and transmit inter-UE coordination information, at least for Scheme 1 with preferred resources.

# References

[1]  R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.

[2]  R1-2108678, “Preliminary RAN1 UE features list for Rel-17 LTE,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.