3GPP TSG RAN WG1 #107bis-e							R1-2200339

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.7

Source: 	OPPO

Title:	Rel-17 UE Power Saving features

Document for:	Discussion

Introduction

In RAN1#107-e, the UE feature of UE power saving enhancements for NR was discussed. Including Feature Groups 29-1 Paging Enhancement, 29-2 TRS resources for IDLE/Inactive UEs and 29-3 PDCCH monitoring adaptation within an active BWP [1]. In this contribution, we share our views on NR Rel-17 UE Power Saving fratures.

UE Features of UE Power Saving Enhancements for NR

- UE feature for paging enhancement (UE feature 29-1)

In RAN1#107-e, the following UE feature 29-1 was agreed.

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the type of the UE feature, in terms of power saving, per band is a little more complicated, we think per-UE is sufficient.

Proposal 1: For the UE feature 29-1, the capability type is per UE.

- TRS resources for idle/inactive UEs (UE feature 29-2)

In RAN1#107-e, the following UE feature 29-2 was agreed.

Agreement

- Component 2 in FG 29-2 for support receiving L1 indication for TRS availability is kept

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS onfigura for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |

Regarding the type of the UE feature, per-UE is sufficient.

Proposal 2: For the UE feature 29-2, the capability type is per UE.

- PDCCH monitoring adaptation within an active BWP (UE feature 29-3)

8.7.2Agreements in RAN1#107-e: Agreement The following agreement in RAN1#106-E is updated as follows, Agreement(extracted from RAN1#106-E) - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case - A new indication field in scheduling DCI is used for indicating PDCCH monitoring adaptation Agreement - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - If M = 1, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A (stay in current SSSG) - [‘11’ is reserved] - If M = 2, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A for duration T1 (stay in current SSSG) - ‘11’ is Beh 1A for duration T2 (stay in current SSSG) - Note: for ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping ) - Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping. FFS SSSG switching timer behavior, and timer value related to the value of T1/T2

In RAN1#107-e, the following UE feature 29-2 was agreed.

Agreement

- FG 29-3 is split into multiple FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: 2 search space sets group switching

  - [FG 29-3c]: 3 search space sets group switching

  - [FG 29-3d]: 2 search space sets group switching with PDCCH skipping

  - FFS: A UE reporting 29-3a and 29-3b does not implicitly mean the UE supports FG 29-3d

  - FFS: FG for 3 search space sets group switching with PDCCH skipping

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of PDCCH skipping Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 2 search space sets grouping switching Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 3 search space sets grouping switching Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2 search space sets grouping switching with PDCCH skipping Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-2b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

In our view, supporting 3a (skipping) and 3b (2 SSSG) does not equals to support 3d (a joint signaling indication of skipping and 2 SSSG). From 8.7.2 Agreements in RAN1#107-e, we think FG 29-3d need to be supported. We don’t think that two separated feature groups 29-3b and 29-3c are needed to indication SSSG switching of 2 SSSGs and 3 SSSGs. We support combine FG 29-3b and FG 29-3c as one FG for SSSG switching up to 3 SSSGs.

Regarding the capabilities of FG 29-3, we think FG 29-3 should be split into three FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: Search space sets group switching (2 or 3 search space sets)

  - FG 29-3c: Search space sets group switching with PDCCH skipping (2 or 3 search space sets)

Proposal 3: FG 29-3 should be split into three FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: Search space sets group switching (up to 3 search space sets)

  - FG 29-3c: Search space sets group switching with PDCCH skipping (up to 3 search space sets)

Proposal 4: For the UE feature 29-3, the capability type is per UE.

Conclusions

In this contribution, RRC parameters supporting the repetition factors are discussed. As summary, we have the following proposals:

Proposal 1: For the UE feature 29-1, the capability type is per UE.

Proposal 2: For the UE feature 29-2, the capability type is per UE.

Proposal 3: FG 29-3 should be split into three FGs as follows:

  - FG 29-3a: PDCCH skipping

  - FG 29-3b: Search space sets group switching (up to 3 search space sets)

  - FG 29-3c: Search space sets group switching with PDCCH skipping (up to 3 search space sets)

Proposal 4: For the UE feature 29-3, the capability type is per UE.

Reference

- R1-2112139 Summary on UE features for UE power saving enhancements	Moderator