3GPP TSG RAN WG1 Meeting #107bis-e	R1-2200366

e-Meeting, January 17 – 25, 2022

Source:	Intel Corporation

Title:	Discussion on initial access aspects for extending NR up to 71 GHz

Agenda item:	8.2.1

Document for:	Discussion/Decision

# Introduction

The WI on NR extension up to 71 GHz in Rel-17 was finalized in the previous RAN1 #107-e meeting. The maintenance phase has started from RAN1 #107bis-e. In this contribution, we discuss remaining open aspects related to initial access for extending NR up to 71 GHz. The list of outcomes on the initial access AI from the previous meeting could be found in Appendix A [1].

# Discussion on Initial Access Aspects

## Confirmation of Working Assumptions

In RAN1 #107-e, the following working assumptions (WAs) were made regarding ‘controlResourceSetZer’ configuration for SCS combinations {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz:

Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration. Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design.

From Table 13-10B and Table 13-10C of the available draft CR [2], it’s evident that there are enough entries to support the configurations described in the WAs above. Therefore, the WAs can be confirmed.

Proposal 1:

- Confirm WAs on ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz.

There is another WA remaining from RAN1 #107-e:

- Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

However, the WAs on the number of bits for Q value indication above is dependent on feedback from RAN2 which is unavailable. Therefore, the confirmation of this WA is pending by RAN1.

## CORESET#0 and Search Space #0 Configuration

The required RB offsets for CORESET#0 for 120 kHz, 480 kHz and 960 kHz depends on supported channel raster and synchronization raster to be defined in RAN4. Therefore, until RAN4 concludes the channel and synchronization raster entries, it will be difficult for RAN1 to conclude on specific RB offset values. In this contribution, we provide feasible RB offsets based on channelization proposal in [3]. The referenced channelization proposal is summarized in Appendix B.

In general, supporting offset is 0 is preferred. This results in the minimization of resource fragmentation by synchronization signal block (SSB). For example, DCI format 1-0 scrambled with SI-RNTI only supports scheduling of contiguous frequency resources and the PDSCH scrambled with SI-RNTI does not rate-match or puncture around REs potentially used by SSB. Therefore, scheduling and transmission od PDSCH scrambled with SI-RNTI in the same slot as SSB requires SSB to not fragment frequency resources given by the span of CORESET#0, which is equal to the initial active BWP.

The channelization proposal in [3], allows support of 0 RB offset for all cases, with the exception of 48 PRB case for 120 kHz. The existing specification supported 0 RB offset for 24 RB and 14 RB offset for 48 RB, therefore to minimize the changes for 120 kHz, the channelization proposal in [3] picked out the raster entries such that these RB offset values can be re-utilized for FR2-2.

The following table 13-10A, 13-10B, and 13-10C are CORESET#0 configuration table for 120, 480, and 960 kHz cases, respectively.

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 48 | 1 | 14 |
| 2 | 1 | 48 | 2 | 14 |
| 3 | 1 | 96 | 1 | 0 |
| 4 | 1 | 96 | 2 | 0 |
| 5 | 3 | 24 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 6 | 3 | 48 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Table 13-10B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 48 | 1 | 0 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 96 | 2 | 0 |
| 4 | 3 | 24 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 5 | 3 | 48 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Table 13-10C: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {960, 960} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 48 | 1 | 0 |
| 2 | 1 | 48 | 2 | 0 |
| 3 | 1 | 96 | 2 | 0 |
| 4 | 3 | 24 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 5 | 3 | 48 | 2 | -20 if k_ssb =0 -21 if k_ssb >0 |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

If the proposed channelization in [3] is adopted in RAN4, we suggest agreeing to support of 0 RB offset for all multiplexing pattern 1 cases, except 48 RB with 120 kHz, support 14 RB offset for 48 RB with 120kHz, and support -20 or -21 depending on k_ssb value for multiplexing pattern 3 cases.

Proposal 2:

- Support the following RB offset values for CORESET#0 configured in MIB:

  - Multiplexing pattern 1 with 120 kHz SCS and 24 or 96 RBs: 0 RB offset

  - Multiplexing pattern 1 with 120 kHz SCS and 48 RBs: 14 RB offset

  - Multiplexing pattern 1 with 480 kHz SCS and 24, 48, or 96 RBs: 0 RB offset

  - Multiplexing pattern 1 with 960 kHz SCS and 24, 48, or 96 RBs: 0 RB offset

  - Multiplexing pattern 3 with 120, 480, or 960 kHz SCS: -20 or -21 (depending on k_ssb) RB offset

Assuming we will have sufficient entries in the CORESET#0 configuration to support multiplexing pattern 3 for 480 and 960 kHz, we need to agree to search space configuration for 480 and 960 kHz. We suggest to agreeing to Table 13-15A below.

Proposal 3:

- Support the following starting symbol location for multiplexing pattern 3 with 480 or 960 kHz SCS in Table 13-15A

Table 13-15A: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {480, 480} kHz or {960, 960} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k=0,1, … 31) |
| --- | --- | --- |
| 0 | SFNc=SFNSSB,i nc=nSSB,i | 2, 9 in i=k |

# Conclusions

In this contribution, we discussed issues related to initial access for extending NR up to 71 GHz. The following is a summary of the proposals:

Proposal 1:

- Confirm WAs on ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz.

Proposal 2:

- Support the following RB offset values for CORESET#0 configured in MIB:

  - Multiplexing pattern 1 with 120 kHz SCS and 24 or 96 RBs: 0 RB offset

  - Multiplexing pattern 1 with 120 kHz SCS and 48 RBs: 14 RB offset

  - Multiplexing pattern 1 with 480 kHz SCS and 24, 48, or 96 RBs: 0 RB offset

  - Multiplexing pattern 1 with 960 kHz SCS and 24, 48, or 96 RBs: 0 RB offset

  - Multiplexing pattern 3 with 120, 480, or 960 kHz SCS: -20 or -21 (depending on k_ssb) RB offset

Proposal 3:

- Support the following starting symbol location for multiplexing pattern 3 with 480 or 960 kHz SCS in Table 13-15A

Table 13-15A: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {480, 480} kHz or {960, 960} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k=0,1, … 31) |
| --- | --- | --- |
| 0 | SFNc=SFNSSB,i nc=nSSB,i | 2, 9 in i=k |

# References

- 3GPP TSG RAN WG1 Meeting #107-e. RAN1 Chairman’s Notes. e-Meeting, November 11th – 19th, 2021.

- R1-2112931. CR 38.213. Introduction for extending NR operation to 71 GHz. 3GPP TSG RAN WG1 Meeting #107-e. e-Meeting, November 11th – 19th, 2021.

- R4-2201924, “Views on channelization for 52.6 to 71 GHz,” Intel Corporation.

# Appendix A – List of Agreements

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame - Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used R1-2112614 [Draft] LS on initial access for 60 GHz Intel Corporation Final LS endorsed in R1-2112805. Agreement Confirm the following working assumptions: - (From #106-bis-e) Support DBTW for 120 kHz. - (From #106-e) For 120kHz SSB, the number of candidates SSBs in a half frame is 64. Agreement For SCS that support DBTW, UE derives the QCL relation between candidate SSBs by the value of i modNSSBQCL , where i is the candidate SSB index. Conclusion - The bit-width of ssb-PositionsInBurst in SIB1 and ServingCellConfigCommon is kept the same as in Rel-15 (i.e., 16-bits in SIB1 and 64-bits in ServingCellConfigCommon). Agreement If multiplexing pattern 3 for 480 and 960 kHz is supported, the TDRA allocation table C is updated as follows: - Row index 6 (previously reserved) is set to - Dmrs-TypeA-Position: 2,3 - PDSCH mapping type: Type B - K0 : 0 - S = 11 - L = 2 Agreement Finalizing PRACH slot index for 480 and 960 kHz (removal of bracket of previous agreement) - when number of PRACH slots in a reference slot is 1, - nslotRA=7 for 480kHz and nslotRA=15 for 960kHz PRACH - when the number of PRACH slots in a reference slot is 2, - nslotRA=3,7 for 480kHz and nslotRA=7,15 for 960kHz PRACH Agreement Update the Table 8.1-2 in TS38.213 to indicate the Ngap (gap between valid RO and SS/PBCH) for 480 kHz and 960 kHz SCS as follows: - Ngap=8 for 480 kHz - Ngap=16 for 960 kHz; Agreement For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than 𝑁 symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where 𝑁=16 for 𝜇=5, 𝑁=32 for 𝜇=6, and 𝜇 is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214]. Conclusion: ΔDelay as part of gap between last symbol of PDCCH order reception and first symbol of the PRACH transmission for FR2-2 uses the same value as FR2-1 (i.e. single value for FR2). R1-2112451 Summary #1 of email discussion on initial access aspect of NR extension up to 71 GHz Moderator (Intel Corporation) Agreement - For 480 kHz, slot index, n, that contain SSB are: - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31} - For 960 kHz, slot index, n, that contain SSB are: - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31} Agreement For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, - use Table 13-12 in TS38.213 for multiplexing pattern 1, - use Table 13-15 in TS38.213 for multiplexing pattern 3. Agreement For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, parameter X from previous RAN1 agreement is set to: - X = 1.25 for 480 kHz - X = 0.625 for 960 kHz Conclusion: For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH. Agreement For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=16 for μ=5 and N=32 for μ=6, and μ is the SCS configuration for the active UL BWP. Agreement For 480 and 960 kHz, supported DBTW lengths are: - {1.25, 1, 0.75, 0.5, 0.25, 0.125, X} ms, where X = 0.0625 if Q=8 is supported and X is removed if Q=8 is not supported. Agreement SSB-PositionQCL-Relation IE to indicate QCL relationship between SSB positions for FR2-2 are same set of values supported for NSSBQCL in MIB. Agreement For operation with shared spectrum access, for SS/PBCH block and CORESET#0 multiplexing pattern 3, a UE monitors PDCCH in the Type0-PDCCH CSS set over slots that include Type0-PDCCH monitoring occasions associated with SS/PBCH blocks that are quasi co-located with the SS/PBCH block that provides a CORESET for Type0-PDCCH CSS set. Agreement For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, parameter Y from previous RAN1 agreement is Y = NsymbCORESET. Agreement - For 480kHz and 960kHz PRACH, reuse the RA-RNTI and MSGB-RNTI formula as FR2 and express the slot indexes t_id based on 120kHz SCS: - RA-RNTI =1+s_id+14×t_id+14×80×f_id +14×80×8×ul_carrier_id - MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2 - where the subcarrier spacing to determine t_id is based on the value of µ specified in clause 5.3.2 in TS 38.211 [8] for µ = {0, 1, 2, 3} - for µ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80). - Note: As per previous RAN1 agreement, there is only one 480 or 960 kHz PRACH slot in a 120kHz slot, such that RA-RNTI and MSGB-RNTI does not result in ID collision. - Send LS to RAN2 on the updates on RA-RNTI and MSGB-RNTI. R1-2112734 [Draft] LS on RA-RNTI and MSGB-RNTI for 480 and 960 kHz Intel Corporation Final LS endorsed in R1-2112832 (with removal of “first” in text referring to the captured agreement) Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship. Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets), if additional entries are left, support multiplex pattern 3 with 24 PRB and 2 symbol duration, and multiplexing pattern 3 with 48 PRB and 2 symbol duration. Working assumption For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - After supporting entries for multiplexing pattern 1 for the agreed pairs of (NRBCORESET, NsymbCORESET) ={(24, 2), (48, 1), (48,2)} (with required RB offsets) and multiplex pattern 3 with 24 and 48 PRB and 2 symbol duration (with required RB offsets), if additional entries are left, support multiplexing pattern 1 with 96 PRB and 2 symbol duration - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design. Agreement - If NSSBQCL is indicated, the same interpretation of ssb-PositionsInBurst in SIB1 or ServingCellConfigCommon as in Rel-16 is supported, i.e.: - A bit set to 1 at position k∈{1..64} indicates SS/PBCH block index k-1 - The UE assumes that a bit at position k > NSSBQCL is set to 0 - For ssb-PositionsInBurst in SIB1, the UE assumes that a bit at groupPresence corresponding to a SS/PBCH block index ≥ NSSBQCL is set to 0 - Note: for ssb-PositionsInBurst in SIB1, position k corresponds to the SS/PBCH block index indicated by a bit in inOneGroup and a bit in groupPresence - In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommonSIB, - for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst: - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 may be transmitted; - if MSB k of inOneGroup or MSB m of groupPresense is set to 0, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 is not transmitted; - In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommon, - ssb-PositionsInBurst bits correspond to supported ‘SSB indices’, - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to indicated bit(s) may be transmitted; - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to not indicated bit(s) are not transmitted - Note to spec editor: The above three bullets maintain the same behavior as Rel-16 NR-U Agreement Update the Table 6.3.3.2-1 in TS 38.211 as follows: - Table 6.3.3.2-1: Supported combinations of ΔfRA and , and the corresponding value of . ΔfRA for PRACH; for PUSCH; , allocation expressed in number of RBs for PUSCH. ...; ...; ...; ...; .... 139; 120; 120; 12; 2. 139; 120; 480; 3; 2 1. 139; 120; 960; 2; 2 23. 139; 480; 120; 48; 2. 139; 480; 480; 12; 2. 139; 480; 960; 6; 2. 139; 960; 120; 96; 2. 139; 960; 480; 24; 2. 139; 960; 960; 12; 2. 571; 120; 120; 48; 2. 571; 120; 480; 12; 21. 571; 120; 960; 67; 247. 571; 480; 120; 192; 2. 571; 480; 480; 48; 2. 571; 480; 960; 24; 2. 1151; 120; 120; 96 97; 1 6. 1151; 120; 480; 24 25; 1 24 23. 1151; 120; 960; 12 13; 1 48 45.

# Appendix B – Channel Raster for n263 [3]

First non-overlapping 100 MHz channel bandwidth (CBW) were defined for channelization of 100 MHz. Next, 400 MHz, 800 MHz, 1600 MHz, and 2000 MHz CBWs were selected by choosing the center frequency of contiguous 4, 8, 16, and 20 channels of 100 MHz CBW, respectively. Not all possible 400/800/1600/2000 MHz locations were not chosen. In general, 400, 800, 1600 MHz were selected among the possible position such that the channels do not overlap. However, in order to maximize spectrum utilization for various regulatory regions, some overlapping channels were selected. Lastly, only non-overlapping 2000 MHz CBW were selected among the possible positions.

In order to generally achieve RB offset 0 for majority of the cases, synchronization raster was chosen such that SSB are selected closest to the center of each 100 MHz channel bandwidth (CBW). These selected synchronization raster entries are chosen as valid entries for 120 kHz. From the subset of synchronization raster entries (selected for each 100 MHz CBW), the first raster instance among valid SSB candidate positions within the 400 MHz CBW were selected for valid synchronization raster for 480 kHz. This results in raster entries for 480 kHz to be a subset of the raster entries for 120 kHz. An illustration of the synchronization raster selection process is shown in figure below.

Figure 1. Illustration of the synchronization raster selection process

The number of sync raster entries required as shown in Table 1

Table 1. Proposed number of sync raster entries in band n263

| SCS | CBW |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 kHz | 138 | 38 | N/A | N/A | N/A |
| 480 kHz | N/A |  | 31* | 11 | N/A |
| 960 kHz | N/A |  |  |  | 6 |

*Note: 2 raster entries selected for 800 MHz are not a subset of the entries for 400 MHz. This trade off ensures minimizing the required RB offsets.

As shown in the table, this results in a total UE SSB search complexity of 138 + 38 + 2 = 178 for initial access. This is significantly lesser than the target value of 665 specified in the WID (extract pasted below).

It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

Table 2 list the full list of channelization entries for 120, 480, and 960 kHz, and corresponding valid synchronization entry. It should be noted that for some wider channels, other synchronization entries could be used, and the table does not exhaustively list all valid synchronization entries possible for each channel.

Table 2. Proposed channelization for band n263 with 86% spectrum utilization

| Data SCS [kHz] | BW [MHz] | NR Chn Raster [kHz] | NR-ARFCN | SS Raster [kHz] | GSCN |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 57095040 | 2564083 | 57099360 | 24157 |
| 120 | 100 | 57195840 | 2565763 | 57203040 | 24163 |
| 120 | 100 | 57296640 | 2567443 | 57289440 | 24168 |
| 120 | 100 | 57397440 | 2569123 | 57393120 | 24174 |
| 120 | 100 | 57498240 | 2570803 | 57496800 | 24180 |
| 120 | 100 | 57599040 | 2572483 | 57600480 | 24186 |
| 120 | 100 | 57699840 | 2574163 | 57704160 | 24192 |
| 120 | 100 | 57800640 | 2575843 | 57807840 | 24198 |
| 120 | 100 | 57901440 | 2577523 | 57894240 | 24203 |
| 120 | 100 | 58002240 | 2579203 | 57997920 | 24209 |
| 120 | 100 | 58103040 | 2580883 | 58101600 | 24215 |
| 120 | 100 | 58203840 | 2582563 | 58205280 | 24221 |
| 120 | 100 | 58304640 | 2584243 | 58308960 | 24227 |
| 120 | 100 | 58405440 | 2585923 | 58412640 | 24233 |
| 120 | 100 | 58506240 | 2587603 | 58499040 | 24238 |
| 120 | 100 | 58607040 | 2589283 | 58602720 | 24244 |
| 120 | 100 | 58707840 | 2590963 | 58706400 | 24250 |
| 120 | 100 | 58808640 | 2592643 | 58810080 | 24256 |
| 120 | 100 | 58909440 | 2594323 | 58913760 | 24262 |
| 120 | 100 | 59010240 | 2596003 | 59017440 | 24268 |
| 120 | 100 | 59111040 | 2597683 | 59103840 | 24273 |
| 120 | 100 | 59211840 | 2599363 | 59207520 | 24279 |
| 120 | 100 | 59312640 | 2601043 | 59311200 | 24285 |
| 120 | 100 | 59413440 | 2602723 | 59414880 | 24291 |
| 120 | 100 | 59514240 | 2604403 | 59518560 | 24297 |
| 120 | 100 | 59615040 | 2606083 | 59622240 | 24303 |
| 120 | 100 | 59715840 | 2607763 | 59708640 | 24308 |
| 120 | 100 | 59816640 | 2609443 | 59812320 | 24314 |
| 120 | 100 | 59917440 | 2611123 | 59916000 | 24320 |
| 120 | 100 | 60018240 | 2612803 | 60019680 | 24326 |
| 120 | 100 | 60119040 | 2614483 | 60123360 | 24332 |
| 120 | 100 | 60219840 | 2616163 | 60227040 | 24338 |
| 120 | 100 | 60320640 | 2617843 | 60313440 | 24343 |
| 120 | 100 | 60421440 | 2619523 | 60417120 | 24349 |
| 120 | 100 | 60522240 | 2621203 | 60520800 | 24355 |
| 120 | 100 | 60623040 | 2622883 | 60624480 | 24361 |
| 120 | 100 | 60723840 | 2624563 | 60728160 | 24367 |
| 120 | 100 | 60824640 | 2626243 | 60831840 | 24373 |
| 120 | 100 | 60925440 | 2627923 | 60918240 | 24378 |
| 120 | 100 | 61026240 | 2629603 | 61021920 | 24384 |
| 120 | 100 | 61127040 | 2631283 | 61125600 | 24390 |
| 120 | 100 | 61227840 | 2632963 | 61229280 | 24396 |
| 120 | 100 | 61328640 | 2634643 | 61332960 | 24402 |
| 120 | 100 | 61429440 | 2636323 | 61436640 | 24408 |
| 120 | 100 | 61530240 | 2638003 | 61523040 | 24413 |
| 120 | 100 | 61631040 | 2639683 | 61626720 | 24419 |
| 120 | 100 | 61731840 | 2641363 | 61730400 | 24425 |
| 120 | 100 | 61832640 | 2643043 | 61834080 | 24431 |
| 120 | 100 | 61933440 | 2644723 | 61937760 | 24437 |
| 120 | 100 | 62034240 | 2646403 | 62041440 | 24443 |
| 120 | 100 | 62135040 | 2648083 | 62127840 | 24448 |
| 120 | 100 | 62235840 | 2649763 | 62231520 | 24454 |
| 120 | 100 | 62336640 | 2651443 | 62335200 | 24460 |
| 120 | 100 | 62437440 | 2653123 | 62438880 | 24466 |
| 120 | 100 | 62538240 | 2654803 | 62542560 | 24472 |
| 120 | 100 | 62639040 | 2656483 | 62646240 | 24478 |
| 120 | 100 | 62739840 | 2658163 | 62732640 | 24483 |
| 120 | 100 | 62840640 | 2659843 | 62836320 | 24489 |
| 120 | 100 | 62941440 | 2661523 | 62940000 | 24495 |
| 120 | 100 | 63042240 | 2663203 | 63043680 | 24501 |
| 120 | 100 | 63143040 | 2664883 | 63147360 | 24507 |
| 120 | 100 | 63243840 | 2666563 | 63251040 | 24513 |
| 120 | 100 | 63344640 | 2668243 | 63337440 | 24518 |
| 120 | 100 | 63445440 | 2669923 | 63441120 | 24524 |
| 120 | 100 | 63546240 | 2671603 | 63544800 | 24530 |
| 120 | 100 | 63647040 | 2673283 | 63648480 | 24536 |
| 120 | 100 | 63747840 | 2674963 | 63752160 | 24542 |
| 120 | 100 | 63848640 | 2676643 | 63855840 | 24548 |
| 120 | 100 | 63949440 | 2678323 | 63942240 | 24553 |
| 120 | 100 | 64050240 | 2680003 | 64045920 | 24559 |
| 120 | 100 | 64151040 | 2681683 | 64149600 | 24565 |
| 120 | 100 | 64251840 | 2683363 | 64253280 | 24571 |
| 120 | 100 | 64352640 | 2685043 | 64356960 | 24577 |
| 120 | 100 | 64453440 | 2686723 | 64460640 | 24583 |
| 120 | 100 | 64554240 | 2688403 | 64547040 | 24588 |
| 120 | 100 | 64655040 | 2690083 | 64650720 | 24594 |
| 120 | 100 | 64755840 | 2691763 | 64754400 | 24600 |
| 120 | 100 | 64856640 | 2693443 | 64858080 | 24606 |
| 120 | 100 | 64957440 | 2695123 | 64961760 | 24612 |
| 120 | 100 | 65058240 | 2696803 | 65065440 | 24618 |
| 120 | 100 | 65159040 | 2698483 | 65151840 | 24623 |
| 120 | 100 | 65259840 | 2700163 | 65255520 | 24629 |
| 120 | 100 | 65360640 | 2701843 | 65359200 | 24635 |
| 120 | 100 | 65461440 | 2703523 | 65462880 | 24641 |
| 120 | 100 | 65562240 | 2705203 | 65566560 | 24647 |
| 120 | 100 | 65663040 | 2706883 | 65670240 | 24653 |
| 120 | 100 | 65763840 | 2708563 | 65756640 | 24658 |
| 120 | 100 | 65864640 | 2710243 | 65860320 | 24664 |
| 120 | 100 | 65965440 | 2711923 | 65964000 | 24670 |
| 120 | 100 | 66066240 | 2713603 | 66067680 | 24676 |
| 120 | 100 | 66167040 | 2715283 | 66171360 | 24682 |
| 120 | 100 | 66267840 | 2716963 | 66275040 | 24688 |
| 120 | 100 | 66368640 | 2718643 | 66361440 | 24693 |
| 120 | 100 | 66469440 | 2720323 | 66465120 | 24699 |
| 120 | 100 | 66570240 | 2722003 | 66568800 | 24705 |
| 120 | 100 | 66671040 | 2723683 | 66672480 | 24711 |
| 120 | 100 | 66771840 | 2725363 | 66776160 | 24717 |
| 120 | 100 | 66872640 | 2727043 | 66879840 | 24723 |
| 120 | 100 | 66973440 | 2728723 | 66966240 | 24728 |
| 120 | 100 | 67074240 | 2730403 | 67069920 | 24734 |
| 120 | 100 | 67175040 | 2732083 | 67173600 | 24740 |
| 120 | 100 | 67275840 | 2733763 | 67277280 | 24746 |
| 120 | 100 | 67376640 | 2735443 | 67380960 | 24752 |
| 120 | 100 | 67477440 | 2737123 | 67484640 | 24758 |
| 120 | 100 | 67578240 | 2738803 | 67571040 | 24763 |
| 120 | 100 | 67679040 | 2740483 | 67674720 | 24769 |
| 120 | 100 | 67779840 | 2742163 | 67778400 | 24775 |
| 120 | 100 | 67880640 | 2743843 | 67882080 | 24781 |
| 120 | 100 | 67981440 | 2745523 | 67985760 | 24787 |
| 120 | 100 | 68082240 | 2747203 | 68089440 | 24793 |
| 120 | 100 | 68183040 | 2748883 | 68175840 | 24798 |
| 120 | 100 | 68283840 | 2750563 | 68279520 | 24804 |
| 120 | 100 | 68384640 | 2752243 | 68383200 | 24810 |
| 120 | 100 | 68485440 | 2753923 | 68486880 | 24816 |
| 120 | 100 | 68586240 | 2755603 | 68590560 | 24822 |
| 120 | 100 | 68687040 | 2757283 | 68694240 | 24828 |
| 120 | 100 | 68787840 | 2758963 | 68780640 | 24833 |
| 120 | 100 | 68888640 | 2760643 | 68884320 | 24839 |
| 120 | 100 | 68989440 | 2762323 | 68988000 | 24845 |
| 120 | 100 | 69090240 | 2764003 | 69091680 | 24851 |
| 120 | 100 | 69191040 | 2765683 | 69195360 | 24857 |
| 120 | 100 | 69291840 | 2767363 | 69299040 | 24863 |
| 120 | 100 | 69392640 | 2769043 | 69385440 | 24868 |
| 120 | 100 | 69493440 | 2770723 | 69489120 | 24874 |
| 120 | 100 | 69594240 | 2772403 | 69592800 | 24880 |
| 120 | 100 | 69695040 | 2774083 | 69696480 | 24886 |
| 120 | 100 | 69795840 | 2775763 | 69800160 | 24892 |
| 120 | 100 | 69896640 | 2777443 | 69903840 | 24898 |
| 120 | 100 | 69997440 | 2779123 | 69990240 | 24903 |
| 120 | 100 | 70098240 | 2780803 | 70093920 | 24909 |
| 120 | 100 | 70199040 | 2782483 | 70197600 | 24915 |
| 120 | 100 | 70299840 | 2784163 | 70301280 | 24921 |
| 120 | 100 | 70400640 | 2785843 | 70404960 | 24927 |
| 120 | 100 | 70501440 | 2787523 | 70508640 | 24933 |
| 120 | 100 | 70602240 | 2789203 | 70595040 | 24938 |
| 120 | 100 | 70703040 | 2790883 | 70698720 | 24944 |
| 120 | 100 | 70803840 | 2792563 | 70802400 | 24950 |
| 120 | 100 | 70904640 | 2794243 | 70906080 | 24956 |
| 120 | 400 | 57548640 | 2571643 | 57393120 | 24174 |
| 120 | 400 | 57951840 | 2578363 | 57807840 | 24198 |
| 120 | 400 | 58355040 | 2585083 | 58205280 | 24221 |
| 120 | 400 | 58758240 | 2591803 | 58602720 | 24244 |
| 120 | 400 | 59161440 | 2598523 | 59017440 | 24268 |
| 120 | 400 | 59665440 | 2606923 | 59518560 | 24297 |
| 120 | 400 | 60068640 | 2613643 | 59916000 | 24320 |
| 120 | 400 | 60471840 | 2620363 | 60313440 | 24343 |
| 120 | 400 | 60875040 | 2627083 | 60728160 | 24367 |
| 120 | 400 | 61278240 | 2633803 | 61125600 | 24390 |
| 120 | 400 | 61782240 | 2642203 | 61626720 | 24419 |
| 120 | 400 | 62185440 | 2648923 | 62041440 | 24443 |
| 120 | 400 | 62588640 | 2655643 | 62438880 | 24466 |
| 120 | 400 | 62991840 | 2662363 | 62836320 | 24489 |
| 120 | 400 | 63395040 | 2669083 | 63251040 | 24513 |
| 120 | 400 | 63999840 | 2679163 | 63855840 | 24548 |
| 120 | 400 | 64403040 | 2685883 | 64253280 | 24571 |
| 120 | 400 | 64806240 | 2692603 | 64650720 | 24594 |
| 120 | 400 | 65209440 | 2699323 | 65065440 | 24618 |
| 120 | 400 | 65612640 | 2706043 | 65462880 | 24641 |
| 120 | 400 | 66116640 | 2714443 | 65964000 | 24670 |
| 120 | 400 | 66519840 | 2721163 | 66361440 | 24693 |
| 120 | 400 | 66923040 | 2727883 | 66776160 | 24717 |
| 120 | 400 | 67326240 | 2734603 | 67173600 | 24740 |
| 120 | 400 | 67729440 | 2741323 | 67571040 | 24763 |
| 120 | 400 | 68334240 | 2751403 | 68175840 | 24798 |
| 120 | 400 | 68737440 | 2758123 | 68590560 | 24822 |
| 120 | 400 | 69140640 | 2764843 | 68988000 | 24845 |
| 120 | 400 | 69543840 | 2771563 | 69385440 | 24868 |
| 120 | 400 | 69947040 | 2778283 | 69800160 | 24892 |
| 120 | 400 | 70451040 | 2786683 | 70301280 | 24921 |
| 120 | 400 | 59262240 | 2600203 | 59103840 | 24273 |
| 120 | 400 | 61681440 | 2640523 | 61523040 | 24413 |
| 120 | 400 | 62084640 | 2647243 | 61937760 | 24437 |
| 120 | 400 | 62487840 | 2653963 | 62335200 | 24460 |
| 120 | 400 | 62891040 | 2660683 | 62732640 | 24483 |
| 120 | 400 | 63294240 | 2667403 | 63147360 | 24507 |
| 120 | 400 | 63697440 | 2674123 | 63544800 | 24530 |
| 480 | 400 | 57548640 | 2571643 | 57496800 | 24180 |
| 480 | 400 | 57951840 | 2578363 | 57894240 | 24203 |
| 480 | 400 | 58355040 | 2585083 | 58308960 | 24227 |
| 480 | 400 | 58758240 | 2591803 | 58706400 | 24250 |
| 480 | 400 | 59161440 | 2598523 | 59103840 | 24273 |
| 480 | 400 | 59665440 | 2606923 | 59622240 | 24303 |
| 480 | 400 | 60068640 | 2613643 | 60019680 | 24326 |
| 480 | 400 | 60471840 | 2620363 | 60417120 | 24349 |
| 480 | 400 | 60875040 | 2627083 | 60831840 | 24373 |
| 480 | 400 | 61278240 | 2633803 | 61229280 | 24396 |
| 480 | 400 | 61782240 | 2642203 | 61730400 | 24425 |
| 480 | 400 | 62185440 | 2648923 | 62127840 | 24448 |
| 480 | 400 | 62588640 | 2655643 | 62542560 | 24472 |
| 480 | 400 | 62991840 | 2662363 | 62940000 | 24495 |
| 480 | 400 | 63395040 | 2669083 | 63337440 | 24518 |
| 480 | 400 | 63999840 | 2679163 | 63942240 | 24553 |
| 480 | 400 | 64403040 | 2685883 | 64356960 | 24577 |
| 480 | 400 | 64806240 | 2692603 | 64754400 | 24600 |
| 480 | 400 | 65209440 | 2699323 | 65151840 | 24623 |
| 480 | 400 | 65612640 | 2706043 | 65566560 | 24647 |
| 480 | 400 | 66116640 | 2714443 | 66067680 | 24676 |
| 480 | 400 | 66519840 | 2721163 | 66465120 | 24699 |
| 480 | 400 | 66923040 | 2727883 | 66879840 | 24723 |
| 480 | 400 | 67326240 | 2734603 | 67277280 | 24746 |
| 480 | 400 | 67729440 | 2741323 | 67674720 | 24769 |
| 480 | 400 | 68334240 | 2751403 | 68279520 | 24804 |
| 480 | 400 | 68737440 | 2758123 | 68694240 | 24828 |
| 480 | 400 | 69140640 | 2764843 | 69091680 | 24851 |
| 480 | 400 | 69543840 | 2771563 | 69489120 | 24874 |
| 480 | 400 | 69947040 | 2778283 | 69903840 | 24898 |
| 480 | 400 | 70451040 | 2786683 | 70404960 | 24927 |
| 480 | 400 | 59262240 | 2600203 | 59207520 | 24279 |
| 480 | 400 | 61681440 | 2640523 | 61626720 | 24419 |
| 480 | 400 | 62084640 | 2647243 | 62041440 | 24443 |
| 480 | 400 | 62487840 | 2653963 | 62438880 | 24466 |
| 480 | 400 | 62891040 | 2660683 | 62836320 | 24489 |
| 480 | 400 | 63294240 | 2667403 | 63251040 | 24513 |
| 480 | 400 | 63697440 | 2674123 | 63648480 | 24536 |
| 480 | 800 | 57750240 | 2575003 | 57496800 | 24180 |
| 480 | 800 | 58556640 | 2588443 | 58308960 | 24227 |
| 480 | 800 | 59463840 | 2603563 | 59207520 | 24279 |
| 480 | 800 | 60270240 | 2617003 | 60019680 | 24326 |
| 480 | 800 | 61076640 | 2630443 | 60831840 | 24373 |
| 480 | 800 | 61983840 | 2645563 | 61730400 | 24425 |
| 480 | 800 | 62790240 | 2659003 | 62542560 | 24472 |
| 480 | 800 | 63596640 | 2672443 | 63337440 | 24518 |
| 480 | 800 | 64604640 | 2689243 | 64356960 | 24577 |
| 480 | 800 | 65411040 | 2702683 | 65151840 | 24623 |
| 480 | 800 | 66318240 | 2717803 | 66067680 | 24676 |
| 480 | 800 | 67124640 | 2731243 | 66879840 | 24723 |
| 480 | 800 | 67931040 | 2744683 | 67674720 | 24769 |
| 480 | 800 | 68939040 | 2761483 | 68694240 | 24828 |
| 480 | 800 | 69745440 | 2774923 | 69489120 | 24874 |
| 480 | 800 | 70551840 | 2788363 | 70404960 | 24927 |
| 480 | 800 | 58153440 | 2581723 | 57894240 | 24203 |
| 480 | 800 | 58959840 | 2595163 | 58706400 | 24250 |
| 480 | 800 | 59867040 | 2610283 | 59622240 | 24303 |
| 480 | 800 | 60673440 | 2623723 | 60417120 | 24349 |
| 480 | 800 | 61580640 | 2638843 | 61626720 | 24419 |
| 480 | 800 | 62387040 | 2652283 | 62127840 | 24448 |
| 480 | 800 | 63193440 | 2665723 | 62940000 | 24495 |
| 480 | 800 | 64201440 | 2682523 | 63942240 | 24553 |
| 480 | 800 | 65007840 | 2695963 | 64754400 | 24600 |
| 480 | 800 | 65915040 | 2711083 | 66067680 | 24676 |
| 480 | 800 | 66721440 | 2724523 | 66465120 | 24699 |
| 480 | 800 | 67527840 | 2737963 | 67277280 | 24746 |
| 480 | 800 | 68535840 | 2754763 | 68279520 | 24804 |
| 480 | 800 | 69342240 | 2768203 | 69091680 | 24851 |
| 480 | 800 | 70148640 | 2781643 | 69903840 | 24898 |
| 480 | 1600 | 58153440 | 2581723 | 57894240 | 24203 |
| 480 | 1600 | 60673440 | 2623723 | 60417120 | 24349 |
| 480 | 1600 | 62387040 | 2652283 | 62041440 | 24443 |
| 480 | 1600 | 65007840 | 2695963 | 64754400 | 24600 |
| 480 | 1600 | 66721440 | 2724523 | 664651206 | 24699 |
| 480 | 1600 | 69342240 | 2768203 | 69091680 | 24851 |
| 480 | 1600 | 59867040 | 2610283 | 59622240 | 24303 |
| 480 | 1600 | 61479840 | 2637163 | 61229280 | 24396 |
| 480 | 1600 | 63092640 | 2664043 | 62542560 | 24472 |
| 480 | 1600 | 68435040 | 2753083 | 68279520 | 24804 |
| 480 | 1600 | 70148640 | 2781643 | 69903840 | 24898 |
| 960 | 400 | 57548640 | 2571643 | 57496800 | 24180 |
| 960 | 400 | 57951840 | 2578363 | 57894240 | 24203 |
| 960 | 400 | 58355040 | 2585083 | 58308960 | 24227 |
| 960 | 400 | 58758240 | 2591803 | 58706400 | 24250 |
| 960 | 400 | 59161440 | 2598523 | 59103840 | 24273 |
| 960 | 400 | 59665440 | 2606923 | 59622240 | 24303 |
| 960 | 400 | 60068640 | 2613643 | 60019680 | 24326 |
| 960 | 400 | 60471840 | 2620363 | 60417120 | 24349 |
| 960 | 400 | 60875040 | 2627083 | 60831840 | 24373 |
| 960 | 400 | 61278240 | 2633803 | 61229280 | 24396 |
| 960 | 400 | 61782240 | 2642203 | 61730400 | 24425 |
| 960 | 400 | 62185440 | 2648923 | 62127840 | 24448 |
| 960 | 400 | 62588640 | 2655643 | 62542560 | 24472 |
| 960 | 400 | 62991840 | 2662363 | 62940000 | 24495 |
| 960 | 400 | 63395040 | 2669083 | 63337440 | 24518 |
| 960 | 400 | 63999840 | 2679163 | 63942240 | 24553 |
| 960 | 400 | 64403040 | 2685883 | 64356960 | 24577 |
| 960 | 400 | 64806240 | 2692603 | 64754400 | 24600 |
| 960 | 400 | 65209440 | 2699323 | 65151840 | 24623 |
| 960 | 400 | 65612640 | 2706043 | 65566560 | 24647 |
| 960 | 400 | 66116640 | 2714443 | 66067680 | 24676 |
| 960 | 400 | 66519840 | 2721163 | 66465120 | 24699 |
| 960 | 400 | 66923040 | 2727883 | 66879840 | 24723 |
| 960 | 400 | 67326240 | 2734603 | 67277280 | 24746 |
| 960 | 400 | 67729440 | 2741323 | 67674720 | 24769 |
| 960 | 400 | 68334240 | 2751403 | 68279520 | 24804 |
| 960 | 400 | 68737440 | 2758123 | 68694240 | 24828 |
| 960 | 400 | 69140640 | 2764843 | 69091680 | 24851 |
| 960 | 400 | 69543840 | 2771563 | 69489120 | 24874 |
| 960 | 400 | 69947040 | 2778283 | 69903840 | 24898 |
| 960 | 400 | 70451040 | 2786683 | 70404960 | 24927 |
| 960 | 400 | 59262240 | 2600203 | 59207520 | 24279 |
| 960 | 400 | 61681440 | 2640523 | 61626720 | 24419 |
| 960 | 400 | 62084640 | 2647243 | 62041440 | 24443 |
| 960 | 400 | 62487840 | 2653963 | 62438880 | 24466 |
| 960 | 400 | 62891040 | 2660683 | 62836320 | 24489 |
| 960 | 400 | 63294240 | 2667403 | 63251040 | 24513 |
| 960 | 400 | 63697440 | 2674123 | 63648480 | 24536 |
| 960 | 800 | 57750240 | 2575003 | 57600480 | 24186 |
| 960 | 800 | 58556640 | 2588443 | 58412640 | 24233 |
| 960 | 800 | 59463840 | 2603563 | 59311200 | 24285 |
| 960 | 800 | 60270240 | 2617003 | 60123360 | 24332 |
| 960 | 800 | 61076640 | 2630443 | 60918240 | 24378 |
| 960 | 800 | 61983840 | 2645563 | 61834080 | 24431 |
| 960 | 800 | 62790240 | 2659003 | 62646240 | 24478 |
| 960 | 800 | 63596640 | 2672443 | 63441120 | 24524 |
| 960 | 800 | 64604640 | 2689243 | 64460640 | 24583 |
| 960 | 800 | 65411040 | 2702683 | 65255520 | 24629 |
| 960 | 800 | 66318240 | 2717803 | 66171360 | 24682 |
| 960 | 800 | 67124640 | 2731243 | 66966240 | 24728 |
| 960 | 800 | 67931040 | 2744683 | 67778400 | 24775 |
| 960 | 800 | 68939040 | 2761483 | 68780640 | 24833 |
| 960 | 800 | 69745440 | 2774923 | 69592800 | 24880 |
| 960 | 800 | 70551840 | 2788363 | 70404960 | 24927 |
| 960 | 800 | 58153440 | 2581723 | 57997920 | 24209 |
| 960 | 800 | 58959840 | 2595163 | 58810080 | 24256 |
| 960 | 800 | 59867040 | 2610283 | 59708640 | 24308 |
| 960 | 800 | 60673440 | 2623723 | 60520800 | 24355 |
| 960 | 800 | 61580640 | 2638843 | 61436640 | 24408 |
| 960 | 800 | 62387040 | 2652283 | 62231520 | 24454 |
| 960 | 800 | 63193440 | 2665723 | 63043680 | 24501 |
| 960 | 800 | 64201440 | 2682523 | 64045920 | 24559 |
| 960 | 800 | 65007840 | 2695963 | 64858080 | 24606 |
| 960 | 800 | 65915040 | 2711083 | 65756640 | 24658 |
| 960 | 800 | 66721440 | 2724523 | 66568800 | 24705 |
| 960 | 800 | 67527840 | 2737963 | 67380960 | 24752 |
| 960 | 800 | 68535840 | 2754763 | 68383200 | 24810 |
| 960 | 800 | 69342240 | 2768203 | 69195360 | 24857 |
| 960 | 800 | 70148640 | 2781643 | 69990240 | 24903 |
| 960 | 1600 | 58153440 | 2581723 | 57600480 | 24186 |
| 960 | 1600 | 60673440 | 2623723 | 60123360 | 24332 |
| 960 | 1600 | 62387040 | 2652283 | 61834080 | 24431 |
| 960 | 1600 | 65007840 | 2695963 | 64460640 | 24583 |
| 960 | 1600 | 66721440 | 2724523 | 66171360 | 24682 |
| 960 | 1600 | 69342240 | 2768203 | 68780640 | 24833 |
| 960 | 1600 | 59867040 | 2610283 | 59311200 | 24285 |
| 960 | 1600 | 61479840 | 2637163 | 60918240 | 24378 |
| 960 | 1600 | 63092640 | 2664043 | 62542560 | 24472 |
| 960 | 1600 | 68435040 | 2753083 | 67882080 | 24781 |
| 960 | 1600 | 70148640 | 2781643 | 69592800 | 24880 |
| 960 | 2000 | 58355040 | 2585083 | 57704160 | 24192 |
| 960 | 2000 | 60471840 | 2620363 | 59812320 | 24314 |
| 960 | 2000 | 62588640 | 2655643 | 61937760 | 24437 |
| 960 | 2000 | 64806240 | 2692603 | 64149600 | 24565 |
| 960 | 2000 | 66923040 | 2727883 | 66275040 | 24688 |
| 960 | 2000 | 69140640 | 2764843 | 68486880 | 24816 |