3GPP TSG RAN WG1 #107bis-e		R1-2200418

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.7.1.2

Source:	Apple Inc.

Title:	Remaining issues on the indication of TRS configurations for idle/inactive UEs

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss the remaining details for the indication of TRS/CSI-RS to idle/inactive-mode UEs [1], including the configuration signaling, the availability signaling, and some overhead reduction enhancements. (Past agreements are included in Appendix B.)

# TRS Configurations

For the TRS configuration signaling, the following has been agreed:

Agreement

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:

        a TRS resource set can be configured to include

o   a set of TRS resources up to two consecutive slots,

  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol.

o   at least common configuration parameters:

  a QCL reference

  firstOFDMSymbolInTimeDomain,

  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’

  FFS

        scramblingID,

        a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported

        Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

What remains open is how to handle scramblingID and how to indicate different CSI-RS symbols in a TRS resource set. Here are the related proposals discussed in RAN1#107-e:

Proposal 4-1 (v9)

Support a configuration parameter for the number of, X, TRS resources for a TRS resource set.

- applicable values for X: {2, 4}

v1:

One or more scrambling IDs is configured for TRS resources within a TRS resource set.

- If only a single TRS resource is configured with scrambling ID, it applies to all the TRS resources.

- Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where the scrambling ID is configured per TRS resource.

v2:

One or more scrambling IDs is configured for a TRS resource set.

-        If a single scrambling ID is configured, it applies to all the TRS resources.

-        Otherwise, the number of scrambling IDs is equal to the number of TRS resources.

It is straightforward and very beneficial to introduce a new parameter for the number of slots, to avoid duplicating the common parameters for different CSI-RS symbols within a TRS resource set.

Proposal 1: Support a configuration parameter for the number of TRS resources X for a TRS resource set.

- applicable values for X: {2, 4}

For scramblingID, it seemed generally agreeable to support the configuration per TRS resource set (to save overhead when the scrambling ID is the same for all the TRS resources) and per TRS resource (when the scrambling ID is different for different TRS resource). The difference between the two versions (v1 and v2) of the proposal is relatively minor. We prefer v2 because v1 suggests the signaling structure needs to have the concept of TRS resource when it says “If only a single TRS resource is configured with scrambling ID” and “the scrambling ID is configured per TRS resource”. However, so far we don’t have any other TRS resource-specific parameters and there may not be a need to introduce a signaling structure per TRS resource. Therefore, it leaves more flexibility to RAN2 signaling design if we simply say the number of scrambling ID being configured for a TRS resource set, instead of how many TRS resources are configured with scrambling ID.

Proposal 2: One or more scrambling IDs is configured for a TRS resource set.

- If a single scrambling ID is configured, it applies to all the TRS resources.

- Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource.

The field sizes for the TRS resource set parameters are listed in Table 1. Assuming the more optimistic case with one scramblingID per TRS resource set, each TRS resource set takes 53 bits (ignoring any additional overhead in the signaling), and 64 TRS resource sets (one per beam for 64 beams) takes 53 * 64 = 3392 bits. However, the current limit for the SI message size is 2976 bits, as defined in TS 38.214: “The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.” Moreover, SIB segmentation is not supported at the moment. RAN2 is considering the possibility of supporting SIB segmentation.

If we want to further reduce the signaling overhead and/or fit it into a SIB without segmentation, more optimization can be done. For example, it may make sense to support the configuration of some common parameters for some or all TRS resource set configurations. One good candidate is the RB allocation (starting RB and nrofRBs), which is more likely to be the same across the TRS resource sets and it also brings significant overhead saving given its size.

Proposal 3: Further consider introducing common parameters (e.g., starting RB and nrofRBs) for multiple/all TRS resource sets to reduce the signaling overhead.

Table 1 TRS configuration parameters

| TRS configuration parameters | Value Range | Field Size (bits) |
| --- | --- | --- |
| Frequency domain allocation | {0, 1, 2, 3} | 2 |
| firstOFDMSymbolInTimeDomain | 0 to 9 | 4 |
| startingRB | Can reuse the existing range: 0 to 274 | 9 |
| nrofRBs | Can reuse the existing range: 24 to 276 | 9 |
| powerControlOffsetSS | Reuse the existing range: {-3, 0, 3, 6} dB | 2 |
| periodicityAndOffset | Can reuse the existing structure of CSI-ResourcePeriodicityAndOffset, with periodicity limited to {10, 20, 40, 80} ms. | 10 |
| QCL information | 0 to 63, indicating the QCLed SSB index | 6 |
| FFS: Number of slots for TRS | {1, 2} | 1 |
| FFS: scramblingID | Reuse the existing range: 0 to 1023 | 10 |
| FFS: TRS resource set ID | 0 to (max # of TRS resource set – 1) |  |

# Availability indication of TRS occasion(s)

For the validity duration of availability indication, it has been agreed that:

Agreement

The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication

- FFS: Whether the availability indication is transmitted [only once] during the validity duration

Note: Qualcomm and Huawei have concern on Alt a

Agreement

For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support

- time unit is one default paging cycle,

- applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]}

When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s).

One open issue is whether to allow another availability indication to be transmitted before the expiration of the previous availability indication, and if yes, whether there is any restriction and/or how to interpret the indication.

First of all, it is not reasonable to mandate that another availability indication cannot be transmitted before the expiration of the previous availability indication. For example, there may be another paging message that comes before the expiration, and the network should be allowed to transmit it (which also includes the availability indication). This can occur quite often especially if the validity duration is configured with a large value.

If we allow another availability indication to be transmitted before the expiration of the previous availability indication, the next question is whether to have any restriction on what availability indication to be transmitted. One such example is to only allow the same availability indication. However, there is no real benefit in having such a restriction, because the meaning of the indication is still different due to different reference starting point for the validity duration. Keeping transmitting bit ‘1’ would mean the validity duration is extended, which means the gNB would not be able to avoid extending the validity duration if it needs to transmit another availability indication before the expiration.

Therefore, we think it is better not to have any restriction. The interpretation of each availability indication can be independent, and each bit ‘1’ indicates the availability of the corresponding TRS resource set(s) during the corresponding validity duration, while bit ‘0’ is simply ignored. Figure 1 provides an example to illustrate the idea, where the validity duration is configured as 2 paging cycles. The first availability indication has the first bit as ‘1’, which means TRS#1 is available for 2 paging cycles. The 2nd availability indication has the 2nd bit as ‘1’, which means TRS#2 is available for 2 paging cycles. The 3rd availability indication has both bits as ‘1’, which means both TRS#1 and #2 are available for 2 paging cycles. Putting all 3 indications together, we have TRS #1 available in paging cycle 1/2/3/4, and TRS#2 available in paging cycle 2/3/4. All the bits with value of ‘0’ are ignored. This means that bit ‘0’ is not interpreted as unavailable. Instead, it means the availability status does not change for the corresponding TRS resource set(s). From the network perspective, the network just needs to make sure TRS is available for the validity duration for any bit set to 1, which is not complicated to manage.

Figure 1 Interpretation of availability indication

Proposal 4: It is supported that another availability indication is received before the end of a previous validity duration.

- Bit ‘1’ means that the corresponding TRS configuration(s) are available in the corresponding validity duration, while bit ‘0’ is ignored.

# Conclusion

In this contribution, we discussed the remaining issues for TRS indication to idle/inactive UEs and made the following proposals:

Proposal 1: Support a configuration parameter for the number of TRS resources X for a TRS resource set.

- applicable values for X: {2, 4}

Proposal 2: One or more scrambling IDs is configured for a TRS resource set.

- If a single scrambling ID is configured, it applies to all the TRS resources.

- Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource.

Proposal 3: Further consider introducing common parameters (e.g., starting RB and nrofRBs) for multiple/all TRS resource sets to reduce the signaling overhead.

Proposal 4: It is supported that another availability indication is received before the end of a previous validity duration.

- Bit ‘1’ means that the corresponding TRS configuration(s) are available in the corresponding validity duration, while bit ‘0’ is ignored.

# Reference

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc., RAN#88e, June 2020.

# Appendix A: Related Agreements

## RAN1#102-e

Agreements:

- New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE.

Agreements:

The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs.

-  Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share).

-  Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted.

-  Note: Always-on TRS/CSI-RS transmission by gNodeB is not required

-  At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported

- FFS for other RS types

-  FFS: Whether UE blind detection is required or not.

Agreements:

Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as:

-           AGC, time/frequency tracking

-           FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

Observation:

It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.)

Agreements:

The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling

-           FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.)

-           FFS for other signalling candidates (e.g., pre-configuration, etc.)

-           FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.)

Agreements:

Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly).

- Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreements:

- Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s).

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

Send an LS to RAN2 informing the above agreements, and

- To further add that RAN1 is working on the detailed physical layer design

Draft LS is endorsed, with final LS in R1-2009791.

Agreements:

- Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs.

Agreements:

- Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting)

* Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence

Agreements:

- Discuss further based on the following alternatives and down-select at RAN1#104-e:

  - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE.

  - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

  - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

    - The condition can be, e.g., existence of paging.

  - Alt 4. Combination of the above alternatives.

  - FFS for details

  - FFS for UE behavior when the availability is not informed.

  - Other techniques are not precluded.

  - Companies encourage to provide sufficient information for the proposal, e.g.,

    - how to achieve power saving gain

    - how to minimize impact on NW

    - how to minimize extra UE implementation complexity

    - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs

  - Proposals should be consistent with the WID objective.

Conclusion:

- TRS/CSI-RS based PEI is discussed in AI 8.7.1.1.

- PEI functionality is not further discussed under AI 8.7.1.2.

- Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Agreements:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least:

- powerControlOffsetSS,

- scramblingID

- firstOFDMSymbolInTimeDomain,

- startingRB.

- nrofRBs,

- FFS other parameters

- FFS applicable values

Agreements:

The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#104b-eRAN1#105-e:

- Alt1: same as initial BWP

- Alt2: configurable parameter

Agreements:

Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details (including whether or not to restrict the RS to be TRS only)

Agreements:

For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication.

- FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.)

- There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

Conclusion

From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs.

Agreements:

The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e:

- Alt-1: within initial DL BWP

- Alt-2: is not restricted by initial BWP

  - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

Agreements:

To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives:

- Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS

- Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO

  - FFS details

- FFS details

- Other alternatives are not precluded

Conclusion:

Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104bis-e

Agreement:

SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0.

Agreement:

Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details of the QCL information, e.g. associated SSB index

Agreement:

IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

- Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP.

Working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling

Agreement:

Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations

- Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured.

  - The parameter trs-info does not need to be provided in the configuration

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

## RAN1#105-e

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset {10, 20, 40, 80} ms

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

Agreement:

Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

## RAN1#106-e

Agreement

Support at least one of the following alternatives

- Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion.

- Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

Note:  The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication.

Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Agreement

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where

- the time duration can be determined based on at least one from the following (to be down-selected):

  - Alt-1: configured by higher layer

  - Alt-2: a predefined/configured window

  - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s)

  - Alt-4: until when the UE receives another availability indication

  - A combination of alternatives or other alternatives is not precluded.

- the reference point can be determined as at least one from the following (to be down-selected):

  - Alt-1: start of next PO or DRX cycle

  - Alt-2: time location where UE receives the indication

    - Note: the time location is subject to application delay if agreed

  - Alt-3: start of current PO or DRX cycle where UE receive the indication

  - Alt-4: a time location which is configured by higher layer

  - A combination of alternatives or other alternatives is not precluded.

Agreement

For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as

  - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

## RAN1#106bis-e

Conclusion

No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs

Working Assumption

If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

Agreement

For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available.

- support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion

  - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set

  - Bitmap size is up to X bits

    - X = [6] for paging PDCCH based L1 availability indication.

    - FFS X for PEI DCI based L1 availability indication

    - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured)

- for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources

  - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources

- FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion

- FFS: indication of unavailability

Agreement

At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where

- the time duration is a validity duration configured by higher layer,

  - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period)

  - FFS UE doesn’t expect inconsistent L1 based indication during the time duration

- the reference point for start of the validity duration is one of the following alternatives:

  - Alt1: SFN of the first PF from the next DRX cycle

  - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication

  - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration

  - Alt4: start of the PF for the PO where UE receives the indication

  - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB

  - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0

- the time duration can be optionally configured by gNB

  - when the time duration is not configured, one of the following alternatives can be considered:

    - Alt1: the availability indication is valid until when the UE receives another availability indication.

    - Alt2: the availability indication is valid until L1 availability indication is changed by network

    - Alt3: default time duration e.g. default paging cycle

- FFS whether and how to handle the miss detection issue of L1 signaling

Agreement

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:

        a TRS resource set can be configured to include

o   a set of TRS resources up to two consecutive slots,

  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol.

o   at least common configuration parameters:

  a QCL reference

  firstOFDMSymbolInTimeDomain,

  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’

  FFS

        scramblingID,

        a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported

        Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

## RAN1#107-e

Agreement

For the maximum number of TRS resource sets configured by higher layer, X,

- X = 64

- FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

Agreement

For L1 availability indication using a bitmap, the following is supported:

- Number of bits in the bitmap, N, is up to 6 bits

- a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on

  - explicit configuration of TRS resource set group, where

    - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field.

    - the ith bit maps to all the TRS resource set(s) associated with ID i.

- start of the bitmap is the first bit of the reserved bits in paging PDCCH

- Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations.

Agreement

The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication

- FFS: Whether the availability indication is transmitted [only once] during the validity duration

Note: Qualcomm and Huawei have concern on Alt a

Agreement

For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support

- time unit is one default paging cycle,

- applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]}

When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s):

Agreement

Confirm the following working assumption

Working Assumption

- Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.