3GPP TSG RAN WG1 #107bis-e			R1-2200433

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.15.8

Source:                Apple Inc.

Title:                     Views on Rel-17 Coverage Enhancement UE Features

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our analysis on UE features for NR coverage enhancement. The discussion is based on  the upated RAN1 UE features list for Rel-17 NR in [1].

# Discussion

Repetition type A enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes |

In last RAN1 meeting, it was agreed that type 1 CG PUSCH supports 32 repetitions via the parameter RepK-r17, and this parameter is also applied to type 2 CG PUSCH as well. thus, the FG30-1a can be updated to support type 1 configured grant PUSCH type A repetitions, one additional component can be added that repetition can be indicated via RepK-r17. Regarding FG30-2a, due to the counting method is different from FG30-2, it should be an independent FG.

Proposal 1: Keeping FG30-2a as an independent FG group.

Proposal 2: The component of 30-1a is updated to support Type 1 configured grant PUSCH,

K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions.

- The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration.

- The number of repetitions is indicated in RepK-r17 for both Type 1 and Type 2 configured grant PUSCH.

TBoMS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 | TBD | Yes |

For FG30-3, the FG of dynamic grant scheduled TBoMS and configured grant based TBoMS can be separated defined, as the implementation and specification impacts are different, especially how to support type 1 configured grant for TBoMS is not clearly defined yet.

Proposal 3: FG30-3 is split into two separated FGs for dynamic grant based TBoMS and configured grant based TBoMS respectively.

For TBoMS frequency hopping, it was agreed that for a single TBoMS transmission and TBoMS repetitions, the legacy Rel-15/16 intra-slot and inter-slot frequency hopping framework used in PUSCH repetition Type A is supported. Even no new frequency hopping scheme is defined for TBoMS, it still needs a UE feature to differentiate the UE whether inter-slot/intra-slot frequency hopping is supported by TBoMS.

Proposal 4: Introducing FG30-3b to specify intra-slot and inter-slot frequency hopping for TBoMS.

DMRS-bundling

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |
| --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes |

For FG30-4g, it was agreed in last RAN1 meeting that UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events. Thus, FG30-4g component need to update accordingly.

Proposal 5: Updating the FG40-4g to apply to dynamic event only.

For FG30-4a/4b/4c/4d, these features are associated with different transmission schemes. The prerequisite features are quite different. From UE implementation perspective, it’s preferred to keep each FG independent to leave more choices for UE implementation.

Proposal 6: Keeping FG30-4a, 4b, 4c, 4d as independent FG groups.

# Summary

In this contribution, we discuss the UL coverage enhancement related UE features and give the following proposals.

Proposal 1: Keeping FG30-2a as an independent FG group.

Proposal 2: The component of 30-1a is updated to support Type 1 configured grant PUSCH,

K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions.

- The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration.

- The number of repetitions is indicated in RepK-r17 for both Type 1 and Type 2 configured grant PUSCH.

Proposal 3: FG30-3 is split into two separated FGs for dynamic grant based TBoMS and configured grant based TBoMS respectively.

Proposal 4: Introducing FG30-3a to specify intra-slot and inter-slot frequency hopping for TBoMS.

Proposal 5: Updating the FG40-4g to apply to dynamic event only.

Proposal 6: Keeping FG30-4a, 4b, 4c, 4d as independent FG groups.

# References

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 107-e”, Moderators (AT&T, NTT DOCOMO, INC.)