3GPP TSG-RAN WG1#107bis-e	Tdoc R1-2200444

e-Meeting, January 17th-25th, 2022

Agenda Item:	8.15.3

Source:	Ericsson

Title:	Rel-17 UE Features for IIoT/URLLC

Document for:	Discussion, Decision

# Introduction

In this contribution, the remaining issues of Rel-17 UE Features for IIoT/URLLC are discussed [1][2] For each enhancement aspect, the candidate UE features are proposed in each section below.

# Discussion

The remaining open issues from the FL summary [3] are discussed in the following sections.

## UE Feedback Enhancements for HARQ-ACK

### Type of FG 25-1 to 25-10

A question was raised whether to consider FSPC as the Type for these features. From our perspective, the corresponding functionality of the features under FG 25-1 to FG 25-10 are not affected when testing for different bands. Therefore, it is not clear that band differentiation would impact the feature and the corresponding testing. Therefore, we suggest to indicate “Per UE” as the Type, and not accept other proposed alternatives such as FSPC. This suggestion is also in line with RAN2 recommendation, that FSPC indication should be avoided as much as possible (R2-2002378)

Adopt “Per UE” type for FG 25-1 to FG 25-10.

### FG 25-9 and FG 25-10 and numerology

It has been discussed last meeting whether each of these features should be split into two capabilities, one for the same numerology between two cells and the other for different numerologies between two cells.

From our point of view, such split is not needed. During the discussion of PUCCH cell switching, the possibility of having different numerologies has been a central assumption in design. Hence, the corresponding agreements were inclusive with respect to numerologies being the same or different, and there is no agreement to differentiate the behaviour for the same or different numerologies.

Do not split FG 25-9 and FG 25-10 each into two capabilities, one for the same numerology and the other for different numerologies between two cells for cell switching.

## CSI Feedback Enhancements

For FG 25-11 in the preliminary list [1], no prerequisite FG is stated. However, the 4-bit subband CQI reporting should be an enhancement of basic CSI feedback. Hence FG 2-32 “Basic CSI feedback” should be added as a prerequisite as shown below.

Additionally, the 4-bit subband CQI should be confirmed as a “per UE” capability.

Table 1: Proposed FG for enhanced CSI feedback.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband | 2-32 |

Add FG 2-32 “Basic CSI feedback” as a prerequisite  for FG 25-11 “4-bits subband CQI”.

Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

## Enhancements for Unlicensed Band URLLC/IIoT

The remain issue with respect to UE features for operation on shared spectrum was whether to update the component description of FG 25-12 as the following:

Table 2: FG 25-12 with proposed updates for component description

| 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI |
| --- | --- | --- |

From our perspective, the proposed additions are important to be included and tested to ensure proper intra-operability.

Adopt the proposed changes in red in Table 2 for FG 25-12.

## Intra-UE Multiplexing/Prioritization

### Prioritization of overlapping DG-PUSCH and CG-PUSCH

In RAN1#107e UE feature discussion, progress was made for 25-14:

Agreement

- FG 25-14 is kept as “PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH” as follows

| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that any contents highlighted in yellow mean FFS and to be discussed further.

Additionally, in RAN1#107e AI 8.3, the following agreements were made:

Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively.

Thus it is clear that Rel-17 supports [25-15]:

| [25-15] | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 |
| --- | --- | --- | --- |

In our view a single UE feature should be defined for overlapping DG-PUSCH vs CG-PUSCH of different priorities. It does not make sense that a UE would support one of 25-14/[15], but not the other. Thus we propose to expand 25-14 to include the components of [25-15].

Merge [25-15] into 25-14. The description is revised to “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different priorities”.

As shown in the agreements of AI 8.3, the UE is expected to apply cancellation per actual repetition, if (LP/HP) DG-PUSCH and/or (HP/LP) CG-PUSCH is repeated. Thus it’s necessary to add the support of PUSCH with repetition in the merged 25-14 and [25-15].

Add support of PUSCH with repetition in the components for merged 25-14 and [25-15].

Furthermore, the additional time d3 in the agreement of AI 8.3 should be added as a component to [25-15] (to be merged with 25-14.

Add a component for the additional cancellation time d3, together with its candidate value set: “Additional number of symbols (d3) of the preparation time needed for the high priority DG-PUSCH transmission that cancels a low priority CG-PUSCH transmission. Candidate values for d3 are: {0, 1,…,2μ+1}”

In summary, we suggest to update 25-14 to the following:

| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different priorities | - Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH - Cancellation is applied per actual repetition, if low-priority DG-PUSCH and/or high-priority CG-PUSCH is repeated. - Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell - Cancellation is applied per actual repetition, if high-priority DG-PUSCH and/or low-priority CG-PUSCH is repeated. - Additional number of symbols (d3) of the preparation time needed for the high-priority DG-PUSCH transmission that cancels a low priority CG-PUSCH transmission. Candidate values for d3 are: {0, 1,…,2μ+1} | 12-1 |
| --- | --- | --- | --- | --- |

### Simultaneous PUCCH and PUSCH transmission

In RAN1#107e AI 8.3, the following conclusions were made:

Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmission of same priority over different cells in Rel-17. Conclusion There is no consensus in RAN1 to support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA in Rel-17.

Thus, Rel-17 only supports simultaneous PUCCH/PUSCH transmission of different priorities on different cells for inter-band CA. Accordingly FG 25-18 can be finalized as below:

| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells [at least] for inter-band CA. |
| --- | --- | --- | --- |

FG 25-18a component description is revised to: “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.”

## Propagation Delay Compensation

For the topic of propagation delay compensation, significant progress has been made in RAN1#107e, for example, the key agreement below:

Agreement For Rel-17 - Support RTT-based PDC method - Support PDC method based on legacy TA-based mechanism - No RAN1/RAN4 specification impact expected

Thus two FG need to be introduced for propagation delay compensation, one for legacy TA-based method, the other for RTT-based method.  A UE may support the legacy TA-based FG for poor synchronization accuracy on the Uu interface, and/or support the RTT-based FG for tighter synchronization accuracy.

Two feature groups are introduced for supporting propagation delay compensation in Rel-17, one for legacy TA-based method, the other for RTT-based method.

Specifically, we suggest the FGs and components below. Further details for the components (e.g., gNB-based or UE-based compensation) can be discussed when the relevant decisions are made in RAN2.

Table 3: Exemplary FG for propagation delay compensation.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Legacy TA-based propagation delay compensation | Support legacy TA based propagation delay compensation for time synchronization of the Uu interface | N/A | Yes | Per UE | No | No | N/A | Optional with capability signalling |
| RTT-based propagation delay compensation | Support RTT-based propagation delay compensation for time synchronization of the Uu interface | N/A | Yes | Per UE | No | No | N/A | Optional with capability signalling |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt “Per UE” type for FG 25-1 to FG 25-10.

Proposal 2	Do not split FG 25-9 and FG 25-10 each into two capabilities, one for the same numerology and the other for different numerologies between two cells for cell switching.

Proposal 3	Add FG 2-32 “Basic CSI feedback” as a prerequisite  for FG 25-11 “4-bits subband CQI”.

Proposal 4	Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

Proposal 5	Adopt the proposed changes in red in Table 2 for FG 25-12.

Proposal 6	Merge [25-15] into 25-14. The description is revised to “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different priorities”.

Proposal 7	Add support of PUSCH with repetition in the components for merged 25-14 and [25-15].

Proposal 8	Add a component for the additional cancellation time d3, together with its candidate value set: “Additional number of symbols (d3) of the preparation time needed for the high priority DG-PUSCH transmission that cancels a low priority CG-PUSCH transmission. Candidate values for d3 are: {0, 1,…,2μ+1}”

Proposal 9	FG 25-18a component description is revised to: “Support simultaneous PUCCH/PUSCH transmissions of different priorities on different cells for inter-band CA.”

Proposal 10	Two feature groups are introduced for supporting propagation delay compensation in Rel-17, one for legacy TA-based method, the other for RTT-based method.

# References

R1-2112902		Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e; Moderators (AT&T, NTT DOCOMO, INC.)

R1-2112903		LS on updated Rel-17 RAN1 UE features list for NR; RAN1, AT&T, NTT DOCOMO, INC.

R1-2112135		Summary on UE features for enhanced IIoT and URLLC, Moderators (NTT DOCOMO, INC.)