3GPP TSG RAN WG1 #107-bis-e                                                 R1-2200476

e-Meeting, January 17th – 25th, 2022

Source: 	Intel Corporation

Title: 	UE Features for NR Positioning Enhancements

Agenda item:	8.15.5

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide our views on UE features for NR positioning enhancements based on input provided in [1] and considering agreements made at RAN1#107e [2] (please refer to conclusion section).

# Views on UE Features for NR Positioning Enhancements

## Latest Version of UE FGs for NR Positioning Enhancements

The latest available version of UE features for NR positioning enhancements was captured in [1] and is expected to be further revised based on RAN1 discussion (please refer to Table 1 providing information on FGs and Annex A for additional details):

Table 1: Latest version of feature groups for NR positioning enhancements from source - R1-2112902

| Index | Feature group | Pre-requisite FGs |
| --- | --- | --- |
| 27-1-1 | Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | 13-1, one or more of {13-3, 13-4} |
| 27-1-2 | Support of UE-TxTEGs for UL TDOA | [13-4, ]13-8 |
| 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | 13-4, 13-8 |
| 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | [13-4 or 13-8] |
| 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | 27-1-1 |
| 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  |
| 27-2-1 | [UE-assisted] DL PRS RSRP of the first path for DL-AoD | [13-2 or 13-3, 13-4, 13-5, 13-8] |
| 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | [13-5, 13-2] |
| 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [13-1, 13-4, 13-8] |
| 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | [13-1] |
| 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 27-3-2 |
| 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning |  |
| 27-5-1 | [UE-initiated] on-demand PRS | [13-1] |
| 27-6 | DL PRS processing capabilities in RRC inactive state |  |
| 27-7 | Maximum number of measurement instances which can be included in a single measurement report |  |
| 27-8 | Support of PRS TEG association information for UE-based DL-TDOA |  |
| 27-9 | Support of lower Rx beam sweeping factor |  |
| 27-10 | Support of UL MAC CE based MG activation request |  |
| 27-11 | Support of DL MAC CE based MG activation |  |
| 27-12 | LOS/NLOS indicator for UE-based positioning assistance data |  |
| 27-13 | Additional path reporting for UE-assisted DL-TDOA |  |
| 27-14 | Additional path reporting for Multi-RTT |  |
| 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] |  |
| 27-16 | OLPC for positioning SRS in RRC_INACTIVE state |  |
| 27-17 | Support of [PRS measurement in RRC_INACTIVE] |  |
| 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA |  |
| 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD |  |
| 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT |  |
| 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state |  |

In above table, the rows highlighted in yellow are left for further discussion. In this contribution, we use the above table as a starting point for discussion and provide our comments aiming to update the above table based on discussion in the next section.

## Proposed Changes for UE FGs on NR Positioning Enhancements

### Rx/Tx Timing Error Mitigation

Considering that UE RxTEGs are associated with UE measurements (DL PRS-RSTD and UE Rx-Tx time difference measurements), we prefer to have separate UE RxTEG FGs for DL TDOA and Multi-RTT UE-assisted positioning.

  - Split FG 27-1-1 into two FGs:

    - 27-1-1a Support of UE-RxTEGs for UE-assisted DL TDOA

    - 27-1-1b Support of UE-RxTEGs for UE-assisted Multi-RTT positioning

Considering that different methods were agreed for UE TxTEGs reporting to gNB (RRC) and LMF (LPP) for UL TDOA and Multi-RTT respectively, we prefer to have separate UE TxTEG FGs for UL TDOA and Multi-RTT UE-assisted positioning.

  - Rename FG 27-1-2 to:

    - 27-1-2a Support of UE-TxTEGs reporting in RRC for UL TDOA

  - Rename FG 27-1-2a to:

    - 27-1-1b Support of UE TxTEGs reporting in LPP for Multi-RTT positioning

Regarding FG 27-1-4a (Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously), we are open to introduce this feature, if corresponding request / report relationship is introduced in LPP signaling, so that LMF can request UE to perform measurements using different UE RxTEGs for the same DL PRS resource simultaneously.

  - Support FG 27-1-4a only if LPP signaling for request of simultaneous measurement using different RxTEGs for the same DL PRS resource is introduced

### DL-AOD Enhancements

- RAN1 agreed that for the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported

Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE

  - The type of expected angle and uncertainty can be requested by the UE, between the following options

    - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

    - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

  - Introduce new FG 27-2-3 for DL-AoD positioning enhancements:

    - Support of assistance information for DL-AoD measurements that includes following components

      - Support of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s)

      - Support of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s)

The proposed new FG 27-2-3 can be further divided into two FGs:

27-2-3a: Support of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s)

27-2-3b: Support of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s)

- RAN1 agreed that TRP beam/antenna can be optionally provided by the LMF to the UE for UE-based DL-AoD. This requires definition of new additional UE capability.

  - Introduce new FG 27-2-4 for DL-AoD positioning enhancements:

    - 27-2-4: Support of TRP beam/antenna information for UE-based DL-AoD positioning

In our understanding, for DL-AOD UE can report either DL PRS-RSRPP or DL PRS-RSRP or their combination. Additional FG is needed for reporting of both DL PRS-RSRP and DL PRS-RSRPP for the first arrival path.

  - Introduce new FG 27-2-5 for DL-AoD positioning enhancements:

    - 27-2-5: Support of the first path DL PRS RSRPP and DL PRS RSRP reporting for DL-AOD in the same report

RAN1 agreed that for UE-assisted DL-AOD positioning method, the LMF may indicate in the assistance data, one or both of the following options:

Option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting

Option 2: subject to UE capability, for each PRS resource, the boresight direction information

  - Introduce two new FGs 27-2-6 and 27-2-7 for DL-AoD positioning enhancements:

    - 27-2-6: Support of prioritization information for DL PRS reporting (subset of DL PRS resources) for DL-AOD positioning

    - 27-2-7: Support of the boresight direction indication for the DL PRS resource for DL-AOD positioning

### Latency Reduction Enhancements

For FG 27-3-1, the M-sample measurement was agreed only for M = 1. Therefore, we propose the following update for FG 27-3-1.

  - Support FG 27-3-1 and define new FG name

    - 27-3-1: Support of single DL PRS period measurement (M = 1) and reporting

DL PRS measurement outside MG within PRS processing priority window was agreed by RAN1 and therefore corresponding UE capability needs to be introduced. A UE can indicate the number of states supported for DL PRS processing priority handling.

  - Support FG 27-3-2 and slightly modify its name

    - 27-3-2: DL PRS measurement outside MG within PRS processing priority window

The following agreement was reached by RAN1 at (RAN1#104e) with regards to reporting of multiple measurement instances in a single report (batch reporting).

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signaling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

The relevant FG 27-7 was added for discussion in draft table. According to our understanding this specific feature was not finalized and therefore may be dropped. Alternatively, discussion on this aspect may continue in RAN2, if time allows and other essential issues are resolved.

  - Delete FG 27-7: Maximum number of measurement instances which can be included in a single measurement report

### Multipath/NLOS Mitigation Enhancements

For LOS/NLOS indication in case of UE-assisted positioning, we think that the split on hard and soft indication is desirable to avoid potential ambiguity in behavior for LMF and UE. Therefore, we propose to split FG 27-4-1 to FGs 27-4-1a (hard LOS/NLOS indication) and 27-4-1b (soft LOS/NLOS indication).

We also propose to define the above capabilities per positioning method (DL-TDOA, DL-AOD, Multi-RTT) and associated measurements (DL RSTD, DL PRS-RSRP, UE Rx-Tx time difference).

  - Split FG 27-4-1 to two FGs:

    - 27-4-1a: Support of hard LOS/NLOS indication for UE-assisted DL-TDOA positioning

    - 27-4-1b: Support of hard LOS/NLOS indication for UE-assisted DL-AOD positioning

    - 27-4-1c: Support of hard LOS/NLOS indication for UE-assisted Multi-RTT positioning

    - 27-4-1d: Support of soft LOS/NLOS indication for UE-assisted DL-TDOA positioning

    - 27-4-1e: Support of soft LOS/NLOS indication for UE-assisted DL-AOD positioning

    - 27-4-1f: Support of soft LOS/NLOS indication for UE-assisted Multi-RTT positioning

The per path DL PRS-RSRP (DL PRS-RSRPP) and per path relative timing measurements were agreed by RAN1 for DL-TDOA reporting enhancements. We support FG and propose to clarify its components:

  - Rename FG 27-13 and clarify it components of the FG 27-13:

    - 27-13: Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted DL-TDOA:

      - Reporting of up to N = 8 DL PRS-RSRPP values together with relative timing measurement (relative to the first path timing) as part of DL-TDOA reporting

The per path DL PRS-RSRP (DL PRS-RSRPP) and per path relative timing measurements were agreed by RAN1 for Multi-RTT reporting enhancements. We propose to support FG and clarify its components as follows:

  - Support FG 27-14:

    - 27-14: Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted Multi-RTT:

      - Reporting of up to N = 8 DL PRS-RSRPP values together with relative timing measurement (relative to the first path timing) as part of Multi-RTT reporting

The first path DL PRS-RSRP (DL PRS-RSRPP) were agreed by RAN1 for DL-TDOA and Multi-RTT reporting enhancements. We propose to support the corresponding FGs:

  - Introduce new FG 27-13a for DL-TDOA positioning enhancements

    - 27-13a: Support of the first path DL PRS-RSRPP reporting for UE-assisted DL-TDOA positioning

  - Introduce new FG 27-14a for Multi-RTT positioning enhancements

    - 27-14a: Support of the first path DL PRS-RSRPP reporting for UE-assisted Multi-RTT positioning

Finally, the extension of the set of existing values for DL PRS processing capabilities within MG, in particular the processing window T can be beneficial for latency reduction and set of values {1 ms, 2 ms, 4 ms} can be introduced.

Introduce additional values {1ms, 2ms, 4 ms} for parameter T defined for UE DL PRS processing capabilities within MG

Add new values for R17 UEs for Rel-16 FG 13-1 Common DL PRS Processing Capability

  - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

  - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

### On-demand DL PRS Support

For on-demand DL PRS support, the UE capability for UE initiated on-demand DL PRS request was agreed. We propose a minor modification to the existing name of the FG 27-5-1.

  - Rename FG 27-5-1 to:

    - 27-5-1: Support of UE-initiated on-demand DL PRS request

### Support for RRC_INACTIVE UEs

It was agreed in RAN2 that LMF is not aware about RRC state of the UE (i.e., LMF procedures are transparent to the UE RRC state). Therefore, from LMF perspective there is no need to define UE capabilities for positioning support in RRC_INACTIVE state. On the other hand, such UE capabilities may be needed for UE testing purposes to ensure that features are supported by UE. From gNB perspective, it should be aware at least about UE capabilities of SRS for positioning transmission in RRC_INACTIVE state as it provides SRS for positioning configuration to UE.

### FGs for SRS for Positioning in RRC_INACTIVE State

RAN1 agreed that SRS for positioning transmission in RRC_INACTIVE state may be performed inside and outside of the initial UL BWP, subject to certain conditions. Different numerology settings can be configured for SRS for positioning transmission outside the initial UL BWP. This functionality requires definition of two new UE FGs.

  - Split FG 27-15 to two FGs:

    - 27-15a: Support of SRS for positioning transmission inside initial UL BWP in RRC_INACTIVE state

    - 27-15b: Support of SRS for positioning transmission outside initial UL BWP in RRC_INACTIVE state

Considering that gNB needs to provide configuration of SRS for positioning for RRC_INACTIVE UEs, the FGs for SRS for positioning including OLPC and spatial relation need to be discussed. There are two possible ways how it can be handled:

Introduce new FGs for RRC_INACTIVE state based on corresponding Rel.16 FGs

Extend functionality of the Rel.16 FGs including both RRC_INACTIVE and RRC_CONNECTED states

In our view introducing new FGs may be a cleaner solution and can be pursued in Rel.17. RAN1 may also need to consult with RAN2 and elaborate on potential options for UE capability definitions.

  - Further discuss whether/how to reuse Rel.16 FGs for SRS for positioning OLPC (FGs 13-9/9a/9b/9e/9f) and spatial relation (FGs 13-10/10a/10b/10d/10e/10f) or define new FGs for support of SRS for positioning in RRC_INACTIVE state (considering Rel.16 FGs defined for RRC_CONNECTED state), i.e.:

    - OLPC

      - FG 27-16-1: OLPC for SRS for positioning based on DL PRS from serving cell

      - FG 27-16-2: OLPC for SRS for positioning based on DL PRS from neighboring cells

      - FG 27-16-3: OLPC for SRS for positioning based on SSB from serving cells

      - FG 27-16-4: Pathloss estimate maintenance for serving cell

      - FG 27-16-5: Pathloss estimate maintenance across all cells

    - Spatial relation

      - FG 27-19-1: Spatial relation for SRS for positioning based on SSB from serving cell

      - FG 27-19-2: Spatial relation for SRS for positioning based on CSI-RS from serving cell

      - FG 27-19-3: Spatial relation for SRS for positioning based on DL PRS from serving cell

      - FG 27-19-4: Spatial relation for SRS for positioning based on SSB from non-serving cells

      - FG 27-19-5: Spatial relation for SRS for positioning based on DL PRS from non-serving cells

      - FG 27-19-6: Spatial relation maintenance

RAN1 may also need to discuss introduction of new FGs for SRS for positioning resources (including periodic and semi-persistent) for RRC_INACTIVE UEs, unless corresponding Rel.16 FGs can be considered as pre-requisites for SRS for positioning transmission in the RRC_INACTIVE state.

### FGs for DL PRS in RRC_INACTIVE State

Regarding FGs 27-6, 27-17, 27-18a/b/c, based on RAN2 agreements, there is no different handling on PRS for the different RRC state. From LMF perspective, the LMF only needs to know whether the UE supports Rel.16 measurements or not.

If the UE cannot support it in RRC_INACTIVE state, the UE should trigger the transition to the RRC_CONNECTED state. Therefore, dedicated LPP capability for RRC_INACTIVE state is not needed, we should rely on the existing LPP capability. On the other hand, UE should be able to declare that corresponding NR positioning functionality is supported at least for the testing purposes and thus it can be defined without introducing LPP capability signaling.

The first family of the FGs that needs to be added is associated with NR positioning measurement capabilities for UEs in the RRC_INACTIVE state.

  - Select between two options:

    - Option 1: Define FGs associated with NR positioning measurements in RRC_INACTIVE state without introduction of UE LPP and RRC capability signaling

      - FG 27-18a: Support of DL PRS measurement in RRC_INACTIVE state for DL-AoD

      - FG 27-18b: Support of DL PRS measurement in RRC_INACTIVE state for DL-TDOA

      - FG 27-18c: Support of UE Rx-Tx measurement in RRC_INACTIVE state for Multi-RTT

      - FG 27-18d: Support of SS-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18e: Support of SS-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18f: Support of CSI-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18g: Support of CSI-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18h: Support of simultaneous DL-AoD and DL-TDOA processing in RRC_INACTIVE state

      - FG 27-18i: Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

    - Option 2: Discuss UE capability for UEs in RRC_INACTIVE state directly in RAN2 as it is a RAN2-led objective

In Rel.16, the FGs for DL PRS resources for various NR positioning techniques were defined in [3]. The same FGs are in general applicable for NR positioning by RRC_INACTIVE UEs.

  - The Rel.16 NR positioning FGs (13-2/2a/2b, 13-3/3a/3b, 13-4/4a/4b) for DL PRS resources and different NR positioning techniques are reused and defined as pre-requisites for FGs associated with NR positioning measurements by RRC_INACTIVE UEs

## Summary of Proposed Changes for UE FGs on NR Positioning Enhancements

In this section we provide a revised list (please refer to Table 2) of FGs for NR positioning enhancements based on discussion provided in section 2.2. The proposed modifications with respect to the original Table 1 are highlighted in blue.

Table 2: Proposed feature groups for NR positioning enhancements

| Index | Feature group |  |
| --- | --- | --- |
| 27-1-1a | Support of UE-RxTEGs for UE-assisted DL TDOA |  |
| 27-1-1b | Support of UE-RxTEGs for UE-assisted Multi-RTT positioning |  |
| 27-1-2a | Support of UE-TxTEGs reporting in RRC for UL TDOA |  |
| 27-1-2b | Support of UE-TxTEGs reporting in LPP for Multi-RTT positioning |  |
| 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT |  |
| 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource |  |
| 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously [only if LPP signaling for request of simultaneous measurement using different RxTEGs for the same DL PRS resource is introduced] |  |
| 27-2-1 | Support of the first path DL PRS RSRPP for [UE-assisted] DL AoD |  |
| 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL AoD positioning |  |
| 27-2-3 | Support of assistance information for DL AoD measurements - expected DL AoD/ZoD values and uncertainty ranges - expected DL AoA/ZoA values and uncertainty ranges |  |
| 27-2-4 | Support of TRP beam/antenna information for UE-based DL-AoD positioning |  |
| 27-2-5 | Support of the first path DL PRS RSRPP and DL PRS RSRP reporting for DL-AOD in the same report |  |
| 27-2-6 | Support of prioritization information for DL PRS reporting (subset of DL PRS resources) for DL-AOD positioning |  |
| 27-2-7 | Support of the boresight direction indication for the DL PRS resource for DL-AOD positioning |  |
| 27-3-1 | Support of single DL PRS period measurement (M = 1) and reporting |  |
| 27-3-2 | DL PRS measurement outside MG within PRS processing priority window |  |
| 27-3-3 | DL PRS processing capability outside MG – buffering capability |
| 27-4-1a | Support of hard LOS/NLOS indication for UE-assisted DL-TDOA positioning |  |
| 27-4-1b | Support of hard LOS/NLOS indication for UE-assisted DL-AOD positioning |
| 27-4-1c | Support of hard LOS/NLOS indication for UE-assisted Multi-RTT positioning |
| 27-4-1d | Support of soft LOS/NLOS indication for UE-assisted DL-TDOA positioning |  |
| 27-4-1e | Support of soft LOS/NLOS indication for UE-assisted DL-AOD positioning |
| 27-4-1f | Support of soft LOS/NLOS indication for UE-assisted Multi-RTT positioning |
| 27-5-1 | Support of UE-initiated on-demand DL PRS request |  |
| 27-6 | DL PRS processing capabilities in RRC inactive state |  |
| 27-7 | Maximum number of measurement instances which can be included in a single measurement report |
| 27-8 | Support of PRS TEG association information for UE-based DL-TDOA |  |
| 27-9 | Support of lower Rx beam sweeping factor |  |
| 27-10 | Support of UL MAC CE based MG activation request |  |
| 27-11 | Support of DL MAC CE based MG activation |  |
| 27-12 | LOS/NLOS indicator for UE-based positioning assistance data |  |
| 27-13 | Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted DL-TDOA |  |
| 27-13a | Support of the first path DL PRS-RSRPP reporting for UE-assisted DL-TDOA positioning |  |
| 27-14 | Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted Multi-RTT |  |
| 27-14a | Support of the first path DL PRS-RSRPP reporting for UE-assisted Multi-RTT positioning |  |
| 27-15a | Support of SRS for positioning transmission inside initial UL BWP in RRC_INACTIVE state |  |
| 27-15b | Support of SRS for positioning transmission outside initial UL BWP in RRC_INACTIVE state |  |
| 27-16 | OLPC for SRS for positioning in RRC_INACTIVE state |  |
| 27-17 | Support of [PRS measurement in RRC_INACTIVE] |  |
| 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA |  |
| 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD |  |
| 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT |  |
| 27-18d | Support of SS-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning |
| 27-18e | Support of SS-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning |
| 27-18f | Support of CSI-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning |
| 27-18g | Support of CSI-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning |
| 27-18h | Support of simultaneous DL-AoD and DL-TDOA processing in RRC_INACTIVE state |
| 27-18i | Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state |
| 27-19 | Spatial relation for SRS for positioning in RRC_INACTIVE state |  |

  - For NR Positioning enhancements, adopt UE FGs as defined in Table 2

# Discussion on FG Components

In this section we provide a revised list of FGs for NR positioning enhancements supplemented by the detail description of the FG components (please refer to Table 3).

Table 3: Proposed feature groups with components for NR positioning enhancements

| Index | Feature group | Components |
| --- | --- | --- |
| 27-1-1a | Support of UE-RxTEGs for UE-assisted DL TDOA | The maximum number of RxTEG IDs that can be supported and reported by the UE to LMF for UE-assisted DL-TDOA positioning |
| 27-1-1b | Support of UE-RxTEGs for UE-assisted Multi-RTT positioning | The maximum number of RxTEG IDs that can be supported and reported by the UE to LMF for UE-assisted Multi-RTT positioning |
| 27-1-2a | Support of UE-TxTEGs reporting in RRC for UL TDOA | The maximum number of TxTEG IDs that can be supported and reported by the UE to serving gNB in RRC for UL-TDOA positioning |
| 27-1-2b | Support of UE-TxTEGs reporting in LPP for Multi-RTT positioning | The maximum number of TxTEG IDs that can be supported and reported by the UE to LMF in LPP for Multi-RTT positioning |
| 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE RX Rx-Tx TEG IDs that can be supported and reported by the UE to LMF in LPP for Multi-RTT positioning |
| 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE RxTEG IDs that can be supported and reported by the UE to measure the same DL PRS resource |
| 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously [only if LPP signaling for request of simultaneous measurement using different RxTEGs for the same DL PRS resource is introduced] | The maximum number of different UE RxTEG IDs that can be supported and reported by the UE to measure the same DL PRS resource simultaneously |
| 27-2-1 | Support of the first path DL PRS RSRPP for [UE-assisted] DL AoD | - Support for the first path DL PRS-RSRPP measurement and reporting for the DL-AOD positioning - The maximum number of first path DL PRS-RSRPP reported values per TRP |
| 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. |
| 27-2-3 | Support of assistance information for DL AoD measurements - expected DL AoD/ZoD values and uncertainty ranges - expected DL AoA/ZoA values and uncertainty ranges | Support for assistance information reporting for the DL-AOD positioning using: - DL AoD/ZoD values and uncertainty range - DL AoA/ZoA values and uncertainty range |
| 27-2-4 | Support of TRP beam/antenna information for UE-based DL-AoD positioning | Support of TRP beam/antenna information for UE based DL-AoD positioning (DL-AoD estimation) |
| 27-2-5 | Support of the first path DL PRS RSRPP and DL PRS RSRP reporting for DL-AOD in the same report | - Support for the first path DL PRS-RSRPP and DL PRS RSRP measurement and reporting for DL-AOD positioning in the same report - The maximum number of supported first path DL PRS-RSRPP values M - The maximum number of supported DL PRS-RSRP values N, M ≤ N |
| 27-2-6 | Support of prioritization information for DL PRS reporting (subset of DL PRS resources) for DL-AOD positioning | Support of prioritization for DL PRS measurement and reporting for the DL-AOD positioning |
| 27-2-7 | Support of the boresight direction indication for the DL PRS resource for DL-AOD positioning | Support for specific boresight directions measurement and reporting for the DL-AOD positioning |
| 27-3-1 | Support of single DL PRS period measurement (M = 1) and reporting | Support of a single DL PRS sample (M = 1) measurement and reporting |
| 27-3-2 | DL PRS measurement outside MG within PRS processing priority window | Supported DL PRS processing types determining that the DL PRS measurements are prioritized over other DL signals/channels outside MG within PRS processing priority window |
| 27-3-3 | DL PRS processing capability outside MG – buffering capability | TBD |
| 27-4-1a | Support of hard LOS/NLOS indication for UE-assisted DL-TDOA positioning | Support of hard LOS/NLOS indicator values reporting (i.e., 0 or 1) for DL-TDOA positioning |
| 27-4-1b | Support of hard LOS/NLOS indication for UE-assisted DL-AOD positioning | Support of hard LOS/NLOS indicator values reporting (i.e., 0 or 1) for DL-AOD positioning |
| 27-4-1c | Support of hard LOS/NLOS indication for UE-assisted Multi-RTT positioning | Support of hard LOS/NLOS indicator values reporting (i.e., 0 or 1) for Multi-RTT positioning |
| 27-4-1d | Support of soft LOS/NLOS indication for UE-assisted DL-TDOA positioning | Support of soft LOS/NLOS indicator values reporting (i.e., 0.1, 0.2, …, 1) for DL-TDOA positioning |
| 27-4-1e | Support of soft LOS/NLOS indication for UE-assisted DL-AOD positioning | Support of soft LOS/NLOS indicator values reporting (i.e., 0.1, 0.2, …, 1) for DL-AOD positioning |
| 27-4-1f | Support of soft LOS/NLOS indication for UE-assisted Multi-RTT positioning | Support of soft LOS/NLOS indicator values reporting (i.e., 0.1, 0.2, …, 1) for Multi-RTT positioning |
| 27-5-1 | Support of UE-initiated on-demand DL PRS request | Support of UE-initiated on-demand DL PRS request |
| 27-6 | DL PRS processing capabilities in RRC inactive state | 1) DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2) Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3) Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |
| 27-7 | Maximum number of measurement instances which can be included in a single measurement report | --- |
| 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP TxTEG for UE-based positioning |
| 27-9 | Support of lower Rx beam sweeping factor | - Support of the lower Rx beam sweeping factor than 8 for FR2 - Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |
| 27-10 | Support of UL MAC CE based MG activation request | Support of using UL MAC CE to request measurement gap |
| 27-11 | Support of DL MAC CE based MG activation | 1) Support of pre-configuration of MGs in RRC 2) Support of using DL MAC CE to activate the MG |
| 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator - LOS/NLOS indicator type: {softValue, hardValue, both} - LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |
| 27-13 | Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted DL-TDOA | - Support of DL PRS-RSRPP measurement and reporting per additional path for DL-TDOA positioning - Support of relative timing reporting per additional path with respect to the first detected path - Maximum number of measured and reported additional paths (N ≤ 8) |
| 27-13a | Support of the first path DL PRS-RSRPP reporting for UE-assisted DL-TDOA positioning | Support of first path DL PRS-RSRPP measurement and reporting for DL-TDOA positioning |
| 27-14 | Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted Multi-RTT | - Support of DL PRS-RSRPP measurement and reporting per additional path for Multi-RTT positioning - Support of relative timing reporting per additional path with respect to the first detected path - Maximum number of measured and reported additional paths (N ≤ 8) |
| 27-14a | Support of the first path DL PRS-RSRPP reporting for UE-assisted Multi-RTT positioning | Support of first path DL PRS-RSRPP measurement and reporting for Multi-RTT positioning |
| 27-15a | Support of SRS for positioning transmission inside initial UL BWP in RRC_INACTIVE state | SRS for positioning allocated within initial UL BWP with the same CP type and SCS settings TBD SRS resources |
| 27-15b | Support of SRS for positioning transmission outside initial UL BWP in RRC_INACTIVE state | SRS for positioning is configured with the CP type and SCS, bandwidth and frequency start point TBD SRS resources |
| 27-16 | OLPC for SRS for positioning in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |
| 27-17 | Support of [PRS measurement in RRC_INACTIVE] | --- |
| 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follow the same as the RRC_CONNECTED state for DL-TDOA. No new LPP signaling defined. |
| 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follow the same as the RRC_CONNECTED state for DL-AoD. No new LPP signaling defined. |
| 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | - Support of PRS measurement in RRC_INACTIVE state for Multi-RTT - Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follow the same as the RRC_CONNECTED state for Multi-RTT. No new LPP signaling defined. |
| 27-18d | Support of SS-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning | Support of SS-RSRP PRM measurements in RRC_INACTIVE state for NR E-CID positioning No new LPP signaling defined. |
| 27-18e | Support of SS-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning | Support of SS-RSRQ PRM measurements in RRC_INACTIVE state for NR E-CID positioning No new LPP signaling defined. |
| 27-18f | Support of CSI-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning | Support of CSI-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning No new LPP signaling defined. |
| 27-18g | Support of CSI-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning | Support of CSI-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning No new LPP signaling defined. |
| 27-18h | Support of simultaneous DL-AoD and DL-TDOA processing in RRC_INACTIVE state | Support of simultaneous DL-AoD and DL-TDOA processing in RRC_INACTIVE state No new LPP signaling defined. |
| 27-18i | Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state | Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state No new LPP signaling defined. |
| 27-19 | Spatial relation for SRS for positioning in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |

  - Consider FG components provided in Table 3 as an input for discussion on NR Positioning FG components

# Conclusions

In this contribution, we have provided our initial views on UE features for Rel.17 WI on NR positioning enhancements. In summary, we have following proposals:

  - Split FG 27-1-1 into two FGs:

    - 27-1-1a Support of UE-RxTEGs for UE-assisted DL TDOA

    - 27-1-1b Support of UE-RxTEGs for UE-assisted Multi-RTT positioning

  - Rename FG 27-1-2 to:

    - 27-1-2a Support of UE-TxTEGs reporting in RRC for UL TDOA

  - Rename FG 27-1-2a to:

    - 27-1-1b Support of UE TxTEGs reporting in LPP for Multi-RTT positioning

  - Support FG 27-1-4a only if LPP signaling for request of simultaneous measurement using different RxTEGs for the same DL PRS resource is introduced

  - Introduce new FG 27-2-3 for DL-AoD positioning enhancements:

    - Support of assistance information for DL-AoD measurements that includes following components

      - Support of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s)

      - Support of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s)

  - Introduce new FG 27-2-4 for DL-AoD positioning enhancements:

    - 27-2-4: Support of TRP beam/antenna information for UE-based DL-AoD positioning

  - Introduce new FG 27-2-5 for DL-AoD positioning enhancements:

    - 27-2-5: Support of the first path DL PRS RSRPP and DL PRS RSRP reporting for DL-AOD in the same report

  - Introduce two new FGs 27-2-6 and 27-2-7 for DL-AoD positioning enhancements:

    - 27-2-6: Support of prioritization information for DL PRS reporting (subset of DL PRS resources) for DL-AOD positioning

    - 27-2-7: Support of the boresight direction indication for the DL PRS resource for DL-AOD positioning

  - Support FG 27-3-1 and define new FG name

    - 27-3-1: Support of single DL PRS period measurement (M = 1) and reporting

  - Support FG 27-3-2 and slightly modify its name

    - 27-3-2: DL PRS measurement outside MG within PRS processing priority window

  - Delete FG 27-7: Maximum number of measurement instances which can be included in a single measurement report

  - Split FG 27-4-1 to two FGs:

    - 27-4-1a: Support of hard LOS/NLOS indication for UE-assisted DL-TDOA positioning

    - 27-4-1b: Support of hard LOS/NLOS indication for UE-assisted DL-AOD positioning

    - 27-4-1c: Support of hard LOS/NLOS indication for UE-assisted Multi-RTT positioning

    - 27-4-1d: Support of soft LOS/NLOS indication for UE-assisted DL-TDOA positioning

    - 27-4-1e: Support of soft LOS/NLOS indication for UE-assisted DL-AOD positioning

    - 27-4-1f: Support of soft LOS/NLOS indication for UE-assisted Multi-RTT positioning

  - Rename FG 27-13 and clarify it components of the FG 27-13:

    - 27-13: Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted DL-TDOA:

      - Reporting of up to N = 8 DL PRS-RSRPP values together with relative timing measurement (relative to the first path timing) as part of DL-TDOA reporting

  - Support FG 27-14:

    - 27-14: Support of DL PRS-RSRPP and relative timing for additional path reporting for UE-assisted Multi-RTT:

      - Reporting of up to N = 8 DL PRS-RSRPP values together with relative timing measurement (relative to the first path timing) as part of Multi-RTT reporting

  - Introduce new FG 27-13a for DL-TDOA positioning enhancements

    - 27-13a: Support of the first path DL PRS-RSRPP reporting for UE-assisted DL-TDOA positioning

  - Introduce new FG 27-14a for Multi-RTT positioning enhancements

    - 27-14a: Support of the first path DL PRS-RSRPP reporting for UE-assisted Multi-RTT positioning

Introduce additional values {1ms, 2ms, 4 ms} for parameter T defined for UE DL PRS processing capabilities within MG

Add new values for R17 UEs for Rel-16 FG 13-1 Common DL PRS Processing Capability

  - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.

  - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

  - Rename FG 27-5-1 to:

    - 27-5-1: Support of UE-initiated on-demand DL PRS request

  - Split FG 27-15 to two FGs:

    - 27-15a: Support of SRS for positioning transmission inside initial UL BWP in RRC_INACTIVE state

    - 27-15b: Support of SRS for positioning transmission outside initial UL BWP in RRC_INACTIVE state

  - Further discuss whether/how to reuse Rel.16 FGs for SRS for positioning OLPC (FGs 13-9/9a/9b/9e/9f) and spatial relation (FGs 13-10/10a/10b/10d/10e/10f) or define new FGs for support of SRS for positioning in RRC_INACTIVE state (considering Rel.16 FGs defined for RRC_CONNECTED state), i.e.:

    - OLPC

      - FG 27-16-1: OLPC for SRS for positioning based on DL PRS from serving cell

      - FG 27-16-2: OLPC for SRS for positioning based on DL PRS from neighboring cells

      - FG 27-16-3: OLPC for SRS for positioning based on SSB from serving cells

      - FG 27-16-4: Pathloss estimate maintenance for serving cell

      - FG 27-16-5: Pathloss estimate maintenance across all cells

    - Spatial relation

      - FG 27-19-1: Spatial relation for SRS for positioning based on SSB from serving cell

      - FG 27-19-2: Spatial relation for SRS for positioning based on CSI-RS from serving cell

      - FG 27-19-3: Spatial relation for SRS for positioning based on DL PRS from serving cell

      - FG 27-19-4: Spatial relation for SRS for positioning based on SSB from non-serving cells

      - FG 27-19-5: Spatial relation for SRS for positioning based on DL PRS from non-serving cells

      - FG 27-19-6: Spatial relation maintenance

  - Select between two options:

    - Option 1: Define FGs associated with NR positioning measurements in RRC_INACTIVE state without introduction of UE LPP and RRC capability signaling

      - FG 27-18a: Support of DL PRS measurement in RRC_INACTIVE state for DL-AoD

      - FG 27-18b: Support of DL PRS measurement in RRC_INACTIVE state for DL-TDOA

      - FG 27-18c: Support of UE Rx-Tx measurement in RRC_INACTIVE state for Multi-RTT

      - FG 27-18d: Support of SS-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18e: Support of SS-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18f: Support of CSI-RSRP RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18g: Support of CSI-RSRQ RRM measurements in RRC_INACTIVE state for NR E-CID positioning

      - FG 27-18h: Support of simultaneous DL-AoD and DL-TDOA processing in RRC_INACTIVE state

      - FG 27-18i: Support of simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state

    - Option 2: Discuss UE capability for UEs in RRC_INACTIVE state directly in RAN2 as it is a RAN2-led objective

  - The Rel.16 NR positioning FGs (13-2/2a/2b, 13-3/3a/3b, 13-4/4a/4b) for DL PRS resources and different NR positioning techniques are reused and defined as pre-requisites for FGs associated with NR positioning measurements by RRC_INACTIVE UEs

  - For NR Positioning enhancements, adopt UE FGs as defined in Table 2

  - Consider FG components provided in Table 3 as an input for discussion on NR Positioning FG components

# References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e, Moderators (AT&T, NTT DOCOMO, INC.), November 2021

- R1-2111810, Summary of UE features for NR positioning enhancements, Moderator (AT&T), November 2021

- R1-2102006, Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e, Moderators (AT&T, NTT DOCOMO, INC.)

# Annex A: UE features for NR positioning enhancements from [R1-2112902] – After RAN1#107-e

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | [per band or FS] | n/a | n/a | n/a | The candidate values are {[1,] 2,[ 3,] 4, 6, 8[, 12, 16, 24, 32]} Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | [13-4, ]13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: per band or per FS | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8} [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | per band | n/a | n/a | n/a | The candidate values are {[1, ]2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first path for DL-AoD | 1.) Support of [measuring and reporting the] PRS RSRP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRP per TRP | [13-2 or 13-3, 13-4, 13-5, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | Yes | n/a | The candidate values are {[12, ]16, 24[, 32, 64]} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set ] | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-7 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | [1. Support of TOA reporting for more than 2 additional paths.] 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} 4. FFS: Applicability for initial BWP OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-17 | Support of [PRS measurement in RRC_INACTIVE] | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |