3GPP TSG RAN WG1 #107bis-e	R1-2200506

e-Meeting, January 17th – 25th, 2022

Source:	Sharp

Title:	UE feature for Rel-17 coverage enhancement

Agenda Item:	8.15.8

Document for:	Discussion and Decision

Background

In [1], the updated list of UE features for Rel-17 has been provided. In this contribution, we discuss UE features for Rel-17 coverage enhancement.

Discussion

  - Enhancements on PUSCH repetition type A

FG 30-1 and FG 30-1a

The updated Rel-17 UE feature list captures two FGs for the increased maximum number of PUSCH Type A repetitions, one is FG 30-1 and the other is FG 30-1a. The description on FG 30-1a still has two FFS bullet points.

| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. |
| --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 |

Regarding the potential merging of FG 30-1a to FG 30-1, both features can be considered as the extensions of Rel-16 dynamic repetition factor indication, and there is no separation in terms of the feature group between Rel-16 repetitions with DC-PUSCH and Type-2 CG-PUSCH. Therefore, we propose merging FG 30-1a into FG 30-1.

Proposal 1:

- FG 30-1a is merged into FG 30-1.

As for the repK-r17, it is not an enhancement of Rel-16 feature, and it does not require dynamic change of repetition factor. Therefore, it is preferable to have a separate FG.

Proposal 2:

- Introduce the following FG 30-1b “Increased maximum number of configured grant PUSCH Type A repetitions”.

| 30. NR_cov_enh | 30-1b | Increased maximum number of configured grant PUSCH Type A repetitions | K = 1, 2, 4, 8, 12, 16, 24, 32 times repetitions. The number of repetitions is indicated by repK-r17. | 5-14 or 5-16 | Yes | N/A | UE does not support more than 16 repetitions with repK-r17 for Type 1 and Type 2 configured grant PUSCH. | Per UE | No | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 30-2 and FG 30-2a

The updated Rel-17 UE feature list captures two FGs for the available slot counting of PUSCH Type A repetitions, one is FG 30-2 and the other is FG 30-2a. The description on FG 30-2a still has an FFS bullet point saying that “FFS whether to merge with FG 30-2”.

| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. |
| --- | --- | --- | --- |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 |

We do not see much difference between them in terms of complexity, because the definition of the available slots is the same for both DG-PUSCH repetitions and CG-PUSCH repetitions. Therefore, for simplicity, our preference is to merge FG 30-2a into FG 30-2.

Proposal 3:

- FG 30-2a is merged into FG 30-2.

  - TB processing over multi-slot PUSCH

FG 30-3 and FG 30-3a

The updated Rel-17 UE feature list captures two FGs for TBoMS, one is FG 30-3 (i.e.  regular TBoMS) and the other is FG 30-3a (i.e.  repetitions of TBoMS). The description on FG 30-3 still has one FFS bullet point saying “FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG”. FG 30-3a also has an FFS point “FFS whether to merge with FG 30-3”.

| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG |
| --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 |

We do not see much difference between DG-TBoMS and CG-TBoMS in terms of complexity, because the only difference is how the UL grant is provided. Therefore, for simplicity, our preference is to not split FG 30-3 into 2 separate FGs.

Proposal 4:

- Keep FG 30-3 as is, i.e., FG 30-3 covers both DG and CG.

As for whether to merge FG 30-3a into FG 30-3 or not, We do not have a strong preference on it. But, the TBoMS with repetitions is not a simple combination of the TBoMS feature and the PUSCH-repetition feature, because the TBoMS itself borrows the Type A repetition framework. Therefore, we are fine with having FG 30-3a separately from FG 30-3.

Proposal 5:

- Keep FG 30-3a as is, i.e., FG 30-3a is not merged to FG 30-3.

  - Joint channel estimation for PUSCH

FG 30-4

The updated Rel-17 UE feature list captures several sets of FGs for DM-RS bundling which still have FFS points. The first one is the maximum duration for DM-RS bundling.

| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |
| --- | --- | --- | --- |

Regarding the dependency on modulation order and back-to-back vs. non-back-to-back, RAN1 has not yet received the formal response from RAN4. Therefore, it is better to postpone the discussion until RAN1 receives the reply.

FG 30-4a, FG 30-4b, FG 30-4c, and FG 30-4d

The second set of FGs is about whether to support DM-RS bundling.

| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |
| --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions FFS whether to merge with FGs 30-4a/4b/4c FFS whether to split to back-to-back transmission and non-back-to-back transmission |

For PUSCH repetitions Type A and Type B and TBoMS, we see no functional difference in terms of DM-RS bundling, and as such a single FG is sufficient. In other words, FG 30-4a, FG 30-4b and FG 30-4c should be merged into one FG. On the other hand, it would be better to define a FG for DM-RS bundling for PUCCH separately from the one for PUSCH, because there could be some functional difference. For example, whether the configuration of DM-RS bundling for PUCCH is per-BWP or per-PUCCH resource/format is still under discussion, while the configuration of DM-RS bundling for PUSCH is per-BWP. To be safer, we propose keeping FG 30-4d separate from 30-4a/4b/4c.

Proposal 6:

- FG 30-4b and FG 30-4c should be merged to FG 30-4a.

- Keep FG 30-4d separately from FG 30-4a.

FG 30-4e and FG 30-4f

The last set of FGs for DM-RS bundling is about inter-slot frequency hopping.

| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH FFS whether to merge with FG 30-4f |
| --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling FFS whether to merge with FG 30-4e |

As discussed above, it is better to have separate FGs for PUSCH and PUCCH, at least for now. Therefore, we propose keeping FG 30-4e and FG 30-4f separate.

Proposal 7:

- Keep FG 30-4e and FG 30-4f separate.

  - PUCCH enhancements

FG 30-5

The updated Rel-17 UE feature list captures one FG for PUCCH enhancement. The description on FG 30-5 still has one FFS bullet point saying “FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4”.

| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 |
| --- | --- | --- | --- |

In our view, there is no need to make separate FGs for formats 0/2 and formats 1/3/4.

Proposal 8:

- Keep FG 30-5 as is, i.e., FG 30-5 covers all the PUCCH formats 0/1/2/3/4.

  - Type A PUSCH repetitions for Msg3

FG 30-6

The updated Rel-17 UE feature list captures one FG for this sub-agenda. The descriptions on FG 30-6 have been stable, and the only thing we should decide on would be its “Mandatory/Optional” column. Currently, the column is filled with “[Optional with capability signalling]”.

| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |
| --- | --- | --- | --- |

In our view, the square bracket should be removed, i.e., this feature should be Optional with capability signalling.

Proposal 9:

- FG 30-6 is Optional with capability signalling.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- FG 30-1a is merged into FG 30-1.

Proposal 2:

- Introduce the following FG 30-1b “Increased maximum number of configured grant PUSCH Type A repetitions”.

Proposal 3:

- FG 30-2a is merged into FG 30-2.

Proposal 4:

- Keep FG 30-3 as is, i.e., FG 30-3 covers both DG and CG.

Proposal 5:

- Keep FG 30-3a as is, i.e., FG 30-3a is not merged to FG 30-3.

Proposal 6:

- FG 30-4b and FG 30-4c should be merged to FG 30-4a.

- Keep FG 30-4d separately from FG 30-4a.

Proposal 7:

- Keep FG 30-4e and FG 30-4f separate.

Proposal 8:

- Keep FG 30-5 as is, i.e., FG 30-5 covers all the PUCCH formats 0/1/2/3/4.

Proposal 9:

- FG 30-6 is Optional with capability signalling.

References

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, November 2021