3GPP TSG RAN WG1 Meeting #107bis-e	R1-2200534

e-Meeting, January 17th – 25th, 2022

Agenda item:	8.7.2

Source: 	Lenovo, Motorola Mobility

Title: 	Enhanced DCI based power saving adaptation

Document for:	Discussion

# Introduction

During RAN1 #107-e meeting, RAN1 made the following agreements for enhanced DCI-based power saving adaptation for connected mode UE [1]:

Agreement

- The value of the timer in slots for monitoring PDCCH in the active DL BWP of the serving cell before moving to the default search space group is

  - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

  - {1,2,3,…,40, 60, 80, 100, 100,160,200} for 30 kHz SCS,

  - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

  - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

Agreement

- For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported

  - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors

    - If M = 1, then

      - ‘00’ is Beh 2

      - ‘01’ is Beh 2A

      - ‘10’ is Beh 1A (stay in current SSSG)

      - [‘11’ is reserved]

    - If M = 2, then

      - ‘00’ is Beh 2

      - ‘01’ is Beh 2A

      - ‘10’ is Beh 1A for duration T1 (stay in current SSSG)

      - ‘11’ is Beh 1A for duration T2 (stay in current SSSG)

- Note: for  ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping )

- Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping.

- FFS SSSG switching timer behavior, and timer value related to the value of T1/T2

Agreement

If a UE is provided with a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

- resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot

  - Alt 2a: for the Type3-PDCCH CSS set or the USS set with group index of either 1 or 2

  - Alt 2b: for the Type3-PDCCH CSS set or the USS set

  - Alt 2c: with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI

- otherwise, decrease the timer value by one after each slot.

- FFS: When the timer expires in a slot

Agreement

- For value X in Beh 1A, candidate skipping values are

  - Up to [100ms] length is supported,

    - The X is configured and indicated in the unit of slot.

      - Working assumption for candidate values for X

        - {1,2,3,…,20,30, 40, 50, 60, 80, 100} for 15 kHz SCS,

        - {1,2,3,…,40, 60, 80, 100, 120,160,200} for 30 kHz SCS,

        - {1,2,3,…,80, 120, 160, 200, 240, 320,400} for 60kHz SCS,

        - {1,2,3,…,160, 240, 320,400, 480, 640,800} for 120kHz SCS

  - FFS: Equal to or longer than the applicable minimum scheduling offset

  - FFS: additional symbol level / PDCCH monitoring period level skipping duration

In this contribution, we discuss remaining details for search space set group (SSSG) switching and dynamic PDCCH skipping.

# Discussion

## Timer based SSSG switching

If a UE is configured with three SSSGs in a given bandwidth part (BWP), one Rel-17 search space switch timer value (i.e. searchSpaceSwitchTimer-r17) can be configured for both SSSG1 and SSSG2 of the BWP. Further, at most one search space switch timer is running at a given time. That is, a UE can reset a search space switch timer after a slot of an active DL BWP of a serving cell when the UE detects a DCI format in a PDCCH reception in the slot for a Type3-PDCCH common search space (CSS) set(s) or UE-specific search space (USS) set(s) with group index of 1 or 2.

When the UE misses a DCI format indicating switching between a SSSG 1 and a SSSG 2 or indicating switching between a default SSSG (i.e. SSSG 0) and a SSSG 1 (or a SSSG 2), different search space switch timer values configured for SSSG 1 and SSSG 2 could cause more ambiguity due to increased number of hypotheses regarding potential SSSG switching instances. Since SSSG configurations are BWP-specific, a Rel-17 search space switch timer value can be configured per BWP to provide more flexibility compared to Rel-16.

Proposal 1: At most one search space switch timer is running at a given time.

Observation 1: Different search space switch timer values for SSSG 1 and SSSG 2 could increase the number of hypotheses on potential SSSG switching instances and could result in more ambiguity, when one or multiple DCI indicating SSSG switching are missed.

Proposal 2: When 2 or 3 SSSGs configured in a BWP, one search space switch timer value can be configured per BWP.

When a search space switch timer expires in a slot,

- if the slot is in an indicated PDCCH skipping duration, UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 after the indicated PDCCH skipping duration ends;

- otherwise, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 at the beginning of the first slot that is at least  symbols after the slot.

Proposal 3: If a search space switch timer expires in a slot that is in an indicated PDCCH skipping duration, UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 after the indicated PDCCH skipping duration ends.

## PDCCH skipping

When a UE receives an indication of PDCCH skipping, the UE may have to delay initiating PDCCH skipping due to on-going transport block (TB) transmissions and/or receptions. For example, if a retransmission related timer is running, the UE may have to continue monitoring PDCCH until the retransmission related timer expires. That is, upon detecting a DCI format indicating PDCCH skipping, the UE stops monitoring PDCCH in relevant UE-specific search spaces and Type-3 common search spaces,

- for DL DCI format(s), upon expiration of drx-RetransmissionTimerDL if drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL is running, and

- for UL DCI format(s), upon expiration of drx-RetransmissionTimerUL if drx-HARQ-RTT-TimerUL or drx-RetransmissionTimerUL is running.

Proposal 4: Upon detecting a DCI format indicating PDCCH skipping, UE stops monitoring PDCCH in UE-specific search spaces and Type-3 common search spaces,

- for DL DCI format(s), upon expiration of drx-RetransmissionTimerDL if drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL is running, and

- for UL DCI format(s), upon expiration of drx-RetransmissionTimerUL if drx-HARQ-RTT-TimerUL or drx-RetransmissionTimerUL is running.

# Conclusion

In summary, we observe and propose the followings for Rel-17 NR UE power saving enhancement for connected mode UEs:

Proposal 1: At most one search space switch timer is running at a given time.

Observation 1: Different search space switch timer values for SSSG 1 and SSSG 2 could increase the number of hypotheses on potential SSSG switching instances and could result in more ambiguity, when one or multiple DCI indicating SSSG switching are missed.

Proposal 2: When 2 or 3 SSSGs configured in a BWP, one search space switch timer value can be configured per BWP.

Proposal 3: If a search space switch timer expires in a slot that is in an indicated PDCCH skipping duration, UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 after the indicated PDCCH skipping duration ends.

Proposal 4: Upon detecting a DCI format indicating PDCCH skipping, UE stops monitoring PDCCH in UE-specific search spaces and Type-3 common search spaces,

- for DL DCI format(s), upon expiration of drx-RetransmissionTimerDL if drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL is running, and

- for UL DCI format(s), upon expiration of drx-RetransmissionTimerUL if drx-HARQ-RTT-TimerUL or drx-RetransmissionTimerUL is running.

Text proposal for TS 38.213 is shown below:

-------------------------- Start of Text Proposal for TS 38.213 ----------------------------------------------------------------

## 10.4	Search space set group switching and skipping of PDCCH monitoring

*** Unchanged text is omitted ***

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

-	decrements the timer after a slot of an active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot for Type3-PDCCH CSS sets or USS sets with group index 1 or 2

-	resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot for Type3-PDCCH CSS sets or USS sets with group index 1 or 2

When the timer expires in a slot,

- if the slot is in an indicated PDCCH skipping duration, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 after the indicated PDCCH skipping duration ends;

- otherwise, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0 at the beginning of the first slot that is at least  symbols after the slot.

*** Unchanged text is omitted ***

-------------------------- End of Text Proposal for TS 38.213 -------------------------------------------------------------

# References

- “RAN1 Chairman’s Notes”, RAN1 107-e meeting, November 11th – 19th, 2021.