3GPP TSG RAN WG1 #107-bis-e			R1-2200538

e-Meeting, Jan. 17th – 25th, 2022

Agenda item:	8.15.7

Source: 	MediaTek Inc.

Title: 	On UE features for UE power saving enhancements

Document for:	Discussion and decision

- Introduction

This paper provides our views on Rel-17 UE features in R1-2112902 (Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e) for UE power saving enhancements. Detailed proposals are described in the following sections.

- Views on UE features for idle mode UE power saving enhancements

In RAN1 #107e, RAN1 agrees the following draft UE features for idle mode UE power saving enhancements:

Note: yellow highlight above means FFS

For “29-1 Paging enhancement“, the “components” cell includes two functions, which are

- Support paging early indication

- Support UE subgroup indication

It may give the impression that UE needs to support both functions to support 29-1.

Observation 1: For the current RAN1 UE feature table (R1-2112902) “29-1 Paging enhancement”, the “components” cell includes two functions, which are

- Support paging early indication

- Support UE subgroup indication

It may give the impression that UE needs to support both functions to support 29-1.

However, for some UEs which do not support UE subgrouping, it can still obtain power saving gain by supporting paging early indication.

Observation 2: For some UEs which do not support UE subgrouping, it can still obtain power saving gain by supporting paging early indication.

Hence, we have the following proposal:

Proposal 1: To allow UEs which do not support UE subgrouping to still support paging early indication (FG 29-1), modify the "Components" to be

- "Support receiving and parsing DCI format 2_7"

For FG 29-2 "TRS resources for idle/inactive UEs", the current contents for “Consequence if the feature is not supported by the UE” is “Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode”, which may be too detailed about UE implementation.

Observation 3: For FG 29-2 "TRS resources for idle/inactive UEs", the current contents for “Consequence if the feature is not supported by the UE” is “Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode”, which may be too detailed about UE implementation.

Therefore, we have the following proposal:

Proposal 2: For FG 29-2 "TRS resources for idle/inactive UEs", modify the contents of “Consequence if the feature is not supported by the UE” to be

- “UE can not receive TRS resources for idle/inactive mode”

- Views on UE features for connected mode UE power saving enhancements

In RAN1 #107e, RAN1 agrees the following draft UE features for connected mode UE power saving enhancements

For [29-3d], the interaction between search group switching and PDCCH skipping can be complincated and may require additional spec definition. Therefore, the bracket should be removed.

Observation 4: For [29-3d], the interaction between search group switching and PDCCH skipping can be complincated and may require additional spec definition. Therefore, the bracket should be removed to introduce a separate feature.

Proposal 4: Remove the bracket of [29-3d]. For “Prerequisite feature groups”, modify the typo “29-2b” to “29-3b”.

For 29-3b and [29-3c], they should be used only when PDCCH skipping is not configured.

Observation 5: For 29-3b and [29-3c], they should be used only when PDCCH skipping is not configured.

Proposal 5: Remove the bracket of [29-3c]. For 29-3b and 29-3c, add the following sentence to “Components”:

- if PDCCHSkippingDurationList is not configured

# Proposed changes on current RAN1 UE features table

# NR_UE_pow_sav_enh (modified contents in red text)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication Support receiving and parsing DCI format 2_7 |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE can not receive TRS resources for idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching, if PDCCHSkippingDurationList is not configured FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-23b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.