3GPP TSG RAN WG1 #107bis-e		    R1-2200550

e-Meeting, January 17th – 25th, 2022

Source: 	MediaTek Inc.

Title:	Remaining issues on improve multicast reliability for RRC_CONNECTED UEs

Agenda item:	8.12.2

Document for:	Discussion & Decision

## Introduction

In last previous e-meeting [1], the mechanisms on improve the multicast reliability was discussed and the some agreements were achieved at the end of meeting. However, there are still some remaining issues that need to be further discussed to finalize the Rel-17 MBS. In this contribution, we will further discuss the detailed design about the remaining issue to improve reliability for RRC_CONNECTED UEs.

## Discussion

  - Configuration/selection of HARQ feedback mode

The ACK/NACK and NACK-only based HARQ-ACK feedback mode for multicast were agreed in RAN1#104-e, RAN1#104bis-e meeting respectively.

Agreement in RAN1#104-e meeting: For RRC_CONNECTED UEs receiving multicast, support the following: - ACK/NACK based HARQ-ACK feedback for multicast, - It is up to network to configure orthogonal PUCCH resources among UEs within the same group. - FFS: NACK-only based HARQ-ACK feedback for multicast, - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group. - FFS details. Agreement in RAN1#104bis-e meeting: Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

As agreed above, both ACK/NACK and NACK-only based HARQ feedback mode are supported for Rel-17 NR multicast service. It will incur one critical question that if both are supported, how to configure or select of ACK/NACK-based and NACK-only based HARQ-ACK feedback? Some companies may think NACK-only based HARQ feedback mode can be used for multiple UEs in one MBS group, which can reduce the PUCCH overhead, and ACK/NACK based HARQ feedback can be used for small UEs in one MBS group. However, there is not a specific criteria to measure how many UEs can be regarded as a “multiple UEs” MBS group and how little UEs can be regarded as a “little UEs” MBS group. Maybe we can set a criteria to measure the UE number, e.g., when UE number more than available PUCCH resource for multicast service, it can regard this MBS group as “multiple UEs” type and use NACK-only based HARQ feedback mode. However, we think it is too restrictive to network scheduling, especially for some UEs newly join or leave the MBS group, gNB need to reschedule the HARQ feedback mechanism. We suggest network can flexibly schedule the HARQ feedback mode based on real transmission condition. The concrete HARQ feedback mode can be indicated dynamically by DCI field, e.g, add a new field “HARQ feedback option” within group common DCI.

Proposal 1: The HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

  - Enable/disable HARQ-ACK feedback

Regarding enable/disable HARQ-ACK feedback mode for multicast, the following agreements were achieved in last RAN1 meeting:

Agreement: Confirm the WA made in RAN1#106-e meeting regarding enabling/disabling HARQ-ACK feedback. Agreement (WA made in RAN1#106-e): Update the WA made in RAN1#105-e meeting regarding enabling/disabling HARQ-ACK feedback as follows: Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH. - UE capability for enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback is introduced and FFS details. Note: It is up to network implementation to avoid any potential HARQ ACK mismatch between different UEs in the same multicast group.

Regarding whether/how to allow UE not to react to the DCI signaling, some companies proposed that some UEs within multicast group have low reliability without HARQ-ACK feedback and other UEs within the same multicast group have higher reliability with HARQ-ACK feedback. However, from our understanding, the HARQ feedback mechanism is the same for all UEs within in the same multicast group. Otherwise, the scheduling processing is more complicated and is not preferred. Besides, the service priority of the same group cast services is the same for the UEs within the same group. Therefore, it is better to define the same HARQ-ACK feedback behaviour for all the UEs in the same group.

Proposal 2: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

As agreed, if RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. In this scenario, whether enabling or disabling the feedback as the default mode is FFS. From our understanding, HARQ ACK is an enhancement for NR MBS, and no HARQ feedback in LTE SC-PTM. Thus, if the RRC signaling doesn’t configure the function, disabling the HARQ feedback can be regarded as a default mode. If the MBS services need the high reliability quality, the RRC signaling can enable the HARQ ACK feedback mechanism.

Proposal 3: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

Regarding the DCI enabling/disabling, the following agreements were achieved in last RAN1 meeting:

Agreement: For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 1_1 scrambled by G-RNTI - For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately. Agreement: For the DCI format including the field of “enabling/disabling HARQ-ACK feedback indication” for multicast scheduling, the field is a new field with 1 bit.

It can be seen that a new field “enabling/disabling HARQ-ACK feedback indication” within multicast DCI format 4_2 (a.k.a, DCI format 1_1 scrambled by G-RNTI) was introduced to indicate the HARQ-ACK enabling/disabling. However, the multicast DCI format 4_1 (a.k.a, the DCI format 1_0 with G-RNTI) does not have any conclusion yet. Considering the DCI format 4_1 with G-RNTI is a new DCI format and there are at least 3 reserved bits, we think it does not need to keep all the DCI 4_1 fields with same as that of DCI 1_0 with C-RNTI. Instead, it is better to align the similar HARQ-ACK behaviour to multicast DCI format. Therefore, we suggest that he “enabling/disabling HARQ-ACK feedback indication” is also included in multicast DCI format 4_1.

Proposal 4: A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for DCI format 4_1.

  - Multicast SPS related HARQ-ACK feedback

Regarding the multicast SPS related HARQ-ACK feedback, the following agreements were achieved in last meeting:

Agreement: For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. Agreement: HARQ-ACK feedback option is configured per G-CS-RNTI. Agreement: For group-common DCI indicating whether ACK/NACK based HARQ-ACK feedback is enabled/disabled, the “enabling/disabling HARQ-ACK feedback indication” is included in DCI format 1_1 scrambled by G-RNTI - For DCI format 1_1 scrambled by G-CS-RNTI, it is discussed separately. Agreement: For multicast SPS PDSCH without PDCCH scheduling, support the following: - RRC signalling configures the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation. - The configuration is per G-CS-RNTI. - Separate UE capability is needed from that for dynamic scheduling for multicast. - RRC signalling configures directly whether the HARQ-ACK feedback is enabled or disabled. - The configuration is per G-CS-RNTI.

Regarding the HARQ-ACK enabling/disabling for multicast DCI with G-CS-RNTI, it has been agreed that RRC signalling can configure the presence of the field “enabling/disabling HARQ-ACK feedback indication” in the group-common DCI for multicast SPS activation, and a “enabling/disabling HARQ-ACK feedback indication” field is natural to be defined for multicast DCI format with G-CS-RNTI.

Proposal 5: A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for multicast DCI format scrambled by G-CS-RNTI.

Although the HARQ-ACK can be disabled for multicast SPS by RRC signalling or DCI dynamic indication, the multicast SPS activation/deactivation cannot be disabled because gNB needs to know whether the UE activate or de-active successfully via HARQ-ACK feedback.

Proposal 6: For multicast SPS activation/activation, disabling the HARQ-ACK feedback is not supported.

  - Retransmission combination with HARQ operation

Regarding the retransmission scheme, some agreements were achieved as following in pervious RAN1 e-meeting:

Agreements: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1. Agreements: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

According to the agreements, it can be seen that PTM scheme 1 can be used for initial MBS transmission and PTM schem1 or PTP can be configured for MBS retransmission. However, which retransmission scheme will be supported for a specific HARQ-ACK feedback option is still pending in previous meeting.

For ACK/NACK based HARQ feedback mode, each UE within one MBS group is allocated orthogonal PUCCH resource and gNB can differentiate which UE feed back ACK or NACK. Thus, the gNB can flexible choose PTM scheme 1 or PTP for retransmission based on the feedback status. E.g., if many UE report NACK to gNB, the PTM scheme 1 is a recommended way for retransmission. If only little UE feed back NACK to gNB, it can use PTP for retransmission using a narrower beam toward the UE and offer higher robustness.

For NACK-only based HARQ feedback mode, all UEs within one MBS group will use the same PUCCH resource for HARQ feedback, and gNB cannot differentiate which UE feedback NACK. Thus, PTP retransmission targeting for specific UE is not supported and only PTM scheme 1 can be scheduled for retransmission.

Proposal 7: PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

Proposal 8: PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

## Conclusion

In this contribution, it further discusses the multicast HARQ reliability issues for RRC_CONNECTED UEs with following proposals:

Proposal 1: The HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

Proposal 2: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

Proposal 3: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

Proposal 4: A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for DCI format 4_1.

Proposal 5: A new field of “enabling/disabling HARQ-ACK feedback indication” should be defined for multicast DCI format scrambled by G-CS-RNTI.

Proposal 6: For multicast SPS activation/activation, disabling the HARQ-ACK feedback is not supported.

Proposal 7: PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

Proposal 8: PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

## Reference

- RAN1 Chairman’s Notes, RAN1 #107-e, November 2021.