3GPP TSG RAN WG1 #107bis-e	                                                                                   R1-2200563

e-Meeting, January 17th – 25th, 2022

Source:	vivo

Title:	Remaining issues on RRC parameters for NR coverage enhancement

Agenda Item:	8.8.4

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e meeting, a list of parameters for NR coverage enhancement topic has been provided to RAN2 in [1]. There’re still some remaining issues of RRC parameters for NR coverage enhancement work item, which will be discussed in this contribution.

# RRC parameters for coverage enhancement

## 2.1	RRC parameters for JCE of PUSCH

Based on the discussions in our contribution [2], we have 2 proposals corresponding to which suggested changes on RRC parameter for JCE of PUSCH are provided in table 1.

Table 1.  RRC parameter updates for JCE of PUSCH

| Parameter Name | RAN2 Parent IE | RAN1 spec/ section | Value range | New or existing? | UE-specific or Cell-specific | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH-TimeDomainWindowLength | PUSCH-config | 38.214 | FFS INTEGER {1…32} | new | UE-specific | Length of a nominal time domain window in number of consecutive slots for DMRS bundling for PUSCH. |
| PUSCH-inter-bundlingFrequencyHopping | PUSCH-config | 38.214 | ENUMERATED {enabled, disabled} | new | UE-specific | Enabling/disabling of frequency hopping with DMRS bundling for PUSCH repetitions. |
| PUSCH-inter-bundlingFrequencyHoppingLength | PUSCH-config | 38.214 | INTEGER {1...32} | new | UE-specific | Length (in slots) of a configured frequency hop for PUSCH with DMRS bundling. |

Proposal 1:

- The time domain window length configured by PUSCH-TimeDomainWindowLength should support 1 slot but not exceed 32 slots, as illustrated in table 1.

Proposal 2:

- The following RRC changes provided in table 1 are needed for inter-bundling frequency hopping.

- Introduce a new RRC parameter to enable/disable PUSCH inter-bundling frequency hopping, e.g. ‘PUSCH-interbundling-FrequencyHopping’ configured in PUSCH-config.

- Introduce a new RRC parameter to configure the hopping interval length when PUSCH inter-bundling frequency hopping is enabled, e.g. ‘PUSCH-interbundling-FrequencyHoppingLength’ configured in PUSCH-config.

## 2.2	RRC parameters for PUCCH enhancement

Based on the discussions in our contribution [3], we have proposal 3 corresponding to which suggested changes on RRC parameter are provided in table 2.

Table 2.  RRC parameter updates for PUCCH enhancement

| Parameter Name | RAN2 Parent IE | RAN1 spec/ section | Value range | New or existing? | UE-specific or Cell-specific | Description |
| --- | --- | --- | --- | --- | --- | --- |
| PUCCH-nrofSlots-r17 | PUCCH-resource | 38.213 | ENUMERATED {2,4,8} | new | [UE-specific] | A new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication. The new repetition parameter is configured per PUCCH resource and should be in PUCCH-Resource. |
| PUCCH-DMRS -Bundling | FFS PUCCH-format1/2/3/4 | 38.213 | ENUMERATED {enabled, disabled} | new | [UE-specific] | Enabling/disabling of DM-RS bundling and time domain window for PUCCH repetitions. |
| PUCCH-TimeDomainWindowLength | [in PUCCH-Config] PUCCH-format1/2/3/4 | 38.213 | FFS INTEGER {1,2,4.8} | new | [UE-specific] | Length of a [nominal] time domain window in slots for DMRS bundling for PUCCH. |
| PUCCH-Window-Restart | PUCCH-config | 38.213 | ENUMERATED {enabled, disabled} | new | [UE-specific] | UE bundles PUCCH DM-RS slots remaining in a bundling nominal time domain window after a slot for which events violate power consistency and phase continuity requirements |
| PUCCH-inter-bundlingFrequencyHopping | PUCCH-format1/2/3/4 | 38.213 | ENUMERATED {enabled, disabled} | new | UE-specific | Enabling/disabling of frequency hopping with DMRS bundling for PUCCH repetitions. |
| PUCCH-inter-bundlingFrequencyHoppingLength | PUCCH-format1/2/3/4 | 38.213 | INTEGER {1,2,4} | new | UE-specific | Length of a configured frequency hopping with DMRS bundling length in slots for PUCCH. |

Proposal 3:

- The following RRC parameter changes provided in table 2 need to be performed in PUCCH enhancement:

- No need to introduce an explicitly indication of repetition factor 1 in the ‘PUCCH-nrofSlots-r17’

- ‘PUCCH-DMRS-Bundling’ is configured per PUCCH resource format and is UE specific.

- ‘PUCCH-TimeDomainWindowLength’ is configured per PUCCH resource format and is UE specific.

- Introduce a new RRC parameter for enabling/disabling frequency hopping with DMRS bundling.

- Introduce a new RRC parameter to configure the frequency hopping interval length when frequency hopping with DMRS bundling is enabled.

## 2.3	RRC parameters for Msg3 repetition:

According to the discussions in our contribution [4], two new parameters provided in table 3 should be introduced in the system information to configure the set of repetition factors and MCS indexes for scheduling an initial or re- transmission of Msg3 repetition.

Table 3.  RRC parameter updates for Msg3 repetition

| Parameter Name | RAN2 Parent IE | RAN1 spec/ section | Value range | New or existing? | UE-specific or Cell-specific | Description |
| --- | --- | --- | --- | --- | --- | --- |
| RepetitionFactorList-Msg3-R17 | SIB1 | 38.213 | List of 4 repetition factors, ENUMERATED {1, 2, 3, 4, 7, 8, 12, 16} | new | Cell-specific | List of 4 repetition factors, each of which has a value in {1, 2, 3, 4, 7, 8, 12, 16}. If this field is absent, the default 4 repetition factors are {1, 2, 3, 4}. |
| MCSIndexList-Msg3-R17 | SIB1 | 38.213 | A list of 4 MCS indexes, INTEGER {0,…,15} | new | Cell-specific | A list of 4 MCS indexes, each of which is a integer value in {0,..,15}. If this field is absent, the default MCS indexes are MCS{0, 1, 2, 3}. |

Proposal 4:

- Two new parameters provided in table 3 should be introduced in the system information to configure a set of repetition factors and a set of MCS indexes respectively, which are applicable for both scheduling an initial and re- transmission of Msg3 repetition.

# Conclusion

In this contribution, we discuss the remaining issues of RRC parameters for NR coverage enhancement work item, and have the following proposals:

Proposal 1:

- The time domain window length configured by PUSCH-TimeDomainWindowLength should support 1 slot but not exceed 32 slots, as illustrated in table 1.

Proposal 2:

- The following RRC changes provided in table 1 are needed for inter-bundling frequency hopping.

- Introduce a new RRC parameter to enable/disable PUSCH inter-bundling frequency hopping, e.g. ‘PUSCH-interbundling-FrequencyHopping’ configured in PUSCH-config.

- Introduce a new RRC parameter to configure the hopping interval length when PUSCH inter-bundling frequency hopping is enabled, e.g. ‘PUSCH-interbundling-FrequencyHoppingLength’ configured in PUSCH-config.

Proposal 3:

- The following changes need provided in table 2 to be performed in PUCCH enhancement:

- No need to introduce an explicitly indication of repetition factor 1 in the ‘PUCCH-nrofSlots-r17’

- ‘PUCCH-DMRS-Bundling’ is configured per PUCCH resource format and is UE specific.

- ‘PUCCH-TimeDomainWindowLength’ is configured per PUCCH resource format and is UE specific.

- Introduce a new RRC parameter for enabling/disabling frequency hopping with DMRS bundling.

- Introduce a new RRC parameter to configure the frequency hopping interval length when frequency hopping with DMRS bundling is enabled.

Proposal 4:

- Two new parameters provided in table 3 should be introduced in the system information to configure a set of repetition factors and a set of MCS indexes respectively, which are applicable for both scheduling an initial and re- transmission of Msg3 repetition.

# References

- R1-2112976, Consolidated higher layers parameter list for Rel-17 NR, 3GPP TSG RAN WG1 #107-e, November, 2021.

- R1-2200088, Remaining issues on joint channel estimation for PUSCH, vivo, 3GPP TSG RAN WG1 #107bis-e, January, 2022.

- R1-2200089, Remaining issues on PUCCH enhancements, vivo, 3GPP TSG RAN WG1 #107bis-e, January, 2022.

- R1-2200090, Remaining issues on Type A PUSCH repetitions for Msg3, vivo, 3GPP TSG RAN WG1 #107bis-e, January, 2022.