3GPP TSG RAN WG1 Meeting #107B-e	R1-2200578

Online, 17 – 25 January, 2022

Agenda Item:	8.12.1

Source: 	LG Electronics Inc.

Title: 	Support of group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss how to support a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service.

# CFR

As discussed in RAN1#105e, RAN1 assumes that Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Considering that UE in RRC_CONNECTED can use the initial BWP in some cases since Rel-15 NR, we think that CFR associated with the initial BWP can be supported. However, we concern that CFR equal to or smaller than the initial DL BWP would lead to low multicast capacity in CFR considering the initial DL BWP is consistently serving system information, paging, initial access from several UEs and additionally new broadcast support for MBS, and furthermore potentially cause overload in initial DL BWP together with broadcast. Thus, it would be beneficial to support possibility of configuring a wider CFR than initial DL BWP for a connected UE receiving multicast. Wider CFR would be also beneficial for idle and inactive UEs receiving broadcast.

Proposal 1: For a connected UE receiving multicast (as well as idle/inactive UEs receiving broadcast), CFR associated to initial DL BWP can be configured with a wider bandwidth than the initial DL BWP or a bandwidth equal to or smaller than the initial DL BWP.

Meanwhile, a connected UE can change unicast BWP switching e.g. from UE’s active BWP#1 to UE’s active BWP#2. Assuming that separate CFRs are associated to different unicast BWPs, unicast BWP switching should trigger change of CFR from CFR#1 associated to UE’s active BWP#1 to CFR#2 associated to UE’s active BWP#2 (whichever CFR option is agreed).

Proposal 2: At least for multicast, unicast BWP switching between UE’s active BWPs may immediately triggers CFR change between different CFRs associated to different UE’s active BWPs.

In addition, a certain CFR could be confined within more than one UE active BWP with a same numerology. If it is the case, upon unicast BWP switching, UE would not need to change CFR because CFR is associated with both unicast BWPs. UE could maintain CFR configuration and continue to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

Proposal 3: If a CFR is confined within more than one UE active BWP with a same numerology, the CFR can be associated to more than one BWP.

- Upon unicast BWP switching between UE’s active BWPs associated to the same CFR, UE does not change CFR and continues to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

For a connected UE receiving broadcast, one CFR of a cell should be associated at least to initial DL BWP of the cell. When connected UE receiving multicast switches to the initial DL BWP, UE would be expected to receive multicast on CFR of the initial DL BWP. If this UE is also receiving broadcast, it would be beneficial that this UE could receive both multicast and broadcast on the same CFR associated to the initial DL BWP.

Meanwhile, it is not clear how a connected UE activating UE’s active BWP different than the initial DL BWP can receive broadcast. One option is for gNB to transmit broadcast as well as multicast on a CFR associated to UE’s active BWP.

Proposal 4: For broadcast, CFR of a cell is associated at least to initial DL BWP of the cell for any RRC state.

- FFS whether broadcast CFR is associated to UE’s active DL BWP for UE in RRC_CONNECTED.

# CORESET/SS

In RAN1#104bis-e, RAN1 discussed the following proposal without any agreement:

For RRC_CONNECTED multicast UEs supporting CA capability, support the following principles for determining MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  and the maximum numbers of BD/CCE UE is required to monitor per slot for a serving cell supporting multicast reception:

- When determining  MPDCCHtotal,slot,μ / CPDCCHtotal,slot,μ  defined in 38.213, the number of DL serving cell(s) supporting multicast reception is increased as R times.

- The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

- R is a value reported by the UE

To our understanding, the maximum BD/CCE numbers can be increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception. However, we think that R is not related to CA capability. For RRC_CONNECTED UE receiving multicast, R can be a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability

Proposal 5: The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

  - R is a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability.

As specified in clause 5.1.5 of 38.214, if the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. In addition, If the PDSCH is scheduled by a DCI format having the TCI field present, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state.

Assuming that the above text in clause 5.1.5 of 38.214 can be also applied to GC-PDCCH/GC-PDSCH, we would need to clarify how the threshold timeDurationForQCL can be determined based on reported UE capability, because multiple UEs receiving the same GC-PDCCH/GC-PDSCH in the same group may report different UE capability for the threshold timeDurationForQCL. One simple approach seems that UEs always go for the worst threshold timeDurationForQCL for GC-PDCCH/GC-PDSCH. Alternatively, gNB could configure threshold timeDurationForQCL per G-RNTI or per CFR based on the worst reported UE capability among UEs in the same group or in the same CFR. If configured, we prefer the threshold timeDurationForQCL per G-RNTI. If not configured, the threshold could be set to the worst value in the current specification.

Proposal 6: For reception of GC-PDSCH scheduled by GC-PDCCH, UE determines whether the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, as specified in clause 5.1.5 of 38.214, where the threshold is configured per G-RNTI by gNB (based on the worst reported UE capability).

  - If the threshold is not configured, the worst value of the threshold in the current specification is used.

# HARQ New TX and ReTX

RAN1 agreed the same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

In RAN1#106bis, RAN1 discussed the following FL’s proposal, but RAN1 did not reach any agreement.

Initial Question 4-1a: Regarding the NDI conflict issue that different UEs in a group may have different NDI values for a certain HPID before performing an initial PTM transmission, which option should be adopted? - Option 1: Rely on gNB implementation to avoid such issue. - Option 2: Resolve this issue with potential specification enhancement.

In our view, as specified in Rel-15/16, UE should manage NDIs in unicast DCIs only across unicast PDCCHs. Meanwhile, UE should manage NDIs in multicast DCIs only within same G-RNTI. That is, for multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Accordingly, NDIs for multicast DCIs and NDIs for unicast DCIs should be independently and separately handled. A NDI value for unicast DCI and a NDI value for subsequent group common DCI can be same or different for a same HPN. Besides, a NDI value for group common DCI and a NDI value for subsequent unicast DCI can be same or different for a same HPN.

In the meantime, RAN1 previously agreed the same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB. We think that the NDI value should be same for a same TB for group common DCIs and PTP retransmissions for a same G-RNTI.

To avoid potential collision between PTP retransmission and unicast transmission for a same HARQ process number in DCIs, UE specific DCI scheduling PTP retransmission can indicate PTP retransmission. Thus, NDIs in UE specific DCIs scheduling PTP retransmission can be managed across PTM and PTP retransmission for a same HARQ process number, separately with NDIs for unicast DCI indicating the same HARQ process number.

Proposal 7: For multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Proposal 8: For multiple GC-DCIs scheduling multiple G-RNTIs with different HPNs, PTP retransmission is associated to PTM initial transmission for one of the G-RNTIs based on the same HARQ process ID and NDI.

Proposal 9: UE specific DCI indicates PTP retransmission.

Proposal 10: For PTP ReTX and unicast with a same HPN, NDI value for PTP ReTX and NDI value for unicast are separately toggled and managed.

Meanwhile, we think that if UE receives PTP retransmission, it is desirable to continue on PTP retransmission, rather PTM retransmission. DRX active time may not be well aligned between PTP retransmission and PTM retransmission because PTP/PTM retransmissions will be likely scheduled after NACKs on PUCCHs separately allocated for PTP/PTM. For saving UE power, it seems beneficial for UE to expect PTP retransmission of a TB after sending NACK to the TB once PTP retransmission of a TB has been received. Otherwise, while monitoring PDCCH for PTP retransmission, UE would need to continuously monitor PDCCH for PTM retransmission. We think that it is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

Proposal 11: Upon receiving PTP retransmission of a TB with a HPN, UE expects PTP retransmission of the TB after sending NACK to the TB.

  - It is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

# Group Common SPS

NOTE: The proposals in this section may be also shown in LG’s other contribution to AI 8.12.2.

When gNB activates group common SPS, a UE in the group may miss the activation DCI. If gNB did not receive confirmation from the UE or gNB did not receive HARQ-ACK to SPS PDSCH of a TB, gNB could schedule PTP transmission of the TB. In this case, initial HARQ transmission of the TB should be supported based on PTP transmission.

Proposal 12: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

When gNB releases group common SPS, a UE in the group may miss the release DCI. If gNB did not receive confirmation from the UE, it is not clear how UE could finally release the group common SPS. Considering such erroneous case, it would be beneficial to introduce UE autonomous SPS release. It seems possible to use a UE’s individual timer based SPS release. However, considering misalignment among different UEs in the group, we should make sure that all UEs autonomously release group common SPS release at a same slot which could be pre-determined e.g. by RRC and/or DCI.

Proposal 13: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

# Multicast scheduling with carrier aggregation

In RAN1#107, RAN1 discussed reception of group-common PDCCH/PDSCH on SCell with self-scheduling and cross-carrier scheduling for multicast. As discussed in RAN1#107, multicast reception on SCell would be useful in scenarios where PCells are different among UEs in the same group, otherwise all UEs in the same group should be configured with the same PCell if they want to receive multicast. Thus, we propose that UE configured with SCell can support reception of multicast transmission on SCell depending on UE capability.

Proposal 14: UE configured with SCell can support reception of multicast transmission on SCell depending on UE capability.

Meanwhile, support of cross-carrier scheduling for multicast seems not essential in Rel-17. Thus, we prefer that cross-carrier scheduling of PTM transmission is not supported in Rel-17 for simplicity.

Proposal 15: Cross-carrier scheduling of PTM transmission is not supported in Rel-17.

Furthermore, assuming that cross-carrier scheduling of PTM transmission is not supported, we could further discuss whether PTM transmission on a serving cell can be supported with cross carrier scheduling of unicast transmission. We are fine to support PTM transmission on a serving cell when UE is configured with CCS for unicast and the serving cell schedules the other cell for unicast transmission,. But, for simplicity, we do not prefer to support PTM transmission on a serving cell when UE is configured with CCS for unicast and the SCell is scheduled by the other cell for unicast transmission. Meanwhile, it seems clear that PTM transmission on a serving cell can be supported if UE is not configured with CCS for unicast.

Proposal 16: PTM transmission on a serving cell can be supported if UE is not configured with CCS for unicast.

Proposal 17: PTM transmission on SCell is not supported if UE is configured with CCS for unicast and the SCell is scheduled by the other cell for unicast transmission.

Proposal 18: PTM transmission on a serving cell can be supported if UE is configured with CCS for unicast and the serving cell schedules the other cell for unicast transmission.

Meanwhile, RAN1 also discussed the feasibility of reception of group-common PDCCH/PDSCH on SCell for broadcast. Considering that MBMS reception on SCell has been supported for LTE, we think that it is useful to support reception of group-common PDCCH/PDSCH on SCell for broadcast. We could not always ensure that UE receives broadcast on PCell. Thus, we propose that UE configured with SCell can support reception of broadcast transmission on SCell depending on UE capability.

Proposal 19: UE configured with SCell can support reception of broadcast transmission on SCell depending on UE capability

# Conclusion

In this contribution, we suggest the following proposals for support of group scheduling in NR MBS:

CFR

Proposal 1: For a connected UE receiving multicast (as well as idle/inactive UEs receiving broadcast), CFR associated to initial DL BWP can be configured with a wider bandwidth than the initial DL BWP or a bandwidth equal to or smaller than the initial DL BWP.

Proposal 2: At least for multicast, unicast BWP switching between UE’s active BWPs may immediately triggers CFR change between different CFRs associated to different UE’s active BWPs.

Proposal 3: If a CFR is confined within more than one UE active BWP with a same numerology, the CFR can be associated to more than one BWP.

- Upon unicast BWP switching between UE’s active BWPs associated to the same CFR, UE does not change CFR and continues to receive PTM/PTP (re-)transmissions on the CFR during/after unicast BWP switching.

Proposal 4: For broadcast, CFR of a cell is associated at least to initial DL BWP of the cell for any RRC state.

- FFS whether broadcast CFR is associated to UE’s active DL BWP for UE in RRC_CONNECTED.

CORESET/SS

Proposal 5: The maximum BD/CCE numbers are increased as R times MPDCCHmx,slot,μ and R times CPDCCHmax,slot,μ for a serving cell supporting multicast reception, where MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are defined in Table 10.1-2 and Table 10.1-3 in 38.213

  - R is a value reported by the UE as part of MBS related UE capability, regardless of whether UE supports CA capability.

Proposal 6: For reception of GC-PDSCH scheduled by GC-PDCCH, UE determines whether the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, as specified in clause 5.1.5 of 38.214, where the threshold is configured per G-RNTI by gNB (based on the worst reported UE capability).

  - If the threshold is not configured, the worst value of the threshold in the current specification is used.

HARQ New TX and ReTX

Proposal 7: For multiple GC-DCIs scheduling multiple G-RNTIs, NDI values for different G-RNTIs are separately toggled and managed.

Proposal 8: For multiple GC-DCIs scheduling multiple G-RNTIs with different HPNs, PTP retransmission is associated to PTM initial transmission for one of the G-RNTIs based on the same HARQ process ID and NDI.

Proposal 9: UE specific DCI indicates PTP retransmission.

Proposal 10: For PTP ReTX and unicast with a same HPN, NDI value for PTP ReTX and NDI value for unicast are separately toggled and managed.

Proposal 11: Upon receiving PTP retransmission of a TB with a HPN, UE expects PTP retransmission of the TB after sending NACK to the TB.

  - It is up to UE whether to additionally receive retransmission of the same TB on group common PDSCH with the same HPN and non-toggled NDI.

Group common SPS

Proposal 12: For a UE not confirming SPS activation, gNB can schedule PTP initial transmission of missed TB(s).

Proposal 13: After group common SPS activation, all UEs autonomously release the group common SPS right after a pre-determined slot

- The pre-determined time is determined by RRC and/or DCI.

CA

Proposal 14: UE configured with SCell can support reception of multicast transmission on SCell depending on UE capability.

Proposal 15: Cross-carrier scheduling of PTM transmission is not supported in Rel-17.

Proposal 16: PTM transmission on a serving cell can be supported if UE is not configured with CCS for unicast.

Proposal 17: PTM transmission on SCell is not supported if UE is configured with CCS for unicast and the SCell is scheduled by the other cell for unicast transmission.

Proposal 18: PTM transmission on a serving cell can be supported if UE is configured with CCS for unicast and the serving cell schedules the other cell for unicast transmission.

Proposal 19: UE configured with SCell can support reception of broadcast transmission on SCell depending on UE capability

# Reference

[1] RP-201038	WID revision: NR Multicast and Broadcast Services,	Huawei, HiSilicon