3GPP TSG RAN WG1 #107bis	R1-2200627

e-Meeting, 17 – 25 January, 2022

Agenda item:		8.15.8

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR coverage enhancement

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined [1, 2]. In this contribution we present our views on the UE features for NR coverage enhancement.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107-e are available in the Annex.

- 30-1:

  - Move candidate values to notes column. No need to list the legacy values, hence only list values for K>16.

  - Add FG11-6 (PUSCH repetition Type A) as pre-requisite

  - Per UE

- 30-1a:

  - Similarly to FG30-1, move values to notes column and restrict range to K>16

  - Per UE

- 30-2:

  - Add 30-1 as pre-requisite

  - Per UE

- 30-2a:

  - Replace pre-requisite FG5-16 with FG30-1a

  - Do not merge with 30-2

  - Per UE

- 30-3:

  - OK to split in 2 FGs for DG ad CG, for consistency with other FGs for coverage enhancements

  - Per UE

- 30-4/a/b/c/d:

  - Do not merge 30-4a/b/c/d, as they refer to FGs that are independently signalled. If the FGs are merged, then it would force a UE to report support for DM-RS bundling on all corresponding cases. This is an unnecessary restriction, as for example a UE could support DM-RS bundling for PUSCH repetition type A, but not for Type B, and vice-versa.

  - No need to split back-to-back and non-back-to-back transmissions, as the constraints for non-back-to-back transmissions are already quite restrictive and should not require more complex UE design and/or testing.

  - Per band

- 30-5:

  - Per UE

- 30-6:

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR coverage enhancement, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107-e, Nov. 2021

- R1-2112902, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Nov. 2021

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. FFS whether to merge with FG 30-3 | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A FFS whether to merge with FGs 30-4b/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B FFS whether to merge with FGs 30-4a/4c/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission FFS whether to split to within-slot back-to-back transmission and across-slot back-to-back transmission | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH FFS whether to merge with FGs 30-4a/4b/4d FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions FFS whether to merge with FGs 30-4a/4b/4c FFS whether to split to back-to-back transmission and non-back-to-back transmission | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH FFS whether to merge with FG 30-4f | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling FFS whether to merge with FG 30-4e | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |