# 3GPP TSG-RAN-WG1 #107bis-e	R1-2200670

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.12.1

Source:	Google Inc.

Title:	Corrections of MBS for RRC_CONNECTED UEs

Document for:	Discussion and decision

# 1. Introduction

In this document, we provide our view on the correction of DCI format 4_2 descriptions

# 2. Discussion

## 2.1 Correction on DCI format 4_2

After RAN1 #107e meeting, the group and the editor discussed how to implement DCI format 1-1 for MBS features. To make the description clearer, new DCI formats 4_0, 4_1, and 4_2 are introduced during the email discussion. However, some descriptions of DCI format 4_2 are still referring to configurations of DCI format 1_1, which may cause confusion.

Proposal: For the description of DCI format 4_2, change the description of “priorityIndicatorDCI-1-1” and “DCI format 1_1” to “priorityIndicatorDCI-4-2” and “DCI format 4_2”, respectively.

Reasons for change To align the description of the new DCI format 4_2 Summary of changes Change “priorityIndicatorDCI-1-1” to “priorityIndicatorDCI-4-2” Change “DCI format 1_1” to “DCI format 4_2” Specs/Sections impacted TS 38.212, Section 7.3.1.5.3

< Start of text proposal > ##### 7.3.1.5.3 Format 4_2 DCI format 4_2 is used for the scheduling of PDSCH in DL cell. The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI: *** Unchanged text omitted *** - Downlink assignment index – number of bits as defined in the following - 4 bits if more than one serving cell are configured in the DL for multicast and the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 2 bits if only one serving cell is configured in the DL for multicast and the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 bits are the counter DAI; - 0 bits otherwise. If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group. If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group. If higher layer parameter priorityIndicatorDCI-4-2 is configured in PDSCH-Config-Multicast, if the bit width of the Downlink assignment index in DCI format 4_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 4_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 4_2 for the two HARQ-ACK codebooks are the same. - PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS 38.213] - PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config-Multicast1 if configured or PUCCH-Config-Multicast2 if configured; otherwise, I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config. If higher layer parameter priorityIndicatorDCI-4-2 is configured in PDSCH-Config-Multicast, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 4_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 4_2 for the two HARQ-ACK codebooks are the same. - Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports {p0,…,pv-1} shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals max⁡{xA,xB}, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. - Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI in PDCCH-Config-Multicast is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214]. - DMRS sequence initialization – 1 bit. - Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-4-2 is not configured in PDSCH-Config-Multicast; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213]. - Enabling/disabling HARQ-ACK feedback indication –1 bit if higher layer parameter harq-FeedbackEnabler-Multicast indicates dci-enabler; 0 bit, otherwise. < End of text proposal >

# 3. Conclusion

Proposal: For the description of DCI format 4_2, change the description of “priorityIndicatorDCI-1-1” and “DCI format 1_1” to “priorityIndicatorDCI-4-2” and “DCI format 4_2”, respectively.

# 4. Reference

[1]	3GPP TS 38.212, v17.0.0, Dec. 2021.