3GPP TSG-RAN WG1 #107bis-e 																	R1-2200686

e-Meeting, January 17th – 25th, 2021

Agenda item:			8.8.1.1

Source:	Moderator (Sharp)

Title:	FL Summary #2 on Enhancements on PUSCH repetition type A

Document for:	Discussion and Decision

Introduction

For PUSCH enahancements the following objectives are described in the Coverage Enhancement WID.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

This document is intended to facilitate view exchange and discussions on the enhancements on PUSCH repetition type A, for the following assigned email discussion.

[107bis-e-R17-CovEnh-01] Email discussion regarding enhancements for PUSCH repetition type A – Toshi (Sharp)

- 1st check point: January 20

- Final check point: January 25

Discussion

RRC parameter related issues

For this meeting, the following remaining issues which have impacts on RRC parameters have been raised.

- Issue#1-1: Whether to support pusch-AggregationFactor-r17

- Issue#1-2: AvailableSlotCounting for CG-PUSCH

[Close] Issue#1-1: Whether to support pusch-AggregationFactor-r17

We have discussed whether to introduce pusch-AggregationFactor-r17 to support up-to-32 repetitions, for quite some times. However, no consensus to support it was made until now.

For RAN1#107bis-e, OPPO [7] and Intel [8] are proposing the introduction of pusch-AggregationFactor-r17 which support up-to-32 repetitions. Meanwhile, Apple [9] is proposing no use of pusch-AggregationFactor to configure the 32 repetitions in Rel-17

Since this issue has RRC impact, we have to make a decision shortly. Moreover, all the arguments have been presented during the past discussions already. Therefore, FL would like to suggest quickly checking companies’ opinions, and if no consensus is made, this issue will be considered closed and no further discussion will be held.

1st round (Issue#1-1)

Companies are asked to express their position on whether pusch-AggregationFactor-r17 supporting up-to-32 repetitions is introduced in Rel-17 or not.

|  | Company name |
| --- | --- |
| Introduce pusch-AggregationFactor-r17 | Nokia, NSB, Panasonic, Intel, Samsung, China Telecom, OPPO, Lenovo, Motorola Mobility |
| Do not introduce pusch-AggregationFactor-r17 | CATT, Panasonic, QC, vivo, ZTE, Sharp, Apple, DOCOMO,Xiaomi, Ericsson,TCL, LG |

1st round summary (Issue#1-1)

9 companies prefer introducing pusch-AggregationFactor-r17 supporting up-to-32 repetitions, while 12 companies do not think it as necessary. Therefore, FL would like to propose the following conclusion.

Proposed conclusion:

No consensus to introduce pusch-AggregationFactor-r17.

Since the above conclusion was agreed in 1/18 GTW2 session, this issue is now considered closed.

[Open] Issue#1-2: AvailableSlotCounting for CG-PUSCH

There is an ongoing discussion in UE features on whether a single FG or separate FGs are defined for DG-PUSCH and CG-PUSCH with the available slot counting. Samsung [5] mentions that, if separate FGs are defined, the parameter AvailableSlotCounting needs to be additionally included in ConfiguredGrantConf, as described in the consolidated higher layers parameter list for Rel-17.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in PUSCH-Config Note: if separate FGs are defined for DG-PUSCH and CG-PUSCH, this field for CG-PUSCH may need to be added in ConfiguredGrantConf, too. | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. Note: if separate FGs are defined for DG-PUSCH and CG-PUSCH, this field may be necessary for each of them. |

This issue should be addressed after the discussion on single FG or separate FGs for DG-PUSCH and CG-PUSCH with the available slot counting concludes in UE feature AI.

2nd round (Issue#1-2)

As per the request from Chair in 1/18 GTW2 session, FL would like to make the following proposal to conclude this issue.

FL Proposal on Issue#1-2:

- Remove the notes from “Per (UE, cell, TRP, …)” and “Comment” columns of the existing AvailableSlotCounting in the consolidated RRC parameter list.

- If separate FGs are defined for DG-PUSCH and CG-PUSCH, add another AvailableSlotCounting to the consolidated RRC parameter list, with the following contents.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in ConfiguredGrantConf | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |

| Company | Comments |
| --- | --- |
| vivo2 | Fine. |
| CATT | OK. |
| ZTE | If separate FGs are defined for DG-PUSCH and CG-PUSCH (not our preference), we are fine with the proposal. One minor editorial suggestion: ‘disable’ should be ‘disabled’. |
| Apple | Just for clarification, with the proposal, there are two rows for parameter AvailableSlotCounting, which is configured in PUSCH-Config or in ConfiguredGrantConfig. Since only one RRC parameter, i.e., AvailableSlotCounting, is introduced, maybe one row is enough, and mention the parent IE could be PUSCH-Config or ConfiguredGrantConfig in column “Per (UE, cell, TRP, …)”. |
| CMCC | Fine with the proposal. From our understanding, CG-PUSCH is configured through ConfiguredGrantConf, and DG-PUSCH is configured through PUSCH-Config separately. The AvailableSlotCounting for CG-PUSCH is supposed to be configured through the ConfiguredGrantConf, which is not related to whether single or two feature groups are defined for the DG and CG PUSCH transmission. |
| Nokia/NSB | Support the FL’s proposal. We also share similar question for clarification as Apple. |
| Ericsson | Support |
| FL | @Apple and Nokia, I have no strong view on the RRC list structure, as far as it reflects our intention clearly enough. Looking at the v001 list, you can see multiple “nrofPRBs-r17” in 60GHz worksheet. So, I don’t think putting two rows for parameter AvailableSlotCounting is a problem. @CMCC, In RAN1#106bis-e, we have already made the following agreement, and only exception is the case when two feature groups would be defined for the DG and CG PUSCH transmission. Agreement A single RRC paramter AvailableSlotCounting that applies to both DG-PUSCH and CG-PUSCH is introduced.. |
| OPPO | Support |

2nd round summary (Issue#1-2)

No negative comment to FL Proposal on Issue#1-2 has been received since it was shared at the beginning of the 2nd round discussion. Therefore, it would be brought for the email approval.

FL Proposal on Issue#1-2 for email approval:

- Remove the notes from “Per (UE, cell, TRP, …)” and “Comment” columns of the existing AvailableSlotCounting in the consolidated RRC parameter list.

- If separate FGs are defined for DG-PUSCH and CG-PUSCH, add another AvailableSlotCounting to the consolidated RRC parameter list, with the following contents.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Enhancement on PUSCH repetition Type A |  |  |  |  | AvailableSlotCounting | new |  | Enabling PUSCH repetitions counted on the basis of available slots | ENUMERATED {enabled, disable } |  | in ConfiguredGrantConf | UE-specific | 38.331 | Agreement: • Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. o RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |

[Close] Issue#1-3: Other RRC parameter related issues

1st round (Issue#1-3)

If there is any other issue which is not captured in this document yet but needs be discussed for the completion of the RRC parameter list in this meeting, please comment below

| Company | Comments |
| --- | --- |

Since no other issue was raised in 1/18 GTW2 session, this issue is closed.

Remaining issues

For this meeting, the following remaining issues have been raised.

- Issue#2-1: Consideration of Rel-17 PUSCH dropping rules

- Issue#2-2: Use of SSBs across multiple TRPs for the available slot determination

- Issue#2-3: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

- Issue#2-4: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

- Issue#2-5: Available slot counting for PUSCH repetition Type A with K=1

- Issue#2-6: The slot indicated by K2 offset for the available slot counting

[Open] Issue#2-1: Consideration of Rel-17 PUSCH dropping rules

In RAN1#106-e, the following 2-step procedure for PUSCH repetition Type A with available slot counting was agreed. This agreement includes one FFS point about applicability of Rel-17 PUSCH dropping rules introduced in other WI(s).

Agreement Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots. - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s)

In Rel-17 eIIoT/URLLC WI, following PUSCH cancellation rules have been made in RAN1#106bis-e and RAN1#107-e meeting.

RAN1#106bis-e: Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH RAN1#107-e Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated.

vivo [2] is proposing that the cancellation of LP PUSCH introduced in Rel-17 eIIoT/URLLC WI should be applied in Step 2 in the same way as Rel-15/16 PUSCH dropping. More specifically, LP DG-PUSCH is cancelled when colliding with HP CG-PUSCH, and LP CG-PUSCH is cancelled when colliding with HP DG-PUSCH. In both cases, the cancellation is applied per actual repetition. Note that this cancellation is already captured in Clause 9 of TS38.213v17.0.0, and the PUSCH dropping procedure in Clause 6.1.2.1 and Clause are saying that a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS 38.213]. Therefore, no specification change would be necessary for supporting of this behaviour.

1st round (Issue#2-1)

Do you agree on the following proposal?

FL proposal to Issue#2-1:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

| Company | Comments |
| --- | --- |
| Nokia/NSB | As explained by the FL, the current specifications have already captured the above proposal. Therefore, we don’t see the need to make an agreement on this issue. |
| CATT | Agree. This may be a conclusion if it is already captured. |
| Panasonic | Agree. |
| QC | Agree |
| Intel | We are fine with the proposal. |
| vivo1 | Agree. |
| ZTE | Fine with the proposal. |
| Samsung | It seems not necessary to have this agreement now. |
| Sharp | Agree. |
| Apple | Fine with the proposal. |
| China Telecom | Agree. Also agree with Nokia the agreement is not necessary. |
| OPPO | Agree. |
| Huawei/HiSilicon | OK. |
| Xiaomi | Agree |
| Ericsson | Support the proposal, since it is aligned with the FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s). |
| Lenovo, Motorola Mobility | Agree |
| TCL | Agree. |
| LG | We support the proposal. |

1st round summary (Issue#2-1)

No companies objected the proposal. Several companies do not see the need to take it as an agreement. FL would like to suggest taking the following conclusion.

Proposed conclusion:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

2nd round (Issue#2-1)

FL would like to bring the above proposed conclusion for email approval. Only if there is a strong concern, please indicate it in the table below.

| Company | Comments |
| --- | --- |
| Samsung | Overall we prefer not to have agreements or conclusions that have no specification impact at this stage of Rel-17. Same comment applies to other proposed conclusions. For this case, given the original agreement for available slot mentions “Rel-15/16” collision rules, we are OK with the proposed conclusion. |

2nd round summary (Issue#2-1)

No objection against the proposed conclusion on Issue#2-1 has been received since it was shared at the beginning of the 2nd round discussion. Therefore, it would be brought for the email approval.

Proposed conclusion for email approval:

- The cancellation of LP PUSCH (introduced in Rel-17 eIIoT/URLLC WI) is applied in Step 2 of the previously agreed 2-step procedure of Rel-17 PUSCH repetitions counted on the basis of available slots (i.e., Option 1-B).

  - No specification impact is expected.

[Open] Issue#2-2: Use of SSBs across multiple TRPs for the available slot determination

In NR Rel-17 feMIMO WI, it was agreed to support different SSB sets with different PCIs transmitted from mTRPs in a same serving cell. In RAN1#107-e, we discussed whether to consider more than one SSB sets for available slot counting for ConEnc, and no consensus to support more than one SSB sets for available slot counting was made.

vivo [2] is raising three possible options to handle the SSBs with PCIs different from the serving cell PCI, and they prefers Options 2 and 3. Also, some other companies are providing their views on this issues as shown below. Ericsson [13] is raising the point that, if a UE considers the SSB configurations of multiple TRPs in the first step of determining of available slots, the TRP that schedules and receives PUSCH needs to know the SSB configuration of other TRP so as to decode a PUSCH repetition correctly. For this issue, Interdigital [11] is suggesting that whether an inter-cell multi-TRP scenario can be considered as a practical use case for coverage enhancement should be discussed before considering SSBs from multiple TRPs to determine available slots, and that would also have some impact on other AIs, such as joint channel estimation.

- Select only one option from:

  - Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

    - ZTE [3],

  - Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

    - vivo [2], Samsung [5], Qualcomm [6], CMCC [12]

  - Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

    - vivo [2], Nokia/Nokia Shanghai Bell [4] , CMCC [12], Ericsson [13]

- Discuss first whether an inter-cell multi-TRP scenario can be considered as a practical use case for coverage enhancement.

    - Interdigital [11]

FL’s understanding is that many companies think multi-TRP scenario can be considered as a practical use case for coverage enhancement. At the same time, multiple SSB configurations of multiple TRPs affect not only Rel-17 PUSCH repetitions with available slot counting but also the legacy PUCCH repetition procedure, because the slots for the legacy PUCCH repetitions are also determined by referring to SSB positions. When we designed the available slot counting for PUSCH repetition Type A, many companies had the alignment with the legacy PUCCH repetitions in mind, and consequently the procedures of PUSCH repetition Type A with the avaialble slot counting is well-aligned to the legacy PUCCH repetition procedure. Therefore, from the FL perspective, it is better to also check with MIMO experts how PUCCH repetitions work with the SSB configurations of multiple TRPs.

1st round (Issue#2-2)

Q1: Any views on how the PUCCH repetitions work with the SSB configurations of inter-cell multiple TRPs? (Note that this question is related to feMIMO WI rather than CovEnh WI.)

Q2: Please provide your preferences/views/clarifications on the following options.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Q1: It is worth clarifying that the case of multiple TRPs (mTRPs) with separate SSB configurations is inter-cell mTRPs with multiple DCIs. Hence, from our understanding, how the legacy UL transmission works with inter-cell multi-DCI mTRPs has not been discussed in feMIMO WI. Q2: Option 3. |
| CATT | Q1: Open to hear other companies’ views. Q2: Option 3 sounds reasonable. If PUSCH repetition is received by one or multiple TRPs, it is natural that the SSB set(s) from related TRPs should be used to determine available slot. The SSBs of irrelevant TRPs does not need to be considered during available slot determination. It may be OK to consider them in Step 2 (actual repetition determination), to avoid potential inter-UE interference, or just simply ignore them to provide more opportunities for actual transmission. |
| FL | As mentioned by Nokia, this issue is about inter-cell mTRPs. |
| QC | Q1: The same issues are being discussed in feMIMO WI, but no conclusions have been drawn. One option for us would be to wait until feMIMO WI makes more progress. The WI is not handled in this meeting. We are okay to mirror the solution that feMIMO WI adopts to handle PUCCH repetitions. Q2: Prefer Option 2. It is information that is available to the UE beforehand and the collision rules dictate that if we don’t take it into account, the transmission will get dropped. Option 2 ensures more repetitions are actually transmitted. |
| Intel | Q1: Suggest to treat this in MIMO session, but not CovEnh session Q2: We prefer Option 3. The SSBs from other TRPs do not need to be considered for available slot determination. |
| vivo1 | Q1: Agree with feature leader that PUCCH repetition related changes are related to feMIMO WI, we should solve the PUSCH available slot determination issue here and it’s up to MIMO topic to decide whether the option we agreed here should be applied to PUCCH as well. Q2: Since option 1 may provide less number of actual repetitions compared to option 2 and option 3, either option 2 or option 3 is fine from our side. From coverage perspective to allow more actual repetitions, option 2 is a bit more preferred as a PUSCH transmission would not be allowed anyway when colliding with SSBs with serving cell PCI according to current spec. in TDD case. |
| ZTE | Q1: In MIMO session, the collision handling between UL channels/signals and non-serving cell SSB is not concluded yet. There are two options (with no difference between different UL channel/signal) on the table for now. We agree that the decision should be made in MIMO session. - Alt 1: The UE does not transmit any UL channel/signal (no matter associated with serving cell PCI or non-serving cell PCI). - Alt 2: The UE can only transmit UL signal/channel associated with the serving cell TRP. Q2: Option 1 and Option 3. We don’t see the difference between Option 1 and Option 3. For Option 1, it means SSBs with PCI the same as the serving cell PCI will be used to determine the available slots, and other set of SSBs are not. Could FL or proponents of Option 3 clarify the difference with Option 1? |
| Samsung | Q1: This should not be discussed in CovEnh – should be in MIMO maintenance. Q2: This should also be discussed in MIMO maintenance. |
| Sharp | Q1: Share the views from other companies. That discussion has not been concluded in feMIMO WI yet. Q2: We prefer waiting the discussions in feMIMO AI on how the legacy PUSCH/PUCCH repetitions work with inter-cell multi-TRPs. |
| Apple | Q1: According to our understanding, there is no conclusion in MIMO discussion on the UL transmission colliding with SSB for M-TRP operation. Q2: For three options, Option 3 is simple and preferred. the wording could be updated. The spatial relation of PUSCH and SSB is only applied to FR2, maybe coresetpoolindex could be used to indicate the SSB. |
| China Telecom | Q1: A unified solution for PUSCH and PUCCH is preferred. Q2: Option 3. |
| OPPO | Q1: We are open on the discussion. Q2: We prefer Option 3. |
| NTT DOCOMO | Q2 : We prefer Option2. In our understanding, whether UL transmission with the symbols overlapping with SSBs of different PCI is allowed or not is not discussed in M-TRP discussion. So that we may wait for the conclusion of M-TRP discussion or we assume one option that UL transmission is not allowed with the symbol since same PDCCH/PDSCH/PUCCH/PUSCH-Config is assumed for the multiple TRPs (In this case Option 2 is preferred.). |
| Xiaomi | Q1: discuss it in MIMO session if it has been concluded. Q2: we prefer Option 3, we agree that SSBs from other TRPs should not be considered for available slot determination. |
| Ericsson | Q1: In our view Rel-17 inter-cell multiple TRPs deals with multi-DCI Multi-PDSCH. Rel-17 Multi-TRP PUCCH repetition is for reliability, where multiple TRPs are in one serving cell. In this case, there is no separate SSB configurations associated with different TRPs, and a UE needs to consider all SSBs of mTRPs in the cell. Q2: With the same logic as Q1, if SSBs with different PCIs are considered, we prefer Option 2. |
| Lenovo, Motorola Mobility | Q1: Should be discussed in MIMO session Q2: Prefer option 3 as SSBs from other TRPs should be considered for available slot determination on one TRP |
| TCL | Q1: We are open on the discussion. Q2: We prefer Option 3. The SSB from other TRPs should not be considered for available slot determination. |
| LG | Q1: The discussion will be handled in feMIMO WI, but it would be better if the aligned solution are adopted for PUCCH and PUSCH. Q2: We prefer to wait the decision in feMIMO. Regarding three options, we are not sure if Option 3 has a benefit compared to Option 1. Option 2 seems to be a solution with both gain (PUSCH can be transmitted in more slots) and loss (latency increases due to postpone). Therefore, Option 1 is preferred, which uses SSB of the serving cell for available slot determination without changing the spec. |
| InterDigital | Q2 : We prefer to wait for the progress in the relevant discussion in the MIMO group. Our understanding is that the MIMO group did not have much discussions on uplink transmission related to inter-cell multi-TRP. |

1st round summary (Issue#2-2)

In feMIMO session the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs has not been concluded yet, where it includes the collision handling between the legacy PUCCH repetitions and multiple SSBs for inter-cell mTRPs and the collision handling between the legacy PUSCH repetitions (i.e., PUSCH repetition Type A with the physical slot counting) and multiple SSBs for inter-cell mTRPs. The decision on these issues should be made in feMIMO session, taking into account mTRP aspects, e.g., what information is shared by multiple TRPs, etc.

Such mTRP aspects would highly affects the discussion on available slot determination in CovEnh as well. Moreover, several companies expressed that we can wait for the conclusion in feMIMO discussion. For the FL perspective, it is suggested waiting further progress in feMIMO session.

Proposed conclusion:

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

For the reference for the future discussion, the companies’ current positions are summarized as follows:

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

  - ZTE, LG

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

  - QC, vivo, NTT DOCOMO, Ericsson

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

  - Nokia/NSB, CATT, Intel, Apple, China Telecom, OPPO, Xiaomi, Lenovo/Motorola Mobility, TCL

2nd round (Issue#2-2)

Do you agree on either conclusion below? If yes, which do you prefer?

Conclusion proposed by FL:

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

Conclusion proposed by vivo:

- Down-select from one of the following options for available slot determination for Type A PUSCH repetition with respect to inter-cell mTRP based on further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

- Option 1: SSBs with PCI different from the serving cell PCI are only used for actual PUSCH repetition determination after the available slots are determined

- Option 2: Both SSBs with serving cell PCI and SSBs with PCI different from the serving cell PCI are used for the available slot determination

- Option 3: SSBs with the same PCI as the SSB used to determine the spatial relation of the PUSCH, are used to determine the available slots. Other set of SSBs are used for actual PUSCH repetition determination after the available slots are determined.

- Option 4: any other options if identified in feMIMO session

| Company | Comments |
| --- | --- |
| QC | Conclusion by FL seems to be enough for now. |
| Intel | We prefer the Conclusion proposed by FL. We should avoid similar or duplicated discussions in feMIMO and CovEnh AIs. |
| vivo2 | We prefer to agree on the options to be down-selected for progress which also helps the discussions in feMIMO, instead of doing nothing while so many companies have proposed all possible options on the table. |
| InterDigital | We prefer Conclusion from the FL. We can come back to the issue if there’s a progress in the feMIMO discussion. |
| Panasonic | We agree on either conclusion. We think “the decision is made in feMIMO session” is also possibility. |
| CATT | FL’s conclusion seems enough. Vivo’s conclusion can be a backup starting point, if feMIMO WI cannot provide solutions in time. In that case, we have to come out a detailed solution by our own. |
| ZTE | No strong preference. We may even no need any conclusion here as we anyway will discuss this issue later on. |
| Apple | Conclusion from FL is enough. |
| Lenovo, Motorola Mobility | FL’s conclusion is sufficient |
| CMCC | As the PUCCH repetition is still under MIMO’s discussion, we prefer unified/aligned solutions for both PUSCH repetitions and PUCCH repetitions under multiple TRP scenarios. Then we support FL’s proposal to defer the discussion until further progress is achieved in feMIMO. |
| Xiaomi | Support the conclusion from FL. |
| Nokia/NSB | We prefer the conclusion from FL. |
| Ericsson | We prefer the conclusion proposed by FL and can come back to these options if they are still valid according to FeMIMO agreement. |
| Samsung | FL conclusion seems to be sufficient. |
| NTT DOCOMO | We are fine with FL conclusion. |
| LG | We prefer the Conclusion proposed by FL. |
| OPPO | We prefer the Conclusion proposed by FL. |

2nd round summary (Issue#2-2)

Companies’ views are summarized as follows:

- Prefer the conclusion proposed by FL

  - QC, Intel, InterDigital, CATT, Apple, Lenovo/Motorola Mobility, CMCC, Xiaomi, Nokia/NSB, Ericsson, Samsung, NTT DOCOMO, LG, OPPO

- Prefer the conclusion proposed by vivo

  - vivo

- Either is fine

  - Panasonic, ZTE

Even if we take the conclusion proposed by FL, options raised by vivo would be a good starting point of the discussion when we come back. In addition, some companies mentioned the possibility that the decision is made in feMIMO session. In such a case, we do not need to come back to this issue.

Based on the above, FL would like to suggests taking the following conclusion for now.

Proposed conclusion:

- The CovEnh discussion on the available slot counting for inter-cell mTRPs is deferred until further progress on the collision handling between UL channels/signals and multiple SSBs for inter-cell mTRPs is made in feMIMO session.

[Open] Issue#2-3: Use of SSBs of other serving cells in half duplex CA operation for the available slot determination

For CA case, the collision handling is performed according to following text in the same section of 38.213 v17.0.0.

If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

For RAN1#107-e, we discussed whether half duplex CA UEs need to take configurations from different DL carriers into account when determining the available slots. For the FL perspective, the agreement in RAN1#106bis-e means only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst of the serving cell where the PUSCH repetitions are performed are considered for the determination of available slots. However, due to the lack of time, FL suggested revisiting this issue in the CR phase.

According to contributions for RAN1#107bis-e, companies’ views for half duplex CA operation are summarized as follows.

- Only SSB in the serving cell where the PUSCH is transmitted is considered for available slot determination for the Type A PUSCH repetition transmissions. SSBs in different serving cells can be considered during PUSCH dropping after the available slot determination.

  - vivo [2], ZTE [3], Nokia/Nokia Shanghai Bell [4], CMCC [12], Ericsson [13]

- SSBs in different serving cells are also considered for the available slot determination.

  - Qualcomm [4]

- UL CA scenario is deprioritized in Rel-17 Coverage Enhancement WI.

  - Samsung [5]

1st round (Issue#2-3)

Do you agree on the following proposal?

FL proposal to Issue#2-3:

- For half duplex CA operation, select only one from:

  - Alt 1: Only SSB in the serving cell where the PUSCH is transmitted is considered for available slot determination for the Type A PUSCH repetition transmissions. SSBs in different serving cells can be considered during PUSCH dropping after the available slot determination.

  - Alt 2: SSBs in different serving cells are also considered for the available slot determination.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Alt. 1 |
| CATT | Fine with the proposal. Prefer Alt1. |
| Panasonic | We agree to the FL proposal. Our preference is Alt.1. |
| QC | Agree |
| Intel | For coverage enhancement, our view is that CA operation should be deprioritized. Even for HD-FDD case, it is mainly for single cell operation. We do not see the need to discuss this issue. |
| vivo1 | Fine. |
| ZTE | Support Alt 1, which aligns with previous agreements. |
| Samsung | We don’t agree. It was already discussed and there was no consensus in considering UL CA scenario, the reason being that UL CA is not a realistic use case for a UE in need of coverage enhancement. No need to split power, and also lose several dB due to MPR on top, when the UE is coverage limited. Also, phase consistency and DMRS bundling gains will be lost. Overall, that is a scenario that does not justify any further consideration at this stage. |
| Sharp | Support the proposal. We prefer Alt 1. |
| Apple | The proposal is for TDD half duplex CA UE, this is not the typical coverage enhancement scenario. In case of DL CA, the simple solution is enough. We prefer Alt1. |
| China Telecom | Support Alt 1. |
| OPPO | We prefer Alt1. |
| Huawei/HiSilicon | Alt.1. In our understanding, in the CR phase, spec impact is the focus. Considering that Alt 1 has been reflected in the current spec, we could directly discuss whether the additional spec impact required by Alt2 is necessary. We would like to suggest to rephrase the proposal to whether additional spec impact for Alt 2 is necessary. |
| Xiaomi | We prefer alt 1. |
| Ericsson | Support Alt1. |
| Lenovo, Motorola Mobility | Support Alt 1 |
| TCL | Alt 1 is prefer |
| LG | We prefer Alt 1. |

1st round summary (Issue#2-3)

Companies’ views are summarized as follows:

- Fine with the proposal

  - Prefer Alt 1: Nokia/NSB, CATT, Panasonic, ZTE, Sharp, Apple, China Telecom, OPPO, Huawei/HiSIlicon, Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL, LG

  - QC, vivo

- Do not discuss this issue

  - Intel, Samsung

The large majority prefers Alt 1. At the same time, Huawei provided a good suggestion rephrasing the proposal to whether additional spec impact for Alt 2 is necessary. Considering this and the 1st round inputs, FL would like to make the following proposal.

Updated FL proposal to Issue#2-3:

- For half duplex CA operation, no additional spec impact is necessary in terms of consideration of SSB(s) for the available slot determination.

2nd round (Issue#2-3)

Do you agree on the above updated FL proposal to Issue#2-3?

| Company | Comments |
| --- | --- |
| Intel | We are fine with the proposal. Our original intension was that CA with HD-FDD operations is not a typical scenario for coverage enhancement and should be deprioritized during maintenance phase, but we are okay with the proposal. It may be good to make a conclusion rather than a proposal. |
| vivo2 | Fine with the proposal. The proposal is necessary given the available slot determination with respect to half duplex CA operation is introduced in NR Rel-17, which should still be clear no matter which scenario we consider. Capacity and coverage may need to be considered at the same time in some scenarios, and we should consider future use cases as well. |
| Panasonic | Agree. |
| CATT | Fine to have this agreement or conclusion. |
| ZTE | Our understanding is we will consider not only the SSB in the serving cell but also the TDD configurations for available slot determination for half duplex CA. So, we may no need to emphasize SSB here, i.e., we can simplify the proposal to ‘For half duplex CA operation, no additional spec impact is necessary in terms of consideration of SSB(s) for the available slot determination’ . |
| Apple | Agree with FL’s proposal. |
| Lenovo, Motorola Mobility | Support the FL’s proposal |
| CMCC | Fine with FL’s proposal. We share the similar idea that the UL CA is not a typical use case for the coverage limited scenario. |
| Xiaomi | Fine with the proposal |
| Nokia/NSB | Support the FL’s proposal. |
| Ericsson | Agree ZTE’s proposal. The original use case is different SSB configurations and/or DL-UL-Configurations across multiple serving cells. |
| FL | @all, If everyone is OK, the proposal can be for a conclusion, not for an agreement. @ZTE and Ericsson, Thank for the suggestion. I agree with you that technically there is no need to limit to SSB. However, the issue we have been discussing here is the handling of SSBs for half duplex CA operation. Therefore, I prefer mentioning “SSB(s)” in the conclusion of this issue, unless it causes any harm. |
| Samsung | UL CA is not a typical scenario for coverage enhancement and some companies expressed this view even in this second round. Besides this conclusion has no specification impact. It seems no need for this conclusion. |
| LG | We are fine to make a conclusion. |
| OPPO | Fine with FL’s proposal. |

2nd round summary (Issue#2-3)

Companies’ views are summarized as follows:

- Fine with the conclusion proposed by FL

  - Intel, vivo, Panasonic, CATT, Apple, Lenovo/Motorola Mobility, CMCC, Xiaomi, Nokia/NSB, LG

- Prefer not limit to SSB

  - ZTE, Ericsson

- No conclusion is necessary

  - Samsung

The large majority supported the proposed conclusion. Moreover, although Samsung did not think making this conclusion is necessary, they also think that no additional spec impact is necessary.

FL would like to suggest closing this issue without a conclusion. However, according to the discussions, it seems to be commonly understood that no further discussion on the available slot determination for half duplex CA operation would be necessary.

[Open] Issue#2-4: Available slot counting for DG-PUSCH scheduled by DCI format 0_0 with other RNTI than TC-RNTI

So far, we have not made any agreement on whether available slot counting is applied to DG-PUSCH scheduled by DCI format 0_0 with other RNTIs than TC-RNTI. In the current TS38.214, the behaviors related to the available slot counting is specified for DCI format 0_1 and 0_2 but not for DCI format 0_0. Sharp [10] is proposing that available slot counting is NOT applied to DG-PUSCH scheduled by DCI format 0_0 with other RNTIs than TC-RNTI even when AvailableSlotCounting is enabled.

1st round (Issue#2-4)

Do you agree on the following proposal?

FL proposal to Issue#2-4:

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - No specification impact is expected.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Given that there is no specification impact, we don’t see the need to make an agreement on this issue. |
| CATT | Agree. |
| Panasonic | Agree. |
| QC | Legacy R15/R16 spec does not support PUSCH repetitions when triggering PUSCH using DCI Format 0_0. R17 made no changes in this regard. If repetitions cannot be triggered, then the question of available slot-based counting does not arise. We don’t think any clarification on this is necessary. |
| Intel | Given that there is no spec impact, we think it is more appropriate to capture this as conclusion, rather than proposal. |
| vivo1 | Agree. |
| ZTE | Fine with the proposal. |
| Samsung | No need for this agreement. UE behavior for the cases that the new feature applies (for DCI format 0_1 and 0_2) is already captured in the specifications. |
| Sharp | Agree. |
| Apple | Fine with the proposal. |
| OPPO | Agree. |
| Huawei/HiSilicon | Since no spec impact, we prefer not to spend time on this. |
| Xiaomi | Agree |
| Ericsson | Agree. |
| Lenovo, Motorola Mobility | Agree |
| TCL | Agree. |
| LG | We are not sure whether the clarification is necessary, but ok to make a conclusion. |

1st round summary (Issue#2-4)

No company objected the proposal. Several companies do not see the need to take it as an agreement. Therefore, FL would suggest taking it as a conclusion.

Proposed conclusion:

- A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is determined by physical slots (not available slots), irrespective of whether AvailableSlotCounting is enabled or not.

  - No specification impact is expected.

2nd round (Issue#2-4)

FL would like to bring the above proposed conclusion for email approval. Only if there is a strong concern, please indicate it in the table below.

| Company | Comments |
| --- | --- |
| QC | I understand that no spec impacted is expected. But what is the relationship between this conclusion and the K2 timeline? Wouldn’t K2 timeline clearly indicate the slot to transmit PUSCH in? Do we really need this conclusion? |
| FL | A slot for PUSCH scheduled by DCI format 0_0 with C-RNTI / CS-RNTI with NDI=1 / MCS-C-RNTI is the slot indicated by K2 value. We’ve already spent some time to collect companies’ views on this, and it seems companies have common understanding on the intended behavior. Why don’t we have a conclusion here? That may help to avoid unnecessary clarification discussions in the future. |

2nd round summary (Issue#2-4)

QC did not think the proposed conclusion is necessary. At the same time, this issue is somehow related to the Issue#2-6. Therefore, FL would like to suggest revisiting this issue after reviewing the companies’ opinions in Issue#2-6.

[Open] Issue#2-5: Available slot counting for PUSCH repetition Type A with K=1

So far, we have not explicitly made any agreement on whether available slot counting is applied to PUSCH with K=1. Note that, in Rel-16, the PUSCH repetition Type A concept covers both K=1 and K>1. Sharp [10] is proposing that available slot counting is applied to DG-PUSCH and CG-PUSCH with K=1 when AvailableSlotCounting is enabled.

1st round (Issue#2-5)

Do you agree on the following proposal?

FL proposal to Issue#2-5:

- A slot for DG-PUSCH repetition Type A with K=1 is determined by available slots when AvailableSlotCounting is enabled.

  - This includes the case of DG-PUSCH with CSI only.

  - No specification impact is expected.

- A slot for CG-PUSCH repetition Type A with K=1 is determined by available slots when AvailableSlotCounting is enabled.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Given that there is no specification impact, we don’t see the need to make an agreement on this issue. |
| CATT | For PUCCH, available counting happens only when NPUCCHrepeat>1, - 9.2.6 PUCCH repetition procedure … For NPUCCHrepeat>1, - the UE repeats the PUCCH transmission with the UCI over NPUCCHrepeat slots - a PUCCH transmission in each of …. If PUCCH is not repeated, available counting is not applied, and gNB should indicate a usable slot/symbol groups for PUCCH transmission. So, if we prefer that PUSCH strictly follows PUCCH mechanism, then when K=1, available slot counting should not be used. The gNB should indicate a correct slot (not overlapped with DL or SSB) by TDRA indication. But we are open to consider different behavior between PUSCH and PUCCH, if it is beneficial. |
| Panasonic | Agree |
| QC | Its not clear why AvailableSlotCounting would come into play for a dynamically triggered PUSCH with no repetitions. The slot where the first transmission is to be carried out should be clear from the grant itself. Even for CG-PUSCH with K=1, we think the gNB should be able to configure in such a manner that the first slot is available for transmission (gNB knows the tdd configs and ssb positions). We don’t see the value to this relaxation. |
| Intel | It is not clear to us whether this is needed. The enhancement on PUSCH repetition type A is intended for the case when repetition is applied for the PUSCH type A. K = 1 is clearly not for the repetition. We do not see the need to capture the proposal. |
| vivo1 | For the first bullet, the single slot for DG PUSCH normally can be always available slot which can be controlled by network. For the 2nd bullet, this seems to be not necessary. For coverage limited scenario, network will not configure repK=1. If latency is important, DG PUSCH will be used. So in such case, PUSCH transmission based on physical slot may be enough. |
| ZTE | We have similar view as CATT. In case of K=1, available slot counting should not be used to follow similar rules of PUCCH repetition. In addition, UE behavior would be also aligned with that of PUSCH scheduled by DCI format 0_0 discussed above. |
| Samsung | No need to introduce applicability of AvailableSlotCounting with no repetitions. |
| Sharp | Although this is our proposal, we also understand the point raised by CATT that available counting is not applied to the legacy PUCCH without repetitions. So, we are open to further discuss this issue. |
| Apple | Seems current specification already support k=1, or repK=1. If proposal is not agreeable, the related configuration need to be restricted. |
| China Telecom | Not necessary support AvailableSlotCounting for K=1. |
| OPPO | It is not necessary to capture this proposal. |
| Huawei/HiSilicon | For any K, the gNB should ensure the first indicated slot is an available slot. However, the only value provided by the proposal is to allow a gNB to indicate an unavailable slot as the first slot when K=1. It is unclear why it is beneficial. Therefore, for the simplicity of spec, we don’t feel the second bullet is necessary. |
| Xiaomi | It seems to be not necessary to introduce the AvailableSlotCounting when K=1 |
| Ericsson | For CG-PUSCH, it is hard to say if the available slot counting framework is beneficial CG-PUSCH with K=1. A Rel-15/16 UE transmits CG-PUSCH in a flexible slot, only if it is indicated as UL by dynamic SFI. If a flexible slot is indicated as DL by dynamic SFI, a legacy UE doesn't transmit PUSCH in the slot and looks for another UL slot or flexible slot, which is indicated as UL by dynamic SFI. But if a Rel-17 UE applies the rule "flexible symbols are available for CG-PUSCH" to determine available slots, the PUSCH transmission in the flexible slot, which is indicated by DL by dynamic SFI, will be dropped. We prefer that PUSCH repetition based on available slot applies to PUSCH transmission with K>1. |
| Lenovo, Motorola Mobility | Not needed to capture this proposal |
| TCL | For DG-PUSCH transmission with no repetition, gNB can dynamic indicated an available slot to UE, AvailableSlotCounting is no need. For CG-PUSCH transmission with no repetition, an available slot for UE could be also configured by gNB. |
| LG | No need to agree on this proposal. |

1st round summary (Issue#2-5)

Companies’ views are summarized as follows:

- Available slot counting is applicable to PUSCH repetition Type A with K=1

  - Sharp, Panasonic, vivo (for DG-PUSCH)

- Available slot counting is not applicable to PUSCH repetition Type A with K=1

  - CATT, QC, Intel, vivo (for CG-PUSCH), ZTE, Samsung, China Telecom, OPPO, Huawei/HiSilicon (for CG-PUSCH), Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL, LG

Companies have different views. In addition, several companies prefer different counting schemes for DG-PUSCH and CG-PUSCH when K=1. Therefore, FL suggests having further discussions.

Updated FL proposal to Issue#2-5:

For DG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for DG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for DG-PUSCH repetition Type A with K=1 is determined by available slots.

For CG-PUSCH when AvailableSlotCounting is enabled, select one from:

- A slot for CG-PUSCH repetition Type A with K=1 is determined by physical slots.

- A slot for CG-PUSCH repetition Type A with K=1 is determined by available slots.

2nd round (Issue#2-5)

Q1: Do you agree on the above updated FL proposal to Issue#2-5?

Q2: Please provide your preference for DG-PUSCH and for CG-PUSCH.

For your information, FL’s understanding on the current TS38.214 is described hereafter. When the spec says just “PUSCH repetition Type A”, it covers both K>1 and K=1. As described below, both N and K can be equal to 1, for DG-PUSCH. Therefore, “N∙K slots” in the spec covers the case of a single slot. This means the current TS38.214 already captures the UE behaviours for the PUSCH repetition Type A with K=1 with the available slot counting. If we agree on not support of the available slot counting for K=1, we have to make some changes to the current TS38.214.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. - the number of slots used for TBS determination N is equal to 1.

| Company | Comments |
| --- | --- |
| QC | Q1: For DG-PUSCH with K=1, we don’t think any clarification is necessary. The K2 timeline suffices. Q2: How is this related to 38.321 Section 5.8.2? The section seems to provide guidance on how to select the first slot. We don’t see a strong need to revisit this. |
| Intel | Q1: We prefer not to change current spec in case of no repetition. Q2: Our understanding is that K = 1 is a default value for PUSCH transmission. This does not mean that this is for PUSCH repetition. |
| vivo2 | Single slot DG-PUSCH can be said to be based on either available slot or physical slot since the dynamically scheduled physical slot is normally available. For the CG PUSCH, when K=1, it can still be up to network to decide whether to enable available slot counting for CG or not based on whether all first slots in each CG period can be available or not. For CG Type 2, network can even decide when to activate or deactivate the CG which is more flexible. Anyway, K=1 is not what we should optimize for coverage enhancement in our view as well. For the spec. text excerpted from 38.214 seems only for repetition factor determination and not change seems necessary. |
| Panasonic | Q1: Agree. Q2: We are OK with either way, but we prefer unified mechanism between DG-PUSCH and CG-PUSCH. We agree with FL’s understanding that if a slot is determined by physical slot, the specification change is necessary. |
| CATT | Q1: OK. Q2: We agree with FL’s understanding that K>1 and K=1 are coupled in many places in current spec. However, for the quoting part above, we have similar understanding with vivo, i.e. it is about how to ‘determine the value of K’, but does not touch ‘how to determine K slots’. We slightly prefer that K=1 cannot use available slot counting, since it seems not the goal of this feature, as pointed out by other companies. And, if spec change is necessary, it should be made in another place of 214. For example, adding ‘N*K>1’ as one of the condition of available slot determination. |
| ZTE | Q1: Ok Q2: In case of K=1, we prefer physical slots for slot determination for both DG and CG. The intention of introducing available slot counting is because gNB may not be able to avoid some collisions when scheduling/configuring PUSCH with repetitions K>1. However, it is not the case for K=1. |
| Apple | In our view, if k=1 is not applicable for available slot counting, then we need check any specification impacts. Otherwise, if k=1 is allowed, we go with FL’s proposal. For DG PUSCH, there is no difference whether PUSCH is based on physical slot or available slot. For CG PUSCH, re-use existing physical slot counting is simple and no change to TS38.321. |
| Lenovo, Motorola Mobility | Q1: Agree Q2: Generally either of the forms are acceptable, but slight preference towards physical slot for the case of K=1, as this is not a repetition but just a single transmission |
| CMCC | For the repetition factors, K=1is included in the set according to the previous agreements and also mentioned by the FL. Whether the counting is based on physical slots or available slots depends on the configuration AvailableSlotCounting is enabled or not. We do not see any need to set exceptional case for the available slot counting. We share the views from some company in the last round that there is no spec impact and there is no need to make any agreement for this issue. For Q1, we see no need for discuss this issue and make any agreement. For Q2, only available slot is selected for both CG and DG transmission. |
| Xiaomi | Q1: It is not necessary to further clarification since for DG-PUSCH, it is always scheduled by gNB and determined based on K2 value and for CG-PUSCH, reuse existing slot counting is enough. Q2: same view with intel. |
| Nokia/NSB | Q1&Q2: We share similar view with the FL that the current spec is clear enough for the cases K=1 and AvailableSlotCounting is enabled or not. We don’t see any ambiguity between K2 and the available slot when AvailableSlotCounting is enabled. This should be clear for counting on available slots for any K, not only for the case when K=1. |
| Ericsson | Q1: no need to discuss this or change the specification. Q2: We don't see the difference between two counting methods for CG-PUSCH, when K=1. For CG-PUSCH based on physical slots with K=1, if the allocated slot is a DL slot, the UE doesn't transmit in this periodicity and transmits the UL data in the slot of the next periodicity, as long as the data is still in the buffer. For CG-PUSCH with K=1, this is not cancellation, but deferral, which has the same effect as Rel-17 CG-PUSCH based on available slots. Therefore, we don't see the need to support counting available slots for CG-PUSCH with K=1. The current specification is fine. |
| FL | Thank you all for the feedback! @vivo, CATT, The intention I copied the above specification description is to show that “PUSCH repetition Type A” covers K=1 case. I did not intend that the copied description should be changed. Sorry if my explanation was confusing. @all, FL’s intention is not to convince anyone that no spec change is necessary. I’m open to either option - the physical slot counting or the available slot counting, for the case of K=1. My point is that the current TS38.214 already captures the available slot counting procedures for DG-PUSCH with both K>1 and K=1. If we agree on the physical slot counting for K=1, some specification changes are necessary, for example, adding ‘N*K>1’ to the associated sentences, as mentioned by CATT. If we agree on the available slot counting for K=1, no specification changes may be necessary. On the other hand, for CG-PUSCH, the current TS38.214 defines the available slot counting procedures only for K>1. If we agree on the available slot counting for K=1, specification changes may be necessary. Anyway, what we should do here is to pick one option for each of DG-PUSCH and CG-PUSCH. Hope this clarifies my intention. |
| LG | We have the similar understanding with Vivo. There is no need to differentiate available slot based counting and physical slot based counting for DG-PUSCH with K=1. It is desirable to assume that the network allocates PUSCH with K=1 in the available resource regardless of counting method. In case of CG-PUSCH transmission, PUSCH transmitting slot can be not available, and CG-PUSCH transmission is canceled in that case. So PUSCH transmission using physical slot and available slot based method cab be different. |
| OPPO | Q1: Agree. Q2: The current spec is clear enough for the cases K=1. |

2nd round summary (Issue#2-5)

For the counting method for K=1, companies’ preferences are summarised as follows:

- Physical slot counting

  - CATT, ZTE, Xiaomi (for CG-PUSCH), Apple?

- Available slot counting

  - CMCC, Nokia/NSB

- Either is OK

  - Panasonic, Lenovo/Motorola Mobility

- No difference (The physical slot indicated by K2 offset is ensured to be available)

  - QC, Intel?, vivo, Apple? (for DG-PUSCH), Xiaomi (for DG-PUSCH), Ericsson (for CG-PUSCH), LG (for DG-PUSCH)

This issue is highly related to the Issue#2-6. Therefore, FL would like to suggest revisiting this issue after reviewing the companies’ opinions in Issue#2-6.

[Open] Issue#2-6: The slot indicated by K2 offset for the available slot counting

During the 2nd round email discussion, Qualcomm suggested having the clarification that whether the slot indicated by K2 offset is by default an available slot or not. The clarification on this point seems important, because different understanding on this point would cause mis-aligned assumptions between the UE and the network.

2nd round (Issue#2-6)

Q1: Do you agree on that, for DG-PUSCH, the slot indicated by K2 offset shall be an available slot?

Q2: If yes to Q1, does that need to be captured in the specification?

Q3: Do you agree on that, for CG-PUSCH, the slot determined in 38.321 Section 5.8.2 shall be an available slot?

Q4: If yes to Q4, does that need to be captured in the specification?

| Company | Comments |
| --- | --- |
| QC | Q1: We believe this to be the current/legacy assumption. We prefer to retain this. We are not sure of impact to timelines otherwise (UCI Multiplexing is one example). Q2: If not explicitly captured so far, it might be good to clarify. Q3: Similar thinking as above. This aligns/reinforces legacy behavior. Q4: We may need to refer this to RAN2 as it could impact 38.321. |
| LG | Q1: It is our understanding that UE assumes the slot indicated by K2 offset is an available slot for DG-PUSCH. Q2: It seems not necessary. In TS 38.214, the omission of PUSCH transmission in not available slot is specified for multi-slot PUSCH transmission. The description is not applied for K=1. Thus, we understand the current spec already captures the behavior. For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].. Q3: CG-PUSCH slot indicated by K2 can be not available and the related behavior is specified in TS 38.214. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS 38.213].. Q4: It is not necessary since it is already captured. |
| Sharp | Q1: No strong view, as either works. Q2: If RAN1 agrees on “the slot indicated by K2 offset shall be an available slot”, it has to be captured in the specification. Q3: Similar to DG-PUSCH, no strong view. Q4: If RAN1 agrees on “the slot determined in 38.321 Section 5.8.2 shall be an available slot”, it has to be captured in the specification. @LG, “available slot” here should means “the slot where any symbol indicated by TDRA does not overlap with a DL symbol or an SSB symbol”, but not the slot without actual PUSCH transmission. Regarding the paragraphs (i.e., dropping rules for multi-slot PUSCH) you refer to above, we have different understanding. When the available slot counting is configured, the dropping rules for multi-slot PUSCH is applied only to the PUSCHs in available slots, not applied to unavailable slots, because there is no “PUSCH transmission” in an unavailable slot. In addition, these paragraphs have existed since Rel-15, and the intention is to clarify that, when the PUSCH is transmitted over multiple slots, PUSCH dropping defined in Clause 9, Clause 11.1 and Clause 11.2A is applied with slot-by-slot basis. It does NOT mean that UE can skip the PUSCH dropping procedures defined in Clause 9, Clause 11.1 and Clause 11.2A when the PUSCH transmission is performed within a single slot |
| OPPO | Q1&Q3: Agree. Q2&Q4: It seems not necessary. |

2nd round summary (Issue#2-6)

As this issue was raised in the middle of the 2nd round discussion, only 4 companies have provided the inputs. Therefore, FL suggests continuing the discussion in the next round.

Correction proposals

For this meeting, the following proposals for corrections on Rel-17 RAN1 specifications have been raised.

- Issue#3-1: Corrections on the definition of available slots

- Issue#3-2: Corrections on frequency hopping

- Issue#3-3: Corrections on available slot counting for FDD

- Issue#3-4: Corrections on available slot counting for TBoMS

[Open] Issue#3-1: Corrections on the definition of available slots

Agreement For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16.

In RAN1#106-e, it was agreed that for DG-PUSCH, the count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16, and this agreement means the K available slots are the earliest available slots. Huawei/HiSilicon [1] is raising the issue that this is not correctly captured in TS38.214v17.0.0. In order to fix this issue, Huawei/HiSilicon [1] is providing the following TP.

TP in R1-2200051

<Unchanged parts are omitted – 38.214> 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted – 38.214> For unpaired spectrum: - When AvailableSlotCounting is enabled, the UE determines the earliest N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. <Unchanged parts are omitted – 38.214>

1st round (Issue#3-1)

Companies are invited to provide their views on the above TP in R1-2200051.

| Company | Comments |
| --- | --- |
| CATT | Even without the modification, the definition is still clear to us. But we are fine with the TP if it can improve the reading. |
| Panasonic | We are fine with the above TP. |
| QC | Its should be clear to all companies that we have been referring to the earliest K or N*K slots. Don’t think there are any divergent views here. If we make this change here, and leave all other instances where we refer to K slots or N*K slots unchanged, it can create some confusion. The language is rather consistent all through the spec. Assuming “earliest” is the common understanding, we prefer to leave things unchanged. |
| Intel | Do not support the TP. The existing spec is already clear. |
| vivo1 | The TP is not necessary. For DG PUSCH the first slot can be always controlled by network and can be configured to be on available slot in our view (and we cannot find a reason why a network would dynamically schedule a first repetition to be on unavailable slot) . And after the first slot, the remaining available slot will be the latest available slots anyway with current wording. |
| ZTE | It’s not clear for us why such modification is needed. Further clarification is needed. |
| Samsung | It seems no need to add it as the UE behavior is already clear. Also there are other instances of “UE determines N∙K slots for a PUSCH transmission” when AvailableSlotCounting is enabled throughout 38.214. |
| Sharp | We are fine with the TP, but also agree with other companies that the current text is interpreted as “earliest”. In fact, we already have the following agreement: Agreement - For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2. - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2. - Note: The available slot determination is to be specified. - For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig. - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig. - Note: The available slot determination is to be specified.. |
| Apple | Seems current spec is clear enough. |
| OPPO | The definition of the existing spec seems to be already clear. |
| Huawei/HiSilicon | Agree. Thus we can correctly capture the previous agreement. |
| Ericsson | No need for the change, because there is no specification to allow UE to skip an available slot. |
| Lenovo, Motorola Mobility | Current spec seems to be correct |
| TCL | We are fine with the TP. |
| LG | The existing specification seems clear. |

1st round summary (Issue#3-1)

Companies’ views are summarized as follows:

- Accept the TP

  - CATT (if it can improve the reading.), Panasonic, Sharp, Huawei/HiSilicon, TCL

- The TP is not necessary

  - CATT, Intel, vivo, ZTE, Samsung, Apple, OPPO, Ericsson, Lenovo/Motorola Mobility, LG

Since companies have different views, FL suggest continuing the discussion.

2nd round (Issue#3-1)

Companies can provide additional comments below if any.

| Company | Comments |
| --- | --- |
| QC | Don’t think this TP is necessary. |
| CMCC | We prefer the original TP without modifications. Current specification is clear enough. |
| Nokia/NSB | We don’t think this TP is necessary. The current specification is clear. |

2nd round summary (Issue#3-1)

Companies’ views from the 1st and 2nd rounds are summarized as follows:

- Accept the TP

  - CATT (if it can improve the reading.), Panasonic, Sharp, Huawei/HiSilicon, TCL

- The TP is not necessary

  - CATT, Intel, vivo, ZTE, Samsung, Apple, OPPO, Ericsson, Lenovo/Motorola Mobility, LG, QC, CMCC, Nokia/NSB

Based on the above, FL’s observation is that the TP is not necessary.

[Open] Issue#3-2: Corrections on frequency hopping

For TS38.214v17.0.0, Apple [9] is raising the issue that, for PUSCH repetition type A frequency hopping, the current descriptions limit the PUSCH is scheduled by DCI format 0_1 or 0_2, RAR UL grant or DCI format 0_0 with TC-RNTI. However, intra-slot frequency hopping is also applicable to a single slot PUSCH transmission, for example, the case when PUSCH is scheduled by DCI format 0_0 with C-RNTI. In order to resolve this issue, Apple [9] is providing the following TP.

TP in R1-2200420

- 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

1st round (Issue#3-2)

Companies are invited to provide their views on the above TP in R1-2200420.

| Company | Comments |
| --- | --- |
| Nokia/NSB | The later sentence in the first paragraph quoted above should clarify the concern, i.e., “For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively” |
| CATT | We have the same understanding with Nokia. |
| FL | FL’s above explanation was wrong. This issue seems not about RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI but related to a single slot PUSCH transmission (e.g., PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI). Therefore, I made updates accordingly. Sorry for the confusion. |
| QC | This change seems reasonable. |
| Intel | We do not support this. For Msg3 initial and retransmission, it was captured under “For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI,…”. |
| vivo1 | Agree that something is wrong when both Msg3 repetition and TBoMS were captured in this section for Rel-17. However, simply removing the text (scheduled by DCI format 0_1 or 0_2) as proposed by Apple may not solve this issue as this would mean that FH of Msg3 repetition will also rely on the parameter frequencyHopping provided in pusch-Config to indicate which FH type (intra or inter-slot FH) to use which is wrong since we agreed that only inter-slot FH is supported when Msg3 is repeated meaning that no intra/inter-slot FH type indication is needed. For PUSCH repetition Type A other than the PUSCH scheduled by 0_1 or 0_2 RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| ZTE | We suggest to directly add the missing case (i.e., PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI) or choose the proposed TP from vivo. |
| Sharp | We support the proposal, as the proposed change would more aligned with Rel-16 descriptions, in terms of a single slot PUSCH transmission. |
| Apple | As indicated by ZTE, adding missing part is another choice. For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 or 0_0 with CRC scrambled by C-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, |
| Ericsson | Support Vivo’s TP. |
| Lenovo, Motorola Mobility | We are fine with th TP |

1st round summary (Issue#3-2)

All the companies that mentioned the support of a single slot PUSCH were thinking the specification should be fixed, though several variants of the TP were provided during the 1st round. Therefore, FL would like to suggest agreeing on the following proposal and then exact TP will be discussed in the next round.

FL proposal to Issue#3-2:

- Clause 6.3.1 of TS38.214v17.0.0 is updated such that it covers single slot PUSCH transmissions.

  - FFS: exact TP.

2nd round (Issue#3-2)

FL would like to bring the above proposed conclusion for email approval. Only if there is a strong concern, please indicate it in the table below.

| Company | Comments |
| --- | --- |
| Intel | We understand the intention but the proposal itself is somehow misleading. Can we simply agree on the TP directly rather than the proposal? |
| CATT | After exchanging the views, now we think the safest choice is adding the missing part (0_0 with CRC scrambled by C-RNTI) in the current spec, as pointed out by ZTE and Apple. Because we are not sure if there will be new feature/DCI/RNTI touching the intra-slot hopping in the future. If we adopt a TP excluding ‘the PUSCH scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI’, it somehow implies that any new feature will be involved by default. This potentially creates a hole to fix in the future. |
| ZTE | Agree with Intel that we can directly try to agree the TP. |
| Apple | Same view as Intel. |
| Lenovo, Motorola Mobility | Share same view to directly agree the TP |
| Nokia/NSB | We prefer to have a proposal such that in case RAN1 cannot agree to a TP in this meeting, the editor can help to fix the issue based on the proposal. However, the proposal should be clear by listing all cases instead of making it too generic. From FL’s explanation, the intention here seems to cover the case of PUSCH transmission scheduled by DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI. Then these cases should be specifically mentioned in the proposal. |
| FL | As several companies expressed their views that it is better to try to agree the TP directly, I would like to provide the following attempt. Is this agreeable? - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI or DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| vivo2 | We have different views on the TPs proposed by companies with respect to which one would be more future proof. In our view, it would be good to just exclude Msg3 repetition (this is the original intention of the editor to add DCI 0_1 and DCI 0_2 since intra-slot FH is not support for Msg3 repetition) instead of listing all possible RNTIs/DCI-formats (for a PUSCH supporting intra-slot FH) other than the DCI scheduling Msg3 repetition. This would also be good to avoid any future modifications in case type A repetition could be scheduled by any other DCI formats with CRC scrambled by any RNTI while intra-slot FH is still supported. With that, we tend to use following TP as we commented earlier: For PUSCH repetition Type A other than the PUSCH scheduled by 0_1 or 0_2 RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured …. Note that in Rel-17 before Msg3 repetition (and TBoMS) was introduced it’s For PUSCH repetition Type A (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured …. |
| FL | @all, Now we have four options. Is everyone OK to select one from them? Option 1: Adding the missing cases, as in the TP#1. Note that I made an editorial change (replacing “or” between MCS-C-RNTI and DCI format 0_1 by “,”), based on Intel’s feedback. TP#1 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. Option 2: Adding the exclusion of Msg3, as in the TP#2, proposed by vivo. TP#2 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A other than the PUSCH scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. Option 3: Adding the missing cases to the different place from Option 1, as in the TP#3, proposed by CATT. TP#2 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) or PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. Option 4: Adding a separate paragraph for single slot PUSCH transmission scheduled by DCI 0_0, proposed by Nokia. @Nokia, If you can provide some TP, that would be very helpful for companies when comparing the options. |
| Nokia/NSB | @FL Option 3 from CATT looks good to us. We suggest a minor modification highlighted in yellow as follows. TP#2 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) or and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |
| FL | @Nokia, Thank you for the feedback! Then, I will remove option 4. @all, TP#3 is updated according to the suggestion from Nokia. Option 1: Adopt TP#1 TP#1 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. Option 2: Adopt TP#2 TP#2 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A other than the PUSCH scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. Option 3: Adopt TP#3 TP#3 - 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots For PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and for TB processing over multiple slots (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH) and for PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingDCI-0-2 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. For PUSCH repetition Type A scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, a UE is configured for frequency hopping by the frequency hopping flag information field of the RAR UL grant, and by the frequency hopping flag information field of DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. One of two frequency hopping modes can be configured: - Intra-slot frequency hopping, applicable to single slot and multi-slot configured PUSCH transmission and multi-slot PUSCH transmission scheduled by DCI format 0_1 or 0_2 and each of multiple PUSCH transmissions scheduled by a DCI if the higher layer parameter pusch-TimeDomainAllocationListForMultiPUSCH is configured. - Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.. |

2nd round summary (Issue#3-2)

Several companies expressed their views that it is better to try to agree the TP directly, rather than agreeing the FL proposal. During the 1st and 2nd round discussions, 4 options of the TP have been raised to resolve the issue. FL would like to make the following proposal as a result of this round.

Updated FL proposal to Issue#3-2:

- Select one from the following options:

  - Option 1: Adopting TP#1 (Adding the missing cases)

  - Option 2: Adopting TP#2 (Adding the exclusion of Msg3)

  - Option 3: Adopting TP#3 (Adding the missing cases to the different place from Option 1)

[Open] Issue#3-3: Corrections on available slot counting for FDD

For TS38.214v17.0.0, Sharp [10] is pointing an inconsistency between the main bullet and its sub-bullet, because main bullet says “N∙K consecutive slots” but the sub-bullet implies “N∙K slots” may be non-consecutive. In order to resolve this issue, Sharp [10] is providing the following TP.

TP in R1-2200500

- 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - For the case of non reduced capability half-duplex UE irrespective of whether AvailableSlotCounting is enabled or not, or the case of reduced capability half-duplex UE and when AvailableSlotCounting is not enabled, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, he UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - In case of non reduced capability half-duplex UE irrespective of whether AvailableSlotCounting is enabled or not, or in case of reduced capability half-duplex UE when AvailableSlotCounting is not enabled, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

1st round (Issue#3-3)

Companies are invited to provide their views on the above TP in R1-2200500.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Adding “For the case of non reduced capability half-duplex UE…” in the main bullet and making equal bullet level makes these paragraphs look like focusing on half-duplex UE only. In fact, the current structure of the specification is clearer since the sub-bullet can be seen as an exception of the main bullet. |
| CATT | OK. |
| Panasonic | We are fine with the above TP. |
| QC | It is correct that for redcap HD UEs, the N*K slots need not be consecutive. This needs to be fixed. |
| Intel | We do not see the need to change the current spec. The existing spec is already clear. |
| vivo1 | Seems fine except a typo “the UE”. |
| ZTE | Fine with the TP. Two minor editorial comments: 1) ‘he’ should be ‘the’ for the second sub-bullet of the TP in section 6.1.2.1, and 2) the last sub-bullet of the TP in section 6.1.2.3.1 should have the same level with the sub-bullet above. |
| Sharp | Support the TP with the corrections suggested by vivo and ZTE. We do not agree on Nokia’s statement “the sub-bullet can be seen as an exception of the main bullet”. For the paired spectrum, people can see almost the same structure of the main bullet and the sub-bullet (see below), where the sub-bullet is the further clarification of the main-bullet, but not an exception. Two parts with the same bullet-point structure leading to two different interpretations would cause unnecessary confusion and have to be avoided. - When AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.. |
| Apple | According to our understanding, HD-FDD UE supporting TBoMS is still open for now. |
| OPPO | OK with the TP. |
| Xiaomi | Fine with the TP |
| Ericsson | Agree with ZTE’s comments. |
| Lenovo, Motorola Mobility | Fine with the TP |
| TCL | Agree with ZTE’s comments. |

1st round summary (Issue#3-3)

Companies’ views are summarized as follows:

- Support the TP (with a couple of typos fixed)

  - CATT, Panasonic, QC, vivo, ZTE, Sharp, OPPO, Xiaomi, Ericsson, Lenovo/Motorola Mobility, TCL

- The TP is not necessary

  - Nokia/NSB, Intel

Since companies have different views, FL suggest continuing the discussion.

2nd round (Issue#3-3)

Companies can provide additional comments below if any.

| Company | Comments |
| --- | --- |
| Intel | For the proposed TP, we do not have definition of non-RedCap UEs in the spec. We still prefer the original structure and if needed, we can make the paragraph for RedCap HD-FDD UEs as a sub-level for the first paragraph. Further, it is not clear to us whether the following update is necessary. At least based on our understanding, RedCap HD-FDD UE does not support repetition scheme introduced in NR-U. If there is further agreement to support this feature from RedCap, we can update the text accordingly. “If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. |
| CATT | FYI, In the 1st GTW, we discussed the following proposal in 8.8.1.2: FL’s proposal 5 The use of TBoMS for HD-FDD UE with counting on available slot is supported.. The majority seems fine with it, but still need to have offline discussion, since a few companies have some concern. We can wait for more progress in 8.8.1.2, if not hurry. |
| Nokia/NSB | We share similar view with Intel that the definition of non RedCap UE has not been discussed and it should not be used in specs languages. The concern from the original text seems only be that the main bullet referred to “N*K consecutive slots” but the sub-bullet referred to “N*K slots”. However, the sub-bullet can be considered as an exception of the main bullet (that’s why it’s a sub-bullet but not equal bullet level). Therefore, we don’t see the need to this TP. |
| FL | @all, During the 1st round, quite a few companies accepted/agreed on the proposed change, that means many companies see the inconsistency in the current specification. At the same time, Intel pointed out that the proposed TP includes the case of RedCap HD-FDD operation with NR-U functions. In addition, Nokia’s interpretation is that the sub-bullet should be considered as an exception of the main bullet. To make a progress, FL would like to suggest considering the following alternative TP, which FL believes is a middle ground compromise. Companies are encouraged to check it and to provide their comment on it. - 6.1.2.1 Resource allocation in time domain [Omitted] For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2, irrespective of whether AvailableSlotCounting is enabled or not. - Except for the case of reduced capability half-duplex UEwith AvailableSlotCounting enabled, in which case a slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on the TDRA information field value in the RAR UL grant. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on the TDRA information field value in the DCI scheduling the PUSCH. [Omitted] - 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant [Omitted] For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum: - Irrespective of whether AvailableSlotCounting is enabled or not, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - Except for the case of reduced capability half-duplex UE with AvailableSlotCounting enabled, in which case a slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.. |
| QC | I tend to agree with the original assessment that this should not be a sub-bullet under the current main bullet. Determining N*K consecutive slots is not applicable to RedCap HD UEs. I think we should state clearly what the behavior for such UEs should be as a separate clause. Would the following work: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2, irrespective of whether AvailableSlotCounting is enabled or not. -Reduced capability half-duplex UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. |
| Sharp | We support QC’s TP with adding “configured with AvailableSlotCounting enabled” as below, since HD-FDD UEs use physical slot counting when the available slot counting is not configured. - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2, irrespective of whether AvailableSlotCounting is enabled or not. -Reduced capability half-duplex UE configured with AvailableSlotCounting enabled determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. We can also live with FL’s compromise. |

2nd round summary (Issue#3-3)

Quite a few companies accepted/agreed on the proposed change, that means many companies see the inconsistency in the current specification. At the same time, Intel pointed out that the proposed TP includes the case of RedCap HD-FDD operation with NR-U functions. In addition, Nokia’s interpretation is that the sub-bullet should be considered as an exception of the main bullet. Hence, Intel and Nokia/NSB prefer keeping the current spec.

During the 2nd round, several compromise solutions have been provided. FL would like to suggest having further discussions based on those solutions.

[Pending] Issue#3-4: Corrections on available slot counting for TBoMS

For TS38.214v17.0.0, Sharp [10] is pointing out that the definition of available slots for TBoMS with a configured grant is missing. In order to resolve this issue, Sharp [10] is providing the following TP.

TP in R1-2200500

##### 6.1.2.3.3 Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission ocasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at - The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}. - Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}. If the UE determines that, for a transmission occasion, the number of symbols available in a slot for the PUSCH transmission of TB processing over multiple slots is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion. For unpaired spectrum: - The UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst. - A slot is not counted in the number of N∙K slots for PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For paired spectrum and SUL band: - For the case of non reduced capability half-duplex UE, the UE determines N∙K consecutive slots for a PUSCH transmission of a TB processing over multiple slots. - For the case of reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a TB processing over multiple slots. A slot is not counted in the number of N∙K slots for a PUSCH transmission of a TB processing over multiple slots, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots, the UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. A Type 2 PUSCH transmission with a configured grant of TB processing over multiple slots is omitted in a slot according to the conditions in clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213].

The same TP is also proposed in AI 8.8.1.2. Since this proposal is more related to TBoMS, FL suggests discussing this proposal in AI 8.8.1.2.

References

- R1-2200051	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2200086	Remaining issues on enhancement for PUSCH repetition type A	vivo

- R1-2200111	Discussion on remaining issues for enhanced PUSCH repetition type A	ZTE

- R1-2200160	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

- R1-2200205	Enhancements on PUSCH repetition type A	Samsung

- R1-2200301	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2200334	Enhancements on PUSCH repetition type A	OPPO

- R1-2200379	Remaining details of enhancements on PUSCH repetition type A	Intel Corporation

- R1-2200420	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2200500	Enhancements on PUSCH repetition type A	Sharp

- R1-2200518	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2200588	Remaining issues on PUSCH repetition type A enhancement	CMCC

- R1-2200655	Remaining Issues for PUSCH Repetition Type A Enhancement	Ericsson

List of agreements

Agreements in RAN1#104-e

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Agreements in RAN1#105-e

Agreement:

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Agreement:

- Down-selection in RAN1#106-e:

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

Agreement:

In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17.

- {20, 24, 28}

Agreement:

- Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition.

  - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

Agreement:

- If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Agreement:

Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded)

- Alt 1-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- Alt 1-B’ consisting of two steps

  - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

  - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability

- Alt 2-A consisting of a single step

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Alt 2-B consisting of two steps

  - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

    - FFS timeline for the dynamic signalling

  - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

Agreements in RAN1#106-e

Agreement:

- For Rel-17 PUSCH repetition Type A without joint channel estimation, no new inter-slot frequency hopping mechanism is introduced.

Agreement

Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots.

- Alt 1-B consisting of two steps

- Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

- FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s)

Agreement

For PUSCH repetition Type A for Rel-17 CG-PUSCH, semi-static flexible symbol is considered as available.

Agreement

For PUSCH repetition Type A for Rel-17 DG-PUSCH, semi-static flexible symbol is considered as available.

Note: The applicability for Msg 3 is to be discussed in 8.8.3

Agreement

- DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists.

Agreement

- For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16.

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Agreements in RAN1#106bis-e

Working Assumption is confirmed

Working Assumption

The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

Conclusion:

For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots.

Agreement

For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots.

Observation

- Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions.

Agreement

- For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2.

  - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2.

  - Note: The available slot determination is to be specified.

- For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig.

  - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig.

  - Note: The available slot determination is to be specified.

Agreement

- Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots.

  - Any other RRC configuration is not considered for the determination of available slots.

Agreement

- The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots.

- The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.

Agreements in RAN1#107-e

Agreement

- The counting based on available slots is applicable to unpaired spectrum, paired spectrum and SUL

  - For paired spectrum and SUL except HD-FDD, all slots are considered as available slots in the first step of determining the available slots.

Agreement

- For HD-FDD RedCap Ues supporting the counting based on available slots.

  - For CG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

    - A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  - FFS: For DG-PUSCH

  - Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.

Agreement

- Rel-17 does not support numberOfRepetitions-r17 for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- repK-r17 supporting up-to-32 repetitions is introduced and is applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH (irrespective of the activating DCI format).

  - Note: No RAN1 spec impact is expected.

  - The possible values of repK-r17 includes 16 and 32. FFS: other values.

- numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition type A.

Agreement

- All the following combinations support the counting based on available slots.

  - DG-PUSCH with Rel-15 repetition factor

  - Type-1 CG-PUSCH with Rel-15 repetition factor

  - Type-2 CG-PUSCH with Rel-15 repetition factor

  - DG-PUSCH with Rel-16 repetition factor

  - Type-2 CG-PUSCH with Rel-16 repetition factor

  - DG-PUSCH with Rel-17 repetition factor

  - Type-1 CG-PUSCH with Rel-17 repetition factor, if supported in Issue#1-1

  - Type-2 CG-PUSCH with Rel-17 repetition factor

Conclusion

  - Rel-17 PUSCH repetition Type A with K>1 does not support PUSCH transmission without UL-SCH.

Agreement

  For repK-r17,

  The value range of repK-17 is {1, 2, 4, 8, 12, 16, 24, 32}.

  repK-r17 is included in ConfiguredGrantConfig.

  When repK-r17 is provided, the legacy repK is not provided.

Agreement

  For HD-FDD RedCap Ues supporting the counting based on available slots.

  For DG-PUSCH, ssb-PositionsInBurst is used in the first step of determining of available slots.

  A slot is not counted in the number of available slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionInBurst.

  Note: Neither tdd-UL-DL-ConfigurationCommon nor tdd-UL-DL-ConfigurationDedicated is configured for FDD.