3GPP TSG RAN WG1 #107-bis	                      R1-2200727

# e-Meeting, January 17th – 25th, 2021

Agenda Item:		8.7.1.2

Source:				Moderator (Samsung)

Title:					Moderator summary #2 on TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion/Decision

# Introduction

This document provides summary of contributions [1-23] submitted to agenda item 8.7.1.2 for RAN1#107bis-e meeting. According to the proposals in contributions [1-23], the remaining issues for supporting TRS/CSI-RS occasion(s) for idle/inactive UEs can be divided into the following parts, wherein the details are captured in Section 2 to 6, respectively:

- Section 2: TRS availability indication

  - Issue 1-1: TRS availability indication during a valid duration

  - Issue 1-2: Consistent TRS availability indication from PEI and paging PDCCH

  - Issue 1-3: Additional design for PEI PDCCH based TRS availability indication

- Section 3: TRS validity duration

  - Issue 2-1: Confirm applicable values

  - Issue 2-2: Clarifications on reference point

- Section 4: Signalling method of TRS availability indication

  - Issue 3-1: How to enable L1 based TRS availability indication

  - Issue 3-2: Default assumption of TRS availability

  - Issue 3-3: Whether support SIB based availability indication

- Section 5: TRS resources configuration

  - Issue 4-1: Determine number of TRS resources per TRS resources set

  - Issue 4-2: How to configure scrambling ID

  - Issue 4-3: Whether to reduce maximum number of TRS resource sets

  - Issue 4-4: Other potential configuration issues

- Section 6: Others

The issues in this document are color coded with High Priority, Medium Priority, or Low Priority.

Per chairman’s instruction, this document will be used for the following email discussion:

[107bis-e-R17-PowSav-02] Email discussion regarding TRS/CSI-RS occasions for idle/inactive UEs – Qiongjie (Samsung) - 1st check point: January 20 - Final check point: January 25

For the first round discussion, please kindly provide your comments on moderator proposals or questions tagged ‘[1RD]’ before 01/18 UTC 4:00.

For the second round discussion, please kindly provide your comments on moderator proposals or questions tagged ‘[2RD]’ before 01/19 UTC 7:00.

# TRS availability indication

The following were agreed regarding TRS availability indication:

From RAN1#107-e: Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration From RAN1#106bis-e Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling

In contributions [1-22], proposals regarding remaining issues for TRS availability indication are captured in table below:

| Huawei, HiSilicon | Proposal 1: Availability indication with value 1 can be transmitted multiple times during the validity duration after the previous indication of value 1 Proposal 2: Different value of availability indication filed is allowed during the validity duration, if the value ‘0’ means ‘ignored’. Proposal 3: Value ‘0’ of TRS availability indication means ‘ignored’. Proposal 4: The UE behavior on triggering validity duration is defined as - If there is at least one bit indicating value ‘0’ while the current UE assumption of this bit is ‘1’ (i.e. at least one bit ‘1 --> 0’), the indication is ignored; - Otherwise, the validity duration is extended. Proposal 6: Adopt text proposal 1 in section 2.1 ------------------------ Start of Text Proposal 1--------------------------------------------- < Unchanged parts are omitted > 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to includes the [first/last] MO of a PO, where the PO frame that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. UE ignores the TRS availability indication field if there is at least one bit indicating value ‘0’ while the previous indication of the bit is ‘1’. < Unchanged parts are omitted > --------------------------- End of Text Proposal 1-------------------------------------------- Proposal 7: Support to only include one indication bit in TRS availability indication field in DCI format 2_7, which is associated with the same QCL reference (SSB index) of the monitoring occasion for the DCI format 2_7, in addition to the support of the same TRS availability indication field size as that in paging DCI. - The same association between SSBs and an indication bit is applied to both paging DCI and PEI DCI |
| --- | --- |
| ZTE, Sanechips | Proposal 1: It should be clarified that whether UE can detect DCI format 2-7 for TRS availability indication without supporting paging enhancement. Proposal 2: The start of TRS indication in DCI format 2-7 should be explicitly configured. Proposal 3: The code point of “0” of the L1 based indication is interpreted as “reserved” to avoid the extension of the on-going validity time duration. Observation 1: The drawback of interpreting the code point of “0” of the L1 based indication as “reserved” is that for the UEs which newly camp on the cell cannot use TRS for sync even though the TRS is transmitted by NW. Proposal 4: The transmission times of availability indication during the validity duration depend on NW implementation. Proposal 5: A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a paging cycle. |
| Vivo | Proposal 1: If L1 availability indication is enabled, and if PEI is configured, availability indication is provided in both PEI and paging DCI. Observation 1: PO for a UE can be located in the first frame of a DRX cycle, and PEI is offset to PO in frame level, which lead to the frame for PEI may be in previous DRX cycle of the associated paging PDCCH. Proposal 3: Availability indication can be transmitted multiple times during the validity duration. Proposal 4: For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI, i.e., UE assumes the L1 availability is detected in the associated paging DCI when determine the starting time of validity duration. - Adopt the TP in Annex of R1-2200084. ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap in the DCI format 1_0 with the TRS availability indication field indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of ‘1’ for the bitmap in the DCI format 2_7 indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH format 1_0 associated with the DCI format 2_7. |
| TCL | Proposal 1: Support the same principle/mechanism for TRS availability indication of paging PDCCH and PEI based signalling. PEI DCI should not be restricted to the availability indication of only those RS resources, which have the same QCL reference as L1 availability indication. Proposal 2: The number of transmissions of availability indication during the validity duration can be up to the gNB Implementation. |
| Spreadtrum | Proposal 2: There is no need to specify that the availability indication is transmitted only once during the validity duration. |
| CATT | Proposal 3: The size and content of TRS availability indication field of DCI format 2_7 should be same as that of the paging DCI. Proposal 5: Inconsistent L1 based indication during the time duration should not be supported for small validity duration length. Proposal 6: The following scheme can be used for support of TRS availability indication in validity duration: • gNB transmits unavailability indication in validity duration. - gNB should ensure presence of TRS at configured occasion(s) before transmitting unavailability indication to UE during validity duration. Proposal 7: PEI based availability indication should have same reference point and validity duration length with that of its associated paging DCI |
| OPPO | ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an Id i which indicate the association to the ith bit within the bitmap. The UE can be additionally provided a multiple, by validityDuration, of a number of frames provided by defaultPagingCycle; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] which is the first frame within the default DRX cycle that a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets is received, where T is provided by defaultPagingCycle. ****************** End of text Proposal*************************************** |
| Sony |  |
| Intel | Proposal 2: It is expected that TRS availability indication would be consistent across PEI DCI and paging DCI received in a given DRX cycle, i.e., paging DCI would not change UE’s assumption on the availability of TRS resource indicated by PEI DCI. Proposal 3: Same definition/configuration for validity duration and reference point is applicable to both PEI DCI and paging DCI based TRS availability indication. Proposal 4: The bit value 1 and 0 in the bitmap of the TRS availability indication field have the following definitions: • Bit value 1: For each bit indicated as ‘1’ in the availability indication field, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 indication. • Bit value 0: For each bit indicated as ‘0’ in the availability indication field, the UE ignores the bit and keeps the existing/current assumption on the availability of the corresponding TRS resource set(s). |
| Xiaomi | Proposal 3: For L1 indication configuration in paging DCI and PEI, Alt 1: UE does not expect L1 TRS availability indication is configured both in paging DCI and PEI. Alt 2: If L1 TRS availability indication is configured both in paging DCI and PEI, for UE monitoring PEI, UE would acquire L1 indication in PEI and ignore L1 indication in paging DCI. Proposal 4: L1 indication can be transmitted multiple times during validity duration. Proposal 5: For a TRS resource set group that is indicated available, during its validity time, if another L1 TRS availability indication is detected and bit “1” is indicated for the TRS resource set group, the validity time of the TRS resource set group is extended; if bit “0” is indicated for the TRS resource set group, the validity time of the TRS resource set group is not extended. |
| CMCC | Proposal 1. The availability indication can be transmitted more than once during the validity duration. For value ‘0’, UE assumes the TRS is not present, and for value ‘1’, UE assumes the TRS is present for the validity duration if it is not changed to value ‘0’ during the validity duration. |
| Panasonic | Proposal 2: To extend the current agreement to support unavailability indication by the following: - For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability and unavailability information for configured RS resources using a bitmap, where each bit indicates whether associated TRS resource(s) are available or not by ‘1’ and ‘0’. Proposal 3: To monitor the PEI-based L1 indication is up to UE implementation Proposal 4: No need to specify the availability indication is transmitted only once during the validity duration. Proposal 6: No need to limit the UE assumption on whether UE expects inconsistent L1 based indication during the validity time duration. |
| Samsung | Proposal 2: L1 availability indication based on DCI format 2_7 and paging DCI are enabled simultaneously, and UE expects the same indication content and validity duration if TRS availability is transmitted in both Proposal 3: A UE can receive a TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication • if the bitmap of the new TRS availability indication field is all “0s”, the UE ignores the TRS availability indication field; otherwise, the UE assumes the new TRS availability indication field overwrites the previous TRS availability indication • Adopt TP#1 for TS 38.213 |
| Apple | Proposal 4: It is supported that another availability indication is received before the end of a previous validity duration. • Bit ‘1’ means that the corresponding TRS configuration(s) are available in the corresponding validity duration, while bit ‘0’ is ignored. |
| InterDigital | Proposal 1: PEI availability information is the same as the paging PDCCH availability information Proposal 2: The same availability information should be transmitted in all paging PDCCH occasions in a validity duration. |
| Sharp | Proposal 4: Support providing availability indication via PEI for RS resources with the same QCL references Proposal 5: TRS availability indications in PEI-DCI should have higher priority than the availability info in paging PDCCH |
| LG |  |
| Ericsson | Proposal 2 For supporting TRS availability in the PEI DCI, following is supported : a. Configuration of the bitfield within the DCI via an explicitly configured start position b. Configuration of separate validity time value for PEI DCI (vs Paging DCI). c. Reference point is the SFN of the first PF associated with the default paging cycle for which UE receives PEI |
| Qualcomm | Proposal 2: Network configures the start bit location of the TRS availability bitmap in PEI PDCCH in TRS configuration. Observation 2: It is an unnecessary waste of signaling if the TRS availability indication is transmitted only once during the validity duration, e.g., by transmitting all 0 bits in the other bitmap in PEI and paging PDCCHs. Observation 3: The validity duration of TRS availability indication with floating start does not solve the missed detection problem. Observation 4: Validity duration is intended to avoid a prolonged transmission of the TRS if network does not have PEI or paging PDCCH to terminate the transmission of the TRS when no UE is paged for a long time. Consistency of TRS availability information is the result of the validity duration if no more TRS availability indication is received before the expiration of the validity duration. There is no need to force the consistency on a new TRS availability indication before the expiration of the validity duration. Proposal 4: UE follows the last received TRS availability information until validity duration for this indication signaling expires or UE receives another TRS availability indication, whichever occurs to the UE first. |
| MediaTek |  |
| Nokia | Observation: From UE perspective there does not appear to be inconsistency in TRS availability based on L1 availability indication. Observation: If PO and associated PEI fall to different default paging cycles, the L1 availability indication could be different in the paging DCI and PEI. Observation: Having different L1 indication value in paging DCI and associated PEI would not cause any UE ambiquity as UE can determine the availability assumption based on the latest L1 availabilty indication with value ‘1’. |
| Nordic | Proposal-1: When UE receives a first indication indicating TRS is present within a DRX cycle and later receive a second indication indicating that TRS are not present within the DRX cycle, a UE assumes that TRS is present in the DRX cycle. Proposal-5: The same DCI field (as in Paging DCI) is present also in PEI, except that validation is determined based on DRX cycle of associated PO • UE expects that PEI and associated PO would have consistent indication. • DCI field is present in PEI when its payload-position is configured by gNB in PEI DCI format. |
| Lenovo | Observation 1: If TRS availability indications in paging DCI are transmitted in multiple paging cycles within a validity duration, different UEs detecting paging DCI in different paging cycles would differently interpret a validity interval for the TRS availability indication. Proposal 2: TRS availability indication is transmitted only in one default DRX cycle within a validity duration of the TRS availability indication. |

According to the above proposals, the remaining issues regarding TRS availability indication include:

- Issue 1-1: TRS availability indication during a valid duration

- Issue 1-2: Consistent TRS availability indication from PEI and paging PDCCH

- Issue 1-3: Additional design for PEI PDCCH based TRS availability indication

## 2.1 <1st round discussion>

Companies views regarding Issue 1 from contributions [1-23] were summarized in sub-sections below:

### Issue 1-1: Availability indication during a validity duration

Open problems:

- Whether the availability indication is transmitted [only once] during the validity duration

  - If not, UE can receive new TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication, then

    - Whether the new availability indication can be same or different from previous availability indication

    - Whether the reference point of validity duration slide/extend/restart simultaneously or independently for all groups of TRS resource sets

- Whether and how to keep consistent availability information between new TRS availability indication and previous TRS availability indication

- Whether to support unavailability indication or clarify bit “0” in TRS availability indication field

Potential solutions:

Based on the proposals in Section 2, there are the following are alternatives proposed companies to resolve issue 1-1.

- Alt1: UE can receive new TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication

  - Alt1-1: the new TRS availability indication can be same or different from previous TRS availability indication.

    - Bit ‘1’ means that the corresponding TRS configuration(s) are available in the corresponding validity duration, while bit ‘0’ is ignored

  - Alt1-2: Availability indication with value 1 can be transmitted multiple times during the validity duration after the previous indication of value 1.

    - If there is at least one bit indicating value ‘0’ while the current UE assumption of this bit is ‘1’ (i.e. at least one bit ‘1 --> 0’), the indication is ignored

    - Otherwise, the validity duration is extended

    - Value ‘0’ of TRS availability indication means ‘ignored’

  - Alt1-3: For value ‘0’, UE assumes the TRS is not present, and for value ‘1’, UE assumes the TRS is present for the validity duration if it is not changed to value ‘0’ during the validity duration.

  - Alt1-4: UE ignores the new TRS availability indication if it has bitmap of all “0s”, otherwise the new TRS overwrites the previous TRS availability indication

  - Alt1-5: UE follows the last received TRS availability information until validity duration for this indication signaling expires or UE receives another TRS availability indication, whichever occurs to the UE first.

- Alt2: UE can’t receive new TRS availability information before the expiration/end of validity duration associated with previous TRS availability indication

  - the new TRS availability indication field if received has bitmap of all “0s”

Motivations

- for Alt1, may include

  - To allow UE who newly enters a cell or UEs miss detect previous availability to utilize the TRS occasions in the cell for power saving

  - It is an unnecessary waste of signaling if the TRS availability indication is transmitted only once during the validity duration

- for Alt2, may include

  - Only consistent TRS availability information can be transmitted before the validity duration expires to avoid

    - miss detection problem

    - different UEs detecting paging DCI in different paging cycles would differently interpret a validity interval for the TRS availability indication

  - To avoid NW overhead for different TRS availability indications

[1RD]

Please a) update or provide your views for alternative(s) you support to resolve open problems for issue 1-1

| Support | Companies |
| --- | --- |
| Alt1-1 | Apple, ZTE, [vivo, TCL, Spreadtrum], Intel, Xiaomi, Nordic (in principle), MediaTek, LG |
| Alt1-2 | HW |
| Alt1-3 | CMCC, Panasonic |
| Alt1-4 | Samsung |
| Alt1-5 | Qualcomm |
| Alt2 | CATT, Lenovo |
| Others |  |

and, b) provide additional comments (e.g. justifications/clarification, concerns, or solutions to open problems) below.

| Company | Comments |
| --- | --- |
| Nordic | word “ignore” is not a spec language, how about something like this When UE receives a first indication indicating idle mode TRS resource set group is present within a DRX cycle and later receive a second indication indicating that idle mode TRS are not present within the DRX cycle, a UE assumes that TRS resource set group is present in the DRX cycle. other option is to specify only “1”, and not to saying anything about “0” in the spec When UE receives a first indication indicating ‘1’, UE assumes idle mode TRS resource set group is present within a DRX cycle. |
| Xiaomi | There can be multiple bits in a TRS availability indication, each bit corresponds to one TRS set group. It is more clear to describe from each TRS set group’s perspective instead of describing from a TRS availability indication’s perspective. The following example can be considered, For a TRS resource set group that is indicated available, during its validity time, if another L1 TRS availability indication is detected and bit “1” is indicated for the TRS resource set group, the validity time of the TRS resource set group is extended; if bit “0” is indicated for the TRS resource set group, the validity time of the TRS resource set group is not extended. |
| CMCC | For Alt1-1, it may cause inconsistence about the meaning of bit “0”. For example, if UE receive new TRS availability indication before the end of validity duration associated with previous TRS availability indication, UE ignores “0”, which means the corresponding TRS is still available until the end of validity time. But after the validity time, UE receives a new TRS availability indication, the bit “0” means corresponding TRS is not available during the validity time. That is the bit “0” can either means “available” or “unavailable”. Thus, we think the behavior of bit “0” should be specified, e.g., bit “0” means TRS is unavailable or not present. |
| MediaTek | Alt1-1 has minimum spec impact |
| Qualcomm | For Alt1-1, we would like to ask the proponent whether the “validity duration” is triggered and maintained per bit or per PDCCH? It did not say when a validity duration is triggered or extended. |
| Sharp | We generally support Alt1 , and we think alt1/3/5 can be applied per bit of the indication and alt 2/4 may need consider the indication bits as a whole, is it right? |
| vivo | Alt 1 can achieve higher flexibility compared to compared with Alt-2; Besides, availability of each group of TRS resource sets should be separately indicated by each bit of the bitmap to achieve better NW flexibility. Hence, Alt 1-1 is preferred over other alternatives. |
| Intel | Instead of “UE ignores bit 0”, it could be rephrased as “UE keeps its current assumption on the availability of TRS resource.” We think there is no issue with receiving multiple indications during validity duration. Once TRS resource is indicated available, it is necessary to make sure those are available until end of validity duration for a given L1 indication for consistency. Hence, after ‘1’ is indicated, either 0 or 1 can be indicated which have similar meaning, i.e., TRS resource is still available. In our view, validity duration is triggered when at least one bit is 1 in the bitmap and reference point is taken with respect to the current L1 indication received by the UE. |
| Ericsson1 | We are OK with principle of Alt 1-1, i.e. in our understanding only the red part below needs to be added (in 38.213, sec 10.4B) instead of reformulation of already specified behavior for value 1 in the bitmap. A value of '1' for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for the bitmap is ‘reserved’. |
| Samsung | For Alt 1-1, our understanding is that the validity duration is maintained per bit or per group of TRS resources sets. For Alt1-X, the validity duration is triggered per PDCCH and common for all TRS resources, which is more feasible regarding implementation. |
| ZTE, Sanechips | Alt 1-1 is preferred. We think whether to extend the on-going validity time duration should depends on NW implementation if the sliding window issue can be resolve. The alt 2 is too restrictive for both UE and NW, and the flexibility of the bitmap indication for each TRS resource set group is impacted. Among the alternatives in Alt1, Alt 1-1 is preferred. The issues for other alternatives are as below. For alt 1-3, the indication of “0” within the on-going validity time duration will lead to inconsistence. The function of valid time duration will be impacted. For alt 1-4, the interpretation of the “0” in not-all-zero code point indication is unclear. For example, if NW indicates “111” in the first paging cycle, and then indicates “011” in the second paging cycle. The first bit in the bitmap in the second paging cycle in unclear. |
| Huawei, HiSilicon | I see some questions by other companies regarding whether the validity duration is triggered per bit or per PDCCH. This is not clear in Alt.1-1. Some response to Sharp’s question. Yes. Alt.1-2 considers the indication bits as a whole indication other than one validity duration per one bit. Before the determination among alternatives, maybe we could fist decide a single validity duration or multiple validity durations are maintained. In our view, the design of validity duration should follow several principle. The first principle is no ‘immediate stopping’. If ‘immediate stopping’ is allowed which makes the validity duration is meaningless. So whenever gNB indicates ‘0’, gNB should ensure the transmission of a TRS before the expiration of any UE’s validity time. Therefore, ‘0’ does not extend the validity duration. The second principle is to avoid multiple validity duration. We share the similar view as Samsung (in their contribution R1-2200202), Alt 1-1 means one validity duration for each group of TRS resource sets, which is not friendly to UE implementation. And finally, the power saving gain should be maximized as much as possible. As analyzed in our contribution (R1-2200033), Alt 1-4 forces gNB transmitting ‘all 0’ which cannot inform UE the presence of all TRS resource sets if the UE has not received the earlier PDCCH with L1 indication of value ‘1’. This restricts the power saving gain for a new coming UE. |
| LG | We believe Alt 1 is more flexible than other alternatives and spec impact can be minimized. Maybe, introducing “unavailability indication” could serve more flexibility from gNB side, but as we discussed so far, problem due to the missing case shall be avoided. |
| Apple | We support Alt 1-1 because it is the most flexible approach and we do not think it is complicated for UE implementation. The question whether the validity duration is per PDCCH or per bit is a bit unclear. According to the agreement, the validity duration is associated with the bits indicated in a PDCCH, so it is common for all the bits in a PDCCH. Different PDCCH would have different definition of validity duration. Over time, different indications can be provided for different TRS resource set groups, and this needs to be tracked per group. However, we think this is very reasonable because the availability depends on the connected UEs, and it is more natural that they are indeed tracked separately. In addition, we have a max of 6 TRS resource set groups, so the additional tracking complexity for the UE (probably more associated with memory) should be minimal. |
| Panasonic | It depends on how flexibility and frequently gNB needs to change the TRS availability, especially the timing to switch off the TRS. If UE assumes a TRS is available but it has been switched off, it is problematic for UE. Thus we propose to look at the need to possibly switch off a certain TRS even it is within a validity period, which can be up to 512 paging cycles. As the TRS is shared from RRC CONNECTED mode UEs, we prefer not to restrict the gNB flexibility on when to switch it off as this is up to the RRC configuration procedure for CONNECTED UEs, but not confined with a number of paging cycles. Thus we propose Alt-3 to keep the flexibility and in the meanwhile avoid miss detection issue, assuming more than one indication can be transmitted within a validity period. |

### Issue 1-2: Consistent availability indication from PEI and paging PDCCH

Open problems:

- Whether UE can detect DCI format 2-7 for TRS availability indication without supporting paging enhancement.

- Whether UE expects the availability indication is provided in both PEI and paging DCI, or paging DCI only, or PEI only

- FFS a UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a paging cycle

- How to keep consistent start of validity duration when PEI-O and PO are not within the same default DRX cycle

Potential solutions:

Based on the proposals in Section 2, the following were proposed solutions/clarifications to support both PEI and paging PDCCH based TRS availability indication

- P1: If L1 availability indication is enabled, and if PEI is configured, availability indication is provided in both PEI and paging DCI.

- P2: A UE does not expect to receive inconsistent TRS availability indication from PEI and paging PDCCH within a paging cycle

- P3: For PEI based availability indication, the reference time of the start of validity duration follows the reference time for the associated monitoring occasion for the paging DCI

[1RD]

Please a) update or provide your views about whether or not to support or further discuss the proposals.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: Vivo, Spreadtrum, Nordic, Xiaomi(Alt 2), Qualcomm, Sharp, Intel, Samsung, LG No: Xiaomi(Alt 1), CMCC, MediaTek, Ericsson1, Huawei, HiSilicon, Apple |
| P2 | Yes: ZTE, Intel, InterDigital, Samsung, Spreadtrum, Nordic, Xiaomi, CMCC, MediaTek (but not preferred), Qualcomm, Huawei, HiSilicon No: Sharp, Ericsson1, Apple |
| P3 | Yes: Vivo, CATT, Nordic, Qualcomm No: Intel, Ericsson, Xiaomi, CMCC, MediaTek, Samsung, Apple |
| Others | Ericsson1 (see comments below) |

and, b) provide additional comments (e.g. justifications/clarification, concerns, or solutions to open problems) below

| Company | Comments |
| --- | --- |
| Xiaomi | For P1, our view is not fully captured. In our contribution R1-2200464 Proposal 3, Alt 1 is, UE does not expect L1 TRS availability indication is configured both in paging DCI and PEI. For Alt 2, if L1 availability indication is enabled, and if PEI is configured, from gNB’s perspective, availability indication can be provided in both PEI and paging DCI if availability indication field are configured in both PEI and paging DCI. But from UE’s perspective, if a UE is configured to monitor PEI, then UE would acquire L1 indication in PEI and ignore L1 indication in paging DCI, so that UE does not need to monitor TRS availability indication twice in order to save power. For P2, we support it. For P3, we do not support it, since it is against the previous agreement, Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication FFS: Whether the availability indication is transmitted [only once] during the validity duration |
| CMCC | If UE receives PEI which providing the TRS availability indication, it is unnecessary to be repeated in the paging DCI. |
| MediaTek | P2 means the two indications are somehow duplicated. It is more reasonable for network to only utilize one of them via SIB configuration. |
| Qualcomm | Even when UE receives a PEI with TRS availability indication, paging PDCCH containing TRS availability indication may still need to be transmitted for UEs that do not support PEI. For a UE that supports PEI, the TRS availability indication should be always transmitted in PEI. Otherwise, the UE will not receive the TRS availability indication if it is not paged. Based on these, P1 should be supported. For P3, the problem is a UE’s PEI may fall into a PF before its PF due to the frame-level and symbol-level offsets. This problem may even exist for paging PDCCH as subset of PMOs of a UE’s PO may fall into a PF after its PF due to multi-beam and repetition for NR-U. For this, our understanding is reflected by the clarification of the agreement. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle associated with the PO where UE receives the availability indication |
| Sharp | P1: we support P1, and UE can apply one type of indication based on its capability. P2: PEI-based indication can have higher priority to get more power-saving gain. P3: we believe that the PEI based availability indication should only be used for the associated paging PDCCH’s and the start point need to be offset from the associated PO to ensure that the UE monitoring the first PO can have a valid reference TRS before the PO. |
| vivo | For P1, we support. when PEI is configured, and - if L1 availability is only transmitted in PEI, for UEs not capable of receiving PEI, UE can not take advantage of the TRS. - if L1 availability is only transmitted in paging DCI, for UEs capable of PEI, UE should first receive PEI to obtain subgroup indication in PEI, and then receive paging DCI to obtain TRS availability. When UE is not paged (for 90% occasions), additional UE reception is required to take advantage of TRS, which deviates from UE power saving. For P2, we are generally Fine Understand the intention, but PEI and its associated paging DCI may transmitted in different paging cycle, hence we suggest the following revision. - P2: A UE does not expect to receive inconsistent TRS availability indication from PEI and its associated paging PDCCH monitoring occasion within a paging cycle For P3 PEI and paging DCI may be transmitted in different default paging cycles, the availability obtained from PEI is one DRX cycle shorter than that from paging DCI in current mechanism. It may limit the power saving benefits at UE. Besides, validity duration with 1 DRX cycle have been supported in RAN1#107. - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When validity duration with 1 DRX cycle is configured, and if TRS is transmitted before PEI in the default DRX cycle, the validity is expired immediately. UE can not take advantage of TRS before PEI occasion in next DRX cycle, which makes the availability indication meaningless, as shown in following figure. P3 provide a solution to address this issue. |
| Intel | We think P1 should still hold and is not an alternative by any means to P2. We updated lists of support. Reference point definition should be same for both PEI and Paging DCI based indication. Hence, P3 seems not supported by the agreement. |
| Ericsson1 | P1 : Our preference is to have presence of availability indication field in PEI to be independently configurable. P2 : Meaning of ‘inconsistent’ is not clear. P3 : Our proposal is as follows instead- Reference point is the SFN of the first PF associated with the default paging cycle for which the UE receives PEI |
| Samsung | For P1, gNB has to provide TRS availability indication in both PEI and paging PDCCH as NW cannot assume all UEs support PEI PDCCH. For P2, UE can receive both PEI and paging PDCCH when UE is paged regardless of UE supports PEI or not. We didn’t see any use case for different indication content. For P3, we prefer to reuse the existing framework for the validity duration as we agreed. This can be considered as an example when UE receive a second TRS availability indication before expires of previous one. |
| ZTE, Sanechips | P1: okay to consider it P2: Support. The indication should be consistent, otherwise, UE needs to detect both L1 signaling. P3: More discussion is needed. First of all, it should be clarified whether it will lead to ambiguity about the understanding of the reference point of validity time duration. Otherwise, we think using the first SFN of the first PF within the paging cycle where the PEI is detected. |
| Huawei, HiSilicon | For P1, we should give gNB the flexibility to control it, e.g. only indicate the TRS availability in PEI. We can discuss the Alt.4 of proposal 3-1. We also fully agree with CMCC regarding the following comments. In our view, it is not good to only rely on paging DCI to indicate the availability indication which may cause unnecessary paging DCI transmission. If UE receives PEI which providing the TRS availability indication, it is unnecessary to be repeated in the paging DCI. For P2, we support. For P3, we can further discuss it considering the figure provided by vivo. |
| LG | P1: For UEs which do not have PEI capability, indicating TRS availability via paging DCI shall be supported. Also, it should be noted that PEI capable UE would not monitor PO if it is not indicated for PO monitoring for power saving. So, it would be worth to convey TRS availability indication via PEI as well. P2: We would like to clarify that whether the gNB have to transmit paging PDCCH for TRS availability indication if it decide to transmit PEI for TRS availability indication, or vice versa. Also, it seems like we need to discuss the issue raised by vivo that PEI and its associated paging DCI may transmitted in different paging cycle P3: we are generally fine with the proposal, but it seems like we need further discussion |
| Apple | P1: we prefer to have separate control for the two to allow more flexibility. P2: the proposal is not clear because “inconsistent” is not defined, especially given the ongoing discussion for issue 1-1. P3: we do not support because it is clearly against the agreement. We also think it is not reasonable to require the gNB to predict the TRS availability for the future. |
| Panasonic | On P1, we think if L1 availability indication is enabled, paging DCI should at least support it. PEI can be optional. On P2, we prefer not to restrict as such. UE should take paging PDCCH as higher priority. On P3, we support. |

### Issue 1-3: Additional design for PEI PDCCH based TRS availability indication

Based on the proposals in Section 4, the following were proposed regarding additional design of TRS availability indication field in DCI format 2_7:

- P1: Support to only include one indication bit in TRS availability indication field in DCI for-mat 2_7, which is associated with the same QCL reference (SSB index) of the monitoring occasion for the DCI format 2_7, in addition to the support of the same TRS availability indication field size as that in paging DCI.

- P2: The start of TRS indication field in DCI format 2-7 should be explicitly configured

- P3: Support configuration of separate validity time value for PEI DCI (vs Paging DCI)

[1RD]

Please a) update or provide your views about whether to support or further discuss the proposals.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: HW, Sharp, Nordic, LG, Apple No: TCL, Qualcomm, vivo, Intel, Samsung |
| P2 | Yes: ZTE, Ericsson, QC, Nordic No: Intel, Samsung, Huawei, HiSilicon, LG, Apple |
| P3 | Yes: Ericsson, LG, Apple No: Nordic, Qualcomm, vivo, Intel, Samsung, Huawei, HiSilicon |
| Others |  |

and, b) provide additional comments below

| Company | Comments |
| --- | --- |
| Xiaomi | For P1, and P3, we are open to discuss For P2, we are not sure about the intention. From our understanding, UE can determine the start of RS indication field in DCI format 2-7 based on the configuration of paging indication field. Then why need to configure the start of TRS indication field in DCI format 2-7 explicitly? |
| Qualcomm | P1 does not follow the RAN plenary conclusion “If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats”. It is not essential to support this design. It will also cause the discussion on whether and how PEI and paging PDCCH based TRS availability should be used if they are transmitted by network in the same paging cycle. |
| Sharp | P3, we think the validity time value for PEI DCI can be different with that for paging DCI and can be the same with the duration of associated paging occasions. |
| vivo | For P1, The bit filed length in PEI DCI should be same as that in paging DCI to limit spec impact. |
| Intel | P1 and P3 do not follow same design/principle for PEI and paging DCI. Regarding P2, we think explicit configuration is not needed. Based on configuration of paging indication field, UE may identify start of TRS availability field. Suggest to add this as an alternative to P2 |
| Samsung | On P1, the new design for PEI is not essential, and should not be considered during CR phase. On P2, we think the start of TRS availability indication can be the first bit after paging indication. On P3, we think a common validity duration is sufficient. |
| ZTE, Sanechips | We agree with Qualcomm that P1 and P3are not aligned with RAN-P guidance. For P2, as to the questions by Xiaomi, it is noticed that the number of subgroups via UE-ID based sub-grouping and CN-assigned sub-grouping may be different, and these two solutions can work simultaneously. Therefore, different UEs may have different understanding about the bitwidth of paging indication carried by DCI format 2-7. So explicit indication is better for UE to parse the TRS availability indication in DCI format 2-7. |
| Huawei, HiSilicon | For P1, If we follow the same design principle, we need to confirm the paging DCI based and PEI based TRS availability indication as a bundled UE feature. Otherwise, it would violate the intention to have the same design mechanism for paging DCI and PEI DCI to reduce UE implementation complexity. For P2, we don’t see any reason to configure the start of TRS availability indication field. We can just hard coded it in the specification. Actually, it is already hard coded in the endorsed 212 CR. For P3, we don’t support it. The benefit is not clear and it introduces more complexity on UE and gNB. |
| LG | P1 and P3: From the power saving and signaling overhead perspective, PEI that conveys one bit for TRS availability indication that is for TRS resource sets with the same QCL reference of the MO would be beneficial. Also, availability indication for TRS occasion(s) between a PEI-O and a PO would be useful from gNB scheduling flexibility while guarantees power saving efficiency for PO monitoring. P2: we do not see needs and benefits from explicit configuration for the start of TRS indication field. PEI capable UE can read SIB for PEI configuration, and the start of TRS indication field can be determined directly based on configuration of paging indication field. |
| Apple | We are generally supportive of P1 and P3. P1 allows indication of the TRS resource sets with the same QCL reference, which is what matters to the UE. This avoids the potentially coarse availability indication per TRS resource set group. P3 is also useful to PEI-based indication because the validity duration can be potentially set to a small value. For P2, we do not see the need. |
| Panasonic | On P1, current functionality for this feature is sufficient in our understanding. On P2, it is okay. On P3, no need. |

## 2.2 <2nd round discussion>

For issue 1-1 and issue 1-2, the discussion in the first round are summarized below. Some proposals are drafted accordingly for further discussion.

For issue 1-3, the majority object P1, P2, and P3. For the sake of time, the discussion on them are suggested to be suspended.

### Issue 1-1: Availability indication during a validity duration

On Issue 1-1, the majority support the direction of Alt1.

For the down-selection from Alt 1-x,

- the main controversial problem is whether the validity duration is triggered per bit or per PDCCH. In the other word, whether the validity duration is maintained per group of TRS resource sets or common for all TRS resources. The majority support Alt 1-1 to allow validity duration maintained per group of TRS resources sets with higher indication flexibility.

- For bit of “0”, the majority support “ignore”, i.e. the UE keeps the current assumption on the availability of the corresponding TRS resource set(s). When UE receives bit “0”, the corresponding TRS resources can be available only if UE receives bit of “1” in previous indication that has not been expired yet.

The following proposal 1-1 (v1) is drafted based on the majority view to support Alt1-1.

[2RD] Proposal 1-1 (v1) UE can receive TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication - For each bit indicated as ‘1’ in the availability indication field, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 indication. - For each bit indicated as ‘0’ in the availability indication field, the UE ignores the bit.

Please a) provide your views about whether or not to support proposal 1-1 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple, Nordic，OPPO, vivo, Nokia2, Samsung, ZTE, Intel, TCL, LG, MediaTek (in principle),Xiaomi |
| No | Panasonic, CATT, CMCC, Ericsson2 |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Panasonic | It depends on how flexibility and frequently gNB needs to change the TRS availability, especially the timing to switch off the TRS. If UE assumes a TRS is available but it has been switched off, it is problematic for UE. Thus we propose to look at the need to possibly switch off a certain TRS even it is within a validity period, which can be up to 512 paging cycles. As the TRS is shared from RRC CONNECTED mode UEs, we prefer not to restrict the gNB flexibility on when to switch it off as this is up to the RRC configuration procedure for CONNECTED UEs, but not confined with a number of paging cycles. Thus we propose Alt-3 to keep the flexibility and in the meanwhile avoid miss detection issue, assuming more than one indication can be transmitted within a validity period. |
| Nokia2 | A minor clarification: “For each bit indicated as ‘1’ in the availability indication field of the current TRS availability indication,” to differentiate from the “previous” TRS availability indication. |
| CATT | UE could receive the availability indication from PEI/paging DCI at any DRX cycle. Most UEs would not know the validity duration had started or not. The indicated value “0” as ignored would imply that most UEs would not be able to use TRS most of time. Thus, no power saving gain would be achieved with configured TRS for IDLE/Inactive UE. |
| Samsung | We are OK with the proposal. We think the main technical issue that should be avoided is the case when gNB indicates TRS resources are not available but UE miss-detect it. So, the dynamic indication of unavailability based on “0” is problematic. Although gNB can transmit multiple indication within a validity duration, gNB may not always do that in practice. |
| IDCC | We are fine with the proposal. |
| CMCC | Both stop transmitting an available TRS or start transmitting an unavailable TRS should be supported in the TRS availability indication. |
| Lenovo | We propose the following modification: For each bit indicated as ‘0’ in the availability indication field, the UE ignores the bit keeps the current assumption on the availability of the corresponding TRS resource set(s). |
| Intel | Agree in principle. Suggest to add the interpretation of ignoring ‘0’ bit in second sub-bullet, such as follows ”, i.e. the UE keeps the current assumption on the availability of the corresponding TRS resource set(s). |
| Ericsson2 | First bullet is not needed – it does not add to existing agreement/existing spec as previously commented. On 2nd sub-bullet, UE ignores is not clear. We suggest reformulating as “If value ‘0’ is indicated for a bit, the bit is considered ‘reserved’.” |
| Huawei, HiSilicon | We agree that explicitly indicting termination of TRS is not a good IDEA, considering UE may miss-detect the indication, as commented by Samsung. However, on the other hand, it is also correct as CATT mentioned that “The indicated value “0” as ignored would imply that most UEs would not be able to use TRS most of time.”. We actually also worry the situation commented by Samsung that “Although gNB can transmit multiple indication within a validity duration, gNB may not always do that in practice.”. It may really eliminates the power saving benefit on UEs for this feature, especially considering some companies are pushing a very large validity duration, e.g. 512 default paging cycles which can be 512*2.56s = 21.x minutes. The UE may not be able to get the L1 indication information, if gNB just indicate “1” at the beginning of the 21 minutes. However, it seems the reason of this hard situation is due to the selected option of sliding validity duration. Therefore, we are wondering why UE needs to have the complexity to maintain six different validity durations if the TRS availability indication may not be always obtained. We are OK with the main bullet. According to Moderator’s explanation, it seems the Alt.1-1 is maintaining different validity durations of different indication bits. However, what would be the additional benefit to maintain up to six different validity durations compared with a single validity duration? We are reluctant to agree the sub-bullets before we have clear understanding on the benefit to maintain different multiple durations. |
| LG | We prefer Nokia’s modified version for more clarification. |
| MediaTek | We are fine with the modification form Nokia and Lenovo. As for the problem that UE cannot use the available TRS because UE ignores the value ‘0’. It can be resolved by defining the indication with bitmap of all ‘0’s, which is mentioned in Alt 2 in 1RD, to terminate all the validity duration. That is, gNB can try to prolong the validity duration as long as the TRS is available, and it also has the flexibility to stop transmitting all the TRS. |
| Xiaomi | Support the proposal |
| Spreadtrum | We shared the similar views that different UEs could receive the availability indication from PEI/paging DCI at different DRX cycles and most UEs would not know the validity duration had started or not. |

### Issue 1-2: Consistent availability indication from PEI and paging PDCCH

On P1

- the motivation is that Gnb’s can provide TRS availability indication in both PEI and paging PDCCH in order to serve Ues with different capabilities.

- The corresponding spec impact can be whether or not PEI and paging PDCCH based TRS availability indication are enabled at the same time or separately when the L1 availability indication is enabled.

On P2

- The intention is to resolve the ambiguity for the case when UE support both PEI and paging DPCCH based availability indication. The common understanding seems to be that UE can ignore TRS availability indication in paging PDCCH if UE receives TRS availability indication in PEI PDCCH.

On P3, several companies object it as it against the agreement in RAN1#107-e.

So, the following proposal 1-2 (v1) is drafted based on P1 and P2.

[2RD] Proposal 1-2 (v1) When L1 availability indication is enabled, - TRS availability indication field is configured in DCI format 2_7 (if configured) and DCI format 1_0 with CRC scrambled by P-RNTI. - If UE receives TRS availability indication in DCI format 2_7, the UE ignores TRS availability indication in DCI format 1_0 with CRC scrambled by P-RNTI.

Please a) provide your views about whether or not to support proposal 1-2 (v1)

| Support | Companies |
| --- | --- |
| Yes | OPPO(if 2nd bullet is removed),Sharp, LG(1st bullet only), Xiaomi( for the second bullet) |
| No | Apple, Nordic, Panasonic, Nokia2, CATT, CMCC, Lenovo, ZTE, Ericsson2, TCL, Huawei, HiSilicon, MediaTek,Xiaomi(for the first) |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Apple | The 1st bullet is not clear to us. Does it mean we support the separate configuration for DCI 2_7 and paging DCI. If yes, we are supportive. We do not support the 2nd bullet because we do not see why the TRS indication in DCI 1_0 needs to be ignored. It can be handled the same way as in P1-1 (v1), and it does not matter whether the indication is carried in paging DCI or PEI. |
| Nordic | The issues is that in some situations UE that supports PEI may rely on Paging DCI instead. It would be desirable if indication is consistent between PEI and associated PO |
| OPPO | We share similar view with apple that the 2nd is not necessary, it can be up to UE implementation to receive which L1 signaling. |
| Vivo | We don’t think the second sub bullet is needed, if the motivation is guarantee the consistency between paging DCI and PEI, the second bullet can be revised as - A UE does not expect to receive inconsistent TRS availability indication from PEI and its associated paging PDCCH monitoring occasion |
| Nokia2 | We would prefer to leave it for network configuration whether to provide L1 availability field in PEI (in addition to paging DCI). |
| CATT | First, DCI format 2_7 for PEI had CRC scrambled by PEI-RNTI as discussed in AI-8.7.1.1. The first bullet is not technical correct. The TRS availability indication should be included in both PEI and paging DCI since Gnb does not know whether any UE supporting FG29-1 paging enhancement. Since paging probability is extremely low for each UE, UE would occasionally receive PEI with correct CRC check. If UE receive PEI with its paging subgroup indication, the paging DCI would be in the same DRX cycle. Thus, the TRS availability indication would be the same in both PEI and paging DCI at the same DRX cycle. UE would not have inconsistent information of TRS availability. If the decoded PEI does not consist the UE’s paging subgroup, UE would not decode paging DCI. Thus, there is no inconsistent information. We don’t find the 2nd bullet of proposal 1-2(v1) in practical deployment scenario. |
| Samsung | For the first bullet, our understanding is that it indicates L1 TRS availability indication is enabled at the same time for paging PDCCH and PEI PDCCH. Whether or not PEI PDCCH based TRS availability is available depends on whether DCI format 2_7 is configured. There is no need to configure PEI based TRS availability indication separately. For the second bullet, if P1-1 was supported we agree with Apple it’s not needed. |
| IDCC | We think second bullet is not needed. |
| Sharp | We support the proposal, and for bullet2, indication in DCI format 2_7 should not change the status of TRS that has been indicated as available by paging PDCCH in an overlapping window. |
| CMCC | We also prefer where to provide TRS availability indication in PEI or paging DCI can be configurable. |
| Lenovo | We don’t think the second bullet is necessary. |
| ZTE,Sanechips | As to the first bullet, it seems to imply that these two L1 signaling should be configured with availability indication simultaneously. However, we think NW should have the flexibility to indicate availability indication via one of these signaling. For the second bullet, UE implementation is restricted, i.e., UE always needs to detect PEI for TRS availability indication. Therefore, we prefer the indication between PEI and paging is consistent so that UE can detect any L1 signaling based on the implementation/paging rate. |
| Intel | Agree in principle with first bullet. We do not think “When L1 indication is enabled” is needed. This can be replaced by “If SIB configures TRS resource”. Also, we suggest to replace “configured” with “present” in first bullet. If SIB configures TRS resource, - TRS availability indication field is present in DCI format 2_7 (if configured) and DCI format 1_0 with CRC scrambled by P-RNTI. Do not support second bullet. It is up to Gnb to ensure indications are consistent across the two DCIs in a given cycle. |
| Ericsson2 | Similar view as Nokia2. Regarding 2nd bullet, we don’t see need for it – it is up to UE implementation which indication it wants to use. |
| TCL | In our view, the transmission of TRS availability indication in both L1 signaling (PEI and paging PDCCH) will unnecessarily wake up a UE in the previous PO to decode the paging PDCCH based TRS indication. We do not support the transmission of TRS availability indication in PEI and paging PDCCH at the same time. |
| Huawei, HiSilicon | We would like to support the configurability to give Gnb flexibility, as commented by CMCC. We don’t think there should be priority difference between PEI and Paging DCI if they are both configured. Gnb should ensure consistent information in PEI DCI and paging DCI, considering value ‘0’ is reserved which means nothing actually. |
| LG | We are fine with the first bullet. As commented by several companies, we think 2nd bullet is not needed. |
| MediaTek | Inconsistency issue can be fundamentally resolved by configuring only one of the L1-based indication. Our first preference is to have full flexibility in selecting L1-based indication, including PEI only and paging DCI only. |
| Xiaomi | For the first bullet, we don’t support it We think Gnb should be able to configure separately whether the L1 indication is transmitted in PEI or paging DCI or both. For the second bullet, we support it . If L1 availability indication is enabled, and if PEI is configured, from Gnb’s perspective, availability indication can be provided in both PEI and paging DCI if availability indication field are configured in both PEI and paging DCI. But from UE’s perspective, if a UE is configured to monitor PEI, then UE would acquire L1 indication in PEI and ignore L1 indication in paging DCI, so that UE does not need to monitor TRS availability indication twice in order to save power. |
| Spreadtrum | We are fine with the first bullet and the second is not needed. |

## 2.3 <3rd round discussion>

### Issue 1-1: Availability indication during a validity duration

For Alt1-3 or the proposal to support indication of “unavailability” from Panasonic/CMCC, the technical issue is:

- the miss-detection issue can’t be resolved. Because although gNB can transmit multiple indication within a validity duration, gNB may not always do that in practice. If UE miss detect an indication to stop transmitting TRS resources, UE may measure random samples.

For the question about motivation for multiple validity durations maintained by UE, raised by HW

- One benefit is to allow higher indication flexibility. In practice, the availability duration can be different for different group of TRS resources.

For the concern, “The indicated value “0” as ignored would imply that most UEs would not be able to use TRS most of time”, from CATT.

- It doesn’t seem to be about UE behaviour within a validity duration. UE has to receive L1 availability indication in order to use TRS resources no matter we support unavailability indication or not.

For the comment, “the first bullet is not needed”, from Ericsson,

- it’s true no more spec change is expected for the first bullet. However, we don’t have any agreement for the first bullet yet. It should be fine to keep it just to provide backup for the last CR or spec update.

Proposal 1-1(v1) is updated by

- integrating the modification from Nokia and Lenovo/Intel, also address the concern from Ericsson.

[3RD] Proposal 1-1 (v2) UE can receive TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 indication. - For each bit indicated as ‘0’ in the availability indication field, the UE ignores the bit keeps the current assumption on the availability of the corresponding TRS resource set(s).

Please a) provide your views about whether or not to support proposal 1-1 (v2)

| Support | Companies |
| --- | --- |
| Yes |  |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 1-2: Consistent availability indication from PEI and paging PDCCH

For the first bullet

- the intention is to clarify the TRS availability indication field is enabled at the same time in both PEI PDCCH and paging PDCCH since gNB does not know whether any UE supporting FG29-1.

  - UE has the flexibility about which PDCCH to receive according to its own capability

- In the 2nd round, some companies [Apple, Nokia, CMCC, ZTE, HW, MediaTek, Xiaomi] proposed to support separate configuration for DCI format 2_7 and paging DCI based TRS availability indication, i.e. NW configures whether to provide L1 availability indication field in DCI format 2_7.

The second bullet addresses the case when UE receives TRS availability indication in both PEI PDCCH and paging PDCCH. The two PDCCHs may in the same DRX cycle or different DRX cycles due to frame-level offset. The intention is to avoid inconsistent availability information. There are the following views about UE behaviour in this case

- Alt1: the UE ignores TRS availability indication in DCI format 1_0 with CRC scrambled by P-RNTI.

  - Sharp, Xiaomi

- Alt2: It can be handled the same way as in P1-1. The second bullet is not needed.

  - [Apple, OPPO, Samsung, IDCC, Lenovo]

- Alt3: TRS availability indication would be the same/consistent in both PEI and paging DCI

  - Nordic, CATT

Given above, proposal 1-2 (v1) is updated by

- modifying the first bullet from P1-2(v1) to include the two views for further discussion and down-selection

- For the second bullet, it’s removed/deprioritized since the majority think it’s not needed.

- integrating the modification from intel on the main text

[3RD] Proposal 1-2 (v2) When L1 availability indication is enabled If SIB configures TRS resource, support one of the following - Alt1: TRS availability indication field is present configured in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI. - Alt2: TRS availability indication field is present in DCI format 1_0 with CRC scrambled by P-RNTI. TRS availability indication field in DCI format 2_7 with CRC scrambled by PEI-RNTI is enabled/disabled by a higher layer configuration parameter. - Alt3: a configuration parameter to select the DCI format for the L1 available indication - If UE receives TRS availability indication in DCI format 2_7, the UE ignores TRS availability indication in DCI format 1_0 with CRC scrambled by P-RNTI.

Please a) provide your views about whether or not to support proposal 1-2 (v2)

| Support | Companies |
| --- | --- |
| Yes | - w/ Alt1: - w/ Alt2: - w/ Alt3: |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

# TRS Validity duration

The following were agreed regarding validity time for L1 based availability indication:

From RAN1#107-e: Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Agreement For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support - time unit is one default paging cycle, - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s)

In contributions [1-24], proposals regarding the validity time for L1 based availability indication are captured in table below:

| Huawei, HiSilicon | Proposal 5: The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first or last MO of the PO/PEI-O where UE receives the availability indication. |
| --- | --- |
| ZTE, Sanechips |  |
| Vivo | Proposal 2: Validity duration with {64, 128, 256, 512} default paging cycles should be supported. |
| TCL |  |
| Spreadtrum |  |
| CATT | Proposal 4: The Applicable values of 64, 128, 256, and 512 should be supported for configuration of validity duration. |
| OPPO |  |
| Sony |  |
| Intel |  |
| Xiaomi |  |
| CMCC |  |
| Panasonic | Proposal 5: For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support applicable values: {1, 2, 4, 8, 16, 32, 64, 128, 256, 512}. |
| Samsung | Proposal 7: Confirm the following WA on the applicable values for validity duration configured by higher layer - [64], [128], [256], [512] |
| Apple |  |
| InterDigital |  |
| Sharp |  |
| LG | Proposal 2: Applicable values for the validity duration larger than 32 are not supported. |
| Ericsson | Proposal 1 For L1-based TRS availability indication, the supported values for validity timer are {1, 2, 4, 8, 16, 32, 64, 128, 256, 512}. - Adopt TP2 related to 38.213 subclause indication of TRS resources. <begin TP2> ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set group includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for the bitmap indicates presence of TRS in associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame default DRX cycle that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability |
| Qualcomm | Observation 1: Without application delay, the reference point for the start of the validity duration agreed in RAN1 #107-e is non-causal. The UE may use noise to update its tracking loops before it receives a new availability indication that switches an available TRS to unavailable. Proposal 3: Define a time offset or application delay on top of the reference point for the start of the validity duration for the TRS availability indication. |
| MediaTek | Proposal 4: Application delay of TRS availability indication, [5] ms, is introduced. Proposal 5: Adopt the following text proposal in Section 10.4B of TS 38.213: <Unchanged parts are omitted> The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of ‘1’ for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from (SFN+PF_offset)modT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. The UE is not expected to receive the indicated TRS resources set within application delay of 5 ms after TRS availability indication. <Unchanged parts are omitted>. |
| Nokia | Proposal-2: It is up to RAN2 whether [64], [128], [256],[512] DRX cycle long Idle TRS validation windows are supported. (Note: This was not from Nokia contribution) |
| Nordic |  |

According to the above proposals, the remaining issues regarding validity duration for L1 based TRS availability indication include:

- Issue 2-1: Confirm applicable values

- Issue 2-2: Clarifications on reference point

## 3.1 <1st round discussion>

Companies views regarding Issue 2-1/2-2 from contributions [1-23] were summarized in sub-sections below:

### Issue 2-1: Confirm applicable values

Open problem:

- whether or not to confirm WA from RAN1#107-e to support applicable values of [64], [128], [256], [512].

The following proposal is drafted based on the majority view to confirm the larger applicable values.

[1RD] Proposal 2-2 (v0) The applicable values:{64, 128, 256, 512} should be supported for the validity duration configured by higher layer,

Please a) update or provide your views about whether or not to support proposal 2-2 (v0)

| Support | Companies |
| --- | --- |
| Yes | Vivo, CATT, Panasonic, Samsung, Ericsson, CMCC, MediaTek, Qualcomm, Intel, ZTE, Apple, |
| No | LG, Huawei/HiSilicon(but OK with the suggested note to clarify UE’s understanding) |
| Up to RAN2 | Nokia, Nordic, Xiaomi |

and, b) provide additional comments (e.g. concerns, modifications), if any, below

| Company | Comments |
| --- | --- |
| Xiaomi | Can leave is to RAN2 |
| Huawei, HiSilicon | We fully agree with Nokia’s online comment that the understanding of UE should be clarified. If a UE does not support eDRX feature, this means the UE may not maintain the time duration larger than 10.24 seconds. The UE has no capability to maintain longer time duration from standard perspective. This is also the reason why RAN2 introduce and discuss the range of hyper radio frame. Some companies argued online that network indicates a longer time than 10.24s, and it is left to UE to assume how long the TRS(s) are available. However, considering the current validity duration design is a sliding duration triggered by an earlier L1 availability indication, the majority view seems consider value ‘0’ as reserved or ignored. This may cause the UE not supporting eDRX can only assume the TRS is available in the first 10.24s after the first L1 availability signaling. For the remaining time of e.g. 512*DRX cycles, the UE may not be able to get the availability information of TRS considering the gNB may always transmit ‘0’ or ‘reserved’ in the remaining time in 512*DRX cycles. Therefore, we would like to have the following note. The intention is to make clear that UE not supporting e-DRX may not know the presence of TRS even though the gNB configures the validity duration as a super long value. gNB may need to handle it by implementation, e.g. do the signaling properly (at least every 10.24s) or take care of that the UE may not be able to get the power saving benefit. Proposal 2-2 (v0 revised by Huawei) The applicable values:{64, 128, 256, 512} should be supported for the validity duration configured by higher layer Note: a UE may assume the validity duration length is no larger than 10.24s if the UE does not support eDRX. |
| LG | We do not prefer to confirm the applicable values larger than 32. Beside the issue on eDRX support, we see several issues as follow: If validity duration is indicated at once, gNB can indicate additional availability indication only when the validity duration for the indicated TRS resource set group can be extended. However, if gNB plans to stop transmitting the TRS, the additional availability indication for the validity of the TRS resource set group shall not be transmitted during the ongoing validity duration. In this case, a UE which newly camp on the cell and did not achieve a previous validity indication could not take advantage of the TRS for a long time. Note that if 512 is supported for the applicable value, validity duration could be configured up to 256*512 radio frames (around 21 min). Also, we would like to point out that gNB can extend the validity duration at several POs during the indicated validity duration even if applicable value equal or smaller than 32 is configured. We generally assume 10% paging probability for a PO. Thus it can be expected that gNB will transmit another PEI and/or paging DCI with high probability before the indicate validity duration expires (about 96% when 32 default paging cycle for a validity duration is assumed). |
| Apple | We are fine with the note suggested by Huawei. |

### Issue 2-2: Clarification on reference point

Based on the proposals in Section 3, the following were proposed by companies about clarification on reference point for TRS availability indication

- P1: The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the [first/last] MO of the PO/PEI-O where UE receives the availability indication.

- P2: Define a time offset or application delay on top of the reference point for the start of the validity duration for the TRS availability indication.

[1RD]

Please a) update or provide your views about whether or not to support or further discuss the proposals above.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: HW/HiSi, Xiaomi, CMCC, Qualcomm, Samsung, No: Nordic, MediaTek, Intel, LG |
| P2 | Yes: QC, MediaTek, Nordic No: Intel, Ericsson1, LG |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nordic | Should not depend on MO offset of PO, |
| Xiaomi | For P1,we can support it. we understand the intention is to guarantee the multiple MOs corresponding to multiple beams within one PO can have the same start of validity duration. For P2, we are not sure about the intention. |
| CMCC | We support P1 to clarify the starting point. |
| MediaTek | For P1, the fundamental solution is utilizing only one of the L1 indications |
| Qualcomm | For P1, we prefer “first”. For P2, it depends on whether a TRS availability indication can indicate a TRS is switched from available to unavailable before validity duration expires. We suggest discussing P2 after this problem is resolved first under Issue 1-1. |
| Sharp | We don’t see the need for more discussion for the reference point of Paging PDCCH based indication for it is clear that the reference point is the SFN of first PF from the current DRX of indication. |
| vivo | For P1, fine with the clarification. |
| Intel | We are not sure why this clarification is needed. For both PEI and paging DCI, UE takes the default DRX cycle when indication is received for identifying the reference point. In principle, it should be corresponding to the MO where L1 indication is received. We do not see strong justification for connection to first/last MO of PO or PEI-O. We do no think P2 is needed, if bit value 0 in the bitmap is ignored and defined as “UE keeps current assumption on the availability of the TRS resource”. Unavailability is not dynamically indicated. |
| Ericsson1 | P2 : we do not see need for additional offset. For P1, we prefer below change instead as it is more clear considering current 38.213 text. A value of '1' for the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame default DRX cycle that includes a PDCCH providing the DCI format 2_7 or the DCI format 1_0 with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. |
| ZTE, Sanechips | In our understanding, there is no ambiguity without the clarification. If the clarification is acceptable by the group, we think the first MO is better. |
| Huawei, HiSilicon | For P1, we think it is essential to clarify it. Some further explanation/response to MTK, Sharp and Intel: The key issue is the reference point is defined as SFN of the first PF from the current default DRX cycle where UE receives the availability indication. UE is clear on where it detects the L1 indication, however, gNB does not, as shown in the following figure. The gNB and UE may have different assumption on the start of the validity duration incase the configured MOs of a PEI-O/PO crosses the boundary of default DRX cycle. For P2, in our understanding, the validity duration is restart/triggered by L1 indication of value ‘1’, i.e. ‘available’. So, even considering there is some time for DCI processing, it can fully leave to UE implementation. The UE can utilize the TRS after the processing of DCI and the validity duration could be actually conjured as multiple DRX cycles. It seems not very essential to introduce this. |
| LG | P1: As far as I understand, PF is a radio frame where contains starting point of a PO. Thus we believe that all MOs within a PO can have same reference point and the further clarification may not needed. P2: We do not see strong motivation for defining a time offset or application delay. |
| Apple | P1: the motivation is not clear to us. In the previous agreements we had “The SFN for the first PF is calculated by (SFN + PF_offset) mod T = 0”. What is the reason to change it now? P2: we do not see a need as long as we do not have bit ‘0’ interpreted as unavailable. So this proposal is related to the discussion for issue 1-1. |
| Panasonic | Using the PO as reference to determine the SFN of the first PF should be a straight forward solution. |

## 3.2 <2nd round discussion>

### Issue 2-1: Confirm applicable values

On issue 2-1, the majority support the initial proposal. A note is added to address the concern on eDRX operation from HW/LG. It seems there is no consensus on whether or not eDRX is an issue due to different UE implementation.

[2RD] Proposal 2-1 (v1) The applicable values:{64, 128, 256, 512} should be supported for the validity duration configured by higher layer, Note: a UE may assume the validity duration length is no larger than 10.24s if the UE does not support eDRX.

Please a) provide your views about whether or not to support proposal 2-1 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple,OPPO, CATT (without Note), Samsung,CMCC, TCL, Huawei, HiSilicon, MediaTek |
| No | Nordic |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nordic | We prefer to first conclude that idle TRS will not have impact on reception of other signals. For example there are proposals where idle UE should rate-match PDSCH etc. |
| Nokia2 | Maybe it would be good to further clarify the note a bit: Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX; a UE may assume that the validity duration length is no larger than 10.24s. |
| CATT | We don’t see the need of Note since the counting of the number of DRX cycles for the validity duration does not need to associate with the SFN and is UE implementation. |
| ZTE, Sanechips | Is there any spec impact w/wo the note? |
| Intel | Support. Fine with revision from Nokia |
| Ericsson2 | We are OK with the applicable values but not OK with the Note in the proposal. It is up to UE implementation how to make use of TRS that are indicated as being available. |
| Huawei, HiSilicon | We are OK with the proposal as a compromise. We are also fine with Nokia’s revision. For ZTE’s question, our understanding is a note can have specification impact. We are not OK without the note. |
| LG | We do not prefer the value larger than 32. As we commented in a previous round, we are worried about the case where a UE which newly camp on a cell could not take advantage of the TRS for a long time. However, if we are the only company who have concern on this issue, we can compromise the majority view for the progress. |
| MediaTek | We can support this proposal with revised note by Nokia. |

### Issue 2-2: Clarification on reference point

On P1,

- the proposal is to address the issue when a PEI-O or a PO consisting of a number of PDCCH MOs are located around the boundary of default DRX cycle, i.e. the PDCCH MOs are spread into to DRX cycles. Although there is no ambiguity of paging frame, the ambiguity can still be there for UEs not configured to monitor the first PF.

On P2

- the motivation seems to clear. As suggested by QC, it depends on whether a TRS availability indication can indicate a TRS is switched from available to unavailable before validity duration expires. So, it can be revisited if needed after discussion of Issue 1-1.

[2RD] Proposal 2-2 (v1) The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first PDCCH MO of the PO/PEI-O where UE receives the availability indication.

Please a) provide your views about whether or not to support proposal 2-2 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple, Nordic, vivo (for PO part), Nokia2, CATT, Samsung, Lenovo, ZTE, TCL, Huawei, HiSilicon, LG(for PO) |
| No | vivo (for PEI part), Panasonic (using PO for both paging PDCCH and PEI case) ,Sharp(for PEI part), Intel, Ericsson2 |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Apple | We missed the intention in R1. Now we are fine with it. |
| OPPO | If first PDCCH MO of the PO and PEI-O are different, how to determine the reference point? |
| Vivo | Agree with the PO part. For PEI, as we mentioned for P3 issue#1-2. The reference point for PEI should be separately discussed. |
| Panasonic | Using the PO as reference to determine the SFN of the first PF should be a straight forward solution. |
| Nokia2 | We are fine to discuss further whether to align the PEI-O and PO reference points. |
| CATT | For a given UE, the PEI-O and PO would only occur in the same DRX cycle. |
| Samsung | The design on validity duration should be same for PEI based and PDCCH based signaling. When the reference points are different, we think it’s the same issue as discussed under P1-1. |
| IDCC | We are ok with the proposal. |
| Sharp | We can agree the proposal if it’s only for paging-PDCCH based indication. |
| ZTE,Sanechips | The clarification is applicable for both paging DCI and PEI. There is no need to define another reference point for PEI. |
| Intel | Thanks to Huawei and FL for further clarifications. However, we do not think PO or PEI-O spanning over multiple default DRX cycles needs special handling. We do not see the need for changing the agreement. Both gNB and UE are aware of the boundary of the cycles and for a L1 indication sent in a given default DRX cycle, first PF of that default DRX cycle is used as reference point. In the example mentioned by Huawei in the first round, it may be possible that L1 indications in MOs of PO or PEI-O may have different reference points if they are sent in different default DRX cycles. We agree that gNB may not know L1 indications in which MOs are actually received. However, we do not see why it can create ambiguity. UE takes current L1 indication for identifying reference point. It is upto gNB to maintain consistent indications. In Huawei’s example, gNB may need to trigger validity duration from second reference point again. |
| Huawei, HiSilicon | We support the proposal. To resolve other companies concern on PEI part, I think it is fair to make working assumption for PEI part, but it is not reasonable to delay the discussion here due to another individual issue. |
| LG | We are fine with the proposal for PO. We tend to agree with vivo that it would be worth to discuss the reference point for PEI separately. |

## 3.3 <3rd round discussion>

### Issue 2-1: Confirm applicable values

I don’t see additional technical concern for the proposal.

The suggestion from Nordic doesn’t seem to be relevant to the issue we discuss in this proposal.

For the note, the common understanding is no spec impact. However, HW thinks there is spec impact.

The proposal is further updated by

- integrating the modification from Nokia

- changing UE to “UE implementation” in the note to clarify the common understanding that no spec impact is expected

[3RD] Proposal 2-1 (v2) The applicable values:{64, 128, 256, 512} should be supported for the validity duration configured by higher layer, - Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX; a UE implementation may assume the validity duration length is no larger than 10.24s if the UE does not support eDRX.

Please a) provide your views about whether or not to support proposal 2-2 (v2)

| Support | Companies |
| --- | --- |
| Yes |  |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 2-2: Clarification on reference point

For the following question from OPPO and P3 from issue#1-2 supported by [Vivo, Panasonic, Sharp]

-“If first PDCCH MO of the PO and PEI-O are different, how to determine the reference point?”

We have discussed it in the first round, i.e. P3 in Issue 1-2. Many companies argued P3 is against the agreement in RAN1#107-e. According to previous agreement, the reference point can be different. Whether or not need additional design to keep consistent TRS availability indication is under discussion in Issue 1-2 and 1-1. For example, some companies think the two indication can be independent based on the solution in P1-1. For another example, some companies think UE can only receive PEI and ignore paging PDCCH.

For the concern from Intel:

- the proposal addresses the case when the multiple paging PDCCH MOs are located in two DRX cycles. Based on the existing agreement, there will be two first PFs associated with the two DRX cycles.

Given above, there is no change on the proposal. The positions from the 2nd round were copied for further check and continued discussion.

[3RD] Proposal 2-2 (v1) The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle corresponding the first PDCCH MO of the PO/PEI-O where UE receives the availability indication.

Please a) update or provide your views about whether or not to support proposal 2-2 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple, Nordic, vivo (for PO part), Nokia2, CATT, Samsung, Lenovo, ZTE, TCL, Huawei, HiSilicon, LG(for PO) |
| No | vivo (for PEI part), Panasonic (using PO for both paging PDCCH and PEI case) ,Sharp(for PEI part), Intel, Ericsson2 |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

# Signalling method of TRS availability indication

The following were agreed regarding enabling/disabling L1 availability indication

From RAN1#106bis-e: Conclusion No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. From RAN1#107-e: Agreement Confirm the following working assumption Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. - Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

In contributions [1-24], proposals regarding remaining issues for enabling of L1 availability indication are captured in table below:

| Huawei, HiSilicon |  |
| --- | --- |
| ZTE, Sanechips |  |
| Vivo |  |
| TCL | Proposal 3: Support SIB based signaling for availability indication of TRS occasions to the idle/inactive UEs. Proposal 4: SIB based singling and L1 based signaling can be configure simultaneously via SIB_X or Pre-Configuration. Proposal 5: Consider a NewBitField of size one bit in the SIB_X to enable/disable SIB based or L1 based signaling of TRS availability indication |
| Spreadtrum | Observation 1: The PEI based availability indication may have function overriding the paging DCI based availability indication. Observation 2: If the PEI based availability indication is disabled by RRC signaling, UE may still monitor PEI for availability indication indirectly. Proposal 1: The PEI based availability indication can be enabled/disabled by RRC signaling |
| CATT | Proposal 1: The TRS availability indication field should be included in both DCI format 2_7 and paging DCI or paging DCI only. Proposal 8: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X. Observation 1: TRS availability should not change frequently by L1 based signaling to achieve the power saving gain. Observation 2: There are at least following problems for L1 based availability indication. - It is difficult to provide persistent availability indication in the practical system for L1 signalling. - UE does not know which beam (SSB) is under covered after waking up from deep sleep, which will have a significant impact on performance of paging reception, network energy saving and system overhead. - Current L1 based availability indication mechanism cannot provide temporary TRS for IDLE UE with eDRX> 8 or [128] hyper frames. - The inherent miss detection issue of L1 based availability indication will create misalignment between gNB and UE’s understandings on the availability of TRS/CSI-RS. Observation 3: Compared with L1 signaling, SIB based availability indication of TRS/CSI-RS at a given cell for IDLE/Inactive mode UE can provide affirmative TRS/CSI-RS resource availability to achieve UE power saving gain and no additional signaling overhead. Proposal 9: The availability of TRS/CSI-RS at a given cell should be indicated to the UE by SIB-based signaling, which is indicated by the presence/absence of SIB-X. |
| DOCOMO | Proposal 1: Confirm the following working assumption: If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. |
| OPPO |  |
| Sony |  |
| Intel | Observation 1: A UE receiving TRS availability information from PEI DCI may or may not monitor PO, subject to the information provided by the Paging indication field in PEI DCI. • If the UE monitors PO and receives paging DCI, it is expected that TRS availability indication would consistent across the two DCIs. Proposal 1: If SIB configures TRS resource, • TRS availability indication is always included in the paging DCI. • TRS availability indication is always included in the PEI DCI when PEI is configured. Proposal 5: The default assumption, i.e., before UE receives L1 availability indication after camping on a cell, is that the configured group(s) of TRS resource set(s) are unavailable. |
| Xiaomi | Proposal 1: The availability of TRS/CSI-RS can be implicitly indicated by the existing of the TRS/CSI-RS configuration in SIB if L1 TRS availability indication is not configured. Proposal 2: If L1 indication is configured, before UE receive a first L1 indication after entering a cell, UE should assume the TRS/CSI-RS for idle is not available. Proposal 6: If L1 TRS availability indication is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable. |
| CMCC |  |
| Panasonic | Proposal 1: Confirm the working assumption that if TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. |
| Samsung | Proposal 1: Deprioritize the discussion on SIB based availability indication. |
| Apple |  |
| InterDigital |  |
| Sharp |  |
| LG | Proposal 1: Confirm the working assumption regarding enabling L1 based availability indication - If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. |
| Ericsson |  |
| Qualcomm |  |
| MediaTek | Proposal 6: Confirm the following working assumption: Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.. Proposal 7: Introduce a RRC parameter ‘AvalIndDCIFormat’ to select the DCI format for the L1-based available indication. Proposal 8: Adopt the following text proposal in Section 10.4B of TS 38.213: <Unchanged parts are omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI, provided by TRS-AvalIndDCIFormat, to carry includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged parts are omitted>. |
| Nokia | Proposal: Discuss whether L1 availability indication needs to be always configured also to PEI when TRS occasions are configured. |
| Nordic |  |

According to the above proposals, the remaining issues regarding enabling of L1 based availability indication include:

- Issue 3-1: How to enable L1 based TRS availability indication

- Issue 3-2: Default assumption of TRS availability

- Issue 3-3: Whether support SIB based availability indication

## 4.1 <1st round discussion>

Companies views regarding Issue 4 from contributions [1-23] were summarized in sub-sections below:

### Issue 3-1: How to enable L1 based TRS availability indication

Open problems:

- Whether or not to confirm the following working assumption

Working Assumption

If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

- Whether and how to enable PEI PDCCH and paging PDCCH based TRS availability indication at the same time or separately

Potential solutions:

- Alt1: Confirm the working assumption

- Alt2: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X

- Alt3: If SIB configures TRS resource

  - TRS availability indication is always included in the paging DCI.

  - TRS availability indication is always included in the PEI DCI when PEI is configured

- Alt4: Introduce a RRC parameter ‘AvalIndDCIFormat’ to select the DCI format for the L1-based available indication.

- Alt5: The PEI based availability indication can be enabled/disabled by RRC signaling

[1RD]

Please a) update or provide your views for alternative(s) you support to resolve issue 4-1.

| Support | Companies |
| --- | --- |
| Alt1 | DOCOMO, Intel, Panasonic, LG, MediaTek (together with Alt4), Nordic, Ericsson1, Samsung, ZTE, Apple |
| Alt2 | TCL, CATT |
| Alt3 | Intel,Sharp, vivo |
| Alt4 | MediaTek, Huawei, HiSilicon, Apple |
| Alt5 | Spreadtrum |

and, b) provide additional comments below

| Company | Comments |
| --- | --- |
| Xiaomi | Our first preference is Alt2. But we are open on this issue and can go with majority. |
| CMCC | Alt 2 as the first preference. |
| MediaTek | As mentioned in Issue 1-2 and Issue 2-2, it is efficient and necessary to select one of the DCI formats to indicate TRS availability. |
| vivo | Alt-3, similar to our comments to P1 of issue#1-2. |
| Intel | There is no strong need to introduce an additional RRC parameter here. It does not make sense for a PEI to not indicate availability if TRS resource is configured. Otherwise, UE would need to wake up to receive paging DCI for availability indication, which is not desirable from power saving perspective. |
| Samsung | We think both can be enabled at the same time. From UE’s perspective, UE may only need to receive PEI PDCCH for the TRS availability indication when UE is not paged. But from gNB’s perspective, gNB may need to transmit the TRS availability indication in both paging PDCCH and PEI PDCCH as some UE may not support PEI PDCCH. |
| Huawei, HiSilicon | We support to agree Alt.4. However, we think we need to discuss which combinations are supported for the configuration. In our view, it could be “PEI DCI”, “PEI DCI and paging DCI” etc. |
| Apple | For Alt 4, it is worthwhile to consider whether to allow enabling/disabling paging DCI-based indication. The motivation is to allow a bit more forward compatibility/flexibility, in case we may want to configure the reserved bits for other purposes in the future. |

### Issue 3-2: Default assumption of TRS availability

Based on the proposals in Section 5, the following were proposed by companies regarding the default assumption of TRS availability

- P1: If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell, the UE assumes the configured group(s) of TRS resource set(s) are unavailable

- P2: If L1 TRS availability indication is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable.

[1RD]

Please a) update or provide your views about whether to support or further discuss the proposals.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: Intel, Xiaomi, Nordic, CMCC, MediaTek, Qualcomm, vivo, Samsung, ZTE, DOCOMO, Huawei, HiSilicon, LG, Apple, Panasonic No: Ericsson1 |
| P2 | Yes: Xiaomi, Nordic, CMCC, MediaTek, Qualcomm, Samsung, ZTE, DOCOMO, Huawei, HiSilicon, Apple, Panasonic No: Ericsson1 |
| Others |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| vivo | For P2, it is nature that after SI update for TRS configurations, previous TRS configuration are not valid anymore, which seems aligned with existing mechanisms on SI change. Hence, no spec impacts is needed in our understanding. |
| Ericsson1 | We do not think any additional clarifications are needed. The current spec (38.213 sec 10.4B) explains when TRS is available. Rel-16 applies otherwise. |
| ZTE, Sanechips | We think the intention of P1 and P2 has been agreed in the previous meeting. |
| Huawei, HiSilicon | Technically it is OK. But we share similar view that it does not need to impact the spec, which is the default assumption, especially for P2. |
| LG | P1: We are fine with the proposal. P2: We are fine with the intention of the proposal. However, we wonder whether UE can assume TRS transmission during the modification period where SI update procedure has performed. Before UE starts SI update procedure, it may use TRS resources that was indicated before UE obtains the SIB for TRS configuration. Thus, it seems like gNB shall guarantee ongoing TRS transmission at least for the modification period where SI update procedure is performed. |
| Apple | Agree in principle, but we think LG has a valid point. |
| Panasonic | On P1, we think it is by default and no specification impact. Thus no need to agree. On P2, we think it is reasonable assumption and can leave it to UE implementation. If majority view is to introduce specification impact, we are also fine. |

### Issue 3-3: Whether support SIB based availability indication

We have conclusion from RAN1#106bis-e that no consensus to support SIB based availability indication. However, there are several proposals on supporting SIB based availability indication as captured in Section 5.

- P1: Support SIB based signaling for availability indication of TRS occasions to the idle/inactive UEs.

[1RD]

Please a) update or provide your views for whether to support or discuss the proposal.

| Proposal | Companies |
| --- | --- |
| P1 | Yes: TCL, CATT, Xiaomi, vivo, Apple No: Nordic, MediaTek, Ericsson1,ZTE, LG |

and, b) provide additional comments (e.g. justification, concern) below

| Company | Comments |
| --- | --- |
| Xiaomi | We think this issue is related to Issue 3-1. If Alt 1 in Issue 3-1 is selected, the there is noe need to discuss Issue 3-3. |
| MediaTek | We already have the conclusion that no consensus to support SIB based availability indication. Instead of introducing a new feature, we suggest prioritizing the detailed designs of L1-based indication in the stage of maintenance. |

## 4.2 <2nd round discussion>

### Issue 3-1: How to enable L1 based TRS availability indication

The proposal is drafted based on the majority view to support Alt1. Whether or not to further separate the enabling of PEI based and paging PDCCH based TRS availability indication can be discussed separately under issue 1-2.

[2RD] Proposal 3-1 (v1) Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. - Whether and how to enable PEI PDCCH and paging PDCCH based TRS availability indication at the same time or separately

Please a) provide your views about whether or not to support proposal 3-1 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple, Nordic,OPPO, Nokia2, Samsung,Sharp, CMCC, Lenovo,ZTE, Intel, Ericsson2, TCL, LG, MediaTek |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 3-2: Default assumption of TRS availability

For P1, the only concern is there is no spec change is needed. So, a conclusion can be considered

For P2, spec update may be necessary as reconfiguration of TRS resources may terminate an on-going validity duration.

[2RD] Conclusion 3-2 (v1) If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell, the UE assumes the configured group(s) of TRS resource set(s) are unavailable Proposal 3-2 (v1) If L1 TRS availability indication is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable.

Please a) provide your views about whether or not to support conclusion/proposal 3-2 (v1)

|  | Support | Companies |
| --- | --- | --- |
| Conclusion 3-2(v1) | Yes | Apple, Nordic,OPPO, Nokia2, Samsung,Sharp, CMCC, Lenovo,ZTE, Intel, TCL, Huawei/HiSilicon(only with the revision), LG, Xiaomi |
|  | No | Ericsson2 (not needed) |
| Proposal 3-2(v1) | Yes | Nordic, Samsung ,CMCC, TCL,Xiaomi |
|  | No | vivo, Ericsson2 (not needed), Huawei, HiSilicon |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Apple | We agree with P3-2 in principle. But we would like to get clarification on the point raised by LG in R1. Does the UE assume all TRS resources are unavailable only in the next modification period? |
| vivo | We don’t think this proposal is needed, although it is technically right. UE will not use the TRS configuration before SI change in current mechanism, according to section 5.2.2.2.1 of 38.331, as follows. ...... The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication.. Hence, the TRS configuration before SI change, including the validity duration configuration, will not be used at UE any more. Hence, we do not see any spec impact. |
| Nokia2 | For proposal 3-2, it is not clear if specification change is needed (or proposed). While we are Ok with the proposal in high level, it is not clear if any specification change is needed. If there is SI update, it could result, not just changing TRS resources, but omitting them all together, thus availability should be confirmed (via L1 indication if any) after SI update. |
| Lenovo | Proposal 3-2 can be a conclusion, and can be merged with Conclusion 3-2 as follows: “If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE assumes the configured group(s) of TRS resource set(s) are unavailable |
| ZTE,Sanechips | We agree that there is no spec impact for the P3-2, the UE behavior during SI change is a common issue. |
| Ericsson2 | We don’t see the need for these given below agreement. Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). |
| Huawei, HiSilicon | For the conclusion we suggest the following revision: Conclusion 3-2 (v1) If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell, the UE can assumes the configured group(s) of TRS resource set(s) are unavailable For the proposal 3-2, we don’t need to agree whether the TRS is available or not before a next L1 indication. If the configuration of TRS resource sets are changed, the TRS resource sets may be changed totally. We agree with Nokia’s comment. Also, this does not need to agree additionally. |
| LG | For proposal 3-2, As we commented in a previous round, we are fine with the proposal in principle, but it seems like further clarification is needed. In my understanding, UE does not need to know updated SI other than MIB and SIB1 for paging reception. Meanwhile, validity for TRS would be used by UE for paging procedure. Furthermore it can be used for SI update procedure which includes PDSCH decoding. From this perspective we prefer to discuss further on this issue. |
| Xiaomi | Support the conclusion and the proposal. |

## 4.3 <3rd round discussion>

### Issue 3-1: How to enable L1 based TRS availability indication

No objection for the proposal.

[2RD] Proposal 3-1 (v1) Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. - Whether and how to enable PEI PDCCH and paging PDCCH based TRS availability indication at the same time or separately

Please a) provide your views about whether or not to support proposal 3-1 (v1)

| Support | Companies |
| --- | --- |
| Yes |  |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 3-2: Default assumption of TRS availability

For Proposal 3-2,

- The following question was raised by LG/Apple

  - whether UE can assume TRS transmission during the modification period where SI update procedure has performed.

- [Vivo, ZTE, HW, Lenovo] think it’s not needed as it’s achieved by current mechanism.

[3RD] Conclusion 3-2 (v2) If L1 TRS availability indication is enabled, before UE receives a L1 availability indication after camping on a cell or after receiving an updated TRS configuration, the UE can assumes the configured group(s) of TRS resource set(s) are unavailable Question [LG/Apple]: whether UE can assume TRS transmission during the modification period where SI update procedure has performed. Proposal 3-2 (v1) If L1 TRS availability indication is configured, and during the validity time, the TRS resource configuration is changed by SI update procedure, before UE detects a following L1 TRS availability indication, all TRS resources are assumed as unavailable.

Please a) provide your views about whether or not to support conclusion 3-2 (v2)

|  | Support | Companies |
| --- | --- | --- |
| Conclusion 3-2(v2) | Yes |  |
|  | No |  |

and, b) provide additional comments or answer to the question, below

| Company | Comments |
| --- | --- |

# TRS resources configuration

The following were agreed in RAN#107-e regarding configuration of TRS resources for idle/inactive UEs:

From RAN1#107-e: Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

The following were discussed in RAN#107-e but not handled online due to time limitation:

From RAN1#107-e: FL Proposal 4-1 (v9) Support a configuration parameter for the number of, X, TRS resources for a TRS resource set. - applicable values for X: {2, 4} One or more scrambling IDs is configured for TRS resources within a TRS resource set. - If only a single TRS resource is configured with scrambling ID, it applies to all the TRS resources. - Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where the scrambling ID is configured per TRS resource.

In contributions [1-24], proposals regarding the TRS availability indication in DCI format 2_7 are captured in table below:

| Huawei, HiSilicon | Proposal 8: For TRS configuration - Support a configuration parameter for the number of, X, TRS resources for a TRS resource set. i. applicable values for X: {2, 4} - One or more scrambling IDs is configured for TRS resources within a TRS resource set. i. If only a single TRS resource is configured with scrambling ID, it applies to all the TRS resources. ii. Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where the scrambling ID is configured per TRS resource. |
| --- | --- |
| ZTE, Sanechips | Observation 2: The number of configured TRS resource sets can be larger than the number of actual transmitted SSB. Proposal 6: The scramblingID is configured per TRS resource set. Observation 3: At least for FR2, the number of slots is needed to distinguish the TRS resource set with one or two slots. Proposal 7: At least for FR2, the number of slots is configured per TRS resource set. Observation 4: The parameter of periodicityAndOffset only determines TRS transmission occasion in one slot within a periodicity. However, one TRS resource set for idle/inactive state UE may comprise TRS occasions in one or two slots. Proposal 8: The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. |
| Vivo | Proposal 5: The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. |
| TCL |  |
| Spreadtrum |  |
| CATT | Proposal 10: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit. Proposal 11: Scrambling ID should be common parameters of the TRS resource set to meet the requirement of SIB message size limit. Proposal 12: Slot number of the TRS resource set should be indicated implicitly, i.e., slot number=2 for FR1 and slot number=1 for FR2. Proposal 13: The following TRS resource set configuration parameters: startingRB, nrofRBs, periodicity and validity duration and TRS resource set group ID could be common for all the TRS resources sets. |
| DOCOMO | Text proposal 1: -------------------------- Start of Text Proposal for TS 38.214 -------------------------- <Unchanged parts omitted> 5.1.6.1.1 CSI-RS for tracking ... Each NZP CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter [TRS-ResourceSet] with the following restrictions for a UE in RRC_IDLE or RRC_INACTIVE: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), is one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - where the first symbol location in a slot is indicated by firstOFDMSymbolInTimeDomain in the [TRS-ResourceSet] and the second symbol location in a slot is firstOFDMSymbolInTimeDomain + 4 - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211]. - the bandwidth and the frequency location of the NZP CSI-RS resource, is given by the higher layer parameter [nrofRBs], [startingRB] and [frequencyDomainAllocation] in a [TRS-ResourceSet] and applies to all resources in a [TRS-ResourceSet]. The [frequencyDomainAllocation] configuration is not restricted by initial DL BWP. - UE is not required to receive TRS occasions outside the initial DL BWP. - the periodicity and slot offset for periodic NZP CSI-RS resources, is given by the higher layer parameter periodicityAndOffset configured by a [TRS-ResourceSet], is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211], applies to all resources in a [TRS-ResourceSet]. - the UE does not expect the [TRS-ResourceSet] to be configured with the periodicity of slots if the bandwidth of NZP CSI-RS resource is larger than 52 resource blocks. - the UE may assume the sub-carrier spacing of the NZP CSI-RS resources configured by [TRS-ResourceSet] to be same as the sub-carrier spacing of the initial DL BWP. - powerControlOffsetSS given by a [TRS-ResourceSet] applies to all resources in a [TRS-ResourceSet]. - the QCL information for periodic NZP CSI-RS resources, is given by the higher layer parameter ssb-Index configured by a [TRS-ResourceSet], is a SS/PBCH block, applies to all resources in a [TRS-ResourceSet]. - the UE may assume the following quasi co-location type(s): - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block. - One or more scrambling IDs is configured for TRS resources within a TRS resource set.  If only a single TRS resource is configured with scrambling ID, it applies to all the TRS resources. Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where the scrambling ID is configured per TRS resource. <Unchanged parts omitted> -------------------------- End of Text Proposal for TS 38.214 -------------------------- |
| OPPO |  |
| Sony |  |
| Intel | Proposal 6: One or more scrambling IDs can be configured for TRS resources within a TRS resource set. |
| Xiaomi |  |
| CMCC | Proposal 2. Both four periodic TRS resources in two consecutive slots and two periodic TRS resources in one slot can be configured to RRC_IDLE/INACTVE UEs for FR1. |
| Panasonic | Proposal 7: The maximum number of configured TRS resource sets is 64 and no need to limit by the number of actual transmitted SSBs in ssb-PositionsInBurst. |
| Samsung | Proposal #4: Support a configuration parameter for the number of, X, TRS resources for a TRS resource set with applicable values for X: {2, 4} - Adopt TP#2 for TS 38.214 Proposal 5: One or more scrambling IDs is configured for a TRS resource set. - - Adopt TP #3 for TS 38.214 Proposal 6: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - adopt value range: up to ssb-PositionsInBurst TRS-ResourceSet for TRS-ResourceSetConfig in TS 38.331. |
| Apple | Proposal 1: Support a configuration parameter for the number of TRS resources X for a TRS resource set. • applicable values for X: {2, 4} Proposal 2: One or more scrambling IDs is configured for a TRS resource set. • If a single scrambling ID is configured, it applies to all the TRS resources. • Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource. Proposal 3: Further consider introducing common parameters (e.g., starting RB and nrofRBs) for multiple/all TRS resource sets to reduce the signaling overhead. |
| InterDigital |  |
| Sharp | Proposal 1: Support more TRS resource set configurations than the number of actual transmitted SSBs Proposal 2: Support using offset values to configure multiple scrambled IDs in a resource set Proposal 3: Support using one bit to indicate the number of the resources in one set for FR2 band |
| LG |  |
| Ericsson | - For configuration of TRS occasions in FR1, there is no need to introduce RRC parameter for indicating number of slots. - For the TRS used for connected mode UEs, specification allows scramblingID to be different among resources within a resource set, and the same flexibility should be ensured for TRS occasions for idle mode UEs. Proposal 3 For configuration of TRS resource set, a. If only one TRS resource is configured with a scrambling ID for a TRS resource set, the same parameter scramblingID applies to all resources of the TRS resource set, otherwise, scramblingID is configured per resource in a TRS resource set. b. For FR2, introduce an optional parameter (twoSlotTRSforFR2) to indicate number of slots i.e. if twoSlotTRSforFR2 is configured/present, then the CSI-RS resources of the TRS resource set are in two consecutive slots, otherwise the CSI-RS resources of the TRS resource set are in one slot. |
| Qualcomm | Proposal 1: Number of configured TRS resource sets is not larger than twice the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. |
| MediaTek | Proposal 1: Scrambling ID in a TRS resource set can be different (same as Rel-16). Proposal 2: In TRS resource set, the applicable value X for number of symbols per TRS resource can be 2 or 4. |
| Nokia | Observation: As there are no requirements for the IDLE/Inactive UE in relation to TRS occasions, it does not seem neccesary to restrict the number TRS resource set configurations. Observation: For FR1, two slot/4 symbol TRS resource set could always be assumed. Proposal: For FR1, either 4 separate scrambling ID IEs can be configured, one for each resource, or one scrambling ID IE is provided, being common for all four resources. Observation: For FR2, one slot/2 symbol and two slot/4 symbol TRS resource set configurations are possible. Proposal: For FR2, either 4 or 2 separate scrambling ID IEs can be configured, one corresponding to each available resource. If only one scrambling ID IE is provided, it is common for all four resources. |
| Nordic | Proposal-3: The maximum number of TRS resource sets per cell within a band is function of L_max. |
| Lenovo | Proposal 1: An SSB index indicating a QCL source of a TRS resource set implicitly indicates a TRS resource set ID. |

According to the above proposals, the remaining issues regarding TRS resources configuration may include:

- Issue 4-1: Determine number of TRS resources per TRS resources set

- Issue 4-2: How to configure scrambling ID

- Issue 4-3: Whether to reduce maximum number of TRS resource sets

- Issue 4-4: Other potential configuration issues

## 5.1 <1st round discussion>

Companies views regarding Issue 5 from contributions [1-23] were summarized in sub-sections below:

### Issue 4-1: number of TRS resources

Open problem:

- Whether or not support additional configuration parameter for the number of, X, TRS resources for a TRS resource set

  - Whether or not for FR2 only, or for both FR1 and FR2

Potential solution:

- Alt1: Support a configuration parameter for the number of, X, TRS resources for a TRS resource set

  - Applicable values for X: {2, 4}

- Alt2: For FR2, the number of slots is configured per TRS resource set. Fixed value (4) for FR1.

- Alt3: Slot number of the TRS resource set should be indicated implicitly, i.e., slot number=2 for FR1 and slot number=1 for FR2.

- Alt4: For FR2, the number of slots is configured per TRS resource set. For FR1, X = 4, except when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon in which case X=2.

[1RD]

Please a) update or provide your views for alternative(s) you support to resolve issue 5-1.

| Support | Companies |
| --- | --- |
| Alt1 | HW/HiSi, Samsung, Apple, MediaTek, CMCC, Intel, Panasonic |
| Alt2 | ZTE, Sharp, Ericsson, Nordic, LG |
| Alt3 | CATT |
| Alt 4 | Ericsson1 |

and, b) provide additional comments (e.g. justification, concern) below

| Company | Comments |
| --- | --- |
| CMCC | We can not accept neither Alt 2 nor Alt 3, we don’t understand why to exclude one slot TRS in FR1 which has been supported in Rel-16. It’s gNB’s implementation to share which TRS using by RRC_CONNECTED UEs, and all Rel-15/16 TRS configurations should be supported. |
| Sharp | For FR1, The number of resources can be 4 by default and can be 2 if the symbols in second slot is not indicated as downlink by tdd-UL-DL-ConfigurationCommon. And we can accept alt1 if it’s majority selection. |
| Ericsson1 | Added Alt 4 since our proposal for FR1 was missing from the list. |
| LG | We prefer Alt 2, but also ok with Alt 1. |
| Apple | We think we should provide full flexibility that is available for connected UEs, which is why we think Alt 2/3 are too restrictive. For FR1, “For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RSResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DLConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.” So for connected UEs, the determination also depends on tdd-UL-DL-ConfigDedicated, and this info is not available to idle/inactive UEs. By using tdd-UL-DLConfigurationCommon only, the idle/inactive UEs may derive different configuration from connected UEs. Therefore, it is much easier just to directly indicate it, and it is just a single bit. |

### Issue 4-2: Scrambling ID

Open problem:

- Whether and how to configure scrambling ID per TRS resource or TRS resource set

  - If per TRS resource, whether consider additional RRC parameter, e.g. TRS-Resource

Note: the number of scrambling IDs if more than one is determined based on number of TRS resources per TRS resource set, which is under discussion in issue 4-1.

Potential solution:

- Alt1: One or more scrambling IDs is configured for TRS resources within a TRS resource set.

  - If only a single TRS resource is configured with scrambling ID, it applies to all the TRS resources.

  - Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where the scrambling ID is configured per TRS resource.

- Alt2: One or more scrambling IDs is configured for a TRS resource set.

  - If a single scrambling ID is configured, it applies to all the TRS resources.

  - Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource.

- Alt3: Support using offset values to configure multiple scrambled IDs in a resource set

[1RD]

Please a) update or provide your views for alternative(s) you support to resolve issue 5-2.

| Support | Companies |
| --- | --- |
| Alt1 | HW/HiSi, Ericsson, DOCOMO, Intel, Nordic |
| Alt2 | Apple, ZTE, CATT, Samsung, Ericsson1, Panasonic |
| Alt3 | Sharp |

and, b) provide additional comments (e.g. justification, concern), if any, below

| Company | Comments |
| --- | --- |
| Xiaomi | Our preference is scrambling ID is common for a TRS resource set. we do not see clear benefit of having different scrambling IDs. But to make progress we can support Alt 1 or Alt 2. |
| CMCC | We can accept Alt 1 or Alt 2. |
| vivo | Is the difference between Alt1 and Alt2 is signalling structure? If YES, can we leave the detailed to RAN2. RAN1 only focus on the flexibility on TRS configuration. |
| Samsung | We prefer Alt2 as there is no need for additional RRC parameter of TRS-Resource. |
| DOCOMO | Same question as vivo. I’m not sure the difference Alt1and Alt2. |
| Huawei, HiSilicon | Alt.1 just follows the configuration of legacy TRS resource. A question for clarification, should Altl.2 be the same as Alt.1? In the list, is the scrambling ID configured per TRS resource? Or it uses some other way? |
| LG | We are fine with Alt 1 and Alt 2. |
| Apple | As we explained in our contribution, Alt 1 and Alt 2 are almost the same, but they may have different implication in signaling design. The only reason we prefer Alt 2 is that it leaves more flexibility to RAN2 signaling design, because Alt 1 implies that we have a parameter defined per TRS resource, which is not really necessary in our view. However, this can be left to RAN2 to decide and Alt 2 serves the purpose. |

### Issue 4-3: Maximum number of TRS resource sets

Open issue:

- FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

Potential solution

- Alt1: the number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

  - The maximum number of configured TRS resource sets is 64 and no need to limit by the number of actual transmitted SSBs in ssb-PositionsInBurst.

- Alt2: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

- Alt3: Number of configured TRS resource sets is not larger than twice the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

- Alt4: The maximum number of TRS resource sets per cell within a band is function of L_max.

[1RD]

Please a) update or provide your views for alternative(s) you support to resolve issue 5-3.

| Support | Companies |
| --- | --- |
| Alt1 | Vivo, Panasonic, Sharp, ZTE, Nokia, MediaTek, Ericsson1, Huawei, HiSilicon, LG |
| Alt2 | Samsung |
| Alt3 | Qualcomm, Apple |
| Alt4 | Nordic |

and, b) provide additional comments (e.g. clarification, concern, or suggestion) below

| Company | Comments |
| --- | --- |
| Nordic | We are OK with Alt2 and Alt 3 as well |
| Xiaomi | Support Alt 1. Alt 1 is the most flexible. there can be multiple TRS resource set on one SSB. what’s reason to introduce restrictions? |
| CMCC | Support Alt 1. |
| DOCOMO | Support Alt 1. |
| Apple | Alt 3 seems to be a good compromise that addresses UE implementation complexity without restricting the benefit in practical sense. |

### Issue 4-4: Others

Based on the proposals in Section 5, the following were proposals regarding other issues on TRS resources configuration:

- P1: The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set.

- P2: The following TRS resource set configuration parameters, e.g. startingRB, nrofRBs, periodicity, for multiple/all TRS resource sets to reduce the signaling overhead

- P3: An SSB index indicating a QCL source of a TRS resource set implicitly indicates a TRS resource set ID.

- P4: For QCL configuration, extend the configuration to configure the QCL source of a TRS resource set as a list of SSBs.

[1RD]

Please a) update or provide your views for whether to support or discusse the proposals.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: ZTE, Nordic, Sharp, Samsung, Apple No: |
| P2 | Yes: CATT, Apple, Nordic No: Samsung |
| P3 | Yes: Lenovo, Nordic No:Samsung, Apple |
| P4 | Yes: Huawei, HiSilicon |

and, b) provide additional comments (e.g. justification, concern) below

| Company | Comments |
| --- | --- |
| Nordic | P2: In general signaling optimization could apply to any parameter of a resource set P3: This depends on 4-3 |
| Ericsson1 | OK to discuss P2. |
| Samsung | For P2 or P3, we think they may be some optimization which are not essential. |
| Huawei, HiSilicon | P1: support. We assume this is the common understanding. P2: we are open to discuss this. P3: So far, we don’t see the need to define the TRS resource set ID. More justification is needed. Beside the three proposals, we also want to discuss the QCL configuration, which is captured by the FL in section 6. For the convenience of companies, we summarize the core idea below: In real NR deployment, it may happen that SSBs use narrow beams while TRS resources use wide beam(s). In the following figure, if TRS1 is only configured to be QCL-ed with SSB1, when UE moves under SSB2 the UE cannot jointly use TRS1 and SSB2. This reduces the power saving gain of TRS. As suggested by Mr. Chairman in Monday GTW session, we added the P4 to collect companies’ views. |
| Apple | P3: it is not clear to us whey a TRS resource set ID is needed. |

## 5.2 <2nd round discussion>

### Issue 4-1: number of TRS resources

The majority support explicit configuration of X for FR2. For FR1, the views are more divergent.

[2RD] Proposal 4-1 (v1) Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is down-select from one of the following alternatives - Alt1: X is based on configuration parameter with applicable value {2, 4} - Alt2: X = 4 - Alt3: X = 4, except when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon in which case X=2.

Please a) provide your views about whether or not to support proposal 4-1 (v1)

| Support | Companies |
| --- | --- |
| Yes | w/ Alt1: Apple,OPPO, Panasonic, Samsung, CMCC, Lenovo,ZTE, Intel, TCL, Huawei, HiSilicon, MediaTek,Xiaomi w/ Alt2: Nordic, LG w/ Alt3:Sharp, Ericsson2 |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Apple | This is the same comment as we provided for R1: We think we should provide full flexibility that is available for connected UEs, which is why we think Alt 2/3 are too restrictive. For FR1, “For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RSResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DLConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.” So for connected UEs, the determination also depends on tdd-UL-DL-ConfigDedicated, and this info is not available to idle/inactive UEs. By using tdd-UL-DLConfigurationCommon only, the idle/inactive UEs may derive different configuration from connected UEs. Therefore, it is much easier just to directly indicate it, and it is just a single bit. |
| Nokia2 | In proposal 4-2 the number of TRS resources should be aligned for the number of scrambling IDs, thus it should be possible to differentiate the number of TRS resources X={2,4} when number of scrambling IDs is two or four. While fixed value (e.g. X=2 for FR2) would need to be configured for the case when number of scrambling ID is one, it would not anyway restrict the network flexibility. Regarding the FR1, having a UL/DL slot configuration without any consecutive DL slots makes it rather difficult to operate/configure IDLE/Inactive mode operation (or contradict with the configuration for dedicated), thus it could be omitted for the IDLE TRS case without significant loss of generality. |
| CMCC | As we commented in the first round, both 1 slot or 2 slots TRS are supported for FR1, there is no reason to preclude the TRS configuration flexibility. |
| LG | We prefer Alt 2, but also ok with Alt 1 if it is supported by majority companies. |

### Issue 4-2: Scrambling ID

The difference between Alt1 and Alt2 is whether or not the scrambling ID are configured per TRS resource or per TRS resource set. For Alt2, it’s always per TRS resource set as scrambling ID is the only parameter that may be different for TRS resources within the TRS resource set.

[2RD] Proposal 4-2 (v1) One or more scrambling IDs is configured for TRS resources within a TRS resource set or for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources within a TRS resource set. - Otherwise, the number of scrambling IDs is equal to the number of TRS resources - Note: It is left to RAN2 decision on whether the one or more scrambling IDs are configured per TRS resource or per TRS resource set.

Please a) provide your views about whether or not to support proposal 4-2 (v1)

| Support | Companies |
| --- | --- |
| Yes | Apple, Nordic,OPPO, Panasonic, Samsung,Sharp, CMCC,ZTE(without note), Intel, TCL, Huawei, HiSilicon, LG,Xiaomi |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Nokia2 | Note seems to imply that multiple scrambling IDs could be configured per TRS resource, thus would need to be in minimum updated. It is not clear there is any need to introduction of TRS-resource IE, thus would prefer put the scrambling ID(s) to TRS-resource set. |
| Lenovo | We think one or more scrambling IDs should be configured per TRS resource set. |
| Ericsson2 | Similar comment as Nokia – simpler to have it per TRS resource set like all the other parameters (i.e. powerControlOffsetSS, firstOFDMSymbolInTimeDomain, startingRB, nrofRBs, ssb-Index, periodicityAndOffset, frequencyDomainAllocation) |
| Huawei, HiSilicon | If we leave the detailed signalling design to RAN2, we would like to make the following revision to be more neutral between Alt.1 and Alt.2: Proposal 4-2 (v1) One or more scrambling IDs is configured for TRS resources within a TRS resource set or for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources within a TRS resource set. - Otherwise, the number of scrambling IDs is equal to the number of each TRS resources is provided with a scrambling ID - Note: It is left to RAN2 decision on whether the one or more scrambling IDs are configured per TRS resource or per TRS resource set. |

### Issue 4-3: Maximum number of TRS resource sets

The conclusion is drafted based on the majority view to support Alt1 without additional spec effort.

[2RD] Conclusion 4-3 (v1) The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

Please a) provide your views about whether or not to support Conclusion 4-3 (v1)

| Support | Companies |
| --- | --- |
| Yes | Panasonic, Nokia2,Sharp,ZTE, LG, MediaTek,Xiaomi |
| No | Nordic |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |
| Apple | We understand that Alt 1 has the majority view, but we still wonder if Alt 3 can be considered as a compromise. It helps the UE without meaningfully degrade the operation in our view. Another alternative is to set a smaller limit for FR1, e.g. 12. |
| Nordic | Supporting 64 resource sets for bands with max 4 SSBs is criminal. UE is not required to support 64 resource sets in FR1 even in RRC connected Component 3) Max # of TRS resource sets configured to UE per CC Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. |
| Nokia2 | IDLE mode UE is not required to monitor all 64 TRS in my understanding, thus it seems to be a victimless crime. |
| Lenovo | We don’t see the need for conclusion 4-3. |
| Ericsson2 | OK in principle, although conclusion is not needed. |

### Issue 4-4: Others

A new proposal (P4) was added by HW. Many companies haven’t provided views yet. So, FL suggests to continue the same discussion in the first round.

[2RD] - P1: The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. - P2: The following TRS resource set configuration parameters, e.g. startingRB, nrofRBs, periodicity, for multiple/all TRS resource sets to reduce the signaling overhead - P3: An SSB index indicating a QCL source of a TRS resource set implicitly indicates a TRS resource set ID. - P4: For QCL configuration, extend the configuration to configure the QCL source of a TRS resource set as a list of SSBs.

Please a) update or provide your views for whether to support or discuss the proposals.

| Proposals | Companies |
| --- | --- |
| P1 | Yes: ZTE, Nordic, Sharp, Samsung, Apple, Nokia2, Lenovo, intel, Huawei, HiSilicon, LG,Xiaomi No: |
| P2 | Yes: CATT, Apple, Nordic No: Samsung, Nokia2, Lenovo |
| P3 | Yes: Lenovo, Nordic No: Samsung, Apple, Huawei, HiSilicon, LG |
| P4 | Yes: Huawei, HiSilicon No: Samsung, Lenovo |

and, b) provide additional comments (e.g. justification, concern) below

| Company | Comments |
| --- | --- |
| Nordic | P2: In general signaling optimization could apply to any parameter of a resource set P3: This depends on 4-3 |
| Ericsson1 | OK to discuss P2. |
| Samsung | For P2 or P3, we think they may be some optimization which are not essential. |
| Huawei, HiSilicon | P1: support. We assume this is the common understanding. P2: we are open to discuss this. P3: So far, we don’t see the need to define the TRS resource set ID. More justification is needed. Beside the three proposals, we also want to discuss the QCL configuration, which is captured by the FL in section 6. For the convenience of companies, we summarize the core idea below: In real NR deployment, it may happen that SSBs use narrow beams while TRS resources use wide beam(s). In the following figure, if TRS1 is only configured to be QCL-ed with SSB1, when UE moves under SSB2 the UE cannot jointly use TRS1 and SSB2. This reduces the power saving gain of TRS. As suggested by Mr. Chairman in Monday GTW session, we added the P4 to collect companies’ views. |
| Apple | P3: it is not clear to us whey a TRS resource set ID is needed. |
| Nokia2 | P1: As the periodicityAndOffset is provided in resource set, not in resource it would seem natural that it would point to the first resource of the set (rest of the resources of the set coming in same or consecutive slots). P2: No, TRS resource sets could have different configurations. P3: If we want to map multiple TRS to same QCL source, the ID needs to be independent of the QCL source. Whether there is need for resource set ID can be left to RAN2. P4: Would not the configuration be achieved by configuring identical TRS resource set to different SSBs? Thus it seems like nothing additional is needed. |
| Samsung | For P4, we think SSBs used for initial access should have wider beams than TRS/CSI-RS for connected UEs. We think the QCL configuration should be same as for TRS resources in connected mode. |
| Lenovo | For P2, we think each TRS resource set should be able to configure different values for parameters startingRB, nrofRBs, and periodicity. For P3, ‘TRS-ResourceSetConfig’ can include a list of TRS resource sets addressed by TRS resource set ID. We are okay to leave this up to RAN2 signaling design. For P4, it is not clear to us that using TRS beam wider than SSB beam is a useful/typical implementation scenario. |
| Huawei, HiSilicon | We are fine with P1. And OK to further discuss P2. We don’t think P3 is needed before we have clear understanding on where the TRS resource set ID shall be used. If the intention is not explained clearly, there is even no need to be discussed in RAN2. Some feedback to Nokia on P4: If it is provided as multiple TRS configurations even though it is the same TRS resource, it would reduce the possibility on UE to do T/F tracking by using more SSBs and TRS resource, which may impact power saving benefit. Some feedback to Samsung and Lenovo on P4: It is not restricted by the specification that TRS beam cannot be wider than SSB beam. It can happen. |
| LG | For P2, we do not prefer further restriction, but ok with discuss the issue. For P4, as far as I concern, we have similar understanding with Samsung. |

## 5.3 <3rd round discussion>

### Issue 4-1: number of TRS resources

According to the discussion in 2nd round, no critical issue for each alternative. The proposal is updated by selecting Alt1 based on the majority view.

[3RD] Proposal 4-1 (v2) Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is down-select from one of the following alternatives - Alt1: X is based on configuration parameter with applicable value {2, 4} - Alt2: X = 4 - Alt3: X = 4, except when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon in which case X=2.

Please a) provide your views about whether or not to support proposal 4-1 (v2)

| Support | Companies |
| --- | --- |
| Yes |  |
| No |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 4-2: Scrambling ID

[Nokia, Lenovo, Ericsson] still prefer to consider multiple scrambling IDs. One benefit is that no need to introduction of TRS-resource IE.

So, option 2 is included for further check. For option 1, the modification from HW is integrated.

[2RD] Proposal 4-2 (v3) Down-select one of the following option Option 1 One or more scrambling IDs is configured for TRS resources within a TRS resource set or for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources within a TRS resource set. - Otherwise, the number of scrambling IDs is equal to the number of each TRS resources is provided with a scrambling ID - Note: It is left to RAN2 decision on whether the one or more scrambling IDs are configured per TRS resource or per TRS resource set. Option 2(Alt2 in 1st round discussion) One or more scrambling IDs is configured for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources. - Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource.

Please a) provide your views about whether or not to support proposal 4-2 (v1)

| Proposal | Companies |
| --- | --- |
| Option 1 | - Support: - Not support: |
| Option 2 | - Support: - Not support: |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 4-3: Maximum number of TRS resource sets

We have discussed different alternatives in 1st round, but only a few companies support additional restriction on the maximum number of TRS resources sets. The conclusion is drafted based on the majority view to support Alt1 without additional spec effort.

Since there is no consensus for the conclusion in the 2nd round,

- Proposal 4-3a (the original Alt3) supported by QC and apple is added for further discussion

[3RD] Conclusion 4-3 (v1) The number of configured TRS resource sets can be larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. Proposal 4-3a (v1): Number of configured TRS resource sets is not larger than twice the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

Please a) update or provide your views for alternative you support to resolve issue 4-3.

| Support | Companies |
| --- | --- |
| Conclusion 4-3 (v1) | Panasonic, Nokia2, Sharp, ZTE, LG, MediaTek, Xiaomi |
| Proposal 4-3a (v1): | Qualcomm, Apple |
| Others |  |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

### Issue 4-4: Others

For P1, no objection. So, a formal proposal is drafted accordingly for further check.

For others, the views haven’t converged yet. The positions were copied for continue discussion.

[3RD] - P1: The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set. - P2: The following TRS resource set configuration parameters, e.g. startingRB, nrofRBs, periodicity, for multiple/all TRS resource sets to reduce the signaling overhead - P3: An SSB index indicating a QCL source of a TRS resource set implicitly indicates a TRS resource set ID. - P4: For QCL configuration, extend the configuration to configure the QCL source of a TRS resource set as a list of SSBs. Proposal 4-4 (v1) The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set.

Please a) update or provide your views for whether to support or discuss the proposals.

| Proposals | Companies |
| --- | --- |
| Proposal 4-4 (v1) | - Support: ZTE, Nordic, Sharp, Samsung, Apple, Nokia2, Lenovo, intel, Huawei, HiSilicon, LG, Xiaomi - Not support: |
| P2 | - Support: CATT, Apple, Nordic - Not support: Samsung, Nokia2, Lenovo |
| P3 | - Support: Lenovo, Nordic - Not support: Samsung, Apple, Huawei, HiSilicon, LG |
| P4 | - Support: Huawei, HiSilicon - Not support: Samsung, Lenovo |

and, b) provide additional comments, if any, below

| Company | Comments |
| --- | --- |

# Others

In addition to the main topics in Section 2-5, some other issues or design aspects have been discussed by a few companies, and the corresponding proposals are captured below.

| HW | Proposal 9: For QCL configuration, extend the configuration to configure the QCL source of a TRS resource set as a list of SSBs. |
| --- | --- |
| Vivo | Proposal 6: RRC connected UE are not required to monitor availability indication field in paging DCI. It can be up to UE implementation whether to use the available TRS for T/F tracking. - If paging indicated TRS resources are overlapping with PDSCH, RRC connected UE does not perform rate matching on these TRS resources. |
| CATT | Proposal 2: The L1 signaling for dynamic availability indication should be one bit to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell. Observation 4: When CSI-RS resources are configured by SI without association with the paging occasion(s), UE might not use the TRS for channel tracking to achieve the UE power saving gain. Observation 5: gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE. Proposal 14: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead Proposal 15: The following procedure can be used for TRS/CSI-RS occasion(s) configuration: Step1) Configured parameters of TRS/CSI-RS resource from the resourceMapping and periodicityAndOffset of nzp-CSI-RS-Resource set Step 2) SIB indicates parameters details, including - QCL assumption of the configured TRS/CSI-RS resources associated with a SSB - TRS resource set group ID Step 3) TRS occasion(s) after the SSB is obtained based on the configured TRS/CSI-RS resource grid and periodicity in step1 and step 2 |
| Sharp | Proposal 6: Support zero-power PDSCH punching for REs overlapping with TRS resources configured in SIB |
| LG | Proposal 3: For REs that are configured for a TRS occasion(s) for idle/inactive UEs and that UEs can assume actual TRS transmission o The UE expects TRS transmission in the REs which are overlapped with scheduled PDSCH o The UE expects PDSCH REs, which overlaps with actual TRS transmission, are punctured after PDSCH RE mapping |
| Nordic | Observation-1: When TRS periodicity is larger than SSB periodicity, UEs with TRS location being far ahead of PF nominal location will have unnecessarily large power consumption. Proposal-4: For the case when TRS periodicity is larger than SSB cycle, consider delaying UE’s PF from nominal location to frame after TRS, in order to facilitate power saving. In other words, consider defining separate PF for R17 UEs supporting Idle TRS. |

Due to time limitation, the discussion for Section 6 is deprioritized and not addressed in first round discussion.

# Proposals for GTW handling

## 7.1 <GTW on 01/17>

The following proposals are suggested for GTW handling on 01/17, Monday.

Proposal 2-2 (v0) The applicable values: {64, 128, 256, 512} should be supported for the validity duration configured by higher layer,

## 7.2 <GTW on 01/19>

The following proposals are suggested for GTW handling on 01/19, Wednesday.

Proposal 4-1 (v2) Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2 - Applicable values for X: {2, 4} - For FR1, X is down-select from one of the following alternatives - Alt1: X is based on configuration parameter with applicable value {2, 4} - Alt2: X = 4 - Alt3: X = 4, except when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon in which case X=2. Proposal 4-2 (v3) Down-select one of the following option Option 1 One or more scrambling IDs is configured for TRS resources within a TRS resource set or for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources within a TRS resource set. - Otherwise, the number of scrambling IDs is equal to the number of each TRS resources is provided with a scrambling ID - Note: It is left to RAN2 decision on whether the one or more scrambling IDs are configured per TRS resource or per TRS resource set. Option 2(Alt2 in 1st round discussion) One or more scrambling IDs is configured for a TRS resource set. - If a single scrambling ID is configured, it applies to all the TRS resources. - Otherwise, the number of scrambling IDs is equal to the number of TRS resources, where each scrambling ID corresponds to one TRS resource. Proposal 1-1 (v2) UE can receive TRS availability indication before the expiration/end of validity duration associated with previous TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 indication. - For each bit indicated as ‘0’ in the availability indication field, the UE ignores the bit keeps the current assumption on the availability of the corresponding TRS resource set(s). Proposal 2-1 (v2) The applicable values :{64, 128, 256, 512} should be supported for the validity duration configured by higher layer, - Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX; a UE implementation may assume the validity duration length is no larger than 10.24s if the UE does not support eDRX. Proposal 3-1 (v1) Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. - Whether and how to enable PEI PDCCH and paging PDCCH based TRS availability indication at the same time or separately Proposal 1-2 (v2) When L1 availability indication is enabled If SIB configures TRS resource, support one of the following - Alt1: TRS availability indication field is present configured in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI. - Alt2: TRS availability indication field is present in DCI format 1_0 with CRC scrambled by P-RNTI. TRS availability indication field in DCI format 2_7 with CRC scrambled by PEI-RNTI is enabled/disabled by a higher layer configuration parameter. - Alt3: a configuration parameter to select the DCI format for the L1 available indication - If UE receives TRS availability indication in DCI format 2_7, the UE ignores TRS availability indication in DCI format 1_0 with CRC scrambled by P-RNTI.

# Conclusion

The following agreements were made in this meeting.

# References

[1]R1-2200033	Remaining issue on assistance TRS occasions for IDLE/inactive mode	Huawei, HiSilicon

[2]R1-2200067	Remaining issues of TRS for RRC idle and inactive UEs	ZTE, Sanechips

[3]R1-2200084	Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

[4]R1-2200150	Remaining issues of TRS/CSI-RS for paging enhancement	CATT

[5]R1-2200156	Remaining issues of TRS/CSI-RS occasion(s) for idle/inactive UEs	TCL Communication Ltd.

[6]R1-2200202	Maintenance on TRS/CSI-RS occasion(s) for idle/inactive UEs	Samsung

[7]R1-2200235	Discussion on TRS/CSI-RS occasion for idle/inactive Ues	NTT DOCOMO, INC.

[8]R1-2200277	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

[9]R1-2200299	TRS and CSI-RS for idle and inactive UE power saving	Qualcomm Incorporated

[10]R1-2200332	Further discussion on RS occasion for idle/inactive UEs	OPPO

[11]R1-2200363	Open issues on n TRS information forto IDLE/INACTIVE mode UEs	Nokia, Nokia Shanghai Bell

[12]R1-2200377	Discussion on the remaining details of TRS/CSI-RS occasions in idle/inactive time	Intel Corporation

[13]R1-2200398	Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs 	Panasonic

[14]R1-2200418	Remaining issues on indication of TRS configurations for idle/inactive UEs	Apple

[15]R1-2200464	Remaining issues on TRS/CSI-RS configuration and indication for idle/inactive UEs	xiaomi

[16]R1-2200479	Remaining aspects of TRS occasion provisioning for Idle/Inactive UEs	Ericsson

[17]R1-2200498	Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs	Sharp

[18]R1-2200533	Provision of TRS/CSI-RS for idle/inactive UEs	Lenovo, Motorola Mobility

[19]R1-2200576	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

[20]R1-2200586	Remaining issues on TRS/CSI-RS occasion(s) for idle/inactive UEs	CMCC

[21]R1-2200607	Maintenance on Design of TRS/CSI-RS Information for idle/inactive UEs	MediaTek Inc.

[22]R1-2200610	On TRS design for idle/inactive UEs	Nordic Semiconductor ASA

[23]R1-2200619	Remaining issues on TRS/CSI-RS occasions for idle/inactive UEs	InterDigital, Inc.

# Appendix: Previous Agreements

## RAN1#102-e

Agreements: - New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE. Agreements: The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. - Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share). - Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. - Note: Always-on TRS/CSI-RS transmission by gNodeB is not required - At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported - FFS for other RS types - FFS: Whether UE blind detection is required or not. Agreements: Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as: - AGC, time/frequency tracking - FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication Observation: It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.) Agreements: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling - FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.) - FFS for other signalling candidates (e.g., pre-configuration, etc.) - FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.) Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreement: - Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s). Agreements: - SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s). - Up to RAN2 to decide which SIB is to be used. - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2 Send an LS to RAN2 informing the above agreements, and - To further add that RAN1 is working on the detailed physical layer design Agreement: - Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs. Agreements: - Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting) - Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence Agreements: - Discuss further based on the following alternatives and down-select at RAN1#104-e: - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE. - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - The condition can be, e.g., existence of paging. - Alt 4. Combination of the above alternatives. - FFS for details - FFS for UE behavior when the availability is not informed. - Other techniques are not precluded. - Companies encourage to provide sufficient information for the proposal, e.g., - how to achieve power saving gain - how to minimize impact on NW how to minimize extra UE implementation complexity - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs - Proposals should be consistent with the WID objective. Conclusion: - TRS/CSI-RS based PEI is discussed in AI 8.7.1.1. - PEI functionality is not further discussed under AI 8.7.1.2. - Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Update on 1/28 email: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#105-e: - Alt1: same as initial BWP - Alt2: configurable parameter Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only) Update on 1/31: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). Conclusion From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs. Agreements: The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e: - Alt-1: within initial DL BWP - Alt-2: is not restricted by initial BWP - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. Agreements: To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives: - Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS - Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO - FFS details - FFS details - Other alternatives are not precluded Conclusion: Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104b-e

Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling To further check on 4/19 Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded

## RAN1#105-e

Agreement: Confirm the following working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources Agreement: Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 Agreement: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source Agreement: Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

## RAN1#106-e

Agreement Support at least one of the following alternatives - Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion. - Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion Note: The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication. Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only. Agreement L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where - the time duration can be determined based on at least one from the following (to be down-selected): - Alt-1: configured by higher layer - Alt-2: a predefined/configured window - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s) - Alt-4: until when the UE receives another availability indication - A combination of alternatives or other alternatives is not precluded. - the reference point can be determined as at least one from the following (to be down-selected): - Alt-1: start of next PO or DRX cycle - Alt-2: time location where UE receives the indication - Note: the time location is subject to application delay if agreed - Alt-3: start of current PO or DRX cycle where UE receive the indication - Alt-4: a time location which is configured by higher layer - A combination of alternatives or other alternatives is not precluded. Agreement For a RS resource configured for TRS/CSI-RS occasion(s) for idle/inactive UEs, a quasi co-location type can be determined as - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block

## RAN1#106bis-e

Conclusion No consensus to support SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement For L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability information for configured RS resources using a bitmap. where each bit indicates whether associated TRS resource(s) are available. - support L1 availability indication at an occasion can provide availability information RS resources with QCL references not confined to be the same as for the L1 availability indication occasion - FFS associated TRS resource(s) per bit, e.g. a bit is associated with a TRS resource set - Bitmap size is up to X bits - X = [6] for paging PDCCH based L1 availability indication. - FFS X for PEI DCI based L1 availability indication - FFS details about how to configure the DCI field: e.g. start and length of bitmap (e.g. explicitly/implicitly configured) - for paging PDCCH based L1 availability indication, support L1 availability indication at an occasion can provide availability information for all configured RS resources - FFS whether this needs to be supported regardless of the number of beams or for some configured RS resources - FFS: PEI DCI provides L1 availability indication information only for RS resources with QCL references to be the same as for the L1 availability indication occasion - FFS: indication of unavailability Agreement At least for paging PDCCH based L1 availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, the L1 availability indication is valid for a time duration starting from a reference point, where - the time duration is a validity duration configured by higher layer, - FFS applicable values, e.g. # of DRX cycles, or multiple of default paging cycle duration (i.e. modification period) - FFS UE doesn’t expect inconsistent L1 based indication during the time duration - the reference point for start of the validity duration is one of the following alternatives: - Alt1: SFN of the first PF from the next DRX cycle - Alt2: SFN of the first PF from the current DRX cycle where UE receives the indication - Alt3: based on SFN configured by higher layer, i.e. modification period configured as multiple of default paging cycle duration - Alt4: start of the PF for the PO where UE receives the indication - Note: the DRX cycle in Alt1 and Alt2 is the default paging cycle broadcast in SIB - Note: The SFN for the first PF is for (UE mod N) = 0, and can be calculated by (SFN + PF_offset) mod T = 0 - the time duration can be optionally configured by gNB - when the time duration is not configured, one of the following alternatives can be considered: - Alt1: the availability indication is valid until when the UE receives another availability indication. - Alt2: the availability indication is valid until L1 availability indication is changed by network - Alt3: default time duration e.g. default paging cycle - FFS whether and how to handle the miss detection issue of L1 signaling Agreement Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include a list of one or more TRS resource sets, where:  a TRS resource set can be configured to include o a set of TRS resources up to two consecutive slots,  Note: a TRS resource is same as Rel-15/16, i.e. a CSI-RS in a symbol. o at least common configuration parameters:  a QCL reference  firstOFDMSymbolInTimeDomain,  ‘frequencyDomainAllocation for row1’, ‘startingRB’ ,‘nrofRBs’,’powerControlOffsetSS’, periodicityAndOffset’  FFS  scramblingID,  a TRS resource set ID, number of slots {1, 2} or number of symbols {2, 4} if supported  Note: the ‘TRS resource set’ configuration is not (necessarily) identical to ‘NZP-CSI-RS-ResourceSet’ configuration for TRS in R15/16.

## RAN1#107-e

Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations. Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Note: Qualcomm and Huawei have concern on Alt a Agreement For the validity duration configured by higher layer at least for paging PDCCH based L1 availability indication, support - time unit is one default paging cycle, - applicable values: {1, 2, 4, 8, 16, 32, [64], [128], [256],[512]} When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s): Agreement Confirm the following working assumption Working Assumption - Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. - Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.