3GPP TSG RAN WG1 #107-bis-e			           R1-220XXXX

e-Meeting, January 17th – 25th, 2022

Agenda Item: 8.7.1.1

Source: Moderator (MediaTek)

Title: Summary of Maintenance on Paging Enhancement

Document for: Discussion and Decision

# Introduction

In this feature lead summary, there will collect, discuss and decide identified issues and proposed solutions/text proposals for the maintenance on paging enhancement related specifications [2]-[5]. In particular, the following show the category of topics to be discussed in each Section:

- Section 2: Whether and how a new PEI-RNTI for DCI format 2_7 is supported (including fixed or configurable)

- Section 3: The ranges of the offsets for PEI-O location determination:

  - PEI-F_offset

  - firstPDCCH-MonitoringOccasionOfPEI-O

  - Necessary structure change for the time offset parameters, if needed

- Section 4: Maintenance for PEI monitoring, including

  - Potential monitoring constraint, analogous to the following

If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

  - Assumption on UE behaviour, including what is expected if UE chooses not to monitor PEI

  - Necessary PEI monitoring related changes, if needed

- Section 5: Other issues, including proposals/changes for new functionality

As per chair’s guidance, as quoted below, any issue that has impact to other WG(s), should be prioritized. This principle will be strictly followed for the upcoming discussions and decisions.

- For all Rel-17 items that are treated in RAN#107bis-e (except MIMO and positioning), all RAN1 decisions that impact other WGs should be finalized in RAN1#107bis-e. For the remaining WIs, plan is to do so in the first week of RAN1#108-e.

# PEI-RNTI for DCI Format 2_7

In RAN1#107-e meeting, it is agreed that payload size of DCI format 2_7 can be the same as paging DCI:

Agreement For PEI DCI format, defined as DCI format 2_7, - Total number of bits for paging indication filed is POnumPerPEI, if is absent or set to 0 or 1, and the number is , if is configured. - For Rel-17, UE does not expect paging indication filed size is larger than the DCI payload size - Whether and how TRS availability indication field is included is up to Agenda Item 8.7.1.2 - Support configurable DCI payload size which should be no larger than payload size of paging DCI - Unused bits, when applicable, are regarded as reserved bits - Note: A smaller payload size is beneficial for PEI detection performance

In Table 1, there summarize companies’ views from [6]-[28] related to whether and how to support a new PEI RNTI:

Table 1

| Company | Companies’ views and proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: A new RNTI, PEI-RNTI, is used to scramble the CRC of PEI-DCI for idle/inactive mode UEs. And adopt the TP 2 in TS38.213. |
| ZTE, Sanechips |  |
| vivo | Proposal 5: Support a new PEI-RNTI for DCI format 2_7 and adopt the text proposal 2 provided in Appendix 2. |
| CATT | Proposal 6: A new RNTI is used to scramble the CRC of PDCCH-based PEI. |
| TCL | Proposal 1: Support PEI-RNTI with a fixed RNTI value for DCI format 2_7 |
| Samsung | Proposal 1: Support a new RNTI, e.g. PEI-RNTI, for DCI format 2_7 |
| DOCOMO |  |
| Spreadtrum |  |
| Qualcomm | Proposal 1: A dedicated PEI-RNTI is configured by network for CRC scrambling of PEI PDCCH. |
| OPPO | Proposal 1: Support new PEI-RNTI for DCI format 2_7. |
| Nokia | Proposal: Introduce PEI-RNTI with fixed value. |
| Intel | Proposal 1: New PEI-RNTI can be used for PEI DCI and it can be specified. |
| Panasonic | Proposal 3: Define a new semi-statically configurable RNTI for Rel.17 PEI. |
| Apple |  |
| xiaomi |  |
| Ericsson | - Proposal 1: PEI design should allow the use of reserved bits in paging DCI in one PO as paging early indication for UEs in one or more groups in other POs. - Proposal 2: For the PEI DCI, the RNTI used for CRC masking is configured via higher layers. - Proposal 3: If full configurability is not agreeable, it should at least be possible to configure the UE with either the P-RNTI or PEI-RNTI for PEI CRC scrambling. |
| Lenovo, Motorola Mobility | Proposal 2: RNTI for a PEI PDCCH is determined based on the first (i.e. earliest) PF of one or multiple PFs associated with the PEI PDCCH. |
| Transsion | Proposal 2: New and fixed PEI-RNTI for PEI is supported. |
| LG Electronics | Proposal 2: A new RNTI value dedicated for PEI is supported. The new RNTI value is fixed and not configurable. |
| CMCC | Proposal 1. Support a new PEI-RNTI with a fixed value for DCI format 2_7 and the following TPs are suggested: TS 38.213 section 10.1: <Unchanged text is omitted> - a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG <Unchanged text is omitted> TS 38.213 section 10.4B: <Unchanged text is omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged text is omitted> |
| MediaTek | Proposal 1: A new fixed PEI-RNTI is supported for DCI format 2_7. Value of PEI-RNTI is up to RAN2 design. |
| Nordic Semiconductor ASA |  |
| InterDigital |  |

From the above views, it is observed that 16 out of 16 companies all support a new RNTI. In this regard, moderator would like to support a quick proposal, regarding the consensus:

Proposal 2-1:

A new PEI-RNTI is supported for DCI format 2_7. The following text proposals are adopted:

TS 38.213 section 10.1: <Unchanged text is omitted> - a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG <Unchanged text is omitted>

TS 38.213 section 10.4B: <Unchanged text is omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged text is omitted>

Regarding the value of the new RNTI, the following statistics show slight majority on a fixed value. Accordingly, Proposal 2-2 is suggested for further discussion, and companies are encouraged to provide comments/suggested changes to Proposals 2-1 and 2-2 in the table below.

- Fixed value (5): TCL, Nokia, Transsion, LG, MTK, Huawei, HiSilicon

- Configurable value (3): QC, Panasonic, Ericsson

- Other: Lenovo (function of 1st PF)

Proposal 2-2:

PEI-RNTI is of fixed value, and value design is up to RAN2

Table 2: Companies’ comments/suggested changes to Proposal 2-1 and Proposal 2-2

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | 2-1 OK, 2-2 OK |
| Xiaomi | OK with Proposal 2-1 and 2-2 |
| Spreadtrum | OK for both |
| Qualcomm | For 2-2, we prefer to have a network configured PEI-RNTI as the PEI PDCCH DCI size can be different than the paging PDCCH DCI size. Then, network does not need to use different RNTI value to CRC scramble the PEI PDCCH. |
| Samsung | Since PEI is not mandatory, we prefer configurable RNTI instead of fixed value. |
| CMCC | Support 2-2 |
| LG | OK with proposals. |
| Sharp | OK |
| ZTE, Sanechips | A configurable RNTI is preferred |
| Ericsson1 | We support to have configurable RNTI. Configurable RNTI does not introduce extra complexity for the UEs and we do not see any advantage of using a predefined RNTI. The disadvantage is that it unnecessarily precludes configuring P-RNTI. |
| Apple | Fine with both proposals |
| Huawei, HiSilicon | We support proposal 2-2, and there is no clear benefit to introduce additional configuration signalling. |
| Intel | Support 2-2. We do not see strong need for additional configurability here. Similar to P-RNTI, a fixed value can be specified. |
| Nokia | We support 2-2. |
| Transsion | Both 2-1 and 2-2 are OK |
| vivo | OK for these two proposals. |
| IDCC | Ok. |
| OPPO | Support the proposals. |
| DOCOMO | Support |
| MediaTek | We support Proposal 2-2, i.e., fixed value for PEI-RNTI. The intention to have configurable PEI-RNTI looks to map DCI format 2_7 content into paging DCI. We, however, think it may not be useful for the following reasons: - The content multiplexing can save a PDCCH “only when” there is paging DCI for another PO. Network is required to send a dummy paging DCI (using PEI-RNTI set to P-RNTI) for the target PO if there is no paging to the “sharing” PO. This will increase false paging probability for the sharing PO. Since most of content of the dummy paging DCI is not used, this also means waste of resource. - If reducing PDCCH overhead for paging is the target, an alternative is to reduce total PO number while using PEI to carry paging indications for multiple POs/subgroups. Since legacy UE power consumption is dominated by SSB processing instead of false paging rate, there is minimum impact to legacy UE. On the other hand, 1-PEI-multi-PO characteristics can be used to reduce the overall PDCCH overhead for paging, as illustrated in the figure below (Figure 5 of R1-2112308) - If PO location is not friendly for UE power saving, PEI-O location should be properly adjusted to achieve power saving. Reusing PO location for PEI-O cannot achieve useful power saving, as shown in Figure 1 of R1-2112308 and quoted below: By the above check, more clarification on the benefit of mapping DCI format 2_7 to paging DCI is needed. Currently, a fixed RNTI looks sufficient. p.s. Another interesting sharing is to share DCI format 2_7 with DCI format 2_6. Since PS-RNTI is already configurable, we also see no need to have a configuration PEI-RNTI. |
| Panasonic | On 2-1, we support. On 2-2, we support configurable value as first priority but can accept fixed value for sake of progress. |
| CATT | We are OK with proposals 2-1 and 2-2. For IDLE UE, it would not be possible to configure PEI-RNTI UE-specifically by gNB. The PEI-RNTI could only be broadcasted by SIB-X, which has the same meaning of fixed value |
| TCL | We are ok with this proposal and support fixed value for PEI-RNTI |

# Ranges of The Time Offsets for PEI-O Location Determination

In RAN1#107-e [4], the ranges of a frame-level offset and a symbol-level offset for PEI-O location determination remain to be specified (as quoted below), and Figure 1 illustrates how the offsets are utilized for the PEI-O location, for the case one PEI-O is mapped to POs of a PF or two PFs.

Agreement - Determination of PEI-O location for UE’s PO is based on deciding a reference point and an offset from the reference point to the start of the first PDCCH MO of the PEI-O. o The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O and configured via SIB for the cell. - FFS: The range of the frame-level offset o There is a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O and configured via SIB for the cell. - FFS: The range of the symbol-level offset - Note: When PEI-O is placed close to or overlapped with an earlier SS burst before its associated POs, the total UE wake-up time can be reduced for better power saving gain. Network can configure the PEI-O location accounting the power saving benefit and potential impact on gNB flexibility.

Figure 1: Illustration of PEI-O location determination

In Table 3, there summarize companies’ views related to time offsets for PEI-O location determination:

Table 3

| Company | Companies’ views and proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: For the values for the frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, support an integer number X of half SS burst periodicity, where X is a number in the value range of {0,1,2,3,4,5,6}. Proposal 2: For the symbol-level offset from the reference point, determined by the start of frame-level offset, to the start of the first PDCCH monitoring occasion of PEI-O, support the values of: - {0..139} for SCS 15kHz - {0..279} for SCS 30kHz - {0..559} for SCS 60kHz - {0..1119} for SCS 120kHz |
| ZTE, Sanechips | - Proposal 4: The range of the frame-level offset can be {0, 1, 2, 3, 4, 5, 6}. - Proposal 5: The range of the symbol-level offset is suggested as 0-139 for 15 KHz SCS, 0-279 for 30KHz SCS, 0-559 for 60KHz SCS, 0-1119 for 120KHz SCS. - Proposal 6: One frame-level offset and multiple symbol-level offsets are needed for the PEI-to-PO mapping when the number of the POs associated with one PEI (NPOPEI) is smaller than the number of POs in one PF (NS). - Proposal 7: The symbol-level offset of the PEI corresponding the (iS + 1)th PO is the (floor((iS + 1)/NPOPEI)+1)th value configured by the symbol-level offset. |
| vivo | Proposal 1: The value range of the frame-level offset (PEI-F_offset) should be {4, 5, 6, 7, 8…}. Proposal 2: The value range of the symbol-level offset (firstPDCCH-MonitoringOccasionOfPEI-O) can be one of the following: - Option 1: the value range is from 0 to the length of PEI-frame i.e., (0…10*2μ*14-1) where μ is the subcarrier spacing (SCS) configuration. - Option 2: Similar to the configuration of firstPDCCH-MonitoringOccasionofPO defined in TS 38.331 i.e., the value range is (0… 10*2μ*14x-1), where x=1, 1/2, 1/4, 1/8, 1/16… represents the density of PEI-frame in a paging cycle and is configured by RRC. - Option 3: the value range is from 0 to the length of the configured PEI-F_offset i.e., (0…f*10*2μ*14-1), where f is the value of the PEI-F_offset. Proposal 3: Configure the list of firstPDCCH-MonitoringOccasionOfPEI-O per PEI-frame or per paging cycle. Each of PEI-Os in the list corresponds to a firstPDCCH-MonitoringOccasionOfPEI-O parameter. |
| CATT | Proposal 1: The range of frame-level offset PEI_offset is {0, 1}. Proposal 2: The range of symbol-level offset, i.e. firstPDCCH-MonitoringOccasionOfPEI-O, is same as the range of firstPDCCH-MonitoringOccasionOfPO. Proposal 3: The PEI-O location could be determined by one of the following methods: - Method 1: Four steps are used to determine the PEI-O location. - Method 1_step 1: Calculate PF_Index. - PF_Index = (floor(SFN_PF*N /T)) mod (Ceil(PONumPerPEI/Ns)) - PF_Index is the index of PF within the PF(s) associated with the PEI-O - SFN_PF is the SFN of UE’s Paging Frame in the DRX cycle - N is the number of total paging frames within T - T is a DRX cycle - Ns is the number of paging occasions for a PF - PONumPerPEI is the number of PO associated with the PEI-O - Method 1_step 2: The first PF of PF(s) associated with one PEI is the PF with PF_Index = 0. - Method 1_step 3: Calculate the reference frame based on the first PF and frame-level offset. - (SFN_PEI + PEI_offset) mod T = SFN_FirstPF - SFN_PEI is the SFN of reference frame of PEI occasion - PEI_offset is a frame-level offset used in determining the reference frame of PEI occasion - T is a DRX cycle - SFN_FirstPF is the SFN of first PF of the PF(s) associated with a given PEI - Method 1_step 4: Calculate the index of the PEI within reference frame. - PEI_i_s = (floor(UE_ID/N)mod Ns) mod Ceil(Ns/PONumPerPEI) - PEI_i_s is the index of the PEI within reference frame - N is the number of total paging frames in T - Ns is the number of paging occasions for a PF - PONumPerPEIis the number of PO associated with the PEI-O - UE_ID is the 5G-S-TMSI mod 1024 - Method 2: Two steps are used to determine the PEI-O location. - Method 2_step 1: Calculate reference frame based on the first PF and frame-level offset. - (SFN_PEI + PF_offset + PEI_offset) mod T = (T div N) * floor((UE_ID mod N) /A) * A - SFN_PEI is the SFN of reference frame - PF_offset is a frame-level offset used for PF determination - PEI_offset is a frame-level offset used for reference frame determination - T is a DRX cycle - N is the number of paging frames in a DRX cycle - Ns is the number of paging occasions in a Paging Frame - PONumPerPEI is the number of PO associated with the PEI-O - A = Ceil(PONumPerPEI/Ns) - UE_ID is the 5G-S-TMSI mod 1024 - Method 2_step 2: Calculate the index of the PEI within the reference frame of PEI occasion. - PEI_i_s = (floor(UE_ID/N)mod Ns) mod Ceil(Ns/PONumPerPEI) - PEI_i_s is the index of the PEI within the reference frame - N is the number of total paging frames in T - Ns is the number of paging occasions for a PF - PONumPerPEI is the number of PO associated with the PEI-O - UE_ID is the 5G-S-TMSI mod 1024 |
| TCL | Observation 1: A frame level offset of 2 radio frames satisfy the condition of one SSB burst with 20m sec periodicity between the reference point and the first PO of the first PF of the PF(s) associated with the PEI-O. Observation 2: A frame level offset of 6 radio frames satisfy the condition of three SSB bursts with 20m sec periodicity between the reference point and the first PO of the first PF of the PF(s) associated with the PEI-O. Proposal 2: For the frame level offset between the PEI-O and the first PF of the PFs associated with PEI-O, a flexible range of frame level offset; i.e. {2, …. , 48} radio frames offset should be defined to guarantee the idle/inactive UE synchronization before paging. Proposal 3: Define the symbol level offset from the reference point to the start of the first PDCCH MO of PEI-O according to the firstPDCCHmonitoringOccasionOfPO as defined in TS 38.331. |
| Samsung | Proposal 2: Adopt applicable value for PEI-F_offset in the range of [Omin, …, PagingCycle] for TS 38.331. - Omin = [3] Proposal 3: Adopt applicable value for firstPDCCH-MonitoringOccasionOfPEI-O 0, …, 140*2μ-1, where is SCS configuration of initial DL BWP. |
| DOCOMO |  |
| Spreadtrum | Proposal 4: The range of PF offset can be reused for the range for the frame-level offset for PEI occasion. Proposal 5: The range of the symbol-level offset for PO can be reused for the range for the symbol-level offset for PEI occasion. |
| Qualcomm | Proposal 4: Range of the frame-level offset for PEI-O is {0, 1, 2}. Range of the symbol-level offset is from 0 to 140×2μ-1 to cover the duration of a radio frame where μ is the numerology factor of the initial DL BWP. |
| OPPO | Proposal 2: When the POs in same PF are associated with different PEI-Os, how to configure the frame-level offset and symbol-level offset need to be clear. Proposal 3: When the POs in same PF are associated with different PEI-Os, (Ns/POnumPerPEI) frame-level offsets and symbol-level offsets could be configured. - If PEI-Os are located in the same PF, corresponding frame-level offsets have the same value, while the corresponding symbol-level offsets have the different value. - If PEI-Os are located in different PFs, corresponding frame-level offsets have different value, while corresponding symbol-level offsets could have the same or different value. |
| Nokia | Proposal: PEI frame offset (PEI-F_offset) could be defined in radio frames, and be separate for each PEI, with range of {1,2,3,…,15,16}. Proposal: PEI symbol level offset (firstPDCCH-MonitoringOccasionOfPEI-O) range could be defined to cover at least following range: - for 15kHz sub-carrier spacing: {0,1,2,..,139} - for 30kHz sub-carrier spacing: {0,1,2,..,279} - for 60kHz sub-carrier spacing: {0,1,2,..,559} - for 120kHz sub-carrier spacing: {0,1,2,..,1119} - for 480kHz sub-carrier spacing: {0,1,2,..,4479} - for 960kHz sub-carrier spacing: {0,1,2,..,8959} - FFS for additional values for N>1 cases. |
| Intel | Proposal 4: For determination of PEI-O location, - Possible values of frame-level offset include 0 and 1. - Unit and range of symbol level offset follow same design as firstPDCCH-MonitoringOccasionOfPO |
| Panasonic | Observation 1: An frame-level offset with range {0, 1, 2, …, 15} is able to provide sufficiently small gap between SSB and PEI. Between PEI and paging PDCCH, there can also possibly be one or more SSB bursts for UE to use to improve paging PDSCH performance, depending on the SSB periodicity configuration. Observation 2: Although the maximum gap between the first PFs of two PEI-Os can be 32 radio frames, it is not so necessary to support the full range offset as the allocation of PFs is already sufficiently flexible. Proposal 1: To determine the PEI-O location, the range of the frame-level offset used to determine the start of the reference frame should be from 0 to no larger than 15. We are also open to discuss a smaller range. Proposal 2: To determine the PEI-O location, the symbol-level offset from the reference point reuses the range of high layer parameter firstPDCCH-MonitoringOccasionOfPO. The exact name and structure definition of this symbol level offset can be up to RAN2. |
| Apple |  |
| xiaomi |  |
| Ericsson | - Proposal 6: PEI transmissions should not be restricted to be in conjunction/adjacent to other transmission. - Proposal 7: The range of the frame-level offset (PEI-F-offset) is up to 8 frames. - Proposal 8: The range of the symbol-level offset (firstPDCCH-MonitoringOccasionOfPEI-O) is up to 1119 symbols. |
| Lenovo, Motorola Mobility |  |
| Transsion | Proposal 3: Frame-level offset can select from the set of {0,1}. Proposal 4: Symbol-level offset can follow the PO principle. |
| LG Electronics | Proposal 1 - The value range of the frame level offset, which is for determining a reference point of PEI, is {1, 2, 3 … 16} radio frames. - The value range of the symbol level offset for PEI is same as the value range of the symbol level offset for PO. |
| CMCC | Proposal 2. The SFN for PFI is determined by: (SFN + PF_offset - PEI-F_offset) mod T = (T div (N/PFnumPerPEI))*((UE_ID mod N) mod PFnumPerPEI) Where PFnumPerPEI =1 if Ns>= POnumPerPEI and PFnumPerPEI =2 if Ns < POnumPerPEI. The value range of PEI-F_offset can be {0,1,2,3…,16}. Proposal 3. firstPDCCH-MonitoringOccasionOfPEI-O comprises PFnumPerPEI*Ns/POnumPerPEI entries and the same range definition as firstPDCCH-MonitoringOccasionOfPO is reused for each entry of firstPDCCH-MonitoringOccasionOfPEI-O. |
| MediaTek | Observation 1: Design of the PEI-F-offset time offset range should ensure: 1) The reference frame contains a SS burst, and 2) There is at least one SS burst between PEI-O and PO. Proposal 3: The range of the frame-level offset for PEI-O, i.e., PEI-F_offset, is {3, 4, 5, 6}. Proposal 4: The range of the symbol-level offset for PEI-O, i.e., firstPDCCH-MonitoringOccasionOfPEI-O, has effective minimum of 1 slot, i.e., 14 symbols, and effective maximum of 2 ms, i.e., 28, 56, 112, 224 symbols for μ=1, 2, 4, 8, respectively. |
| Nordic Semiconductor ASA | Proposal-1: Range of frame-level offset could be from 1 to 16 frames. Reuse firstPDCCH-MonitoringOccasionOfPO ranges for PEI. |
| InterDigital | Proposal 1: The range of the frame level offset is {0, 1}. Proposal 2: When firstPDCCH-MonitoringOccasionOfPEI-O is not present, the symbol-level offset is set to 0. |

Although Figure 1 shows one frame-level offset and one symbol-level offset will be sufficient, multiple time offsets will be necessary for the case one PEI-O is only mapped to part of POs of a PF, as shown in the figure below quoted from [7].

Figure 2: Need of multiple time offsets for the case one PEI-O is mapped part of POs of a PF [7]

In analogy of PO location determination design where multiple POs can apply multiple symbol-level offsets from a common PF start, the method of multiple symbol-level offsets can be applied to PEI-O. Accordingly, the following proposal is suggested, and companies are encouraged to provide your comments/suggested changes to Proposal 3-1 in the table below:

Proposal 3-1:

One frame-level offset and multiple symbol-level offsets are supported for the PEI-to-PO mapping when the number of the POs associated with one PEI (NPOPEI) is smaller than the number of POs in one PF (NS).

Table 4: Companies’ comments/suggested changes to Proposal 3-1

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We do not support, different MO-offsets should be configured for different PEIs |
| Xiaomi | Support Proposal 3-1 in general. |
| Spreadtrum | Different PEI-O should have different starting symbol? If it is true, the proposal is reasonable. Oppositely, if the different two PEI-Os can share the same starting symbol, but FDMed in a CORESET, the proposal can be postponed. |
| Qualcomm | “number of the POs associated with one PEI (NPOPEI) is smaller than the number of POs in one PF (NS)” should be precluded. There is no clear benefit why such a design flexibility should be supported by Rel-17 PEI design. It only makes the design unnecessarily complicated without UE power saving gain benefit. Our understanding is the PEI should be always aligned with a SSB so that UE can wakeup once to receive both the SSB for tracking loop update and receive the PEI. If “number of the POs associated with one PEI (NPOPEI) is smaller than the number of POs in one PF (NS)”, the alignment cannot be achieved for all PEI-Os anymore. Based on this, we do not support the proposal. |
| Samsung | We share similar view as QC. We don’t see the need to address the case when multiple PEI-O are mapped to a single PF. We think one symbol-level offset is enough. |
| CMCC | Support 3-1, it gives gNB more flexibility to configure the mapping between PO and PEI. |
| LG | We support the proposal. To avoid overlap between multiple PEI-Os correspond to different POs in a same PF, it is necessary to have symbol level offset configuration per PEI-O. If not, UE would miss understand the paging indication and it would wake up due to a PEI transmission for other PO. |
| Sharp | We support one symbol-level offset for the PEI-to-PO mapping, and the PEI occasion associated to one PEI frame can be arranged in the similar way as that for paging occasion associated to one PF. |
| ZTE, Sanechips | We support this proposal. In the previous meetings, there is no consensus to convey paging indication for Ns POs in one PF (or reuse the legacy parameter, i.e., Ns). As a consequence, it was agreed that the PEI can be used to indication information for 1, 2,4 POs for the sake of better flexibility. In addition, carrying less information in one PEI would beneficial to reduce the payload size, reduce the paging alarming rate, and improve the performance, etc. Therefore, “number of the POs associated with one PEI (NPOPEI) is smaller than the number of POs in one PF (NS)” is a reasonable case. In this case, it is a straightforward solution to map different PEIs in different time location, which is similar with PO mapping. We agree with Spreadtrum that multiplexing these PEIs in frequency domain may not possible or lead to congestion considering the limited bandwidth for initial DL BWP. Moreover, as these POs are in the same PF, there is no need to have multiple frame level offsets, using multiple symbol level offset is sufficient. |
| Ericsson1 | In our understanding, configuring of one symbol level offset should also be supported for this case, which is already in the spec. |
| Apple | Our understanding has always been that both the frame-offset and symbol-level offset are configured per PEI-O. If this is the case, Proposal 3-1 would not be necessary. |
| Huawei, HiSilicon | The benefit of such design seems not clear. Actually, we are not sure why gNB wants to only associate partial POs in a PF. gNB could configure only two POs in a PF or associate all POs in the same PF. We are reluctant to optimize this during CR phase. |
| Intel | We do not support. Different PEI-Os should have different symbol offsets based on configuration. Agree with QC that PEI addressing number of POs less than Ns is not a useful case to study/optimize for. |
| Nokia | After last meeting discussion, my understanding of the conclusion was similar as Apple that we would do the configuration per PEI, thus each PEI could have it’s own frame- and symbol-level offset. |
| Transsion | Support multiple PEIs associated with POs of the same PF start with different symbols. |
| vivo | We can accept with proposal 3-1, if the intention is to configure the list of firstPDCCH-MonitoringOccasionOfPEI-O per PEI-frame to be as similar as that of firstPDCCH-MonitoringOccasionOfPO configured per PF given below. And the maximum size of the list (i.e., the maximum number of associated symbol-level offsets for each PEI-frame) is equal to Ns. |
| IDCC | Agree with most other companies that one offset value should be sufficient. But we can accept multiple values (analogous to PO design) if there is a clear technical need. |
| OPPO | Support the proposal. |
| MediaTek | We think Proposal 3-1 is an analogous solution as POs that shares the same PF (i.e. same understanding as vivo). Regarding the benefit that the structure of can be firstPDCCH-MonitoringOccasionOfPO literally reused for firstPDCCH-MonitoringOccasionOfPEI-O, we are supportive to Proposal 3-1. On the other hand, the suggestion from Qualcomm is also one alternative solution which can simplify the feature configurations. Apple’s understanding is also reasonable. Yet, how UE chooses the corresponding pair of frame-level and symbol-level offsets needs to be clarified for the case there are multiple PEI-Os associated with the POs of a PF. |
| Panasonic | We do not think such optimization is necessary at this stage. |
| CATT | We are OK with the principle to have one or more symbol-level offset. Thus, we suggest to change “many symbol-level offset” to “one or more symbol-level offset” |
| TCL | We are generally fine with this proposal. |

For the range of the frame-level offset, i.e., PEI-F_offset, the following summarize the statistics of companies’ views:

- Maximum:

  - < 6 (5 companies) : CATT (max = 1), QC (max = 2), Intel (max = 1), Transsion (max = 1), IDC (max = 1)

  - 6 (3 companies): HW, ZTE, MTK

    - HW: Unit can be (SS burst period / 2)

  - >6 (10 companies): vivo, TCL (max = 48), Samsung (max = paging cycle), Spreadtrum (max = paging cycle), Nokia (max = 16), Panasonic (max = 15), Ericsson (max = 16), CMCC (max = 16), Nordic (max = 16)

- Minimum:

  - 0 (10 companies): HW, ZTE, CATT, spreadtrum, QC, Intel, Panasonic, Transsion, CMCC, IDC

    - HW: Need a minimum time gap between PEI and PO

  - >0 (7 companies): vivo (min = 4), TCL (min = 2), Samsung (min = 3), Nokia (min = 1), 
LG (min = 1), MTK (min = 3), Nordic (min = 1)

From the above, one of the possible considerations for the maximum is to align a SS burst. Since maximum SS burst period can be 160 ms, we also see max = 15/16 has the most support. Since a larger maximum also include a smaller maximum, moderator would like to suggest companies to further check whether maximum of 16 frames can be acceptable. For the minimum value, 0 frame looks of the most support, while a time gap between PEI and PO would be necessary to define (otherwise, the time for UE preparation for PO can be “tighter” than connected-mode DCP and first slot of DRX on-duration). An alternative is to set a smallest non-zero number, i.e., 1 frame, so that to waive the need for additional PEI-PO time gap. Moderator thinks either of the two alternatives can work and would like suggest companies to further check and discuss. Companies are encouraged to provide comments/suggested changes on Proposal 3-2 in the table below.

Proposal 3-2:

For the range of frame-level offset, PEI-F_offset,

- Maximum value is 16 frames

- Minimum value is :

  - Alt-1: 0 frame

    - FFS: minimum time gap between PEI and the indicated PO

  - Alt 2: 1 frame

Table 5: Companies’ comments/suggested changes to Proposal 3-2

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support. And prefer Alt-1. Considering the fact that SSB periodicity can be 5ms in minimum, so it is possible that two SSBs are contained in 10ms, that is within the same frame. In this situation, even PEI and PO are located in one frame, power saving gain still exists. So 0 frame offset is possible for PEI. |
| Spreadtrum | We support the full flexibility of PEI_F-offset, if it is up to UE whether or not to detect PEI. If UE has freedom to monitor PEI or paging DCI, the large PEI_F-offset is not so problematic. Suggest to agree it is up to UE whether or not to detect PEI at first. However, if it is the majority view, we support Alt-1 for simplicity. |
| Qualcomm | Considering the maximum 160ms, the maximum value of frame-level offset 16 is acceptable. However, we prefer to the have this large value only for largest SSB periodicity. Based on this we propose to change the maximum value proposal to - Maximum value is SSB periodicity/10ms The minimum value should be Alt-1 as Xiaomin mentioned. |
| Samsung | We suggest to clarify whether the proposal indicates to support all applicable values between the minimum and the maximum? For Alt-1, the FFS is unclear to us. Is the minimum time gap a RRC parameter or UE capability? Can gNB configure frame-level offset with value smaller than the minimum time gap if supported? |
| CMCC | Support Alt-1 in general, One clarification question about the FFS in Alt-1, we think it is the same minimum gap as in proposal 3-3. |
| LG | We are fine with the proposal and prefer Alt 2. |
| Sharp | Support FL’s proposal |
| ZTE,Sanechips | We prefer a smaller “maximum value ” considering that the largest gap between PEI and PO across companies’ evaluation results is 3SSB, which corresponds to 6 frames. As to the minimum value, as the power saving gain will shrink if the PEI is located closely with PO according to the previous evaluation. From this perspective, we think Alt2 is more reasonable. However, if it is common understanding that it depends on UE implementation to detect PEI, we are open to consider alt1 without the sub-bullet. |
| Ericsson1 | We are OK with FL proposal. Support Alt-1, but do not see the need for FFS. |
| Apple | Instead of using “frame” as unit, it may be a good idea to use “SSB periodicity/10 frames” as the basic unit, because what really matters is the number of SSBs in between. In this case, we don’t need to support large values like 16. But if majority prefer to indicate the absolute number of frames, we would be fine with 16 frames. For minimum value, we are fine with either Alt-1 or Alt-2. |
| Huawei, HiSilicon | Regarding the maximum value of 16, we share quite similar concern with Qualcomm that 16 may only applicable for large SSB periodicity. To allow 16 for 20ms SSB periodicity is not a good idea. However, we would like to support configure the PEI close to the third or second earlier SS bursts. Qualcomm’s proposal cannot support this. We would like to ask the group consider the following method: - X*SS burst period/2, where X is {0, 1,2,3,4,5,6} We are also open to consider Qualcomm’s proposal with following revision: Y*SSB periodicity/10ms, where Y is 0, 1, 2. |
| Intel | 20ms SSB periodicity is the typical periodicity and this is what UE assumes during initial access. On the other hand, having PEI occasions far ahead of the Pos have other consequences such as increase in power consumption at least for the Ues belonging to the later Pos addressed by the PEI, paging latency etc. We also do not quite agree that at low SNR, 1 SSB processing is sufficient for PEI detection. Hence, we prefer a smaller maximum value and PEI should be placed between the nearest SSB and the PO. We can compromise to QC proposal for maximum value. For minimum value, Alt-1 should be supported. |
| Nokia | We support 2-2. Keeping the frame-level offset independent of the SSB period would, at least for the shorter SSB periods, allow more flexibility when trying to place the PEI closer to SSB. |
| Transsion | For Maximum value , we share the same view as QC. For Minimum value, we think one frame is enough for SSB processing. |
| Vivo | As mentioned by companies, SSB period is a critical factor for the decision of the range of frame-level offset. For clarification, our intention for setting nonzero value (e.g., 4) as the starting value of the frame-level offset is to ensure a certain number of SSB or TRS between PEI-O and PO as shown in Figure 1. Besides, in RAN1# 106bis meeting, whether any SSB or TRS is needed between PEI-O and PO has been approved as an FFS, but not discussed sufficiently in previous meetings. Hence, we suggest to firstly discuss whether at least [one or two or three] SSB(s) or TRS(s) is needed between PEI-O and PO, followed by the discussion on the range of frame-level offset. In this way, we think we can be more convergent on the range of frame-level offset. Figure 1: An illustration for the two offsets for PEI-O location determination. |
| IDCC | Support and prefer Alt-1. |
| OPPO | Support the proposal and prefer Alt 2. |
| DOCOMO | Support FL’s proposal |
| MediaTek | We think restricting PEI-PO gap smaller than 20 ms or one SS burst period would be too restrict to REDCAP Ues where reduced antenna number requires more resource to converge sync error for paging PDSCH reception. In this regard, HW proposal that keeps the flexibility of 2 SS burst periods will be the bottom line we can accept. Regarding Alt-1 or Alt-2, we slightly prefer Alt-1 with a minimum time gap between PEI and PO. UE warm-up is a fundamental requirement to allow UE to stay in low-power state initially. For Rel-16 DCP/wake-up PDCCH, there can allow UE to report a time gap of 3 ms. For idle/inactive-mode, a larger time gap should be allowed; otherwise it is difficult to understand how UE can achieve better power saving than connected-mode. A suggested minimum gap for idle/inactive mode UE is [5] ms, and we also welcome more UE vendors’ inputs. |
| Panasonic | We are okay with maximum value is 16. On minimum value, we support 0 frame. |
| ZTE, Sanechips2 | As to the suggestion that taking SSB periodicity/2 as the basic unit of frame level offset, we think it should be noticed that the SSB periodicity is 20ms for initial access, other periodicity values (esp, larger SSB periodicity) are only applicable for non-initial access case. In the scenario for PEI deployment, the idle/inactive state UE camps on the cell for paging reception, and potential initial access. Therefore, larger SSB periodicity is not a valid case in the consideration about PEI design. So we think there is no need to define this new unit, frame level granularity is sufficient. |
| CATT | We don’t support the proposal with maximum value of 16 frames. The UE power saving gain would be diminished if the PEI-O is far away from PO since UE could not have long deep sleep time. |
| TCL | For maximum frame level offset we support the maximum value of 48 frames. Our intention of selecting 48 frames is to cover the case of SSB burst periodicity with 160m sec. By using a frame level offset range of {0…47} the gNB may have the flexibility of selecting any maximum frames offset value according to the SSB periodicity. For minimum offset value we support Alt2, |

For the range of the symbol-level offset, firstPDCCH-MonitoringOccasionOfPEI-O, there is only one company (MTK) should like to suggest restriction on maximum value to 14 symbols, while all other companies think minimum value of 0 symbol. For the maximum value of the symbol-level offset, the following statistics can be identified:

- <10 ms: MTK (2 ms)

- 10 ms (6 companies): HW, ZTE, vivo, Samsung, QC, Nokia

- Same as PO, i.e., T/N (10 companies): vivo, CATT, TCL, Spreadtrum, Intel, Panasonic, Transsion, LG, CMCC, Nordic

- Other: vivo (up to span of PEI-F_offset), Ericsson (up to 1119 symbols)

By the above, the following proposal is suggested, and companies are encouraged to provide comments/ suggested changes to Proposal 3-3 in the table below.

Proposal 3-3:

For the range of symbol-level offset, firstPDCCH-MonitoringOccasionOfPEI-O,

- Minimum value is 0 symbol

- Maximum value:

  - Alt-1: Same as ‘firstPDCCH-MonitoringOccasionOfPO’, i.e., symbol number corresponding to T/N as defined in TS 38.304

  - Alt-2: Symbol number corresponding to 10 ms, i.e., (0…10*2μ*14-1) where μ is the subcarrier spacing (SCS) configuration

- FFS, if PEI and the indicated PO can overlap in the same slot: Minimum time gap between PEI and the indicated PO

Table 6: Companies’ comments/suggested changes to Proposal 3-3

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support the proposal. and prefer Alt-2. |
| Spreadtrum | Alt-1 |
| Qualcomm | It is not clear why Alt-1 should be supported to cover the range of the duration equal to interval between two PFs as indicated by the following 38.331 IE. For example, when PF is configured once in every two radio frames, the range is 240 symbols for 15kHz SCS which is equal to 20ms. firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL, -- Need R. For PEI-PDCCH, we already have the frame-level offset. Then the symbol-level offset only needs to cover the range of a frame duration. Then for the maximum value we support Alt-2. |
| Samsung | WE prefer Alt-2, and we suggest to determine the applicable values directly. |
| CMCC | Alt 1, which gives gNB the same flexibility to configure the MO of PEI as MO of PO. |
| LGE | We are fine with the proposal, and prefer Alt 1. As discussed in a previous proposal, it is worth to note that multiple PEI-Os can be configured within a same PEI frame. To avoid overlap between PEI-O, symbol level offset larger than 10ms should be supported. |
| ZTE,Sanechips | We support alt-2. We agree with QC that different with PO determination, we already agreed a frame-level offset, therefore, it is sufficient for the symbol-level offset to cover a frame duration. For the FFS bullet, we think it can be discussed separately or if conclusion 4-2 is agreed, we think there is no need to consider it. |
| Ericsson1 | We support Alt-2 and do not see need for the FFS. |
| Apple | We think Alt-1 should be supported for max value. The reason is that the first paging MO can come long time after the start of the PF if firstPDCCH-MonitoringOccasionOfPO is configured as a large value. An example: assume 15 kHz and PF is configured once every 8 radio frames (80 ms, 1120 symbols). For PF n, assuming the corresponding firstPDCCH-MonitoringOccasionOfPO is 980 symbols, the first paging PO is in frame n+7. On the other hand, the frame-level offset for PEI is relative to the start of the PF, so it can never occur after PF n. If we limit the symbol offset to 1 frame, the latest time for the first PEI MO is in frame n+1, which is 6 frames earlier than the first paging MO. This greatly limits the configuration flexibility. Therefore, to accommodate the different configurations for firstPDCCH-MonitoringOccasionOfPO, the same range should be supported for PEI also. |
| Huawei, HiSilicon | The minimum value should be zero. We support it. We support Alt.2, and have concern on Alt.1. The symbol level value range of PEI does not need to associate with T/N。 |
| Intel | Additional flexibility beyond the possible values of symbol offset applicable for PO identification seems not strongly needed. Support Alt-1 |
| Nokia | We support proposal 3-3. For Alt1 and Alt2, we think that Alt-1 could suffice but the need of additional values could be also left to RAN2. |
| Transsion | Alt-1，which can cover T/N radio frames and cover different UE’s paging occasion. |
| vivo | We agree with the minimum value of the symbol-level offset. And for the maximum value, Alt-1 is more straightforward and flexible configuration, which we support. By adopting Alt-1, we can reuse the configuration of firstPDCCH-MonitoringOccasionofPO, so the value range of firstPDCCH-MonitoringOccasionOfPEI-O depends on both the value of SCS and the density of PEI-frame in a paging cycle. As such, Alt-1 can not only provide a larger value range of the offset (i.e., the value range of symbol-level offset is from 0 to the length of PEI-frame interval), but give more configuration flexibility as well. For Alt-2, the value range of firstPDCCH-MonitoringOccasionOfPEI-O is within the length of a PEI-frame. And we have several concerns as below: - Since the value range is limited to a frame, the restriction of the network flexibility will be caused. - Besides, in case that 1 PEI indicates 1 PO and 1 PF contains 4 POs, there will be four PEI-Os in a PEI-frame, if we still restrict all of them into one frame, it will be so crowded that some of PEI-MOs corresponding to different PEI-Os will overlap with each other as shown in Figure 2. And as we analysed in our contribution [R1-2200083], on these overlapped PEI-MOs, a UE cannot distinguish which PO(s) the detected PEI DCI is used to indicate, in other words, the UE has no idea that whether the detected PEI DCI belongs to itself or not. - Even we already have the frame-level offset, it still cannot be an excuse for limiting symbol-level offset into one frame. Taken figure 3 as an example, the drawback of Alt-2 can be clearly illustrated. Given the reasons above, we do think Alt-1 is more justifiable. Figure 2: An illustration for the overlapping of PEI-MOs corresponding to different PEI-Os. Figure 3: Comparison for Alt-1 and Alt-2. |
| IDCC | Ok with the proposal. |
| OPPO | Support the proposal and prefer Alt 2. For the FFS, in our view, the minimum time gap between PEI and the indicated PO should be at least 1 slot referring to DCI 2_6. We also think it can be discussed separately. |
| MediaTek | We support Alt 2 and think 10 ms can be sufficient. It is noticed that the symbol offsets of POs is up to network configuration, and excess symbol offset can be avoided. Then it will be sufficient for symbol-level offset to provide the granularity within a frame (complementing the frame-level offset) Regarding the FFS, we think it is necessary. UE warm-up is a fundamental requirement to allow UE to stay in low-power state initially. For Rel-16 DCP/wake-up PDCCH, there can allow UE to report a time gap of 3 ms. For idle/inactive-mode, a larger time gap should be allowed; otherwise it is difficult to understand how UE can achieve better power saving than connected-mode. A suggested minimum gap for idle/inactive mode UE is [5] ms, and we also welcome more UE vendors’ inputs. |
| Panasonic | Okay with us. |
| CATT | We support Alt 2. Alt 1 would have issue in mapping with the corresponding POs when Ns is greater than 1. We don’t see the need of FFS. |
| TCL | We are ok with the proposal and prefer alt2 for maximum value of symbol level offset. |

# Maintenance for PEI Monitoring

In Table 7, there summarize companies’ views and proposals related to maintenance for PEI monitoring:

Table 7

| Company | Companies’ views and proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: UE is not required to monitor PDCCH PEI during the Y ms prior to the start of the target PO, which is used for UE processing of PEI before the PO. Proposal 8: Make the proposed conclusion in RAN1#107-e, which was not concluded due to the time limit: For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI. - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. |
| ZTE, Sanechips | - Proposal 1: Clarification of the determination of the first PF of the PF(s) associated with one PEI is needed. - Proposal 2: The SFN of the first PF of the PFs associated with one PEI can be determined by SFNfirstPF=SFNPO-flooriPO/NS⋅(T/N), where SFNPO is the SFN of the UE’s PF, iPO is a PO index among the POs associated with the PEI, N, NS, and T are defined in TS 38.304. |
| vivo | Proposal 4: PEI DCI should carry the indicated PO group index to avoid the ambiguity e.g., adding 0-2bits PO group index indication in PEI DCI and adopt the text proposal 1 provided in Appendix 1. we suggest to discuss this restriction for UE monitoring of DCI format 2_7 after that. If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.. |
| CATT | Proposal 4: If a UE is configured with an eDRX cycle longer than 1024 radio frames, PEI Time Window is defined and related to the frame-level offset of reference frame determination. Proposal 5: ‘peiSearchSpace’ can be configured with SearchSpaceZero. |
| TCL | Observation 3: If an idle/inactive UE has decided not to monitor a PEI and monitor the UEs PO, it will unnecessarily wake up the UE for paging PDCCH monitoring even if the UE is not paged by the network. Proposal 4: If an idle/inactive UE decides not to monitor PEI, the UE has not to monitor the PO and stay in sleeping mode. |
| Samsung |  |
| DOCOMO | In the case that the size of DCI Format 2_7 is less than total number of bits of information informed via PEI, UE should monitors a paging occasion determined according to [17, TS 38.304]; |
| Spreadtrum | Proposal 2: Re-check the equation of the relative PO index agreed in RAN1#107e. If it is problematic, consider the original equation proposed in RAN1#107e, i.e., i_po = ((UE_ID mod N) mod PFnumPerPEI)*Ns + i_s, where PFnumPerPEI = ceil(POnumPerPEI/Ns). Proposal 3: If the spec should describe the equation of the first PF of the PF(s) associated with the PEI-O, the equation could be: (SFN_pei + PEI_F_offset) mod T = (T / N_pei ) * i_pei, where SFN_pei is the SFN of the first PF of the PF(s) associated with the PEI-O, PEI_F_offset is the frame-level offset of PEI, i_pei = floor(UE_ID/(N/N_pei)) mod N_pei, and N_pei is the number of PEI occasions in a paging cycle. |
| Qualcomm | It does not preclude the configuration that one PEI is mapped to two PFs belonging to two different paging cycles. Note that one paging cycle has an even number of PFs. There seems no need to consider the case that the first PF mapped to the PEI is an odd numbered PF within the paging cycle although there seems no obvious harm to have such a configuration. It may be more convenient for network implementation to limit paging operation for all UEs within each paging cycle. There can be a similar concern for UE implementation. To keep the design simple without potential implantation issues, we propose to preclude PEI mapping across paging cycle boundary. Proposal 2: If two PFs are associated with a PEI, the two PFs belong to the same paging cycle. Proposal 3: Clarify whether the first PF of PFs associated with a PEI-O is based on - Understanding 1: PDCCH monitoring occasions of peiSearchSpace on the PEI occasion have a floating start (similar to paging PDCCH). Need to explicitly define the first PF associated with the PEI, e.g., every even numbered PF in the paging cycle is a first PF if two PFs are associated with a PEI-O. - Understanding 2: Configuration of peiSearchSpace provides start of the first PEI PDCCH monitoring occasion of the PEI-O based on the offset information embedded in monitoringSlotPeriodicityAndOffset in IE SearchSpace. UE hypothetically (i.e., at most twice) derives the PEI-O location by either assuming its PF is the first PF or the PF prior to its PF is the first PF. The one that complies with peiSearchSpace configuration and the frame/symbol-level offsets is used to determine the PEI-O. Proposed conclusion: For a UE supporting R17 paging enhancement feature, it is RAN1 understanding that it is up to UE implementation whether the UE monitors the MO(s) for a PEI (No RAN1 spec impact) - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304 (No RAN1 spec impact). Proposal 5: RAN1 agrees on the following proposed conclusion that has been thoroughly discussed during RAN1 #107-e email discussions. - Proposed conclusion: - For a UE supporting R17 paging enhancement feature, it is RAN1 understanding that it is up to UE implementation whether the UE monitors the MO(s) for a PEI (No RAN1 spec impact) - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304 (No RAN1 spec impact) |
| OPPO | UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO when provided the parameter for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state. |
| Nokia | Proposal: The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. Proposal: Support also searchSpaceSetZero for PEI monitoring. |
| Intel |  |
| Panasonic |  |
| Apple | Proposal 1: All the UEs in a cell determine the mapping of PEI to POs/PFs based on a reference DRX cycle, which is signaled by the gNB in SIB. The gNB shall guarantee that the DRX cycle of any UE is not smaller than the reference DRX cycle. Proposal 2: To address the potential issue for UEs not configured with subgrouping, adopt one of the following alternatives: - Option 1: mandate that (1) all UEs supporting PEI also support subgrouping; (2) gNB is required to configure subgrouping for a capable UE if PEI is enabled. - Option 2: For a UE monitoring PEI but not configured with subgrouping, - Alt 1: Add a separate bit in the PEI field for UEs not configured with subgrouping - Alt 2: Use the last bit for a PO in the PEI field for UEs not configured with subgrouping, and introduce a separate parameter for the number of subgroups for UE to derive the subgroup ID for UE_ID-based subgrouping. Proposal 4: Support separate PO configurations for UEs supporting sub-grouping and UEs not supporting sub-grouping. |
| xiaomi | Proposal 1: SFN of the first PF of the PF(s) associated with the PEI-O should be defined. SFN of “the first PF” = SFN of UE’s PF - . Proposal 2: If PEI search space reuses paging search space, PEI DCI will not wake up UE if there is only short message, PEI DCI will only wake up UE if there is paging message or paging message plus short message. Proposal 3: If PEI search space reuses paging search space, UE can directly try to decode the paging DCI in the same monitoring occasion carrying PEI, to get the short messages. Proposal 4: If PEI search space reuses paging search space, PEI DCI size should be aligned with paging DCI size. Proposal 5: If PEI search space does not reuse paging search space, no reserved bits would be needed for PEI. PEI DCI size is only the sum of total number of bits for paging indication filed and TRS availability indication field, if configured. |
| Ericsson | - Proposal 9: No special handling is introduced for PEI when eDRX PTW is configured (if eDRX PTW is supported). The UE wakes up at configured PTW during which PEI is applicable to the POs within. |
| Lenovo, Motorola Mobility |  |
| Transsion |  |
| LG Electronics |  |
| CMCC | Proposal 4. The TP suggestion for TS 38.213 section 10.1 is as the following: <Unchanged text is omitted> If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, peiSearchSpace or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a PEI-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. <Unchanged text is omitted> |
| MediaTek | Observation 2: UE expects at most one PEI outcome from each PEI monitoring occasion. Proposal 5: The following text proposal to Section 10.1 of TS 38. 213 is adopted: ## *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. ## *** Unchanged text is omitted ***. |
| Nordic Semiconductor ASA | Proposal-2: If nrofCandidates is not configured, number of candidates for PEI CSS is given by Table 10.1-1 in TS38.213 - Note: This allows gNB to reduce SIB1 overhead by 15bits Proposed conclusion: Alt 1: It is up to UE implementation whether the UE monitors the MO(s) for a PEI - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. It has been agreed that a frame offset of PEI frame form PO frame is configured by gNB. As a consequence, a gNB saving resources and UE with unfortunate paging frame location may not achieve power-saving. Therefore, it should be up to UE implementation whether UE monitors MO(s) for PEI or monitors as in legacy, when a gNB configuration cannot achieve power saving. If Conclusion is agreed, RAN2/4 should be informed. Proposal-3: Conclude above Alt 1 and inform RAN2/RAN4 about the conclusion. |
| InterDigital |  |

For the case one PEI-O is associated with POs of 2 PFs, the following proposals of, suggested by a) Qualcomm and b) xiaomi, respectively, will be useful to simplify and clarify UE processing. In this regard, companies are encouraged to check Proposal 4-1 and provide comments/suggested changes in the table below.

Proposal 4-1:

If one PEI-O is associated with POs of 2 PFs,

- If two PFs are associated with a PEI, the two PFs belong to the same paging cycle.

- SFN of the first PF of the PF(s) associated with the PEI-O should be defined. SFN of “the first PF” = SFN of UE’s PF - .

Table 8: Companies’ comments/suggested changes to Proposal 4-1

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support the proposal. |
| Spreadtrum | We prefer the equation like PF. However, if it is the majority view, we can live with it. |
| Qualcomm | For b), what is the physical meaning of the formula? We want to avoid the erroneous case that a UE uses the PF after its own PF as the first PF of two PFs associated with the PEI-O. |
| Samsung | support |
| CMCC | Support |
| LG | We are fine with the proposal. |
| Sharp | Support a), and for b), we have a similar view as Spreadtrum that the formula of PEI SFN can be similar to that of PF SFN |
| ZTE, Sanechips | support |
| Ericsson1 | We are OK with a). For b), like Qualcomm, the meaning of the formula is unclear to us. It seems sufficient to say “the first of the two PFs associated with a PEI is an even numbered frame”. |
| Apple | Fine with the proposed mechanism in principle. But we have another proposal (P4-6) that handles different UE DRX cycles, which may have an impact on the exact formula here. |
| Huawei, HiSilicon | For bullet a), it is not clear. We think we only need to agree that the two PFs are consecutive PFs from network perspective, which is not explicitly mentioned in the last meeting. For bullet b), we don’t see a need for further agreements. The agreement in the last meeting is already clear enough. |
| Intel | Ok with bullet a). Bullet b) needs more discussion |
| Nokia | We share a similar view as Huawei that it could be sufficient to agree that the PO/PFs are ‘consecutive’. |
| Transsion | What's the meaning of ipo ? We think this equation means POs in two PFs that need to have consecutive numbers. For example, when Ns=2 and PONumPerPEI=4, the numbers of ipo are 0~3. If we do this, why don't we simply set the even PF of a paging cycle as the first PF? |
| vivo | We agree with bullet a). Bullet b) needs more discussion. |
| IDCC | Fine with the proposal. |
| OPPO | Support the proposal. |
| Panasonic | We are okay. |
| CATT | We are OK with the Proposal. |
| TCL | We are fine with this proposal. |

In the final round of email discussion, the following conclusion is close to be agreed/approved. Principally, the conclusion is to allow UE’s freedom in PEI monitoring, subject to PO performance guarantee. Companies are encouraged to check Conclusion 4-2 and provide comments/suggested changes in the table below.

Conclusion 4-2 (left-over from RAN1#107-e email discussion):

For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI.

- If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304.

Table 9: Companies’ comments/suggested changes to Conclusion 4-2

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support the proposal. |
| Spreadtrum | Support. gNB should have large flexibility to place the PEI, and UE should have freedom to ignore the PEI which is not power efficient for detection. |
| Qualcomm | This conclusion was thoroughly discussed in email discussion of the last meeting. We support to adopt it as an official conclusion in this meeting. |
| Samsung | Support |
| CMCC | Support |
| LG | We support this proposal. |
| Sharp | We support FL’s proposal |
| ZTE, Sanechips | If this conclusion is agreed, we think there is no need to further consider the minimum gap between PEI and PO/SSB. |
| Ericsson1 | We would be OK if “No spec impact” is explicitly captured. |
| Apple | Support |
| Huawei, HiSilicon | Support. |
| Intel | Support |
| Nokia | Support |
| Transsion | Support |
| Vivo | Support. |
| IDCC | Support. |
| OPPO | Support. |
| DOCOMO | Support |
| Panasonic | Support. |
| CATT | We are OK with the conclusion. |
| TCL | Support. It seems that we are the only one company with different understanding of this proposal, so we will go with the majority. |

During the discussion on feature CR to TS 38.213, spec editor suggests further discussion on whether to include PEI monitoring in the following paragraph. The paragraph is to state that UE only expects one PEI outcome from each PDCCH MO of PEI-O. Given the analogy to PO, the following proposal is suggested, and companies are encouraged to provide comments/suggested changes to Proposal 4-3 in the table below.

Proposal 4-3:

UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted.

## Section 10.1 of TS 38. 213 ## *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. ## *** Unchanged text is omitted ***

Table 10: Companies’ comments/suggested changes to Proposal 4-3

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support the proposal. |
| Spreadtrum | Fine |
| Qualcomm | We support the proposal. |
| Samsung | Support |
| CMCC | Support |
| LG | We are fine with the proposal. |
| Sharp | support |
| ZTE, Sanechips | Support |
| Ericsson1 | OK with the TP. |
| Apple | support |
| Huawei, HiSilicon | A question for clarification. Does this preclude or not the case UE can monitor two monitoring occasions, one is QCLed with SSB1 and the other one is QCLed with SSB2? Also this seems relates with the issue of proposal 3-1. We suggest to hold on. |
| Intel | Support |
| Nokia | Support |
| Transsion | Support |
| vivo | From our understanding, if there will be case that partial PEI-MOs corresponding to different PEI-Os can overlap with each other, UE could be required to detect more than one PEI DCI in the overlapped PEI-MOs. So, we suggest to defer the discussion on this TP until the decision on the symbol-level offset related proposal. |
| OPPO | Support. |
| DOCOMO | Support |
| MediaTek | We support Proposal 4-3 and would like to keep at most one PEI to be detected from each PEI PDCCH MO. |
| Panasonic | For clarification, in case that the PEI-RNTI can be configured, which is not yet agreed and under discussion, the value of PEI-RNTI could be same with P-RNTI. We are okay with the TP, given that we assume this text proposal can still cover this case. If not the case, we may further clarify on it. |
| CATT | We are OK with the proposal. |
| TCL | Support |

PEI-O definition:

In TS 38.304, PO definition include the description to skip UE symbols. In Nokia contribution [16], there suggests to include the same description to avoid such overlapping. Moderator thinks this is essential for a correct definition of PEI-O and would like to suggest companies to check the following Proposal 4-4, as quoted from Nokia contribution [16] and provide your comments/suggested revision to the proposal in the table below.

Proposal 4-4: The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF.

Table 11: Companies’ views on Proposal 4-4

| Company | Companies’ views |
| --- | --- |
| Apple | Support. We also think it was the intention to follow the same mechanism as paging MOs. |
| Qualcomm | We support this proposal. |
| Intel | Support. Minor revision seems needed? The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PF. |
| Nokia | We agree with the update proposed by Intel, maybe even further: “…sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-OPF. |
| ZTE,Sanechips | Agree with Intel/Nokia’s update |
| MediaTek | Support the proposal with the revision from Intel and Nokia |
| CATT | We agree with Intel/Nokia’s correction of Proposal 4-4. |
| Ericsson2 | Support the proposal with revisions from Intel and Nokia. |

Inclusion of pagingSearchSpace and searchSpaceSetZero:

During GTW2 on 1/17, CATT raised the remaining FFS on whether/how to include pagingSearchSpace

and searchSpaceSetZero, as the agreement in RAN1#107-e only address the search space set with SearchSpaceId > 0.

In previous discussion, the concern on reusing searchSpaceSetZero is that SSB can be ahead of associated PEI so the additional UE processing complexity (e.g., buffering PEI before SSB processing is done) will be required. On the other hand, since the FFS is not completely addressed, moderator should like to suggest companies to check the following Proposal 4-5 (as Proposal 5 in CATT contribution [9]) and provide views to the table below.

Agreement (RAN1#106-bis-e) Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero

Agreement (RAN1#107-e) ‘peiSearchSpace’ can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0

Proposal 4-5: ‘peiSearchSpace’ can be configured with SearchSpaceZero

Table 12: Companies’ views on Proposal 4-4

| Company | Companies’ views |
| --- | --- |
| Apple | We could be fine with the proposal, but we think it is a good idea to exclude some configurations that are obviously not friendly for UE power saving, as discussed before. |
| Huawei, HiSilicon | We don’t see a need to repeat a discussion made in previous meetings. |
| Qualcomm | Need to additionally clarify the peiSearchSpace has a floating start and SearchSpaceZero does not provide the starting location of the first PMO of the PEI-O. |
| Intel | Support |
| Nokia | We support. |
| ZTE, Sanechips | support |
| Transsion | Support |
| vivo | Not support, this has already been fully discussed and precluded in the previous meeting. |
| MediaTek | Not support. We already have very flexible PEI-O location framework, and it is suggested network can optimize the PEI-O location for optimizing the UE power saving gain (intention of introducing PEI) |
| CATT | The search space in addition to peiSearchSpace, such as SearchSpaceZero and pagingSearchSpace, should be supported. No company had shown the evidence of power saving degradation with SearchSpaceZero. |
| Ericsson2 | OK with the proposal. |
| Sharp | We support FL’s proposal |

Handling of different DRX cycles [19]:

In Apple contribution, there observes an issue: Current agreement doesn’t address the case UE applies different idle DRX cycles. Since UE identifies its paging bit according to its paging configuration, current mapping can cause complicated paging indication management for the network. Also, false paging probability can be increased since some PEI can associate with more POs than others. Accordingly, Proposal 4-6, as Proposal 1 in [19], is suggested, and companies are encouraged to provide views in the table below.

Proposal 4-6: All the UEs in a cell determine the mapping of PEI to POs/PFs based on a reference DRX cycle, which is signaled by the gNB in SIB. The gNB shall guarantee that the DRX cycle of any UE is not smaller than the reference DRX cycle.

Figure 1 in [19]: PEI-to-PO mapping for UEs with different DRX cycles

Table 13: Companies’ views on Proposal 4-5

| Company | Companies’ views |
| --- | --- |
| Apple | We think the current mapping (as shown in the figure above ) is not aligned with the original intention of mapping at most two PFs to one PEI, because it can result in mapping > 2 PFs to one PEI, when UEs have different DRX cycles. In addition to what is mentioned above, the current mapping also results in unnecessarily large paging delay for some UEs (e.g. UEs with 1.28 sec DRX cycle in PF4). P4-6 results in a much cleaner mapping (as shown in the figure below) and avoids all the disadvantages mentioned. |
| Huawei, HiSilicon | In my understanding, the PEI association with POs are discussed from gNB perspective. Therefore, we assume the common understanding is based on the paging configuration in SIB1. The mapping is clear already. UE may have different DRX cycles, but the mapping between PEIs and POs shall not be impacted by a DRX cycle length of a specific UE. |
| Qualcomm | Network can handle this issue without UE impact. But we are fine with the proposal. |
| ZTE,Sanechips | If the current paging cycle can be different according to TS 38.304 without impact on UE’s behavior of monitoring PO, we think there is no issue for PEI determination, either. |
| vivo | We have the same view with Huawei that PEI to PO mapping is determined from gNB perspective, and it is clear for gNB to know how two PFs can be associated with one PEI, just as described in the figure provided by Apple in the comment row. Instead, it makes no sense for UE to assume the mapping according to its own paging cycle. However, we think the point is how could a UE figure out whether its PF is the first PF for the determination of frame-level offset without awareness of the reference paging cycle for PEI to PO mapping. For this intention, a reference paging cycle (e.g., default paging cycle given in TS 38.840) may be needed. |
| MediaTek | We are supportive to this proposal since PO to PEI-O mapping in TS38.304 is based on UE perspective. Although network configuration can handle/avoid this issue, it will be beneficial to avoid such case and allow more flexible network configuration. |
| CATT | We don’t see the need of this proposal since the mapping of PEI-O and PO would be in the same DRX cycle if we restrict the frame offset value to small value “1”. |
| Ericsson2 | We do not support. PEI relates to the configured successive PFs, and since those are configured in cell-specific manner, there is no need for this. |
| Sharp | We do not support the proposal. From UE perspective, a UE monitoring a special PO can find it’s associated PEI-O based on current agreement(offset + POnumPerPEI) without caring the duration of paging cycle |

In additional to the above proposals, moderator would like to use the following table to collect companies’ inputs on missing issues and proposals related to PEI monitoring.

Table 14: Companies’ inputs on missing issues and proposals related to PEI monitoring

| Company | Companies’ views |
| --- | --- |

# Other Issues

In Table 11, companies’ additional proposals are collected:

Table 11

| Company | Companies’ views and proposals |
| --- | --- |
| Huawei, HiSilicon |  |
| ZTE, Sanechips | The transmission power of PEI needs to be determined. And the transmission power determination method of paging PDCCH can be reused. |
| vivo |  |
| CATT |  |
| TCL |  |
| Samsung |  |
| DOCOMO | In NR paging, UE determines PO (Paging Occasion) for the UE based on the calculation by e.g., UE ID, and the PO includes PDCCH monitoring occasion(s) which corresponds to each SSB. A PO can consist of a set of monitoring occasions corresponding to different beams in multi-beam operations. A large number of UEs share the same PO. These UEs wake up to monitor paging PDCCH in the same PO and have to receive corresponding PDSCH including paging message. If the paging message does not have the information for UEs, power consumption increases by unnecessary paging reception. … In the last meeting, we also agreed to support configurable DCI payload size which should be no larger than payload size of paging DCI and unused bits, when applicable, are regarded as reserved bits. In my understanding of it, there is a room to introducing Short Message in PEI. |
| Spreadtrum | For the DCI size, it was agreed that it should be no larger than paging DCI size. To reduce the DCI size of DCI format 2_7 for better PEI detection performance, we prefer the DCI size is configurable independent of the paging DCI size. Proposal 1: The DCI size of DCI format 2_7 is configurable independent of the paging DCI size. |
| Qualcomm | Proposal 6: If the working assumption for RedCap separate initial DL BWP that “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell” is agreed, network configures PEI in the separate initial DL BWP. |
| OPPO |  |
| Nokia | Proposal: Network flexibility to choose in which cells/beams paging is sent, should be maintained and applied also to PEI. Proposal: To enable/disable broadcast beam specific PEI, bit map could be used to indicate the SSBs to which the PEI is active. |
| Intel | Proposal 2: TRS availability indication field is always included in PEI DCI if SIB configures TRS resource. - Bits of TRS availability indication field starts after the paging indication field. Proposal 3: Bit size of TRS availability indication field is same in PEI DCI and paging DCI. Proposal 5: UE may follow TRS availability indication by PEI regardless of whether UE is indicated to monitor PO or not by the same PEI. |
| Panasonic |  |
| Apple | Proposal 3: It is supported that UE transmits assistance information on the preferred offset between PEI and PO, in unit of number of SSBs in between. |
| xiaomi |  |
| Ericsson | - Observation 1: Use of reserved bits in paging DCI (as a PDCCH-PEI) in one PO as paging early indication for UEs in one or more groups in other POs can further reduce PEI signalling overhead and NW power consumption. - Observation 2: Reusing a predefined RNTI for PEI detection and a fixed payload position limits the possibility to multiplex PDCCH PEI on top of a PDCCH PO. - Observation 3: Compared to a predefined RNTI, a configurable RNTI for PEI and configurable payload position does not introduce considerable complexity for the UEs. - Proposal 1: PEI design should allow the use of reserved bits in paging DCI in one PO as paging early indication for UEs in one or more groups in other POs. - Proposal 4: PEI design should allow configurable start position of the payload, bits before this starting point are treated as reserved bits by the UE. - Proposal 5: The locations (start position) of the following information elements within the PEI DCI is configurable via higher layer broadcast configuration: - Start of paging indication field (S1), 0≤S1 ≤42 - Start of TRS availability indication field (S2), 0≤S2 ≤42 |
| Lenovo, Motorola Mobility | Proposal 1: Support a DCI format 2_7 configuration in an RRC release message for an RRC inactive mode UE. |
| Transsion | Proposal 1: If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. |
| LG Electronics | Observation 1: Once the SI change indication is transmitted, repetitions of SI change indication may occur within preceding modification period. Observation 2: Conveying information with regard to SI change indication and/or ETWS/CMAS notification over PEI is beneficial from power saving perspective. Observation 3: Compared to the Alt 1-a, conveying information with regard to SI change indication and/or ETWS/CMAS notification over PEI does not increase the NW overhead. Proposal 3: Support two bits indication over PEI for SI change indication and ETWS/CMAS notification |
| CMCC |  |
| MediaTek |  |
| Nordic Semiconductor ASA | Observation-1: For Idle UEs, gNB must avoid overlapping of CORESET#0/CommonCORESET and PDSCH. For RRC connected UEs, gNB may configure UE to rate-match dynamically around entire CORESET#0/CommonCORESET (as mandatory feature). Observation-2: When gNB indicates transmitting PDCCH DMRS in entire CORESET#0/CommonCORESET, the UE may use DMRS as sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization. There is clear benefit for UE. Proposal-4: Consider introducing PDCCH DMRS transmitted in an entire CORESET#0/CommonCORESET configured by SIB1/MIB during PEI monitoring occasions to facilitate sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization. Regarding whether and how to support separate PO configuration, we believe it would be beneficial e.g. in case of PO determination would be different for R17 UEs supporting iTRS, as proposed in Proposal-3. Transmit power of PEI should be the same as for other CSS PDCCH. |
| InterDigital |  |

From the above, PEI transmission power is one fundamental performance aspect. Accordingly, the following proposal is suggested, and companies please provide comments/suggested changes to Proposal 5-1 in the table below.

Proposal 5-1:

The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP is adopted:

................................................................ Text Proposal for 38.213.................................................................................. 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI. ............................................................... Text Proposal for 38.213..................................................................................

Table 12: Companies’ comments/suggested changes to Proposal 5-1

| Company | Companies’ comments/suggested changes |
| --- | --- |
| Nordic | We support |
| Xiaomi | Support the proposal. |
| Qualcomm | We support the proposal. |
| Samsung | Support |
| CMCC | Support |
| LG | We are fine with the proposal. |
| Sharp | Support |
| Ericsson1 | OK with the TP. |
| Apple | support |
| Huawei, HiSilicon | OK with it. |
| Nokia | We are OK. |
| ZTE, Sanechips | Support. |
| vivo | OK |
| OPPO | Support. |
| DOCOMO | Support |
| Panasonic | We support. |
| CATT | OK |

Regarding sharing the resource with paging DCI, Ericsson further suggests one solution to embed PEI DCI fields to the reserved bits of paging DCI. The intention is to minimize the resource overhead when there is transmitted paging DCI for other UEs. To enable such sharing, being able to adjust the start of the paging indication field and TRS indication field is needed. To allow flexible implementation for paging early indication, companies are encouraged to check the Proposal 5-2 and provide your views in the table below.

Proposal 5-2:

The locations (start position) of the following information elements within the PEI DCI is configurable via higher layer broadcast configuration:
- Start of paging indication field (S1), 0≤S1 ≤42
- Start of TRS availability indication field (S2), 0≤S2 ≤42

Table 13: Companies’ views on whether/how to support Proposal 5-2

| Company | Companies’ views |
| --- | --- |
| Nordic | We support |
| Xiaomi | Not support the proposal. we do not see why such explicit configuration is needed. from our understanding, paging DCI is the first field, and TRS availability indication field is the second field, and its start position is based on the configuration of subgrouping number and PEI-PO mapping in SIB. |
| Qualcomm | We think the second sub-bullet is sufficient which can help decouple the sub-grouping feature and TRS feature support. It would be fine to assume paging early indication field starts from the first bit of the PEI payload. |
| Samsung | We don’t see the need for this proposal. S1 is 0 as the first field. S2 is the second field. As long as UE knows the size of the fields, UE can locate the start position. |
| CMCC | Don’t support, S1 can be before S2 and hard-coded in spec. |
| LG | We have same understanding with Xiaomi. We do not see the need for these parameters. |
| Sharp | Not support and We have similar views as Xiaomi |
| Ericsson1 | We support. Explicit configuration of these parameters allows multiplexing the PEI for one group of UEs on top of other UEs’ paging DCI transmission using the reserved bits in paging DCI as paging indicator field/TRS availability field. This allows PEI operation with less overhead and reduced NW power consumption, as discussed in our paper. Fixed location for these fields is unnecessary constraint – having flexible locations is also aligned with connected mode WUS design principle. |
| Apple | We don’t see the need for such explicit indication. |
| Huawei, HiSilicon | We don’t agree with the proposal. It was already hard coded in 38.212 CRs. There is no need for this proposal. |
| Intel | We don’t think such explicit indication is needed |
| Nokia | As discussed in last meeting, providing the starting point for a given PO’s field could have been the alternative, but in my understanding, RAN1 chose not to take that approach. Hence, don’t see absolute need for this based on last meeting agreements. |
| ZTE, Sanechips | We think the explicit configuration of the location for TRS availability indication is needed considering the number of sub-groups by UE-ID based sub-grouping and CN-assigned sub-grouping can be different, UE may have different understanding about the size of paging indication field. |
| OPPO | It is not necessary for such explicit indication. |
| MediaTek | We are not supportive to this proposal for the same reason as in Table 2 |
| CATT | We don’t see the need of this proposal |

Because of the flexibility of PEI DCI format 2_7, there are multiple companies suggesting inclusion of a 2-bit short message field in the DCI format. Since there is constant support on the short message field, moderator would like to suggest companies’ check and provision of views on whether/how to include a 2-bit short message field.

Proposal 5-3:

Support two bits indication in DCI format 2_7 for SI change indication and ETWS/CMAS notification.

Table 14: Companies’ views on whether/how to support Proposal 5-3

| Company | Companies’ views |
| --- | --- |
| Nordic | We do not support |
| Qualcomm | We do not support |
| Samsung | We do not support |
| LG | We support the proposal. It is obvious that power saving efficiency will be increased when UE receive SI change and ETWS/CMAS notification via PEI and skipping additional PO monitoring. Also required payload for these indication is 2 bits, which is small fraction of total payload size of PEI DCI. We do not see any drawback from it. |
| Ericsson1 | We do not support. |
| Apple | Support. We share the same view as LG. It is a small overhead. |
| Huawei, HiSilicon | We support the proposal. We think it can be configurable to be right after the TRS availability indication field. |
| Intel | Do not support |
| ZTE, Sanechips | We think the explicit configuration of the location for TRS availability indication is needed considering the number of sub-groups by UE-ID based sub-grouping and CN-assigned sub-grouping can be different, UE may have different understanding about the size of paging indication field. |
| vivo | Not support. |
| OPPO | Not support. |
| DOCOMO | Support. we agreed to support configurable DCI payload size which should be no larger than payload size of paging DCI and unused bits are regarded as reserved bits. In the case of there being reserved bits, there is a room to introducing Short Message in PEI. |
| MediaTek | We support the proposal and think configurable presence of this small field can be a compromise |
| CATT | We object this proposal |

According to section chair’s guidance, we should address the issues with RRC/cross-WG impact in this meeting. Moderator would like to create the following table for collecting companies’ inputs on missing topic(s) in the above sections. Your check and feedback, if any, are highly appreciated.

Table 19: Companies’ inputs on missing topic(s) with RRC/cross-WG impact

| Company | Companies’ views |
| --- | --- |
| Apple | As captured in Section 5, we have the following proposal: “It is supported that UE transmits assistance information on the preferred offset between PEI and PO, in unit of number of SSBs in between.” We think this is useful assistance information for the network to have to determine a good value for the configured offset for PEI, which in turn is also good for UE power saving. Companies had expressed different views on how this offset should be set, but in the end, it is very dependent on UE implementation. Most likely there wil not be a single optimal value for all UEs, but having some UE info is still much better than doing blind decision at the gNB. |
| Huawei, HiSilicon | Apple’s comments may also relate with the following proposal made by Nokia. If the group is happy to optimize the PEI configuration for better power saving, we are open to further discuss the two issues together. Proposal: RAN1 discuss configuration of more than one PEI per PO to accommodate different device preferences. |
| vivo | For the symbol-level offset, another concern should be discussed as described below. There could be the case that some PEI-MOs respectively corresponding to different PEI-Os in a PEI-frame overlap with each other. Of course, there will be the same case for PO, however, the causing result for PO is only to increase the false alarm rate while the worse result for PEI is to increase the miss detection rate i.e., UE detect a wrong PEI DCI and follow the false indication e.g., UE may not be woken up to receive PO. For the sake of both configuring PEI-Os to be as close to SSB as possible and reducing PEI resource overhead, the network can configure multiple PEI-Os to be overlapped with each other i.e., multiple PEI-Os have to share partial PEI-MOs. On these overlapped PEI-MOs, a UE cannot distinguish which PO(s) the detected PEI DCI is used to indicate, in other words, the UE has no idea that whether the detected PEI DCI belongs to itself or not. To address this issue, the direct way is to limit the length difference of any two symbol-level offsets which should be larger than the duration length of PEI-O. But, it is a strong limitation for configuring the symbol-level offset and a departure from the principle to configure PEI-O to be as close to SSB as possible. Hence, we give following proposal. Proposal: PEI DCI should carry the indicated PO group index to avoid the ambiguity e.g., adding 0-2bits PO group index indication in PEI DCI. |

# Summary

Suggested Proposals for GTW2 on 1/19:

Section 10.4A of TS 38.213 … A UE can be provided the following for detection of a DCI format 2_7 in RRC_IDLE state or in RRC_INACTIVE state [12, TS 38.331] - … - a number of frames, by PEI-F_offset, from the start of a first paging frame of paging frames associated with a number of PDCCH monitoring occasions for DCI format 2_7 [17, TS 38.304] to the start of a frame - a number of symbols, by firstPDCCH-MonitoringOccasionOfPEI-O, from the start of the frame to the start of the first PDCCH monitoring occasion for DCI format 2_7 - …

Proposal 3-1:

For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives

- Alt-1: It is not supported, and POnumPerPEI can only be one or multiple of Ns

- Alt-2a: It is supported, and UE applies a single value of PEI-F_offset for frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for symbol-level offset

- Alt-2b: It is supported, and UE applies the (flooris/POnumPerPEI+1)-th value out of Ns/POnumPerPEI configured values in PEI-F_offset (firstPDCCH-MonitoringOccasionOfPEI-O) for frame-level (symbol-level) offset

Proposal 3-2:

For PEI-O location determination:

- Range of PEI-F_offset, in unit of frame, is 0, 1,2, …, 16

- Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, follows the same design as firstPDCCH-MonitoringOccasionOfPO

- Note: When PEI-O is placed close to or overlapped with the slot of the associated PO, UE is required to be ready for paging PDSCH demodulation in a short or little time duration after detecting the paging indication from PEI, which can cause UE complexity and potentially higher UE power consumption.

Proposal 2-2:

PEI-RNTI is of fixed value, and value design is up to RAN2.

Conclusion 4-2:

For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI.

- If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304.

Proposal 4-4:

The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging PEI in the PF PEI-O.

Proposal 5-1:

The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP is adopted:

.............................................................. Text Proposal for 38.213.............................................................................. 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI. ............................................................... Text Proposal for 38.213..................................................................................

Proposal 4-3:

UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted.

Section 10.1 of TS 38. 213 *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. *** Unchanged text is omitted ***

# Reference

- R1-2112943, “38.212 CR - Introduction of Rel-17 UE power saving enhancements”, RAN1#107-e

- R1-2112936, “38.213 CR - Introduction of Rel-17 UE power saving enhancements for NR”, RAN1#107-e

- R1-2112957, “38.214 CR - Introduction of Rel-17 UE power saving enhancements”, RAN1#107-e

- R1-2112977, “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN1, RAN1#107-e

- R1-2112886, “Summary #3 of Paging Enhancements”, Moderator (MediaTek), RAN1#107-e

- R1-2200032, “Remaining issues on Paging enhancements for UE power saving in IDLE/inactive mode”, Huawei, HiSilicon

- R1-2200066, “Remaining issues of power saving enhancements for paging”, ZTE, Sanechips

- R1-2200083, “Remaining issues on paging enhancements for idle/inactive mode UE power saving”, vivo

- R1-2200149, “Paging enhancement for UE power saving”, CATT

- R1-2200155, “Remaining issues on paging enhancements for idle/inactive mode UE power saving”, TCL Communication Ltd.

- R1-2200201, “Maintenance on paging enhancements”, Samsung

- R1-2200234, “Discussion on paging enhancement”, NTT DOCOMO, INC.

- R1-2200276, “Discussion on potential paging enhancements for UE power saving”, Spreadtrum Communications

- R1-2200298, “Paging enhancements for idle and inactive UE power saving”, Qualcomm Incorporated

- R1-2200331, “Further discussion on Paging enhancements for power saving”, OPPO

- R1-2200362, “Open items on paging enhancements for UE power saving”, Nokia, Nokia Shanghai Bell

- R1-2200376, “Discussion on the remaining aspects of paging enhancements”, Intel Corporation

- R1-2200397, “On paging enhancement”, Panasonic

- R1-2200417, “Remaining issues on paging enhancements for idle/inactive UEs”, Apple

- R1-2200463, “Remaining issues on paging enhancement for power saving”, xiaomi

- R1-2200478, “Remaining aspects of Paging Enhancements”, Ericsson

- R1-2200532, “Paging enhancement for UE power saving”, Lenovo, Motorola Mobility

- R1-2200560, “Discussion on PEI Design”, Transsion Holdings

- R1-2200575, “Discussion on potential paging enhancements”, LG Electronics

- R1-2200585, “Remaining issues on paging early indication”, CMCC

- R1-2200606, “Maintenance on Design of Paging Early Indication”, MediaTek Inc.

- R1-2200609, “On paging early indication”, Nordic Semiconductor ASA

- R1-2200618, “Remaining issues on paging enhancements”, InterDigital, Inc.