3GPP TSG RAN WG1 Meeting #107bis-e	  R1-2200756

E-meeting, January 17th – 25th, 2022

Agenda Item:	8.3.4

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on propagation delay compensation enhancements

Document for:	Discussion and Decision

Introduction

The revised IIoT / URLLC work item description for Rel-17 [1] has enhancements for time synchronization as one of its main objectives:

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

This document summarizes the key issues discussed under agenda item 8.3.4 based on the views in [2][3][4][5][6][7][8][9][10], and aims to discuss a set of issues in RAN1#107bis-e. The agreements in past meetings are captured in the Appendix.

Remaining open issues for PDC

There are a few open issues raised in the contributions submitted in RAN1#107b-e.

Issue #2-1: Whether or not SRS for Positioning should be supported for RTT-based PDC?

- Option 1: SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC.

  - Huawei/HiSilicon, ZTE, OPPO, Nokia/NSB (1st),

  - Pros

    - Low specification impact

    - Not necessary from performance perspective

  - Cons

    - May result in a waste of SRS air interface resources for a UE supporting both PDC and a Positioning service

- Option 2: SRS for positioning is supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC, i.e. SRS for positioning can be used along with either TRS or PRS.

  - Intel, [LG]

  - Pros

    - Avoid unnecessary waste of SRS resource

    - More flexibility

      - Note that SRS for positioning can be used independent of PRS in Rel-16/17 positioning

  - Cons

    - High specification impact

- Option 3: SRS for positioning is supported for RTT-based PDC and only along with PRS.

  - Nokia/NSB (2nd), Intel (fine),

  - Pros

    - May avoid unnecessary waste of SRS resource when both PRS and SRS for positioning are used for positioning purpose already

  - Cons

    - Medium specification impact

Feature lead: More views are needed before making any decision here.

First round discussion

The following questions are set for the first round email discussions.

Question 2-1-1: Which option among the above three options do you prefer? Please explain your reasons for your choices also.

| Company | View |
| --- | --- |
| Nokia, NSB | Option 1 (first preference) or Option 3 (if SRS for PDC is supported). Option 1 should be sufficient, but if SRS for positioning is supported, then Option 3 gives the best compromise as it avoids the waste of resource in case a UE has been configured with SRS for positioning and is simple to specify by just adding a PDC usage indicator in the SRS Resource Set configuration. |
| Intel | Both Option 2 and Option 3 are fine. Option 1 looks restrictive for the cases when SRS for positioning is supported by the UE. Furthermore, RAN4 already has requirements and tests for SRS for positioning, so that Option 2/3 looks easily supportable. |
| OPPO | We support Option 1. We think the important question here is whether PRS (for PDC purpose) can be taken as pathloss reference signal (for power control purpose) of at least one UL signal/channel (including SRS for PDC purpose). Apparently RAN1 does not have such agreement in place. Then one of most outstanding reasons to introduce positioning SRS in positioning scope does not apply here for PDC. All the “Pros” listed in Option 2 for positioning SRS can all be satisfied by normal SRS. For “Pros” listed in Option 3, we think the normal SRS can be more likely a “must-have” signal used by a UE, even when the UE is running positioning, i.e., it is unlikely for a UE to transmit only positioning SRS but no normal SRS when the UE is in RRC-connected state. For the same reason, we disagree with the “cons” in Option 1. We would like to highlight that Option 1 does not necessarily mean the existing normal SRS configuration should be changed to include PRS as a supported pathloss reference – the SSB can be maintained as pathloss reference signal. |
| New H3C | For Option 1, we want to clarify whether SRS for positioning means special SRS for positioning. If yes, we support option 1 because special SRS for positioning isn’t necessary. [Feature lead] Yes it means the SRS designed specific for positioning in positioning work items. |
| vivo | Option 1. For normal SRS and positioning SRS, configurations still may be different, such as time-frequency density of REs. A UE can be configured with both the normal SRS and positioning SRS for different purpose. We think the “cons” for Option 1 is not valid. On the other hand, in the maintenance phase it is unnecessary to consider positioning SRS. |
| HW/HiSi | Option 1. In our view the mandatory legacy SRS can be used and there is no need to support the positioning SRS for RTT-based PDC. Since the legacy SRS is mandatory anyway, we do not see why SRS air interface resources would be wasted when adopting this solution. And the accuracy of the MIMO SRS can be the same as positioning SRS. |
| Spreadtrum | Option 1 We agree position SRS has similar performance as SRS we have agreed. |
| LG | We prefer Option 3 (we removed our company name from option 2). If UE is configured with PRS and SRS for positioning, Option 3 can re-use the positioning framework without any redundancy. Option 1 would be sufficient to support RTT-based PDC so we are open to discuss. |
| ZTE | Option 1 We think SRS is sufficient for RTT-based PDC from the perspective of PDC performance. In addition, NR SRS has flexible configuration. There is no need to introduce another specific SRS. |

Summary of first round email discussion for question 2-1-1 above:

- Option 1: SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC.

  - Huawei/HiSilicon, ZTE, OPPO, Nokia/NSB (1st), vivo, Spreadtrum

  - Pros

    - Low specification impact

    - Not necessary from performance perspective

  - Cons

    - May result in a waste of SRS air interface resources for a UE supporting both PDC and a Positioning service

- Option 2: SRS for positioning is supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC, i.e. SRS for positioning can be used along with either TRS or PRS.

  - Intel

  - Pros

    - Avoid unnecessary waste of SRS resource

    - More flexibility

      - Note that SRS for positioning can be used independent of PRS in Rel-16/17 positioning

  - Cons

    - High specification impact

- Option 3: SRS for positioning is supported for RTT-based PDC and only along with PRS.

  - Nokia/NSB (2nd), Intel (fine), LG

  - Pros

    - May avoid unnecessary waste of SRS resource when both PRS and SRS for positioning are used for positioning purpose already

  - Cons

    - Medium specification impact

Feature lead: Based on the summary above, it is recommended to go with the majority view, i.e. option 1, especially at this late stage to save some workload for different working groups considering the additional benefits is not that significant.

Question 2-1-2: If option 2 or option 3 is supported, is it also necessary to use PRS along with SRS for MIMO for RTT-based PDC also?

| Company | View |
| --- | --- |
| Feature lead | Note that if option 1 is agreed, then it is clear that PRS should be used along with SRS for MIMO. However, if option 2 or option 3 is agreed, then it is not really necessary for PRS to work along with SRS for MIMO, e.g. PRS can always along with SRS for positioning, while TRS can always work along with SRS for MIMO. Just to further clarify that the intention for this question is to see whether/what impact on UE feature definition and RRC parameter design. For example, if PRS along with SRS for MIMO is also supported, then for UE feature we may need on FG for PRS with SRS for positioning and another FG for PRS with SRS for MIMO, while if PRS along with SRS for MIMO is not supported, then we only need one FG to reflect PRS with SRS for positioning. On RRC parameter, if PRS along with SRS for MIMO is also supported, we may also need to provide the flexibility to take PRS as the pathloss for SRS for MIMO, which will have impact on RRC parameters for MIMO. Of course, at least OPPO already expressed that SRS for MIMO can always just takes SSB as the pathloss reference signal, but some other companies may want to go the other way based on the contributions. |
| Nokia, NSB | We don’t see that is necessary to use PRS along with SRS for MIMO for RTT-based PDC. As FL says, TRS can be used along SRS for MIMO. |
| Intel | It is not necessary |
| OPPO | Per functionality wise, it is not necessary to use PRS along with SRS for MIMO for PDC, but this should not lead to the necessity to bind positioning SRS “only with PRS”, because it is an UE implementation restriction that UE has to support positioning PRS (which is new to 38.331 in Rel-17) if UE plans to just support positioning SRS (which is already in 38.331 since Rel-16) for PDC. This is why we do not prefer to Option-3. |
| New H3C | It isn’t necessary. We needn’t limit PRS along with SRS for MIMO for RTT-based PDC. |
| Vivo | It is not necessary. It seems more related to implementation. |
| HW/HiSi | Generally it is not necessary for option 2/3. But for a UE already supporting Rel-16 DL-TDOA positioning but not supporting multi-RTT positioning, it only supports PRS but not the positioning SRS, it is then still beneficial to use PRS with MIMO SRS to simplify the UE’s implementation for RTT-based PDC. |
| LG | We think it is not necessary. |
| ZTE | It should be up to implementation. The restriction is not necessary. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2-1-1: SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC.

| Company | View |
| --- | --- |
| Feature lead | @ Intel @ LG Please check the discussion in the first round, and hopefully the proposal can be acceptable for you. |
| Ericsson | Support |
| Samsung | Support |
| New H3C | Support |
| vivo | Support |
| OPPO | Agree. |
| Nokia, NSB | Support |
| ZTE | Support |
| Spreadtrum | Support |
| HW/HiSi | Support |
| Intel | We can accept majority view for the progress |

Summary of the status for proposal 2-1-1 in 2nd round

| Support | Ericsson, Nokia, NSB, vivo, Samsung, ZTE, Huawei, HiSilicon, New H3C, OPPO, Spreadtrum, Intel |
| --- | --- |
| Not support |  |

FL recommendation: The proposal is stable.

Issue #2-2: Whether measurement gap should be mandatory for a UE to process PRS for PDC purpose?

Nokia R1-2200019 Additionally, it will need to be discussed if same processing constraints as present for PRS for Positioning also applies for PRS for PDC purposes. For example 38.214 states that: “UE is not expected to process DL PRS without configuration of measurement gap”, which we think is an undesired property considering the IIoT use case and that the PRS transmission only comes from the serving cell. Proposal 4: Measurement gaps should not be mandatory for a UE to process PRS for PDC purposes.

Feature lead: According to TS38.214 g80 copied below, UE is not expected to process DL PRS without configuration of measurement gap in Rel-16 positioning. However in Rel-17 positioning, this sentence is removed accordingly in the TS38.214 h00 copied below, and the CR is in [12] for your information. It seems the corresponding behaviors for Rel-16 PRS reception and Rel-17 PRS reception are a little bit different. Regardless of the reasons for positioning, for PDC it seems the reason given by Nokia is reasonable, considering that the PRS transmission only comes from the serving cell for IIoT use case. However, let’s hear more views from companies.

Copied from TS38.214 g80 If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with QCL set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE. UE is not expected to process DL PRS without configuration of measurement gap. Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority otherwise; - Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the frequency layer are sorted according to priority otherwise.

Copied from TS38.214 h00 5.1.6.5 PRS reception procedure …… If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same dl-PRS-ID. If DL-PRS-QCL-Info is configured to the UE with QCL set to 'type-D' with a source DL PRS resource then the nr-DL-PRS-ResourceSetId and the nr-DL-PRS-ResourceId of the source DL PRS resource are expected to be indicated to the UE. The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter [PRSProcessingWindow]. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, if the UE determines the DL PRS priority is higher than [other DL signals or channels except SSB] as indicated by higher layer parameter [PRS-priority-indicator] or as implied by UE capability, the UE is expected to measure the DL PRS; otherwise, the UE is not expected to measure the DL PRS and expected to receive [other DL signals and channels], subject to UE capabilities. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 1A] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 1B] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 2] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE on the overlapped symbols with the DL PRS. Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority otherwise; - Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the frequency layer are sorted according to priority otherwise. …… 9 UE procedures for transmitting and receiving for RTT-based propagation delay compensation For operation with RTT-based propagation delay compensation, the UE may be configured with either: - one CSI-RS for tracking with higher layer parameter pdc-Info for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-r17, or - one PRS configuration of higher layer parameter NR-DL-PRS-PDC-ResourceSet-r17 [12, TS 38.331] for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-r17. The related UE procedures for transmitting uplink reference signals and receiving downlink reference signals for RTT-based propagation delay compensation are defined as follows: - for reception of CSI-RS for tracking with higher layer parameter pdc-Info, the UE follows the procedures for reception of CSI-RS for tracking defined in Clause 5.1.6.1.1. - for reception of the one PRS configuration provided by RRC [12, TS 38.331] for RTT-based propagation delay compensation, the UE follows the procedure for PRS reception defined in Clause 5.1.6.5 using the configuration information provided by NR-DL-PRS-PDC-ResourceSet-r17 instead of NR-DL-PRS-ResourceSet. - for transmission of an SRS resource set configured with usage-r17, the UE follows the procedures for SRS transmission defined in Clause 6.2.1.

First round discussion

The following question are set for the first round email discussions.

Question 2-2-1: Do you agree that measurement gaps should not be mandatory for a UE to process PRS for PDC purposes? Please explain your reasons also.

| Company | View |
| --- | --- |
| Nokia, NSB | We agree. As per explanation from our tdoc highlighted by the FL above. |
| Intel | Agree. Measurement gaps are necessary for measurements on other cells/carriers, while for PDC purpose only the serving cell/carrier is needed. |
| OPPO | One of the concerned spec texts raised by Nokia’s paper is already removed from Rel-17. We wonder what the proposal means in terms of spec impact. Does it mean the PDC UE should not be configured with or follow “NR-PRS-MeasurementInfoList” for PDC-PRS? Or the “measurement gap” in this proposal not only refers to the gap configured by “NR-PRS-MeasurementInfoList” but also refers to the general measurement gap defined in 38.321? The question is not clear to us. [Feature lead]: In Rel-16, the restriction“UE is not expected to process DL PRS without configuration of measurement gap”is given in the spec, in Rel-17 this sentence is removed together with adding additional specification impact on defining behavior in case of collision between receiving PRS and data. For PDC PRS, if we all agree that gap is optional, then we can further discuss whether/any further specification needed if gap is not configured. By the way, I think MeasGapConfig is the one to configure the gap, not NR-PRS-MeasurementInfoList. |
| New H3C | We agree with this proposal. |
| vivo | We are fine with the proposal considering PRS transmission only from the serving cell. |
| HW/HiSi | Agree that PRS is only needed from the serving cell in case of PDC. But we would like to know if measurement gaps are not configured, does the PDC UE still follow the below copied Rel-17 specification for PRS reception for PDC? For example, if the PDC UE determines the DL PRS priority is higher than [other DL signals or channels except SSB], the PDC UE is expected to measure the DL PRS; otherwise, the PDC UE is not expected to measure the DL PRS and expected to receive [other DL signals and channels]. Then does it mean that we also introduce PRSProcessingWindow, PRS-priority-indicator configuration and capability 1A/1B/2 for PDC UE? Or are we missing something here? “The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter [PRSProcessingWindow]. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, if the UE determines the DL PRS priority is higher than [other DL signals or channels except SSB] as indicated by higher layer parameter [PRS-priority-indicator] or as implied by UE capability, the UE is expected to measure the DL PRS; otherwise, the UE is not expected to measure the DL PRS and expected to receive [other DL signals and channels], subject to UE capabilities. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 1A] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 1B] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap if it is supporting [capability 2] and if the DL PRS is determined to be higher priority than the DL signals and channels inside the PRS processing window, those DL signals and channels are not expected to be measured by the UE on the overlapped symbols with the DL PRS.” |
| LG | Agree. We also think it would be good to specify that in RTT-based PDC capability so we can re-use following text for PDC as well. The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter [PRSProcessingWindow]. |
| ZTE | Since the UE only needs to measure the PRS from serving cell, the measurement gap is not mandatory. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion 2-2-1: Measurement gaps should not be mandatory for a UE to process PRS for PDC purposes.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO Based on the discussion in the first round, it seems the majority company support the conclusion here, while OPPO raised some question. Please check my reply and see if the proposed conclusion here can be acceptable for you. |
| Ericsson | Support the conclusion |
| Samsung | Support |
| New H3C | Support |
| vivo | Support |
| OPPO | We are fine with the proposal. |
| Nokia, NSB | Support |
| ZTE | Support |
| Spreadtrum | Support |
| HW/HiSi | Support |
| Intel | Support |

Summary of the status for proposed conclusion 2-2-1 in 2nd round

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Spreadtrum, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

FL recommendation: The proposed conclusion is stable.

Question 2-2-1: If the above conclusion 2-2-1 is supported, do you see any additional specification impact needed to support this conclusion? For example, UE will just follow the Rel-17 PRS receiving behavior defined in the current specification, or UE is expected to always measure the PRS without considering any other channel/signal?

| Company | View |
| --- | --- |
| Ericsson | There may be additional spec impact if the conclusion is reached. One issue is, how to handle collision of PRS with other DL signals, e.g., DCI, DG PDSCH, SPS PDSCH. Other than RAN1 PRS reception spec, we think there can be additional specification impact, for example, event triggered measurement report. On the other hand, such discussion is up to RAN2 and no need to discuss in RAN1. |
| Samsung | Some spec changes might be needed. We’d like to suggest to describe PRS for PDC together with CSI-RS for tracking (used for PDC). We think only PRS pattern is reused, but not intend to re-use any procedure as for positioning. |
| Vivo | Some additional spec impact may be needed, such as collision handling between PRS and other DL signals/channels measurement. As cited in the first round, three UE capabilities are defined for positioning for different measurement behavior. Defining too many UE capabilities should be avoided for PDC. A rule with minimum spec impact is suggested. |
| OPPO | Because the PRS for PDC and PRS for positioning could be physically the same signal, we suggest to reuse the same handling rule from positioning side at least for “collision with other DL signals” so that no new spec impact is expected at least for collision handling. |
| Nokia, NSB | No. We do not see any additional spec impact. We can reuse the PRS reception procedure from R17. |
| ZTE | Regarding the additional specification impact, we don’t see anything that needs to change. For the collision handing with other DL signal, we wonder why the collision occur in the case of all the DL signals only from the serving cell. This is the same with CSI-RS for tracking. Is there any collision between CSI-RS for tracking and the other DL signal? In addition, the measurement for PDC should not be frequent. Everything related to DL reception can be up to UE implementation. |
| HW/HiSi | We think that there is some more specification impact and at least a collision between PRS and other DL signals needs to be resolved. |
| Intel | Without measurement gaps, a UE needs to know how/whether to handle overlap with other DL channels. Even if gNB would need to ensure no overlap, this should be captured in the spec. |

Feature lead: Looking at the inputs above, it seems the views are still diverse. I am worried that people may have no sufficient time to check the specs and think thoroughly during this busy RAN1 meeting, thus it is recommended to further discuss issue in next meeting, i.e. whether/what additional specification changes needed. For the conclusion itself, it is common understanding and stable, thus we can approve it first.

Whether to clarify the TA value for PDC in case multiple TAGs exist?

Ericsson (R1-2200443) For the legacy TA-based PDC, one issue is to select the timing advance command of a specific timing advance group (TAG) for PDC purpose, if multiple TAGs exist. First, regardless of the PDC method, the propagation delay should be measured between the UE and the TRP associated with referenceTimeInfo. Thus, similar to the conclusion below, with the legacy TA-based method, the timing advance value should also be associated with the PCell. Conclusion (RAN1#107e) For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP. Note: No RAN1 specification impact is expected for this conclusion. Thus, it should be clarified in the specification that the TA value for PDC is the timing advance associated with the timing advance group (TAG) of the PCell, i.e., the Primary Timing Advance Group (PTAG). When in dual-connectivity operation, both master cell group (MCG) and secondary cell group (SCG) have PTAG. In this case, it needs to be clarified that the TA value is associated with PTAG of MCG. Proposal 1: Specify that the TA value for PDC is the timing advance value associated with the PTAG of MCG.

Feature lead: According to the description about the referenceSFN IE below, the SFN of PCell is used as the reference for the reference time information. According to TAG definition in TS38.321 copied below, the PCell belongs to PTAG. Therefore, it looks to me that the current spec is already clear, and there is no need to do further clarifications. However, let’s hear more views from other companies.

Copied from 38.331 #### – ReferenceTimeInfo The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping, see TS 23.501 [32], clause 5.27.1.2. ReferenceTimeInfo information element -- ASN1START -- TAG-REFERENCETIMEINFO-START ReferenceTimeInfo-r16 ::= SEQUENCE { time-r16 ReferenceTime-r16, uncertainty-r16 INTEGER (0..32767) OPTIONAL, -- Need S timeInfoType-r16 ENUMERATED {localClock} OPTIONAL, -- Need S referenceSFN-r16 INTEGER (0..1023) OPTIONAL -- Cond RefTime } ReferenceTime-r16 ::= SEQUENCE { refDays-r16 INTEGER (0..72999), refSeconds-r16 INTEGER (0..86399), refMilliSeconds-r16 INTEGER (0..999), refTenNanoSeconds-r16 INTEGER (0..99999) } -- TAG-REFERENCETIMEINFO-STOP -- ASN1STOP ReferenceTimeInfo field descriptions. referenceSFN This field indicates the reference SFN corresponding to the reference time information. If referenceTimeInfo field is received in DLInformationTransfer message, this field indicates the SFN of PCell..

Copied from 38.321 Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

First round discussion

Question 2-3-1: Do we need to further clarify in RAN1 spec that the TA value for PDC is the timing advance value associated with the PTAG of MCG?

| Company | View |
| --- | --- |
| Nokia, NSB | No need for clarification - we agree with FL. We think that a decent UE implementation should obviously use the TA of the TAG to which the PCell is associated, which is PTAG, so we don’t see a strong need for such clarification. |
| Intel | We don’t have strong preference. The issue is valid, at the same time, as pointed out by Nokia the other interpretations are unreasonable, thus may be handled by proper UE implementation. |
| OPPO | Agree with FL that the current spec is already clear and there is no need to further clarify the TA value for PDC. |
| New H3C | We can’t see any motivation to clarify this. |
| vivo | Agree with FL that the current spec is already clear and there is no need to do further clarification. |
| HW/HiSi | No. Agree with the FL explanation. |
| Spreadtrum | Agree with FL’s assessment. |
| LG | Agree with the explanation from FL. |
| ZTE | We agree with the FL’s view. The spec is clear. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion 2-3-1: No need to further clarify in RAN1 spec that the TA value for PDC is the timing advance value associated with the PTAG of MCG

Revised Proposed conclusion 2-3-1: The TA value for PDC is the timing advance value associated with the PTAG of MCG.

- Note: No RAN1 specification impact is expected for this conclusion

| Company | View |
| --- | --- |
| Feature lead | @ Ericsson Based on the discussion in the first round, almost all companies support the conclusion here. Please check the discussions and see if the proposed conclusion here can be acceptable for you. Note that I revised the proposed conclusion as above to address the comment from Ericsson. |
| Ericsson | We do not agree with proposed conclusion. The discussion above merely show that our proposal is quite reasonable and nobody objects to this understanding. Currently there is nothing linking referenceTimeInfo to TA-based PDC. But it appears that FL linked referenceTimeInfo and PCell to TA value for PDC as if this is already agreed. Recall that the existing conclusion of RAN1#107e was made for RTT-based PDC. Conclusion For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP. Considering there can be multiple TA values on a given Un interface, it is necessary to specify that the TA value for PDC is the timing advance value associated with the PTAG of MCG. [Feature lead]: As explained in section 2.3, according to RAN2 spec TS 38.331, the reference SFN corresponding to the reference time information is the SFN of the PCell, then it should be very straightforward that the TA value for PCell will be used for TA-based PDC, which also means the TA value associated with the PTAG of MCG. Note that in Rel-16, UE can use TA-based PDC to do the compensation also and we don’t have any further clarification specifically for this in the spec I think. As you mentioned above, it is common understanding and nobody has different view. I think the key question/point is whether any additional RAN1 specification impact needed, it seems most companies think that there is no need to do further clarification in the spec. However, maybe we can make a conclusion similar as that for RTT-based PDC, which can make it clearer to all even with no specification impact. I revised the proposal accordingly. |
| Samsung | Support the proposal. |
| Feature lead | Please all check my replies to Ericsson above, and then provide your views on the revised proposed conclusion. |
| New H3C | Support the updated proposal |
| vivo | Support the revised conclusion. |
| OPPO | We prefer to following conclusion to address Ericsson’s concern and to better match what RAN1 has for RTT-based PDC: Potential conclusion: For TA-based PDC, it is assumed that the reference time information is associated with the TRP that is used by MCG. With this conclusion, it is up to UE’s implementation to choose the right TA to estimate the propagation delay, given there has to be a TAG associating with MCG. [Feature lead]: As copied above in section 2.3, TS 38.331 already specified that the SFN of PCell is used as the reference for the reference time information, which reflects the association in a more concrete way already, thus I think we don’t need to make a conclusion as you suggested above. The issue raised by Ericsson is to clarify which TA value to be used, it should be very straightforward to just follow the TA used on PCell, especially considering the potential association of the error components which we had discussed much before for TA-based PDC. The above potential conclusion targets a gNB behavior. In contrast, FL’s revised conclusion seems to talk about a UE behavior (on how to pick TA for one-way propagation delay estimation), which is not reflected in current spec -- so it is a bit self-contradicting by saying the conclusion has no RAN1 spec impact. [Feature lead]: We need to cover the UE behavior also, when UE does the compensation, it needs to pick a TA command to use. The point is that using the TA associated with the PTAG of MCG is very straightforward and should be common understanding, thus no need to really specify this in the spec, however we can have a conclusion to make it clearer or reflect the common understanding. Let me update the conclusion a little bit to avoid misunderstanding. |
| Samsung | Fine with the proposed proposal. |
| Nokia, NSB | We support the revised conclusion. |
| ZTE | Support the revised conclusion. |
| Spreadtrum | Support Revised Proposed conclusion 2-3-1. |
| Hw/HiSi | We think it is already clear in the specification and that there is no further spec impact needed. We are fine with the revised conclusion. |
| Intel | Support the revised conclusion |

Feature lead: Most companies agree with the conclusion, while OPPO raised comments, not sure if OPPO is fine with my replies above. To make this conclusion clearer to avoid any confusion, I updated the conclusion a little bit. Companies are encouraged to check.

3rd round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposed conclusion 2-3-1 (for GTW): For PDC method based on legacy TA-based mechanism, it is assumed that the TA value for PDC is the timing advance value associated with the PTAG of MCG.

- Note: No RAN1 specification impact is expected for this conclusion

Revised Proposed conclusion 2-3-1: For PDC method based on legacy TA-based mechanism, it is assumed that the TA value for PDC is the timing advance value associated with the PTAG of MCG.

- Note: No RAN1 specification impact is expected for this conclusion

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ Ericsson Please check the discussions for second round and see if the proposed conclusion here can be acceptable for you. Note that according the following agreement we made before, no RAN1 specification is expected. Agreement For Rel-17 - Support RTT-based PDC method - Support PDC method based on legacy TA-based mechanism - No RAN1/RAN4 specification impact expected |
| OPPO | We can accept the proposal. |
| New H3C | We are fine with this proposal. |
| Nokia, NSB | We support the proposal. |
| ZTE | We support this proposal. |
| vivo | We support this proposal. |
| Ericsson | We don’t see why “it is assumed”? Based on companies’ input, it’s common understanding that the compensation does not make sense if using TA of a different TAG. In our view, it’s necessary to spell out this clearly, rather than leave it to vague language. TA-based PDC accuracy depends on parameters of the reference cell of PTAG, including SCS of SSB, SCS of uplink signals (see 38.133 tables). Accuracy achievable by TA based method is important to know. For example, if the TA-based accuracy is sufficient, then gNB-UE can choose TA-based method. On the other hand, if TA-based accuracy is inadequate when compared to the target accuracy, then the gNB has to configure RTT-based method instead. Regarding the Note, we suggest to remove if it’s kept as a Conclusion. If the note is kept, then we suggest to make the proposal an agreement. [Feature lead] I think even with “it is assumed” it will not change the common understanding, it is kept here just to align the wording of the conclusion we achieved last meeting. Ok to delete it also unless there is concern from other companies. Similar situation for the note, I think keep it or not doesn’t matter that much, I will suggest removing it if no concern from other companies. In summary, the revised versions are provided below. Revision #1: Proposed conclusion 2-3-1: For PDC method based on legacy TA-based mechanism, it is assumed that the TA value for PDC is the timing advance value associated with the PTAG of MCG. - Note: No RAN1 specification impact is expected for this conclusion Revision #2: Proposal 2-3-1: For PDC method based on legacy TA-based mechanism, it is assumed that the TA value for PDC is the timing advance value associated with the PTAG of MCG. - Note: No RAN1 specification impact is expected for this conclusion |
| Samsung | We support this proposal. We can live with E’s revision #2. [Feature lead] If we go to revision #2, I think Ericsson wants to make it as a proposal instead of conclusion. I think either way can work also, though I feel the main idea is more like a conclusion, then we can remove the note to address E// concern since typically no specification impact for a conclusion anyway. |
| Feature lead | The original conclusion is revised per the comment from Ericsson. The revised one will be treated in the GTW. Please check my replies to Ericsson above also to understand the reasons for the revision. |

Other issues

Issue #2-4: Whether to include SRS-Resource-ID and/or dl-PRS-ID/csi-RS (The ID of a CSI-RS resource) in the Rx-Tx measurement report?

New H3C R1-2200013 In our view, if TRS and PRS are configured for PDC measurement simultaneously, SRS-Resource-ID should be included in the Rx-Tx measurement report provided from the gNB to the UE in order to pair the SRS and TRS/PRS for a gNB Rx-Tx time difference. It is also beneficial for mitigating the Rx/Tx timing errors. Proposal 1: The Rx-Tx measurement report provided from the gNB includes SRS-Resource-ID. In addition, if TRS and PRS are configured for PDC measurement simultaneously, dl-PRS-ID/csi-RS (The ID of a CSI-RS resource) should be included in the Rx-Tx measurement report provided from the UE to the gNB in order to pair the SRS and TRS/PRS for a gNB Rx-Tx time difference. Proposal 2: The Rx-Tx measurement report provided from the UE includes dl-PRS-ID/csi-RS (The ID of a CSI-RS resource).

Feature lead view: The issue doesn’t exist, since TRS and PRS will not be configured for PDC measurement simultaneously according to the agreement below:

Agreement

If RTT-based propagation delay compensation is supported,

- CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE.

- PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

| Company | View |
| --- | --- |
| OPPO | Agree with FL. |
| New H3C | We want to clarify that if both PRS and CSI-RS are configured simultaneously, PRS will be used with high priority. If yes, we agree with FL ‘s view. If not, we need consider this issue. |
| ZTE | We agree with FL. |

Issue #2-5: Whether LS reply is needed for LS in from RAN4 on enhanced TA-based PDC in R1-2200011?

Nokia R1-2200019 RAN1 requested feedback from RAN4 in R1-2108635 on possible enhancements to TA-based PDC and received the reply LS in R1-2200011. As during RAN1#107-e it was decided to support only TA-based PDC based on legacy TA mechanism (without any additional enhancements), the feedback received from RAN4 does not require any additional work on the RAN1 and RAN4 side for the support of TA-based PDC. Moreover, as RAN1 already informed the related RAN1#107-e decision in the LS to RAN2 & RAN4 in R1-2112834 to not further pursue enhanced TA-based PDC, there seems to be no need to further discuss or react on this LS. Observation 2: There is no further RAN1 actions needed on the incoming LS from RAN4 in R1-2200011, as enhanced TA-based PDC is not supported in Rel-17 and RAN4 has been informed about the related decision already in the LS to RAN2 / RAN4 in R1-2112834.

Feature lead view: Agree with Nokia that LS reply is not needed.

| Company | View |
| --- | --- |

Miscellaneous issues on RRC parameters

Several issues on RRC parameters are raised in the contributions submitted to RAN1#107b-e.

RRC parameters for SRS for MIMO

Issue #3-1: RRC parameters for SRS for MIMO

RRC parameters for SRS for MIMO was discussed in RAN1#107-e, but no consensus achieved yet. The two candidate options are summarized as below based on the views in the paper.

- Option 1: Modify usage-r17 in SRS-ResourceSet to indicate that this ResourceSet is used for PDC purposes as below:

  - usage-r17  ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching, PDC}

  - Huawei, HiSilicon, LG, Intel

  - Pros

    - Low specification impact

  - Cons

    - Would reduce the total number of SRS resource sets that can be used for other purposes for MIMO, e.g. Beam management

    - No flexibility to configure the pathloss reference signal as either SSB or PRS, if PRS along with SRS for MIMO is supported

- Option 2: Add new “usage-pdc-r17” field to SRS-ResourceSet to indicate that this ResourceSet is used for PDC purposes, meanwhile also indicate that this ResourceSet is used for other purpose by usage-r17.

  - Nokia, NSB, Huawei, HiSilicon, LG, Intel, ZTE

  - Pros

    - Low specification impact

    - Avoid reducing the total number of SRS resource sets that can be used for other purposes for MIMO, by reusing one of the SRS resource sets for MIMO

  - Cons

    - No flexibility to configure the pathloss reference signal as either SSB or PRS, if PRS along with SRS for MIMO is supported

- Option 3: Introduce a new SRS-PDCResourceSet for SRS for MIMO.

  - Ericsson, [Samsung], [OPPO],

  - Pros

    - No impact on the total number of SRS resource sets that can be used for other purposes for MIMO

    - Provide the feasibility to separate SRS configuration is that pathlossReferenceRS and spatialRelationInfo can be easily customized for PDC

  - Cons

    - More specification impact

    - Potential more RRC signaling overhead

Feature lead: There are pros and cons for each of the above options. In addition, it may also depend on the outcome from issue 2-1. For example, if SRS for positioning is not supported, then SRS for MIMO will have to be used along with PRS, in which case we may need to change pathlossReferenceRS and spatialRelationInfo for more flexibility, though it seems not really necessary.

First round discussion

The following question is set for the first round email discussions.

Question 3-1-1: Which option among the above three options do you prefer? Please explain your reasons for your choices also.

| Company | View |
| --- | --- |
| Nokia, NSB | Option 2 As this has least restrictions and specifications impact. @FL, what is in Cons of Option 2 should be in Pros in our view, as the usage indication for PDC should not exclude that the SRS resource set is also used for MIMO, thus not impacting the total number of SRS resource set for MIMO. [Feature lead] Yes it is a typo, corrected accordingly above and I also added the potential cons compared to option 3. |
| OPPO | Our preference in our contribution is not listed here. Our preference is to define a RRC IE container to hold configurations for PDC, in order to isolate the PDC-related configurations for a DL/UL RS from the general/basic configurations of those RS’s so that the reconfiguration of PDC-oriented RS would not lead to reconfiguration of those RS themselves (imagine those RS’s may be used for non-PDC purpose as well). PDC-Info-container ::= SEQUENCE { DL-RS CHOICE { nzp-CSI-RS-ResourceSetId NZP-CSI-RS-ResourceSetId, /// this is TRS PDC-PRS SEQUENCE { /// This is PRS …… }, }, srs-ResourceSetId SRS-ResourceSetId, /// This is SRS } The logic is that one DL/UL RS is implicitly assigned a “usage=PDC” only if its configuration shows up in that RRC IE container configuring the PDC, and there can be only one DL RS and only one UL RS in that container. [Feature lead] The proposal in your contribution is to be discussed in section 3.6, since it is also related to DL RS. Let’s discuss there. To clarify, we do not support Option 3, which adds PRS as pathloss/spatial reference to SRS. We believe this is unnecessary, unless there is clear evidence showing the fundamental differences between SSB/CSI-RS and PRS for pathloss and spatial relationships in PDC procedure. [Feature lead] Going to option 3 doesn’t necessarily mean the change for pathloss, it just provide the chance if in the end people would like to do it. We also disagree Option-1 has the low spec impacts. In our view, Option-1 could have the largest spec impacts among the three options, because Option-1 should make spec to clarify the UE behaviors for the case when UE was originally given a non-PDC “usage” but now a replacement of “PDC usage”. It is also not clear to us how to make the same SRS for both usages of PDC and non-PDC under Option-1. [Feature lead] For option 1, there is no replacement, since from the beginning the set will be configured for utilization of PDC. If we have to make a choice from these three options, Option-2 has our least dislike. In this case, the new field of “usage-pdc-r17” seems to have no difference from the “pdc-info” used in DL TRS configuration. It is better to use “pdc-info” for SRS as well instead of having two fields to hold the same prefix of “usage”. [Feature lead] Thanks for the flexibility, I do agree option 2 is better than option 1. If we all agree no need to change the pathloss RS, then option 2 should be the choice in my understanding. |
| New H3C | It isn’t necessary to limit SRS for MIMO for PDC purpose. |
| HW/HiSi | We prefer Option 1, since the spec impact is very small and the number of SRS resources for MIMO is only very slightly reduced (considering that the maximum number of SRS-ResourceSet per BWP is 16). |
| LG | We slightly prefer Option 1 or Option 2 but it is related to outcome of issue 2-1. We would like to make decision after resolving issue 2-1. |
| ZTE | We prefer Option 2 since it enable one SRS for both MIMO and PDC. We think any SRS can be used for PDC as long as it has a large bandwidth. Therefore, SRS for MIMO and SRS for PDC can share the same configuration. |
| Intel | Option 1 or 2. For Option 3, we think it is an unnecessary flexibility to separately configure SRS. It would require to also mention this new configuration everywhere for SRS procedures in RAN1 specs. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-1-1: Add new “usage-pdc-r17” field to SRS-ResourceSet to indicate that this ResourceSet is used for PDC purposes, meanwhile also indicate that this ResourceSet is used for other purpose by usage.

| Company | View |
| --- | --- |
| Feature lead | Please all check the discussion in the first round email discussion, especially the pros and cons for each option, and see if you can accept this majority view. |
| Ericsson | Do not agree. Our concern with the proposal above (also Option 1 and Option 2 in section 3.1.1) is, existing SRS-ResourceSet does not allow SRS to be associated with PRS in pathlossReferenceRS and spatialRelationInfo, even if {PRS, SRS} is the pair of reference signal for PDC. If pathlossReferenceRS and spatialRelationInfo can be expanded to include PDC PRS when usage-pdc-r17 indicates PDC, then we can be flexible with the proposal. Modified proposal is recommended: Add new “usage-pdc-r17” field to SRS-ResourceSet to indicate that this ResourceSet is used for PDC purposes, meanwhile also indicate that this ResourceSet is used for other purpose by usage-r17. Add pathlossReferenceRS-r17 and spatialRelationInfo-r17, which are used in place of pathlossReferenceRS and spatialRelationInfo when usage-pdc-r17 is set to True. [Feature lead]: Based on the views shared in the first round email discussion, I think a few companies think that there is no need to link DL PRS as the reference for SRS for MIMO. However, let me make an additional proposal to check the views from other companies. Based on your views here, I think proposal 3-1-1 itself is agreeable to you. |
| Samsung | Fine with the proposal. |
| New H3C | Support this proposal. |
| OPPO | We are fine with the first part talking about “usage-pdc-r17”, but more preferring to using the same field name of “pdc-info=enumerate{true}” as used in TRS for PDC. We do not think it is necessary to create a new usage-r17 structure; instead, the legacy usage=enumerate{beamManagement, codebook, nonCodebook, antennaSwitching} should be maintained. If RAN1 introduce usage-r17= ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching, PDC}, it logically introduces a new Rel-17 SRS that actually does not perform any legacy SRS function. However, this new Rel-17 SRS only helps gNB to measure RTT, which is nothing different from the measurement in legacy specification, where the SRS usage has 4 choices (i.e., w/o PDC) and the legacy positioning has PRS on DL. Update: we messed up the revised proposal with the original proposal in 1st round regarding to “usage-r17 vs. usage”. Given the 2nd round proposal now does not introduce new “usage-r17”, we can accept the proposal although we prefer to using “pdc-info=enumerate{true}” instead of “usage-pdc-r17”. [Feature lead]: Thank you very much for further checking and being flexible. |
| Nokia, NSB | Support the proposal. |
| ZTE | Support this proposal. |
| HW/HiSi | Support the proposal. |
| Intel | The proposal is agreeable. |

Summary of the status for proposal 3-1-1 in 2nd round

| Support | Samsung, New H3C, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

Feature lead: It seems all companies support or accept this proposal, not sure if Ericsson is ok with my replies above though.

Proposal 3-1-2: Add new “pathlossReferenceRS-PDC-r17” field to SRS-ResourceSet to indicate a reference signal (e.g. a CSI-RS config or a SS block or a DL-PRS config) to be used for SRS path loss estimation, with pathlossReferenceRS-PDC-r17 as below:

PathlossReferenceRS-PDC-r17 ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

dl-PRS-PDC                          nr-DL-PRS-ResourceID-r16

}

| Company | View |
| --- | --- |
| Feature lead | Ericsson has strong view to allow SRS to be associated with PRS in pathlossReferenceRS and spatialRelationInfo. In my understanding, it should be fine to do this way to leave more flexibility. Hopefully it can be acceptable for companies. Note that it is assumed that Proposal 2-1-1 in section 2.1.2 is agreed, i.e. SRS for positioning is not supported for RTT-based PDC. |
| OPPO | We do not support this proposal. This is not just for RRC parameter change. The power control section in 38.213 also gets some impacts due to this proposal. In our view, given PDC-SRS is an intra-cell signal, it is sufficient for its power control loop to follow existing procedure, there is no need to introduce new pathloss reference signal. |
| Nokia, NSB | Do not support We don’t think it is strictly needed to allow PRS to be yet another pathloss reference for SRS. In our view it is sufficient for the UE to rely on other references signals to estimate the transmission power needed for SRS (say SSB). If enough motivation is presented for having such flexibility, then the PRS option could also be added to the existing reference signal configuration which can be configured for such particular cases. |
| ZTE | We don’t think it is necessary. If a PRS is configured for PDC, we think configuring a DL-RS (SSB or CSI-RS) QCL-ed with the PRS as path loss RS can work. |
| Intel | Tent to agree that PRS as a pathloss reference is only needed for other cell measurements. For the serving cell there are already signals for that purpose. |

Summary of the status for proposal 3-1-2 in 2nd round

| Support | Ericsson |
| --- | --- |
| Not support | OPPO, Nokia, NSB, ZTE, - Given PDC-SRS is an intra-cell signal, it is sufficient to follow existing procedure to rely on other references signals to estimate the transmission power needed for SRS (i.e. SSB or CSI-RS), there is no need to introduce new pathloss reference signal |

Feature lead: It seems more companies don’t support this proposal. Further discussion is still needed.

Proposal 3-1-3: Add new “spatialRelationInfo-PDC-r17” field to SRS-Resource to indicate the spatial relation between a reference RS and the target SRS, with spatialRelationInfo-PDC-r17 as below:

spatialRelationInfo-PDC-r17 ::=     SEQUENCE {

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

dl-PRS-PDC                          nr-DL-PRS-ResourceID-r16

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

| Company | View |
| --- | --- |
| Feature lead | Ericsson has strong view to allow SRS to be associated with PRS in pathlossReferenceRS and spatialRelationInfo. In my understanding, it should be fine to do this way to leave more flexibility. Hopefully it can be acceptable for companies. Note that it is assumed that Proposal 2-1-1 in section 2.1.2 is agreed, i.e. SRS for positioning is not supported for RTT-based PDC. |
| OPPO | Same comment as for Proposal 3-1-2. |
| Nokia, NSB | Support We are OK by adding PRS as another option for spatial relation for SRS. |
| ZTE | Similar as above, we don’t think it is necessary. In addition, it was agreed that one PRS and one SRS can be configured for PDC. The UE can assume they are QCL. |
| Intel | We are open to spatial relation information for SRS. |

Summary of the status for proposal 3-1-3 in 2nd round

| Support | Ericsson, Nokia, NSB |
| --- | --- |
| Not support | OPPO, ZTE - Given PDC-SRS is an intra-cell signal, it is sufficient to follow existing procedure. |

Feature lead: It seems more companies don’t support this proposal. Further discussion is still needed.

3rd round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-1-1: Add new “usage-pdc-r17” field to SRS-ResourceSet to indicate that this ResourceSet is used for PDC purposes, meanwhile also indicate that this ResourceSet is used for other purpose by usage.

| Company | View |
| --- | --- |
| Feature lead | @ Ericsson All companies support or accept this proposal, not sure if Ericsson is ok with my replies above though. Please check my replies to you in section 3.1.2 and see if the proposal is acceptable for you. |
| Nokia, NSB | We support the proposal. |
| LG | We can accept the proposal. |
| Samsung | OK |

Revised Proposal 3-1-2:

- Alt.1: Add new “pathlossReferenceRS-PDC-r17” field to SRS-ResourceSet to indicate a reference signal (e.g. a CSI-RS config or a SS block or a DL-PRS config) to be used for SRS path loss estimation, with pathlossReferenceRS-PDC-r17 as below:

PathlossReferenceRS-PDC-r17 ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

dl-PRS-PDC                          nr-DL-PRS-ResourceID-r16

}

- Alt.2: No need to add new “pathlossReferenceRS-PDC-r17” field to SRS-ResourceSet to indicate a reference signal (e.g. a CSI-RS config or a SS block or a DL-PRS config) to be used for SRS path loss estimation.

  - Note: With Alt.2, the existing RRC parameter PathlossReferenceRS-Config is used to indicate a reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation.

| Support Alt.1 | Ericsson |
| --- | --- |
| Support Alt.2 | OPPO, Nokia, NSB, ZTE, LG - Given PDC-SRS is an intra-cell signal, it is sufficient to follow existing procedure to rely on other references signals to estimate the transmission power needed for SRS (i.e. SSB or CSI-RS), there is no need to introduce new pathloss reference signal |

| Company | View |
| --- | --- |
| Feature lead | Based on the inputs in the second round, more companies prefer not to introduce new RRC parameter to configure DL-PRS as the pathloss reference signal for SRS. More views are needed before making decision. Please indicate if you have very strong concern on any of the alternatives. |
| OPPO | We maintain our position of supporting Alt-2. |
| Nokia, NSB | Slightly prefer Alt.2. We don’t see the need for the additional flexibility provided with Alt.1 with the current arguments on the table. Alt.2 should be reformulated to reflect existing pathlossReferenceRS in SRS-ResourceSet, that already supports either an SSB or CSI-RS. PRS is not an option there. Alt.2: No need to add new “pathlossReferenceRS-PDC-r17” field to SRS-ResourceSet to indicate a reference signal (e.g. other than a CSI-RS config or a SS block or a DL-PRS config) to be used for SRS path loss estimation. [Feature lead] The contents in the bracket is to describe what the new “pathlossReferenceRS-PDC-r17” to indicate, if we make the change as you suggested above, it looks like that the new RRC parameter can be introduced to indicate CSI-RS or SSB, just not DL-PRS. Therefore, it seems the original wording is better. We can add a sub-bullet to clarify in this case we just following the existing way. |
| ZTE | We support Alt-2. |
| LG | We support Alt. 2 |

Revised Proposal 3-1-3:

- Alt.1: Add new “spatialRelationInfo-PDC-r17” field to SRS-Resource to indicate the spatial relation between a reference RS and the target SRS, with spatialRelationInfo-PDC-r17 as below:

spatialRelationInfo-PDC-r17 ::=     SEQUENCE {

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

dl-PRS-PDC                          nr-DL-PRS-ResourceID-r16

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

- Alt.2: No need to add new “spatialRelationInfo-PDC-r17” field to SRS-Resource to indicate the spatial relation between a reference RS and the target SRS.

  - Note: With Alt.2, the existing RRC parameter SRS-SpatialRelationInfo is used to indicate the spatial relation between a reference RS and the target SRS.

| Support Alt.1 | Ericsson, Nokia, NSB |
| --- | --- |
| Support Alt.2 | OPPO, ZTE, LG - Given PDC-SRS is an intra-cell signal, it is sufficient to follow existing procedure. |

| Company | View |
| --- | --- |
| Feature lead | Based on the inputs in the second round, more companies prefer not to introduce new RRC parameter to configure DL-PRS as the pathloss reference signal for SRS. More views are needed before making decision. Please indicate if you have very strong concern on any of the alternatives. |
| OPPO | We maintain our position of supporting Alt-2. |
| Nokia, NSB | Prefer Alt. 1. However, we believe that both alternatives can work. Either we add the new parameter, or it is stated that the UE can assume that the SRS Resource use spatialRelationInfo from TRS/PRS if not configured for SSB. |
| ZTE | We support Alt-2. The current mechanism works very well. |
| LG | We support Alt. 2. Different from position procedure, all procedure of PDC are performed in same cell. We think it is sufficient to follow existing procedure. |

RRC parameters for PRS for PDC

Issue #3-2: Whether to include RRC parameters dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 for PRS for PDC?

The candidate options and company positions based on the contributions are summarized as below:

- Option 1: Do not include dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 from 37.355 in the PRS PDC configuration in 38.331.

  - Nokia, NSB, New H3C, ZTE

  - Reasons

    - dl-PRS-ResourceRepetitionFactor-r16. Indeed useful for coverage limited use cases, or when extended coverage is needed for PRS, which are not the case when the UE only needs PRS from the serving cell.

    - dl-PRS-ResourceTimeGap-r16. Only needed if dl-PRS-ResourceRepetitionFactor-r16 is provided, as it indicates the time between the same PRS Resource ID is repeated.

- Option 2: Add “dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 in the RRC parameters list for RTT-based PDC

  - Ericsson, Huawei, HiSilicon, Intel, LG

  - Reasons

    - According to the current specification for positioning, it seems that theses parameters can be used for any serving cell, including serving cell and neighbor cell, therefore it should be straightforward to include unless problems identified.

    - PRS may need to rely on repetition to achieve the desired reception accuracy, especially when the PRS bandwidth is limited. Parameter dl-PRS-ResourceTimeGap is a companion to dl-PRS-ResourceRepetitionFactor.

First round discussion

The following questions are set for the first round email discussions.

Question 3-2-1: Which option among the above two options do you prefer? Please explain your reasons for your choices also.

| Company | View |
| --- | --- |
| Nokia, NSB | Option 1. We think this is simpler, as in our view, these parameters are mainly useful for improving the detection of PRS of neighbor cells, while we don’t see a big relevance of these features for serving cell. |
| OPPO | Prefer Option-2. The two fields, if added to 38.331, should be still optional as in 37.355. Reason: In case the same PRS is transmitted for both PDC purpose and positioning purpose and the repetition is needed on positioning side, it is beneficial to let PDC UE know it has repeated PRS resources to utilize (for free). |
| New H3C | We have the same view with Nokia. |
| HW/HiSi | Option 2. It is straight forward to include these values and they are needed to meet the accuracy. Please note that in measurement accuracy requirement e.g. Table 10.1.25.2-2: UE Rx-Tx time difference measurement accuracy in FR1 in fading, the repetition is already used to define this requirement. Without these parameters, RAN4 may not be able to reuse these requirements as much as possible since the configuration may be different. |
| LG | Option 2. We think these parameter would be necessary at least when PRS is configured both for PDC and positioning. Thus, it would be good at least to allow to configure these parameter optionally. |
| ZTE | We prefer not to introduce the parameter that we don’t use. Therefore, Option 1 is preference. |
| Intel | Option 2, for future proofness and better accuracy in BW limited situations. |

Feature lead: Company position are reflected in section 3.2 also. Based on the views from companies, there is no strong majority view. However, I would encourage companies to consider option 2, since anyway no harm to keep them meanwhile these parameters might be used in some cases also.

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-2-1: Add “dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 in the RRC parameters list for RTT-based PDC

| Company | View |
| --- | --- |
| Feature lead | Please all check the discussion in the first round email discussion, especially the pros and cons for each option, and see if you can accept with this proposal. |
| Ericsson | Support We think these parameters are needed for cases where PRS repetitions are necessary to achieve the desired accuracy. See 38.133 V17.3.0, Table 10.1.25.2-2, where “PRS resource repetition” is a parameter of the UE Rx-Tx Time Diff accuracy table. |
| New H3C | Support |
| OPPO | Agree |
| Nokia, NSB | Support For the sake of progress, we can support the proposal. |
| ZTE | Even positioning and PDC can share the same PRS configuration, the configuration should still be separately since their configuration methods are totally different. We don’t think no harm is a reason to introduce something useless. However, we agree the comments on the repetition factor from other companies. So we can accept this parameter. |
| HW/HiSi | Support. |
| Intel | Support as per initial round of dicsussion |

Summary of the status for proposed conclusion 3-2-1 in 2nd round

| Support | Ericsson, New H3C, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

FL recommendation: The proposed conclusion is stable.

Issue #3-3: Whether to include RRC parameter dl-PRS-ResourcePower-r16 for PRS for PDC?

The candidate options and company positions based on the contributions are summarized as below:

- Option 1: Do not include dl-PRS-ResourcePower-r16 from 37.355 in the PRS PDC configuration in 38.331.

  - Nokia, NSB, OPPO, New 3HC, ZTE, Intel

  - Reasons

    - dl-PRS-ResourcePower-r16. This specifies the transmission power for the PRS signal, which is useful for PL estimation based on PRS transmitted from neighbour cells. For PRS used for PDC, it would be sufficient to give the UE the PRS transmission power relative to another DL RS (e.g. SSB).

- Option 2: Include dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC

  - Huawei, HiSilicon, LG, Intel

  - Reasons

    - It is true that dl-PRS-ResourcePower-r16 can be applied for the case of receiving PRS from neighbor cells, but in our understanding it is also used for the case of receiving PRS from the serving cell, therefore it should be straightforward to include this parameter to enable receiving PRS from the serving cell, otherwise we need a new RRC parameter as indicated in option 3.

- Option 3: Do not include dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC, and add an new relative value (e.g., powerControlOffsetSS) to signal the EPRE of PRS for PDC

  - Ericsson, Nokia, Intel

  - Reasons

    - dl-PRS-ResourcePower-r16 is also used for receiving PRS from neighbor cell, thus the range of values to indicate is very wide. A relative value can be introduced to indicate receiving PRS from the serving cell.

First round discussion

The following question are set for the first round email discussions.

Question 3-3-1: Which option among the above three options do you prefer? Please explain your reasons for your choices also.

| Company | View |
| --- | --- |
| Nokia, NSB | Option 1 or Option 3. We don’t see the need of dl-PRS-ResourcePower-r16 with a large range of values for serving cell only. A fixed value relative to e.g. SSB is sufficient. |
| OPPO | Prefer to Option-1. For Option-2, could Huawei please clarify what PDC-related UE behavior (either specified or implementation-based) has to rely on information provided in this field? We’d like to learn whether this new field would bring in somehow new UE behavior. [Feature lead] All behavior is exactly the same as that for positioning, I don’t see any new behavior if I didn’t miss anything. For option 1 or option 3, for sure we need some specification impact due to new introduced RRC parameter (option 3) or no RRC parameter to use (option 2). For Option-3, we did not see fundamental difference from Option-2. |
| New H3C | We support option 1. |
| HW/HiSi | Option 2. We think it is straight forward to include “dl-PRS-ResourcePower-r16” and avoids the introduction of a new RRC parameter. Please note that dl-PRS-ResourcePower-r16 is not a new field, it is an existing field. But Options 1 and 3 would add a new field. |
| LG | Option 2. We think these parameter would be necessary at least when PRS is configured both for PDC and positioning. Thus, it would be good at least to allow to configure these parameter optionally. |
| ZTE | We prefer Option 1. The purpose of the PRS is for DL synchronization and Rx-Tx time difference measurement. The UE does not need to know the transmission power of PRS. |
| Intel | All options are acceptable to us. |

Feature lead: Company position are reflected in section 3.3 also. Based on the views from companies, it seems majority companies want to support option 1. However, one observation is that if we go to option 1, and if DL PRS is also used as the pathloss for SRS, then option 1 doesn’t work well. According to TS 38.213 as below, we need a value for the reference signal power. Therefore, companies are encouraged to consider either option 2 or option 3. With option 2, we can directly reuse the current specifications from both RAN1 and RAN2, though it may include a larger range of candidate values which are not useful for PDC. With option 3 we can narrow down the candidate values to be indicated, but we need to introduce a new RRC parameter and specification impact from both RAN1 and RAN2 are needed.

Copied from TS 38.213 - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos - if a ssb-IndexNcell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 3-3-1: Which option do you prefer? Please indicate if you have strong concern (if any) on any of the options also.

- Option 2: Include dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC, if DL PRS can be used as the pathloss reference signal for SRS also.

  - Huawei, HiSilicon, LG, Intel

- Option 3: Do not include dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC, and add an new relative value (e.g., powerControlOffsetSS) to signal the EPRE of PRS for PDC, if DL PRS can be used as the pathloss reference signal for SRS also.

  - Ericsson, Nokia, Intel, Samsung, New H3C, ZTE, Huawei/HiSilicon (can accept)

| Company | View |
| --- | --- |
| Feature lead | Please check my analysis in section 3.3.1 to understand why only option 2 and 3 are mentioned here. In general, with option 2, we can directly reuse the current specifications from both RAN1 and RAN2, though it may include a larger range of candidate values which are not useful for PDC. With option 3 we can narrow down the candidate values to be indicated, but we need to introduce a new RRC parameter and specification impact from both RAN1 and RAN2 are needed. |
| Ericsson | Option 3 It’s clear that PDC PRS is different from PRS for positioning. We don’t see a good reason to stick with the existing dl-PRS-ResourcePower-r16, which is an inappropriate way to define EPRE for the PDC PRS. |
| Samsung | Perfer Option 3, with similar reason as Ericsson. We don’t think there is a need to “re-use” everything from Pos. We prefer to treat similar as other RS in RAN 1. |
| New H3C | We prefer to Option 3 |
| OPPO | Do not agree Options 2 and 3. We continue supporting Option 1. According to 38.213, “dl-PRS-ResourcePower-r16” is designed for OLPC of positioning SRS given the UE may not be able to identify SSB of a remote assisting cell. But here the positioning SRS is very likely not supported. Even if positioning SRS is supported, the current spec already defines how the OLPC is done for intra-cell transmission -- and that is not related to PRS. Update: The 38.213 text that FL copied in section 3.3.1 analysis is only applicable to positioning SRS, which all companies seem to prefer not to support. So it is not clear to us what this “dl-PRS-ResourcePower-r16” is used for. 38.213 vh00: If a UE transmits SRS based on a configuration by SRS-PosResourceSet on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO_SRS,b,f,cqs and αSRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId from SRS-PosResourceSet, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos - if a ssb-IndexNcell is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower. [Feature lead] Yes the spec is for SRS for positioning, it is an example to show that if DL-PRS will be used as the pathloss reference signal for SRS for MIMO, then something similar is needed for SRS for MIMO. Of course as clarified in the proposal, only if DL-PRS is agreed to be one of the candidate pathloss reference signal. |
| Nokia, NSB | Prefer Option 3 over Option 2. |
| ZTE | Option 3 is preferred to follow the principle of power indication for the RS in the serving cell if DL PRS used for path loss RS is agreed. |
| HW/HiSi | Prefer Option 2 since it is simple due to no additional spec impact. But we can accept Option 3 as well if this is the majority view. |
| Intel | Continue supporting either option with slight preference to Option 3 |

Feature lead: Most companies support or accept option 3, while OPPO doesn’t like either option 2 or option 3. Please OPPO check my replies to your comments and see if you can accept the proposal.

3rd round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-1-1: Do not include dl-PRS-ResourcePower-r16” in the RRC parameters list for RTT-based PDC, and add a new relative value (e.g., powerControlOffsetSS) to signal the EPRE of PRS for PDC, if DL PRS can be used as the pathloss reference signal for SRS also.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO Most companies can accept this proposal based on the discussion in the second round. Please OPPO check my replies to your comments in the previous section and see if you can accept the proposal. |
| OPPO | We can live with the majority view. |
| Nokia, NSB | Support the proposal. |
| Ericsson | Support the main part of the proposal. However, we don’t see the need of adding the condition “if DL PRS can be used as the pathloss reference signal for SRS also”. EPRS of PRS needs to be provided independent of this condition. Suggest to remove “if DL PRS … SRS also” [Feature lead] In the previous discussion, companies asked what the potential impact on dl-PRS-ResourcePower-r16, and it looks like that it is used only when DL-PRS is used as the pathloss reference signal of SRS. So the point here is that if DL-PRS will not be used as the pathloss reference signal for SRS for MIMO, then we don’t need either dl-PRS-ResourcePower-r16 or powerControlOffsetSS. |

Issue #3-4: Whether to update dl-PRS-QCL-Info-r16 to a new RRC parameter for PRS for PDC?

Ericsson (R1-2200443) The QCL configuration of PRS should be updated for PDC purpose, since the PRS is no longer sent from multiple cells. Specifically, NR-DL-PRS-Resource-r16 has a field dl-PRS-QCL-Info-r16, where the following changes are needed for PDC purpose: - Remove ‘pci-r16’ since only one cell (i.e., PCell) is involved in PDC, and its cell ID is already known to the UE. - Add ‘typeA’ to rs-Type-r16. Rs-Type indicates the QCL type between the PRS and the SSB. For PDC, the PRS is expected to be sent from the same TRP as the given SSB, and have the same average propagation delay. Thus ‘typeA’ is a typical QCL type for FR1. - Replace dl-RS by CSI-RS as a choice of QCL source. - For time synchronization use cases, only one PRS configuration (PRS#1) is provided. It cannot be assumed that the UE will also perform positioning at the same time. In other words, it cannot be assumed that the UE has another DL PRS (PRS#2, for positioning) which could possibly be used as QCL source for PRS of PDC (PRS#1). Thus dl-PRS-r16 field should be eliminated from DL-PRS-QCL-Info-r16. - In contrast, CSI-RS can be relied on as a QCL source for PRS of PDC. The CSI-RS may or may not be a TRS configured for PDC purpose (i.e., it may or may not have ‘trs-info’ and ‘pdc-info’ set to true), as long as this CSI-RS has the same average delay as the desired SSB-index, e.g., this CSI-RS has qcl-Type set to ‘typeA’, or ‘typeC’, or ‘typeD’ with reference to the desired SSB-Index. Thus CSI-RS should be added as a QCL choice in DL-PRS-QCL-Info-r16. Proposal 3 For PDC purpose, DL-PRS-QCL-Info-r16 is revised by removing ‘pci-r16’, adding ‘typeA’ to rs-Type-r16, and replace dl-PRS-xx by CSI-RS. In summary, the DL-PRS-QCL-Info-r16 is suggested to be updated as below, where NZP-CSI-RS-ResourceId refers to a proper CSI-RS as discussed above. - DL-PRS-QCL-Info-r16 ::= CHOICE { - ssb-r16 SEQUENCE { - pci-r16 NR-PhysCellID-r16, - ssb-Index-r16 INTEGER (0..63), - rs-Type-r16 ENUMERATED {typeA, typeC, typeD, typeC-plus-typeD} - }, - csi-RS-Index NZP-CSI-RS-ResourceId - dl-PRS-r16 SEQUENCE { - qcl-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, - qcl-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 - } - }

Feature lead: My original thinking is that there is no harm to keep pci-r16 there, since gNB can always indicate the ID of the serving cell here. For DL-PRS-r16, the basic assumptions for PDC is also to include more than one PRS resources in the PRS resource set, considering beamforming case, therefore my understanding is that DL-PRS-r16 is still applicable for PRS for PDC. For CSI-RS, it is not listed as the QCL source for PRS for positioning, then the motivation to add it for PRS for PDC is not that clear to me. But let’s hear more views from companies.

First round discussion

The following question are set for the first round email discussions.

Question 3-4-1: Do you think it is really necessary to make the following changes to DL-PRS-QCL-Info-r16? Please provide your reasons.

- DL-PRS-QCL-Info-r16 ::= CHOICE {

- ssb-r16						SEQUENCE {

- pci-r16							NR-PhysCellID-r16,

- ssb-Index-r16					INTEGER (0..63),

- rs-Type-r16						ENUMERATED {typeA, typeC, typeD, typeC-plus-typeD}

- },

- csi-RS-Index                        NZP-CSI-RS-ResourceId

- dl-PRS-r16					SEQUENCE {

- qcl-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16,

- qcl-DL-PRS-ResourceSetID-r16	NR-DL-PRS-ResourceSetID-r16

- }

- }

| Company | View |
| --- | --- |
| Feature lead | Note that if any of the changes is needed, then we will add a new RRC parameter for QCL for PRS for PDC. |
| Nokia, NSB | No. We agree with FL arguments above. |
| OPPO | Majorly No. The adding of CSI-RS may complicate the logic and need more discussion: for example, what type of QCL relation should be assumed between CSI-RS and PRS? Whether that CSI-RS can be also used (together with PRS) in RTT timing measurement? Adding “Type A” to QCL type seems unnecessary, because the PRS and SSB (and the related SIB-9) have to share the same delay profile, regardless whether “Type-A” is mentioned or not. However, removing “dl-PRS-r16” from the “CHOICE” list seems necessary because: - The field actually asks UE to learn another PRS in PDC procedure – conflicting with earlier RAN1 agreement that UE can have only one PRS for PDC. - It may lead to a mix-up of PRS ID space (by referring to NR-DL-PRS-ResourceID-r16) across positioning protocol and non-positioning protocol, although the RAN1 discussion so far does not clarify the mixed ID space either. [Feature lead] The RAN1 agreement is to have on PRS configuration, but one PRS set can include multiple PRS resource, at least from beamforming perspective this is beneficial |
| New H3C | No need |
| vivo | No. We share the similar view with FL. |
| HW/HiSi | Agree with the FL assessment that it is not really necessary. |
| LG | We share view with FL. |
| ZTE | We share the same view with FL. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-4-2: Add new “DL-PRS-PDC-QCL-Info” field to NR-DL-PRS-Resource-r16 to specify the QCL indication with other DL reference signals for DL PRS for PDC, with DL-PRS-PDC-QCL-Info as below:

DL-PRS-PDC-QCL-Info ::= CHOICE {

ssb-r16						SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

rs-Type-r16						ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

csi-RS-Index                                NZP-CSI-RS-ResourceId

dl-PRS-PDC					SEQUENCE {

qcl-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16,

}

}

| Company | View |
| --- | --- |
| Feature lead | According to the inputs from Ericsson in the table below highlight in grey, Ericsson has strong view to take CSI-RS as one of the QCL source for DL PRS for PDC. Indeed it can provide more flexibility, therefore I made the above proposal for companies to check, and to see if any strong concern. Note that it is assumed that Proposal 2-1-1 in section 2.1.2 is agreed, i.e. SRS for positioning is not supported for RTT-based PDC. |
| New H3C | Support |
| OPPO | We do not support this proposal. We support the original one “No further changes needed for DL-PRS-QCL-Info-r16”. Regarding to Ericsson’s comment “In our view, this CSI-RS can provide better beam info than SSB in FR2”, our understanding is that RAN1’s PDC focus in on 15kHz and 30kHz SCS in FR1. In addition, adding CSI-RS as QCL source of PRS has no direct relation to timing measurement but meanwhile goes beyond Rel-16 UE implementation for PRS reception. |
| Nokia, NSB | Do not support It is not clear to us why CSI-RS needs to be an option for QCL for PRS for PDC. We would appreciate additional arguments. |
| ZTE | If SSB is enough for PRS for positioning, we think there is no need to introduce new RS for PDC. |

Summary of the status for proposal 3-4-2 in 2nd round

| Support | Ericsson, New H3C - Provide better beam info by taking CSI-RS as the QCL source. |
| --- | --- |
| Not support | OPPO, ZTE, Nokia, NSB - RAN1’s PDC focus in on 15 kHz and 30 kHz SCS in FR1, thus better beam info is not the target. - Adding CSI-RS as QCL source of PRS has no direct relation to timing measurement but meanwhile goes beyond Rel-16 UE implementation for PRS reception. |

Feature lead: It seems more companies don’t support this proposal. Further discussion is still needed.

Proposed conclusion 3-4-1: No further changes needed for DL-PRS-QCL-Info-r16.

| Company | View |
| --- | --- |
| Feature lead | @ Ericsson Based on the discussion in the first round, almost all companies support the conclusion here. Please check the discussions and see if the proposed conclusion here can be acceptable for you, though it is not your preference. |
| Ericsson | Do not agree Regarding CSI-RS: it is necessary to have it. Existing CSI-RS/TRS and SRS both have CSI-RS in their QCL relation, see below. In our view, this CSI-RS and can provide better beam info than SSB in FR2. Since PDC PRS behaves the same as TRS for PDC usage, we don’t think PRS should be put in disadvantage by not having this. [Feature lead] I agree listing CSI-RS here can provide more flexibility, though it is not clear yet whether it is really necessary since CSI-RS is not listed there for PRS for positioning. I can check again if any company has strong concern to do this. Regarding ‘typeA’: we are fine not to add it since FL pointed out in section 3.5 that periodic TRS does not have typeA either. Existing QCL info for CSI-RS/TRS: QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } Existing QCL info for SRS: SRS-SpatialRelationInfo ::= SEQUENCE { servingCellId ServCellIndex OPTIONAL, -- Need S referenceSignal CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId, srs SEQUENCE { resourceId SRS-ResourceId, uplinkBWP BWP-Id } } } Regarding removing ‘dl-PRS-r16’: in our understanding, existing ‘dl-PRS-r16’ refers to positioning PRS resources, hence it should be removed. If the intention is to use another PDC PRS resource in the same resource set, we are open to add NR-DL-PRS-PDC, which is the RRC parameter name for PDC PRS. Since there is only one NR-DL-PRS-PDC-ResourceSet, it is not necessary to include this field. In summary, we can recommend the following: DL-PRS-QCL-Info-r16 ::= CHOICE { ssb-r16 SEQUENCE { pci-r16 NR-PhysCellID-r16, ssb-Index-r16 INTEGER (0..63), rs-Type-r16 ENUMERATED {typeC, typeD, typeC-plus-typeD} }, csi-RS-Index NZP-CSI-RS-ResourceId dl-PRS-r16 SEQUENCE { qcl-DL-PRS-ResourceID NR-DL-PDC-PRS-ResourceID, qcl-DL-PRS-ResourceSetID NR-DL-PDC-PRS-ResourcesSetID qcl-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, qcl-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 } } (Blue changes above are optional) [Feature lead] Originally I think it is fine to re-use dl-PRS-r16 here, since it doesn't really refer to real Rel-16 DL PRS, just a name. Please note that NR-DL-PRS-ResourceID-r16 can be reused also, since we didn’t make anything on the resource setting itself, we only introduce an additional set for PRS for PDC. However, if you feel it may cause confusing, we can make the change if other companies are ok with it. |

3rd round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 3-4-2:

- Alt.1: Add new “DL-PRS-PDC-QCL-Info” field to NR-DL-PRS-Resource-r16 to specify the QCL indication with other DL reference signals for DL PRS for PDC, with DL-PRS-PDC-QCL-Info as below:

DL-PRS-PDC-QCL-Info ::= CHOICE {

ssb-r16						SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

rs-Type-r16						ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

csi-RS-Index                                NZP-CSI-RS-ResourceId

dl-PRS-PDC					SEQUENCE {

qcl-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16,

}

}

- Alt.2: No need to add new “DL-PRS-PDC-QCL-Info” field to NR-DL-PRS-Resource-r16 to specify the QCL indication with other DL reference signals for DL PRS for PDC.

  - Note: With Alt.2, the existing RRC parameter DL-PRS-QCL-Info-r16 is used to specify the QCL indication with other DL reference signals for DL PRS for PDC.

| Support Alt.1 | Ericsson, New H3C - Provide better beam info by taking CSI-RS as the QCL source. |
| --- | --- |
| Support Alt.2 | OPPO, ZTE, Nokia, NSB - RAN1’s PDC focus in on 15 kHz and 30 kHz SCS in FR1, thus better beam info is not the target. - Adding CSI-RS as QCL source of PRS has no direct relation to timing measurement but meanwhile goes beyond Rel-16 UE implementation for PRS reception. |

| Company | View |
| --- | --- |
| Feature lead | Based on the inputs in the second round, more companies prefer alternative 2. More views are needed before making decision. Please indicate if you have very strong concern on any of the alternatives. |
| Nokia, NSB | Prefer Alt. 2. However, we believe that both alternatives can work. |
| ZTE | We support Alt.2. Such enhancements on PRS is not needed for PDC. |
| LG | We prefer Alt. 2 |
| Ericsson | Alt 1. We disagree that better reception PRS has no direction relation to timing measurement. For example, in the 38.133 UE Rx-Tx Time Diff measurement accuracy table below, “PRS Ês/Iot” is the first column under condition. In our understanding, it is important to ensure good quality of PRS reception, at least on par with TRS. Also note that positioning PRS couldn’t use CSI-RS, because for positioning purpose, neighbor cell UEs don’t have access to CSI-RS, and had to be limited to broadcast type of signal as reference. But for PDC purpose, this limitation does not exist, and PDC PRS can access the same tools as TRS in the serving cell. [Feature lead] I think companies think that since the target for PDC is FR1 not FR2, then provide better beam information is not the target. On the other hand, people seems don’t want to change the implementation for receiving PRS. Regarding “qcl-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16”: we don’t think the newly introduced PRS can use the same ID name as the positioning PRS. For positioning, this ID NR-DL-PRS-ResourceID-r16 is ‘global’: shared among all gNBs, sent by location server, can be read by UEs in neighbor cells. In contrast, the PDC PRS is ‘local’ in the serving cell. Thus, in our view, a new local ID should be used. qcl-DL-PRS-ResourceID NR-DL-PDC-PRS-ResourceID, This problem should be addressed in both Alt 1 and Alt 2. [Feature lead] We have introduced NR-DL-PRS-PDC-ResourceSet which is specific for PRS for PDC, and then inside NR-DL-PRS-PDC-ResourceSet, there is no need to have differentiated names, i.e. the existing parameter dl-PRS-ResourcePower-r16 can be reused. In the current TS 38.311, there are many this kind of example. One benefit to reuse these RRC parameter is to provide the chance for TS 38.331 to save some potential specification impact, though detailed design still up to RAN2 as the note captured in the excel. I went through the RAN1 specs, and didn’t identify any potential confusion by reusing this parameters, since we already have the differentiated resource set name as the umbrella. Of course, if people prefer new RRC parameter, we can do that also. |

RRC parameters for TRS for PDC

Issue #3-5: Whether to further clarify the QCL type for CSI for tracking (TRS) for PDC?

Ericsson (R1-2200443) TRS is CSI-RS for tracking. Currently TRS can be periodic or aperiodic, where the configuration of aperiodic TRS depends on that of periodic TRS. Aperiodic TRS and periodic TRS resource have the same bandwidth (with same RB location) and the aperiodic TRS being configured with qcl-Type set to ‘typeA’ and ‘typeD’, where applicable, with the periodic TRS resources. When used for time synchronization purpose (i.e., ‘pdc-info’ = true), the gNB should ensure that the NZP CSI-RS resource to be configured as QCL with SSB of Pcell. This ensures that the transmission of DL TRS, UL SRS, and reference time information are associated with a same TRP via the SSB-Index. Furthermore, the qcl-Type of periodic TRS can be set to ‘typeA’, ‘typeC’, or ‘typeD’ with the SSB, where yped is useful for FR2. According to TS 38.214 section 5.1.5, the quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values. For propagation delay compensation, the key metric is average delay. Thus ‘typeB’ is not appropriate for PDC purpose. - ‘typeA’: {Doppler shift, Doppler spread, average delay, delay spread} - ’typeB’: {Doppler shift, Doppler spread} - ‘typeC’: {Doppler shift, average delay} - ‘typeD’: {Spatial Rx parameter} NZP-CSI-RS-Resource ::= SEQUENCE { ... qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index }, qcl-Type ENUMERATED {typeA, typeB, typeC, yped}, ... } Since the existing TRS configuration already allows the desired configuration described above, it is sufficient to describe the proper setting when the TRS is configured for PDC. For example, one sentence can be added to the field description of qcl-InfoPeriodicCSI-RS in 38.331 as shown below. Qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. When the periodic CSI-RS is in a NZP-CSI-RS-ResourceSet with ‘trs-info’ set to true and ‘pdc-info’ set to true, the QCL source is expected to be an SSB-Index in the Pcell, and the QCL type is expected to be typeA, typeC, or yped. Refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList in the PDSCH-Config included in the BWPDownlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1).. For aperiodic TRS, it is adequate to reuse the existing mechanism, i.e., aperiodic TRS has qcl-Type set to ‘typeA’ (for FR1) or ‘typeD’ (for FR2) with the periodic TRS. When configured for PDC purpose, periodic TRS has qcl-Type set to ‘typeA’, ‘typeC’, or ‘typeD’ with an SSB of Pcell.

Feature lead: Based on the spec below, the QCL type for periodic TRS is already only type C or type D, thus seems no additional clarifications needed. However, let’s hear views from other companies first.

Copied from section 5.1.5 in 38.214 For a periodic CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s): - ‘typeC’ with an SS/PBCH block and, when applicable, ‘typeD’ with the same SS/PBCH block, or - ‘typeC’ with an SS/PBCH block and, when applicable,’typeD’ with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition.

First round discussion

The following question is set for the first round email discussions.

Question 3-5-1: Do we need to further clarify in TS 38.331 to preclude ‘type B’ for TRS for PDC?

| Company | View |
| --- | --- |
| Nokia, NSB | No. We agree with FL that RAN1 specs seems already to preclude it for TRS. |
| OPPO | No. Agree with FL comment that there is no need to further clarify. |
| New H3C | No. we agree with FL’s view. |
| Vivo | No. We agree with FL views that no additional clarifications are needed. |
| HW/HiSi | No. Agree with the FL and other companies that commented before us. |
| LG | No. Agree with FL’s comment. |
| ZTE | No. We share the same view with FL. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion 3-5-1: No further clarification needed in TS 38.331 to preclude ‘type B’ for TRS for PDC.

| Company | View |
| --- | --- |
| Feature lead | @ Ericsson Based on the discussion in the first round, almost all companies support the conclusion here. Please check the discussions in the first round email discussions and see if the proposed conclusion here can be acceptable for you, though it is not your preference. |
| Ericsson | Yes, we are fine with the conclusion and explanation. |
| Samsung | Support |
| New H3C | Support this proposal |
| vivo | Support the proposed conclusion. |
| OPPO | Agree. |
| Nokia, NSB | Support |
| ZTE | Support this proposed conclusion. |
| HW/HiSi | Support. |

Summary of the status for proposed conclusion 3-5-1 in 2nd round

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, |
| --- | --- |
| Not support |  |

FL recommendation: The proposed conclusion is stable.

Issue #3-6: Whether to introduce a PDC-specific configuration container (e.g., configured in ServingCellConfig) has one field referring to the DL-RS used for PDC?

OPPO (R1-2200345) Configuration of CSI-RS for tracking in PDC RAN1 #107e agreed in RRC discussion to add a new “pdc-info” field to existing NZP-CSI-RS-ResourceSet, where the presence of “enumerate {true}” for this field means the corresponding NZP-CSI-RS-ResourceSet should be used to measure UE-side RTT in the RTT-based PDC. However, this settlement may have some issues: - RAN1 agreed that UE is configured with only one TRS configuration; however, the above RRC signaling format itself does not prevent UE from being configured with more than one NZP-CSI-RS-ResourceSet with “pdc-info=true”. - RAN2 may certainly restrict the UE to be configured with just one NZP-CSI-RS-ResourceSet with “pdc-info=true” by adding such restriction in the way of RRC configuration (such as in RRC parameter field description), instead of instantiating such restriction by RRC signaling format. But this may introduce an implicit competition among different NZP-CSI-RS-ResourceSets (with trs-info=true), which may add additional complexities, for example, in case the network wants to switch the TRS for PDC – the network has to release the original NZP-CSI-RS-ResourceSets before setting up the new one with “pdc-info=true”, even though the original NZP-CSI-RS-ResourceSets could be in support of some functionalities other than PDC. - The RAN1’s agreement of configuring only one DL-RS between TRS and PRS but not the both is also not reflected in the current RRC framework for PDC. Observation-1: The RRC configuration agreed in RAN1 #107e for TRS in PDC does not fully match RAN1 agreements of supporting no more than one TRS configuration and configuring just one RS between TRS and PRS. So we suggest the RAN1 group to review the earlier agreement made in RRC discussion session for the configuration of TRS for PDC, and to consider the following configuration method which avoids the issues mentioned above. Proposal-1: RAN1 revisits the RAN1 #107e agreement of introducing new RRC field of “pdc-info” in NZP-CSI-RS-ResourceSet, taking into account another configuration solution where a PDC-specific configuration container (e.g., configured in ServingCellConfig) has one field referring to the DL-RS used for PDC, as a “CHOICE” between an existing NZP-CSI-RS-ResourceSetID (corresponding to trs-info=true) and the PRS. Such PDC-specific configuration container could be beneficial or needed anyway from RAN2 perspective to hold RAN2-defined PDC configurations if any.

Feature lead: based on the RAN1 spec below, it is already clear that the UE may be configured with one TRS or one PRS, we also included the agreement on only configuring one TRS or one PRS in the comment column to RAN2, RAN2 would be able to implement it appropriately. As to the switching between TRS for PDC and normal TRS, in my understanding it may only happen in some very corner case, e.g. all the resource set(s) are already configured for other purposes, otherwise no need to release the current one in order to configure one set for PDC. However, let’s hear more views whether there is any change needed here.

Copied from 38.214 For operation with RTT-based propagation delay compensation, the UE may be configured with either: - one CSI-RS for tracking with higher layer parameter pdc-Info for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-r17, or - one PRS configuration of higher layer parameter NR-DL-PRS-PDC-ResourceSet-r17 [12, TS 38.331] for Rx – Tx time difference estimation at UE side and one SRS resource set with usage-r17.

First round discussion

The following question is set for the first round email discussions.

Question 3-6-1: Do we need to introduce a PDC-specific configuration container (e.g., configured in ServingCellConfig) has one field referring to the DL-RS used for PDC, as a “CHOICE” between an existing NZP-CSI-RS-ResourceSetID (corresponding to trs-info=true) and the PRS?

| Company | View |
| --- | --- |
| Nokia, NSB | Not needed. We think the way it has been agreed and informed to RAN2 already meets the purpose, although maybe the container in the first place may not have been a bad idea as such. |
| OPPO | Our full idea is to have a RRC container like below: PDC-Info-container ::= SEQUENCE { DL-RS CHOICE { nzp-CSI-RS-ResourceSetId NZP-CSI-RS-ResourceSetId, /// this is TRS PDC-PRS SEQUENCE { /// This is PRS …… }, }, srs-ResourceSetId SRS-ResourceSetId, /// This is SRS } Then any reconfiguration of PDC RS (such as switching between TRS and PRS, or switching between one TRS (or SRS) to another TRS (or SRS)) only involves reconfiguration of this IE container. The actual PRS/TRS/SRS do not need to be released. From UE perspective, we do not ensure the switching between RS would be a corner case or not. It is about a configuration restriction on network side. Besides, from spec language perspective, there is clear difference between “one RS configuration” and “only one RS configuration”. So if the way of configuration does not prevent more than one RS configuration (e.g., more than one TRS having “pdc-info=true” or more than one SRS having “usage=PDC”), there should be explicit text in RAN1 spec or RAN2 spec to prevent it. As a current spec text example, it is 38.214 that says “Only one SRS resource set can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’”. |
| New H3C | No |
| vivo | No. Current RAN1 spec is already clear and RAN 2 can appropriately implement the agreement. |
| HW/HiSi | We do not need to do that. |
| LG | We don’t think it is necessary. |
| ZTE | No. We don’t see the need since it is clear that only one DL RS and one UL RS can be configured for PDC based RAN1 agreement. |
| Intel | Unnecessary change, the implemented structure works well. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion 3-6-1: No need to introduce a PDC-specific configuration container as proposed in R1-2200345.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO Based on the discussion in the first round, almost all companies support the conclusion here. Please check the discussions in the first round email discussions and see if the proposed conclusion here can be acceptable for you, though it is not your preference. |
| Ericsson | Support |
| Samsung | Support |
| New H3C | Support this proposal |
| vivo | Support |
| OPPO | We respect the majority view, and certainly accept the conclusion given the network vendors believe the potential constraints and inconvenience in current TRS/PRS/SRS configurations for PDC (which we intend to improve) are not issues. |
| Nokia, NSB | Support |
| ZTE | Support |
| Hw/HiSi | Support |
| Intel | Support |

Summary of the status for proposed conclusion 3-6-1 in 2nd round

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

FL recommendation: The proposed conclusion is stable.

Issue #3-7: for PRS configuration for RTT-based PDC, whether to include some parameter(s) in NR-DL-PRS-PositioningFrequencyLayer-r16?

Copied from 37.355 NR-DL-PRS-Resource-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16, dl-PRS-SequenceID-r16 INTEGER (0.. 4095), dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { n2-r16 INTEGER (0..1), n4-r16 INTEGER (0..3), n6-r16 INTEGER (0..5), n12-r16 INTEGER (0..11), … }, dl-PRS-ResourceSlotOffset-r16 INTEGER (0..nrMaxResourceOffsetValue-1-r16), dl-PRS-ResourceSymbolOffset-r16 INTEGER (0..12), dl-PRS-QCL-Info-r16 DL-PRS-QCL-Info-r16 OPTIONAL, --Need ON … } NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE { dl-PRS-SubcarrierSpacing-r16 ENUMERATED {kHz15, kHz30, kHz60, kHz120, …}, dl-PRS-ResourceBandwidth-r16 INTEGER (1..63), dl-PRS-StartPRB-r16 INTEGER (0..2176), dl-PRS-PointA-r16 ARFCN-ValueNR-r15, dl-PRS-CombSizeN-r16 ENUMERATED {n2, n4, n6, n12, …}, dl-PRS-CyclicPrefix-r16 ENUMERATED {normal, extended, …}, … }

Copied from 38.211 - 7.4.1.7.3 Mapping to physical resources in a downlink PRS resource For each downlink PRS resource configured, the UE shall assume the sequence rm is scaled with a factor βPRS and mapped to resources elements k,lp,μ according to ak,lp,μ=βPRS rm m=0, 1, … k=mKcombPRS+koffsetPRS+k' mod KcombPRS l=lstartPRS, lstartPRS+1, …, lstartPRS+LPRS-1 when the following conditions are fulfilled: - the resource element k,lp,μ is within the resource blocks occupied by the downlink PRS resource for which the UE is configured; - the symbol l is not used by any SS/PBCH block used by a serving cell for downlink PRS transmitted from the same serving cell or any SS/PBCH block from a non-serving cell whose time frequency location is provided to the UE by higher layers for downlink PRS transmitted from the same non-serving cell; - the slot number satisfies the conditions in clause 7.4.1.7.4. and where - the antenna port p=5000 - lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter dl-PRS-ResourceSymbolOffset; - the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter dl-PRS-NumSymbols; - the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter dl-PRS-CombSizeN such that the combination LPRS,KcombPRS is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}; - the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is obtained from the higher-layer parameter dl-PRS-CombSizeN-AndReOffset; - the quantity k' is given by Table 7.4.1.7.3-1. The reference point for k=0 is the location of the point A of the positioning frequency layer, in which the downlink PRS resource is configured where point A is given by the higher-layer parameter dl-PRS-PointA. Table 7.4.1.7.3-1: The frequency offset k' as a function of l-lstartPRS. KcombPRS; Symbol number within the downlink PRS resource l-lstartPRS. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11. 2; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1. 4; 0; 2; 1; 3; 0; 2; 1; 3; 0; 2; 1; 3. 6; 0; 3; 1; 4; 2; 5; 0; 3; 1; 4; 2; 5. 12; 0; 6; 3; 9; 1; 7; 4; 10; 2; 8; 5; 11.

Feature lead: In positioning, some PRS parameters e.g. the bandwidth, start RB, point A is not included in the NR-DL-PRS-Resource-r16 configuration. These parameters are included in NR-DL-PRS-PositioningFrequencyLayer-r16 according to the 37.355. Note that the reference point for the PRS frequency mapping is the point A which is configured by dl-PRS-PointA in positioning according to the 38.211. In order to complete the PRS configuration for PDC purpose, we need to discuss whether and how to configure these parameters also.

First round discussion

The following question is set for the first round email discussions.

Question 3-7-1: for the PRS configuration for RTT-based PDC, do we need to also include at least some of the parameter(s) in NR-DL-PRS-PositioningFrequencyLayer-r16? If yes, then which parameter(s) to be included, e.g. dl-PRS-ResourceBandwidth-r16, dl-PRS-StartPRB-r16 and dl-PRS-PointA-r16?

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, …},

dl-PRS-ResourceBandwidth-r16	INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, …},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, …},

…

}

| Company | View |
| --- | --- |
| Nokia, NSB | Not fully clear to us. Is it the intention to pick some of the parameters and add to PRS config in RRC spec? Or is it to check whether to copy-paste the IE NR-DL-PRS-PositioningFrequencyLayer into RRC? dl-PRS-ResourceBandwidth-r16, dl-PRS-StartPRB-r16 and dl-PRS-PointA-r16 seems to be needed. [Feature lead] Yes the intention is to pick some of the parameters and add to the PRS configuration for PDC. The reason is that without some of the parameters, it seems 38.211 cannot do the PRS mapping, or we need some change for 38.211. With these parameters, then it looks to me that 38.211 can be reused. Yes the three parameters are what in my mind also. |
| OPPO | Seems yes. From our view, if it is desirable to maintain the flexibility to reuse the same PRS for both positioning purpose and PDC purpose, RAN1 may want to import all of them, except the ones in list below,; - dl-PRS-SubcarrierSpacing-r16: only 15kHz and 30kHz are needed. - dl-PRS-CyclicPrefix-r16: this field is not needed because PDC is not designed for 60kHz SCS with ECP. As another alternative, RAN1 could import at least resource BW, start-PRB and CombSizeN, and leave UE assuming all others as the same as used by basic DL channels. |
| New H3C | this topic is up to RAN2. |
| HW/HiSi | Yes, at least dl-PRS-StartPRB-r16, dl-PRS-ResourceBandwidth-r16 are needed to locate the frequency location of PRS in the serving cell. Others e.g. numerology, pointA can be the same as the current serving cell. |
| LG | Maybe yes, to configure PRS for any purpose. However, in the perspective of RRC, we are not sure whether we need to make changes. If it is necessary, it would be up to RAN2, based on parameter usages in RAN1 specification. |
| ZTE | We think all the three parameters are needed in order to determine the PRB allocation for PRS. |
| Intel | Support adding the parameters listed by Moderator. For pointA, we think reusing the current serving cell may work, however it may sometimes be hard to align DL PRS configuration for positioning and for PDC in this case. |

Second round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 3-7-1: Include dl-PRS-ResourceBandwidth-r16, dl-PRS-StartPRB-r16 and dl-PRS-PointA-r16 in NR-DL-PRS-Resource-r16 for the PRS configuration for RTT-based PDC.

| Company | View |
| --- | --- |
| Feature lead | It is noted that dl-PRS-ResourceList-r16 in the current specification provide the room to extend the included IEs. |
| Ericsson | For dl-PRS-PointA-r16, we think using a pointA different from the serving cell can be problematic. If the PRBs for PRS and PRBs for other DL signals use different grid, the UE may need time to tune to receive such PRS, but earlier proposal was to remove measurement gap for PRS reception. Also, if using different grid, it may be more difficult to coordinate and avoid PRS colliding in frequency domain with other DL signal in the same cell. [Feature lead] Agree and the intention is not to configure a different point A for PRS. Just by adding this parameter, then there is no need to make RAN1 specification change. My plan to set it as below, then gNB can ensure same point A as the serving cell. dl-PRS-PointA-r16 ARFCN-ValueNR |
| Samsung | Support. Although we prefer to configure PRS using similar way as for TRS, when it applicable. While we also understanding that from resource utilization point of view, it is better to reuse PRS for position from serving cell. |
| New H3C | For PDC purpose, configuration of dl-PRS-PointA-r16 should align with that of serving cell. So it isn’t necessary to configure Point A and PDC can reuse configuration on point A of serving cell [Feature lead] The intention is to also include this parameter then there is no need to make RAN1 specification change. Of course, if people prefer not to include this parameter, then it would be fine also, but we need some RAN1 specification change. |
| Nokia, NSB | Support including dl-PRS-ResourceBandwidth-r16, dl-PRS-StartPRB-r16. We don’t immediately see the need to also include dl-PRS-PointA-r16. It is out understanding that the configured PointA would be the same as used for the serving cell, and hence it is not needed to be configured. [Feature lead] The intention is to also include this parameter then there is no need to make RAN1 specification change. Also gNB can configure it as the same of the serving cell. Of course, if people prefer not to include this parameter, then it would be fine also, but we need some RAN1 specification change. |
| ZTE | We share the same view with other companies that the Point A should be the same. We also agree the intention of minimizing the spec impact. So we can accept this proposal. We are also fine to exclude the PointA parameter. |
| HW/HiSi | Support |
| Intel | Support. Potentially, it can be also captured that a UE does not expect different pointA from serving cell [Feature lead] Maybe one way out, let’s check the views from other companies. |

Summary of the status for proposal 3-7-1 in 2nd round

| Support | Samsung, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support dl-PRS-PointA-r16 | Ericsson, New H3C, Nokia, NSB, ZTE |

FL recommendation: The only controversial point is whether to also include dl-PRS-PointA-r16, let’s further discuss it, while making proposal to approve the other two parameters by email first.

3rd round email discussion

Based on the inputs and analysis above, the following proposals and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 3-7-2 (for GTW):

No need to include dl-PRS-PointA-r16 for the PRS configuration for RTT-based PDC.

- Note: Additional RAN1 specification change is expected

Proposal 3-7-2:

- Alt.1: Include dl-PRS-PointA-r16 in NR-DL-PRS-Resource-r16 for the PRS configuration for RTT-based PDC.

  - UE does not expect a different pointA from the serving cell configured by dl-PRS-PointA-r16.

- Alt.2: No need to include dl-PRS-PointA-r16 in NR-DL-PRS-Resource-r16 for the PRS configuration for RTT-based PDC.

  - Note: Additional RAN1 specification change is expected

| Company | View |
| --- | --- |
| Feature lead | There is pros and cons between Alt.1 and Alt.2, the main intention for Alt.1 is to minimize further RAN1 specification change. But both Alt.1 and Alt.2 can work. |
| New H3C | We still support Alt.2 because dl-PRS-PointA-r16 isn’t necessary to PDC |
| Nokia, NSB | Support Alt. 2 With the formulation in Alt. 1 we can just as well avoid having this parameter and not signal it? [Feature lead] If people think it is ok to make changes for RAN1 specification work instead of reusing existing parameter, then we can just directly go to Alt.2. |
| ZTE | We are fine with either one. |
| LG | We support Alt. 1. Alt. 1 can minimize and it is more aligned with current way of PDC description in spec. |
| Ericsson | Alt 2 Regarding RAN1 spec change, it’s not a concern. One sentence can be added in 38.211 PRS generation, e.g., “for PDC purpose, pointA refers to that of serving cell.” |
| Feature lead | Since most companies prefer Alt.2, and indeed both alternatives can work, it is recommended to go to the majority view. |

Proposals/conclusions for email approval

The proposals/conclusions for email approval after the 2nd round email discussion is summarized as below:

Proposal 2-1-1: SRS for positioning is not supported for RTT-based PDC, regardless of whether TRS or PRS is used for RTT-based PDC.

| Support | Ericsson, Nokia, NSB, vivo, Samsung, ZTE, Huawei, HiSilicon, New H3C, OPPO, Spreadtrum, Intel |
| --- | --- |
| Not support |  |

Proposed conclusion 2-2-1: Measurement gaps should not be mandatory for a UE to process PRS for PDC purposes.

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Spreadtrum, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

Proposal 3-2-1: Add “dl-PRS-ResourceRepetitionFactor-r16 and dl-PRS-ResourceTimeGap-r16 in the RRC parameters list for RTT-based PDC

| Support | Ericsson, New H3C, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

Proposal 3-7-1: Include dl-PRS-ResourceBandwidth-r16 and dl-PRS-StartPRB-r16 in NR-DL-PRS-Resource-r16 for the PRS configuration for RTT-based PDC.

| Support | Samsung, ZTE, Huawei, HiSilicon, Intel, Ericsson, New H3C, Nokia, NSB, ZTE |
| --- | --- |
| Not support |  |

Proposed conclusion 3-5-1: No further clarification needed in TS 38.331 to preclude ‘type B’ for TRS for PDC.

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, |
| --- | --- |
| Not support |  |

Proposed conclusion 3-6-1: No need to introduce a PDC-specific configuration container as proposed in R1-2200345.

| Support | Ericsson, Samsung, New H3C, vivo, OPPO, Nokia, NSB, ZTE, Huawei, HiSilicon, Intel |
| --- | --- |
| Not support |  |

- References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR , Nokia, Nokia Shanghai Bell

- R1-2200013	Discussion on propagation delay compensation enhancements	New H3C Technologies Co., Ltd.

- R1-2200019	On remaining issues of propagation delay compensation	Nokia, Nokia Shanghai Bell

- R1-2200110	Discussion on propagation delay compensation enhancements	ZTE

- R1-2200200	Remaining issues for propagation delay compensation enhancements	Samsung

- R1-2200345	Enhancement for support of time synchronization	OPPO

- R1-2200375	Remaining issues for RTT-based propagation delay compensation	Intel Corporation

- R1-2200677	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

- R1-2200574	Discussion on propagation delay compensation enhancements	LG Electronics

- R1-2200650	Enhancements for support of time synchronization	Huawei, HiSilicon

- R1- 2112978 Summary of Email discussion on Rel-17 RRC parameters for LS to RAN2 Moderator (Ericsson)

- R1-2112953 Introduction of NR Positioning Enhancements

- Appendix Agreements in the past meetings

RAN1#102-e

Agreements:

- Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Agreements:

- ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation.

Agreements:

For 5GS synchronicity budget requirement,

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

Agreements:

For BS transmit timing error, further study the following three options:

- Option 1: 65 ns

- Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario

- Option 3:82.5 ns

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Asymmetry between downlink and uplink channel for control-to-control scenario is not considered.

Agreements:

100 ns is assumed for BS detecting error.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

Agreements:

Send an LS to RAN2 with the content including

- Inform RAN2 the two representative use cases concluded in RAN1 for further study;

- Ask RAN2 for input about Uu interface error budget for each of the two use cases;

Agreements:

The following options for propagation delay compensation are further studied in RAN1

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

Draft LS R1-2007445 is approved, with final LS in R1-2007446.

RAN1#103-e

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

- Asymmetry between downlink and uplink channel for smart grid scenario is not considered.

- TA adjustment accuracy is not considered for the evaluation of time synchronization error.

- errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error.

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

Agreements:

For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error:

- Option 1: 200 ns

- Option 2: 65 ns

RAN1#104-e

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately.

- Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not

  - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background

- FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

Decision: As per email posted on feb 5th, the draft LS is endorsed. Final LS is approved in R1-2102245.

RAN1#104b-e

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation

Agreements: Take the following equation for evaluation of the DL propagation delay estimation error for TA based propagation delay compensation:

- Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245.

- FFS whether errorBS,DL,TX in the above equation should be included or not.

Agreements:

- Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario.

- Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.

Working assumption:

Agreement:

Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC.

- The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure.

- errorUE,DL,RXerrorUE, DL, RX is based on other signals (e.g. CSI-RS) instead of SSB.

- errorBS, UL,RXerrorUE, DL, RX iss based on other uplink signals instead of contention based PRACH, e.g. SRS.

- Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method.

Agreement:

Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- FFS whether PRS can be used for UE Rx – Tx time difference estimation or not

- FFS which DL reference signal(s) to be used if/when PRS is not used

Conclusion:

- Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

RAN1#106-e

Agreement

SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

Agreement

Send LS to RAN4 to ask for feedback on the following questions:

- Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most.

- Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition.

- Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4

- Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also.

- Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information.

- Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

Agreement

If RTT-based propagation delay compensation is supported,

- CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE.

- PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

Agreement

Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

- gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

R1-2108513	Feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

Agreement

Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured

- At least one SRS configuration for Rx – Tx time difference estimation at gNB side

Agreement

If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least:

- gNB Rx-Tx time difference at a given granularity

- FFS whether to include SRS-Resource-ID

Agreement

Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e.

- Alt. 1:

  - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

  - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference

- Alt. 2:

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing

R1-2108618 Draft LS on TA-based propagation delay compensation 	Moderator (Huawei)

Decision: The draft LS is endorsed with the following note

- Note: It’s pending further discussion in RAN1 whether the WA is to be confirmed including which alternative is to be selected

Final LS is approved in R1-2108635.

RAN1#106bis-e

Agreement

For evaluation of the overall time synchronization error for RTT-based propagation delay compensation,

- Alt.1 for RTT-based PDC

Agreement

For evaluation of the overall time synchronization error for TA-based propagation delay compensation,

- Alt.1 for TA-based PDC

Agreement

For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that

- The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy

- The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC

Agreement

For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported.

Agreement

If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least:

- UE Rx-Tx time difference at a given granularity

Conclusion

When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error.

Agreement

Send an LS to RAN2 and CC RAN4 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

R1-2110594	Draft LS on propagation delay compensation	Huawei

Decision: The draft LS is endorsed. Final version is approved in R1-2110647.

Agreement

For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively.

- Note: Detection error achieved by evaluations is not precluded if available.

Agreement

If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC.

- FFS whether/how to configure UL signal for enhanced TA-based PDC

Agreement

If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17,

- The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity.

Agreement

If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5.

- FFS the value of k

- FFS the reporting range of Rx-Tx time difference measurement for PDC

RAN1#107-e

Agreement

If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report.

Agreement

For Rel-17

- Support RTT-based PDC method

- Support PDC method based on legacy TA-based mechanism

  - No RAN1/RAN4 specification impact expected

Agreement

For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates at least to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC.

Agreement

Send an LS to RAN2 and RAN4 with the content including:

- The agreements made in RAN1#107-e for propagation delay compensation.

- Ask RAN4 to define the following for RTT-based propagation delay compensation:

  - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking

  - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate)

  - gNB Rx-Tx time difference absolute accuracy based on SRS (including reuse existing spec if appropriate)

- Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1.

Conclusion

For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP.

Note: No RAN1 specification impact is expected for this conclusion

Agreement

For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signalling.

Conclusion

The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4.

R1-2112729	Draft LS on propagation delay compensation	Huawei

Decision: As per email decision posted on Nov 20th, the draft LS is endorsed. Final LS to RAN2/RAN4 is approved in R1-2112834.